3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2202269

# Electronic Meeting, January 17-25, 2022

Source:	OPPO

Title:	R17 60GHz Tx requirements

Agenda item:	6.16.3.1

Document for:	Approval

Introduction

In this paper, we provide our view on Tx RF requirements in FR2-2.

Discussion

The FR2-2 peak EIRP calculation can be found in below table with 8 antenna elements.

Table 1: UE minimum peak EIRP for handheld UE in FR2-2

| Parameter | Unit | Value |
| --- | --- | --- |
| Antenna element number |  | 8 |
| Element gain | dBi | 4 |
| Polarization gain | dB | 2.5 |
| Antenna roll-off loss vs frequency | dB | -2.5 |
| Pout | dBm | 2.5 |
| Array gain | dB | 9 |
| Total IM Loss (worst case) | dB | -12.5 |
| Minimum Peak EIRP | dBm | 12 |

Proposal 1: For handheld UE with 8 antenna elements, proposed to define min Peak EIRP as 12dBm.

Conclusion

In this contribution, we provided our views on peak EIRP for handheld UE for FR2-2.

Proposal 1: For handheld UE with 8 antenna elements, proposed to define min Peak EIRP as 12dBm.