3GPP TSG-RAN WG4 Meeting #101-bis-e                                      R4-2202294

Electronic Meeting, January 17-25, 2022

Source: 	Huawei, HiSilicon

Title: 	WF on simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combinations

Agenda Item:	5.39

Document for:	Approval

# MSD threshold

Candidate options:

- Option 1: The threshold should be higher than 29.8(2 antenna ports) / 32.5(4 antenna ports) dBm. If there are some difficulties to define those values from the implementation perspective, they should be clarified and considered when defining the threshold.

- Option 2: The threshold value should be higher than 32.5dB.

- Option 3: MSD threshold could be decided from some system level simulation.

- Option 4: It is proposed to consider specify mandatory simultaneous Rx/Tx capability case by case for FR1 FDD-TDD band combinations at least in Rel-17, i.e. whether a FDD-TDD band combination mandatory support simultaneous Rx/Tx is based on operator’s request.

- Option 5: Others (please specify)

Observation:

- Previous agreement is that simultaneous Rx/Tx is the default capability in FDD-TDD FR1 band combinations, the remaining issue is the MSD threshold

- Previously mandating a FR1 FDD-TDD combo to support simultaneous Rx/Tx capability is based on request by operators,  which is not efficient and most of the work are duplicated

- It seems difficult to reach consensus on the specific MSD threshold based on discussions in these two meetings

- For FR1 FDD-TDD band combinations with MSD, so far there is no identified combination which cannot support simultaneous Rx/Tx operation

Proposed WF

- No need to further discuss the MSD threshold for FR1 FDD-TDD band combinations

- If a FR1 FDD-TDD band combination is identified which cannot support simultaneous Rx/Tx operation, a note similar to FR1 TDD-TDD band combination shall be indicated in the specification, and for such operation the minimum requirements are not applicable for this band combination

# References

[1] R4-2202216, Email discussion summary for [101bis-e][116] LTE_NR_Other_WI, Huawei, HiSilicon