3GPP TSG-RAN WG4 Meeting #101-bis-e                                      R4-2202300

Electronic Meeting, January 17-25, 2022

Source: 	Huawei, HiSilicon

Title: 	WF on PC2 intra-band contiguous UL CA with UL MIMO

Agenda Item:	6.3.2

Document for:	Approval

# Topic #1: MPR for contiguous UL CA

## Issue 1-1: MPR for 23+26dBm

Candidate options:

  - Option 1. 1T PC2 MPR for CA is applied (i.e. Current MPR in 6.2A.2 for PC2)

  - Option 2: Same as 2T 23+23 with delta MPR based on measurement data

Agreement: for MPR for 23+26dBm for contiguous UL CA

- For 1LO without TxD indication [and for 2LO cases for bandwidth class C only], 1T PC2 MPR is applied for CA implemented

- For TxD, same as 2T 23+23 with delta MPR based on measurement data

## Issue 1-2: Signalling for UL CA

Candidate options:

  - Remove limitation to absence of dualPA-Architecture IE reporting

  - Add that uplinkTxDC-TwoCarrierReport-r16 is applicable to bandwidth class C

Agreement:

1) dualPA-Architecture IE is only considered for CA bandwidth class C

2) uplinkTxDC-TwoCarrierReport-r16 is not used to differentiate MPR requirements for different architectures

## Issue 1-3: Spec change for delta MPR with indication of TxD

Candidate options:

  - Option 1: Clarification Note in the existing MPR table for PC3 and PC2

  - Option 2: Separate Tables with relaxed MPR

Tentative agreements:

Option 2 with separate tables for the relaxed MPR.

Agreement

- To define separate MPR tables for CA with indication of TxD

  - No new signaling is needed as reply LS from RAN2

Discussion:

# Topic #2: MPR for contiguous UL CA + UL MIMO

## Issue 2-1: Whether UE architectures of 23+23, 23+26, 26+26 should be explicitly indicated in the spec

Candidate options:

  - Option 1. Yes

  - Option 2. No

Tentative agreements:

Option 2. No explicitly indication of specific UE architectures in the spec, i.e. 23+23, 23+26, 26+26.

Agreement

No explicitly indication of specific UE architectures in the spec, i.e. 23+23, 23+26, 26+26.

## Issue 2-2: Whether new UE capabilities needed to be introduced for CA+MIMO other than existing capabilities

Candidate options:

  - Option 1. Yes

  - Option 2. No

Tentative agreements:

Option 1. No new capability needs to be introduced to distinguish the applicable MPR requirements.

Agreement:

No new capability needs to be introduced to distinguish the applicable MPR requirements.

- The existing capability can be reused to ensure correctly mapping the separate requirements to UE

## Issue 2-3: MPR for 23+26dBm

Candidate options:

  - Option 1. 1T PC2 MPR for CA is applied (i.e. Current MPR in 6.2A.2 for PC2)

  - Option 2. Depends on whether TxD is indicated, i.e.

    - 1T PC2 MPR for CA w/o TxD

    - 2T 23+23 relaxed MPR w/ TxD

  - Option 3. Using delta requirements (i.e. 2T 23+23 relaxed MPR) for all architectures for CA+UL-MIMO.

  - Option 4:

    - If PC2 UL contiguous CA and UL MIMO being used simultaneously, MPR for 2T 23+23

    - If PC2 UL contiguous CA but UL MIMO NOT being used simultaneously, MPR for 1T PC2Option 1

Proposed WF

1) MPR for 1T PC2  for PC2 UL contiguous CA w/o UL MIMO or TxD indication

2) MPR for 2T 23+23  for PC2 UL contiguous CA with UL MIMO or TxD indication

Skyworks: we should have exact the same MPR as the non-MIMO case.

Agreement: MPR for 2T 23+23 can apply for PC2 UL contiguous CA with UL MIMO indication.

## Issue 2-4: MPR for 23+23, 26+26dBm

Candidate options:

  - Option 1. 1T PC2 MPR for CA is applied (i.e. Current MPR in 6.2A.2 for PC2)

  - Option 2. Depends on whether TxD is indicated, i.e.

    - 1T PC2 MPR for CA w/o TxD

    - 2T 23+23 relaxed MPR w/ TxD

  - Option 3. Using delta requirements (i.e. 2T 23+23 relaxed MPR) for all architectures for CA+UL-MIMO.

  - Option 4:

    - If PC2 UL contiguous CA and UL MIMO being used simultaneously, MPR for 2T 23+23

    - If PC2 UL contiguous CA but UL MIMO NOT being used simultaneously, MPR for 1T PC2Option 1

Agreement:

- MPR for 1T PC2 is applied for PC2 UL contiguous CA w/o UL MIMO or TxD indication

- MPR for 2T 23+23 is applied for PC2 UL contiguous CA with UL MIMO and/or TxD indication

Skyworks: 26+26 and 23+23 are the baseline assumptions for this work.

- FFS how to map the requirements to UE considering the following proposal

  - Reuse the existing modified MPR signaling rather than delta-MPR to map the requirements to UE

# References

[1] R4-2202218, Email discussion summary for [101-bis-e][118] NR_RF_FR1_enh_IntraHPUE, Huawei, HiSilicon