3GPP TSG-RAN WG4 Meeting # 101-bis-e												R4-2202311

Electronic Meeting, 17 – 25 January, 2022

Agenda item:			5.29

Source:	Man Hung Ng (Nokia)

Title:	Email discussion summary for [101-bis-e][111] LTE_NR_HPUE_FWVM

Document for:	Information

# Introduction

Email discussion for contributions submitted under agenda item 5.29 for High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, Band 13, Band n5, Band n13, and Band n71.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discussion and potential approval of TR 37.828 v0.1.0, TPs to TR 37.828 on coexistence studies and UE RF requirements.

- 2nd round: Approval of TR 37.828 v0.1.0, TPs to TR 37.828 on coexistence studies and UE RF requirements.

# Topic #1: General

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200703 | Nokia, Nokia Shanghai Bell | Proposal 1: Approve TR 37.828 v0.1.0 Observation 1: |

## Open issues summary

### Sub-topic 1-1

Issue 1-1: TR 37.828 v0.1.0

- Proposals

  - Option 1: Approve the TR.

  - Option 2: Revise the TR.

- Recommended WF

  - TBA

### Sub-topic 1-2

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Approve the TR. |
| Ericsson | Option 1 |
| Huawei | Option 1 |

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: All received comments support option 1 to approve the TR, so the TR can be approved. Candidate options: Recommendations for 2nd round: No need for 2nd round. |

### CRs/TPs

## Discussion on 2nd round (if applicable)

# Topic #2: Coexistence Studies

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200410 | Nokia, Nokia Shanghai Bell | Proposal 1: Approve TP to TR 37.828: Coexistence study for High-power UE Vs adjacent channel Public Safety operation for fixed-wireless/vehicle-mounted use cases in Band 5 and Band n5 Observation 1: |

## Open issues summary

### Sub-topic 2-1

Issue 2-1: TP to TP to TR 37.828: Coexistence study for High-power UE Vs adjacent channel Public Safety operation for fixed-wireless/vehicle-mounted use cases in Band 5 and Band n5

- Proposals

  - Option 1: Approve the TP.

  - Option 2: Revise the TP.

  - Option 3: Postpone the TP.

- Recommended WF

  - TBA

### Sub-topic 2-2

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Approve the TP. |
| Ericsson | Option 1 |
| Qualcomm | The study states that blocking is dependent on the selectivity of the PS device and therefore coex with PS may or may not be ensured. Do we know what is the selectivity required? Do we know if PS devices can meet this? Otherwise, it seems that 3GPP can only state that we are unsure whether coexistence can be met or not with PC1 Band 5/n5. And in that case, how should 3GPP proceed with defining PC1 for Band 5/n5? |
| Huawei | Option 1 |

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Three comments support option 1, but one comment raises questions on the required selectivity to ensure coexistence and whether PS devices can meet this, and how should 3GPP proceed with defining PC1 for Band 5/n5 if we are unsure whether coexistence can be met. Further discussion is needed in the 2nd round. Recommendations for 2nd round: Continue to discuss the above questions and revise the TP if needed. |

### CRs/TPs

## Discussion on 2nd round (if applicable)

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Nokia | Response to Qualcomm: As we discussed as an example in the paper, a (31+53=) 84 dB selectivity is required to make the attenuated HPUE carrier power equal to the Band 5/n5 HPUE -53 dBm/6.25 kHz OOBE (if specified in TS 36.101 and TS 38.101-1) received by the NBPS portable device receiver. Given that 72 dB ACS is agreed as system parameter for PSNB portable device [8], 12 dB higher selectivity is required for PSNB portable device against the interfering Band 5/n5 HPUE carrier power which is at least 2 MHz away from the NBPS portable device receive frequency band. RAN4 should investigate whether public safety receiver (especially for the portable device) can meet the selectivity requirement against the interfering Band 5/n5 HPUE carrier power before proceeding with defining Band 5/n5 HPUE to ensure satisfactory coexistence between HPUE in Band 5/n5 and adjacent channel public safety operation. The TP has been revised to clarify the above points, please check if the revision is ok for approval. |
| Qualcomm | To Nokia, thank you for the response and for taking our comments into consideration. We are fine with the revision. |

## Summary for 2nd round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: TP is revised in R4-2202288 to clarify the questions raised in 1st round, revised TP is agreeable and thus can be approved. |

# Topic #3: UE RF requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200704 | Nokia, Nokia Shanghai Bell | Proposal 1: NR PC3 MPR is valid also for NR PC1 operation for all NR bands Observation 1: |

## Open issues summary

### Sub-topic 3-1

Issue 3-1: MPR study for PC1 FWA device

- Proposals

  - Option 1: Approve proposal 1.

  - Option 2: Revise proposal 1.

  - Option 3: Postpone proposal 1.

- Recommended WF

  - TBA

### Sub-topic 3-2

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Approve proposal 1. |
| Ericsson | Option 1 |
| AT&T | Option 1. The existing PC1 MPR table for n14 in Table 6.2.2-5 of 38.101-1 should be updated accordingly to show that the PC1 MPR table is generic at the next meeting. This would be in-line with the discussions concerning PC1 for n14 if the PC1 requirements could be generically applied to other operating bands. |
| Qualcomm | Can the details of the PA used in the simulations be shared? Is this an actual PC1 PA? Is this a PA for which data sheets were provided at the last meeting? Our understanding is that those PA’s were not suitable for an FWA device. The proposal is also to generalize this conclusion to all bands, but is that limited to FWA since that is the subject of this study? |
| Huawei | Could you please provide a reference to the used PA model? (similar to the freescale model used for B5, R4-082001 or a Rapp PA model or … ) |

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Three comments support option 1, but two comments raise questions on of the PA used in the simulations and whether the proposal should be limited to FWA which is the subject of this study. Recommendations for 2nd round: Continue to discuss the above questions and revise the TP if needed. |

### CRs/TPs

## Discussion on 2nd round (if applicable)

Sub topic 3-1

| Company | Comments |
| --- | --- |
| Nokia | To Qualcomm and Huawei about PA model. We have used normal standardization method for determining if PC3 MPR is sufficient for PC1. That is we take a PA model and bias that so that it just meets emission requirements for 5 and 10 MHz channel bandwidths which already are specified for PC1. That is ACLR is 37 dBc, general SEM and spurious emissions are just met. Then we check other channel bandwidths especially those which have highest SU. It turns out that needed backoff is less that current PC3 MPR for all channel bandwidths. Using this kind of method is normal in RAN4 as requirements are forward looking. We are just creating the market and all components are not readily available. This proposal to generalize MPR to all bands could be limited for FWA as it is the objective of this WI. Also there are no PC1 filters which would fit to smart phones anyways. |
| Huawei | Thanks Nokia for the comment. We would like to have more time to make the analysis on our own and bring the result for the next meeting to include the discussion with different perspectives. |
| Qualcomm | We understand the challenges associated with finding a suitable PA model. I’m not confident that I’ll be successful, but there is a single PC1 implementation that we are aware of and are trying to obtain the PA characteristics from it. |
| Nokia | Thank you Huawei and Qualcomm for at least trying to provide data for next meeting. Let’s continue in February then. |

## Summary for 2nd round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Two companies asked for more time to study and provide results for the next meeting, so original TP in R4-2200704 can be noted and revised TP in R4-2202289 can be withdrawn. |

# Recommendations for Tdocs

## 1st round

New tdocs

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-22xxxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2200703 | TR 37.828 v0.1.0 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2200410 | TP to TR 37.828: Coexistence study for High-power UE Vs adjacent channel Public Safety operation for fixed-wireless/vehicle-mounted use cases in Band 5 and Band n5 | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2200704 | MPR study for PC1 FWA device | Nokia, Nokia Shanghai Bell | Noted |  |

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-22xxxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2200410 | TP to TR 37.828: Coexistence study for High-power UE Vs adjacent channel Public Safety operation for fixed-wireless/vehicle-mounted use cases in Band 5 and Band n5 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2200704 | MPR study for PC1 FWA device | Nokia, Nokia Shanghai Bell | Withdrawn | Original TP in R4-2200704 can be noted. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Petri Vasenkari | petri.j.vasenkari@nokia.com |
| Ericsson | Dominique Everaere | dominique.everaere@ericsson.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |
| Qualcomm | Gene Fong | gfong@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)