3GPP TSG-RAN WG4 Meeting # 101-BIS-e 													R4-2202324

Electronic Meeting, 17th – 25th January, 2022

Agenda item:			6.15.1, 6.15.2

Source:	Moderator (LG Electronics)

Title:	Email discussion summary for [101-BIS-e][124] NRSL_enh_Part1

Document for:	Information

Introduction

In this paper, RAN4 treats the SL_enh_part1 in Rel-17 for operating bands, system parameters, and UE transmitter/Receiver requirements for SL enhancement including advanced 5G V2X, Public safety usage and other commercial use cases with PC5 operation.

The provided technical docs list of email discussion are shown in the existing Tdocs in recommenations for Tdocs Part.

Candidate target of email discussion for 1st round is listed as follows.

- 1st round: Discuss NR SL enhancement UE RF requirements to support PS service and other SL operation.

  - Topic #1: SL enhancement UE RF requirements

  - Sub-Topic #1-1: MOP for inter-band V2X con-current operation

    - Sub-Topic #1-1-1: inter-band V2X con-current UE in TS38.101-1 in Rel-17

    - Sub-Topic #1-1-2: inter-band V2X con-current UE in TS38.101-3 in Rel-17

  - Sub-Topic #1-2: REFSENS requirements

    - Sub-Topic #1-2-1: REFSENS requirements in n14

  - Sub-Topic #1-3: TPs and CRs for SL enh. UE RF requirements

    - Sub-Topic #1-3-1: TxD

    - Sub-Topic #1-3-2: PS UE in n14

  - Sub-Topic #1-4: Other RF requirements

    - Sub-Topic #1-4-1: Addition of definitions and symbols to Chapter 3 in TR38.785

    - Sub-Topic #1-4-2: Updated TR 38.785 v0.5.0

- 2nd round: The following issues will be further discussed

  - Alomst open issues are cleared in 1st round. Just focus on the revised Tdocs in 2nd round as following documents

| Tdoc number | Title | Source | Recommandation |
| --- | --- | --- | --- |
| R4-2200139 | Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be endorsed |
| R4-2200141 | Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be endorsed |
| R4-2200833 | TR38.785 v0.5.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | Will be agreed |
| R4-2202355 (Rev_R4-2200946) | TP for TR 38.785: Addition of definitions and symbols to Chapter 3 | vivo | Will be approved |
| R4-2202407 (Rev_R4-2200842) | TP on RF requirements for NR PS UE in n14 for NRSL_enh WI in Rel-17 | LG Electronics France | Will be approved |
| R4-2202408 (Rev_R4-2200848) | Draft CR on RF requirements for SL enhancement for public safety service in n14 | LG Electronics France | Will be endorsed |

- Topic #1: SL enhancements UE RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200138 | CATT | CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16 and Rel-17) Provide draft CR to change MOP for inter-band V2X con-current UE in TS38.101-1 in Rel-16 & Rel-17 6.2E.1.2 UE maximum output power for V2X con-current operation For the inter-band con-current NR V2X operation, the maximum output power specified in clause 6.2.1 shall apply for the uplink in licensed and the maximum output power specified in clause 6.2E.1 shall apply for the sidelink in licensed band or Band n47. The period of measurement shall be at least one sub frame (1ms). - Table 6.2E.1.2-1: void |
| R4-2200140 | CATT | CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16 and Rel-17) Provide draft CR to rchange MOP for inter-band V2X con-current UE in TS38.101-3 in Rel-16 & Rel-17 - 6.2E.1.3 UE maximum output power for Inter-band V2X For inter-band V2X with transmission assigned to one E-UTRA band and one NR band, the maximum output power shall apply on each component carrier as defined in clause 6.2.2 or clause 6.2.2G in TS 36.101 [4] and in clause 6.2.E.1.1 or clause 6.2.1.1 in TS 38.101-1 [2], respectively. The period of measurement shall be at least one sub frame (1ms). - Table 6.2E.1.3-1: void |
| R4-2200509 | Qualcomm Incorporated | n14 REFSENS for PS in licensed band Observation 1: The NR V2X SNR requirements for RB≤24 is 0.5dB higher than for RB>24 due to worse channel estimation performance Observation 2: For PS operating on sidelink the same SNR and IM values must be used for calculating PS REFSENS as used for NR V2X for a given number of RB Proposal 1: Use additional IM=0.5 dB for RB≤24 for calculating REFSENS for PS as was done for NR V2X Proposal 2: Use the REFSENS (RefSensNRn14) values presented in table 1 for NR n14 PS |
| R4-2200842 | LG Electronics | TP on the RF requirements for PS service with sidelink operation in n14 Provide TP for RF requirements for PS UE to support PC1 and PC3 in n14. - Reuse MPR for PC3 for PC1 PS UE using SL operation as same as NR Uu. - Define NS_06 for A-SEM requirements - Configured Tx power, min. output power, off power, ON/OFF time mask and Power control and Frequency error and time aligement error shall follow NR V2X RF requirements. - Transmit modulation quality, occupied bandwidth, out-of-band emission, SEM/A-SEM, ACLR, Spurious emission and transmit intermodulation shall follow NR Uu RF requirements. |
| R4-2200848 | LG Electronics | Draft CR on RF requirements for NR PS service with sidelink operation Provide draft CR to introduce NR PS UE RF requirements to support PC1 and PC3 in n14. - Propose detail RF requirements for NR PS UE based on the agreed RF requirements in TR38.785 and proposed TP in R4-2200842. |
| R4-2200946 | vivo | TP for TR 38.785: Addition of definitions and symbols to Chapter 3 In this text proposal, definitions and symbols for SL enhancement were added on the latest version of TR 38.785 - See the detail contents in TP |
| R4-2201952 | Huawei | TP for 38.785: TxD requirements for NR V2X RAN4 agreed that TxD is a left over issue for NR V2X, and the requirements will be included after the general TxD requirements are available. The proposed contents are based on the approved CR in R4-2115100. |
| R4-2201953 | Huawei | Big CR: introduction of TxD requirements for NR V2X Based on proposed TP, the detail TxD RF requirements for NR V2X UE were provided in TS38.101-1 in Rel-17. |
| R4-2200833 | LG Electronics | TR38.785 v0.5.0 TR Update for SL enhancement in Rel-17 It will be reserved to capture the all approved TPS and WF in this meeting |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Based on provided contributions, RAN4 discusses the detailed NR UE RF Tx/Rx requirements for SL enhancement service.

  - Topic #1: SL enhancement UE RF requirements

  - Sub-Topic #1-1: MOP for inter-band V2X con-current operation

    - Sub-Topic #1-1-1: MOP for inter-band V2X con-current UE in TS38.101-1 in Rel-17

    - Sub-Topic #1-1-2: MOP for inter-band V2X con-current UE in TS38.101-3 in Rel-17

  - Sub-Topic #1-2: REFSENS requirements

    - Sub-Topic #1-2-1: REFSENS requirements in n14

  - Sub-Topic #1-3: TPs and CRs for SL enh. UE RF requirements

    - Sub-Topic #1-3-1: TxD RF requirements

    - Sub-Topic #1-3-2: PS UE RF requirements in n14

  - Sub-Topic #1-4: Other RF requirements

    - Sub-Topic #1-4-1: Addition of definitions and symbols to Chapter 3 in TR38.785

    - Sub-Topic #1-4-2: Updated TR 38.785 v0.5.0

    - Sub-topic #1-1

Sub-topic description: MOP for inter-band V2X con-current operation

Open issues and candidate options before e-meeting:

Issue 1-1-1: MOP for inter-band V2X con-current UE in TS38.101-1 in Rel-16 & Rel-17

- Proposals

  - Option 1: Based on CATT paper (R4-2200138), RAN4 remove the MOP table for inter-band con-current V2X UE and follow indivisual MOP per Uu and SL.

  - Option 2: Keep the current MOP Table and contents for inter-band V2X con-current UE.

- Recommended WF

  - TBA

Issue 1-1-2: MOP for inter-band V2X con-current UE in TS38.101-3 in Rel-16 & Rel-17

- Proposals

  - Option 1: Based on CATT paper (R4-2200140), RAN4 remove the MOP table for inter-band con-current V2X UE and follow indivisual MOP per Uu and SL.

  - Option 2: Keep the current MOP Table and contents for inter-band V2X con-current UE.

- Recommended WF

  - TBA

    - Sub-topic #1-2

Sub-topic description: REFSENS requirements

Open issues and candidate options before e-meeting:

Issue 1-2-1: REFSENS requirements in n14

- Proposals

  - Option 1: REFSENS (R4-2200509, Qualcomm) requirements are acceptable for n14 NR SL UE with 0.5dB IM less than 24RB. The proposed levels are same REFSENS requirements in TR38.785 v0.4.0

  - Option 2: Do not allow 0.5 dB additional IM for REFSENS in PS UE.

- Recommended WF

  - TBA.

    - Sub-topic #1-3

Sub-topic description: TPs and CRs for SL enh. UE RF requirements

Open issues and candidate options before e-meeting:

Issue 1-3-1: TxD RF requirements

- Proposals

  - Option 1: Based on TP (R4-2201952, Huawei) and big CR (R4-2201953), RAN4 can accept the proposed TxD RF requirements in TR38.785 and TS38.101-1.

  - Option 2: Need further discussion on the each RF requirement and determine the detailed contents based on 1st round comments.

- Recommended WF

  - TBA

Issue 1-3-2: PS UE RF requirements in n14

- Proposals

  - Option 1: Based on TP (R4-2200842, LGE) and draft CR (R4-2200848), RAN4 can accept the proposed PS UE RF requirements in TR38.785 and TS38.101-1

  - Option 2: Need further discussion on the each RF requirement and determine the detailed contents based on 1st round comments.

- Recommended WF

  - TBA

    - Sub-topic #1-4

Sub-topic description: Other RF requirements

Open issues and candidate options before e-meeting:

Issue 1-4-1: Addition of definitions and symbols to Chapter 3 in TR38.785

- Proposals

  - Option 1: Based on TP (R4-2200946, vivo), RAN4 can update the TR contents.

  - Option 2: Some definitions are needed in the TR38.785. For example, the con-current operation definition are already specified in TS38.101-1 in Rel-16.

- Recommended WF

  - TBA

Issue 1-4-2: Updated TR 38.785 v0.5.0

- Proposals

  - Option 1: updated TR will be treated in 2nd round to capture the approved TPs in this meeting.

- Recommended WF

  - Option 1 is acceptable

Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1: MOP for inter-band V2X con-current operation

Issue 1-1-1: MOP for inter-band V2X con-current UE in TS38.101-1 in Rel-17

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Either is ok. Slightly prefer Option 1, to not cause further ambiguity in the spec. |
| CATT | Support Option 1 to avoid misunderstanding and ambiguity between MOP and configured transmitted power. |
| LGE | Support option 2. If RAN4 specified licensed band (Uu) + licensed band (SL), then need to meet the restricted power in licensed bands. For the combinations of the licensed band + n47 or B47, we are fine since there are no limitation in ITS spectrum. Anyway, we can prefer to keep and define total power class if the licensed band + licensed band combination lin future release. |
| Vivo | Option 1 is OK if we agreed no upper power bound for inter-band V2X con-current operation. |
| Qualcomm | Option 1 |
| Xiaomi | Option 1 is OK. |

Issue 1-1-2: MOP for inter-band V2X con-current UE in TS38.101-3 in Rel-17

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Either is ok. Slightly prefer Option 1, to not cause further ambiguity in the spec. |
| CATT | Support Option 1 to avoid misunderstanding and ambiguity between MOP and configured transmitted power. |
| LGE | Same comment in issue 1-1-1. |
| Vivo | Option 1 is OK if we agreed no upper power bound for inter-band V2X con-current operation. |
| Qualcomm | Option 1 |
| Xiaomi | Option 1 is OK. |

Sub topic 1-2: REFSENS requirements

Issue 1-2-1: REFSENS requirements in n14

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | For the proposed MSD in R4-2200508, the analysis is for harmonic, but the TP is for cross band isolation. Moderator: sorry for the wrong Tdoc number, the reference paper is R4-2200509. It was fixed in this version in session 1.2.2. |
| CATT | QC’s paper is R4-2200509 and we are OK with Option 1 |
| LGE | Yes, we are fine with option 1 |
| Qualcomm | Option1 |
| AT&T | We are OK with Option 1. |

Sub topic 1-3: TPs and CRs for SL enh. UE RF requirements

Issue 1-3-1: TxD RF requirements

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | The TxD requirements are based on agreed general TxD phase 1 CR in R4-2115100 and existing SL requirements. |
| CATT | OK with Option 1. |
| LGE | Support option 1 |
| vivo | OK with Option 1. |
| Qualcomm | Option1 |

Issue 1-3-2:  NR PS UE RF requirements in n14

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Since n14 only has CBW of 5MHz and 10MHz, for the requirements relevant to CBW, e.g. ACLR, A-MPR, etc, should we also need to consider the requirements other than the supported CBW for n14 as proposed in the draft CR? |
| CATT | OK with option 1. |
| LGE | Support option 1. PS UE can be expanted in other operating bands. So we can refer the exisiting contents in draft CR. If other companies would like to remove other CBW, then we can revised the draft CR based on consensus. |
| Vivo | OK with Option 1. |
| Qualcomm | Option 2: Should further check to determine if the MPR values for PSCCH/PSSCH, S-SSB and PSFCH defined for PC3 should be used for PC1 as well. Also should further discuss whether the FRCs for the RX specifications should be from appendix 7 (V2X) instead of from appendix 3 |
| AT&T | Option 1. Although the brackets can be removed from REFSENS values if we agree with the QC proposal in R4-2200509. We do not think that the MPR values need further checking as suggested by QC as the same MPR assumptions can be taken as determined for Uu operation. |

Sub topic 1-4: Other RF requirements

Issue 1-4-1: Addition of definitions and symbols to Chapter 3 in TR38.785

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Ok with the proposed TP. |
| CATT | For intra-band V2X operation/V2X con-current operation or inter-band V2X con-current operation, we also believe it may cause ambiguity with no further clarification of each definition. Until now it involves several RATs (LTE V2X, NR V2X, LTE Uu and NR Uu) with different resource sharing modes (TDM or FDM) for intra-band or inter-band. In future, it tends to be more complex with a variety of combinations. So we think it is necessary to further clarify these definitions as mentioned in the TP. Intra-band V2X operation: LTE V2X carrier and NR V2X carrier in the same operating band. Intra-band V2X con-current operation: NR SL carrier and NR Uu carrier in the same licensed band. Intra-band contiguous V2X operation：Contiguous LTE V2X carrier and NR V2X carrier are in the same operating band. Intra-band contiguous V2X con-current operation：Contiguous NR SL carrier and NR Uu carrier are in the same licensed band. Inter-band con-current operation: NR Uu carrier and NR SL carrier in different operating bands. Intra-band V2X operating UE: UE support the intra-band V2X con-current operation. In Rel-16, RAN4 defined intra-band V2X operation since only LTE V2X and NR V2X operate with TDM mode in ITS band in which no Uu is involved. When it comes to Rel-17, intra-band V2X and Uu operation with TDM and FDM are introduced. Also, it seems not approriate to call it intra-band con-current operation for the case SL and Uu operate with TDM mode in a licensed band. RAN4 already have the definition for con-current operation that clearly indicate simultaneous transmission and reception of sidelink and Uu. According to the definition, FDM is the case instead of TDM. Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface. |
| LGE | Fine with option 2. Removed the detined definitions in TS38.101-1 or TS38.101-3 in rel-16. And add additional definition and symbols in TR38.785. |
| vivo | If we agreed to introduce the definitions and symbols in TR 38.785, we can revise our tdoc to capture CATT’s clarification. |
| Qualcomm | Option 1 |
| Xiaomi | Option 1. |

Issue 1-4-2:  Updated TR38.785 v0.5.0

| Company | Comments |
| --- | --- |
| LGE | The TR will be treated after 1st round to capture the approved TPs. |
| Qualcomm | Option 1 |

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200138 | Company A |
|  | Company B |
|  | … |
| R4-2200140 | Company A |
|  | Company B |
|  | … |
| R4-2200842 | Qualcomm : Same comment as for issue 1-3-2 |
|  | Company B |
|  | … |
| R4-2200848 | Qualcomm : Why are the FRCs for the RX section referencing appendix 3?. As this is PS on V2X should it not reference appendix 7 in 38.101-1? |
|  | Company B |
|  | … |
| R4-2200946 | Company A |
|  | Company B |
|  | … |
| R4-2201952 | Company A |
|  | Company B |
|  | … |
| R4-2201953 | Company A |
|  | Company B |
|  | … |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1: MOP for inter-band V2X con-current operation | Issue 1-1-1: MOP for inter-band V2X con-current operation in TS38.101-1 in Rel-16 & Rel-17 - Tentative agreements - Option 1: Based on CATT paper (R4-2200138), RAN4 remove the MOP table for inter-band con-current V2X UE and follow indivisual MOP per Uu and SL. - Recommendations for 2nd round: The CATT draft CR (R4-2200138) will be endorsed. And no further discussion in 2nd round. |
|  | Issue 1-1-2: MOP for inter-band V2X con-current operation in TS38.101-1 in Rel-16 & Rel-17 - Tentative agreements - Option 1: Based on CATT paper (R4-2200140), RAN4 remove the MOP table for inter-band con-current V2X UE and follow indivisual MOP per Uu and SL. - Recommendations for 2nd round: The CATT draft CR (R4-2200140) will be endorsed. And no further discussion in 2nd round. |
| Sub-Topic#1-2: REFSENS requirements | Issue 1-2-1: REFSENS requirements in n14 - Tentative agreements - Option 1: REFSENS (R4-2200509, Qualcomm) requirements are acceptable for n14 NR SL UE with 0.5dB IM less than 24RB. The proposed levels are same REFSENS requirements in TR38.785 v0.4.0 - Recommendations for 2nd round: The [] in REFSENS level will be removed in TR. And no further discussion in 2nd round. |
| Sub-topic#1-3: TPs and CRs for SL enh. UE RF requirements | Issue 1-3-1: TxD RF requirements - Tentative agreements - Option 1: Based on TP (R4-2201952, Huawei) and big CR (R4-2201953), RAN4 can accept the proposed TxD RF requirements in TR38.785 and TS38.101-1. - Recommendations for 2nd round: The CATT TP (R4-2201952) and draft CR (R4-2201953) will be approved and endorsed. And no further discussion in 2nd round. |
|  | Issue 1-3-2: NR PS UE RF requirements in n14 As AT&T mentioned the MPR of PC3 UE for NR Uu was reused for PC1 MPR in NR PS Uu in n14. Same principle will be considered since the PC1 UE is not smart phone type UE. - Tentative agreements - Option 1: Based on TP (R4-2200842, LGE) and draft CR (R4-2200848), RAN4 can accept the proposed PS UE RF requirements in TR38.785 and TS38.101-1 - Recommendations for 2nd round: The LGE TP (R4-2200842) and draft CR (R4-2200848) will be approved and endorsed. And no further discussion in 2nd round. |
| Sub-topic#1-4: Other RF requirements | Issue 1-4-1: Addition of definitions and symbols to Chapter 3 in TR38.785 - Tentative agreements - Option 2: It will be revised based on 1st round comments. - Recommendations for 2nd round: Based on the revised TP, RAN4 can further discuss for the additional definitions and synbols in 2nd round |
|  | Issue 1-4-2: Updated TR38.785 v0.4.0 - Tentative agreements - Option 1: updated TR will be treated in 2nd round to capture the approved TPs in this meeting. - Recommendations for 2nd round: The revised TR shall be provided in 2nd round to capture the approved TPs in this meeting. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2200138 | The draft CR can be endorsed. |
| R4-2200139 | The mirror CR will be provided in 2nd round |
| R4-2200140 | The draft CR can be endorsed. |
| R4-2200141 | The mirror CR will be provided in 2nd round |
| R4-2200833 | It will be provided in 2nd round |
| R4-2200842 | The TP will be revised based on 1st round comments |
| R4-2200848 | The draft CR can be revised based on 1st round comments. |
| R4-2200946 | The TP will be revised based on 1st round comments |
| R4-2201952 | The TP will be approved |
| R4-2201953 | The draft CR can be endorsed. |

Discussion on 2nd round (if applicable)

    - Open issues (if applicable)

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2200842 | TP on RF requirements for NR PS UE in n14 for NRSL_enh WI in Rel-17 Qualcomm: Should further discuss whether the FRCs for the RX specifications should be from appendix 7 (V2X) instead of from appendix 3 To QC from Moderator: I requested revisd Tdoc number, but I wait other companies comments for the reference FRC for PS UE in n14. If interested companies are fine with A.7.x, then I will revised TP. AT&T: If the SL RMC can be used and provides the same expected REFSENS performance as with the RMC defined in Annex A.3.2, we would be OK with it. However, how will we treat the UL RMC since this was the previous agreement on REFSENS that we would use the same UL RB allocation as Uu and not a full RB allocation as done for V2X previously? Is the UL configuration specified in Table 7.3E.2-2 sufficient or do we also refer back to the UL RMC Annex in A.2.2.2.1? Also, how do we address 5MHz for SL RMC? Will this be addressed at the next meeting? LGE: we are fine to use RMC for NR SL operation with A.7.x. For the 5MHz RMC, RAN4 can finalize and define the 5MHz RMC in Feb. meeting. |
| R4-2200848 | Draft CR on RF requirements for SL enhancement for public safety service in n14 Qualcomm: Should further discuss whether the FRCs for the RX specifications should be from appendix 7 (V2X) instead of from appendix 3 To QC from Moderator: I requested revisd Tdoc number, but I wait other companies comments for the reference FRC for PS UE in n14. If interested companies are fine with A.7.x, then I will revised draft CR. AT&T: See comment above. If a revised draft CR is produced, please remove the brackets on the REFSENS values. LGE: we are fine to use RMC for NR SL operation with A.7.x. For the 5MHz RMC, RAN4 can finalize and define the 5MHz RMC in Feb. meeting. |
| R4-2200833 | TR38.785 v0.5.0 TR Update for SL enhancement in Rel-17 Updated version will be provided at the formal Tdoc deadline. |
| R4-2200139 | Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) |
| R4-2200141 | Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) |
| R4-2202355 (Rev_R4-2200946) | TP for TR 38.785: Addition of definitions and symbols to Chapter 3 |

    - Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| Tdoc number | Title | Source | Reccomandation |
| --- | --- | --- | --- |
| R4-2200139 | Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be endorsed |
| R4-2200141 | Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be endorsed |
| R4-2200833 | TR38.785 v0.5.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | Will be agreed |
| R4-2202355 (Rev_R4-2200946) | TP for TR 38.785: Addition of definitions and symbols to Chapter 3 | vivo | Will be approved |
| R4-2202407 (Rev_R4-2200842) | TP on RF requirements for NR PS UE in n14 for NRSL_enh WI in Rel-17 | LG Electronics France | Will be approved |
| R4-2202408 (Rev_R4-2200848) | Draft CR on RF requirements for SL enhancement for public safety service in n14 | LG Electronics France | Will be endorsed |

- Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200138 | Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-17) | CATT | Will be endorsed |  |
| R4-2200139 | Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be provided mirror CR in 2nd round |  |
| R4-2200140 | Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-17) | CATT | Will be endorsed |  |
| R4-2200141 | Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be provided mirror CR in 2nd round |  |
| R4-2200509 | n14 REFSENS for PS in licensed band | Qualcomm Incorporated | Noted. The contents are agreed |  |
| R4-2200833 | TR38.785 v0.5.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | Will be provided in 2nd round |  |
| R4-2200842 | TP on RF requirements for NR PS UE in n14 for NRSL_enh WI in Rel-17 | LG Electronics France | Will be approved |  |
| R4-2200848 | Draft CR on RF requirements for SL enhancement for public safety service in n14 | LG Electronics France | Will be endorsed |  |
| R4-2200946 | TP for TR 38.785: Addition of definitions and symbols to Chapter 3 | vivo | Revised to |  |
| R4-2201952 | TP for 38.785: TxD requirements for NR V2X | Huawei, HiSilicon | Will be approved |  |
| R4-2201953 | Big CR: introduction of TxD requirements for NR V2X | Huawei, HiSilicon | Will be endorsed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200139 | Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be endorsed | Shadowing draft CR of R4-2200138 |
| R4-2200141 | Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16) | CATT | Will be endorsed | Shadowing draft CR of R4-2200140 |
| R4-2200833 | TR38.785 v0.5.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | Will be agreed | All Approved TPs are captured in the update TR |
| R4-2202355 (Rev_R4-2200946) | TP for TR 38.785: Addition of definitions and symbols to Chapter 3 | vivo | Will be approved | TP to add definitions and symbols to Chapter 3 |
| R4-2202407 (Rev_R4-2200842) | TP on RF requirements for NR PS UE in n14 for NRSL_enh WI in Rel-17 | LG Electronics France | Will be approved | TP to define RF core requirements for PS UE |
| R4-2202408 (Rev_R4-2200848) | Draft CR on RF requirements for SL enhancement for public safety service in n14 | LG Electronics France | Will be endorsed | Draft CR to specify the RF core requirements for PS UE |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| LG Electronics | Suhwan Lim | Suhwan.lim@lge.com |
| CATT | Yuan Gao | gaoyuan@catt.cn |
| vivo | Shuai Zhou | shuai.zhou@vivo.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)