3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2202239

Electronic Meeting, January 17-25, 2022

Agenda item:	4

Source: 	CMCC

Title: 	Email discussion summary for [101-bis-e][139] R17_feature_list

Document for:	Information

- Introduction

This email discussion focuses on Rel-17 UE feature list discussion (agenda 4). According to RAN4 Chair guidance on UE feature list handling guidance in the meeting arrangement (slide 9), this email thread will focus on feature group discussion in UE RF, RRM demod releated feature groups will be handled in the individual session. The outcome will be taken into account in the UE feature list sent to RAN2.

Before the consensus is reached, the feature or feature group will be captured with index being labeled as FFS. After the consensus is reached, the feature or feature group will be given the index.

In the summary, except for the proposed UE features, I list all the Rel-17 WIs. For the WIs that no contribution submitted, if any feature is needed, please add it in the comments, so that it can be discussed in this meeting.

- RF requirements enhancement for NR frequency range 1 (FR1)

  - Companies’ contributions summary

.Companies’ contributions summary

| Tdocs | Title | Source |
| --- | --- | --- |
| R4-2200286 | Initial views on R17 feature list | Apple |
| R4-2201446 | Initial review of RAN4 UE feature in Rel-17 | ZTE Corporation |

  - Discussion on 1st round

Issue 1-1: Requirements for intra-band UL CA+UL MIMO

Option 1: (Apple):

In the context of requirements for intra-band UL CA + UL MIMO, differentiation of requirement applicability based on PA architecture may be needed.

It is recommended to revisit this issue once the related technical discussion converges.

Option 2: (ZTE):

| UE feature | Description |
| --- | --- |
| Intra-band contiguous CA+MIMO | Indicate whether UE supports Intra-band contiguous CA for UL-MIMO |

    - Companies’ views collection for 1st round

Issue 1-1: Requirements for intra-band UL CA+UL MIMO

| Company | Comments |
| --- | --- |
| Skyworks | We agree that signaling for 2Tx UL CA to support CA for UL MIMO is needed. We also agree that differentiation of architecture is needed between case that can only use TxD and cases with at least one full power PA for 1Tx fallback PA. furthermore, additional signaling may be needed to differentiate 2Tx MPR for case with only one full power PA and case with two full power PA. We agree with Apple that this still needs resolution in the WI. |
| HW | This new signaling is not necessary. Whether UE needs to decide the capability based on PA architecture is under discussion in email thread #118 NR_RF_FR1_enh_IntraHPUE, and we shall refer to that discussion. Even if it’s identified as needed, the existing signaling could be reused. UE could report the MIMO capability per band per band combination since Rel-15. The scheme could be reused for intra-band UL CA+UL MIMO. |
| OPPO | For option 1, the requirements for different UE architectures of CA+MIMO is still under discussion, can come back to this when that finished and clear. For Option 2, with the LS from RAN2 R4-2200016, it is clear that current RAN2 signaling can indicate whether UE supports UL CA and MIMO in one band. No new signaling is needed. |
| Ericsson | Signaling is not needed unless there is a difference is supported functionality/capability for a band combination and the possibility to configure the UE with a certain configuration. If the MPR is excessive for an architecture for a CA/UL-MIMO configuration, then this should not be considered feasible for this configuration. Indication of PA architectures of limited value (supported features/power class relevant). |
| Apple | We are fine to follow the group's decision (outcome of Tuesday's GTW) |

Issue 1-2: Other features groups for NR FR1 enhancement

| Company | Comments |
| --- | --- |
| Skyworks | Other features using 2 transmit paths (TxD, NC UL VA, contiguous ULCA) may need existing/additional signaling for different PA architectures. Again this need to be resolved in the associated WI. At this time it is unclear if NRU contiguous ULCA may require additional signaling |
| China Telecom | For Tx switching (and the corresponding DL interruption, UL-MIMO coherence), LSs on the needed capabilities have seen sent to RAN2 in previous meetings. Maybe these UE features can be added in the list in next RAN4 meeting. |
| Ericsson | A capability for supporting power limits for UL CA (power prioritization) should be introduced to allow indication in earlier releases (the capability mandatory for Rel-17). |
| Qualcomm | We have to be careful in handling the features/capabilities so that we do not have too much fragmentation. |

    - Summary for 1st round

Issue 1-1: Requirements for intra-band UL CA+UL MIMO

Option 1: (Apple):

In the context of requirements for intra-band UL CA + UL MIMO, differentiation of requirement applicability based on PA architecture may be needed.

It is recommended to revisit this issue once the related technical discussion converges.

Agreement: follow the reply LS R2-2111465 from RAN2.

Issue 1-2: Other features groups for NR FR1 enhancement

Agreement: Capture the previous agreements of capabilities for Tx switching.

- NR RF requirement enhancements for frequency range 2 (FR2)

  - Companies’ contributions summary

.Companies’ contributions summary

| Tdocs | Title | Source |
| --- | --- | --- |
| R4-2200286 | Initial views on R17 feature list | Apple |
| R4-2201446 | Initial review of RAN4 UE feature in Rel-17 | ZTE Corporation |

  - Discussion on 1st round

Issue 2-1 ~ 2-3 (Apple)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL gap for Tx power management | 2-1 | Support of UL gap for Tx power management | Capability of performing BPS sensing for Tx power management |  | yes | no | UE does not support UL gap for Tx power management | FR2 only | No | No |  |  | Optional with capability signalling |
|  | 2-2 | Support of UL gap configurations | Capability of support specific UL gap configuration | 2-1 | yes | no |  | FR2 only | No | No |  |  | Optional with capability signalling |
| UL gap for coherent UL MIMO | 2-3 | Support of UL gap for coherent UL MIMO | Capability of performing coherent UL MIMO calibration in UL gap |  | yes | no | UE does not support UL gap for coherent UL MIMO calibration | FR2 only | No | No |  |  | Optional with capability signalling |

Issue 2-4 inter-band UL CA (ZTE)

| UE feature | Description |
| --- | --- |
| Inter-band UL CA | Indicate the supported beam management type for inter-band UL CA within FR2. Beam management type can only be independent beam management (IBM). |

Issue 2-5 inter-band DL CA (ZTE)

| UE feature | Description |
| --- | --- |
| Inter-band DL CA | Indicate the supported beam management type for inter-band CA within FR2. Beam management type can be independent beam management (IBM) or common beam management (CBM), or both |

    - Companies’ views collection for 1st round

Issue 2-1: Support of UL gap for Tx power management

| Company | Comments |
| --- | --- |
| ZTE | In general, we are fine to introduce such kind of UE capability, however it’s better to be discussed under the FR2 UE RF enhancement firstly. Need of FR1 and FR2 differentiation should be yes instead of N |
| Nokia | For the UL gap for Tx power management feature it is important to clearly define as part of the feature and later as part of UE capability that the UE is also required to meet all the enhanced UE requirements and pass the related test cases if supporting this feature and thus, indicate the support of the UE capability. In our view one UE capability 2-1 ‘Support of UL gap for Tx power management’ is sufficient and this UE capability/feature description should include the definition that the UE needs to meet the related enhanced requirements and pass the corresponding test cases. Every UE supporting this feature should also support all the UE gaps that are specified for this feature to enable efficient system operations. Therefore, we propose that 2-2 is not included to the feature list and is removed from the table. |
| Ericsson | We are fine with this as UE capability. |
| Apple | As proposed, support of UL gap for Tx power management is a per UE capability |
| Intel | We support to add the capability and according to email tread discussions, this capability should be per-band. |

Issue 2-2: Support of UL gap configurations

| Company | Comments |
| --- | --- |
| ZTE | Similar comment as Issue 2-1, it’s better to be clarified whether all ULGP is mandatory or only 1st two is mandatory and rest of ULGP is optional. |
| Nokia | See also our comments above for 2-1, In our view one UE capability 2-1 ‘Support of UL gap for Tx power management’ is sufficient and this UE capability/feature description should include the definition that the UE needs to meet the related enhanced requirements and pass the corresponding test cases. Every UE supporting this feature should also support all the UE gaps that are specified for this feature to enable efficient system operations. Therefore, we propose that this 2-2 ‘Support of UL gap configurations ’ is not included to the feature list and is removed from the table. |
| Ericsson | This issue is discussed under thread ([101-bis-e][120] NR_RF_FR2_enh2_Part_22.2.1) Sub-topic 2-1: Optionality of Gap configurations. Discussion on feature list for this issue should be postponed until the conclusion of UL gaps under ([101-bis-e][120] NR_RF_FR2_enh2_Part_22.2.1. Our view is that all UL gap configurations should NOT be optional. At least some of them must be mandatory. |
| Qualcomm | We would like to understand why 2 separate capabilities are needed. Discussion should first be concluded in the specific topic. |
| Apple | Our view is support of UL gap configuration is optional. We are OK to further discuss it in thread #120. |
| Intel | We support to introduce this capability and we can further discuss whether this should be per-UE or per-band in [120]. |

Issue 2-3: Support of UL gap for coherent UL MIMO

| Company | Comments |
| --- | --- |
| ZTE | In general, we are fine to introduce such kind of UE capability, however it’s better to be discussed under the FR2 UE RF enhancement firstly. Need of FR1 and FR2 differentiation should be yes instead of No |
| Nokia | The 2-3 ‘Support of UL gap for coherent UL MIMO’ UE feature/capability should also include as part of the feature description that UE also needs to meet all the enhanced UE requirements defined for this feature and pass the related UE test cases to indicate support of this feature/capability. We agree that one UE capability/feature is sufficient for coherent UL MIMO and the same principle should also be used for the UL gap for Tx power management feature. |
| Ericsson | We do not see the need to specify UL gap for coherent UL MIMO. This can be done autonomously without any specification. But if this is specified then it should be optional and UE capability. |
| Apple | As proposed, we think support of UL gap for coherent UL MIMO should be a UE capability. |
| Intel | We support to introduce this capability as a per-band UE capability. |

Issue 2-4 inter-band UL CA

| Company | Comments |
| --- | --- |
| ZTE | In Rel-17, RAN4 only define and discuss the RF requirements for IBM for inter-band UL CA, which is different with inter-band DL CA. Therefore, we think it should distinguish the beam management capability for inter-band UL CA and inter-band DL CA. |
| HW | This signaling for UL beam management is not needed. RAN4 hasn’t discussed on beam management capability for UL CA. The discussion was mainly on DL CA which is covered by Issue 2-5. |
| Ericsson | This feature is not needed. BM indication is not needed for UL CA. It can be based on the DL indication. The indication is needed for configuring the UE with the BM. The UE should support the same CA configuration in UL and DL (at least) |
| Qualcomm | As commented in GTW, we do not see the need to differentiate between UL and DL, these should be bundled and we already have the capability. |
| Apple | Currently uplink transmission of FR2 is based on beam correspondence. BM should be the same between UL and DL, and this feature is not needed. |
| ZTE | A question: If inter-band DL CA supports CBM, then does it mean CBM is supported/indicated to inter-band UL CA? (In Rel-17, CBM is not supported in inter-band UL CA.) |

Issue 2-5 inter-band DL CA

| Company | Comments |
| --- | --- |
| ZTE | It was already agreed that beam management of ‘both’ and ‘CBM’ are applied to inter-band DL CA. So some corrections should be needed for the current description in TS38.306. In addition, for ’both’ , it is agreed that RAN4 sends out LS to ask RAN2 to define the capability for “both” and inquire whether “both” can be release independent from Rel-16. Current RAN2 signaling didn’t support ‘both’ |
| HW | The signaling is agreeable. The ongoing discussion in email thread #119 needs to be considered. |
| Nokia | Same comments Huawei |
| Ericsson | We are fine with this feature. |

Issue 2-6: Other features groups for NR FR2 enhancement

| Company | Comments |
| --- | --- |

    - Summary for 1st round

Issue 2-1 ~ 2-3 (Apple)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL gap for Tx power management | 2-1 | Support of UL gap for Tx power management | Capability of performing BPS sensing for Tx power management |  | yes | no | UE does not support UL gap for Tx power management | FR2 only | No | No |  |  | Optional with capability signalling |
|  | 2-2 | Support of UL gap configurations | Capability of support specific UL gap configuration | 2-1 | yes | no |  | FR2 only | No | No |  |  | Optional with capability signalling |
| UL gap for coherent UL MIMO | 2-3 | Support of UL gap for coherent UL MIMO | Capability of performing coherent UL MIMO calibration in UL gap |  | yes | no | UE does not support UL gap for coherent UL MIMO calibration | FR2 only | No | No |  |  | Optional with capability signalling |

- Further discuss issue 2-1~2-3 in [120].

Issue 2-4 inter-band UL CA (ZTE)

| UE feature | Description |
| --- | --- |
| Inter-band UL CA | Indicate the supported beam management type for inter-band UL CA within FR2. Beam management type can only be independent beam management (IBM). |

- Discuss issue 2-4 in [119].

Issue 2-5 inter-band DL CA (ZTE)

| UE feature | Description |
| --- | --- |
| Inter-band DL CA | Indicate the supported beam management type for inter-band CA within FR2. Beam management type can be independent beam management (IBM) or common beam management (CBM), or both |

Agreement: Indicate the supported beam management type for inter-band CA within FR2-1. Beam management type can be independent beam management (IBM) or common beam management (CBM), or both

- The capability is defined per-band combination.

- Extending current NR operation to 71GHz

  - .Companies’ contributions summary

| Tdocs | Title | Source |
| --- | --- | --- |
| R4-2200286 | Initial views on R17 feature list | Apple |
| R4-2200544 | Discussion on Rel-17 RAN4 UE feature list | Intel Corporation |

  - Discussion on 1st round

Issue 3-1: UE supported FR2-2 channel bandwidths

Option 1 (Apple)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X.Extending current NR operation to 71GHz | X-1 | UE support of max. CBW for supported SCS | Capability of supported max. CBW - 400MHz for 120kHz SCS - 1600MHz for 480kHz SCS - 2000MHz for 960kHz SCS Note: this capability may need to be split into three capabilities, i.e. one for each supported SCS |  | yes | no | UE cannot transmit or receive with the max. CBW | per Band | No | No |  |  | Optional with capability signalling |

Option 2 (Intel)

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | FR2-2 channel bandwidths for each SCS in each band for DL and UL for a single CC | Support of FR2-2 channel bandwidths 1) 120 kHz SCS: {100, 400} MHz CBW 2) 480 kHz SCS: {400, 800, 1600} MHz CBW 3) 960 kHz SCS: {400, 800, 1600, 2000} MHz CBW | FFS | Yes | No | UE cannot support some UE channel bandwidths | Per band | N/A | Applicable to FR2-2 only | N/A | UE indicating the support of specific SCS per band (RAN1 features X-Y) is required to support all CBWs corresponding to this SCS No additional capability signalling is needed. | NA |

Issue 3-2: UL support of 64QAM in the UL

Option 1 (Apple):

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | X-4 | UL support of 64QAM in the UL | Capability of support of 64QAM in the UL |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

Option 2 (Intel)

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-2 | 64QAM for PUSCH for FR2-2 | 1) Support of 64QAM modulation for FR2-2 PUSCH | FFS | Yes | No | UE cannot support PUSCH 64QAM transmission | Per band | N/A | Applicable to FR2-2 only | N/A |  | Optional with capability signalling |

Issue 3-3: Improved ON/ON transient period

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | [Improved ON/ON transient period] | 1) Support of improved ON/ON transient period of X < 5us (X is FFS) | FFS | Yes | No | UE does not support improved ON/ON transient period and support 5us transient period | Per UE | N/A | Applicable to FR2-2 only | N/A | Further RAN4 discussion is required on whether to support improved ON/ON transient period and X value | Optional with capability signalling |

    - Companies’ views collection for 1st round

Issue 3-1: UE supported FR2-2 channel bandwidths

It seems we need to introduce feature groups for UE supported FR2-2 channel bandwidth. However, whether the bandwidth should be maximum bandwidth or other bandwidths, and the exact bandwidth values to report for each SCS need further discussion.

Recommended WF: More discussion is needed.

| Company | Comments |
| --- | --- |
| ZTE | Option 2 is more preferred, however whether it’s mandatory to support the maximum channel bandwidth per SCS as indicated in both option 1 and option 2 should be further discussed and confirmed. In the previous meeting, this discussion is not confirmed yet. |
| Nokia (TL) | We support the proposal from Intel (option 2). As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required. In the competing proposal from Apple the support for bandwidth below the maximum channel bandwidth is also unclear. |
| HW | Option 2 is preferred. UE might not support all the channel bandwidths at the early deployment stage. It’s beneficial to allow UE to inform the network what specific BW is supported. Similar signaling already exists for FR1 and FR2-1. Need clarification on the note copied below. Is the RAN1 feature already specified? “UE indicating the support of specific SCS per band (RAN1 features X-Y) is required to support all CBWs corresponding to this SCS No additional capability signalling is needed.” |
| Ericsson | Need further discussion |
| Apple | Option 1 is preferred, and we are fine to continue the discussion in email thread 127 |
| Intel | We do not see the need to introduce extra capability for max CBW given that 480kHz and 960kHz SCS features are already optional. We are also ok to split this feature into three, i.e., one for each supported DL/UL SCS Question to Apple: Is the intention that only max CBW per SCS will be optional (e.g., 1600MHz for 480kHz SCS), while other CBWs will be supported (e.g. UE with 480kHz SCS support will always support 400, 800 MHz CBW)? To Huawei: In RAN1 UE feature list (R1-2112902), 480/960 kHz are already defined as optional UE features for FR2-2, so there is no benefit in defining support of maximum CBW as additional optional capabilities. |

Issue 3-2: UL support of 64QAM in the UL

Option 1 and option 2 are similar, while option 2 has more details.

Recommended WF: Agree to introduce “64QAM for PUSCH for FR2-2”, use option 2 as baseline.

Companies please provide comments based on option2.

| Company | Comments |
| --- | --- |
| ZTE | Fine with option 2 to set the optional capability for FR2-2 UL. |
| Nokia (TL) | We support the proposed WF. |
| HW | Option 2 is preferred. |
| Intel | Option 2. RF session agreement was already made. |

Issue 3-3: Improved ON/ON transient period

Recommended WF: More discussion is needed.

| Company | Comments |
| --- | --- |
| ZTE | It’s better to be discussed together with BS on/off transient period. If there is no consensus reached, this could be further discussed in Rel-18 RAN4 package. |
| Nokia (TL) | Further discussion in the corresponding WI is needed before finalizing the capability. It is still under investigation whether 1, 2 and/or 3 us transient period is specified. |
| HW | This is pending discussion in email thread #128. |
| Intel | We are ok to wait on the outcome of the ON/ON transient periods discussion in thread [128]. |

Issue 3-5: Other RF related UE features for NR operation to 71GHz

Note: RRM related UE features will be discussed in RRM session and corresponding email threads.

| Company | Comments |
| --- | --- |

    - Summary for 1st round

Issue 3-1: UE supported FR2-2 channel bandwidths

- Further discuss issue 3-1 in email thread [127].

Issue 3-2: UL support of 64QAM in the UL

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-2 | 64QAM for PUSCH for FR2-2 | 1) Support of 64QAM modulation for FR2-2 PUSCH | FFS | Yes | No | UE cannot support PUSCH 64QAM transmission | Per band | N/A | Applicable to FR2-2 only | N/A |  | Optional with capability signalling |

Agreement: agree on option 2.

Issue 3-3: Improved ON/ON transient period

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | [Improved ON/ON transient period] | 1) Support of improved ON/ON transient period of X < 5us (X is FFS) | FFS | Yes | No | UE does not support improved ON/ON transient period and support 5us transient period | Per UE | N/A | Applicable to FR2-2 only | N/A | Further RAN4 discussion is required on whether to support improved ON/ON transient period and X value | Optional with capability signalling |

- Discuss it in [128].

- UE RF requirements for Transparent Tx Diversity (TxD) for NR

  - .Companies’ contributions summary

| Tdocs | Title | Source |
| --- | --- | --- |
| R4-2200286 | Initial views on R17 feature list | Apple |
| R4-2201446 | Initial review of RAN4 UE feature in Rel-17 | ZTE Corporation |
| R4-2200611 | Discussion on capability signalling for HPUE NR DC with UE supporting TxD | MediaTek Inc. |
| R4-2200612 | Draft LS on signalling clarification of NR CA/DC power class in R17 | MediaTek Inc. |
| R4-2201308 | TxD signalling and inter-band carrier aggregation | Apple |

  - Discussion on 1st round

Issue 4-1: TxD support per band per band combination

Proposal in R4-2200286

- New signaling was agreed to be introduced in Rel-17, and RAN2 has introduced a per-band optional capability with a restriction to FR1 only

- In the case of a UE supporting TxD in a particular band within a band combination, we should consider the scenario of a UE falling back to single Tx operation (i.e. in a single carrier configuration the UE supports TxD, but when used with inter-band CA the UE falls back to single Tx); we do have this signaling for EN-DC, but not for inter-band CA; thus, we need an additional per band per band combination capability that indicates that the UE supports TxD in band X applicable to all inter-band UL CA combinations

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TxD | X-1 | TxD support per band per band combination | Support of transmit diversity per band per band combination | TxD support per band | Yes | No | UE uses a single Tx | Per FSPC (per CC per band per BC) | No | FR1 only | N/A |  | Optional with capability signalling |

Issue 4-2: New power class signalling per band per band combination

Proposal in R4-2200611 (MTK)

A new power class signalling per band per combination is needed for PC2 NR CA/DC starts from Rel-17.

Proposal in R4-2201308 (Apple)

Observation 2: In case of TxD and inter-band carrier aggregation the UE may not be able to achieve advertised power class for single bands and has currently no possibility to signal a different power class.

Proposal: Discuss the issue of TxD and inter-band carrier aggregation to find a potential way forward.

    - Companies’ views collection for 1st round

Issue 4-1: TxD support per band per band combination

| Company | Comments |
| --- | --- |
| Skyworks | For UE using TxD in one or both bands of an inter-band 2UL UL configuration, the current limitation to max 2 transmitter means that UE will have to fall back to 1Tx (and thus 3dB lower power) for these bands. This may be an assumption or a signaling capability, the later may be better if 3Tx is foreseen in R18 |
| MediaTek | We share the Skyworks view and we’ve seen there is issue on existing UE that supporting TxD when the UE also supports inter-band UL CA capability. |
| Nokia(HU) | We understand the point and we are open to discuss it’s necessity. Necessity, however, may depend on if we still keep specifying band combinations with asymmetric power classes per band within a band combination but the total power is capped by PC by the band combination like PC2(PC2, PC3). If so, that signaling may need but the signaling does not have to be TxD specific. If band combinations like PC2(PC2, PC3) will not be specified anymore and higher power which is on-going WI will be used, the signaling may not need since if PC2(23+23) per single band is used as 23 within PC band combination, the UE would not signal higher power capability anymore. |
| HW | This signaling is needed only in case of 3Tx or above. In Rel-17 only 2Tx is assumed for inter-band CA. It’s expected UE will not apply TxD on any band of inter-band CA combo. |
| OPPO | Specific power class was introduced for EN-DC due to TxD supported in one band of the band combination to make it clear which power class requirement this band will apply. Similar for inter-band UL CA, since the MPR is applied per band, therefore it needs to be clear which power class this band will support under a band combination. It can be indicated via TxD per band per band combination capability or other capabilities like new power class per band per band combination. |
| Ericsson | Agree with Apple (could also apply to TX switching cases with CA). Something similar to the EN-DC case could be considered |
| Qualcomm | This capability is non-backwards compatible in our understanding as UEs should not change their power class depending on configuration. We have some concerns on introducing this. Also, the case that UE power class stays the same should always be covered. |
| Apple | If we understood the GTW discussion correctly, then the alternative option of not introducing this proposed per band per BC signaling is motivated by the common understanding that Rel-17 assumes the 2 Tx architecture. This alternative would imply that a UE capable of PC2 via TxD in band X shall be assumed to support PC3 only in any inter-band UL CA configuration with band X. We are not certain whether this restricts implementations too much with, e.g. 23+26 dBm scenarios, where the UE can still support PC2 with a single PA for some bands and would need to fall back to PC3 in other bands. In our understanding, the signaling approach still seems to be the cleanest way forward. |
| Samsung | We are okay with introducing the signaling, but the expected UE behavior should be further clarified, especially because this per-CC-perBand-per-BC signaling is optional one: - If one band is indicated its support of TxD (by using the per-band sub feature which needs to be updated into the feature list table), and if this per-B-per-BC feature is not present, what is the UE behavior? - If the feature group is FSPC as proposed here, we expect the same TxD support among all CCs in the same band. Is that the correct understanding? Furthermore, do we really needs FSPC (per CC per Band per BC) for this feature, or per Band per Band Combo is enough, considering the general TxD feature is per Band. Especially considering the name in the table is “TxD support per band per BC”, obviously update is needed here (either on feature group name or Feature Type). |

Issue 4-2: New power class signalling per band per band combination

| Company | Comments |
| --- | --- |
| ZTE | For PC2 inter-band CA/DC, we think new power class signalling per band per combination is needed |
| MediaTek | As we proposed, a new power class signaling per band per combination can indicate different UE capability under non-CA operation (TxD) and under inter-band UL CA operation. |
| Nokia(HU) | At least if we introduce signaling like this, this does not have be specific to TxD. |
| HW | We have also identified the problem and discussed it in our paper R4-2201836 (#101bis-e) and R4-2118547 (#101-e).The ambiguity of per-band power class for PC2 CA/DC could cause a valid UE to fail the conformance tests of the configured transmitter power. We need a scalable solution that can support both 2Tx and 3Tx. A Lookup Table (LUT) based method is proposed in our paper R4-2201836, which can solve the power class ambiguity issue and increase the CA power limit when needed. This could be considered for inter-band CA and intra-band NC CA. |
| OPPO | Same comment as Issue 4-1. Either per band per BC TxD capability or power class capability can be used. |
| Ericsson | Same comment as for issue 4-1. |
| Qualcomm | Same comment as for issue 4-2 |
| Apple | There may also be one additional scenario for us to consider: a UE may only have 1 PC2 PA and 1 PC3 PA. For single band, PC2 could be shared by a few bands, such as n40 and n41. But when we have UL CA n40+n41, one band would need to fall back to PC3. This scenario does not necessarily involve TxD. |

Issue 4-3: Other UE features for TxD.

| Company | Comments |
| --- | --- |
| Skyworks | TxD for 1CC/2CC may need existing/additional signaling for different PA architectures. this need to be resolved in the associated WI. |
| Ericsson | TxD for 1CC/2CC may need existing/additional signaling for different PA architectures. this need to be resolved in the associated WI. |
| Qualcomm | We have to be careful in not introducing too many capabilities as this will just bring fragmentation. Not all corner cases have to be covered. The capability signaling is only useful if the network will actually use it. If there are too many variations for a feature, it is very unlikely that the network would optimize for each on of them but will rather take the least common denominator. |

    - Summary for 1st round

Issue 4-1: TxD support per band per band combination

Issue 4-2: New power class signalling per band per band combination

- Discuss issue 4-1, 4-2 in this email thread.

Issue 4-3: Others (TxD for different PA architectures)

- Discuss issue 4-3 in [118].

  - Discussion on 2nd round

Issue 4-1: TxD support per band per band combination

Issue 4-2: New power class signalling per band per band combination

Continue to discuss how to solve the ambiguity of per-band power class for PC2 CA/DC in 2nd round.

Option 1: indicate TxD support per band per band combination

Option 2: indicate new power class per band per band combination

Other options are not precluded.

| Company | Comments |
| --- | --- |
| Nokia(HU) | At least we don’t agree with the Option 1. We are open to discuss the Option 2. But we are not convinced if this is needed now or not. Regarding the option 1, TxD is transparent and currently per band capability where the Power Class as TxD needs to follow single Tx PC defined in 6.2. Though not sure what network can do even if a UE can use whether TxD capable or not during CA, but the network can know if PC per band combination is reported indirectly since if TxD is not available during CA, the reported PC within a CA is half of the PC reported as single band. |
| Apple | Option 1 is preferred. We are also open to discuss Option 2 further; perhaps it could be possible to carry both options forward to the next meeting for further analysis? |
| MediaTek | Option 2. The option has no dependency with option 1 thus it can be discussed separately. In response to NBC concern, PC2 NR CA/DC starts from Rel-17 and same is specification suffix G (TXD). The new signaling is needed to solve the power class ambiguity with UE supporting TXD thus we don’t think this is NBC in 38.101-1. In response to UE behavior clarification, if one band is indicated its support of TxD but per-B-per-BC feature is not present, there may be different PHR comparing to single band operation that may cause network uplink power control issue. And the feature type FSPC may not be proper here. There seems no need to signal per CC per Band per BC. Per band per BC is enough. Further, this solution can also be applied for UE supporting more than 2TX, this can be further discussed in later release since 2TX is current RAN4 assumption. At least in current release we need a clear indication. |
| HW | Option 1 and 2 could be discussed separately. Option 2. Reporting new power class is necessary in some scenarios. Example: UE with 23dBm+26dBm implementation. As per current RAN4 agreement it will not report TxD capability. It might report PC2 in all the bands as the 26dBm PA could be used when it works in single band. When it works in CA mode, one of the band can only transmit 23dBm, and this information can’t be communicated to network. By introducing a new power class, network could update its expectation. Option 1. The UE’s TxD capability per band per band combination is not necessary as it could be derived based on the existing per band TxD capability and the new power class signaling as per Option 2. |
| Ericsson | Issue 4-1: reading the proposal again, we propose not to introduce any further TxD indication, the power capability for the BC and its consistent bands should follow from the BC and per-band power classes (see next issue). Issue 4-2: We support Option 2. A new per-band power class could be introduced in the featureSetCombination per band and would apply to intra-band contiguous CCs in the band. If absent the NR power class applies. |

  - Summary for 2nd round

Issue 4-1: TxD support per band per band combination

Issue 4-2: New power class signalling per band per band combination

Continue to discuss how to solve the ambiguity of per-band power class for PC2 CA/DC in 2nd round.

Option 1(Apple): indicate TxD support per band per band combination

Option 2 (MediaTek, Huawei, Ericsson): indicate new power class per band per band combination

Other options are not precluded.

Recommended WF:

More companies prefer to consider option 2 to solve the issue. And some companies believe these two issues can be discussed separately.

Continue to discuss in next meeting.

- NR coverage enhancements

  - .Companies’ contributions summary

| Tdocs | Title | Source |
| --- | --- | --- |
| R4-2200286 | Initial views on R17 feature list | Apple |

  - Discussion on 1st round

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coverage enhancement | X-1 | Maximum duration of UE transmission for joint channel estimation | UE supports the indicated maximum duration of transmission for joint channel estimation: [{2, 4, 8, 16, 32} slots] | N/A | Yes | No | UE does not support transmission for joint channel estimation | Per Band | No | No | N/A |  | Optional with capability signalling |

    - Companies’ views collection for 1st round

Issue 5-1: Maximum duration of UE transmission for joint channel estimation

| Company | Comments |
| --- | --- |
| China Telecom | RAN1 has agreed the following UE feature, as seen in RAN1 LS. So, we propose RAN4 to send the related agreements to RAN1, and RAN1 can capture the RAN4 agreements in RAN1 feature list. 30. NR_cov_enh; 30-4; [The maximum duration for DM-RS bundling]; The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions. |
| ZTE | Based on the discussion in last RAN4 meeting and LS out to RAN1 R4-2120002, maximum duration for joint channel estimation is still under the discussion, it’s per FR or per band is also FFS. |
| HW | There is relevant discussion in email thread #130. |
| Ericsson | We do not support this feature since in our view the requirements on the maximum duration of UE transmission for joint channel estimation should be based on one value in terms of number of slots. In any case the feature’s components/Per band/TDD-FDD differentiation/FR1 FR2 differentiation are currently being discussed in email thread # [130]. Feature discussion on these fields should refer to/depend on email thread # [130] discussion. |
| Intel | We also support to check with RAN1 whether this proposal has overlap with RAN1 features. |

    - Summary for 1st round

Issue 5-1: Maximum duration of UE transmission for joint channel estimation

Agreement: no need to capture the feature in RAN4. Provide the feedback to RAN1 to capture the values.

- Others

  - .Companies’ contributions summary

| Tdocs | Title | Source |
| --- | --- | --- |
| R4-2200286 | Initial views on R17 feature list | Apple |
| R4-2201446 | Initial review of RAN4 UE feature in Rel-17 | ZTE Corporation |
| R4-2201803 | A new Rel-17 per-FR MG capability based on Per BC | Qualcomm Incorporated |

  - Discussion on 1st round

Issue 6-1: Support ot Hybrid duplex operation for HPUE FDD (Apple)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HPUE FDD | X-1 | Hybrid duplex operation | Support of hybrid duplex operation | N/A | Yes | No | UE does not support hybrid duplex operation | Per Band | FDD only | FR1 only | N/A |  | Optional with capability signalling |

Proposal in R4-2200286:

We propose to introduce a new UE capability of supporting hybrid duplex operation for PC2 FDD bands.  It can indicate to the need to configure UE to half-duplex mode or back to full-duplex mode as needed. The mode configuration can potentially be done semi-statically through RRC reconfiguration or dynamically through MAC-CE process.

Issue 6-2: Power high limit for inter-band CA and DC (ZTE)

| UE feature | Description |
| --- | --- |
| [Power high limit for inter-band CA and DC] | [Indicate whether UE supports high power limits for inter-band CA and DC] |

Issue 6-3: maxUplinkDutyCycle-interBandCA-PC2 (ZTE)

| UE feature | Description |
| --- | --- |
| maxUplinkDutyCycle-interBandCA-PC2 | Maximum uplink duty cycle for NR inter-band CA power class 2 |

Issue 6-4: DC location for intra-band CA [more than 2CC]

| UE feature | Description |
| --- | --- |
| DC location for intra-band CA [more than 2CC] | Indicate whether UE support Additional DC location reporting for intra-band UL CA |

Issue 6-5: Per BC indication for the per-FR gap capability (Qualcomm)

Observation 1:  Per-FR gap capability for a UE is not purely depending on RF architecture but also baseband design.

Observation 2:  There are thousands of FR1+FR2 band combinations specified in 3GPP so far and they can be of up to 5 bands of either FDD or TDD in both FR1 and FR2.

Observation 3: The constraints of the per UE indication of per-FR gap come from that UE may not support per-FR gap for certain high order CA combination.

Observation 4: For overloading issues, reverting the assumptions for all related requirements in a case by case manner is not feasible as there will be severe compatibility issues even by introducing new dedicated signaling for each requirements.

Observation 5: The advantages of introducing the new capability are very clear (e.g. enables more UEs to support per-FR gap feature) while there is no foreseen disadvantage. In the worst case scenario, the new capability is simply ignored.

Proposal 1: Keep the original per UE per-FR gap indication and add new Per BC indication for the per-FR gap capacity to Rel-17 UE feature list.

Issue 6-6: Support of MSD reduction for HPUE FDD (China Unicom)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HPUE FDD |  | MSD reduction | Support of reducing UE Tx power for certain bandwidth in specific bands, where the MSD is larger than or equal to [FFS]dB under power class 2 operation. | N/A | Yes | No | UE does not support lowering the MSD by reducing UE Tx power | Per Band | FDD only | FR1 only | N/A | Network can configure whether to enable the UE capability | Optional with capability signalling |

    - Companies’ views collection for 1st round

Issue 6-1: Support ot Hybrid duplex operation for HPUE FDD

| Company | Comments |
| --- | --- |
| ZTE | No agreements so far. In terms of the previous discussion, some companies agree to introduce Hybrid duplex operation for HPUE FDD but still there are several companies agree not to support. |
| HW | This signaling is not necessary. The said Hybrid duplex method adds implementation complexity of network scheduling, and would have unexpected RAN1 impact, therefore should not be introduced. In addition if the signaling as in Issue 6-6 is agreed, the MSD impact could be reduced. |
| OPPO | Need to wait for the outcome of discussion whether the hybrid FDD is needed or not. |
| Ericsson | Not preferred. We do not support introducing Hybrid duplex operation for HPUE FDD. |
| CHTTL | Share the same view as HW and Ericsson. |
| Qualcomm | This first has to be discussed in the work item. The feature is not yet agreed. |
| Apple | We view this proposed capability as the most efficient method of enabling high power UE operation in FDD bands, and we are fine to continue the discussion in the related email thread. |
| Intel | Postpone the discussion till the agreements on Hybrid duplex introduction are made. |

Issue 6-2: Power high limit for inter-band CA and DC

Recommended WF: Companies are welcome to comment whether this feature is needed or not. If needed more details on the feature group are welcome to be provided in order to complete the feature group.

| Company | Comments |
| --- | --- |
| ZTE | In terms of the agreement, the signalling for power high limits for inter-band CA/DC is needed, however, the signalling details are still underway, maybe we postpone to discuss. |
| Nokia(HU) | We agree with ZTE that we anyway need some more discussion. From our perspective, at least signalling per band combination to indicate power high limit support is necessary. At least in our understanding, this capability is power aggregation agnostic for instance, PC2+PC3 or PC5+PC3 can use the same capability since this capability indication is just to lift the restriction and the expected total power can be calculated by power class per band. Also, additional maxUPlinkDutyCycle for higher power may be needed but it depends on the discussion outcome. |
| HW | There is ongoing discussion in email thread #114 under dedicated WI NR_Power limit_CA_DC. |
| OPPO | Generally ok to introduce a UE capability for this power high limit. But more details are needed, and the discussion of handling TxD exception case is still ongoing, like whether it is allowed to indicate this capability if one band in a band combination support TxD, if it is then maybe per band per band combination capability is needed. |
| Ericsson | Not needed. The existing power class per band combination can be extended. |
| Intel | Postpone the discussion till the design becomes clearer. |

Issue 6-3: maxUplinkDutyCycle-interBandCA-PC2

Recommended WF: Companies are welcome to comment whether this feature is needed or not. If needed more details on the feature group are welcome to be provided in order to complete the feature group.

| Company | Comments |
| --- | --- |
| China Telecom | The maxUplinkDutyCycle-interBandCA-PC2 and maxUplinkDutyCycle-SULcombination-PC2 capabilities have been agreed in RAN4 #100e, and LS has been sent to RAN2 in R4-2114933. So the corresponding UE features are needed (but not urgent), and the details on the feature can be finished in the next meeting. |
| ZTE | It was already introduced in TS38.101-1 spec for PC2 inter-band CA duty cycle. However, no corresponding capability in TS38.306 was introduced. if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or.... |

Issue 6-4: DC location for intra-band CA [more than 2CC]

Recommended WF: Companies are welcome to comment whether this feature is needed or not. If needed more details on the feature group are welcome to be provided in order to complete the feature group.

| Company | Comments |
| --- | --- |
| ZTE | Current RAN2 signalling on Tx DC only support two uplink carriers |
| Nokia | A new capability is needed. At least the capability needs to also indicate which “frequency component” is supported to calculate default DC location(s). Moreover, some complementary information is required to address the case where a UE has two DC locations. In any case, it needs more discussion. |
| HW | Support adding this signaling. Still some open issues are under discussion in email thread #121. |
| OPPO | Agree with Nokia, more capability signalling is needed, and can refer to LS R4-2119965, like UE supported default DC location, DC numbers. |
| Qualcomm | A new capability is most likely needed as there will be new signaling for this case. |

Issue 6-5: Per BC indication for the per-FR gap capability

Recommended WF: This issue had been discussed since Rel-16 feature list. Companies please provide technical comments on whether this feature should be introduced in Rel-17..

| Company | Comments |
| --- | --- |
| HW | Support adding this signaling. By introducing the per-BC indication, UE can utilize the per-FR gap if this feature can not be supported only in some extreme BC or when it is overloaded by other RRM requirements. Based on the discussion in previous meetings, no compatibility issues is foreseen. |
| Qualcomm | We support this feature. The proposal was discussed before, is fully backwards compatible and does not require and requirement change in RAN4 |
| Intel | We support the contents of this proposal. But we had RANP agreement that we do not continue discuss this issue since there is no placeholder for this discussion. If the group consensus is that we can use feature list as the place holder we would like to support this proposal. |

Issue 6-6: Per band indication for reducing MSD under NR FDD PC2

| Company | Comments |
| --- | --- |
| HW | Support introducing such a signaling. |
| Qualcomm | We should come back to this discussion after we understand what MSD we are talking about. It is unlikely that the MSD would be so high that a reduced MSD capability would be needed/feasible. |
| Apple | This proposal seems to contradict the work item objective, and we don't think this is the right way to resolve the related MSD issues. In our system level simulation study, which we submitted to the FDD HPUE agenda, we actually made a related assumption by modeling MSD impact as a function of Tx power. In our understanding, this proposed capability would simply shrinks the cell size right back to PC3 cell size, resulting in no overall gain for the network. |

Issue 6-7: Other features for spectrum related WIs or features not covered by Rel-17 WIs

| Company | Comments |
| --- | --- |

    - Summary for 1st round

Issue 6-1: Support ot Hybrid duplex operation for HPUE FDD (Apple)

- Discuss it in [115].

Issue 6-2: Power high limit for inter-band CA and DC (ZTE)

- Discuss it in [114].

Issue 6-3: maxUplinkDutyCycle-interBandCA-PC2 (ZTE)

Agreement: agree to capture feature of maxUplinkDutyCycle-interBandCA-PC2 and maxUplinkDutyCycle-interBandCA-PC2 and maxUplinkDutyCycle-SULcombination-PC2 in RAN4 feature list.

Issue 6-4: DC location for intra-band CA [more than 2CC]

Agreement: Introduce the new capability signalling for DC location for intra-band CA more than 2CC to indicate whether UE support additional DC location reporting for intra-band UL CA.

- Further discuss the details in email thread [121] for both FR1 and FR2.

Issue 6-5: Per BC indication for the per-FR gap capability (Qualcomm)

- Further discuss it in this email thread.

Issue 6-6: Support of MSD reduction for HPUE FDD (China Unicom)

- Discuss it in [115] email thread for FDD HPUE.

  - Discussion on 2nd round

Issue 6-5: Per BC indication for the per-FR gap capability (Qualcomm)

Observation 1:  Per-FR gap capability for a UE is not purely depending on RF architecture but also baseband design.

Observation 2:  There are thousands of FR1+FR2 band combinations specified in 3GPP so far and they can be of up to 5 bands of either FDD or TDD in both FR1 and FR2.

Observation 3: The constraints of the per UE indication of per-FR gap come from that UE may not support per-FR gap for certain high order CA combination.

Observation 4: For overloading issues, reverting the assumptions for all related requirements in a case by case manner is not feasible as there will be severe compatibility issues even by introducing new dedicated signaling for each requirements.

Observation 5: The advantages of introducing the new capability are very clear (e.g. enables more UEs to support per-FR gap feature) while there is no foreseen disadvantage. In the worst case scenario, the new capability is simply ignored.

Proposal 1: Keep the original per UE per-FR gap indication and add new Per BC indication for the per-FR gap capacity to Rel-17 UE feature list.

| Company | Comments |
| --- | --- |
| Apple | Procedure-wise, we share the comment from Intel that if R17 feature list discussion should include this proposal, as it doesn’t belong to any of the R17 WIs. Technically, even though this proposal was discussed during R16 at multiple meetings, what remains to be seen is what constraint a UE might face in terms of baseband processing capability, even though the general understanding is this constraint comes from the baseband. Further clarifications from proponents are appreciated. |
| Huawei | We support the moderator’s proposal. It makes the per-FR gap more flexible to be supported, and it is backwards compatible according the discussion in Rel-16. |
| Ericsson | Our concern is that the Per BC indication for the per-FR gap capability puts more constrain on the network scheduling and measurement configuration. If the BC is reconfigured or changes then depending on the UE per BC capability the UE may or may not support the per FR for the new BC. In this case the network has to change the scheduling method, reconfigure gaps and possibly modify measurement configuration. |

  - Summary for 2nd round

Issue 6-5: Per BC indication for the per-FR gap capability (Qualcomm)

Proposal: Keep the original per UE per-FR gap indication and add new Per BC indication for the per-FR gap capacity to Rel-17 UE feature list.

Recommended WF:

Continue to discuss in next meeting.

- Enhancement for NR high speed train scenario in FR1

Moderator: will be discussed in email thread [203] FR1 HST. The conclusion in [203] FR1 HST will be captured in the feature list document.

- NR support for high speed train scenario in FR2

Moderator: will be discussed in email thread [204] FR2 HST. The conclusion in [204] FR2 HST will be captured in the feature list document.

- Further RRM enhancement for NR and MR-DC

Moderator: will be discussed in email thread [206/207/208] NR feRRM. The conclusion in [206/207/208] NR feRRM will be captured in the feature list document.

- NR and MR-DC measurement gap enhancements

Moderator: will be discussed in email thread [209/210/211] NR MG Enh. The conclusion in [209/210/211] NR MG Enh will be captured in the feature list document.

- UE Power Saving Enhancements for NR

Moderator: will be discussed in email thread [214] NR UE PWS. The conclusion in [214] NR UE PWS will be captured in the feature list document.

- Positioning enhancements for NR

Moderator: will be discussed in email thread [222] NR ePos. The conclusion in [222] NR ePos will be captured in the feature list document.

- Further enhancement on NR demodulation performance

Moderator: will be discussed in demodulation session. The conclusion will be captured in the feature list document.

- Introduction of DL 1024QAM for NR FR1

Recommended WF: At least UE feature of supporting DL 1024 QAM is needed?

| Company | Comments |
| --- | --- |
| ZTE | In general, we are fine to introduce the UE capability similar as LTE. In addition, the UE category or UE types should be clarified further in the corresponding agenda. The original motivation for the support of 1024QAM in DL is for stationary wireless scenarios (e.g. FWA) |
| Ericsson | RAN1 has agreed to introduce the optional per-band UE capability for DL1024 QAM (NR_DL1024QAM_FR1). RAN4 does not need to add UE feature on DL1024QAM for FR1. |
| Intel | RAN1 feature list already includes DL 1024QAM for FR1 and no further RAN4 discussion is needed (see R1-2112902 feature 36-1). |

GTW Tentative agreement: No need to capture the DL 1024QAM feature in RAN4.

After further check, FG36-1 for DL 1024QAM is already captured in RAN1 feature list. The above tentative agreement can be confirmed.

- Solutions for NR to support non-terrestrial networks (NTN)

Any UE features for NTN？

| Company | Comments |
| --- | --- |
| Intel | FFS. Propose to discuss once additional RAN4 progress is made. There are planned features to be added which are under discussion. |

- NR Sidelink enhancement

Any UE features for Sidelink enhancement?

| Company | Comments |
| --- | --- |
| LGE | Provide the expected SL UE features based on the agreements at last RAN4 meeting. Agreement 1: For sidelink, define specific NR V2X power class capability, e.g., PC2 and PC3 per band. Agreement 2: Introduce NR V2X BC power class for intra-band con-current V2X operation, e.g., PC2 and PC3 per band combinations. Agreement 3: Introduce NR V2X BC power class for inter-band con-current V2X operation, e.g., PC2 and PC3 per band combinations. |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL enhancement | 16-1 | NR SL UE power class | 1) Support of NR V2X UE power class for single carrier 2) Support of NR V2X UE power class for intra-band con-current band combination 3) Support of NR V2X UE power class for inter-band con-current band combination | 5G V2X_NRSL | Yes (in-coverage) No (pre-configured parameters) | Yes | PC2 V2X UE or con-current V2X UE do not operated with max. output power. | Per Band or Per Band combination |

- Further enhancements on MIMO for NR

Any UE features for eMIMO?

| Company | Comments |
| --- | --- |
| Intel | FFS. Propose to discuss once additional RAN4 progress is made. |

- Support of reduced capability NR devices

Any UE features for RedCap?

| Company | Comments |
| --- | --- |

- Multi-Radio Dual-Connectivity enhancements

Any UE features for MRDC enhancement?

| Company | Comments |
| --- | --- |

- Enhanced IIoT and URLLC support

Any UE features for IIoT and URLLC?

| Company | Comments |
| --- | --- |

- NR Sidelink Relay

Recommended WF: No UE features for sidelink Relay.

| Company | Comments |
| --- | --- |
| OPPO | Agree with the Recommended WF for ‘sidelink relay RRM requirements. As for LTE SL relay, ‘Sidelink Relay UE operation’ and ‘Sidelink Remote UE operation’ were introduced in TS 36.331. The similar discussion is under discussion in RAN2. From RAN4 perspective, no new UE feature is needed for R17 SL relay. |

Tentative agreement: No UE features for sidelink Relay.

- NR small data transmissions in INACTIVE state

Recommended WF: No UE features for SDT.

| Company | Comments |
| --- | --- |

- NR repeater

Recommended WF: No UE features for NR repeater.

| Company | Comments |
| --- | --- |
| Nokia (TL) | We support the proposed WF. |

Agreement: no UE features for NR repeaters in RAN4.

- Enhancements to Integrated Access and Backhaul (IAB) for NR: Qualcomm

Recommended WF: No UE features for IAB.

| Company | Comments |
| --- | --- |
| Nokia (TL) | We support the proposed WF. |
| Samsung | We agree with the recommended WF. From RAN4 perspective No new UE feature is needed for R17 eIAB according to our observation. |

Tentative agreement: No UE features for IAB.

- Additional enhancements for NB-IoT and LTE-MTC

Any UE features for NB-IoT and LTE-MTC?

| Company | Comments |
| --- | --- |

Recommendations for Tdocs

  - 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| Rel-17 UE feature list | CMCC |  |
| LS on Rel-17 RAN4 UE features list for NR | CMCC |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

  - 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2202400 | Rel-17 UE feature list | CMCC | Return to |  |
| R4-2202401 | LS on Rel-17 RAN4 UE features list for NR | CMCC | Return to |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Hiromasa Umeda | hiromasa.umeda@nokia.com |
| Nokia (TL) | Toni Lähteensuo | Toni.h.lahteensuo (at) nokia.com |
| E/// | Muhammad Kazmi | Muhammad.kazmi@ericsson |
| Intel | Meng Zhang | Meng.zhang@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)