3GPP TSG-RAN WG4 Meeting #101-Bis-e  						        				R4-2202354

Electronic Meeting, Jan. 17-25, 2022

Agenda item:			6.7.3

Source:	Samsung

Title:	WF on ULFPTx requirements for fallback and SRS antenna switching

Document for:	Approval

# 1 Background

The WF is focused on the following remaining issues from the email discussion summary on UE RF requirements for Transparent Tx Diversity (TxD) for NR in RAN4 #101-bis-e, with the email thread identifier “[101-bis-e][122] NR_TxD”:

- Topic #3 (SRS Antenna Switching): Issue 3-1-2

- Topic #4 (ULFPTx and TxD): All issues

# 2 Way Forward

### Topic #3: SRS Antenna Switching

Issue 3-1-2: Is SRS power difference for antenna switching dependent on other features than TxD

- Way forward:

  - Is SRS power difference for antenna switching dependent on other features than TxD:

    - Option 1: Yes

    - Option 2: No

### Topic #4: ULFPTx and TxD

- Way forward:

  - How ULFPTx capable UE’s fallback DCI requirement should be defined?

    - Alt. 1:

      - Default fallback DCI requirement is directed to 6.2, except:

        - Only give exception to ULFPTx Mode 1 UE to 6.2G

      - Independent on UE indicating its support of TxD:

        - Wording of “If UE not indicating Tx Diversity” (already implemented in 38.101-1 v17.4.0) is not needed

      - Typical Text Proposal for Alt. 1:

<from Nokia R4-2200483> If UE not indicating Tx diversity [15, TS 38.306] is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.1 apply for the power class as indicated by the ue-PowerClass field in capability signalling with the following exceptions: for UEs being configured with ul-FullPwrMode1-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass apply.

      - The below table reflect the implication for Alt. 1:

| Single antenna-port Requirements applicability | ul-FullPwrMode1-r16 (Mode-1) | ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 (Mode-2 Mechanism 1) | ul-FullPwrMode2-TPMIGroup-r16 (Mode-2 Mechanism 2) | ul-FullPwrMode-r16 (Mode 0) | No ULFPTx |
| --- | --- | --- | --- | --- | --- |
| txDiversity-r16 | Dual Tx (Exception in Alt.1) | N/A if single antenna-port PUSCH etc.. is considered as exception of requirements in Table 6.2D.1-3 Dual Tx (Exception is allowed) if single antenna-port PUSCH etc.. is considered as exception of all the requirements for Rel-16 UL MIMO | N/A (Given that TxD is half rated declared PC PA x 2, this will in principle not happen) | N/A (Given that TxD is half rated declared PC PA x 2, this will in principle not happen) | Dual Tx |
| No TxD indication | Dual Tx (Exception in Alt.1) | Single Tx | Single Tx | Single Tx | Single Tx |

    - Alt. 2:

      - Default fallback DCI requirement is directed to 6.2, except:

        - Give exception to ULFPTx Mode 1 UE to 6.2G

        - Give exception to UE indicating [txDiversity-416] to 6.2G

      - Typical Text Proposal for Alt. 2:

<from Ericsson R4-2200862> If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2 apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling with the following exceptions: for UEs indicating [txDiversity-r16] or ul-FullPwrMode1-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass.

      - The below table reflect the implication for Alt. 2:

| Single antenna-port Requirements applicability | ul-FullPwrMode1-r16 (Mode-1) | ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 (Mode-2 Mechanism 1) | ul-FullPwrMode2-TPMIGroup-r16 (Mode-2 Mechanism 2) | ul-FullPwrMode-r16 (Mode 0) | No ULFPTx |
| --- | --- | --- | --- | --- | --- |
| txDiversity-r16 | Dual Tx (Exception in Alt.2) | Dual Tx (Exception in Alt.2) | Dual Tx (Exception in Alt.2) | Dual Tx (Exception in Alt.2) | Dual Tx |
| No TxD indication | Dual Tx (Exception in Alt.2) | Single Tx | Single Tx | Single Tx | Single Tx |

    - Alt. 3:

      - For all ULFPTx modes UE, based on [txDiversity-r16] to determine whether 6.2 or 6.2G shall be applied for fallback DCI:

      - Typical Text Proposal for Alt. 3:

<from Samsung R4-2201762> If UE not indicating Tx diversity [xx15, TS 38.306] is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.1 apply for the power class as indicated by the ue-PowerClass field in capability signalling. If UE indicating Tx diversity [15, TS 38.306] is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2G.1 apply for the power class as indicated by the ue-PowerClass field in capability signalling.

      - The below table reflect the implication for Alt. 2:

| Single antenna-port Requirements applicability | ul-FullPwrMode1-r16 (Mode-1) | ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 (Mode-2 Mechanism 1) | ul-FullPwrMode2-TPMIGroup-r16 (Mode-2 Mechanism 2) | ul-FullPwrMode-r16 (Mode 0) | No ULFPTx |
| --- | --- | --- | --- | --- | --- |
| txDiversity-r16 | Dual Tx | Dual Tx | Dual Tx | Dual Tx | Dual Tx |
| No TxD indication | Single Tx | Single Tx | Single Tx | Single Tx | Single Tx |

    - Other alternatives, or combined one if identified.

# References

[1] R4-2202222, “Email discussion summary for [101-bis-e][122] NR_TxD”, Qualcomm