3GPP TSG-RAN WG4 Meeting # 101-BIS-e	      	              							R4-2202361

Electronic Meeting, 17th -25th January, 2022

Title: 	WF on Configured Tx power for intra-band V2X con-current operation UE

Source: 	LG Electronics

Agenda item:	6.15.3.1

Document for:	Approval

Background

- During RAN4#101-e & RAN4 #101-BIS-e meeting, RAN4 discussed on the configured Tx power for intra-band V2X con-current operation UE in licensed band.

- In 1st round discussion, RAN4 discussed how to specify the configured Tx power as follow.

Issue 1-2-1: Configured transmitted power for intra-band V2X con-current operation

  - Proposals

    - Option 1: Reuse the principle of NR inter-band CA.

    - Option 2: Reuse the principle of NR intra-band CA.

- In the 1st round, there was no majority view for the configured Tx power for intra-band V2X con-current operation UE.

  - CATT,Xiaomi, LGE prefer to follow the principle of NR inter-band CA

  - Huawei, QC, vivo prefer to follow the principle of NR intra-band CA

- In WF (R4-2119989) on MPR simulation assumptions, RAN4 agreed the different modulation order and different RB size between NR Uu operation and NR SL operation in a licensed band.

- RAN4 assumed the baseline RF architectures as follow

  - In FDM operation: Separate RF architectures for intra-band con-current V2X UE.

  - In TDM operation, Shared RF architecture for intra-band con-current V2X UE.

- Above RAN4 agreements

WF: Configured Tx power for intra-band V2X con-current UE

- Issue 1-2-1: Configured transmitted power for intra-band V2X con-current operation

  - Observation

    - For TDM operation, the each RAT will satisfy the individual configured Tx power per carrier.

    - For the FDM operation, the configured Tx power of the intra-band V2X con-current UE, the MPR values are applied for both two carriers as same as intra-band CA UE.

    - It is easy way to apply the configured Tx power concept for TDM operation and FDM operation for intra-band V2X con-current operation in a licensed band.

  - WF

    - For the TDM operation intra-band V2X con-current UE, the configured Tx power for each RAT applies per carrier at a given time.

    - For the FDM operation intra-band V2X con-current UE, RAN4 can applied the principle of of NR intra-band CA for the configured Tx power.

    - FDM or TDM operation is determined by whether simultaneous transmission of Uu and SL overlap in time or not.

    - TP and Draft CR will be updated based on the WF.

- Reference

- R4-2202225, “Email discussion summary for [101-BIS-e][125] NRSL_enh_Part1,” Moderator (CATT)

- R4-2200841, “Draft CR on RF requirements for intra-band con-current V2X operation in Rel-17,” LGE

- R4-2200143, “TP on configured transmitted power for intra-band V2X con-current operation,” CATT

- R4-2201500, “Further discussion on configured power for intra-band concurrent operation,” Xiaomi

- R4-2201949, “Draft CR for TS 38.101-1: configured transmitted power for intra-band con-current operation,” Huawei