3GPP TSG-RAN WG4 Meeting # 101-bis-e                                                      R4-2202367

Electronic Meeting, January 17-25, 2022

Agenda item:			6.16.5

Source:	CATT, Qualcomm, Nokia, ZTE, Ericsson, vivo, Korea Testing Laboratory, Huawei

Title:	WF on the decision of DL/UL ACIR and BS/UE ACLR/ACS for FR2-2

Document for:	Approval

- Background

In this RAN4 meeting, several companies provided updated simulation results. Some observations are provided to try to find the reasons why the co-existence simulation results in the 71GHz WI shows more stringent requirement than that in TR 38.803. The following observations are provided in the contributions,

  - Observation 1: The increased number of columns of UE antenna elements significantly increases the 5%-tile throughput of the victim UE, such that the impact of the adjacent channel interference from the interfering system becomes more significant compared to the impact of the co-channel interference from the own system for the cell-edge UE.

  - Observation 2: Antenna elements radiation pattern and transmission power have less influence than the number of transmitting antennas in determining ACIR requirement.

  - Observation 3: ACIR requirements become stringent as the number of transmitting antennas decreases.

  - Observation 4: The impact due to array element parameter selection does not significantly affect the end results.

Companies also provided the ACIR proposals based on the simulation results and also the technology trends. The detail proposal and discussion in the 1st round discussion can be found in [1].

- Way Forward

The ACIR requirements for FR2-2 are agreed to reuse TR 38.803 70 GHz ACIR requirements and round up ACLR and ACS values to the nearest integers.

DL ACIR requirement for 57-71GHz

| DL ACIR | BS ACLR | UE ACS |
| --- | --- | --- |
| ~18.7 dB | 24 dB | 21 dB |

UL ACIR requirement for 57-71GHz

| UL ACIR | BS ACS | UE ACLR |
| --- | --- | --- |
| 13.8 dB | 22 dB | 15 dB |

Reference

[1] R4-2202229, Email discussion summary for [101-bis-e][129] NR_ext_to_71GHz_Part_3