3GPP TSG-RAN WG4 Meeting #101-bis-e                                                    R4-2202385

# Electronic Meeting, January 17-25, 2022

Source:	ZTE Corporation, Xiaomi

Title:	TP to TR 38.862 on introduction to BCS4 and BCS5

Agenda Item:	7.3.2

Document for:	Approval

1	Introduction

In RAN#90-e meeting, a new WID for introduction of bandwidth combination set 4 (BCS4) for NR was agreed in [1]. The main purpose of the WID is to solve the time-consuming new request from operators when creating new NR-CA and SUL band combinations, especially for the cases when new channel bandwidths have been added to existing bands in the band combinations. Since the establishment of the WID, RAN4 has spent a lot of time discussing on this topic and some agreements have been achieved. In this proposal, a TP to capture the related agreements and some guidelines for BCS4/5 definition is proposed.

Proposal 1:	 It is suggested to adopt the following TP related to BCS4/5.

2	References

[1] RP-202832, New WID: Introduction of bandwidth combination set 4 (BCS4)

Text Proposal

<< begin text proposal >>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		RP-210773, “New SID: Study on band combination handling in RAN4”, RAN#91-e.

[3] 		3GPP TR 38.817-01: “General aspects for User Equipment (UE) Radio Frequency (RF) for NR”.

[4] 		3GPP TR 38.101-01: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[5] 		3GPP TR 38.101-02: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[6] 		3GPP TR 38.101-03: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[xx] 		RP-202832, “New WID: Introduction of bandwidth combination set 4 (BCS4) for NR”, RAN#90-e.

<< Unchanged sections omitted >>

5.x	Bandwidth combination set 4 and 5

5.x.1	Introduction of BCS4/BCS5

In NR band combination, it is a challenge for operators to request new NR CA and SUL band combinations when new channel bandwidths are added to existing bands in the band combinations. Operators have to go back and create new Bandwidth Combinations Sets for existing band combinations in order to be able to use those new channel bandwidths in NR CA and SUL. To reduce the workload in RAN4, a new WID for creating a new type of BCS was approved in [xx].

BCS4/BCS5 are allowed to be applied to new band combinations in RAN4 which indicate UE to support up to all of the channel bandwidths for the band in the band combination. BCS4 is release independent to Rel-15 with no new signalling while BCS5 is functionally equivalent to BCS4 except that the new signalling [supportedMinBandwidthDL/supportedMinBandwidthUL] introduced in Rel-17 such as the limitation to the supporting channel bandwidth in each band within the band combination would apply, and BCS5 with the signalling [supportedMinBandwidthDL/supportedMinBandwidthUL] is allowed for early implementation from Rel-15. For a legacy gNB that was not upgraded to understand BCS4 or BCS5 with the new signalling, it would enable to ignore BCS4 or BCS5 with the new signalling.

Considering that BCSs are not defined or reported separately for UL and DL for traditional BCSs, there is also no need to differentiate BCS4/BCS5 for UL and DL. BCS4/BCS5 can be used for FR1 intra-band UL CA. Table 5.x.1-1 shows the template for NR CA configurations for intra-band contiguous CA with BCS4/BCS5. Table 5.x.1-2 shows the template for NR CA configurations for intra-band non-contiguous CA with BCS4/BCS5. For inter-band CA combinations including FR1 intra-band CA and with BCS4/BCS5, the bandwidth combination sets for the FR1 intra-band CA are BCS4/BCS5. The BCS4/BCS5 are represented in the inter-band CA configuration table by using the option which covers inter-band and intra-band as shown in Table 5.x.1-3. For SUL band combinations, BCS4/BCS5 reuse the same template with inter-band CA as in Table 5.x.1-3.

Table 5.x.1-1: Template for NR intra-band contiguous CA configurations with BCS4/BCS5

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_nXC | CA_nXC | 40 | 80, 100 |  |  |  | 180 | 0 |
|  |  |  |  | 50, 60, 80 |  | 60, 80, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | … |  | … |  |  |  |  |  |  |  | … |  | … |  |
|  |  |  |  | See nX channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  |  |  |  |  |  | TBD |  | 4 and 5 |  |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: The aggregated bandwidth must be greater than or equal to the minimum for the bandwidth class defined in Table 5.3A.5-1, and smaller than or equal to the maximum aggregated bandwidth. |  |  |  |  |  |  |  |  |

Table 5.x.1-2: Template for NR intra-band non-contiguous CA configurations with BCS4/BCS5

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_nX(2A) | CA_nX(2A) | 40, 50, 60, 80,100 | 40, 50, 60, 80, 100 |  |  |  | 180 | 0 |
|  |  | … | … |  |  |  | … | … |
|  |  | See nX channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  |  | TBD | 4 and 5 |

Table 5.x.1-3: Template for NR inter-band CA configurations including FR1 intra-band CA with BCS4/BCS5

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | … | 100 |  |
| CA_nXA-nYA | CA_nXA-nYA | nX | 5 | 10 | 15 | 20 |  |  |  |  | 0 |
|  |  | nY | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |
|  |  | nX | See nX channel bandwidths in Table 5.3.5-1 |  |  |  |  |  |  |  | 4 and 5 |
|  |  | nY | See nY channel bandwidths in Table 5.3.5-1 |  |  |  |  |  |  |  |  |
| CA_nXA-nYC | CA_nXA-nYA | nX | 5 | 10 | 15 | 20 |  |  |  |  | 0 |
|  |  | nY | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |
|  |  | nX | See nX channel bandwidths in Table 5.3.5-1 |  |  |  |  |  |  |  | 4 and 5 |
|  |  | nY | See nY channel bandwidths in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |

With regards to the applicability of BCS4/BCS5 to FR2 intra-band combinations, since all FR2 combinations only have BCS0 and new channel bandwidth have not been added, BCS4/BCS5 are probably not needed for FR2. For FR1+FR2 BCS4/BCS5 combinations the configuration table shall state that BCS0 applies for the intra-band FR2 part (when applicable).

- 5.x.2	Guidelines for band combination with BCS4/BCS5

The following are the rules for applying BCS4/BCS5 for band combination request:

-	BCS4/BCS5 apply to SUL, NR CA, NR DC and SUL and/or NR CA part of inter-band MR-DC while it does not apply to intra-band MR DC.

-	BCS4/ BCS5 shall be requested together, but BCS5 can’t be reported together with BCS4.

-	If needed, traditional BCSs are allowed for all releases. For a new band combination in Rel-17 and onwards, if BCS4/BCS5 are requested, traditional BCSs are allowed but are not needed pending on the proponents, the network of the proponents of BCS4/BCS5 is demanded to recognize BCS4/BCS5.

<< end text proposal >>