3GPP TSG-RAN WG4 Meeting #101-bis-e												  	         R4-2202392

Electronic Meeting, January 17-25, 2022

Source: 	Puloli, Baicells

Title: 					TP to TR 36.779: Operating band, channel bandwidths, channel numbering

Agenda Item: 			8.8.1

Document for:	Approval

# 1	Introduction

During 3GPP RAN plenary #93-e and #94-e, the new work item [1] and its revised version [2] were approved to introduce upper 700MHz A Block as a new E-URTA band in US. The core objectives of the WI are:

The following are the core objectives of the Work Item: - Study the need for any applicable co-location and/or co-existence requirements for the 700MHz NB-IoT band, considering regional regulations, and if needed specify the co-existence requirements - Study any identified unique aspects specific to the proposed new band. - Standardize a new FDD E-UTRA operating band based on the 2 MHz paired spectrum consisting of Upper 700 MHz A Block. To be specific, the uplink band for this new band is 1 MHz, 787-788 MHz; the downlink band is 1 MHz, 757-758 MHz; the duplex spacing is 30 MHz. - Support for 200 kHz bandwidth - Specify band numbering and RF characteristics of the new band. - Ensure the new band supports only NB-IoT operation. - Update the related 3GPP E-UTRA technical specifications to include support for the new band

This contribution is a TP on analysis on UE requirements for introduction of the new E-UTRA band as defined in the WID. It should be noted that the new band is introduced to support standalone NB-IoT operation only.

# 2	References

[1] RP-212618, New WI: Upper_700_MHz_A_Block new E-UTRA Band in the US, RAN#93-e

[2] RP-212917, Revised WID: Introduction of upper 700MHz A block E-UTRA band for the US, RAN#94-e

# 3	Discussion

The expected band-specific changes for TS 36.101 due to introduction of Band 103 are analyzed in this document. The upper 700MHz A block resides between Band 13 and Band 14. Hence, this new band and Band 13 / 14 exhibit the same propagation characteristics. Moreover the regulatory requirements for this band are exactly the same as those for Band 13. Therefore, it is proposed to define the requirements based on the work done for band 13 with the support for NB-IoT operation only.

************************ Start of the TP ************************************************

# 6	Operating band, channel bandwidths, channel numbering

The core objective of the WI is to standardize the upper 700MHz A block defined by the FCC in the US to support standalone NB-IoT operation [1][2].

## 6.1	Operating bands

Band number 103 is assigned to the Upper 700 MHz A Block.

Table 6.1-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 103 | 787 MHz | – | 788 MHz | 757 MHz | – | 758 MHz | FDD |

## 6.2	Operating bands for category NB1 and NB2

Category NB1 and NB2 are designed to operate in the E-UTRA operating band 103 which is defined in Table 6.2-1.

Band 103 is subject to the same emission requirements as Band 13, therefore NS_04 is applicable.

In case UE receives network signaling value NS_04 on any of the operating bands listed in Table 6.2-1 then the lower and upper limit of those bands are shown in Table 6.2-1 to account for the USA emission requirements.

Table 6.2-1 E-UTRA operating bands for NB-IoT in the USA

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 13 | 777 MHz | – | 786.9 MHz | 746 MHz | – | 755.9 MHz | FDD |
| 85 | 698.1 MHz | – | 715.9 MHz | 728.1 MHz | – | 745.9 MHz | FDD |
| 103 | 787.1 MHz | – | 787.9 MHz | 757.1 MHz | – | 757.9 MHz | FDD |

## 6.3	UE channel bandwidth

Band 103 is the first band that supports NB-IoT only operation. Thus far, none of the E-UTRA channel bandwidths is supported, as the whole band is only of 1+1 MHz paired spectrum.

************************ End of the TP ************************************************