3GPP TSG-RAN WG4 Meeting #101-bis-e												  	         R4-2202393

Electronic Meeting, January 17-25, 2022

Source: 	Puloli

Title: 					TP to TR 36.779: Analysis on UE requirements for upper 700MHz A block in the US

Agenda Item: 			8.8.3

Document for:	Approval

# 1	Introduction

During 3GPP RAN plenary #93-e and #94-e, the new work item [1] and its revised version [2] were approved to introduce upper 700MHz A Block as a new E-URTA band in US. The core objectives of the WI are:

The following are the core objectives of the Work Item: - Study the need for any applicable co-location and/or co-existence requirements for the 700MHz NB-IoT band, considering regional regulations, and if needed specify the co-existence requirements - Study any identified unique aspects specific to the proposed new band. - Standardize a new FDD E-UTRA operating band based on the 2 MHz paired spectrum consisting of Upper 700 MHz A Block. To be specific, the uplink band for this new band is 1 MHz, 787-788 MHz; the downlink band is 1 MHz, 757-758 MHz; the duplex spacing is 30 MHz. - Support for 200 kHz bandwidth - Specify band numbering and RF characteristics of the new band. - Ensure the new band supports only NB-IoT operation. - Update the related 3GPP E-UTRA technical specifications to include support for the new band

This contribution is a TP on analysis on UE requirements for introduction of the new E-UTRA band as defined in the WID. It should be noted that the new band is introduced to support standalone NB-IoT operation only.

# 2	References

[1] RP-212618, New WI: Upper_700_MHz_A_Block new E-UTRA Band in the US, RAN#93-e

[2] RP-212917, Revised WID: Introduction of upper 700MHz A block E-UTRA band for the US, RAN#94-e

[3] R4-2119425, UE coexistence and emission requirements for 700 MHz NB-IoT, RAN4#101-e

# 3	Discussion

The expected band-specific changes regarding UE RF requirements as defined in Section 6 & 7 of TS 36.101 due to introduction of new band are analyzed in this document. The upper 700MHz A block resides between Band 13 and Band 14. Hence, this new band and Band 13 / 14 exhibit the same propagation characteristics. Moreover the regulatory requirements for this band are exactly the same as those for Band 13. Therefore, it is proposed to define the requirements based on the work done for band 13 with the support for NB-IoT operation only.

************************ Start of the TP ************************************************

# 7	Study of UE and BS requirements

## 7.1	UE requirements

The upper 700MHz A block resides between Band 13 and Band 14. Hence, this new band and Band 13 / 14 exhibit the same propagation characteristics. Moreover the regulatory requirements for this band are exactly the same as those for Band 13. Therefore, it was decided to define the requirements based on the work done for bands 13 with the support for NB-IoT operation only.

### 7.1.1 	UE maximum output power for category NB1 and NB2

Following the above motivation to reuse the Band 13 work, Band 103 was agreed to use the following NB1 and NB2 power classes, as listed in table 7.1.1-1.

Table 7.1.1-1: UE Power Class for Band 103

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 103 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |

### 7.1.2	UE maximum output power with additional requirements for category NB1 and NB2 UE

To address the additional spurious emission requirement of -13 dBm/30 kHz outside of the band (787 - 788 MHz), as described in Section 5.2, a 100 kHz guard band at both band edges is introduced by making use of the NS_04 signaling for US NB-IoT bands [3]. For Band 103, the result is effectively truncating its bandwidth from 1 MHz to 800 kHz. Band 103 is added to the row of NS_04 in the following table.

Table 7.1.2-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_04 | 5.5F | 2, 4, 5, 12, 13, 17, 25, 26, 66, 71, 85, 103 | N/A |

### 7.1.3	Spurious emission band UE co-existence

UE to UE co-existence is specified the same as for Band 13, i.e., adopting the -50 dBm/MHz emission requirement to/from other 3GPP bands.

Table 7.1.3-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 103 | E-UTRA Band 2, 4, 5, 12, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 13, 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 7.1.3-1 … |  |  |  |  |  |  |  |

************************ End of the TP ************************************************