3GPP TSG-RAN WG4 Meeting #101bis-e													R4-2202403

Electronic meeting, 17th – 25th Jan, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On EIS for FR2-2

Agenda item:	6.16.3.2

Document for:	Approval

# Introduction

The latest WID on extending current NR operation to 71GHz [1] was approved at RAN#94. Before that, 3GPP RAN studied requirements for NR beyond 52.6GHz up to 114.25GHz, potential use cases and deployment scenarios, and NR system design requirements and considerations on top of regulatory requirements as captured in [2].

In this contribution UE REFSENS is discussed.

# Discussion

#### UE REFSENS

Achievable UE REFSENS was evaluated and captured to Table 1. We would like to provide some references for the used assumptions:

- Average antenna element gain is conservative, e.g. R4-2200238 [3] uses worst case 4.2 dBi and R4-2200067 [4] reports 5.2 dBi peak gain

- LNA noise figure is based on system evaluations from TR 38.808 [2], 10 dB NF was also use there, but we chose the more conservative value.

- Allowed implementation losses are generous totaling 9 dB

Table 1: REFSENS for UE receiving with 8-element dual polarized antenna array.

| Parameter | Unit | Value |
| --- | --- | --- |
| Thermal noise floor | dBm/Hz | -174 |
| Signal BW (400MHz : 264 PRB / 120 kHz SCS) | MHz | 380.16 |
| LNA noise Figure | dB | 13 |
| # ant elements per polarization |  | 8 |
| Avg. element gain | dBi | 4 |
| Further antenna losses including roll-off vs. frequency | dB | -3 |
| Realized antenna array gain excluding polarization gain | dBi | 10.0 |
| other losses, including routing | dB | -6 |
| Polarization gain | dB | 2.8 |
| REFSENS | dBm | -82.0 |

Any additional implementations losses would naturally lower either REFSENS value, as well as impact the EIS spherical coverage.

As a summary, we propose UE REFSENS of -82 dBm for 400 MHz channel bandwidth based on very conservative analysis.

Proposal 1: In FR2-2 band n263, handheld UE REFSENS for 400 MHz channel bandwidth is specified -82.0 dBm.

# Conclusion

In this contribution, we discuss REFSENS for UE for a NR band in the range 52.6GHz – 71GHz. We have made following observations and proposals:

Proposal 1: In FR2-2 band n263, handheld UE REFSENS for 400 MHz channel bandwidth is specified -82.0 dBm.

# References

- RP-213540, Revised WID:  Extending current NR operation to 71 GHz

- TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz”, 3GPP

- R4-2015444, “UE RF for NR beyond 52.6 GHz”, Nokia, Nokia Shanghai Bell

- R4-2200238, “60 GHz UE Tx”, Qualcomm Incorporated

- R4-2200067, “Views on UE antenna elements from FR2-2”, Murata Manufacturing Co., Ltd.