3GPP RAN WG4 Meeting #101bis-e		R4-2202414

Electronic meeting, Jan. 17 - Jan. 25, 2021

Agenda item:	6.16.3

Source:	Apple

Title:	Peak EIRP and EIS for 60 GHz

WI/SI:	NR_ext_to_71GHz-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

In this contribution we provide our initial views on the minimum peak EIRP and EIS considering our analysis on the PA and LNA challenges at 60 GHz with 8 antenna elements configuration.

# Discussion

In subsection 2.1 we derive the minimum peak EIRP requirement and in subsection 2.2 the minimum peak EIS requirement, both requirements assume 8 antenna elements.

## 2.1		Peak EIRP for 60 GHz

Collecting all the parameters for the peak EIRP and considering the PA performance analysis, we have estimated the minimum peak EIRP for 60 GHz with 8 antenna elements is 9.2 dBm.

| Parameter | Unit | Proposal |  |
| --- | --- | --- | --- |
|  |  | Nominal | Tolerance |
| Frequency range | GHz | 57 – 71 | 57 – 71 |
| Pout per element | dBm | 2.50 | 0.00 |
| # of antennas in an array |  | 8.00 |  |
| Total conducted power per polarization | dBm | 11.53 | -2.00 |
| Average antenna element gain | dBi | 3.00 |  |
| Antenna gain roll-off loss versus frequency | dB | -2.50 |  |
| Realized antenna array gain | dBi | 9.53 | 0 |
| Polarization gain | dB | 2.80 |  |
| Mismatch and transmission line loss including load pull | dB | -2.90 | -0.70 |
| Beamforming loss (phase shifter and amplitude error) | dB | -2.50 | -0.60 |
| Form factor integration losses | dB | -4.00 | -2.00 |
| Total implementation loss (nominal) | dB | -9.40 | -3.30 |
| Total implementation loss (worst case) | dB |  |  |
| Peak EIRP (Nominal) | dBm | 14.46 |  |
| Tolerance (+/-) | dB |  | -5.30 |
| Peak EIRP (Minimum) | dBm | 9.2 |  |

Proposal 1: The single-band minimum peak EIRP requirement for 60 GHz is 9.2 dBm.

## 2.2	Peak EIS for 60 GHz

Collecting all the parameters for the REFSENS for 60 GHz considering the LNA performance analysis, we have estimated the REFSENS budget with 8 antenna elements is -67.81 dBm/400 MHz.

| Parameter | Unit | Proposal |
| --- | --- | --- |
| Frequency range | GHz | 57 – 71 |
| SNR Requirement | dB | -1.00 |
| Bandwidth | MHz | 400.00 |
| Thermal Noise | dBm/Hz | -174.00 |
| Noise Figure | dB | 18.00 |
| # of antennas in an array |  | 8.00 |
| Average antenna element gain | dBi | 3.00 |
| Realized antenna array gain |  | 9.50 |
| Diversity Gain | dB | 0.00 |
| Antenna gain roll-off loss versus frequency | dB | -2.50 |
| Mismatch and transmission line loss |  | -3.60 |
| Beamforming Loss | dB | -3.10 |
| Total Insertion Loss | dB | -12.70 |
| Form factor Integration losses |  | -6.00 |
| REFSENS | dBm | -67.81 |

Proposal 2: The single-band peak EIS requirement for 60 GHz is -67.81 dBm/400 MHz.

# 3	Conclusion

In this contribution, we have provided an estimate of the expected minimum peak EIRP and EIS considering the challenges to support the wideband 60 GHz operation with 8 antenna elements.
In summary, we have made the following proposals:

Proposal 1: The single-band minimum peak EIRP requirement for 60 GHz is 9.2 dBm.

Proposal 2: The single-band peak EIS requirement for 60 GHz is -67.81 dBm/400 MHz.

# 4	References

RP-202925 “Revised WID: Extending current NR operation to 71 GHz”, CMCC, 3GPP TSG-RAN Meeting #90-e, December 7th – 11th, 2020

R4-2109446 “60 GHz UE RF”, Apple Inc., 3GPP TSG-RAN WG4 Meeting #99-e, May 19th – 27th, 2021