3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2202555

Electronic Meeting, January 17-25, 2022

Agenda item:			6.9.4

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [101bis-e][204] NR_HST_FR2_RRM_1

Document for:	Approval

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

## Background and scope

This T-doc will be used to guide and summarize the email discussion for the topic of Rel-17 NR HST FR2 enhancements RRM core requirements, with the email thread identifier “[101-bis-e][204] NR_HST_FR2_RRM_1”.

In this email thread, the following agenda items will be discussed:

- 6.9.4	RRM core requirements

  - 6.9.4.1	General

  - 6.9.4.2	Number of RX beams

  - 6.9.4.3	RRC Idle/Inactive and connected state mobility requirements

  - 6.9.4.5	Signalling characteristics requirements

  - 6.9.4.6	Measurement procedure requirements

As a moderator for FR2 HST enhancements RRM discussion, we would like to suggest the following candidate target of 1st and 2nd round email discussion:

- 1st round: Focus on the discussion of needed network signalling (Sub-topic 1-1) and deployment-related issues (Sub-topic 1-2) due to a need to prepare LS to RAN2. Take into account achieved agreements in the sub-topics above to agree on the measurement procedure requirements (Sub-topic 2-2).

- 2nd round: Based on the results from the 1st round, identify a few issues that have the potential to achieve agreements and discuss them further. Achieve agreements as much as possible.

## Email discussion guidelines

Unless different guidance is received from the session chairs, the moderator would like to ask companies to adhere to the following guidelines, when taking part in [101-bis-e][204] NR_HST_FR2_RRM_1.

Please also check the “RAN4#101-bis-e meeting arrangements and guidelines”, available on the reflector, for fundamental guidelines and deadlines [R4-2200003].

The preferred method of commenting is to add/update your company’s view directly in this email summary document (use change marks whenever appropriate) and upload it to [101-bis-e][204] NR_HST_FR2_RRM_1 draft folder.

- Draft folder:
[101-bis-e][204] NR_HST_FR2_RRM_1
https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_101-bis-e/Inbox/Drafts/%5B101-bis-e%5D%5B204%5D%20NR_HST_FR2_RRM_1

- It is expected that delegates will download the latest version (including other companies’ versions) of the summary document, insert comments and upload it again.

- To ensure the comments are captured timely and correctly, delegates are encouraged to:

  - Rename the file by adding your company name and changing the file version.
Example: “Summary_204_HST_FR2_RRM_1_v0_1st_round_v05_CATT_Nokia.docx” -> “Summary_204_HST_FR2_RRM_1_v0_1st_round_v06_Nokia_QC.docx”

  - There is no need to send e-mails in the reflector when comments in the summary are added.

  - Please, check for updated base document versions, right before uploading your updates.

- Please, do not hesitate to mark your company as supporting a certain option directly in this document.
Please refrain from rewriting existing options and proposed WFs; ask the moderator (in your company’s comment) to modify/add.

- It is encouraged to give a short reasoning for each view expressed (1-2 sentences are recommended).
Please avoid statements like “Option X”, without further explication or reasoning.

- Please, update your company contact information in the Error! Reference source not found..
It is also recommended to explicitly mention delegate’s name next to company name in the comments if multiple delegates from the same company are commenting.
Alternative way is to identify different delegates in the Track changes username.

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200098 | CATT | Discussion on general issues for FR2 HST Proposal 1: Network signals of uni-/bi-directional operation is not needed from RRM perspective. We can accept it considering only 1 bit overhead. Proposal 2: To resolve potential mobility issue when the train is travelling direction is opposite to the serving beam orientation, it’s better to define the assistance signalling for beam direction. Proposal 3: It is not necessary to introduce UE capability to indicate UE can support 2 RX beams or 6 RX beams. UE supports both 2 RX and 6 RX beams operations. |
| R4-2200099 | CATT | Discussion on requirements on scenario-B with two-side RRH Proposal 1: It is not necessary to separate the requirements for one side of the track and two sides of the track. Use unified RX beams for Scenario B. NW assistance indication to UE may be helpful to solve the two-side RRH. |
| R4-2200259 | Apple | Discussion on general aspects for FR2 HST Proposal 1: Network signals the type of deployment (uni- or bi- direction) to UE. Proposal 2: Support network assisted information, i.e., enable network assisted signaling of SSB index and order per RRH. Proposal 3: UE support Set 1 and Set 2 RRM requirement based on network signaling without a capability. |
| R4-2200260 | Apple | Discussion on number of Rx beam for FR2 HST Proposal 1: Enable cell-specific signalling to indicate set 1 or set 2 RRM requirement when enhanced RRM requirements flag is set in HST FR2 deployments. Proposal 2: It is not necessary to define additional RRM requirement for scenario B (set 2) with two side RRH. |
| R4-2200328 | Qualcomm, Inc. | On NR FR2 HST RRM Requirements Proposal 1: Capture the following description of set 1 requirements applicability in TR: When 2Rx beam sweep based requirement (set 1) applies to the deployment scenario with Dmin > 10m or Hdiff (height difference between train rooftop mounted CPE and RRH) > 10m, performance degradation is expected. Proposal 2: Define different UE capabilities for the two sets of requirements. The 2Rx and 6Rx sweep agreements only apply to neighboring cell and L1-RSRP measurement requirement Proposal 3: Network signals uni-direction or bi-direction deployment to UE. In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes. [Moderator]: Observations and proposal below till Proposal 7 are not treated in RRM-1. Observation 1-1: The RSRP measurement accuracy is off by 0.6dB with 2CP timing offset. Detection large timing change by SSB timing for measurement purpose is not reliable. Observation 1-2: Without timing accuracy requirement on SSB measurement, network or UE can not determine a proper threshold for large timing jump detection. Proposal 4: Add a MAC-CE command to inform UE of the TCI state switch is across RRH and send an LS to RAN2. No performance requirement after TCI state switch across different RRHs before the first TRS is received by UE. Proposal 5: Network can estimate the new NTA by Ds, and then use TA command to correct the remaining timing errors. Proposal 6-1: Introduce UL scheduling restriction after cross-RRH TCI state switch until the first TRS is received after the TCI state switch. Proposal 6-2: Gradual timing adjustment Tq requirement is not applicable to the first transmission after TCI state switch with the MAC-CE command received before the switch, given the scheduling restriction in proposal 6-1. Observation 2: Propagation delay difference between two RRHs can cause large UL to DL interference when two UEs are close two each other and an RRH. Proposal 7: Network applies different offsets to DL frame boundaries of different RRHs to pre-compensate the propagation delay difference across different RRHs to eliminate UL to DL interference across UEs. Network then inform UE the TA change needed after TCI state change across RRHs. Proposal 8: For FR2 HST neighboring cell search enhancement, consider 6Rx sweeping and an additional 1.5 scaling factor to account for the two-side RRH search if network can provide assistant information to UE as proposed in section 2.7. Proposal 9: Keep the following agreement from RAN4#101e WF and remove the square bracket. - For scenario A Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NA)*TSSB). DRX cycle ≤ 80ms; max(TReport, ceil(1.5*M*P*NA)*max(TDRX,TSSB)). Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. NA = 2 - For scenario B Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NB)*TSSB). DRX cycle < 80ms; max(TReport, ceil(1.5*M*P*NB)*max(TDRX,TSSB)). Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. NB = 6 Proposal 10: Select option 1 for neighboring cell measurement in idle mode. Proposal 11: Following R15 requirement, do not consider known cell connection re-establishment in FR2. No enhancement is needed for connection re-establishment for unknown cell due to lack of use cases in FR2 HST. Proposal 12: Follow FR1 HST, do not introduce RLM/BFD enhancement due to lack of use cases in FR2 HST. Proposal 13: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs. Network only signals this mapping when the QCL property relationship can be established across RRHs. If deployment scenario doesn’t satisfy the QCL property similarity across RRHs, network doesn’t have to signal these flags. Send LS in the Appendix to RAN2 for this signaling design. Proposal 14: The following additional beam coverage related information can be signaled to UE - Distance between the projections of adjacent beam peaks on the track - Beam peak direction angle relative to track - The 6 dB beam-width projection on track In addition, UE can report speed to the network. Appendix: LS Draft (Beam Management Enhancement) |
| R4-2200581 | ZTE Corporation | General RRM requirements for HST FR2 Proposal 1: No matter whether the NW signaling of uni-/bi-directional deployments is indicated to UE, the UE can operate properly. Such NW signaling can be helpful for UL timing issue and large Doppler jump issue. [Moderator]: The second part of the proposal rather belongs to RRM-2 email discussion thread. Proposal 2: To resolve the handover issue for the case of opposite direction moving, configuring a different offset value in the same trigger event is enough. Proposal 3: he necessity of introducing NW assistance information signalling is not obvious. Proposal 4: Considering the possible mixed deployment of Scenario A and Scenario B, UE should support both 2 and 6 Rx beam sweeping, not need any corresponding UE capability. |
| R4-2200582 | ZTE Corporation | Discussion on RX beam number for HST FR2 Proposal 1: The applicability restriction of 2 Rx beam requirements is necessary. For the detailed range of Dmin and Hdiff, we are open to discuss. Proposal 2: To address the two-side RRH deployment, two possible solution should be further discussed. - Solution 1: Introduce additional scaling factor - Solution 2: Indicated to UE to switch antenna panel |
| R4-2200627 | CMCC | Discussion on general RRM requirements for FR2 HST Proposal 1: since it was agreed that no separate requirements for uni-/bi-directional deployments are needed, it is not necessary to introduce the network signaling to indicate type of deployment (uni- or bi-direction) to UE. Proposal 2: UE can support both 2Rx and 6Rx beams operations and adapt the number of Rx beams accordingly. No capacity is needed. |
| R4-2200878 | Ericsson | General requirements for HST FR2 Proposal 1: RAN4 sends LS to RAN2 for signaling definition of Set 1 or Set 2 RRM requirements. Proposal 2: Support Option 1: Network signals type of deployment (uni- or bi-direction) to UE. Proposal 3: Slightly prefer option1, enable network assisted signaling of SSB index and order per RRH, if have to choose between option1 and option2. UE may improve operation to some extent with assistance of this information. Option2 is rather complex, it may introduce more uncertainty in practice. Proposal 4: Suggest discussing the necessity of signaling of information on direction of RRH panel in case that occasional change of direction of RRH panel orientation. Proposal 5: Suggest discussing the necessity of signaling of RRH side position information. Proposal 6: Support Option 2: UE shall support both 2Rx and 6Rx beams operations and adapt the number of Rx beams accordingly. No capacity is needed. |
| R4-2200881 | Ericsson | Number of RX beams for HST FR2 Proposal 1: Send LS to RAN2 to define the following signaling: - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements Proposal 2: There are several options to deal with the case of scenario-B (Set 2) with two-side RRH: - No special consideration for two-side RRH, rely on UE’s implementation to cover both directions. - Introduce signaling to indicate the queue of directions of RRHs in one cell to UE. - Extra scaling factor is used for scenario-B (Set 2) to allow UE more time if UE justifies the necessity. We prefer option2 but we’re open to discussion. |
| R4-2200883 | Ericsson | LS on FR2 HST RRM [Draft] LS on network signalling for Rel-17 NR HST RRM |
| R4-2201156 | OPPO | Discussion on general requirements for FR2 HST Proposal 1: Signalling of uni-/bi-directional operation is not needed. Proposal 2: No UE capability is need to support different Rx beam sweeping number. |
| R4-2201175 | Huawei, Hisilicon | General discussion on network signaling and UE capability in FR2 HST Proposal 1: Signalling of uni-/bi-directional operation is not needed from RRM perspective. Proposal 2: Define different UE capabilities to support 2Rx beams and 6Rx beam operation. |
| R4-2201666 | Nokia, Nokia Shanghai Bell | LS on network signalling for Rel-17 FR2 NR HST RRM |
| R4-2201667 | Nokia, Nokia Shanghai Bell | Discussion on network signalling for FR2 HST RRM Observation 1: For deployment Scenarios A or B, uni- and bi-directional modes have the same set of RRM requirements. Observation 2: No obvious potential benefits in defining network signalling to indicate uni- or bi-directional mode. Proposal 1: Network signalling of uni- or bidirectional is not needed. Observation 3: With planned SSB without DRX and DRX 40 ms work reasonably well, but already DRX 80 ms has clearly too long delays to provide robust mobility. Observation 4: For unidirectional Scenario A, the DRX upper bound is set to 40 ms for UE moving in the opposite direction to the RRH TX beam pointing direction and 80 ms for UE moving in the same direction as the RRH TX beam pointing direction. Proposal 2: For unidirectional Scenario A, set the DRX upper bound to [40 ms]. |
| R4-2201766 | Samsung | Discussion on network signaling and UE capability for FR2 HST <Network Signaling> Proposal 1: RAN4 needs to inform RAN2 to introduce the network signaling to configure UE to follow either Set 1 or Set 2 enhanced RRM requirements in terms of different RX beams (i.e. RX beam sweeping scaling factor) per UE. Proposal 2: Network signaling of uni-/bi-directional operation shall not be introduced in Rel-17 FR2 HST WI. Proposal 3: The network assistance of SSB configuration (e.g., RRC signaling for SSB index and order per RRH, or other indication for inter-/inter-RRH) shall not be introduced in Rel-17 FR2 HST WI. Proposal 4: A new NW signaling is introduced for NW to enable the one shot large UL timing adjustment. [Moderator]: The proposal note treated because it rather belongs to RRM-2 email discussion thread. <UE Capability > Proposal 5: FR2 HST UE (power class 6 UE) shall mandatorily support both Set 1 and Set 2 enhanced RRM requirements, in terms of different RX beams (i.e. RX beam sweeping scaling factor) per UE. Proposal 6: FR2 HST UE (power class 6 UE) shall mandatorily support the mechanism of one shot large UL timing adjustment. [Moderator]: The proposal note treated because it rather belongs to RRM-2 email discussion thread.. |
| R4-2201767 | Samsung | LS on Network Signalling for Rel-17 NR FR2 HST RRM |
| R4-2201847 | Nokia, Nokia Shanghai Bell | TP to TR 38.854 on Mobility Performance in HST FR2 Deployment Scenarios |
| R4-2201848 | Nokia, Nokia Shanghai Bell | TP to TR 38.854 on the Number of Rx beams |
| R4-2200544 | Intel Corporation | Discussion on Rel-17 RAN4 UE feature list The following UE features are proposed for the Rel-17 NR FR2 HST WI. More details can be found in our companion paper [R4-2201877, Views on FR2 HST PDSCH performance requirements, Intel Corporation, RAN4 #101-bis-e, January 2022.]. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Network signaling

Sub-topic description:

Open issues and candidate options before e-meeting:

#### Issue 1-1-1: Signaling of uni-/bi-directional operation

- Background

Way forward from RAN4#101-e:

Discuss further a need to introduce network signaling:

  - Option 1: Network signals type of deployment (uni- or bi-direction) to UE

  - Option 2: Signaling of uni-/bi-directional operation is not needed.

- Proposals and/or Observations

  - Proposal 1 (CATT): Network signals of uni-/bi-directional operation is not needed from RRM perspective. We can accept it considering only 1 bit overhead.

  - Proposal 2 (Apple): Network signals the type of deployment (uni- or bi- direction) to UE.

  - Proposal 3 (Qualcomm): Network signals uni-direction or bi-direction deployment to UE. In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes.

  - Proposal 4 (CMCC): since it was agreed that no separate requirements for uni-/bi-directional deployments are needed, it is not necessary to introduce the network signaling to indicate type of deployment (uni- or bi-direction) to UE.

  - Proposal 5 (Ericsson): Support Option 1: Network signals type of deployment (uni- or bi-direction) to UE.

  - Proposal 6 (OPPO): Signalling of uni-/bi-directional operation is not needed.

  - Observation 1 (Nokia): For deployment Scenarios A or B, uni- and bi-directional modes have the same set of RRM requirements.

  - Observation 2 (Nokia): No obvious potential benefits in defining network signalling to indicate uni- or bi-directional mode.

  - Proposal 7 (Nokia): Network signalling of uni- or bidirectional is not needed.

  - Proposal 8 (Samsung): Network signaling of uni-/bi-directional operation shall not be introduced in Rel-17 FR2 HST WI.

- Candidate options:

  - Option 1 (Apple, Qualcomm, Ericsson): Network signals type of deployment (uni- or bi-direction) to UE

  - Option 2 (CATT, CMCC, OPPO, Nokia, Samsung): Signaling of uni-/bi-directional operation is not needed.

- Recommended WF

  - No consensus of on the Issues. Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Support Option 1 for the following reasons: - No potential benefits are observed as it is possible for the direction of DL beams of unidirectional to switch from one direction to the opposite when UE is moving from one RRH to the next assuming non-uniform deployment scenarios. - It is not obvious how such signalling can address open issues (e.g., Doppler shift jump, one-shot timing adjustment). |
| QC | Firstly, this signaling is helpful for the UL timing issue in the second thread. In addition, although the requirements are the same for uni and bi directional model, this information is necessary for UE. Note that most of the previous analysis focus on number of beams calculated in one direction and one side. Uni vs bi-directional signaling is useful when considering two side RRHs and two directions of beams. Moreover, the analysis is targeting minimum requirement, while additional information easily signaled by network can help to improve system performance and speed up measurement a lot, which is beneficial in practice. Given that this information is only one bit signaling and available based on deployment, we support option 1 to add this signaling. |
| Ericsson | Support Option 2, we suppose NW signaling of Uni- or Bi-directional deployment can and optimize specific issues occurs in HST FR2 scenario, e.g. UE may optimize beam management or inter-cell mobility in Uni- or Bi-directional deployment and also benefit to two-side RRH along with railway. |
| Apple | It seems the candidate option 1 and 2 in summary and background are switched. We support uni-directional deployment signaling, i.e., option 1 in candidate options. Signaling whether this is the uni-directional or bi-directional operation can simplify UE’s operation for beam management and the large propagation delay correction. For example, if this is uni-directional, UE will perform beam management using only one panel as UE move along the track. While on the other hand, if this is bi-directional deployment, UE needs to perform beam management using both antenna panels. It is also observed that to uni-directional deployment experience large propagation issue, therefore network signaling can help UE to perform one-time large time adjustment to handle the delay. |
| Huawei | Prefer option 2. As it is agreed on RX beam number in RAN4#100e meeting that [2] RX beams in scenario A and [6] RX beams in scenario B for both uni-/bi-directional scenarios [1]. From UE beam sweeping perspective, there is no different UE behaviour in uni-directional and bi-directional deployment in the same scenario A/B |
| ZTE | We have no strong view. Since only 1 bit payload, so we can support such signalling. |
| Samsung | By following the “candidate options” listed by Moderator, we support Option 2, no need for signaling. At least till now, no requirement differentiation is provided for RF, RRM and Demod, so we are questioning about the expected UE behavior should be different or not if this signaling is provided. |
| CMCC | It was agreed that no separate requirements for uni-/bi-directional deployments. From this point of view, we do not see the necessity to have the network signaling to indicate type of deployment (uni- or bi-direction) to UE. |
| Nokia | As there is an editorial mistake in our earlier comment, we would like to correct it to support Option 2. Thanks for spotted the mistake! Based on the comments from the proponents of Option 1, the signalling consumes one bit, which can only indicate whether uni- or bi-directional mode to UE. We can see some benefits in terms of beam management. However, it is unclear how such one bit signalling can be useful to deal with the UL timing issue (or one-time large time adjustment). Assuming the issue only exists in the uni-directional mode might not be valid. We think this issue is related to Issue 1-1-4. Thus, it can be further discussed in Issue 1-1-4. |
| CATT | From RRM perspective, we prefer option 2. |

#### Issue 1-1-2: Signaling to indicate set 1 or set 2 RRM requirement

- Background

GtW Agreement from RAN4#101-e:

  - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements

- Proposals and/or Observations

  - Proposal 1 (Apple): Enable cell-specific signalling to indicate set 1 or set 2 RRM requirement when enhanced RRM requirements flag is set in HST FR2 deployments.

  - Proposal 2 (Ericsson): RAN4 sends LS to RAN2 for signaling definition of Set 1 or Set 2 RRM requirements.

  - Proposal 3 (Ericsson): Send LS to RAN2 to define the following signaling:

    - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements

  - Proposal 4 (Samsung): RAN4 needs to inform RAN2 to introduce the network signaling to configure UE to follow either Set 1 or Set 2 enhanced RRM requirements in terms of different RX beams (i.e. RX beam sweeping scaling factor) per UE.

- Recommended WF

  - Agree on tentative agreement:
Include signaling of Set 1 or Set 2 enhanced RRM requirement into the LS to RAN2.

  - Continue the discussion of LSs in Issue 1-1-6.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Nokia has contributed LS requesting RAN2 to specify such signalling. |
| QC | Support recommended WF |
| Ericsson | Agree with recommended WF. |
| Apple | Network signaling of set 1 and set 2 were agreed in previous meeting. Our proposal is cell specific signaling since set 1 and set 2 requirements are mainly depends on network deployment. |
| Intel | Agree with recommended WF |
| Huawei | Agree with the recommended WF and cell specific signaling is fine. |
| ZTE | Agree with recommended WF. |
| Samsung | Agree with recommended WF, and as we mentioned in draft LS, per-cell signaling is okay. |
| CMCC | OK with the recommended WF |
| CATT | Support the tentative agreement. Also agree that it is per cell level. |

#### Issue 1-1-3: Applicability of enhanced RRM requirements in TP

- Background

It was noted at RAN4#101-e GTW that

  - Note: the applicability of Set 1/2 requirements to the FR2 HST scenarios will be captured in the TR

- Proposals and/or Observations

  - Proposal 1 (Qualcomm): Capture the following description of set 1 requirements applicability in TR:

    - When 2Rx beam sweep based requirement (set 1) applies to the deployment scenario with Dmin > 10m or Hdiff (height difference between train rooftop mounted CPE and RRH) > 10m, performance degradation is expected.

  - Proposal 2 (Qualcomm): Define different UE capabilities for the two sets of requirements. The 2Rx and 6Rx sweep agreements only apply to neighboring cell and L1-RSRP measurement requirement

  - Proposal 3 (ZTE): The applicability restriction of 2 Rx beam requirements is necessary. For the detailed range of Dmin and Hdiff, we are open to discuss.

- Proposals for the TR:

  - Proposal 1 (Qualcomm, ZTE): When 2Rx beam sweep based requirement (set 1) applies to the deployment scenario with Dmin > 10m or Hdiff (height difference between train rooftop mounted CPE and RRH) > 10m, performance degradation is expected.

  - Proposal 2 (Qualcomm): The 2Rx and 6Rx sweep agreements only apply to neighboring cell and L1-RSRP measurement requirement

- Recommended WF

  - In the 1st round, companies are invited to share their opinions whether the proposals above can be included in the TR.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Regarding Proposal 1, it is not necessary to agree on the proposal as such, but prefer to see the text proposal to the TR. Regarding Proposal 2, we prefer to consider a case-by-case basis rather than restricting the agreement to these two requirements. |
| QC | We are fine with capturing proposal 1 text in TR. Proposal 2 is related to other topics, can be discussed separately in those topics as Nokia suggests. |
| Ericsson | Agree on Nokia’s comments on proposal 1 and proposal 2. |
| Intel | For Proposal 1: We prefer to capture different points in TR. Based on our observations, 2 Rx can be applicable even for Dmin = 50m with no significant performance degradation. For Proposal 2: We believe that it should not be considered as a proposal for TR. And as mentioned by Nokia, it should be considered on case-by-case basis. |
| ZTE | For Proposal 1: We do not mean performance degradation is expected once Dmin > 10m or Hdiff (height difference between train rooftop mounted CPE and RRH) > 10m. We just believe the deployment in the field would not always guarantee Dmin = 10m and Hdiff = 10m, so the applicable value for Dmin and Hdiff without performance degradation should be discussed, such as Dmin = 50m. For Proposal 2: Agree with Nokia’s view, we have not discuss this question until now. |
| Samsung | P1: Needs clarification, Dmin > 10m is the proposed condition for set 1? Seems the proponent intention is opposite, as I captured this from R4-2200328: “Therefore, we propose that the 2Rx beam sweep based requirement only applies to the cases in which Dmin and Hdiff are both no greater than 10m. When Dmin or Hdiff is larger than 10m, DL Tx beam angle range becomes larger with the same switching point, and 2Rx beams can not cover it.” P2: Nokia’s proposal on case-by-case basis is reasonable. |
| QC | To Samsung: Dmin > 10m is the proposed condition for expected performance degradation when set 1 is signaled. To ZTE: could you share Dmin = 50m analysis to demonstrate that the performance degradation is not observed? To Intel: could you share the observation of Dmin <= 50m without performance degradation, and we want to know the observation is based which settings and what’s the performance derived with 2 and 6 beams? We use the analysis framework in R4-2104905, and found that when Dmin is 50m, the optimal switching point when following scenario A setting is at Ds_offset = 125m. When we follow scenario B setting and allow 6 beams, the optimal Ds_offset becomes 50m and the throughput is 20% higher than following scenario A setting (applying set 1 requirement). Therefore, it is reasonable to say that Dmin = 50m exhibits performance degradation, and consider our original proposal, expect performance degradation when Dmin > 10m and using set 1 requirement. |

#### Issue 1-1-4: Signaling of SSB configuration

- Background

Way forward from RAN4#101-e:

  - Further discuss signaling of RRH SSB configuration:

    - Option1: Enable network assisted signaling of SSB index and order per RRH

    - Option 2: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs when deployment parameters Dmin and Ds are similar for adjacent RRHs.

    - Option 3: The system can work without such assistance signaling

- Proposals and/or Observations

  - Proposal 1 (Apple): Support network assisted information, i.e., enable network assisted signaling of SSB index and order per RRH.

  - Proposal 2 (Qualcomm): Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs. Network only signals this mapping when the QCL property relationship can be established across RRHs. If deployment scenario doesn’t satisfy the QCL property similarity across RRHs, network doesn’t have to signal these flags. Send LS in the Appendix to RAN2 for this signaling design.

  - Proposal 3 (ZTE): he necessity of introducing NW assistance information signalling is not obvious.

  - Proposal 4 (Ericsson): Slightly prefer option1, enable network assisted signaling of SSB index and order per RRH, if have to choose between option1 and option2. UE may improve operation to some extent with assistance of this information. Option2 is rather complex, it may introduce more uncertainty in practice.

  - Proposal 5 (Samsung): The network assistance of SSB configuration (e.g., RRC signaling for SSB index and order per RRH, or other indication for inter-/inter-RRH) shall not be introduced in Rel-17 FR2 HST WI.

- Candidate options:

  - Option 1 (Apple, Ericsson): Enable network assisted signaling of SSB index and order per RRH

  - Option 2 (Qualcomm): Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs.

  - Option 3 (ZTE, Samsung): The network assistance of SSB configuration shall not be introduced in Rel-17

- Recommended WF

  - Continue the discussion in the 1st round

  - It recommended to discuss on the necessity of signaling but other than for UL timing adjustment. Solutions to the problem of a large jump in propagation delay should be treated in the RRM-2 email discussion thread.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | The present description for Options 1 and 2 is abstract. It is not clear how the signalling of either option is realized even though we know it is RAN2 responsibility. Without concrete details, it is hard to evaluate the full benefits. In a uniform deployment scenario (where the number of TX beams per RRH is the same and RRH TX beam pointing directions are the same), Options 1 and 2 would give similar end results. However, the signalling overhead may not be the same. Do we understand correctly Option 2 require the signalling in Issue 1-1-5? Is it not standalone? We’re open to further discuss. |
| QC | As we explained in our contribution, we propose to signal this type of mapping to UE when UE is connected to RRH1 before (or during) the switch to RRH2 via MAC-CE, which indicates that SSB 1 and 5 are with similar QCL properties (also 2 to 6 etc.) The major difference between option 1 and 2 is the information provided: it’s not necessary that cross RRH distances are identical, and track is absolutely straight. In this case, SSB index itself doesn’t imply similar QCL properties, and option 2 mapping is needed to distinguish which SSB indexes has the similar QCL properties and which doesn’t. RRH1; RRH2. 1; 5. 2; 6. 3; 7. 4; 8. |
| Ericsson | The signaling in proposal 1 can be defined as a list or matrix, (1,2,3,4); (5,6,7,8); (9,10,11,12); (13,14,15,16), which build the SSB indexes for a cell. each bracket is for one RRH in the cell: the numbers in each bracket are SSB index. |
| Apple | Other than one time large TA adjustment, the SSB index and order information can also help UE to derive the Rx beam as UE move along the track. Agree with the signaling example Ericsson provided. It is minimum change since current specification already signal the SSB index per cell. For option 2, our understanding is a new QCL definition needs to be introduced in this case. Current defined QCL types A-D all assume the UE is in a fixed location. However, the QCL discussed here assumed UE is in different location and using the same Rx beam to receive different SSBs on different RRHs. This requires RAN1 discussion on defining new QCL type to different QCL relationship between SSB of different RRH with UE in different locations. If this is agreed, RAN4 needs to send LS to RAN1. |
| Intel | Support option 3. For Option 1 we also see benefits, which are mostly related to UL timing adjustment issue treated in thread [205]. |
| ZTE | Support Option 3. Even without such NW assistance signaling, the system can also work, we can not find any indispensable excuse for Option 1 or Option 2. Considering the signaling overhead is not negligible, so we support Option 3. |
| Samsung | Option 3. From the necessity of the network assistance information perspective: For bi-directional RRH deployment, even inter-RRH beam switching does not necessarily mean there is large propagation delay difference. In other words, even RAN4 introduce the network assistance of indication of inter-RRH, a complicated applicability rule (involving uni-/bi-directional deployment, etc.) is still required, otherwise the information of inter-/intra-RRH cannot be useful. From the complexity of introducing network assistance information perspective: For the RRC signaling for SSB index and order per RRH, obviously it is needed to be broadcast in SIB which will incur additional overhead, and more seriously we have not yet seen a solid and simple proposal which can be delivered to RAN2 for implementation directly before the completion deadline of Rel-17 work item (i.e., within Q1 2022). |
| CATT | We agree option 1 has benefit. but how to handle the tight schedule issue for other group before the R17 deadline? |

#### Issue 1-1-5: Signaling of beam coverage related information

- Background

The proposal was already introduced in the previous RAN4 meetings but didn’t get enough support among the companies. Therefore, the moderator’s opinion is that it is doubtable that it may get standardized in terms of HST FR2 Rel-17 WI.

- Proposals and/or Observations

  - Proposal 1 (Qualcomm): The following additional beam coverage related information can be signaled to UE

    - Distance between the projections of adjacent beam peaks on the track

    - Beam peak direction angle relative to track

    - The 6 dB beam-width projection on track

In addition, UE can report speed to the network.

- Recommended WF

  - It is recommended to discontinue the discussion unless new strong arguments are disclosed.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Can the proponent explain why 6 dB beamwidth projection on track is signalled instead of 3 dB beamwidth? This signalling seems to be per RRH or per cell? |
| QC | We are opening to discuss the beam width parameter. We expect SSB index switch happens at 6dB beam width in HST, but other numbers are possible. Since we are in a late stage of this WI, we can accept moderator’s recommendation. |
| Intel | Ok with recommended WF. |
| ZTE | OK with recommended WF. |
| Samsung | Ok with recommended WF. |
| CATT | Support Recommended WF |

#### Issue 1-1-6: LS to RAN2 on Network Signaling

- Background

A number of Draft LSs were submitted to the meeting:

  - LS Draft on Beam Management Enhancement in the Appendix of R4-2200328 by Qualcomm

    - Pending on agreements in Issues 1-1-4 and 1-1-5

  - [Draft] LS on network signaling for Rel-17 NR HST RRM, R4-2200883 by Ericsson

    - Includes signaling of Set 1 or Set 2 enhance RRM requirements

    - Pending on agreements in Issues 1-2-2 and 1-2-3.

  - [Draft] LS on network signalling for Rel-17 FR2 NR HST RRM, R4-2201666 by Nokia

    - Focusing on signaling of Set 1 or Set 2 enhance RRM requirements

  - [Draft] LS on Network Signalling for Rel-17 NR FR2 HST RRM, R4-2201767 by Samsung

    - Includes signaling of Set 1 or Set 2 enhance RRM requirements

    - Pending on the final agreement on One Shot Large UL Timing Adjustment Flag in RRM-2 email discussion thread

In the WF from RAN4#99-e it was agreed to have a combined LS to RAN2 on all HST FR2 signaling flags:

  - Add HST FR2 Network deployment flag to the combined LS to RAN2 on all HST FR2 signalling flags.

  - At that meeting Nokia has volunteered to edit the combined LS.

A need to include signaling of Set 1 or Set 2 enhance RRM requirements follows directly from the agreement in RAN1#1-1e (see also Issue 1-1-2).

- Recommended WF

  - Since some of LSs are still pending on agreements in the other Issues and have overlapping requests to RAN2, in the 1st round it is recommended to agree on the combined LS editor, i.e., whether it is Nokia or the other volunteering company(ies).

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia1 | Nokia is still interested to be the editor of combined CR on network signaling to RAN2. |
| Ericsson | Agree with Nokia’s comments. |
| Samsung | We volunteer to be LS drafter, and as we comment in two meeting earlier, a combined LS for all relevant signaling should be drafted when the clear requirement with different requirement sets and different signaling can be provided. We only have agreement after last meeting’s GTW. Content-wise, also pending on the discussion on the timing part. |

### Sub-topic 1-2: Deployment-related issues

Sub-topic description

Open issues and candidate options before e-meeting:

#### Issue 1-2-1: Train travelling opposite to the serving beam

- Background

One of the companies has demonstrated that that there are no mobility problems in this opposite scenario-A when DRX is off or for DRX cycle length 40. However, problems start to appear for DRX 80 and above. Other companies think that alternative solution would be selection of different offset.

- Proposals and/or Observations

  - Proposal 1 (CATT): To resolve potential mobility issue when the train is travelling direction is opposite to the serving beam orientation, it’s better to define the assistance signalling for beam direction.

  - Proposal 2 (ZTE): To resolve the handover issue for the case of opposite direction moving, configuring a different offset value in the same trigger event is enough.

  - Observation 1 (Nokia): With planned SSB without DRX and DRX 40 ms work reasonably well, but already DRX 80 ms has clearly too long delays to provide robust mobility.

  - Observation 2 (Nokia): For unidirectional Scenario A, the DRX upper bound is set to 40 ms for UE moving in the opposite direction to the RRH TX beam pointing direction and 80 ms for UE moving in the same direction as the RRH TX beam pointing direction.

  - Proposal 3 (Nokia): For unidirectional Scenario A, set the DRX upper bound to [40 ms].

  - Proposal 4 (CATT): For the potential mobility issue when the train is travelling direction is opposite to the serving beam orientation, it can be resolved by using proper event offset in NW if NW knows the UE moving direction.

  - Proposal 5 (Ericsson): Based on our understanding, the following are possible solutions to deal with the problem (we suppose a similar problem may occur in L1 mobility):

    - Shorter DRX may alleviate the problem, but we doubt it will completely resolve it.

    - Implement UE-initiated TCI activation/deactivation, but no additional signaling is allowed with regard to agreement in previous meetings.

    - Alter the conditions of L1 mobility and L3 mobility, for example, parameters in A3/A5  in L3 mobility adopts different values, before the SNR drops. The addition of signaling to UE, for example intra-RRH mobility or inter-RRH mobility, may be beneficial to the solution, but this needs to be investigated.

We encourage open discussions on the issue.

- Candidate options:

  - Option 1 (ZTE, CATT): Configure a different mobility parameter, e.g., HO offset for opposite direction

  - Option 2 (Nokia): For unidirectional Scenario A, set the DRX upper bound to [40 ms]

- Recommended WF

  - Discuss candidate options in the 1st round.

  - Clarify whether and which network assistance signaling is needed for Option 1

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Options 1 and 2 do not contradict with each other. Option 2 is simpler but it can be used in combination with Option 1. |
| QC | According to analysis from Nokia, option 1 may not resolve the issue completely. We can support option 2 as a resolution. |
| Ericsson | Option2a or Option 2b. Even the deployment scenario isn’t in WI, we suggest to check the practical risk of improper UE implementation which isn’t optimized for the issue. |
| Intel | Support Option 2 |
| ZTE | Support both Option 1 and 2. |
| Samsung | Support Option 1. Both Option 1 and Option 2 can be achieved by proper NW configuration. Restriction on DRX upper bound in the RAN4 spec may not be needed for other NW vendors, so we don’t see the necessity of introducing additional restriction here. |
| CATT | Support option 1. And option 1 and option 2 can be NW implementation. No spec impact. |

#### Issue 1-2-2: Two-side RRM deployment in Scenario-B

- Background

It RAN4#101-e it was agreed that

Consideration of RRH positions at one/both sides of rail track doesn’t have impact on 6Rx beams agreement in Scenario B (set 2)

However, it left open whether it is necessary and how to address scenario-B (Set 2) with two-side RRH.

Some companies propose not to consider two-side RRH deployments in a special way. At the same time, two groups of solution are proposed:

  - Based on network signaling

- Proposals and/or Observations:

  - Proposal 1 (CATT): It is not necessary to separate the requirements for one side of the track and two sides of the track. Use unified RX beams for Scenario B.  NW assistance indication to UE may be helpful to solve the two-side RRH.

  - Proposal 2 (Apple): It is not necessary to define additional RRM requirement for scenario B (set 2) with two side RRH.

  - Proposal 3 (Qualcomm): For FR2 HST neighboring cell search enhancement, consider 6Rx sweeping and an additional 1.5 scaling factor to account for the two-side RRH search if network can provide assistant information to UE as proposed in section 2.7.

  - Proposal 4 (ZTE): To address the two-side RRH deployment, two possible solution should be further discussed.

    - Solution 1: Introduce additional scaling factor

    - Solution 2: Indicated to UE to switch antenna panel

  - Proposal 5 (Ericsson): Suggest discussing the necessity of signaling of RRH side position information.

  - Proposal 6 (Ericsson): There are several options to deal with the case of scenario-B (Set 2) with two-side RRH:

    - No special consideration for two-side RRH, rely on UE’s implementation to cover both directions.

    - Introduce signaling to indicate the queue of directions of RRHs in one cell to UE.

    - Extra scaling factor is used for scenario-B (Set 2) to allow UE more time if UE justifies the necessity.

We prefer option2 but we’re open to discussion.

- Candidate options:

  - Option 1 (Apple): No special consideration for two-side RRH deployment

  - Option 2a (Ericsson): Introduce signaling to indicate the queue of directions of RRHs in one cell to UE

  - Option 2b (ZTE, Ericsson): Indicated to UE to switch antenna panel/RRH side position information

  - Option 3 (Qualcomm, ZTE): Use extra [1.5] scaling for DRX [<=80] ms

- Recommended WF

  - Discuss candidate options in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Support Option 1. |
| QC | We support option 3. Note that option 3 is not asking for a separate set of requirement. The proposal is to apply a factor of 1.5 on under discussion Mpss/sss and Mmeaurement factors and the agreed number of Rx beams. According to our analysis, with 1.5 factor, the detection and measurement is still fast enough for ISD = 700m. We are open to the signaling, but since it is per RRH configuration, MAC-CE command might be needed instead of RRC configuration? |
| Apple | Based on agreement that “RRH positions at one/both sides of rail track doesn’t have impact on 6Rx beams agreement in Scenario B (set 2)” , it was our understanding that the doubling effect is already included in the requirement for Scenario B with 6 Rx beams. For Option 3, when 1.5 scaling is used with 6 Rx beam, the requirement becomes 9. It is even larger than 8 Rx beam. For option 2b, the UE switches antenna panel based on bi-directional deployment signaling. This is one of the reasons we think uni-directional or bi-directional signaling is helpful other than the one-time large TA adjustment. However, it is not clear why UE switch panel based on one side or two side deployment. Is the assumption here still UE has 2 panels, one facing front, one facing back? |
| Intel | Support Option 1 and agree with comment from Apple – 6 Rx beams already cover both sides. |
| ZTE | Support Option 2b and 3. For Option 2b, to Apple: As shown in Fig.1, UE can switch the receiving panel for two-side RRH deployment, then better beam coverage can be guaranteed especially when UE is close to the serving RRH. Here one panel facing the RRHs on the left of the railway, while another panel facing the RRHs on the right of the railway. Fig.1 Two CPE panels for two sides deployment(uni-directional) |
| Samsung | Support Option 1 and agree with comment from Apple. |
| CATT | Support option 1. |
| QC | Based on our understanding, all the previous analysis are based on one-side RRH, could Apple, Samsung or Intel provides the Tdoc number showing that 6Rx is enough to cover both sides? If all the previous analysis is on one-side, then additional factor is needed if we consider both sides. Note that the agreement “RRH positions at one/both sides of rail track doesn’t have impact on 6Rx beams agreement in Scenario B (set 2)” doesn’t exclude using an additional scaling factor to account for two-side RRHs, since the complete agreement is (R4-2120292): Agreements: Consideration of RRH positions at one/both sides of rail track doesn’t have impact on 6Rx beams agreement in Scenario B (set 2) Way forward: FFS, whether it is necessary and how to address scenario-B (Set 2) with two-side RRH Way forward from GtW: - Baseline: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1 FFS if a different scaling factor is needed for scenario-B with two-side RRH |

#### Issue 1-2-3: Change of RRH panel orientation in uni-directional deployments

- Proposals and/or Observations:

  - Proposal 1 (Ericsson): Suggest discussing the necessity of signaling of information on direction of RRH panel in case that occasional change of direction of RRH panel orientation.

  - Proposal 2 (Qualcomm): Network signals uni-direction or bi-direction deployment to UE. In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes.

- Candidate options:

  - Option 1: Network signals the beam direction of new RRH

  - Option 2: Other options are not precluded

- Recommended WF

  - It is recommended to discuss the candidate options in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | The rationale behind Option 1 is to reduce UE RX beam searching time. What is the difference with Issue 1-1-5, e.g., beam peak direction angle relative to track? |
|  | To Nokia: if 1-1-5 is agreed, we don’t need option 1. However, option 1 is a signaling solution with less complexity, when compared to issue 1-1-5. |
| Intel | There is no need in additional signalling. If there will be no uni-/bi- directional signalling, then UE will always assume bi-directional where we don’t have that issue. If there will be uni-/bi- directional signalling, then the issue can be resolved with correct combination of that signalling: operating in uni-directional -> switching to bi-directional during the beam direction change -> switch back to uni-directional after new direction is identified |
| Samsung | Agree with Intel’s comment. |

### Sub-topic 1-3: UE capabilities

Sub-topic description

Open issues and candidate options before e-meeting:

#### Issue 1-3-1: Capability to support different RX beam sweeping number

- Background

Way forward from RAN4#101-e:
Further discuss UE capability to support different RX beam sweeping number:

  - Option 1: Define different UE capabilities to support 2Rx beams and 6Rx beam operation.

  - Option 2: UE can support both 2Rx and 6Rx beams operations and adapt the number of Rx beams accordingly. No capacity is needed.

- Proposals and/or Observations

  - Proposal 1 (CATT): It is not necessary to introduce UE capability to indicate UE can support 2 RX beams or 6 RX beams. UE supports both 2 RX and 6 RX beams operations.

  - Proposal 2 (Apple): UE support Set 1 and Set 2 RRM requirement based on network signaling without a capability.

  - Proposal 3 (Qualcomm): Define different UE capabilities for the two sets of requirements. The 2Rx and 6Rx sweep agreements only apply to neighboring cell and L1-RSRP measurement requirement

  - Proposal 4 (ZTE): Considering the possible mixed deployment of Scenario A and Scenario B, UE should support both 2 and 6 Rx beam sweeping, not need any corresponding UE capability.

  - Proposal 5 (CMCC): UE can support both 2Rx and 6Rx beams operations and adapt the number of Rx beams accordingly. No capacity is needed.

  - Proposal 6 (Ericsson): Support	Option 2: UE shall support both 2Rx and 6Rx beams operations and adapt the number of Rx beams accordingly. No capacity is needed.

  - Proposal 7 (OPPO): No UE capability is need to support different Rx beam sweeping number.

  - Proposal 8 (Samsung): FR2 HST UE (power class 6 UE) shall mandatorily support both Set 1 and Set 2 enhanced RRM requirements, in terms of different RX beams (i.e. RX beam sweeping scaling factor) per UE.

- Candidate options:

  - Option 1 (Qualcomm): Define different UE capabilities to support 2Rx beams and 6Rx beam operation.

  - Option 2 (CATT, Apple, ZTE, CMCC, Ericsson, OPPO, Samsung): UE can support both 2Rx and 6Rx beams operations and adapt the number of Rx beams accordingly. No capacity is needed.

- Recommended WF

  - Check in the 1st round if Option 2 is agreeable:
FR2 HST UE (power class 6 UE) shall mandatorily support both Set 1 and Set 2 enhanced RRM requirements, in terms of different RX beams (i.e., RX beam sweeping scaling factor) per UE.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Although we believe the capabilities provide flexibility for UE implementation, we can compromise to the recommended WF if issue 1-1-3 proposal 1 can be agreed and our compromise proposal for issue 3-1-1 is considered agreeable. If the assumption is that the deployment can include both scenarios on the same route, we need to capture the application guidance to ensure correct network signaling. With our 3-1-1 compromise proposal, no separate capabilities provide less implementation flexibility but reduce implementation complexity, then it becomes a good trade-off from UE perspective. |
| Ericsson | We are ok with option2. |
| Apple | Support option 2 |
| Intel | Support recommended WF Since mixed Scenario A/B deployment can be considered, it is important that UE can satisfy the requirements for both Set 1 and Set 2. |
| ZTE | Support Option 2. |
| Samsung | Option 2 and agree with recommended WF. |
| CMCC | OK with the recommended WF |
| CATT | Support Recommended WF |

#### Issue 1-3-2: UE feature list

- Background

The following UE features are proposed for the Rel-17 NR FR2 HST WI. More details can be found in the paper [R4-2201877, Views on FR2 HST PDSCH performance requirements, Intel Corporation, RAN4 #101-bis-e, January 2022.].

Component (2) “Support of enhanced RRM requirements for FR2 HST” is relevant to HST FR2 RRM.

- Recommended WF

  - Discuss whether “Support of enhanced RRM requirements for FR2 HST” UE feature is sufficient for Rel-17 NR FR2 HST WI.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Ok with the list. Just a question: shall we use power class to identify the feature support? Is it possible that power class 6 will used in another scenario in the future? |
| ZTE | OK with this UE feature, but we are not sure whether per-band type is necessary? Or per-UE type is enough? |
| Samsung | Generally okay with list, but it is better to be discussed in next meeting, after a more understanding of the newly introduced features after the discussion in this meeting. |

### Sub-topic 1-4: Other

Sub-topic description

Open issues and candidate options before e-meeting:

#### Issue 1-4-1: CR work split

- Background

At the previous RAN4#101 that companies have identified their interest providing CRs for the following sections (CR work Split):

| TS 38.133 section number | TS 38.133 section title | Responsible company |
| --- | --- | --- |
| 4 | SA: RRC_IDLE state mobility |  |
| 4.1 | Cell Selection |  |
| 4.2 | Cell Re-selection | ZTE |
| 5 | SA: RRC_INACTIVE state mobility |  |
| 5.1 | Cell Re-selection |  |
| 6 | RRC_CONNECTED state mobility |  |
| 6.1 | Handover | Ericsson |
| 6.2 | RRC Connection Mobility Control | Ericsson |
| 7 | Timing |  |
| 7.1 | UE transmit timing | Nokia |
| 7.2 | UE timer accuracy |  |
| 7.3 | Timing advance |  |
| 7.4 | Cell phase synchronization accuracy |  |
| 7.7 | deriveSSB-IndexFromCell tolerance |  |
| 8 | Signalling characteristics |  |
| 8.1 | Radio Link Monitoring | CATT |
| 8.5 | Link Recovery Procedures | CATT |
| 8.10 | Active TCI state switching delay | Samsung |
| 8.12 | Uplink spatial relation switch delay | Samsung |
| 9 | Measurement Procedure |  |
| 9.2 | NR intra-frequency measurements | Nokia |
| 9.5 | L1-RSRP measurements for Reporting | ZTE |
| 9.8 | L1-SINR measurements for Reporting | Huawei |
| 9.10 | CSI-RS based L3 measurements | Huawei |

- Recommended WF

  - The companies are invited to verify whether all of the clauses are with enhancements are listed

  - The companies are encouraged to volunteer for the BigCR editor for the TS 38.133.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia1 | We can be bigCR editor for 38.133. |
| YYY |  |
| ZZZ |  |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| R4-2201847, TP to TR 38.854 on Mobility Performance in HST FR2 Deployment Scenarios, Nokia, Nokia Shanghai Bell |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| YYY |  |
| ZZZ |  |

| R4-2201848, TP to TR 38.854 on the Number of Rx beams, Nokia, Nokia Shanghai Bell |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #1-1: Network signaling | Issue 1-1-1: Signaling of uni-/bi-directional operation Background: The companies still have opposite opinions about a need for network signaling. Some companies indicate that no requirement differentiation is provided in between uni- and bi-directional deployments and signaling is not addressing directly the open issues. On the other hand, other companies think that the flag can help to optimize UE beam management. Tentative agreements: None Candidate options: - Option 1 (QC, Ericsson, Apple): Network signals type of deployment (uni- or bi-direction) to UE - Option 2 (Nokia, Huawei, Samsung, CMCC, CATT): Signaling of uni-/bi-directional operation is not needed Recommendations for 2nd round: Continue the discussion in the 2nd round. Issue 1-1-2: Signaling to indicate set 1 or set 2 RRM requirement Background: Based on the proposed LSs and the comment by Apple in the 1st round it is moderator’s opinion that it is a common understanding that Set1/2 enhanced requirements signaling is cell-specific. Tentative agreements: Include cell-specific signaling of Set 1 or Set 2 enhanced RRM requirement into the LS to RAN2. Recommendations for 2nd round: Confirm the tentative agreement in the 2nd round. Issue 1-1-3: Applicability of enhanced RRM requirements in TP Background: Based on the company comments, Proposal 2 shall be considered on case-by-case basis. Regarding the Proposal 1, there is no common agreement about the recommended ranges of values for Dmin and Hdiff with Set-1 requirements. Looks like more analysis is needed. The proposing companies are also encouraged to bring TP to the TR at the next meeting. Tentative agreements: None Candidate options: - Option 1: 2Rx beam sweep based requirement (set 1) applies to the deployment scenario with Dmin > [10] m or Hdiff (height difference between train rooftop mounted CPE and RRH) > [10] m, performance degradation is expected. - Option 2: Other options are not precluded Recommendations for 2nd round: It is recommended to add directly the issue to the WF without further discussion in the 2nd round: FFS, whether it should be clarified in the TR that 2Rx beam sweep based requirement (set 1) applies to the deployment scenario with Dmin > [10] m or Hdiff (height difference between train rooftop mounted CPE and RRH) > [10] m, performance degradation is expected. Issue 1-1-4: Signaling of SSB configuration Background: The candidate options discussed in the first round: - Option 1: Enable network assisted signaling of SSB index and order per RRH - Option 2: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs. - Option 3: The network assistance of SSB configuration shall not be introduced in Rel-17 In general, the companies are doubting whether it is possible to introduce any complicated signaling, e.g., based on MAC-CE, in terms of HST FR2 Rel-17 WI, especially, due to a need clarify further the details of signaling. Next, the moderator’s observation is that the most supported is Option 3, but there is still some discussion of Option 1. However, Option 2 was not supported by the companies that much. Tentative agreements: None Candidate options: - Option 1 (Apple, Ericsson): Enable network assisted signaling of SSB index and order per RRH. - Option 2 (Intel, ZTE, Samsung): The network assistance signaling of SSB configuration shall not be introduced in Rel-17. Recommendations for 2nd round: Discuss further the candidate options in the 2nd round. Issue 1-1-5: Signaling of beam coverage related information Background: Proposal 1 (Qualcomm): The following additional beam coverage related information can be signaled to UE - Distance between the projections of adjacent beam peaks on the track - Beam peak direction angle relative to track - The 6 dB beam-width projection on track In addition, UE can report speed to the network. The proposal didn’t get sufficient support in between companies in terns of HST FR2 Rel-17. Tentative agreements: None Candidate options: None Recommendations for 2nd round: It is recommended to discontinue the discussion in Rel-17. Issue 1-1-6: LS to RAN2 on Network Signaling Background: There is still a number of open issues on NWA signaling that prevent the submission of combined LS to RAN2 that could cover all potentially needed signaling. Tentative agreements: None Candidate options: None Recommendations for 2nd round: A TDoc number will be requested for the LS in case of significant progress in the 2nd round. Otherwise, the LS should be postponed till the next meeting, and the LS responsible can be agreed in between the volunteered companies. |
| Sub-topic #1-2: Deployment-related issues | Issue 1-2-1: Train travelling opposite to the serving beam Background: In the first round the following options were discussed: - Option 1 (ZTE, CATT): Configure a different mobility parameter, e.g., HO offset for opposite direction - Option 2 (Nokia): For unidirectional Scenario A, set the DRX upper bound to [40 ms] The companies still have different opinions about the resolution of potential issues with mobility in the HST FR2 Scenario-A where the train travelling direction is opposite to the serving beam orientation. The companies have pointed out that the Options are not mutually exclusive, however, the adjustment of HO parameters is an implementation issue without standardization impact. Even though only one company has referred to this issue from Issue 2-2-2, the majority of the companies still see it possible to agree on the upper bound = 80 ms both to Sets 1 (Scenario-A) and 2. However, some companies still support Option 2. This issue directly impacts the following requirements: - Time period for PSS/SSS detection (Issue 2-2-3) - Measurement period for intra-frequency measurements without gaps (Issue 2-2-3) - L1-RSRP measurement requirement (Issue 2-2-4) - RLM/BFD evaluation period (Issue 3-1-1) Tentative agreements: None Candidate options: - Option 1: The DRX upper bound = 80 ms applies both to Sets 1 (Scenario-A) and 2 (Scenario-B). - Option 2 (Nokia, QC, Intel): For unidirectional Scenario A, set the DRX upper bound to [40 ms]. Recommendations for 2nd round: The discussion of the DRX upper bound should continue in this issue only, i.e., the Issue 2-2-2 will be discontinued. It is recommended to discuss the Issue at the GtW, and then apply achieved agreement to all other pending Issues (2-2-3, 2-3-4, and 3-1-1). Issue 1-2-2: Two-side RRM deployment in Scenario-B Background: The following candidate options were discussed in the first round: - Option 1 (Apple): No special consideration for two-side RRH deployment - Option 2a (Ericsson): Introduce signaling to indicate the queue of directions of RRHs in one cell to UE - Option 2b (ZTE, Ericsson): Indicated to UE to switch antenna panel/RRH side position information - Option 3 (Qualcomm, ZTE): Use extra [1.5] scaling for DRX [<=80] ms It looks like Options 2a and 2b that involve signaling have some further questions from the companies, and, thus, need more clarification. On the other hand. Option 1 and 3 have clear proponents. Many companies think that 6 Rx beams is already sufficient to cover the deployments with the RRH on both sides of the track. Tentative agreements: None Candidate options: - Option 1 (Apple, Samsung, CATT, Intel, Nokia): No special consideration for two-side RRH deployment - Option 2 (QC, ZTE): Use extra [1.5] scaling for DRX [<=80] ms for all SMTC periodicities Recommendations for 2nd round: The discussion should continue in the 2nd round and the issue will be recommended for the GtW session. The agreement here will directly impact the agreements in the Issues 2-2-3, 2-2-4. Issue 1-2-3: Change of RRH panel orientation in uni-directional deployments Background: In general, it seems that companies do not see a need in such additional signaling. However, the agreement is pending on the agreements in Issues 1-1-1 and 1-1-2. Tentative agreements: None Candidate options: - Option 1: Network signals the beam direction of new RRH - Option 2: No need in additional signaling Recommendations for 2nd round: It is recommended to postpone further discussion until other related issues are clarified. Include directly in the WF: FFS, whether additional signaling should be defined to identify to the UE a change of RRH panel orientation in uni-directional deployment, considering the outcomes of the discussions on signaling of uni-/bi-directional operation and on two-side RRM deployment. |
| Sub-topic 1-3: UE capabilities | Issue 1-3-1: Capability to support different RX beam sweeping number Background: All of the companies except one think that UE (power class 6 UE) shall mandatorily support both Set 1 and Set 2 enhanced RRM requirements. At the same time, it is not clear to the moderator why an applicability note/recommendation in the TR should have an impact in this agreement. It is not clear either why specifically BLM/BFD evaluation period requirements should be taken into account whereases there are in general two different sets of enhanced RRM requirements (Set 1 and Set 2) defined For HST FR2. The discussion in Issues 3-1-1 can continue. Tentative agreements: FR2 HST UE (power class 6 UE) shall mandatorily support both Set 1 and Set 2 enhanced RRM requirements, in terms of different RX beams (i.e., RX beam sweeping scaling factor) per UE. Candidate options: None Recommendations for 2nd round: It is recommended to agree on tentative agreement. Issue 1-3-2: UE feature list Background: Seems that companies need more time to check the definition of HST FR2 RRM feature group component. Tentative agreements: None Candidate options: None Recommendations for 2nd round: Add FFSs directly to the WF: Companies are encouraged to check further UE feature needed for the support of enhanced RRM requirements for FR2 HST: - FFS, whether power class shall be used to identify the feature support - FFS, whether per-band type is necessary or per-UE type is enough |
| Sub-topic #4-1: Other | Issue 2-1-1: CR work split Background: Nokia has volunteered to the Big CR editor for TS 38.133. Tentative agreements: None Candidate options: None Recommendations for 2nd round: Include the informative Table with the CR work split to the WF. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2201847 | Since no comments were received on the TP in the 1st round, it is recommended to agree on the TP. |
| R4-2201848 | Since no comments were received on the TP in the 1st round, it is recommended to agree on the TP. |

## Discussion on 2nd round (if applicable)

TBA

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| TDoc |  |
| --- | --- |
| Company | Comments |

### Sub-topic 1-2: TBA

#### Issue 1-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Mobility and Measurement procedure

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200100 | CATT | Further discussion on RRC Idle/Inactive and connected state mobility requirements for FR2 HST Proposal 1: Use the same DRX upper round for both Set1 and Set2. Proposal 2: The same M2 can be used in Scenario-B with two-side RRH. [Moderator]: treated in Topic#1, Issue 1-2-2 Proposal 3: For the potential mobility issue when the train is travelling direction is opposite to the serving beam orientation, it can be resolved by using proper event offset in NW if NW knows the UE moving direction. [Moderator]: treated in Topic#1, Issue 1-2-1 Proposal 4: Define two sets of requirements for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra . N1 refers to the agreed Rx numbers. |
| R4-2200261 | Apple | Discussion on mobility requirement for FR2 HST Proposal 1: For connected state mobility, requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers. Proposal 2: For connected state mobility, no need to further define different scaling factor for scenario-B with two-side RRH. [Moderator]: treated in Topic#1, Issue 1-2-1 Proposal 3: Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC). Proposal 4: Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC). Proposal 5: Handover requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers. Proposal 6: For idle/inactive mode cell reselection enhancement, Use 320 ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the set 1 and set 2 Rx beam numbers. |
| R4-2200583 | ZTE Corporation | Discussion on RRC Idle Inactive and Connected state mobility requirements for HST FR2 Proposal 1: Using 80 ms as the DRX upper bound for enhanced RRM HST FR2 requirements for both Scenario A and Scenario B. Proposal 2: For time to identify Known NR cell, we prefer Option 1b. For time to identify unknown NR cell, we prefer Option 1. Proposal 3: For each delay component, only T∆ is candidate which needs possible enhancement, but after further analysis, we suggest no requirement enhancement of handover is necessary. Proposal 4: Enhancement of requirement at DRX cycle length 320ms is needed, and should be consistent with the agreed RX number, so we prefer Option 1-b. Proposal 5: The value of N1 refers to agreed RX beam number. [Moderator]: This proposal seems to contradict the former agreement that requirements are enhanced for DRX 320 only. |
| R4-2200628 | CMCC | Discussion on mobility requirements for FR2 HST CONNECTED State Mobility Proposal 1: for the enhancement in connected mode, it is proposed to define the requirements following the 2 sets of number of RX beams. Idle/inactive mode requirements Proposal 2: for the enhancement in idle/inactive mode, it is proposed as following: DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; [2.56 x N1 x M2 (8 x N1 x M2)]; [0.32 x N1 x M3 (1 x N1 x M3)]; [0.96 x N1 x M4 (3 x M4)]. Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: N1 refers to the agreed Rx numbers.. |
| R4-2200880 | Ericsson | RRC Idle/Inactive and connected state mobility requirements for HST FR2 Proposal 1: DRX upper bound for enhanced RRM HST FR2 requirements is 80ms. Proposal 2: Issue’ M2 scaling factor for short DRX’ can refer to Issue’ Requirements for RRH deployment on both sides of the track’ in Clause: 6.9.4.2 Number of RX beams. [Moderator]: Treated in Topic#1, Issue 1-2-2. Proposal 3: Based on our understanding, the following are possible solutions to deal with the problem (we suppose a similar problem may occur in L1 mobility): - Shorter DRX may alleviate the problem, but we doubt it will completely resolve it. - Implement UE-initiated TCI activation/deactivation, but no additional signaling is allowed with regard to agreement in previous meetings. - Alter the conditions of L1 mobility and L3 mobility, for example, parameters in A3/A5 in L3 mobility adopts different values, before the SNR drops. The addition of signaling to UE, for example intra-RRH mobility or inter-RRH mobility, may be beneficial to the solution, but this needs to be investigated. We encourage open discussions on the issue. [Moderator]: Treated in Topic#1, Issue 1-2-1. Proposal 4: We support option1 in Issue’ Time to identify unknown NR cell’ . If option1 in issue’ Time to identify unknown NR cell’ is agreed, we have not strong view on Issue ‘Time to identify Known NR cell’. Proposal 5: No enhancement for HO is needed, if known cell is assumed. Proposal 6: The scaling factor N1 in Issue ‘IDEL/INACTIVE state mobility’ refers to agreed Rx numbers. |
| R4-2201176 | Huawei, Hisilicon | Discussion on RRC Idle/Inactive and connected state mobility requirements for HST in FR2 Proposal 1: In connected mode, Mpss/sss is at least 10 and 24 for scenario A and Scenario B respectively. Proposal 2: For 320ms DRX cycle, the scaling factor N1 need to be changed according to RX beam number and the sample number for Tdetect, Tmeasure and Tevaluate can refer to the existing enhancement requirements for R16 FR1 HST, i.e., 8,1,3 respectively. Proposal 3: No enhancements in HO requirement corresponding the number of RX beam sweep are needed. Proposal 4: In FR2 HST RRC re-establishment, Tidentify are only specified for unknown cell, - for intra-frequency Tidentify= MAX (1000 ms, 10*N* TSMTC) when SINR≥ -8dB; - for inter-frequency Tidentify= MAX (1000 ms, 13*N* TSMTC) when SINR≥ -8 dB. |
| R4-2201651 | Nokia, Nokia Shanghai Bell | On RRM measurement requirements for FR2 HST Observation 1: The DRX upper bound = 80 ms applies to Sets 1 and 2. Observation 2: No enhancements are made for DRX cycle length > 80 ms. Proposal 1: In connected mode, the 2 sets of enhanced intra-frequency measurement requirements are specified the tables below. PSS/SSS detection Set 1: Table 1: Time period for PSS/SSS detection when [flag1] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(6 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(600ms, ceil(6 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Set 2: Table 2: Time period for PSS/SSS detection when [flag2] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(18 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(600ms, ceil(18 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. SSB measurement period Set 1: Table 3: Measurement period for intra-frequency measurements without gaps when [flag1] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(6 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(400ms, ceil(6 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Set 2: Table 4: Measurement period for intra-frequency measurements without gaps when [flag2] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(18 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(400ms, ceil(18 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Observation 3: For Option 2, 6 UE Rx beams (N1B = 6) for Scenario B is not feasible for cell reselection in idle mode with DRX cycle = 320 ms. Proposal 2: For SA cell reselection in idle mode, the scaling factor N1 is 3 with DRX cycle = 320 ms for Scenarios A and B. |
| R4-2201652 | Nokia, Nokia Shanghai Bell | CR to TS 38.133: intra-frequency measurements without gaps for for FR2 NR HST |
| R4-2200877 | Ericsson | CR On RRC_CONNECTED state mobility for HST FR2 RRM |
| R4-2200103 | CATT | Further discussion on measurement procedure requirements for FR2 HST Proposal 1: The different scaling factor is not needed for Scenario-B with two-side RRH. For each scenario, define unified requirement. For PSS/SSS and intra-frequency measurement, Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps can be 6 (Scenario A) and 18 (Scenario B). [Moderator]: Partly treated in Topic#1, Issue 1-2-1 |
| R4-2200264 | Apple | Discussion on measurement procedure requirement for FR2 HST Proposal 1: - Mpss/sss_sync_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2. - Mmeas_preriod_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2. Proposal 2: No separate sets of scaling factor for scenario-B with two side RRH. [Moderator]: Treated in Topic#1, Issue 1-2-1. |
| R4-2200879 | Ericsson | Measurement procedure requirements for HST FR2 Proposal 1: Agree with separating sets of requirements for deployment Scenarios A and B. |
| R4-2201178 | Huawei, Hisilicon | Discussion on RRM requirements for high speed train scenario in FR2 Proposal 1: For 320ms DRX cycle, the scaling factor N1 need to be changed according to RX beam number and the sample number for Tdetect, Tmeasure and Tevaluate can refer to the existing enhancement requirements for R16 FR1 HST, i.e., 8,1,3 respectively. Proposal 2: In connected mode, Mpss/sss is at least 10 and 24 for scenario A and Scenario B respectively. Proposal 3: The intra-frequency cell detection delay for FR2 HST can be specified as below, where Xpss/sss_sync_w/o_gaps is Mpss/sss in proposal 2. DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤80ms; max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms<DRX cycle≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Proposal 4: The intra-frequency time index detection delay for FR2 HST can be specified as below, where Xpss/sss_sync_w/o_gaps is Mpss/sss in proposal 2. DRX cycle; TSSB_time_index_intra. No DRX; max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤80ms; max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms<DRX cycle≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. Proposal 5: The intra-frequency measurement period for FR2 HST can be specified as below, where Xpss/sss_sync_w/o_gaps is Mpss/sss in proposal 2. DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Xmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤80ms; max(400ms, ceil(Xmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms<DRX cycle≤ 320ms; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: RRC CONNECTED and IDLE state mobility requirements

Sub-topic description:

Open issues and candidate options before e-meeting:

#### Issue 2-1-1: Handover

- Background

Agreement from RAN4#101-e:

  - Do not enhance requirements for HO to unknown cell.

  - If the target cell is a known cell, then Tsearch = 0 ms.

Way forward:

  - FFS, whether enhancements in HO requirement corresponding the number of RX beam sweep are needed.

- Proposals and/or Observations

  - Proposal 1 (Apple): Handover requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers.

  - Proposal 2 (Ericsson): No enhancement for HO is needed, if known cell is assumed.

  - Proposal 3 (Huawei): No enhancements in HO requirement corresponding the number of RX beam sweep are needed.

  - Proposal 4 (ZTE): For each delay component, only T∆ is candidate which needs possible enhancement, but after further analysis, we suggest no requirement enhancement of handover is necessary.

- Candidate options:

  - Option 1 (Apple): Enhance requirements corresponding to the set 1 and set 2 UE Rx beam numbers

  - Option 2 (Ericsson, Huawei, ZTE): No enhancement is needed

- Recommended WF

  - Check whether Option two is agreeable:
No enhancement is needed in HO requirement.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support option 2 |
| Ericsson | Support Option2 |
| Apple | We can accept with option 2 based on the analysis. |
| Intel | Agree with recommended WF |
| Huawei | Agree with recommended WF |
| ZTE | Support Option2 |
| Samsung | Option 2, and agree with recommended WF |
| CATT | Support option 2. |

#### Issue 2-1-2: RRC Re-establishment requirement to known cell

- Background

Exiting requirement in TS 38.133:

Way forward from RAN4#101-e

Further study the requirement on the time to identify target cell for RRC connection re-establishment to NR intra-frequency cell in FR2 HST deployments:

  - Time to identify Known NR cell

    - Option 1: MAX (TBD, 5 x N1 x TSMTC), the value of N1 refers to agreed RX beam number

      - Option 1a: TBD is 400 ms

      - Option 1b: TBD is: 200 ms

    - Option 2: Do not introduce any enhancements

- Proposals and/or Observations

  - Proposal 1 (Apple): Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC).

  - Proposal 2 (ZTE): For time to identify Known NR cell, we prefer Option 1b. For time to identify unknown NR cell, we prefer Option 1.

  - Proposal 3 (Ericsson): We support option1 in Issue’ Time to identify unknown NR cell’ . If option1 in issue’ Time to identify unknown NR cell’ is agreed, we have not strong view on Issue ‘Time to identify Known NR cell’.

  - Proposal 4: In FR2 HST RRC re-establishment, Tidentify are only specified for unknown cell,

    - for intra-frequency Tidentify= MAX (1000 ms, 10*N* TSMTC) when SINR≥ -8dB;

    - for inter-frequency Tidentify= MAX (1000 ms, 13*N* TSMTC) when SINR≥ -8 dB.

  - Proposal 5 (Qualcomm): Following R15 requirement, do not consider known cell connection re-establishment in FR2. No enhancement is needed for connection re-establishment for unknown cell due to lack of use cases in FR2 HST.

- Candidate options:

  - Option 1 (Apple): Tidentify_intra_NR = MAX (400 ms, 5 x N1 x TSMTC)

  - Option 2 (ZTE): Tidentify_intra_NR = MAX (200 ms, 5 x N1 x TSMTC)

  - Option 3 (Huawei, Qualcomm): Specify Tidentify_intra_NR only for unknown cell

- Recommended WF

  - Discuss candidate options in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support option 3, do not see the necessity of adding known requirement for HST while there is no corresponding requirement for legacy FR2. |
| Ericsson | We have not strong view on known cell. in this sense, we support Option 3. |
| Apple | 400ms is calculated as 1000-600ms. We can compromise to enhance unknown case as well, since known FR2 was not defined in legacy. |
| Huawei | Support option 2. In legacy requirements, for FR2, there are no cases for known NR cell. The reason is that UE is not required to store the information of ever measured cells at re-establishment procedure as UE needs to perform beam sweeping in FR2. The same logic for FR2 HST. |
| ZTE | We can compromise to Option 3. |
| Samsung | Option 3. |
| CATT | Support option 3. Follow legacy FR2. |
| QC | Does Huawei mean option 3 in their comment? |

#### Issue 2-1-3: RRC Re-establishment requirement to unknown cell

- Background

Way forward from RAN4#101-e

Further study the requirement on the time to identify target cell for RRC connection re-establishment to NR intra-frequency cell in FR2 HST deployments:

  - Time to identify unknown NR cell

    - Option 1: MAX (1000 ms, 10 x N1 x TSMTC)), the value of N1 refers to agreed RX beam number

    - Option 2: Do not introduce any enhancements

Note, that it was agreed at RAN4#98-bis-e: “Do not define inter-frequency measurements requirements for FR2 HST”.

- Proposals and/or Observations

  - Proposal 1 (Apple): Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC).

  - Proposal 2 (ZTE): For time to identify Known NR cell, we prefer Option 1b. For time to identify unknown NR cell, we prefer Option 1.

  - Proposal 3 (Ericsson): We support option1 in Issue’ Time to identify unknown NR cell’ . If option1 in issue’ Time to identify unknown NR cell’ is agreed, we have not strong view on Issue ‘Time to identify Known NR cell’.

  - Proposal 4 (Huawei): In FR2 HST RRC re-establishment, Tidentify are only specified for unknown cell,

    - for intra-frequency Tidentify= MAX (1000 ms, 10*N* TSMTC) when SINR≥ -8dB;

    - for inter-frequency Tidentify= MAX (1000 ms, 13*N* TSMTC) when SINR≥ -8 dB.

  - Proposal 5 (Qualcomm): Following R15 requirement, do not consider known cell connection re-establishment in FR2. No enhancement is needed for connection re-establishment for unknown cell due to lack of use cases in FR2 HST.

- Candidate options:

  - Option 1 (Apple, ZTE, Ericsson, Huawei): MAX (1000 ms, 10 x N1 x TSMTC)), the value of N1 refers to agreed RX beam number

  - Option 2 (Qualcomm): Do not introduce any enhancements

- Recommended WF

  - Companies are invited to check whether Option 1 is agreeable:
Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell Tidentify_intra_NR = MAX (1000 ms, 10 x N1 x TSMTC) when SINR≥ -8dB.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | As we explained, we don’t see the use case for RRC connection re-establishment. However, if companies insist to have it, we should define requirement for unknown instead of known case to follow R15 principle. Therefore, we can compromise to option 1 if known cell is aligned to legacy (no requirement). |
| Ericsson | Support option1. |
| Apple | Support option 1 |
| Intel | Agree with recommended WF |
| Huawei | Agree with the recommended WF. |
| ZTE | Support Option 1. |
| Samsung | Agree with the recommended WF. |
| CATT | Support Recommended WF |

#### Issue 2-1-4: IDEL/INACTIVE state Cell Re-selection

- Background

At the last RAN4#101-e meeting it was agreed that

  - Defined enhanced requirements for DRX 320 ms only

  - Requirements for longer DRX cycles are left without changes

The only Option for the enhancement of Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE in HST FR2 deployments was the table. This format

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | [2.56 x N1 x M2 (8 x N1 x M2)] | [0.32 x N1 x M3 (1 x N1 x M3) ] | [0.96 x N1 x M4 (3 x M4) ] |  |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16]. |  |  |  |  |

The choice of N1 parameter left open.

For reference, enhanced requirements are defined for HST FR1 in TS 38.133:

- Proposals and/or Observations

  - Proposal 1 (CATT): Define two sets of requirements for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra . N1 refers to the agreed Rx numbers.

  - Proposal 2 (Apple): For idle/inactive mode cell reselection enhancement, Use 320 ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the set 1 and set 2 Rx beam numbers.

  - Proposal 3 (ZTE): Enhancement of requirement at DRX cycle length 320ms is needed, and should be consistent with the agreed RX number, so we prefer Option 1-b.

  - Proposal 4 (CMCC): Note 2: N1 refers to the agreed Rx numbers.

  - Proposal 5 (Ericsson): The scaling factor N1 in Issue ‘IDEL/INACTIVE state mobility’ refers to agreed Rx numbers.

  - Proposal 6 (Huawei): For 320ms DRX cycle, the scaling factor N1 need to be changed according to RX beam number and the sample number for Tdetect, Tmeasure and Tevaluate can refer to the existing enhancement requirements for R16 FR1 HST, i.e., 8,1,3 respectively.

  - Observation 3 (Nokia): For Option 2, 6 UE Rx beams (N1B = 6) for Scenario B is not feasible for cell reselection in idle mode with DRX cycle = 320 ms.

  - Proposal 7 (Nokia): For SA cell reselection in idle mode, the scaling factor N1 is 3 with DRX cycle = 320 ms for Scenarios A and B.

  - Proposal 8 (Qualcomm): Select option 1 for neighboring cell measurement in idle mode.

- Candidate options:

  - Option 1 (CATT, Apple, ZTE, CMCC, Ericsson, Huawei, Qualcomm): Two sets of requirements with N1 equal to the number of Rx beams in Set 1 (=2) and Set 2 (=6).

  - Option 2 (Nokia, Qualcomm): N1 = 3.

- Recommended WF

  - Check whether DRX cycle = 320 ms is feasible for cell reselection with scaling factor N1=6

  - Companies are invited to verify whether Option 1 is agreeable: 
Defined enhanced requirements for Cell reselection in IDLE/INACTIVE mode for DRX 320 ms in HST FR2 deployments

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x N1 x M2 (8 x N1 x M2) | 0.32 x N1 x M3 (1 x N1 x M3) | 0.96 x N1 x M4 (3 x M4) |  |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16]. |  |  |  |  |

N1 refers to the number of Rx beams and equals 2 for Set 1, and 6 for Set 2.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Option 1 will result in suboptimal performance as shown in our analysis because the distance travelled by UE exceeds the cell ISD for N1B = 6 and DRX cycle = 320 ms. Thus, Option 2 is preferred, but can compromise on Option 1. |
| QC | We can support recommended WF |
| Ericsson | Support Option1 |
| Apple | Support option 1 |
| Intel | Ok with recommended WF |
| Huawei | Support Option1 |
| ZTE | Support Option 1. |
| Samsung | We support Option 1 i.e., recommended WF |
| CMCC | Option 1 |
| CATT | Support option 1. |

### Sub-topic 2-2: Measurement procedure requirements

Sub-topic description

Open issues and candidate options before e-meeting:

#### Issue 2-2-1: Need for enhancements

- Background

At RAN4#101-e, it was already agreed that tow sets of requirement shall be defined:

GtW Agreements: - Define only two sets of enhanced RRM requirements in terms of number of RX beams (i.e. RX beam sweeping scaling factor) per UE - Set 1: 2 RX beams - Set 2: 6 RX beams - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements - Note: the applicability of Set 1/2 requirements to the FR2 HST scenarios will be captured in the TR

- Proposals and/or Observations

  - Proposal 1 (Apple): For connected state mobility, requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers.

  - Proposal 2 (CMCC): for the enhancement in connected mode, it is proposed to define the requirements following the 2 sets of number of RX beams.

- Recommended WF

  - It is recommended not to discuss further the proposals above due to exiting agreement and to focus on concrete enhancements in requirements, Issue 2-2-3.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support recommended WF |
| Intel | Agree with recommended WF |
| Huawei | Proposal 1 and 2 seems the same. 2 sets of RX beam number shall be used for measurement requirements. |
| ZTE | Agree with recommended WF. |
| Samsung | Agree with recommended WF to focus on requirement directly. |
| CMCC | OK with the recommended WF |
| CMCC | OK with the recommended WF |

#### Issue 2-2-2: DRX upper bound

- Background

Way forward from RAN4#101-e:

Agreement: Baseline: DRX upper bound for enhanced RRM HST FR2 requirements is [80]ms. Way forward: FFS, whether different requirements shall be defined for requirement Set1 and Set 2.

Reduction of DRX upper bound to 40 is discussed in Topic#1 Issue 1-2-1.

The issue covers all requirements

  - Time period for PSS/SSS detection

  - Measurement period for intra-frequency measurements without gaps

  - L1-RSRP measurement requirement

- Proposals and/or Observations

  - Proposal 1 (CATT): Use the same DRX upper round for both Set1 and Set2.

  - Proposal 2 (ZTE): Using 80 ms as the DRX upper bound for enhanced RRM HST FR2 requirements for both Scenario A and Scenario B.

  - Proposal 3 (Ericsson): DRX upper bound for enhanced RRM HST FR2 requirements is 80ms.

  - Observation 1 (Nokia): The DRX upper bound = 80 ms applies to Sets 1 and 2.

  - Observation 2 (Nokia): No enhancements are made for DRX cycle length > 80 ms.

- Recommended WF

  - Agree, firstly, on the solution in Issue 1-2-1 (train travelling in the opposite direction)

  - Agree on tentative agreement:
The DRX upper bound = 80 ms applies to Sets 1 and 2

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | The recommended WF is Ok. |
| QC | Support recommended WF |
| Ericsson | Agree with recommended WF |
| Apple | Agree with the recommended WF |
| Intel | Support reduction of DRX upper bound to 40ms proposed in Issue 1-2-1 |
| Huawei | Support recommended WF. |
| ZTE | Agree with recommended WF. |
| Samsung | Support recommended WF. |
| CATT | Support Recommended WF |

#### Issue 2-2-3: Time period for PSS/SSS detection and Measurement period for intra-frequency measurements

- Background

For reference, the exiting requirements from TS 38.133 are listed below

The Issue is also pending on the agreements in Issue 1-2-2 on two-sided RRM deployments.

- Proposals and/or Observations

  - Proposal 1 (Nokia): In connected mode, the 2 sets of enhanced intra-frequency measurement requirements are specified the tables below.
[Moderator]: See Option 1a below.

  - Proposal 2 (CATT): For PSS/SSS and intra-frequency measurement, Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps can be 6 (Scenario A) and 18 (Scenario B).

  - Proposal 3 (Apple):

    - Mpss/sss_sync_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2.

    - Mmeas_preriod_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2.

  - Proposal 4 (Huawei): In connected mode, Mpss/sss is at least 10 and 24 for scenario A and Scenario B respectively.
[Moderator]: See tables in Option 2 below.

- Candidate options:

  - Option 1 (CATT, Apple):

    - Mpss/sss_sync_w/o_gaps = Mmeas_preriod_w/o_gaps = 6 for Set 1; DRX cycle≤ [80ms] and No DRX

    - Mpss/sss_sync_w/o_gaps = Mmeas_preriod_w/o_gaps = 18 for Set 2; DRX cycle≤ [80ms] and No DRX

  - Option 1a (Nokia):

PSS/SSS detection

Set 1:

Table 1: Time period for PSS/SSS detection when [flag1] is configured, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(6 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ [80ms] | max(600ms, ceil(6 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| [80ms]< DRX cycle≤ 320ms | max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Set 2:

Table 2: Time period for PSS/SSS detection when [flag2] is configured, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(18 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ [80ms] | max(600ms, ceil(18 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| [80ms]< DRX cycle≤ 320ms | max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

SSB measurement period

Set 1:

Table 3: Measurement period for intra-frequency measurements without gaps when [flag1] is configured (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(6 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ [80ms] | max(400ms, ceil(6 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| [80ms]< DRX cycle≤ 320ms | max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Set 2:

Table 4: Measurement period for intra-frequency measurements without gaps when [flag2] is configured (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(18 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ [80ms] | max(400ms, ceil(18 x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| [80ms]< DRX cycle≤ 320ms | max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

  - Option 2 (Huawei):

The intra-frequency cell detection delay for FR2 HST can be specified as below, where Xpss/sss_sync_w/o_gaps is Mpss/sss in proposal 2. [Moderator]: See proposal 4 above.

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤80ms | max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms<DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The intra-frequency time index detection delay for FR2 HST can be specified as below, where Xpss/sss_sync_w/o_gaps is Mpss/sss in proposal 2.

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤80ms | max(600ms, ceil(Xpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms<DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |

The intra-frequency measurement period for FR2 HST can be specified as below, where Xpss/sss_sync_w/o_gaps is Mpss/sss in proposal 2.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Xmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤80ms | max(400ms, ceil(Xmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms<DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

- Recommended WF

  - In moderator’s opinion, Time period for time index detection is not defined for FR2. Companies are invited to verify whether the requirement on time index detection delay is needed.

  - Take into account the outcomes of the discussion in Topic#1 Issue 1-2-2 on two-sided RRM requirements

  - Discuss candidate options in the 1st round

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Share the same view as moderator that the time period for time index detection does not apply to FR2. For Option 2, it seems the scaling factor M2 is missing and not according to the agreement in the WF (R4-2120416) as outlined below: For PSS/SS and intra-freq measurement - Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST with the same SMTC periodicity bound of 40ms unless technical issues identified Support Options 1a and 1. |
| QC | Option 1 and 2 seems to be the same except M2 factor. As we commented in 1-2-2, a scaling factor of 1.5 regardless of SMTC period is needed to account for possibilities of RRH on two sides, which is likely unavoidable in practice. |
| Ericsson | Ok with option 1a and 1. Time index detection does not apply to FR2 |
| Apple | Support option 1. Option 1 and 1a are the same. M2 factor was agreed separately. |
| Huawei | M2 can be scaled and we agree with Nokia time-index delay for intra-frequency in FR2 is not needed. Our point is the Mpss/sss value. In FR2 HST, as the Rx beam number for scenario A and scenario B are respectively 2 and 6, considering about 5 samples the scaling factor M for FR2 HST would be at least 10 and 24. |
| Samsung | Support Option 1a and 1, and time index detection is not needed because it is assumed that without the time index sync, FR2 system can’t work so the flag of deriveSSB-IndexFromCell should be always set. For FR2 HST, the same principle can be followed. |
| CATT | Support option 1. In addition, option 1a is the same as option 1. |

#### Issue 2-2-4: L1-RSRP measurements for Reporting

- Background

For reference, exiting requirements from TS 38.133:

N=8

Way forward from RAN4#101-e:

  - Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms unless technical issues identified

  - Separate sets of requirements for deployment Scenarios A and B

    - For scenario A

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NA)*TSSB) |
| DRX cycle ≤ [80ms] | max(TReport, ceil([1.5]*M*P*NA)*max(TDRX,TSSB)) |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NA = 2

    - For scenario B

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NB)*TSSB) |
| DRX cycle < [80ms] | max(TReport, ceil([1.5]*M*P*NB)*max(TDRX,TSSB)) |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NB = 6

- Proposals and/or Observations

  - Proposal 1 (Qualcomm): Keep the following agreement from RAN4#101e WF and remove the square bracket.

  - Proposal 2 (Ericsson): Agree with separating sets of requirements for deployment Scenarios A and B.

  - Proposal 3 (Huawei): Factor 1.5 can be removed for DRX cycle≤ 80ms in L1-RSRP measurement period for FR2 HST.

- Candidate options:

  - Option 1: Keep 1.5 scaling factor

  - Option 2: Remove 1.5 scaling factor

- Recommended WF

  - Companies are encouraged to check the upper bound for DRX cycle in Scenario-A and Scenario-B.

  - Companies are invited to discuss candidate options taking the outcomes of Issues 1-2-2 and 2-2-2 into account

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| XXXNokia | For the L1-RSRP measurement, the agreements in the WF (R4-2120416) are For L1-RSRP measurement - Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms unless technical issues identified - Separate sets of requirements for deployment Scenarios A and B It seems there is an editorial mistake in the table in which the 1.5 scaling factor should be replaced with M2 and the following sentence should added to the table: - M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |
| QC | As we explained in our contribution, the factor of 1.5 regardless of SMTC period is needed due to possible two side RRH deployment. |
| Apple | Same understanding as Nokia. |
| Intel | Agree with Nokia’s comment and prefer to align DRX limit with Issue 1-2-1 and Issue 2-2-2. |
| Huawei | No strong opinion. |
| Samsung | Option 1, keeping 1.5 scaling factor by following last meeting’s WF by just removing square bracket is okay to us. |
| CATT | Agree with Nokia’s modification. |
| QC | We consider L1-RSRP as more performance impact to the system than neighboring cell search because if wider beam is used in L1-RSRP stage to cover both sides, longer beam refinement time is needed and throughput degradation is observed. In contrast, wider beam can degrade SSB detection and measurement, but as long as it is still detectable, the impact to system performance is acceptable. Therefore, if the 1.5 scaling factor can be kept for L1-RSRP, we can compromise to remove it from neighboring cell measurement. |

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| R4-2201652, CR to TS 38.133: intra-frequency measurements without gaps for for FR2 NR HST, Nokia, Nokia Shanghai Bell |  |
| --- | --- |
| Company | Comments |
| QC | General comment for all the CRs: since many issues are pending, we suggest to focus on finalizing the requirement, and handle the CRs in the next meeting. |
| Samsung | One question is about do we need to specify the requirement is only applicable to PC6 UE with [flag1] or [flag2] is configured, in other words, other UE should not be applied by the enh. requirement even the per-cell flag is configured. |
| Nokia | In response to Samsung’s question concerning PC6 UE, this applicability issue has not been discussed in RAN4. So, it should be discussed first. |

| R4-2200877, CR On RRC_CONNECTED state mobility for HST FR2 RRM, Ericsson |  |
| --- | --- |
| Company | Comments |
| XXXNokia | There is no applicability defined for the number of RX sweeping beams for FR2 non-HST. The same approach should be used for FR2 HST. |
| Ericsson | We can adopt Nokia’s suggestion. |
| Samsung | Similar view as Nokia that set 1 and set 2 by following flag is not better to be handled in applicability section. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #2-X: RRC CONNECTED and IDLE state mobility requirements | Issue 2-1-1: Handover Background: The companies have agreed on the tentative agreement from the first round. Agreements: No enhancement is needed in HO requirement Candidate options: None Recommendations for 2nd round: Add agreement to the WF. Issue 2-1-2: RRC Re-establishment requirement to known cell Background: The companies share the opinion that since legacy FRE requirement are not defined for RRC Re-establishment requirement to known cell, they shall not be defined for HST FR2 either. Tentative agreements: Do not define RRC Re-establishment requirement to known cell for HST FR2. Candidate options: None Recommendations for 2nd round: Agree on tentative agreement. Issue 2-1-3: RRC Re-establishment requirement to unknown cell Background: The companies have agreed to enhance the requirement following the agreed Set-1 and Set 2 parameters. Tentative agreements: Define RRC connection re-establishment requirement to unknown NR intra-frequency cell Tidentify_intra_NR = MAX (1000 ms, 10 x N1 x TSMTC) when SINR≥ -8dB, the value of N1 refers to agreed RX beam number. Candidate options: None Recommendations for 2nd round: Agree on tentative agreement. Issue 2-1-4: IDEL/INACTIVE state Cell Re-selection Background: All companies have compromised to the Option 1: Two sets of requirements with N1 equal to the number of Rx beams in Set 1 (=2) and Set 2 (=6). It is moderator’s opinion that Note 2 in the tentative agreement still refers to HST FR1 requirement. Thus, it needs to be updated. Tentative agreements: Defined enhanced requirements for Cell reselection in IDLE/INACTIVE mode for DRX 320 ms in HST FR2 deployments DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: [When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraRAT-MeasurementEnhancement-r16].. N1 refers to the number of Rx beams and equals 2 for Set 1, and 6 for Set 2. Candidate options: None Recommendations for 2nd round: Agree on a tentative agreement. Add directly to the WF that The companies are encouraged to check the Notes and to reformulate Note 2. |
| Sub-topic #2-2: Measurement procedure requirements | Issue 2-2-1: Need for enhancements Background: It was confirmed that the need for enhancements already follows from the former agreements. Tentative agreements: None Candidate options: None Recommendations for 2nd round: None Issue 2-2-2: DRX upper bound Background: In general, the companies mostly agree that The DRX upper bound = 80 ms applies to Sets 1 and 2. However, the agreement is pending on the agreement on Issue 1-2-1. Tentative agreements: None Candidate options: - Option 1: The DRX upper bound = 80 ms applies to Sets 1 and 2 Recommendations for 2nd round: The discussion is merged with Issue 1-2-1. Issue 2-2-3: Time period for PSS/SSS detection and Measurement period for intra-frequency measurements Background: The companies seem to converge to Option 1a as a baseline. There is one company that highlights that that the value of M2 is pending on the agreement in Issue 1-2-2. Another company proposes to use larger scaling factors considering that 5 measurement samples are needed. Tentative agreements: Use the following requirements as a baseline: PSS/SSS detection Set 1: Table 1: Time period for PSS/SSS detection when [flag1] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil([6] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(600ms, ceil([6] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: [M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1]. Set 2: Table 2: Time period for PSS/SSS detection when [flag2] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil([18] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(600ms, ceil([18] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: [M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1]. SSB measurement period Set 1: Table 3: Measurement period for intra-frequency measurements without gaps when [flag1] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil([6] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(400ms, ceil([6] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: [M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1]. Set 2: Table 4: Measurement period for intra-frequency measurements without gaps when [flag2] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil([18] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(400ms, ceil([18] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. [80ms]< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: [M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 ]. Candidate options: - Option 1 (Nokia, Ericsson, Apple, Samsung, CATT): Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2 - Option 2 (Huawei): Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 10 for Set 1 and 24 for Set 2 Recommendations for 2nd round: - Use tentative agreement as a baseline. - Take into account the outcomes of Issue 1-2-2 for M2 and Issue 1-2-1 for the Upper DRX bound. - Discuss further the candidate options for scaling factors in the 2nd round. Issue 2-2-4: L1-RSRP measurements for Reporting Background: The companies identified some inaccuracies in the previous agreements. Otherwise the Tentative agreements: Use the following requirements as a baseline: - For Set 1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NA)*TSSB). DRX cycle ≤ [80ms]; max(TReport, ceil([1.5]*M*P*NA*M2)*max(TDRX,TSSB)). Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: [M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1]. NA = 2 - For Set 2 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NB)*TSSB). DRX cycle ≤ [80ms]; max(TReport, ceil([1.5]*M*P*NB*M2)*max(TDRX,TSSB)). Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: [M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1]. NB = 6 Candidate options: None Recommendations for 2nd round: - Use tentative agreement as a baseline. - Take into account the outcomes of Issue 1-2-2 for M2 and Issue 1-2-1 for the Upper DRX bound. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2201652 | CR to TS 38.133: intra-frequency measurements without gaps for for FR2 NR HST, Nokia, Nokia Shanghai Bell It is recommended to discuss further in a separate issue whether Enhanced RRM requirement is only applicable to PC6 UE with [flag1] or [flag2] is configured, in other words, other UE should not be applied by the enh. requirement even the per-cell flag is configured. Due to a need to agree on several Issues related to the CR it is doubtable the CR can be agreed yet. It is recommended not to pursue it any more. |
| R4-2200877 | CR On RRC_CONNECTED state mobility for HST FR2 RRM, Ericsson It seems that the CR can be rather easily reviewed considering received comments and agreements in Issues 2-1-2, 2-1-3. Hence, it is recommended to revise the CR. |

## Discussion on 2nd round (if applicable)

TBA

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| TDoc |  |
| --- | --- |
| Company | Comments |

### Sub-topic 2-2: TBA

#### Issue 2-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Signaling characteristics

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200102 | CATT | Discussion on RLM/BFD requirement for FR2 HST Proposal 1: For RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios, use agreed RX beam factor for set-1 and set-2. |
| R4-2200104 | CATT | Draft CR on RLM/BFD requirement for FR2 HST |
| R4-2200263 | Apple | Discussion on signaling characteristic requirement for FR2 HST Proposal 1: Enhanced RLM/BFD requirement with scaling Rx beam factor for set 1 and set 2. |
| R4-2200585 | ZTE Corporation | Discussion on Signaling characteristics for HST FR2 Proposal 1: The memory capacity of UE is not bottleneck for the case of high speed moving. Therefore, no need to reduce the TCI state switching requirement so as to relax the memory capacity of UE. Proposal 2: For RLM/BFD requirements for DRX <=80ms, following RX beam factor for Set-1 and Set-2. |
| R4-2200884 | Ericsson | Signalling characteristics requirements for HST FR2 Proposal 1: With regard to RLM/BFD requirements, support option 1: following RX beam factor for set-1 and set-2. Proposal 2: Regarding PSS/SS and intra-freq measurement, it can refer to discussion and agreement on Issue ‘Requirements for RRH deployment on both sides of the track’ in WF on FR2 HST RRM (part 1) [Moderator]: Treated in the previous Topic. |
| R4-2201177 | Huawei, Hisilicon | Discussion on signaling characteristics requirements for high speed train scenario in FR2 Proposal1: The framework of existing RLM/BFD requirements for FR2 can be reused except the scaling factor N (Rx beam sweeping) which follows set1 and set 2. Proposal 2: Factor 1.5 can be removed for DRX cycle≤ 80ms in L1-RSRP measurement period for FR2 HST. Proposal 3: The known condition of TCI state can be reduced to 480ms for FR2 HST. Proposal 4: The existing TCI switching delay in known case can be reused in FR2 HST with updated known TCI state condition. Proposal 5: Prefer to only consider known TCI switching in FR2 HST. |
| R4-2201770 | Samsung | Remaining Issues on signaling characteristics requirements for FR2 HST <RLM/BFD evaluation period> Proposal 1: For SSB-based RLM and BFD evaluation period, the scaling factor N should be defined: - by following the number of RX beams per UE for Set 1 and Set 2, for no DRX or DRX cycle <=80ms: - Set 1: 2 RX beams; (2) Set 2: 6 RX beams. - The applicable of set 1 and set 2 is dependent on the introduced network signaling Proposal 2: The example text proposal for FR2 HST UESSB-based RLM evaluation period is provided as: <Active TCI State Switching Delay> Observation 1: If the target TCI state is known, and the target TCI state is in the active TCI state list for PDSCH, there is no interruption allowed during MAC-CE based TCI state switching. Observation 2: During TCI switching between RRHs in FR2 HST scenario, it is possible to have inter-symbol interference which cannot be accommodated by the CP length of the OFDM symbol from the target RRH. Proposal 3: One more slot is allowed for interruption during TCI switching for FR2 HST scenario. <Uplink Spatial Relation Switch Delay> Proposal 4: For the known conditions for spatial relation when associated with DL-RS, the requirement defined in Rel-16 shall be reused, and no standard impact is expected for Rel-17 FR2 HST. |
| R4-2201771 | Samsung | Draft CR to introduce active TCI state switching delay requirement for FR2 HST UE |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

#### Issue 3-1-1: RLM/BFD evaluation period

- Background

For reference, FR2 requirements from TS 38.133:

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with scaling factor N=8.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-2 for FR2 with scaling factor N=8

Way forward from RAN4#101-e:

  - RAN4 will further study the RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios

    - Option 1: following RX beam factor for set-1 and set-2

    - Option 2: The existing RLM/BFD requirements for DRX <=80ms is applied

- Proposals and/or Observations

  - Proposal 1 (CATT): For RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios, use agreed RX beam factor for set-1 and set-2.

  - Proposal 2 (Apple): Enhanced RLM/BFD requirement with scaling Rx beam factor for set 1 and set 2.

  - Proposal 3 (ZTE): For RLM/BFD requirements for DRX <=80ms, following RX beam factor for Set-1 and Set-2.

  - Proposal 4 (Ericsson): With regard to RLM/BFD requirements, support option 1: following RX beam factor for set-1 and set-2.

  - Proposal5 (Huawei): The framework of existing RLM/BFD requirements for FR2 can be reused except the scaling factor N (Rx beam sweeping) which follows set1 and set 2.

  - Proposal 6 (Samsung): For SSB-based RLM and BFD evaluation period, the scaling factor N should be defined:

    - by following the number of RX beams per UE for Set 1 and Set 2, for no DRX or DRX cycle <=80ms:

      - Set 1: 2 RX beams; (2) Set 2: 6 RX beams.

    - The applicable of set 1 and set 2 is dependent on the introduced network signaling

  - Proposal 7 (QC): Follow FR1 HST, do not introduce RLM/BFD enhancement due to lack of use cases in FR2 HST.

- Candidate options:

  - Option 1 (CATT, Apple, ZTE, Ericsson, Huawei, Samsung): Define scaling factor N for no DRX and DRX cycle <=80ms by following the number of RX beams per UE for Set 1 and Set 2: Set 1: 2 RX beams; (2) Set 2: 6 RX beams.

  - Option 2 (QC): Do not introduce RLM/BFD enhancement

- Recommended WF

  - Check if Option 1 is agreeable in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | The recommended WF is Ok. |
| QC | We understand that option 1 is the majority view. As we explained in our contribution, we don’t see many use case for RLM/BFD procedure in HST. In addition, as we commented previously, we can compromise to no UE capability distinguishment for set 1 and 2. Therefore, for this relatively corner case procedure, we want to simplify UE implementation and set requirement according to set 2 uniformly. Note that RLM/BFD measurements are on serving beams, while all the other measurements are on target beams, therefore the two sets implementation to satisfy other requirement can not be directly applied to RLM/BFD measurement, and have only one set requirement helps reducing UE complexity. Hence here is the compromised proposal: Option 3: To simplify UE implementation, set RLM/BFD requirement according to set 2: 6Rx. |
| Ericsson | The recommended WF is Ok. |
| Apple | Support WF |
| Intel | Ok with recommended WF, but prefer to align DRX limit with Issue 1-2-1 and Issue 2-2-2. |
| Huawei | The recommended WF is Ok |
| ZTE | The recommended WF is OK. |
| Samsung | The recommended WF is Ok |
| CATT | Support Recommended WF |
| QC | Since no one comments on our compromise proposal, could we further discuss it in second round or GTW? |

#### Issue 3-1-2: TCI switching delay

- Background

Agreement from RAN4#101-e:

  - The existing 1280ms duration for known condition is applied for FR2 HST scenario

Current requirements are defined in Clause 8.10 of TS 38.133.

- Proposals and/or Observations

  - Proposal 1 (ZTE): The memory capacity of UE is not bottleneck for the case of high speed moving. Therefore, no need to reduce the TCI state switching requirement so as to relax the memory capacity of UE.

  - Proposal 2 (Huawei): The known condition of TCI state can be reduced to 480ms for FR2 HST.

  - Proposal 3 (Huawei): The existing TCI switching delay in known case can be reused in FR2 HST with updated known TCI state condition.

  - Proposal 4 (Huawei): Prefer to only consider known TCI switching in FR2 HST.

  - Observation 1 (Samsung): If the target TCI state is known, and the target TCI state is in the active TCI state list for PDSCH, there is no interruption allowed during MAC-CE based TCI state switching.

  - Observation 2 (Samsung): During TCI switching between RRHs in FR2 HST scenario, it is possible to have inter-symbol interference which cannot be accommodated by the CP length of the OFDM symbol from the target RRH.

  - Proposal 3 (Samsung): One more slot is allowed for interruption during TCI switching for FR2 HST scenario.

- Recommended WF

  - It is recommended not to update known TCI state condition due to existing agreement from RAN4#101-e.

  - Companies are invited to share their views on Proposal 3 in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Follow the agreement on known TCI state condition from the last meeting. Proposal 3 is FFS. |
| QC | Support recommended WF |
| Ericsson | known TCI state condition was agreed in last meeting. FFS on Proposal 3, it shall be studied more. |
| Intel | Ok with recommended WF For Proposal 3: we recognize the issue of inter-symbol interference during TCI state switching due to different propagation delays. The propagation delay difference is larger than CP length, but it is smaller than symbol length. Thus, we are wondering whether we need to waste the whole slot while only one symbol is affected. |
| Huawei | Support recommended WF. |
| ZTE | Fine with recommended WF. For Proposal 3, we agree some update is necessary for known condition. Maybe 240 ms, 480 ms can be candidates. |
| Samsung | For known TCI state condition, last meeting we have agreement. For our proposed P3, “One more slot is allowed for interruption during TCI switching for FR2 HST”, the problem is described in our paper, without additionally allow interruption here, how the proposed problem can be solved? |
| CATT | Support Recommended WF. |

#### Issue 3-1-3: Uplink Spatial Relation Switch Delay

- Proposals and/or Observations

  - Proposal 1 (Samsung): For the known conditions for spatial relation when associated with DL-RS, the requirement defined in Rel-16 shall be reused, and no standard impact is expected for Rel-17 FR2 HST

- Recommended WF

  - Companies are invited to check in the first round whether Proposal 1 is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support recommended WF |
| Ericsson | Ok with Proposal 1. |
| Huawei | Support recommended WF |
| ZTE | Fine with Proposal 1. |
| Samsung | As proponent of P1, we support this P1. |
| CATT | Fine with P1. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| R4-2201770, Remaining Issues on signaling characteristics requirements for FR2 HST, Samsung - TP on text proposal for FR2 HST UE SSB-based RLM evaluation period - TP for MAC-CE based TCI state switching delay |  |
| --- | --- |
| Company | Comments |
| Nokia | Some of the text proposals can be found in R4-2201771. |
| Samsung | In R4-2201770, just proposed some text proposal for SSB-based RLM, which is not CR, so we can use others’ draft CR to discuss further. No need further discuss this as draftCR |
| ZZZ |  |

| R4-2200104, Draft CR on RLM/BFD requirement for FR2 HST, CATT |  |
| --- | --- |
| Company | Comments |
| XXXNokia | The CR seems OK. |
| Samsung | Suggest company to compare with the TP given in our discussion paper (R4-2201770): - TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with scaling factor N=8, for FR2 power classes other than power class 6. - TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with scaling factor N = 2 or 6, by following the network configuration [highSpeedMeasFR2Flag] = [set1] or [set2] respectively, for FR2 power class 6. The requirement for power class 6 is only applicable to the configuration with no DRX or DRX cycle≤80ms. Open to discuss pro and cons: (1) from our understanding PC6 should be introduced here to restrict the enhancement only to PC6; (2) We think the sentence of “The requirement for power class 6 is only applicable to the configuration with no DRX or DRX cycle≤80ms.” is also needed. |
| ZZZ |  |

| R4-2201771, Draft CR to introduce active TCI state switching delay requirement for FR2 HST UE, Samsung |  |
| --- | --- |
| Company | Comments |
| XXXNokia | The CR pending the outcome of Issue 3-1-2. |
| YYY |  |
| ZZZ |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #3-1: Signaling characteristics | Issue 3-1-1: RLM/BFD evaluation period Background: The majority of the companies agree on the Option 1: Define scaling factor N for no DRX and DRX cycle <=80ms by following the number of RX beams per UE for Set 1 and Set 2: Set 1: 2 RX beams; (2) Set 2: 6 RX beams. However, one of the companies made a new compromise proposal: Option 3: To simplify UE implementation, set RLM/BFD requirement according to set 2: 6Rx. One more company prefers to align upper DRX bound with the outcomes of Issue 1-2-1. Tentative agreements: None Candidate options: - Option 1 (CATT, Apple, Intel, ZTE, Ericsson, Huawei, Samsung): Define scaling factor N for no DRX and DRX cycle <=[80] ms by following the number of RX beams per UE for Set 1 and Set 2: Set 1: 2 RX beams; (2) Set 2: 6 RX beams. - Option 2 (QC): Set RLM/BFD requirement according to set 2: 6Rx. Recommendations for 2nd round: Adjust the upper DRX bound with the outcomes of Issue 1-2-1. Discuss new Option 2 in the 2nd round. Issue 3-1-2: TCI switching delay Background: All the companies reconfirm that the existing 1280ms duration for known condition is applied for FR2 HST scenario. Regarding the proposal on the one slot interruption during TCI state switching, seems that the companies need to study the potential issue more. Tentative agreements: None Candidate options: None Recommendations for 2nd round: Add directly to the WF: FFS, whether one more slot is allowed for interruption during TCI switching for FR2 HST scenario to avoid inter-symbol interference. Issue 3-1-3: Uplink Spatial Relation Switch Delay Background: All the companies agree on the proposal. Agreements: For the known conditions for spatial relation when associated with DL-RS, the requirement defined in Rel-16 shall be reused, and no standard impact is expected for Rel-17 FR2 HST. Candidate options: None Recommendations for 2nd round: Include agreement into the WF. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2201770 | Remaining Issues on signaling characteristics requirements for FR2 HST, Samsung It is recommended to follow the comment by the author “No need further discuss this as draftCR” |
| R4-2200104 | Draft CR on RLM/BFD requirement for FR2 HST, CATT Since the final agreement on the related Issue 3-1-1 was not achieved in the 1st round, and taking into account the comment on the PC6, it is recommended not to pursue the CR at this meeting. |

## Discussion on 2nd round (if applicable)

TBA

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| TDoc |  |
| --- | --- |
| Company | Comments |

### Sub-topic 2-2: TBA

#### Issue 2-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR2 HST RRM requirements (part 1) | Nokia, Nokia Shanghai Bell |  |
| LS on network signaling for Rel-17 NR HST RRM | ZZZ | To: RAN_2; Cc: RAN_1 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2201847 | TP to TR 38.854 on Mobility Performance in HST FR2 Deployment Scenarios | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2201848 | TP to TR 38.854 on the Number of Rx beams, Nokia, Nokia Shanghai Bell | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2201652 | CR to TS 38.133: intra-frequency measurements without gaps for for FR2 NR HST | Nokia, Nokia Shanghai Bell | Not Pursued |  |
| R4-2200877 | CR On RRC_CONNECTED state mobility for HST FR2 RRM | Ericsson | Revise |  |
| R4-2200104 | Draft CR on RLM/BFD requirement for FR2 HST | CATT | Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Nokia, Nokia Shanghai Bell) | Dmitry Petrov | Dmitry.a.petrov@nokia-bell-labs.com |
| QC | Chu-Hsiang (Sean) Huang | chuhsian@qti.qualcomm.com |
| Intel | Ilya Bolotin | ilya.bolotin@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when making comments, i.e., Company A (XX, XX)