3GPP TSG-RAN4 Meeting #101-bis-e	R4-2202557

# Electronic Meeting, January 17-25, 2022

Agenda item:			6.10.2.1

Source:	Moderator (Apple)

Title:	Email discussion summary for [101-bis-e][206] NR_RRM_enh2_1

Document for:	Information

# Introduction

This email discussion summary includes general (6.10.1) and SRS antenna port switching (6.10.2.1).

# Topic #1: SRS antenna port switching (6.10.2.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200068 | CATT | Proposal 1: It is captured in WF only that the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs. Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration. The SRS resource configuration can be considered in test case. Proposal 3: The L1-RSRP/L1-SINR measurement will not be impacted by SP/P SRS antenna port switching, but will be interrupted and the measurement period will be extended when overlapped with aperiodic SRS antenna port switching. Proposal 4: No specification impact of SRS antenna port switching to other specific RRM requirements except for NR measurement is needed. Proposal 5: RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. Proposal 6: Prefer RAN4 to define different requirements between sync and async cases. Proposal 7: The interruption requirement is defined based on slot level. Proposal 8: No need to discuss and clarify on the requirement applicability when SRS antenna switching is colliding with other transmission with higher priority defined in clause 6.2.1 of TS 38.214. Proposal 9: Agree option 1 in WF [1], i.e. this is up to RAN1 discussion, and no need to discuss this case in RAN4. |
| R4-2200069 | CATT | Draft CR. |
| R4-2200179 | MediaTek Inc. | Proposal 1: No requirement applies for aperiodic L1-RSRP/L1-SINR measurement collides with aperiodic SRS in the same OFDM symbol. Otherwise, when SRS resource and the NR measurement are scheduled in the same OFDM symbol, NR measurements (including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR) are always prioritized. Proposal 2: Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is no UE requirement is imposed. Proposal 3: RAN4 to clarify in Section 3 of TS38.133 that other RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities. Proposal 4: To revise the agreement in the last meeting that “RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1, NR measurement and the potential conclusions on the collision rules for all reference signals, including CSI-IM, except DMRS and UCI containing CSF report.”. Proposal 5: Define the interruption requirement for SRS antenna port switching based on slot level. Proposal 6: For interruption, to define a generic requirement regardless of SRS resource configuration. Proposal 7: RAN4 to define one single requirement to cover the synchronous and asynchronous scenarios with or without UL TA. Proposal 8: The SRS antenna switching interruption requirement should be specified as follows. Table X. Interruption length (slots) due to SRS antenna switch Victim cell SCS(KHz); Aggressor Cell SCS (KHz). 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. Proposal 9: For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol: - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation. - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any. |
| R4-2200288 | Apple | Proposal 1: No need to specify performance degradation in issue 1-1-1. Proposal 2: RAN4 to define requirements based on SRS resource configurations in scenario 1 and 2 in agreed WF of RAN4 #101e meeting. Proposal 3: RAN4 to choose one from following options: - Option 1: - For NR SA, EN-DC and NE-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized. - For NR-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol of same CG; otherwise, NR SRS antenna port switching shall be prioritized. - Option 2: - NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. Proposal 4: In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching is configured during those RRM activities. Proposal 5: Regarding impact of SRS antenna port switching to CSF and other RS, RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. Proposal 6: No need to further discuss the minimum interruption requirements for sync cases. Proposal 7: Interruption requirement of SRS antenna port switching shall be defined based on slot level. Proposal 8: Scenario 1: The interruption requirement of SRS antenna port switching for scenario 1 is 2 slots of victim carrier’s SCS, as following, Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15 (NR or LTE); 2; 2; 2. 30; 2; 2; 2. 60; 2; 2; 2. 120; 2; 2; 2. Unit of interruption requirement is slot for NR and subframe for LTE. Scenario 2: The interruption requirement of SRS antenna port switching for scenario 2 is summarized as: Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15 (NR or LTE); 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. Unit of interruption requirement is slot for NR and subframe for LTE. Proposal 9: regarding “Interruption requirement related with prioritization rule in RAN1”, RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. Proposal 10: For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol: - This is up to RAN1 discussion, and no need to discuss this case in RAN4. |
| R4-2200322 | Qualcomm, Inc. | Proposal 1: No need to capture the performance degradation during transient period in RRM specification. Proposal 2: Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed. Proposal 3: Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement based on aperiodic CSI-RS. When the collisions happen, it’s up to UE implementation for collision resolution. Proposal 4: Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply: UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies. Proposal 5: Specify the interruption requirement for scenario 2 (X=6 SRS symbols) in slot unit. Observation 1: Utilizing the uninterrupted symbols in an interrupted slot even in scenario 1 (X=1 SRS symbol is configured) is not an ideal implementation in practice for general SRS antenna switching cases Proposal 6: Specify the interruption requirement for scenario 1 (X=1 SRS symbol is configured) if gNB can not utilize the uninterrupted symbols in an interrupted slot for communication. Proposal 7: Interruption requirement is the same for synchronized and asynchronized carriers. Proposal 8: SRS antenna switching interruption for scenario 2 is specified as Table 2-2 for NR SA. In EN-DC, interruption on LTE carrier is the same as victim SCS = 15kHz case in NR SA. Scenario 2; Interruption Length (slots). Victim SCS (kHz); 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. Table 2-2 SRS antenna switch interruption Proposal 9: SRS antenna switching interruption for scenario 1 is 2 slots for all aggressor/victim SCS combinations. |
| R4-2200531 | Intel Corporation | Proposal 1: Impact of SRS antenna port switching to RRM requirements in NR-SA will be: - NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. - No requirement applies for AP/P/SP L1-RSRP/L1-SINR measurement colliding with AP SRS Proposal 2: No requirement will be defined if collision happens between scheduling of SRS antenna switching and all reference signals including CSI-IM except DMRS and UCI containing CSF report. Proposal 3: If interruption length based on symbol level is defined, MRTD/MTTD and TA margin needs to be considered. Proposal 4: When X=1 SRS symbol is configured in a slot for SRS antenna port switching, the interruption lengths are as follows: Tab.1 Interruption Length (symbols) aggressor SCS. Victim SCS; 15kHz; 30kHz; 60kHz. 15kHz; 10; 10; 10. 30kHz; 12; 10; 10. 60kHz; 17; 13; 10. Proposal 5: For other cases, using X=6 SRS symbols in a slot as assumption of SRS transmission time and requirement are defined based on slot level. |
| R4-2200561 | LG Electronics Inc. | - Proposal 1: Capture the description of performance degradation due to SRS antenna port switching in the TS38.133. - Proposal 2: For the requirement scope with different SRS resource configurations, wait for the conclusion of the interruption length depending on the components of the interruption time. - Proposal 3: The antenna switching time (15us) should be applied only when the symbol before or after SRS transmission occasion is uplink symbol since the switching time is for Tx-to-Tx. - Observation 1: In most synchronous MR-DC cases which have 35.21us MTTD, the large number of potential interrupted symbols are observed even if a single SRS resource is configured. - Observation 2: In asynchronous MR-DC cases, the exact interrupted symbols cannot be predictable. - Proposal 4: Define slot level based interruption requirements for scenario 1 (X=1), and FFS for intra-band EN-DC having 5.21us MTTD. - Proposal 5: Define slot level based interruption requirement for scenario 2 (X=6). - Proposal 6: When the symbol before or after configured SRS resource for antenna port switching is the downlink symbol, no interruption is applicable for the downlink symbol in TDD synchronous case. - Proposal 7: Introduce interruption requirements for SRS antenna port switching in Table 1. Table 1 Interruption length for SRS antenna port switching Victim cell SCS [kHz]; Interruption length [slot]. Aggressor cell SCS [kHz]. 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 2. Note 1: In TDD synchronous case, if the slot after SRS resources for antenna port switching is a downlink slot, the corresponding downlink slot is excluded from the interruption requirements. Note 2: In TDD synchronous case, if the downlink symbols and SRS resources for antenna port switching are in the same slot, the downlink symbols are excluded from the interruption requirements.. |
| R4-2200597 | vivo | Proposal 1 Interruption of SRS antenna switching is applicable only when the SRS transmission time and the guard period are not colliding with any other transmission in the same carrier with higher priority defined in clause 6.2.1 of TS 38.214. Note 1: No collision handling rule can be inferred from clause 6.2.1 of TS 38.214 for the case between DL carrier and UL carrier in NR CA, or between any two cross-CG carriers in DC. Note 2: Interruption of SRS antenna switching is still applicable to UL CA no matter whether the collision handling case is specified in TS 38.214 or not. Proposal 2 NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. Clarify in the WF to L1-RSRP/L1-SINR measurement period requirements, ‘Note: Longer measurement period is expected if a PUCCH carrying the semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier, or if PUCCH/PUSCH carrying the L1-RSRP or L1-SINR report in one carrier is collided with SRS interruption time in another carrier’. Proposal 3 To the applicability of other specific RRM requirements in which NR measurements are involved, do not add any clarifications in the spec regarding whether SRS antenna port switching is configured or not. Proposal 4 The interruption requirement is preferred to be defined based on slot level. Proposal 5 For the async case of scenario 1, the maximum interruption for SRS antenna switching is 2 slots for all kinds of subcarrier spacing. Proposal 6 For the sync case of scenario 1, if the last symbol in the slot on the aggressor CC is not used for SRS transmission, the maximum interruption for SRS antenna switching is 1 slots for all 15kHz and 30kHz aggressor CC SCS cases. |
| R4-2200640 | CMCC | Proposal 1: for the case of using 6 SRS symbols in a slot as assumption of SRS transmission time, the interruption requirements can be specified based on slot level. Proposal 2: for the case of using 1 SRS symbols in a slot as assumption of SRS transmission time, whether the interruption requirements are specified based on symbol level or slot level can be further discussed taking the misalignment between CCs into account Proposal 3: asynchronous case, for the case that SRS transmission time is 6 symbols, the interruption requirements are proposed as: Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. |
| R4-2200684 | Xiaomi | Proposal 1: RAN4 to define a generic requirement regardless of SRS resource configuration. Proposal 2: The interruption requirement for SRS antenna port switching is defined based on slot level. Proposal 3: The interruption requirement of SRS antenna port switching is defined as 2 slots for scenario 1 for both sync and async cases. Proposal 4: The interruption requirement of SRS antenna port switching for both sync and async cases is defined as: Victim CC SCS (kHz); Interruption length (slot). Aggressor CC SCS(kHz). 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. |
| R4-2200739 | ZTE Corporation | Proposal 1: Take option 1: RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. Proposal 2: Interruption requirement of SRS antenna port switching is defined based on slot level. |
| R4-2200895 | Nokia, Nokia Shanghai Bell | Proposal 1: For synchronous scenarios, when X=1 SRS symbol is configured in a slot for SRS antenna switching, the interruption length shall be defined as X1 in Table 1. Otherwise, the interruption length is defined as X2 in Table 2. Proposal 2: For asynchronous scenarios, when X=1 SRS symbol is configured in a slot for SRS antenna switching, the interruption length shall be defined as X1+1 in Table 1. Otherwise, the interruption length is defined as X2+1 in Table 2. Proposal 3: The interruption requirement applies only if SRS resources are allowed to be configured in the last 6 OFDM symbols in a slot. Proposal 4: The interruption requirement does not apply if the SRS resources of a set in a slot are configured in non-consecutive manner. Proposal 5: Do not define the requirements when AP NR SRS resource and the P/SP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol, or the prioritization needs to be clarified for this particular case. |
| R4-2201134 | OPPO | Proposal 1: Define the requirement clearly in TS 38.133 that no scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols are expected, but performance degradation on these symbols can be allowed. Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration, e.g., interruption requirements for a limited set of SRS configurations. Proposal 3: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. Proposal 4: Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. Proposal 5：RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities. Proposal 6: The same interruption requirements apply for sync and async cases. Proposal 7: For either 6 SRS symbols or 1SRS symbol, the same interruption requirement should be defined for the two scenarios in slot level. Proposal 8: Interruption of SRS antenna switching is applicable only when the interruption time is not colliding with any other transmission with higher priority defined in clause 6.2.1 of TS 38.214. |
| R4-2201201 | Huawei, Hisilicon | Proposal 1: It is preferred to capture the clarification about performance degradation on impacted symbols in RRM spec. Observation 1: When SRS resources within the same slot are configured with non-consecutive transmission, 6 SRS symbols are assumed as SRS transmission time. Proposal 2: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. Proposal 3a: No specification impact is expected of SRS antenna port switching to other specific RRM requirements except for NR measurement Proposal 3b: Capture in the WF that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities Observation 2: According to the agreement that RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement, there is no need to discuss the impact of SRS antenna port switching on CSF and other RS, and two SRS colliding. Proposal 4: Define the interruption requirements for scenarios 1 and 2 according to table II and table I. Table I Interruption requirement in number of slots for scenarios 2 Aggressor CC SCS(kHz). Victim CC SCS (kHz); 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. Table II Interruption requirement in number of slots for scenarios 1 Aggressor CC SCS(kHz). Victim CC SCS (kHz); 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 2; 2; 2. 120; 2; 2; 2. |
| R4-2201378 | Ericsson | Proposal 1: RAN4 shall focus on defining requirements assuming SRS resources in a set are transmitted in the same slot with consecutive SRS resources being separated at most by one OFDM symbol for µ=0,1,2. Proposal 2: Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. Proposal 3: RAN4 to define interruption requirement for sync case also. Proposal 4: Interruption requirements shall be based on slot granularity for async cases. Proposal 5: Interruption requirements shall be based on symbol granularity for sync cases. Proposal 6: Two SRS colliding on same symbol is RAN1 issue and RAN4 do not need to discuss this. |
| R4-2201379 | Ericsson | Draft CR |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Scope of SRS antenna switching requirement

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: Whether to capture performance degradation clarification in RRM spec:

Agreement in last meeting: The performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs.

- Proposals

  - Option 1 (LGE, OPPO, Huawei): yes, clarify it in RRM Spec

  - Option 2 (CATT, Apple, QC): no, it was captured in last meeting’s agreed WF

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 2. But since there is no any technical issue with option 1, if majority companies agree with option1, we can compromise to option 1 as well. |
| QC | We support option 2. |
| Huawei | No strong views. Prefer to capture in spec. |
| LGE | We are open to option 2, but prefer to capture it in the spec and not sure that similar intention of the agreement is captured in RF spec. |
| OPPO | Prefer Option 1 to make the spec clear to us. Do not see any technical concern on the clarification. |
| MediaTek | No strong view. Prefer Option 1. |
| Ericsson | Since there is an explicit agreement that performance degradation can be expected, it is better to capture it in Spec. No strong views though. |
| ZTE | No strong view. It has been captured in last WF. Option 1 could be supported. |
| QC | In RF spec, the following description is captured already: The transmit power time mask defines the transient period(s) allowed - between transmit OFF power as defined in clause 6.3.2 and transmit ON power symbols (transmit ON/OFF) - between continuous ON-power transmissions with powerchange or RB hopping is applied. Our understanding is that power transition/change happens at transient period, and the performance degradation is the implication of power transition/change. Capturing this implication for SRS but not other transient periods may lead to confusion of whether other transient periods specified in 38.101-1 6.3.3 and other sections have the same performance degradation. |
| Nokia | Slightly prefer Option 1 so that vendors are aware of the potential performance degradation when configuring SRS antenna switching. But we also agree with QC’s comments and can accept Option 2. |
| vivo | No strong view. Probably RAN4 can discuss this in the CR phase. In case companies can not reach consensus in the detailed wording in the spec, it can be removed. |
| CATT | Support option 2. Generally only the requirements that can be tested will be captured in the spec. |

Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration

- Proposals

  - Option 1 (CATT, Xiaomi, OPPO, MTK):

    - RAN4 to define a generic requirement regardless of SRS resource configuration.

      - Option 1a(CATT):  The SRS resource configuration can be considered in test case.

  - Option 2 (Apple):

    - RAN4 to define requirements based on SRS resource configurations in scenario 1 and 2 in agreed WF of RAN4 #101e meeting.

  - Option 3 (LGE):

    - For the requirement scope with different SRS resource configurations, wait for the conclusion of the interruption length depending on the components of the interruption time.

  - Option 3(Ericsson):

    - RAN4 shall focus on defining requirements assuming SRS resources in a set are transmitted in the same slot with consecutive SRS resources being separated at most by one OFDM symbol for µ=0,1,2.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 1 and option 2. It has already been agreed that interruption requirement would be designed based on scenario 1 and 2, it’s no need to discuss other SRS configuration. |
| QC | We have the same view as Apple |
| Huawei | Support option 2. According to the agreements in last meeting, there is no need to discuss this issue. |
| LGE | We are fine for option 2. But we think the requirements could be generic if slot based interruptions are defined for both X=1 and X=6. |
| OPPO | Support Option 1. |
| Xiaomi | Support option 1 and option 2. |
| MediaTek | Either option 1 or 2 is fine to us. |
| Ericsson | We are fine with option 2. However, there is a clarification needed on X=6; That is X=6 includes both SRS and guard period. |
| ZTE | We support option 1 and can compromise to option 2. |
| Intel | Support option 2. |
| Nokia | Option 2. Following agreements from last meeting, option 2 should be the starting point. But under scenario 2, we believe the non-consecutive SRS resources case shall be excluded as it is being discussed in RAN1 FeMIMO WI. It would be good to have this clarified in scenario 2 e.g. by adding a note as below? Note: In scenario 2, SRS resources in a set are transmitted in the same slot with consecutive SRS resources being separated at most by one OFDM symbol for µ=0,1,2 |
| vivo | Support option 2. |
| CATT | Support option 1 and can compromise to option 2. |

### Sub-topic 1-2: Impact of SRS antenna port switching to other requirements

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA

- Proposals

  - Option 1 (CATT): The L1-RSRP/L1-SINR measurement will not be impacted by SP/P SRS antenna port switching, but will be interrupted and the measurement period will be extended when overlapped with aperiodic SRS antenna port switching.

  - Option 2 (Apple): UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

  - Option 3 (Apple, Intel, vivo, OPPO, HW, MTK): NR measurements are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

      - Option 3a (MTK): No requirement applies for aperiodic L1-RSRP/L1-SINR measurement collides with aperiodic SRS in the same OFDM symbol.

      - Option 3b (Intel): No requirement applies for AP/P/SP L1-RSRP/L1-SINR measurement colliding with AP SRS.

      - Option 3c (vivo): Clarify in the WF to L1-RSRP/L1-SINR measurement period requirements,

        - ‘Note: Longer measurement period is expected if a PUCCH carrying the semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier, or if PUCCH/PUSCH carrying the L1-RSRP or L1-SINR report in one carrier is collided with SRS interruption time in another carrier’.

  - Option 4 (QC):

    - Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement based on aperiodic CSI-RS. When the collisions happen, it’s up to UE implementation for collision resolution.

    - Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply: UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies

  - Option 5 (Nokia): Do not define the requirements when AP NR SRS resource and the P/SP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol, or the prioritization needs to be clarified for this particular case.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 2 and option 3. The option 2 is using the similar rule of the SRS prioritization in section 6.2.1.3 for the collision between CCs. However, in order to move forward, we could compromise to the simple solution for RAN4 requirement and UE implementation, i.e., option 3. |
| QC | Support option 3a. Part of option 4 aligns to option 3a and can be combined. |
| Huawei | Support option 3. It could be seen in 9.5.6.3 that A/SP/P CSI-RS for L1-RSRP are all considered when colliding with SRS. |
| OPPO | Support Option 3, as commented by Apple. |
| MediaTek | Prefer option 3a. The reason is because UE may not be able to handle the collision between aperiodic SRS and aperiodic CSI-RS within a limited time. |
| Ericsson | Just a clarification question. Are we talking about measurement report instance. If so we are fine with option 3. |
| Intel | Prefer option 3b to consider AP SRS case. we can compromise to option 3 to move forward. |
| Nokia | Option 5. This can also be merged into Option 3 with small difference from Option 3b: Option 3d: No requirement applies for AP/P/SP L1-RSRP/L1-SINR measurement colliding with AP SRS. There seems to be different understanding on the exceptional case. We refer to below in TS38.214 section 6.2.1, which provides the general prioritization principle for SRS transmission. Following the text, PUCCH is prioritized only if AP-SRS is overlapping with SP/P L1-RSRP. We wonder why AP L1-RSRP is also included in Option 3b? PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). For Option 2, the prioritization is following the principle dedicated to SRS carrier-based switching which is different function from SRS antenna switching. |
| vivo | Support option 3. |
| CATT | Option 1. This can be merged into option 3b. |

Issue 1-2-2: Impact of SRS antenna port switching to other specific RRM requirements

- Agreements in RAN4#100e meeting - RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. - RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements.

- Proposals

  - Option 1 (CATT, vivo, HW): No specification impact of SRS antenna port switching to other specific RRM requirements except for NR measurement is needed.

    - Option 1a (HW): Capture in the WF that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities

  - Option 2 (MTK, OPPO): RAN4 to clarify in Section 3 of TS38.133 that other RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities.

  - Option 3 (Apple): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching is configured during those RRM activities.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Fine with option 2 and 3. We prefer to capture the clarification in spec since we did the same clarification for SRS carrier switching in LTE spec, e.g., LTE SCell activation requirement. |
| QC | Option 1,2,3 seem similar. Capture in WF is good for us as a simpler solution. |
| Huawei | We support option 1 and option 1a which is a compromised solution. |
| OPPO | Support Option 2. Also agree to capture it in the spec. |
| MediaTek | Support option 2. In Issue 1-2-1, RAN4 almost agreed that NR measurement may have impact on SRS antenna port switching. To us, it would be mis-understanding when the SRS antenna port switching collides with other RRM requirement having NR measurement procedure. To avoid such mis-understanding, it would be better to clearly define in spec. |
| Ericsson | Here also are we talking about CSI report in other RRM requirements being overlapped with SRS on the same OFDM symbols? If it is the case, we are fine with it in principle. However, current wording needs to be clarified/modified to reflect this. |
| ZTE | Prefer option 2 and option 3. |
| Nokia | Option 1. In existing spec, the impact due to SRS transmission has been specified in relevant clauses. We don’t see any immediate specification impact right now. If additional impact is identified due to SRS antenna switching e.g. on SCell activation, we can consider adding conditions to the specific RRM requirements as in Option 3 but it should be clear what RRM requirements are impacted. |
| vivo | Support option 1. Capture in WF is already enough |
| CATT | Support option 1 and 1a. |

Issue 1-2-3: Impact of SRS antenna port switching to CSF and other RS

- Proposals

  - Option 1 (CATT, Apple, ZTE, HW): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

  - Option 2 (MTK, QC, Intel, OPPO): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed.

  - Option 3 (Ericsson): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. From technical perspective we are fine with option 2, but we don’t think it needs to be captured in RRM spec for any requirement if we have the clarification in issue 1-2-2. |
| QC | We support option 2 but can compromise to option 1 due to now it’s in a late stage in this WI. |
| Huawei | Support option 1 according to agreements in last meeting. |
| OPPO | Prefer option 2. It may lead to some uncertain for implementation if going to option 1. |
| MediaTek | Prefer option 2. |
| Ericsson | Option 1. If something is not clear we should send LS to RAN1 (option 3). |
| ZTE | Prefer option 1. |
| Intel | Prefer option 2 and can compromise to option 1. |
| Nokia | Option 1. We believe this is RAN1 issue and should not be discussed in RAN4. |
| vivo | OK to option 1. |
| CATT | Prefer option 1. |

### Sub-topic 1-3: Interruption requirement applicability

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3-1: Whether and how to specify interruption requirement for sync case

- Proposals

  - Option 1 (CATT, Ericsson): RAN4 to define different requirements between sync and async cases.

  - Option 2 (NTK, Apple, QC, Xiaomi, OPPO): No need to further discuss a separate interruption requirement for sync cases.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. We clarified the reason in our paper for couple meetings, e.g., considering the TA, MTTD/MRTD. |
| QC | Support option 2 and same view as Apple. |
| LGE | Generally, we are fine with the same requirements for sync and async. However, in TDD synchronous case, the downlink symbols should be excluded for interruption requirements when the symbol before and/or after SRS resource for antenna switching is downlink symbol. |
| OPPO | Support option 2 and same view as Apple. |
| Xiaomi | Support Option 2. |
| MediaTek | Option 2. Same view as Apple. |
| Ericsson | Option 1. MRTD/MTTD difference between sync and async cases is high. For sync cases it is possible to compute the location of interruption (on which OFDM symbol) due to short MRTD. Hence, we prefer defining requirements in symbols for sync case. For async case, since RTD/TTD is high and computing the location of interruption (which symbols are going to be affected) is tough. Hence, we are OK defining it in slots for async case. |
| Intel | Support option 2. |
| Nokia | Option 1. We tend to agree with the impact due to MRTD/MTTD. It adds 33us interruption in sync and half a slot in async scenarios. As 33us counts around 1 symbol, the interruption length in sync scenarios shall be defined in symbol-level. For async scenario, if 8-10 symbols are resulted, we may consider round up to 1 slot. About TA, we wonder how it would impact the interruption. As we agreed to define the interruption length for DL and UL respectively, TA has nothing to do with the interruption on UL. Even for DL interruption, the TA ensures SRS transmission arriving at DL at exact timing with MRTD shift. We’d like to better understand the issue. Anyway, as discussed in last meeting, it would be reasonable to calculate the interruption lengths for sync and async scenarios respectively, before we could make comparison and conclude on the scenarios. |
| vivo | We support option 1, especially for scenario 1. For scenario 2, we do not have strong view. |
| CATT | Prefer option 1. |

### Sub-topic 1-4: Interruption requirement design

Sub-topic description

- Agreements in last RAN4 meeting - The components of interruption time of SRS antenna port switching in FR1 are - Antenna switching time before and after SRS transmission occasion (2*15us) - SRS transmission time of X symbols - Requirements would be defined for two scenarios: - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, the configured number of SRS symbols is used as SRS transmission time - Scenario 2: otherwise, using X=6 SRS symbols in a slot as assumption of SRS transmission time

Open issues and candidate options before e-meeting:

Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level

- Proposals

  - Option 1 (CATT, MTK, Apple, QC, vivo, xiaomi, ZTE, OPPO, HW, LGE): based on slot level

    - Option 1a (QC): Specify the interruption requirement for scenario 1 (X=1 SRS symbol is configured) if gNB can not utilize the uninterrupted symbols in an interrupted slot for communication.

    - Option 1b (LGE): FFS for intra-band EN-DC having 5.21us MTTD

  - Option 2 (Nokia, ): based on symbol level

  - Option 3 (Intel):

    - based on symbol level for scenario 1.

    - based on slot level for scenario 2.

  - Option 4 (CMCC):

    - FFS for scenario 1.

    - based on slot level for scenario 2.

  - Option 5 (Ericsson):

    - based on slot level for async cases.

    - based on symbol level for sync cases.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 1. The interruption itself is not controllable by network (different from scheduling restriction), the impact of losing one or more symbols would eventually make the whole slot unusable; and in addition, test case is designed to count the ACK/NACK loss for interruption requirement like RAN4 design for SRS carrier based switching. |
| QC | Support option 1, can compromise to option 4. |
| Huawei | Support option 1, but compromise to option 4. Our concerns about having symbol level requirements are the performance can not be tested. |
| LGE | We support option 1 since several symbols are interrupted even if X=1 considering MTTD. But, FFS for intra-band EN-DC with 5.21us MTTD. |
| OPPO | Support option 1. Agree that the impact of losing one or more symbols would eventually make the whole slot unusable. |
| Xiaomi | Support Option 1. |
| MediaTek | Support option 1. We are wondering how does network know which symbol is interrupted if we go with option 2. |
| Ericsson | Due to large MRTD/MTTD, we may not be able to compute the interruption location even if X=1; Whether X=1 or 6 interruption location may shift by same value. Due to this we propose option 5. |
| ZTE | We support option 1. |
| CMCC | For scenario 1, even though X=1, we not sure whether the interruption requirements are specified based on symbol level or slot level taking the misalignment between CCs into account. We are open to discussion and would like to hear companies’ views. |
| Intel | Option 3 is a compromise solution. We can also agree with option 1 and option 4. |
| Nokia | We can compromise to Option 5 considering the MRTD impact in async scenario. |
| vivo | Support option 1. |
| CATT | Support option 1. |

Issue 1-4-2: Antenna switching time

- Proposals

  - Option 1 (LGE): The antenna switching time (15us) should be applied only when the symbol before or after SRS transmission occasion is uplink symbol since the switching time is for Tx-to-Tx

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Different view from option 1. The Rx would be also impacted during the transient period since UE would adjust its RF, as long as UE indicates the impact from SRS antenna switching to Rx carrier. |
| QC | SRS is scheduled in special slot in most cases, therefore UL symbols follows almost all SRS Tx in practice. Even if Rx follows SRS Tx, non-zero transient time is needed (note that even w/o antenna switching, transient time is 10us). Since it’s not seen in practice, RAN4 should consider only Tx to Tx case when setting requirements. |
| Huawei | Suggest to define the requirements in a general approach. If SRS resource is located at the end of an UL slot and followed by DL slot, transient time is still needed. |
| LGE | Based on RF spec., the 15us transient time for antenna switching is between Tx to Tx. When we consider the transient time (e.g., 10us) for Tx to Rx or Rx to Tx in TDD, NTA_offset (20us or 13us) can be covered. So, we don’t need additional transient time when the symbol before and/or after SRS resource for antenna switching is downlink symbol. |
| Xiaomi | The transition time is needed for either TX/TX switching or TX/RX switching, and the transition time would have impact on the indicated CC. We prefer to define unified requirement. |
| MediaTek | Same view as Apple, QC and HW |
| Ericsson | We are fine to further look into it. |
| vivo | Same view as Apple, QC and HW |
| CATT | If the requirements are defined as slot level, no need to discuss such detail. |
| LGE | For some response, This proposal is to clarify the interruption in the case where the symbol before/after SRS resource is downlink symbol in TDD sync. The 15us transient time for SRS antenna port switching is between SRS resources, i.e., Tx-to-Tx switching. And in my understanding,10us the transient time Rx-to-Tx or Tx-to-Rx switching in TDD is already existed, and NTA_offset covers the impact on this transient time. So, even if the slot based requirements is defined, the DL symbols should be excluded from interrupted slot especially in special slot. |

Issue 1-4-3: Interruption requirement proposals for scenario 1

- Proposals

  - Option 1 (MTK):

- Table X. Interruption length (slots) due to SRS antenna switch

| Victim cell SCS(KHz) | Aggressor Cell SCS (KHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |

  - Option 2 (Apple, QC, xiaomi, HW): The interruption requirement of SRS antenna port switching for scenario 1 is 2 slots of victim carrier’s SCS,

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Unit of interruption requirement is slot for NR and subframe for LTE.

  - Option 3 (Intel):

- Tab.1 Interruption Length (symbols)

|  | aggressor SCS |  |  |
| --- | --- | --- | --- |
| Victim SCS | 15kHz | 30kHz | 60kHz |
| 15kHz | 10 | 10 | 10 |
| 30kHz | 12 | 10 | 10 |
| 60kHz | 17 | 13 | 10 |

  - Option 4 (LGE):

    - When the symbol before or after configured SRS resource for antenna port switching is the downlink symbol, no interruption is applicable for the downlink symbol in TDD synchronous case.

- Table 1 Interruption length for SRS antenna port switching

| Victim cell SCS [kHz] | Interruption length [slot] |  |  |
| --- | --- | --- | --- |
|  | Aggressor cell SCS [kHz] |  |  |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 2 |
| Note 1: In TDD synchronous case, if the slot after SRS resources for antenna port switching is a downlink slot, the corresponding downlink slot is excluded from the interruption requirements. Note 2: In TDD synchronous case, if the downlink symbols and SRS resources for antenna port switching are in the same slot, the downlink symbols are excluded from the interruption requirements. |  |  |  |

  - Option 5 (vivo):

    - For the async case of scenario 1, the maximum interruption for SRS antenna switching is 2 slots for all kinds of subcarrier spacing.

    - For the sync case of scenario 1, if the last symbol in the slot on the aggressor CC is not used for SRS transmission, the maximum interruption for SRS antenna switching is 1 slots for all 15kHz and 30kHz aggressor CC SCS cases.

  - Option 6 (Nokia):

    - For synchronous scenarios, when X=1 SRS symbol is configured in a slot for SRS antenna switching, the interruption length shall be defined as X1 in Table 1.

    - For asynchronous scenarios, when X=1 SRS symbol is configured in a slot for SRS antenna switching, the interruption length shall be defined as X1+1 in Table 1.

- Table 1: Interruption length X1 (symbols)

|  | NR Slot length(ms) of victim cell | NR Symbol length (us) of victim cell | Interruption time (us) |  | Interruption length X1 (symbols) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub carrier spacing for aggressor cell (kHz) |  | Sub carrier spacing for aggressor cell (kHz) |  |
|  |  |  | 15 | 30 | 15 | 30 |
| 0 | 1 | 71.35 | 101.35 | 65.68 | 2 | 1 |
| 1 | 0.5 | 35.68 | 101.35 | 65.68 | 3 | 2 |

  - Option 7 (OPPO): For either 6 SRS symbols or 1SRS symbol, the same interruption requirement should be defined for the two scenarios in slot level.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2, up to the conclusion of issue 1-4-1 and issue 1-3-1. |
| QC | Support option 2 |
| Huawei | Support option 2 |
| LGE | The Note 1 and 2 of option 4 should be considered to avoid unnecessary interruption for downlink symbols. |
| OPPO | OK with option 2. |
| Xiaomi | Support Option 2. |
| MediaTek | Option 2 after checking other companies’ tdocs |
| Ericsson | Can be FFS till the interruption granularity is agreed. |
| Intel | We can compromise to option 2. |
| Nokia | We would like to update Option 6 as below taking into account MRTD. Taking into account 33us MRTD, the interruption for sync scenario can be reformulated as below. And for async scenario, additional 6 symbols are added on top of it. Table 1: Interruption length X1 (symbols) NR Slot length(ms) of victim cell; NR Symbol length (us) of victim cell; Interruption time (us); Interruption length X1 (symbols). Sub carrier spacing for aggressor cell (kHz); Sub carrier spacing for aggressor cell (kHz). 15; 30; 15; 30. 0; 1; 71.35; 101.35; 65.68; 2; 2. 1; 0.5; 35.68; 101.35; 65.68; 4; 3. |
| vivo | Option 2 can be supported for the async case. For sync case we prefer FFS. |
| CATT | OK with option 2 for asyn case. |

Issue 1-4-4: Interruption requirement proposals for scenario 2

- Proposals

  - Option 1 (MTK(without 120kHz aggressor), Apple, QC, CMCC(for asynch), xiaomi, HW): the interruption requirement of SRS antenna port switching is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

Unit of interruption requirement is slot for NR and subframe for LTE.

  - Option 2 (LGE):

    - When the symbol before or after configured SRS resource for antenna port switching is the downlink symbol, no interruption is applicable for the downlink symbol in TDD synchronous case.

- Table 1 Interruption length for SRS antenna port switching

| Victim cell SCS [kHz] | Interruption length [slot] |  |  |
| --- | --- | --- | --- |
|  | Aggressor cell SCS [kHz] |  |  |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 2 |
| Note 1: In TDD synchronous case, if the slot after SRS resources for antenna port switching is a downlink slot, the corresponding downlink slot is excluded from the interruption requirements. Note 2: In TDD synchronous case, if the downlink symbols and SRS resources for antenna port switching are in the same slot, the downlink symbols are excluded from the interruption requirements. |  |  |  |

  - Option 3 (Nokia):

    - For synchronous scenarios, the interruption length is defined as X2 in Table 2.

    - For asynchronous scenarios, the interruption length is defined as X2+1 in Table 2

- Table 2: Interruption length X2 (symbols)

|  | NR Slot length(ms) of victim cell | NR Symbol length (us) of victim cell | Interruption time (us) |  | Interruption length X2 (symbols) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub carrier spacing for aggressor cell (kHz) |  | Sub carrier spacing for aggressor cell (kHz) |  |
|  |  |  | 15 | 30 | 15 | 30 |
| 0 | 1 | 71.35 | 458.1 | 244.08 | 7 | 4 |
| 1 | 0.5 | 35.68 | 458.1 | 244.08 | 13 | 7 |

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1, up to the conclusion of issue 1-4-1 and issue 1-3-1. |
| QC | Option 1. |
| Huawei | Support option 1. |
| LGE | Same comments as scenario 1. |
| OPPO | Option 1 is fine. |
| Xiaomi | Support Option 1. |
| MediaTek | Option 1. |
| Ericsson | Can be FFS till the interruption granularity is agreed. |
| CMCC | We are OK with option 1. |
| Intel | Option 1. |
| Nokia | We would like to update Option 3 as below. Taking into account 33us MRTD, the interruption length for scenario 2 in sync scenario can be reformulated as below. For async scenario, additional half a slot is added. We may then consider using slot-level interruption for async scenario 2. Table 2: Interruption length X2 (symbols) NR Slot length(ms) of victim cell; NR Symbol length (us) of victim cell; Interruption time (us); Interruption length X2 (symbols). Sub carrier spacing for aggressor cell (kHz); Sub carrier spacing for aggressor cell (kHz). 15; 30; 15; 30. 0; 1; 71.35; 458.1; 244.08; 7; 4. 1; 0.5; 35.68; 458.1; 244.08; 14; 8. |
| vivo | OK to option 1. |
| CATT | Fine with option 1 with asyn case. |

### Sub-topic 1-5: Miscellaneous issues

Sub-topic description

Open issues and candidate options before e-meeting:

- Agreements in last RAN4 meeting - RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - FFS: Interruption of SRS antenna switching is applicable only when the interruption time is not colliding with any other transmission with higher priority defined in clause 6.2.1 of TS 38.214.

Issue 1-5-1: Interruption requirement related with prioritization rule in RAN1

- Proposals

  - Option 1 (CATT, Apple):

    - RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

      - No need to discuss and clarify on the requirement applicability when SRS antenna switching is colliding with other transmission with higher priority defined in clause 6.2.1 of TS 38.214.

  - Option 2 (MTK): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1, NR measurement and the potential conclusions on the collision rules for all reference signals, including CSI-IM, except DMRS and UCI containing CSF report.

  - Option 3 (vivo, OPPO): Interruption of SRS antenna switching is applicable only when the SRS transmission time and the guard period are not colliding with any other transmission in the same carrier with higher priority defined in clause 6.2.1 of TS 38.214.

    - Option 3a (vivo):

      - Note 1: No collision handling rule can be inferred from clause 6.2.1 of TS 38.214 for the case between DL carrier and UL carrier in NR CA, or between any two cross-CG carriers in DC.

      - Note 2: Interruption of SRS antenna switching is still applicable to UL CA no matter whether the collision handling case is specified in TS 38.214 or not.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. RAN4 only defines the applicability/priority for RRM related activities, and in issue 1-2-2 we clarified no SRS antenna switching is assume during the other RRM activities for requirement applicability. |
| QC | Support option 1, same view as apple. |
| Huawei | Support option 1. |
| OPPO | We can compromise to option 1 as well, with the agreements in issue 1-2-2 as Apple clarified. |
| MediaTek | Support option 2. We are wondering if RAN4 go with option 1. Then, do we need to discuss the issue 1-2-3. In fact, there is an open issue which is not included in RAN1 spec and NR measurement, right? |
| Ericsson | Option 1 |
| ZTE | Support option 1. |
| CMCC | OK with option 1. |
| Intel | Fine with Option 1. |
| Nokia | We support Option 1 and Option 2 that this prioritization rule shall be defined in RAN1. About the collision rules, we understood the collision rules are defined in RAN1 as well, but the collision with RRM measurements are defined in RAN4. If this issue relates to the LS sent to RAN1, we should wait reply LS before any further discussion. |
| vivo | Do not see the difference between the main bullet of option 1 and option 3. Do RAN4 still need to introduce interruption requirement if SRS is not transmitted according to RAN1 spec? We think the collision rule defined in RAN1/2 is not clear for the SRS ant switching. Therefore, some clarifications would be fine, similar to the SRS carrier switching case. |
| CATT | Support option 1. |

Issue 1-5-2: Two SRS colliding on same symbol

For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

- Proposals:

  - Option 1 (CATT, Apple, Ericsson): This is up to RAN1 discussion, and no need to discuss this case in RAN4.

  - Option 3 (MTK): For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol:

    - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation.

    - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. We think this R17 FeRRM WI shall not be based on any other parallel R17 feature, and we could leave this issue to RAN1. |
| QC | Option 1. |
| Huawei | Support option 1. |
| MediaTek | More discussion is needed. For option 1, does “no discussion” mean no UE requirement applied when the collision happen? |
| Ericsson | Option 1. |
| ZTE | Support option 1. |
| Nokia | Option 1. |
| vivo | Option 1 |
| CATT | Support option 1. |

Issue 1-5-3: Impacts from SRS antenna port switching enhancement in R17 FeMIMO

- Proposals:

  - Option 1 (Nokia):

    - The interruption requirement applies only if SRS resources are allowed to be configured in the last 6 OFDM symbols in a slot.

    - The interruption requirement does not apply if the SRS resources of a set in a slot are configured in non-consecutive manner.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Don’t need to discuss the FeMIMO related issue in this SRS antenna port switching requirement. This R17 FeRRM WI shall not be based on any other parallel R17 feature. |
| QC | FeMIMO is out of scope for R17 FeRRM, only R15 procedures are discussed. |
| Huawei | Fine with the first bullet. There is no need to consider the second bullet which is already covered by agreed scenario 1/2. |
| LGE | We don’t need to consider Rel-17 FeMIMO related issue. |
| Xiaomi | The issue related to R17 FeMIMO is out of scope of R17 FeRRM WI. |
| MediaTek | Same view as Apple, QC and LGE. |
| Ericsson | DO not understand the proposal intention. Can Nokia please clarify, why it has to be discussed separately for FeMIMO? Any FeMIMO specific issue is present? |
| Intel | Don’t need to consider Rel-17 FeMIMO issue. |
| Nokia | Option 1. The purpose of the proposal is exactly to exclude any scenarios introduced by R17 FeMIMO. For the first bullet, the intention is to avoid configuring SRS resources in symbols other than the last 6 symbols in a slot. Note that in R17 FeMIMO, SRS resources can be configured on 12 symbols in a slot. This should not be considered when defining the requirements. The 2nd bullet is about SRS configuration as discussed in Issue 1-1-2. For scenario 2, we should assume SRS resources are configured in consecutive manner. |
| vivo | Out-of-scope. The related discussion is precluded from R17 feMIMO WI. We also think it should not be discussed here. |
| CATT | Same view as Apple. |

### Sub-topic 1-6: Feature list for SRS antenna port switching

Sub-topic description

Open issues and candidate options before e-meeting:

Proposals in R4-2200286 (Apple), R4-2200544 (Intel).

- Proposals

  - Option 1 (Apple):

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | SRS antenna port switching | 1) Support RRM requirements of SRS antenna port switching | [Rel-15 NR RAN1 UE feature list feature 2-55 (SRS Tx switch)] | Yes | N/A | Network cannot know the interruption time when SRS antenna port switching happens for this UE. Therefore, either network may not trigger SRS antenna port switch or there will be performance degradation | Per UE | No | FR1 | N/A | Functionality of SRS antenna port switching has already been supported since R15. RRM requirement is expected to be introduced in R17. Thus, R17 UE shall meet corresponding RRM requirement. The requirements apply when SRS antenna port switching is on NR carrier. | Optional with capability signalling |

  - Option 2 (Intel):

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | SRS antenna port switching RRM requirements | 1) Support of SRS antenna port switching RRM requirements | R15 RAN1 feature group 2-55 SRS Tx switch | Yes | No | The UE does not meet the specified RRM requirements for SRS antenna port switching in Rel-17 | Per UE | No | No | N/A |  | Mandatory with capability signalling if the prerequisite group is supported by the UE |

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. Even though the UE may indicate to support SRS antenna port switching from R15, it doesn’t mean UE is mandated to meet the R17 SRS antenna port switching requirement. |
| QC | Since the objective of R17 FeRRM is define requirement for R15 features, R15 feature capability implies the support of the feature and the requirement simultaneously for R17 and future UEs. The support of RAN4 requirement should always couple with the associated feature. It doesn’t have its own capability, as separate capabilities can decouple the feature and the requirement while complete spec is defined. The separate capabilities not only confuses network but also leave system without performance guarantee. |
| Huawei | Similar discussion happened in Rel-16 RRM enhancement. From our understanding, we suggest not to have dedicated capability to indicate whether UE can meet certain requirement. |
| OPPO | Ok to support RRM requirements of SRS antenna port switching for FR1. Since the related RF requirements for SRS antenna port switching have been already defined since R17, e.g., insert loss per band comb, RRM requirements can be agreed as optional. In addition, defining UE features of R17 FeRRM as a package is also fine to us. The details could be further discussed case by case. |
| MediaTek | Support QC and HW. |
| Ericsson | Same view as QC, HW and MTK. As per our understanding we agreed in Rel-16 that RRM requirements will not be optional if UE supports the feature. We can follow the same agreement. |
| CMCC | No need to introduce UE capability for requirements. In Rel-16, we have similar discussion. And the conclusion is not to introduce UE capability for the requirements, and the requirements are only applied to Rel-16 UE and later release UE. It is preferred to follow the same approach. For SRS antenna port switching, no need to introduce UE capability for requirements. Since the requirements are defined in Rel-17, it can be clarified that the requirements are only applied to R17 and future UEs. |
| Intel | We want to double check with companies that whether the requirement defined in Rel-17 for SRS antenna port switching can apply for legacy Rel-15/Rel-16 UE. - If no, then we are fine not to introduce extra UE capability. - If Yes, we think it’s better to define new Rel-17 capability. Otherwise, there are some existing Rel-15/Rel-16 UE, which may indicate to support the capability while not meet the Rel-17 requirement. |
| Nokia | Share the view from QC and HW. We can follow the agreements in R16 feature list discussion. |
| vivo | Same view as QC and HW. For UE cannot meet the requirements in R17, UE can report ‘not supported’ for the SRS ant switching capability. |
| CATT | Same view as Huawei and QC. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section 1.2

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200069 (CATT CR) | Apple: may need to be revised based on the conclusions from all above issues. |
|  | QC: suggest to come back next meeting until the relevant agreements are reached. |
|  | Huawei: suggest to focus on the issue first. Some contents depend on pending issues. |
|  | Nokia: Can come back when open issues are concluded. |
| R4-2201379 (Ericsson CR) | Apple: may need to be revised based on the conclusions from all above issues, e.g., we may not need to list all the components of interruption in requirement but we only need a table to summarize the interruption values. |
|  | QC: suggest to come back next meeting until the relevant agreements are reached. |
|  | Huawei: suggest to focus on the issue first. Some contents depend on pending issues. |
|  | Nokia: Can come back when open issues are concluded. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 1-1: Scope of SRS antenna switching requirement

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Whether to capture performance degradation clarification in RRM spec: | Tentative agreements: Since companies have no strong views on two options but QC provided the RF spec to support option 2, recommend to adopt Option 2. Candidate options: Recommendations for 2nd round: Please @OPPO check if the tentative agreement is acceptable or not. Conclusions would be captured in the WF. |
| Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration | Tentative agreements: RAN4 to define requirements based on SRS resource configurations in scenario 1 and 2 in agreed WF(R4-2120295) of RAN4 #101e meeting. Candidate options: Recommendations for 2nd round: Please @OPPO check if the tentative agreement is acceptable or not. Conclusions would be captured in the WF. |

Sub-topic 1-2: Impact of SRS antenna port switching to other requirements

|  | Status summary |
| --- | --- |
| Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA | Tentative agreements: Based on the 1st round comments collection, option 3 could be a general tentative agreement, but the option a/b/c risen by companies shall be discussed in 2nd round. - NR measurements are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement, and FFS on following options for specific scenarios: - Option 1 (MTK, QC, Intel): No requirement applies for aperiodic L1-RSRP/L1-SINR measurement collides with aperiodic SRS in the same OFDM symbol. - Option 2 (Intel, CATT): No requirement applies for AP/P/SP L1-RSRP/L1-SINR measurement colliding with AP SRS. - Option 3 (Nokia): No requirement applies for P/SP L1-RSRP/L1-SINR measurement colliding with AP SRS. Candidate options: Recommendations for 2nd round: Please @Ericsson check if the tentative agreement is acceptable or not. Continue discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-2-2: Impact of SRS antenna port switching to other specific RRM requirements | Tentative agreements: Candidate options: - Option 1 (CATT, vivo, HW, QC, Nokia): No specification impact of SRS antenna port switching to other specific RRM requirements except for NR measurement is needed. - Option 1a (HW, CATT): Capture in the WF that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities - Option 2 (MTK, OPPO, Apple, ZTE): RAN4 to clarify in Section 3 of TS38.133 that other RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities. - Option 3 (Apple, ZTE): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching is configured during those RRM activities. Recommendations for 2nd round: Continue discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-2-3: Impact of SRS antenna port switching to CSF and other RS | Tentative agreements: Candidate options: - Option 1 (CATT, Apple, ZTE, HW, QC, E///, Intel, Nokia, vivo): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - Option 2 (MTK, QC, Intel, OPPO): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed. Recommendations for 2nd round: Please @OPPO and @MTK check if could compromise to option 1. Continue discussion in 2nd round. Conclusions would be captured in the WF. |

Sub-topic 1-3: Interruption requirement applicability

|  | Status summary |
| --- | --- |
| Issue 1-3-1: Whether and how to specify interruption requirement for sync case | Tentative agreements: Candidate options: - Option 1 (CATT, Ericsson, Nokia, vivo(for scenario 1)): RAN4 to define different requirements between sync and async cases. - Option 2 (MTK, Apple, QC, Xiaomi, OPPO, LGE(with clarification for TDD sync), Intel): No need to further discuss a separate interruption requirement for sync cases. Recommendations for 2nd round: Continue discussion in 2nd round. Conclusions would be captured in the WF. |

Sub-topic 1-4: Interruption requirement design

|  | Status summary |
| --- | --- |
| Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level | Tentative agreements: Candidate options: Based on the 1st round collection, following options are down selected, please companies check if option 2 or 3 could be a middle ground to compromise. - Option 1 (CATT, MTK, Apple, QC, vivo, xiaomi, ZTE, OPPO, HW, LGE, Intel): based on slot level - Option 2 (Intel): - based on symbol level for scenario 1. - based on slot level for scenario 2. - Option 3 (CMCC, QC, HW, Intel): - FFS for scenario 1. - based on slot level for scenario 2. - Option 4 (Ericsson, Nokia): - based on slot level for async cases. - based on symbol level for sync cases. Recommendations for 2nd round: Continue discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-4-2: Antenna switching time | Tentative agreements: Candidate options: - Option 1 (LGE): The antenna switching time (15us) should be applied only when the symbol before or after SRS transmission occasion is uplink symbol since the switching time is for Tx-to-Tx - Option 2 (Apple, HW, Xiaomi, MTK, vivo, CATT, QC): generic requirement is preferred and no need to consider option 1. Recommendations for 2nd round: Please companies to check how this discussion could help to define the requirement, since interruption itself is defined from one CC to another CC. Continue discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-4-3: Interruption requirement proposals for scenario 1 | Tentative agreements: Candidate options: Based on the 1st round collection, following options are down selected, - Option 1 (Apple, QC, xiaomi, HW, OPPO, MTK, Intel, vivo(async), CATT(async), LGE): The interruption requirement of SRS antenna port switching for scenario 1 is 2 slots of victim carrier’s SCS, Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15 (NR or LTE); 2; 2; 2. 30; 2; 2; 2. 60; 2; 2; 2. 120; 2; 2; 2. Unit of interruption requirement is slot for NR and subframe for LTE. - Option 1a (LGE): Note 1: In TDD synchronous case, if the slot after SRS resources for antenna port switching is a downlink slot, the corresponding downlink slot is excluded from the interruption requirements. Note 2: In TDD synchronous case, if the downlink symbols and SRS resources for antenna port switching are in the same slot, the downlink symbols are excluded from the interruption requirements. - Option 2 (Nokia): - For synchronous scenarios, when X=1 SRS symbol is configured in a slot for SRS antenna switching, the interruption length shall be defined as X1 in Table 1. - For asynchronous scenarios, when X=1 SRS symbol is configured in a slot for SRS antenna switching, the interruption length shall be defined as X1+1 in Table 1. Table 1: Interruption length X1 (symbols) NR Slot length(ms) of victim cell; NR Symbol length (us) of victim cell; Interruption time (us); Interruption length X1 (symbols). Sub carrier spacing for aggressor cell (kHz); Sub carrier spacing for aggressor cell (kHz). 15; 30; 15; 30. 0; 1; 71.35; 101.35; 65.68; 2; 2. 1; 0.5; 35.68; 101.35; 65.68; 4; 3. Recommendations for 2nd round: Continue discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-4-4: Interruption requirement proposals for scenario 2 | Tentative agreements: Candidate options: Based on the 1st round collection, following options are down selected, - Option 1 (MTK, Apple, QC, CMCC, xiaomi, HW, LGE, OPPO, Intel, vivo, CATT(async)): the interruption requirement of SRS antenna port switching is summarized as: Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15 (NR or LTE); 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. Unit of interruption requirement is slot for NR and subframe for LTE. - Option 1a (LGE): Note 1: In TDD synchronous case, if the slot after SRS resources for antenna port switching is a downlink slot, the corresponding downlink slot is excluded from the interruption requirements. Note 2: In TDD synchronous case, if the downlink symbols and SRS resources for antenna port switching are in the same slot, the downlink symbols are excluded from the interruption requirements. - Option 2 (Nokia): - For synchronous scenarios, the interruption length is defined as X2 in Table 2. - For asynchronous scenarios, the interruption length is defined as X2+1 in Table 2 Table 2: Interruption length X2 (symbols) NR Slot length(ms) of victim cell; NR Symbol length (us) of victim cell; Interruption time (us); Interruption length X2 (symbols). Sub carrier spacing for aggressor cell (kHz); Sub carrier spacing for aggressor cell (kHz). 15; 30; 15; 30. 0; 1; 71.35; 458.1; 244.08; 7; 4. 1; 0.5; 35.68; 458.1; 244.08; 14; 8. Recommendations for 2nd round: Continue discussion in 2nd round. Conclusions would be captured in the WF. |

1.2.5	Sub-topic 1-5: Others

|  | Status summary |
| --- | --- |
| Issue 1-5-1: Interruption requirement related with prioritization rule in RAN1 | Tentative agreements: Candidate options: - Option 1 (CATT, Apple, QC, HW, OPPO, Ericsson, ZTE, CMCC, Intel, Nokia, vivo): - RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - No need to discuss and clarify on the requirement applicability when SRS antenna switching is colliding with other transmission with higher priority defined in clause 6.2.1 of TS 38.214. - Option 2 (MTK, Nokia): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1, NR measurement and the potential conclusions on the collision rules for all reference signals, including CSI-IM, except DMRS and UCI containing CSF report. - Option 3 (vivo, OPPO): Interruption of SRS antenna switching is applicable only when the SRS transmission time and the guard period are not colliding with any other transmission in the same carrier with higher priority defined in clause 6.2.1 of TS 38.214. - Option 3a (vivo): - Note 1: No collision handling rule can be inferred from clause 6.2.1 of TS 38.214 for the case between DL carrier and UL carrier in NR CA, or between any two cross-CG carriers in DC. - Note 2: Interruption of SRS antenna switching is still applicable to UL CA no matter whether the collision handling case is specified in TS 38.214 or not. Recommendations for 2nd round: Please companies check if can compromise to option 1. Moderator understanding to MTK’s comment is: issue 1-2-3 could be discussed separately from this issue 1-5-1 (the yellow highlighted part is up to the conclusion of issue 1-2-3, so we may not need to mention it in this issue). Continue discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-5-2: Two SRS colliding on same symbol | Tentative agreements: Candidate options: - Option 1 (CATT, Apple, Ericsson, QC, HW, ZTE, Nokia, vivo): This is up to RAN1 discussion, and no need to discuss this case in RAN4. - Option 2 (MTK): For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol: - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation. - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any. Recommendations for 2nd round: Can MTK compromise to option 1? Moderator understanding to MTK’s comment is: RAN4 would not discuss it and leave it to RAN1(no RAN4 spec impact). Continue discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-5-3: Impacts from SRS antenna port switching enhancement in R17 FeMIMO | Tentative agreements: Candidate options: - Option 1 (Nokia): - The interruption requirement applies only if SRS resources are allowed to be configured in the last 6 OFDM symbols in a slot. (HW) - The interruption requirement does not apply if the SRS resources of a set in a slot are configured in non-consecutive manner. - No need to discuss option 1. (Apple, QC, LGE, Xiaomi, MTK, Intel, vivo, CATT) Recommendations for 2nd round: Due to the time left for this WI completion, can Nokia compromise to option 2? Continue discussion in 2nd round. Conclusions would be captured in the WF. |

1.2.6	Sub-topic 1-6: Feature list for SRS antenna port switching

|  | Status summary |
| --- | --- |
| Sub-topic 1-6: Feature list for SRS antenna port switching | Tentative agreements: Candidate options: - Option 1 (Apple, OPPO): - Introduce UE capability for SRS antenna port switching requirement in RAN4. - Option 2 (QC, HW, MTK, E///, CMCC, Nokia, vivo, CATT): - No need to introduce UE capability for SRS antenna port switching requirement in RAN4. - Option 3 (Intel): - No need to introduce UE capability for SRS antenna port switching requirement in RAN4, and - R15/R16 UE who supports SRS antenna port switching is not required to meet R17 SRS antenna port switching requirement. Recommendations for 2nd round: Based on the 1st round discussion, we added one more option as a compromise proposed by Intel. Please companies check if option 3 is agreeable or not. Continue discussion in 2nd round. Conclusions would be captured in the WF. |

## Discussion on 2nd round (if applicable)

In 2nd round companies could discuss directly on the WF, and all the comments recorded in WF would be moved to this summary after 2nd round.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching | Apple | Wayfoward |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200069 | Interruption requirement to LTE serving cell, and impacts to other LTE RRM requirement | CATT | Postponed | Companies commented to come back next meeting. |
| R4-2201379 | Draft CR on Interruption requirement to NR serving cell, and impacts to other NR RRM requirement (if applicable) | Ericsson | Postponed | Companies commented to come back next meeting. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Apple | Jie Cui | Jie_cui@apple.com |
| MediaTek | ChihKai Yang | ck.yang@mediatek.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)