3GPP TSG-RAN WG4 Meeting # 101-bis-e 													R4-2202558

Electronic Meeting, 17th – 25th January 2022

Agenda item:			6.10.2.2

Source:	Moderator (vivo)

Title:	Email discussion summary for [101-bis-e][207] NR_RRM_enh2_2

Document for:	Information

# Introduction

This email discussion summary covers topic HO with PSCell under agenda 6.10.2.2.

# Topic #1: HO with PSCell

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200070 | CATT | Proposal 1: Prefer to adopt option 1 i.e. UE use the SMTC configured by PCell if both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell. Proposal 2: The Tprocessing for HO with PSCell for both parallel processing cases and sequential processing cases can be defined as one value that equals to max of Tprocessing for PCell handover and for PSCell addition/change. Proposal 3: The delay requirements for HO with PSCell are defined as PCell handover delay requirements and PScell addition delay requirements respectively. Proposal 4: The delay requirements for PScell addition are defined with parameter of o×TPCell_timing, and defining o = 0 for parallel cases and o = 1 for sequential cases. Proposal 5: No interruption requirement on PCell should be defined during HO with PSCell. Proposal 6: The requirements defined for handover with PSCell will be applied both 2-step RA and 4-step RA. No need to mention it in the specification. Proposal 7: The requirement for handover with PSCell will be defined for no collision of PSCell PRACH with PCell PRACH, and adding clarification that additional uncertainty delay can be expected for this case. Proposal 8: Do not consider CSI-RS based CFRA for HO with PSCell in this WI. Proposal 9: Support option 1 in WF[1], i.e. postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band. |
| R4-2200289 | Apple | Proposal 1: detailed requirement of HO with PSCell for NR SA to EN-DC could be determined after receiving the reply LS from RAN2. Proposal 2: for HO with PSCell for NR-DC to NR-DC and for EN-DC to EN-DC, if both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, adopt either ‘using the SMTC in the MO configured by source PCell’ or ‘up to UE implementation’. Proposal 3: Timeline of Tprocessing (UE SW processing and RF warm-up(if needed) time) for HO with PSCell is: - For parallel processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. - For sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in sequential. Proposal 4: - For parallel processing for HO with PSCell, Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) - For sequential processing for HO with PSCell, Tprocessing for HO with PSCell = sum(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) Proposal 5: the UE processing time for HO with PSCell is: UE processing margin (Tprocessing); Target PCell and PSCell is in the same FR as old serving cell; Target PCell and/or target PSCell is in the different FR from old serving cell. Sequential processing; 40ms; 60ms. Parallel processing; 20ms; 40ms. Proposal 6: regarding issue 2-3-2b, interruption requirements on PCell/PSCell due to PSCell/PCell RF retuning is: - If sequential processing is used for HO with PSCell, UE may have an interruption on new PCell due to the PSCell addition. - If parallel processing is used for HO with PSCell, no need to define interruption requirement. Proposal 7: No need to mention 2-step RA or 4-step RA in the requirement of HO with PSCell. Proposal 8: When UE is performing HO with PSCell, - for FR1+FR1 EN-DC, adding clarification that additional uncertainty delay can be expected for PSCell RACH collision with PCell UL channels if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1; - for FR1+FR1 NE-DC, adding clarification that additional uncertainty delay can be expected for PCell RACH collision with PSCell RACH if the PCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.2; - otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider RO collision issue. Proposal 9: - Not define detailed requirement for HO with PSCell when CSI-RS based CFRA is used, but only clarify in spec that longer delay would be expected for HO with PSCell when CSI-RS based CFRA is used. |
| R4-2200290 | Apple | Draft CR on HO with PSCell for NR SA to EN-DC_R17 |
| R4-2200416 | Qualcomm Incorporated | Proposal 1: In the case of NR-DC to NR-DC and EN-DC to EN-DC, it is up to UE implementation to utilize MOs from source Pcell and source PSCell. UE can use the larger SMTC in the configured MO to relax the UE processing time. Proposal 2: Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [5] ms. |
| R4-2200532 | Intel Corporation | Proposal 1: For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. Proposal 2: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel, Tprocessing applies independently for PCell and PSCell, it’s FFS whether any margin is needed. Proposal 3: The requirement for handover with PSCell will be defined for no collision of PSCell PRACH with PCell PRACH, and adding clarification that additional uncertainty delay can be expected for this case. Proposal 4: The delay requirements for HO with PSCell for NR-DC can be described as: - THO = TRRC_delay + Tsearch_PCell + Tprocessing + TIU + T∆_PCell + Tmargin - TPSCell= TRRC_delay + a*(Tsearch_PCell + T∆_PCell + Tmargin) + Tsearch_PSCell + T∆_PSCell + Tprocessing +TPSCell_ DU + Tmargin ms Where a=1 if targetCellSMTC-SCG is configured but not configured in reconfigurationWithSync; a = 0 otherwise. |
| R4-2200598 | vivo | Observation 1 For the sequential case, there is enough room for sequential processing of PSCell RF warm-up and PCell RF warm-up, if parallel processing of PCell cell search and PSCell RF warm-up can be assumed. Proposal 1 For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are assumed to be performed in parallel. No interruption on PCell will be introduced due to PSCell RF retuning. Proposal 2 Tprocessing for HO with PSCell is max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [X] ms, and X can be different for different HO with PSCell scenarios. Proposal 3 For NR-DC to NR-DC and EN-DC to EN-DC, Tprocessing for PCell HO is 20ms, and Tprocessing for PSCell change/addition is 20ms for the intra-FR case, and 40ms for the inter-FR case. Proposal 4 For NE-DC to NE-DC, Tprocessing for PCell HO is 20ms, and Tprocessing for PSCell change/addition is also 20ms. Proposal 5 For NRSA to EN-DC, Tprocessing for PCell HO is 50ms, and Tprocessing for PSCell addition is 20ms if NR PSCell in FR1, and is 40ms if NR PSCell is in FR2. Proposal 6 Clarify in the spec that if both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, it is up to UE to decide which SMTC in the configured MOs is used. Observation 2 For NR-SA to EN-DC, based on R16 RAN2 signaling, UE can only perform sequential processing if SMTC of target unknown PSCell is configured the HO command. Proposal 7 In R17, RAN4 will further discuss and conclude the feasibility and necessity of UE parallel processing in HO with PSCell from NR-SA to EN-DC for the case when network can provide SMTC of target unknown PSCell outside the container obtained from target E-UTRAN PCell, and if needed, send LS to RAN2 asking for the corresponding signalling design. Proposal 8 In R17, for HO with PSCell from NR-SA to EN-DC, RAN4 work on RRM requirements firstly assuming - parallel processing for the case when target PSCell is known, and - sequential processing for the case when SMTC of target unknown PSCell is provided to UE in the container obtained from target E-UTRAN PCell, and - parallel processing for the case when SMTC of target unknown PSCell is obtained by UE from the MOs of source PCell Proposal 9 If UE assumes 5ms SSB periodicity for the target PSCell by default, parallel processing is assumed. RAN4 may further discuss whether to allow larger X in Tprocessing if the SSB periodicity for either PCell HO or PSCell change is not more than 5ms. Proposal 10 On PRACH collision handling for the parallel processing, - If target cells are in FR1+FR1 EN-DC, an additional uncertainty delay due to PSCell PRACH collision with PCell UL channels may be introduced if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1; - If target cells are in FR1+FR1 NE-DC, an additional uncertainty delay due to PCell PRACH collision with PSCell UL channels may be introduced if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1A; - Otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider PRACH collision issue. Proposal 11 RAN4 concludes whether to specify one delay requirement for PCell HO after the impacts from PSCell addition to PCell HO are all clear. Proposal 12 No new UE capability is introduced for sequential processing in HO with PSCell. UE should be mandatory to support both parallel processing and sequential processing if it supports HO with PSCell. Proposal 13 RAN4 include both 2-step RA and 4-step RA into the new requirements made for handover with PSCell. No need to mention 2-step or 4-step in HO with PSCell requirements. Proposal 14 CSI-RS based CFRA is deprioritized in the discussion of HO with PSCell in R17 WI. |
| R4-2200630 | CMCC | Proposal 1: if both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE either to use the SMTC in the MO configured by PCell or up to UE implementation Proposal 2: for both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. Proposal 3: if UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel, Tprocessing is applied independently for PCell and PSCell (20ms or 40ms depending on whether target is same or different FR). Proposal 4: for the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately: PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing + T∆ + Tmargin ms |
| R4-2200673 | Xiaomi | Proposal 1: The timeline of UE SW processing and RF warm-up for PCell HO and PSCell addition/change is defined as follows: - For parallel processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. - For sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in sequential. Proposal 2: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel, the Tprocessing for HO with PSCell is defined as follows: Tprocessing = max(Tprocessing_PCell HO, Tprocessing_PSCell addition/change) + 5 ms Proposal 3: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in sequential, the Tprocessing for HO with PSCell is defined as follows:  Tprocessing = Tprocessing_PCell HO + Tprocessing_PSCell addition/change Proposal 4: If the SMTC of target unknown PSCell is not configured, and if both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE use the SMTC in the MO configured by the source PCell. Proposal 5: Interruption requirements on PCell due to PSCell RF retuning are defined as follows: - If sequential processing is used for HO with PSCell, UE may have interruption on new PCell due to PSCell RF retuning. - If parallel processing is used for HO with PSCell, no need to define interruption requirement. Proposal 6: RAN4 not to mention 2-step RA or 4-step RA in the requirement of HO with PSCell. Proposal 7: The requirement for handover with PSCell is defined without considering additional uncertainty delay. Proposal 8: RAN4 add the clarification in TS38.133 that additional uncertainty delay for HO with PSCell can be expected when PCell RACH or PSCell RACH cannot be transmit based on the criteria in TS38.213. Proposal 9: RAN4 not to consider CSI-RS based CFRA for handover with PSCell in Rel-17. |
| R4-2200740 | ZTE Corporation | Proposal 1: For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. Proposal 2: No additional interruption requirements should be defined during HO with PSCell. Proposal 3: No need to mention 2-step RA or 4-step RA in the requirement of HO with PSCell. Proposal 4: The requirement for handover with PSCell will be defined for no collision of PSCell PRACH with PCell PRACH, and adding clarification that additional uncertainty delay can be expected for this case. |
| R4-2201135 | OPPO | Proposal 1: If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE use the SMTC in the MO configured by PCell. Proposal 2: For parallel processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. Proposal 3: For sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in sequential. - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) Proposal 4: If sequential processing is used for HO with PSCell, UE may have an interruption on new PCell due to the PSCell addition. |
| R4-2201202 | Huawei, Hisilicon | Proposal 1: If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE use the SMTC in the MO configured by PCell or up to UE implementation. Observation 1: It is clearer to discuss the exact value of Tprocessing in case by case manner. Proposal 2: Define Tprocessing for HO with PSCell according to Table I. Table I. Tprocessing_HO and Tprocessing_PSCell for HO with PSCell Scenarios; Source PCell; Target PCell; Source PSCell; Target PSCell; Tprocessing_HO; Tprocessing_PSCell. NR SA to EN-DC; FR1; LTE; NA; FR1; 20ms; 20ms. FR1; LTE; NA; FR2; 40ms; 40ms. FR2; LTE; NA; FR1; 40ms; 40ms. FR2; LTE; NA; FR2; 40ms; 40ms. EN-DC to EN-DC; LTE; FR1; LTE; FR1; 20ms; 20ms. LTE; FR1; LTE; FR2; 40ms; 40ms. LTE; FR2; LTE; FR1; 40ms; 40ms. LTE; FR2; LTE; FR2; 20ms; 20ms. NE-DC to NE-DC; FR1; LTE; FR1; LTE; 20ms; 20ms. NR-DC to NR-DC; FR1; FR2; FR1; FR2; 20ms; 20ms. Observation 2: According to RAN2 agreement, UE applies the PSCell SMTC configuration based on the timing reference of target EUTRA PCell for the case of NR SA to EN-DC HO with PSCell addition. Proposal 3: No need to define interruption requirements on target PCell due to PSCell addition/change. Proposal 4: for EN-DC, an additional uncertainty delay due to PSCell RACH collision with PCell UL channels may be introduced if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1; for NE-DC, an additional uncertainty delay due to PCell RACH collision with PSCell RACH may be introduced if the PCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.2; otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider RO collision issue. |
| R4-2201203 | Huawei, Hisilicon | Draft CR on requirements for HO with PSCell from EN-DC to EN-DC |
| R4-2201380 | Ericsson | Proposal 1: UE shall be capable of software processing and RF warm-up in parallel for PCell and PSCell regardless whether other activities (e.g., cell detection) are carried out in parallel or in sequence. Proposal 2: Tprocessing should be the maximum value between UE processing timing of PCell HO and UE processing timing of PSCell addition/change. Tprocessing for HO with PSCell = max (Tprocessing for PCell HO, Tprocessing for PSCell addition/change) Proposal 3: RAN4 to agree that no additional interruption requirements on PCell should be defined during HO with PSCell due to sequential RF retuning of PSCell. Proposal 4: RAN4 to agree that the requirements defined for handover with PSCell will be applied both 2-step RA and 4-step RA. No need to mention it in the specification. Proposal 5: Requirements for HO with PSCell when PSCell is on NR-U shall be discussed in parallel with licensed carrier requirements and be specified in Rel-17. Proposal 6: Tsearch + T∆ + Tmargin for PCell HO and PSCell addition/change, when PSCell is on unlicensed carrier can be performed in sequence. Proposal 7: For EN-DC to EN-DC HO with PSCell with PSCell under CCA, the delay requirement shall be specified separately for PCell and PSCell and shall use ending points as PRACH preamble transmission in PCell and PSCell, respectively. Proposal 8: For NR PSCell change with target NR PSCell under CCA (band n46), the NR PSCell addition requirement in TS 36.133 clause 7.31A.2 can be used as baseline, with the following modification: - Tprocessing = 20ms when source and target cells are in same FR - Tprocessing = 40ms when source and target cells are in different FRs Proposal 9: When PSCC is under CCA, if UE is incapable of simultaneous PRACH preamble transmission in PCell and PSCell, and RACH occasions in PCell and PSCell collide, then UE shall prioritize PRACH preamble transmission on the carrier with CCA. An additional uncertainty term or redefinition of TIU is introduced for the leg without CCA. |
| R4-2201381 | Ericsson | Drfat CR on HO with PSCell requirements for NE DC to NE-DC |
| R4-2201542 | Nokia, Nokia Shanghai Bell | - In HO with PSCell for NR-DC to NR-DC, parallel processing delay requirements which will reuse legacy HO and PSCell addition will fulfill the delay in this specific case when SMTC of target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. - HO with PSCell RRM requirements can refer to existing handover requirements and PSCell addition requirements directly. Define the delay requirements for HO with PSCell for NR-DC to NR-DC as reusing current HO and PSCell addition/change requirements directly for both parallel processing and sequential processing. No additional interruption should be defined during HO with PSCell due to PSCell RF retuning. Both 2-step RA and 4-step RA are applicable for HO with PSCell and no need to mention 2-step or 4-step in HO with PSCell requirements. For EN-DC and NR-DC, the requirement for handover with PSCell will be defined for no collision of PSCell PRACH with PCell PRACH, and adding clarification that additional uncertainty delay can be expected for the case of RACH occasion collision between PCell and PSCell. Follow the same assumption as legacy HO requirements and do not need to discuss CSI-RS based CFRA in this WI. |
| R4-2201543 | Nokia, Nokia Shanghai Bell | dratCR on HO with PSCell |
| R4-2201851 | MediaTek Inc. | Proposal 1: If both source PCell and source PSCell have configured the UE with MOs, it is up to UE implementation to use the SMTC of which MO. Proposal 2: For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change can be performed in parallel. Proposal 3: The overall Tprocessing for HO with PSCell should be max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) +10ms . Proposal 4: Whether interruption requirements on PCell/PSCell due to PSCell/PCell RF retuning are needed depend on RAN4 conclusion on whether SW processing and RF warm-up can be performed in parallel. Proposal 5: No need to mention 2-step RA or 4-step RA in the requirement of HO with PSCell. Proposal 6: Do not consider CSI-RS based CFRA for handover with PSCell in this WI. Proposal 7: Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 Scenarios for RRM requirement of HO with PSCell

Sub-topic description:

There is no open issue for this sub-topic.

### Sub-topic 2-2 Delay requirement design of HO with PSCell

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-2-2c-1: If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE use the SMTC in the MO

The issue is related to scenarios of HO with PSCell for NR-DC to NR-DC and for EN-DC to EN-DC.

The principle may also be applied to HO with PSCell for NR-SA to EN-DC, depending on further RAN2 reply.

- Proposals

  - Option 1 (Apple, CATT, CMCC, Xiaomi, OPPO, Huawei)

    - Configured by PCell

  - Option 2 (Apple, Qualcomm, CMCC, vivo, Huawei, MediaTek)

    - Up to UE implementation

- Recommended WF

  - Agree on option 2

Note: It seems difficult to coverage and not so meaningful for open discussions. Moderator would like to check if option 2 is agreeable to all companies.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| CMCC | OK with recommended WF. |
| Apple | Either option is fine to us. |
| Qualcomm | Agree with recommended WF. |
| OPPO | Agree with recommended WF. |
| Xiaomi | Fine with option 2. |
| ZTE | Agree with the recommended WF. |
| Huawei | We prefer option 1. But also fine with option 2 as suggested by moderator, then it means requirements are considered based on the longer SMTC period. |
| Intel | Agree with recommended WF. |
| Ericsson | Either options are OK with us |
| vivo | Agree with recommended WF. |
| Nokia | We have different view. we think UE should use the shortest SMTC in the MOs. Option 3, the shortest SMTC in the configured MOs |
| MediaTek | Option 2 is agreeable from our view, this option can also follow the specs defined for PSCell change in clause 8.11 of 38.133: “If the SMTC periodicity of the target cell is not provided within the PSCell change message, and measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation.” |
| CATT | Fine with the recommended WF. |

Issue 2-2-3a: Timeline of Tprocessing (UE SW processing and RF warm-up(if needed) time) for HO with PSCell

- Proposals

  - Option 1: (CATT, Intel, vivo, ZTE, Ericsson, MTK, Intel, vivo, Nokia)

    - For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel.

  - Option 2: (Apple, Xiaomi, OPPO)

    - For parallel processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel.

    - For sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in sequential.

- Recommended WF

  - For parallel processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel.

  - FFS for sequential processing cases

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| CMCC | Option 1. For parallel processing case, there is no doubt that UE SW processing and RF warm-up for PCell handover and PSCell addition/change will be performed in parallel. For sequential case, considering the timing of target PSCell is based on the timing of the target PCell, it was agreed that sequential processing is used for cell search, SSB processing margin and fine time/frequency tracking and acquiring timing information for PCell. But for Tprocessing, since it is about UE SW processing and RF warm-up, which is not related with timing reference, they can be performed in parallel for PCell handover and PSCell addition/change. |
| Apple | Support option 2. We can also agree with moderator suggestion to conclude on parallel case first. For sequential processing, we don’t see the strong justification to mandate UE to tune RF for both PCell HO and PSCell addition/change simultaneously, and as we discussed in our paper, in this case UE would just waste power if the RF is tuned to cover PSCC during the PCell HO but UE cannot do any DL synchronization on that PSCell. |
| Qualcomm | We support Option1. UE SW processing and RF warm-up can be performed in parallel regardless of sequential or parallel processing for cell search. |
| OPPO | Support option 2. Both sequential and parallel processing could be possible UE implementation. We should not restrict the requirements for different UEs. We also agree with Apple’s comments that UE maynot do any DL synchronization on that PSCell during the PCell HO. |
| Xiaomi | Support option 2, for sequential processing cases, from UE perspective, all the possible UE implementation should be allowed for UE SW processing and RF warm-up operations. UE may operate UE SW processing and RF warm-up for PSCell addition/change after the timing of target PCell is ready which is beneficial for power saving. |
| ZTE | We support option 1. UE SW processing and RF warm-up for PCell handover and PSCell addition/change could be performed in parallel. |
| Huawei | We are wandering what is the impact on spec if agreeing on above proposal. From our understanding, components contained in Tprocessing are implementation specific. It is only the assumption to discuss the value, we suggest to discuss the value directly. |
| Intel | Support option 1. For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed independently and in parallel. |
| Ericsson | We support option 1. We are OK with Huawei suggestion too. |
| vivo | We support option 1 and the recommended WF. For sequential case, we agree with the proponents on the power consumption part. We share the same concern with Huawei on the potential spec impact for this discussion, especially if we need to consider X as margin. Therefore, we prefer to continue discussion based on the recommended WF. |
| Nokia | We support option 1. UE SW processing and RF warm-up for PCell HO and PSCell addition/change can be performed in parallel regardless of sequential or parallel processing. |
| MTK | We support option 1, where UE SW processing and RF warm-up for PCell handover and PSCell addition/change can be performed in parallel for both parallel and sequential processing. We can also agree with the WF recommended by the moderator. |
| CATT | Support option 1. |

Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel

There are proposals that the processing time would be better to be discussed for different scenarios in a tabulated form. Moderator still thinks it should be simpler agreeing on high level principle by taking previous agreements/common understanding into consideration. There final processing time can be derived by taking all relevant agreements into account.

In the previous two meetings, following common understanding is reached regarding processing timing for legacy PCell handover and PSCell addition/change.

- Tprocessing for PCell HO

  - Reuse the Tprocessing defined for legacy PCell HO

    - 20ms, when source and target cells are in the same FR

    - 40ms, when source and target cells are in different FRs

- Tprocessing for PSCell change for NR-DC and EN-DC

  - 20ms, when source and target cells are in the same FR

  - 40ms, when source and target cells are in different FRs

- Tprocessing for PSCell addition for NR-DC and EN-DC

  - 20ms, when NR PSCell is in the same FR as PCell

  - 40ms, when NR PSCell is in the different FR from PCell

- Proposals

  - Option 1: (CATT, Apple, OPPO, Ericsson)

    - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change)

  - Option 1a: (Qualcomm, vivo, Xiaomi, MTK)

    - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [X] ms.

    - X=5 (Qualcomm, Xiaomi)

    - X=10 (MTK)

    - X=FFS and can be different for different HO with PSCell scenarios (vivo)

  - Option 2a: (Intel, CMCC)

    - Tprocessing applies independently for PCell and PSCell.

    - FFS whether margin is needed

- Recommended WF

  - Further discussion in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| CMCC | Option 2a. For parallel processing case, it was agreed to define delay requirements for HO and PSCell addition/change separately with the ending points defined as PCell PRACH and PSCell PRACH respectively, which, in our understanding, means that PSCell addition delay and HO delay are specified separately and independently. With this understanding, at least for parallel processing case, Tprocessing applies independently for PCell and PSCell, and no additional margin is needed. |
| Apple | We support option 1 and can compromise to option 1a with X=10. |
| Qualcomm | We support option1a X=5ms/10ms. Simple requirements need to be defined rather than define the requirements per cases. |
| OPPO | Option 1 and can compromise to option 1a with X=5/10ms. At least we can firstly agree on the majority view by defining joint Tprocessing for HO with PSCell. |
| Xiaomi | Option 1a, additional margin should be considered for the UE SW processing and RF warm-up in parallel, and we think 5ms is a proper value. |
| ZTE | Support option 1. |
| Huawei | Support option 1. |
| Intel | Support option 2a at least for parallel processing. For this issue, the assumption is that UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel, there is no dependency between PCell and PSCell processing. In last meeting, RAN4 agreed to define delay requirements for HO and PSCell addition/change separately, the agreement is as follows: Issue 2-2-5: Ending point of the delay requirement for HO with PSCell Defining delay requirements for HO and PSCell addition/change separately with the ending points defined as PCell PRACH and PSCell PRACH respectively.. Therefore, we don’t need to define a unified Tprocessing for HO with PSCell. Tprocessing for PCell and PSCell can be used in each requirement respectively. We are also open to define extra margin for Tprocessing, e.g. 5 or 10ms. By the way, the issue is also related to Issue 2-2-8a, it seems that many companies prefer option 3. In option 3, Tprocessing is defined for PCell and PSCell respectively in parallel processing. We don’t know what’s the assumption here and why a combined value is introduced. |
| Ericsson | Support option 1 |
| vivo | Support option 1a. X can be FFS. |
| Nokia | We have agreed the Tprocessing each for PCell HO and PSCell addition/change in previous meeting, Since UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel, Tprocessing applies independently for PCell HO and PSCell addition/change with no margin and can reuse the Tprocessing in legacy requirements directly. |
| MTK | Support option 1a. If the UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel, the overall processing time needs to choose the maximum value between PCell HO and PSCell addition/change plus an additional margin of 10 ms. This margin is introduced to consider worst-case scenario when some RF components or SW resources might be shared between PCell and PSCell. Therefore, we support option 1a with X=10ms. |
| CATT | Support option 1. No additional margin is needed. |

Issue 2-2-3c: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in sequential

It further depends on conclusion of Issue 2-2-3a whether this is needed or not.

- Proposals

  - Option 1: (Apple, Xiaomi)

    - Tprocessing for HO with PSCell = sum (Tprocessing for PCell HO, Tprocessing for PSCell addition/change)

- Recommended WF

  - Hold on discussion in the 1st round.

Issue 2-2-3d: Processing timeline for NR SA to EN-DC

- Proposals

  - Option 1 (vivo)

    - In R17, RAN4 will further discuss and conclude the feasibility and necessity of UE parallel processing in HO with PSCell from NR-SA to EN-DC for the case when network can provide SMTC of target unknown PSCell outside the container obtained from target E-UTRAN PCell, and if needed, send LS to RAN2 asking for the corresponding signalling design.

    - In R17, for HO with PSCell from NR-SA to EN-DC, RAN4 work on RRM requirements firstly assuming

    - parallel processing for the case when target PSCell is known, and

    - sequential processing for the case when SMTC of target unknown PSCell is provided to UE in the container obtained from target E-UTRAN PCell, and

    - parallel processing for the case when SMTC of target unknown PSCell is obtained by UE from the MOs of source PCell

- Recommended WF

  - Further discussion in the 1st round

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We suggest waiting the LS reply from RAN2 on this issue, and we don’t think it’s reasonable to extend the signaling design to cover a new case where PSCell configuration is outside the container (in our understanding it not a valid option in RAN2 but we can check in the reply LS). Regarding the current RAN4 work, in our previous paper, we think: - sequential processing is used for the case when SMTC of target unknown PSCell is provided to UE in the container obtained from target E-UTRAN PCell - otherwise, parallel processing is used (all the other cases shall assume parallel processing since UE doesn’t need to wait for new LTE PCell timing acquisition) |
| Qualcomm | Per RAN2 agreements, sequential processing shall be used for NR SA to EN-DC. RAN2 confirms that UE applies the PSCell SMTC configuration based on the timing reference of target EUTRA PCell for the case of NR SA to EN-DC HO with PSCell addition (if explicit SMTC configuration is present in RRCConnectionReconfiguration).- RAN2#116-e, [010][NR16] connection Control |
| Huawei | For the first bullet, it is possible for parallel processing if UE obtains the SMTC in MO. So we didn’t see the strong need to add another signaling for SMTC configuration. And I think it is similar to the case of NR-DC. |
| vivo | It is OK to FFS for this issue until the 2nd round. To Qualcomm, RAN2 agreements was only for R16, and if RAN4 can reach consensus, we still see some signaling impact is possible in R17. For NR-SA to EN-DC, the SMTC configuration and the SMTC used by UE are: The main issue is that: - Even if network knows the timing reference of the target PCell, it may miss the chance to provide UE such information via MO configuration before the HO command take place. - Network has no information whether the target PSCell is known or unknown from UE perspective, therefore, it is difficult for network to differentiate between Row1 and Row 2 or 3 in above table. Therefore, we think the necessity of new signaling can be justified. To Huawei, yes, it is similar to the case of NR-DC, and we do not think anything is new here. |
| Nokia | LS has sent to RAN2 in last RAN4 meeting. We can wait the response from RAN2. |
| MTK | For NR SA to EN-DC scenario, we think the processing timeline can be based on the following: - If the SMTC of the target PSCell is configured in HO command: - UE follows the SMTC in the HO command, which, in this case, is based on the timing reference of target E-UTRAN PCell. Therefore, sequential processing should be performed to get the target PCell timing first. - If the SMTC of the target PSCell is not configured in HO command: - If UE is configured with source PCell MO: - UE follows the SMTC in this MO. Therefore, parallel processing can be performed. - If UE is not configured with source PCell MO: - UE assumes SSB has 5ms periodicity. Therefore, parallel processing can be performed. |

Issue 2-2-8a: How the HO with PSCell delay requirements are specified

- Proposals

  - Option 1: (Intel)

    - The delay requirements for HO with PSCell for NR-DC can be described as:

    - THO = TRRC_delay + Tsearch_PCell + Tprocessing + TIU  + T∆_PCell + Tmargin

    - TPSCell= TRRC_delay + a*(Tsearch_PCell + T∆_PCell + Tmargin) + Tsearch_PSCell + T∆_PSCell + Tprocessing +TPSCell_ DU + Tmargin ms

      - Where a=1 if targetCellSMTC-SCG is configured but not configured in reconfigurationWithSync; a = 0 otherwise.

  - Option 2: (CATT)

    - The delay requirements for HO with PSCell are defined as PCell handover delay requirements and PScell addition delay requirements respectively.

    - The delay requirements for PScell addition are defined with parameter of o×TPCell_timing, and defining o = 0 for parallel cases and o = 1 for sequential cases.

  - Option 3: (CMCC)

    - For the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately:

    - PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

    - HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

  - Option 4: (Nokia)

    - HO with PSCell RRM requirements can refer to existing handover requirements and PSCell addition/change requirements directly.

    - Define the delay requirements for HO with PSCell for NR-DC to NR-DC as reusing current HO and PSCell addition/change requirements directly for both parallel processing and sequential processing.

  - Option 5: (vivo)

  - RAN4 concludes whether to specify one delay requirement for PCell HO after the impacts from PSCell addition to PCell HO are all clear.

- Recommended WF

  - Further discussion in the 1st round

Moderator thinks it would be helpful for CR structure if high level principle can be agreed. It can also be totally up to CR discussion. Company’s views are welcome.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| CMCC | Option 3. For parallel processing case, it was agreed to define delay requirements for HO and PSCell addition/change separately with the ending points defined as PCell PRACH and PSCell PRACH respectively, which, in our understanding, means that PSCell addition delay and HO delay are specified separately and independently. With this understanding, option 3 is proposed for the parallel processing case. |
| Apple | Agree with moderator, up to all above issues, we could agree on the CR structure first. Option 3 is a good starting point for parallel processing. |
| Qualcomm | For parallel processing: Option1, 3 For sequential processing: Option1. |
| OPPO | We can firstly agree on parallel processing case. Option 3 is fine. For sequential processing, it is related to other issues like 2-2-3c. We are ok to further discuss in 2nd round or CR discussion. |
| Xiaomi | For parallel processing case: Option 3 For sequential processing case: depend on the conclusion of issue 2-2-3c. |
| vivo | For parallel processing: Option1. Option 3 is the same. Moreover, TIU for the PCell Handover can be impacted by ‘a’ factor in option 1, since it is quite unlikely to consider PRACH collision for the sequential case, but quite likely to consider PRACH collision for the parallel case. For sequential processing: Option1. |
| Huawei | For CR structures, we suggest to define requirements taking following structures as exmaple: 5.x EN-DC Handover with PSCell 5.x.y.1 Handover with PSCell Interruption time 5.x.y.2 Handover with PSCell - NR PSCell Changing Delay requirements |
| Intel | Fine with Option 3. |
| Ericsson | Tprocessing is not agreed yet. If all components can be performed in parallel, option 3 is OK. If at least some are sequential, one equation may be considered. |
| Nokia | We support option 4. From the delay requirements point of view, Option 3 is similar to option 4. The difference is on the specification structure. We think that it would be helpful on the agreement of CR structure. From our view, since parallel processing is our baseline, and the delay requirements for parallel processing would be same as legacy PCell HO and PSCell addition/change requirements, we can refer to current legacy PCell HO and PSCell addition/change requirement directly to reduce the duplicated requirements in specification, then if extra delay is needed for specific cases like sequential processing, we can add the extra delay as incrementation based on the reference to legacy requirements. |
| MTK | Ok with option 1 to define the delay requirements for HO with PSCell for both parallel and sequential processing. |
| CATT | Support option 1 and option 2 which are the same in our view. |

### Sub-topic 2-3 Interruption requirement design of HO with PSCell

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-3-2b: Interruption requirements on PCell due to PSCell RF retuning

It further depends on conclusion of Issue 2-2-3a whether this is needed or not.

- Proposals

  - Option 1 (CATT, ZTE, Huawei, vivo, Ericsson, Nokia)

    - No additional interruption requirements should be defined during HO with PSCell

  - Option 2 (Apple, Xiaomi, OPPO)

    - If sequential processing is used for HO with PSCell, UE may have an interruption on new PCell due to the PSCell addition.

    - If parallel processing is used for HO with PSCell, no need to define interruption requirement.

  - Option 3 (MTK)

    - Whether interruption requirements on PCell/PSCell due to PSCell/PCell RF retuning are needed depend on RAN4 conclusion on whether SW processing and RF warm-up can be performed in parallel

- Recommended WF

  - Hold on discussion in the 1st round. RAN4 needs to conclude issue 2-2-3a firstly.

### Sub-topic 2-4 Generic RACH assumption for HO with PSCell

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-4-1: 2 step and 4 step RACH for HO with PSCell

- Proposals

  - Option 1a (CATT, vivo, Nokia, Ericsson):

    - The requirements defined for handover with PSCell will be applied both 2-step RA and 4-step RA. No need to mention it in the specification.

  - Option 1b (Apple, Xiaomi, ZTE, MTK):

    - No need to mention 2-step RA or 4-step RA in the requirement of HO with PSCell.

  - Option 2 (Apple, Qualcomm, OPPO, MTK):

    - For requirement of HO with PSCell, RAN4 starts the discussion with 4 step RACH first and FFS on 2 step RACH.

- Recommended WF

  - Further discussion in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1b or 2. We didn’t see big difference between option 1a and 1b if it’s not mentioned in the spec. |
| Qualcomm | We support Option2. Once the requirements are finalized with 4-step RACH then discuss about 2-step RACH. |
| Xiaomi | Option 1b or 2, In legacy handover requirement, 2-step RACH and 4-step RACH are not mentioned in the requirement. Similarly, we think there is no need to mention 2-step RACH or 4-step RACH in the requirement of HO with PSCell. Regarding whether 2-step RACH is supported for HO with PSCell is another issue, RAN4 can define the requirement for 4-step based HO with PSCell in this release, as 2-step RACH is introduced in Rel-16, and this WI is assumed to be enhanced based on Rel-15 features. |
| ZTE | We support option 1a or 1b. No big difference between these two options. |
| Intel | Prefer Option 1b. |
| Ericsson | Option 1a |
| vivo | Support option 1a but also OK to option 1b. |
| Nokia | We support option 1a, option 1b is also fine. |
| MTK | Support option 1b. No need to mention 2-step RA or 4-step RA in the requirement of HO with PSCell. |
| CATT | Support option 1a and 1b. |

Issue 2-4-2: RACH occasion collision between Pcell and PSCell

- Proposals

  - Option 1a (Apple, Huawei):

    - For FR1+FR1 EN-DC, adding clarification that additional uncertainty delay can be expected for PSCell RACH collision with PCell UL channels if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1;

    - For FR1+FR1 NE-DC, adding clarification that additional uncertainty delay can be expected for PCell RACH collision with PSCell RACH if the PCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.2;

    - Otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider RO collision issue.

  - Option 1b (vivo):

    - If target cells are in FR1+FR1 EN-DC, an additional uncertainty delay due to PSCell PRACH collision with PCell UL channels may be introduced if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1;

    - If target cells are in FR1+FR1 NE-DC, an additional uncertainty delay due to PCell PRACH collision with PSCell UL channels may be introduced if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1A;

    - Otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider PRACH collision issue.

  - Option 2a (Intel, CATT, ZTE, Nokia):

    - The requirement for handover with PSCell will be defined for no collision of PSCell PRACH with PCell PRACH, and adding clarification that additional uncertainty delay can be expected for this case.

  - Option 2b (Xiaomi):

    - The requirement for handover with PSCell is defined without considering additional uncertainty delay.

    - RAN4 add the clarification in TS38.133 that additional uncertainty delay for HO with PSCell can be expected when PCell RACH or PSCell RACH cannot be transmit based on the criteria in TS38.213.

- Recommended WF

  - Further discussion in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1a has typo about the section number of TS38.213. It shall be: - For FR1+FR1 EN-DC, adding clarification that additional uncertainty delay can be expected for PSCell RACH collision with PCell UL channels if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1; - For FR1+FR1 NE-DC, adding clarification that additional uncertainty delay can be expected for PCell RACH collision with PSCell RACH if the PCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1A; - Otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider RO collision issue. We didn’t see difference between option 1a and 1b, and we can support both. For option 2a, if the PCell and PSCell belongs to different FRs, the additional uncertainty delay is not needed and HO requirement shall still apply. |
| Qualcomm | Okay with option1a, 1b. |
| OPPO | Okay with option1a, 1b. |
| Xiaomi | Fine with option 1a or 1b |
| Huawei | Support option 1a/1b, which are the same. |
| Intel | Fine with option 2a, can compromise to option 1a or 1b to move forward. |
| vivo | 1b is just to prevent mis-interpretation, and make sure NR-SA to EN-DC is also counted in the first bullet. If companies are fine, we can go with 1b. |
| Nokia | We support option 2a and 1a. option 1a and option 2a are talking the same things, adding clarification that additional uncertainty delay can be expected for the specific case based on the criteria in TS 38.213. For option 1a, Could we change the wording as LTE+FR1 in EN-DC and FR1+LTE in NE-DC to make it a little clear? |
| MTK | Options 1a and 1b look similar. We are fine with either. |
| CATT | Support option 2a and compromise to option 1a to move forward. |

Issue 2-4-4: CSI-RS based CFRA

- Proposals

  - Option 1 (Apple):

    - Not define detailed requirement for HO with PSCell when CSI-RS based CFRA is used, but only clarify in spec that longer delay would be expected for HO with PSCell when CSI-RS based CFRA is used.

  - Option 2a (vivo):

    - CSI-RS based CFRA is deprioritized in the discussion of HO with PSCell in R17 WI.

  - Option 2b (Nokia):

    - Follow the same assumption as legacy HO requirements and do not need to discuss CSI-RS based CFRA

  - Option 2c (CATT, Xiaomi, MTK):

    - Do not consider CSI-RS based CFRA for handover with PSCell in this WI.

- Recommended WF

  - Check if option 2c is agreeable.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1 to clarify if such CSI-RS based CFRA is configured. |
| Qualcomm | Option 2a,2b, and 2c are okay. |
| Xiaomi | Option 2c, not consider CSI-RS based CFRA for handover with PSCell. |
| ZTE | Support option 2a. |
| vivo | Support recommended WF. Whether clarification is needed can be discussed in the CR phase. |
| Nokia | We support option 2b and 2c. |
| MTK | Our view is to focus on the SSB measurements as a baseline for HO with PSCell and not consider CSI-RS based CFRA. Therefore, we are fine with options 2c, 2b and 2a. |
| CATT | Support option 2c. |

### Sub-topic 2-5 Requirements for NR-U

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-5-1: Requirements for HO with PSCell for NR-U

There are proposals from one company on requirements for HO with PSCell for NR-U. It is moderator’s understanding that it is not time to discuss the requirements detail without agreements on whether or not the requirements should be introduced in Rel-17. Moderator would encourage companies to agree on whether or not to introduce NR-U requirements firstly.

- Proposals

  - Option 1 (CATT, MTK):

    - Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band.

  - Option 2 (Ericsson):

    - Requirements for HO with PSCell when PSCell is on NR-U shall be discussed in parallel with licensed carrier requirements and be specified in Rel-17

- Recommended WF

  - Further discussion in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1 or we may discuss it in R18 if RAN4 companies can have common interest on it. |
| Qualcomm | We support Option 1. |
| OPPO | Option 1. |
| ZTE | Support option 1. |
| Intel | Support option 1. |
| Ericsson | Option 2. For NR-U we need to only discuss the differences w.r.t licensed carrier for PSCell and we do not see the dependency of licensed carrier agreements on the differences part, and it can go on in parallel. Moreover, most of the issues for licensed carrier are sorted now. |
| vivo | Support option 1. We are also OK to discuss it in R18. |
| MTK | Support option 1. Our view is that to focus on the licensed band requirements for HO with PSCell as a baseline. Therefore, we support to postpone the requirement design of NR-U HO with PSCell until RAN4 completes the requirements for HO with PSCell on licensed band. |
| CATT | Support option 1. |

### Sub-topic 2-6 UE feature

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-6-1: UE feature group for HO with PSCell

- Proposals

  - Option 1 (vivo):

    - No new UE capability is introduced for sequential processing in HO with PSCell. UE should be mandatory to support both parallel processing and sequential processing if it supports HO with PSCell.

  - Option 2 (Apple [R4-2200286]):

    - Support of RRM requirements of handover with PSCell, including the following scenarios:

      - from NR SA to EN-DC

      - from EN-DC to EN-DC

      - from NE-DC to NE-DC

      - from NR-DC to NR-DC

  - Option 2a (Intel [R4-2200544]):

    - Support of Handover with PSCell RRM requirements

- Recommended WF

  - Further discussion in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Support Option 2 (UE needs to indicate if it can support HO with PSCell, and if yes, then that implies parallel/sequential processing is supported). |
| Qualcomm | Issue 2-2-3a should be finalized first. |
| Xiaomi | We are fine with option 1 and option2 |
| Huawei | We suggest not to have dedicated UE capability to indicate whether UE can meet requirements. From our understanding, parallel or sequential process are not different UE capability but assumption in discussion for different cases (e.g. whether target cell timing is needed as reference timing) |
| Intel | Support option 2 and 2a. |
| Ericsson | Similar view as Huawei. Sequential processing is agreed due to the fact that timing acquisition from PCell is needed for PSCell timing acquisition and not due to processing capability of UE. |
| vivo | We support option 1. For option 2, we think this is repeating discussion the same as RAN4 did in R16. Moreover, HO w PSCell is not mandatory feature, especially for UE not supporting DC band combinations. |
| Nokia | Similar view as Huawei and Ericsson, parallel processing or sequential processing for HO with PSCell are for different cases when discussing delay requirements but not related to UE capability. |
| MTK | Agree with option1. Our understanding is that as long as the UE feature is supported (in this case HO with PSCell) its requirements should be mandatory not optional. |
| CATT | Same view as Huawei. |

## Companies’ views collection for 1st round

### Open issues

Comments are collected in section 1.2.

### CRs/TPs comments collection

Companies are encouraged to provide comments on the structure and technical part of requirements.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200290 |  |
| R4-2201203 |  |
| R4-2201381 |  |
| R4-2201543 |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 2-2 Delay requirement design of HO with PSCell

|  | Status summary |
| --- | --- |
| Issue 2-2-2c-1: If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE use the SMTC in the MO | 12 companies support option 2. 1 company proposed new option 3, which was also discussed in the last meeting. Given the obvious majority view, moderator would like to propose option 2 as tentative agreement. Tentative agreements: - In HO with PSCell for NR-DC to NR-DC, if SMTC of target unknown PSCell is not configured in either targetcellSMTC-SCG-r16 or reconfigurationWithSync, - If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, it is up to UE implementation which SMTC in the MOs are used. - In HO with PSCell for EN-DC to EN-DC, if SMTC of target unknown PSCell is not configured in either targetcellSMTC-SCG-r16 or reconfigurationWithSync, - If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, it is up to UE implementation which SMTC in the MOs are used. Recommendations for 2nd round: Confirmation from Nokia is expected and comments are welcome if there are technical concerns on the tentative agreement. |
| Issue 2-2-3a: Timeline of Tprocessing (UE SW processing and RF warm-up(if needed) time) for HO with PSCell | 9 companies support option 1. 3 companies support option 2. 1 company suggest to discuss the Tprocessing value directly. However, it is not clear to moderator how values can be decided if high level principle cannot be agreed. Given this issue has been discussed for a long time and there are other issues depending on it, Moderator would like to suggest to treat this issue in GTW session. Candidate options: Issue 2-2-3a: Timeline of Tprocessing (UE SW processing and RF warm-up(if needed) time) for HO with PSCell - Proposals - Option 1: (CMCC, Qualcomm, ZTE, Intel, Ericsson, vivo, Nokia, MTK, CATT) - For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. - Option 2: (Apple, Xiaomi, OPPO) - For parallel processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. - For sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in sequential. Recommendations for 2nd round: Discuss in the GTW session. |
| Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel | Views from companies are diverse. Given this issue has been discussed for a long time and is very difficult to converge by email discussion, Moderator would like to suggest to treat this issue in GTW session. Candidate options: Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel - Proposals - Option 1: (CATT, Apple, OPPO, ZTE, Huawei, Ericsson) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) - Option 1a: (Qualcomm, vivo, Xiaomi, MTK, Apple, OPPO) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [X] ms. - X=5 (Qualcomm, Xiaomi, OPPO) - X=10 (MTK, Apple, Qualcomm) - X=FFS and can be different for different HO with PSCell scenarios (vivo) - Option 2a: (Intel, CMCC, [Nokia]) - Tprocessing applies independently for PCell and PSCell. - FFS whether margin is needed Recommendations for 2nd round: Discuss in the GTW session. |
| Issue 2-2-3d: Processing timeline for NR SA to EN-DC | Diverse views from companies. Candidate options: Issue 2-2-3d: Processing timeline for NR SA to EN-DC - Proposals - Option 1 (vivo) - In R17, RAN4 will further discuss and conclude the feasibility and necessity of UE parallel processing in HO with PSCell from NR-SA to EN-DC for the case when network can provide SMTC of target unknown PSCell outside the container obtained from target E-UTRAN PCell, and if needed, send LS to RAN2 asking for the corresponding signalling design. - In R17, for HO with PSCell from NR-SA to EN-DC, RAN4 work on RRM requirements firstly assuming - parallel processing for the case when target PSCell is known, and - sequential processing for the case when SMTC of target unknown PSCell is provided to UE in the container obtained from target E-UTRAN PCell, and - parallel processing for the case when SMTC of target unknown PSCell is obtained by UE from the MOs of source PCell - Option 2 (Apple) - sequential processing is used for the case when SMTC of target unknown PSCell is provided to UE in the container obtained from target E-UTRAN PCell - otherwise, parallel processing is used (all the other cases shall assume parallel processing since UE doesn’t need to wait for new LTE PCell timing acquisition) - Option 3 (Qualcomm) - sequential processing shall be used - Option 4 (MTK) - If the SMTC of the target PSCell is configured in HO command: - UE follows the SMTC in the HO command, which, in this case, is based on the timing reference of target E-UTRAN PCell. Therefore, sequential processing should be performed to get the target PCell timing first. - If the SMTC of the target PSCell is not configured in HO command: - If UE is configured with source PCell MO: - UE follows the SMTC in this MO. Therefore, parallel processing can be performed. - If UE is not configured with source PCell MO: - UE assumes SSB has 5ms periodicity. Therefore, parallel processing can be performed. - Option 5 (Apple, Nokia) - Wait for RAN2 reply Recommendations for 2nd round: Continue discussion in the 2nd round. |
| Issue 2-2-8a: How the HO with PSCell delay requirements are specified | Some companies think option 1 and option 3 are same (similar) when it comes to parallel processing. Proponent of option 1 is also fine with option 3. One company thinks option 4 is similar to option 3 but spec structure may be different. Only one company proposed CR structure. So, option 3 can be used as baseline for defining HO with PSCell delay requirements for parallel processing. The CR structure can be further discussed. No comments are collected for CRs in section 1.3.2. For sequential processing, it can be holding on and wait for outcome of issue 2-2-3a. Tentative agreements: - For the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately: - PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms - HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Recommendations for 2nd round: Confirm tentative agreements in the 2nd round. Collect comments for each CR, especially comments on CR structure. |

Sub-topic 2-3 Interruption requirement design of HO with PSCell

|  | Status summary |
| --- | --- |

Sub-topic 2-4 Generic RACH assumption for HO with PSCell

|  | Status summary |
| --- | --- |
| Issue 2-4-1: 2 step and 4 step RACH for HO with PSCell | 9 companies are fine with option 1a and/or 1b. 1 company supports option 2. There is obvious majority view. Moderator would like to propose option 1b as a compromised option with the condition it can be revisited if issue is identified in the future. Tentative agreements: - No need to mention 2-step RA or 4-step RA in the requirement of HO with PSCell. - It can be revisited if issue is identified. Recommendations for 2nd round: Check if tentative agreements are agreeable in the 2nd round. |
| Issue 2-4-2: RACH occasion collision between Pcell and PSCell | Majority companies think option 1a (with typo corrected) and option 1b are almost the same. 2 companies supporting option 2a are fine with option 1a. The term “FR1+FR1 EN-DC” and “FR1+FR1 NE-DC” may not be clear enough, at least to 2 companies. Thus, moderator would like to propose option 1a (with typo corrected) as tentative agreements to be further confirmed in the 2nd round. In addition, clarification on “FR1+FR1 EN-DC” and “FR1+FR1 NE-DC” can also be further discussed. Tentative agreements: - For RACH occasion collision between Pcell and PSCell, - For [FR1+FR1 EN-DC], adding clarification that additional uncertainty delay can be expected for PSCell RACH collision with PCell UL channels if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1; - For [FR1+FR1 NE-DC], adding clarification that additional uncertainty delay can be expected for PCell RACH collision with PSCell RACH if the PCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1A; - Otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider RO collision issue. Recommendations for 2nd round: Check if tentative agreements are agreeable in the 2nd round. Companies are encouraged to provide suggestions on how to improve the term [FR1+FR1 EN-DC] and [FR1+FR1 NE-DC]. |
| Issue 2-4-4: CSI-RS based CFRA | 6 companies are fine with option 2c. 1 company supports option 1. 1 company supports option 2a. Candidate options: Issue 2-4-4: CSI-RS based CFRA - Proposals - Option 1 (Apple): - Not define detailed requirement for HO with PSCell when CSI-RS based CFRA is used, but only clarify in spec that longer delay would be expected for HO with PSCell when CSI-RS based CFRA is used. - Option 2c (CATT, Xiaomi, MTK, Qualcomm, vivo, Nokia, ZTE?): - Do not consider CSI-RS based CFRA for handover with PSCell in this WI. Recommendations for 2nd round: Further discussion on the two options in the 2nd round. Apple is encouraged to check if option 2c is acceptable. |

Sub-topic 2-5 Requirement for NR-U

|  | Status summary |
| --- | --- |
| Issue 2-5-1: Requirements for HO with PSCell for NR-U | Majority view is to postpone the rrequirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band. 1 company thinks it can be discussed in parallel with requirements for licensed band. 2 companies think it can be discussed in Rel-18. Since this has been discussed for several meetings and one company has strong view to define HO with PSCell requirements for NR-U in Rel-17, moderator would like to suggest to treat this issue in the GTW session. Candidate options: Issue 2-5-1: Requirements for HO with PSCell for NR-U - Proposals - Option 1 (CATT, MTK, Apple, Qualcomm, ZTE, Intel, vivo): - Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band. - Option 2 (Ericsson): - Requirements for HO with PSCell when PSCell is on NR-U shall be discussed in parallel with licensed carrier requirements and be specified in Rel-17 - Option 3 (Apple, vivo): - Discuss requirements for NR-U in Rel-18. Recommendations for 2nd round: Further discuss in the GTW session. |

Sub-topic 2-6 UE feature

|  | Status summary |
| --- | --- |
|  | Based on comments, moderator added option 1a as one of the candidate options, which would be similar to option 1 in moderator’s understanding. Option 2a is combined to option 2 as proponent of option 2a also supports option 2. Candidate options: Issue 2-6-1: UE feature group for HO with PSCell - Proposals - Option 1 (vivo, Xiaomi, MTK): - No new UE capability is introduced for sequential processing in HO with PSCell. UE should be mandatory to support both parallel processing and sequential processing if it supports HO with PSCell. - Option 1a (Huawei, Ericsson, Nokia, CATT): - No dedicated UE capability to indicate whether UE can meet requirements - Option 2 (Apple, Intel, Xiaomi): - Support of RRM requirements of handover with PSCell, including the following scenarios: - from NR SA to EN-DC - from EN-DC to EN-DC - from NE-DC to NE-DC - from NR-DC to NR-DC Recommendations for 2nd round: Continue discussion in the 2nd round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

## Summary on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on further RRM enhancement for NR and MR-DC – HO with PSCell | vivo |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2200070 | Further discussion on HO with PSCell | CATT | Noted |  |
| R4-2200289 | Discussion on RRM requirement for handover with PSCell | Apple | Noted |  |
| R4-2200290 | Draft CR on HO with PSCell for NR SA to EN-DC_R17 | Apple | Return to |  |
| R4-2200416 | HO with PSCell | Qualcomm Incorporated | Noted |  |
| R4-2200532 | Discussion on HO with PSCell | Intel Corporation | Noted |  |
| R4-2200598 | Discussion on RRM requirements for HO with PSCell | vivo | Noted |  |
| R4-2200630 | Discussion on HO with PSCell | CMCC | Noted |  |
| R4-2200673 | Further discussion on RRM requirements for handover with PSCell | Xiaomi | Noted |  |
| R4-2200740 | Discussion on RRM requirements for HO with PSCell | ZTE Corporation | Noted |  |
| R4-2201135 | RRM requirements for HO with PSCell | OPPO | Noted |  |
| R4-2201202 | Discussion on RRM requirements for HO with PSCell | Huawei, Hisilicon | Noted |  |
| R4-2201203 | Draft CR on requirements for HO with PSCell from EN-DC to EN-DC | Huawei, Hisilicon | Return to |  |
| R4-2201380 | RRM requirements for handover with PSCell | Ericsson | Noted |  |
| R4-2201381 | Drfat CR on HO with PSCell requirements for NE DC to NE-DC | Ericsson | Return to |  |
| R4-2201542 | discussion on HO with PSCell | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2201543 | dratCR on HO with PSCell | Nokia, Nokia Shanghai Bell | Return to |  |
| R4-2201851 | Discussion on HO with PSCell | MediaTek Inc. | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Apple | Jie Cui | Jie_cui@apple.com |
| MediaTek | Ogeen Toma | Ogeen.hanna@mediatek.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)