3GPP TSG-RAN WG4 Meeting # 101bis-e 												R4-2202574

Electronic Meeting, January 17 - 25, 2022

Agenda item:			6.21.2

Source:	Moderator (CATT)

Title:	Email discussion summary for [101-bis-e][223] NR_pos_enh_2

Document for:	Information

Introduction

This email discussion summary contains agenda 6.21.2.1, 6.21.2.3 and 6.21.2.5 which include the following topics:

- Topic 1: UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation

- Topic 2: Measurement in RRC_INACTIVE state

- Topic 3: Enhancements of A-GNSS positioning (No documents submitted)

Topic #1: UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200118 | CATT | Proposal 1: Define multiple margin values associated with TEGs per UE/TRP. Proposal 2: The association between TEG ID and margin value is decided and reported by UE/TRP itself. The margins for different TEGs can be same or different. Proposal 3: The following options can be supported for margin configuration and reporting: - Option 1: NW configure requested margins to UE/TRP based on demand. - Option 2: UE/TRP report used margins to NW based on implementation. - Option 3: The margins are fixed in the specification. Proposal 4: Based on the approaches for defining margins in section 2.1, there is no need to consider dymatic TEG. Proposal 5: RAN4 to define accuracy requirements for the TEG framework based on the different margins. Proposal 6: Whether and how to report the measurement without TEG association should be within RAN1/2 scope. |
| R4-2200119 | CATT | Proposal 1: It is feasible for UE to report the TEG association information between UE Tx TEG IDs and SRS resources for positioning based on a configured periodicity. Proposal 2: The UE Tx TEG association between UE Tx TEG IDs and SRS resources for positioning is up to UE implementation, so it is not necessary and practical to define the condition when the TEG association is changed. Proposal 3: The following solutions can be considered for the Tx TEG association change - Option 1: Allow UE to report the TEG association information with a selected periodicity which is not larger than configured periodicity. - Option 2: Allow UE to record the association information with time stamp when it is changed during the configured periodicity. |
| R4-2200540 | Intel Corporation | Proposal 1: RAN4 shall define a static timing error margin (e.g. ≤2 fixed margin) associated with all TEGs per UE/TRP. Proposal 2: RAN4 can define the maximum possible timing error margins in TS38.133. Whether UE needs to report them to NW is up to RAN1/2. Observation 1: Whether to define time variant (semi-static or dynamic) TEGs can be up to RAN1’s agreements. Observation 2: The exact TEG from UE can be changed time to time. RAN4 can investigate the impacts due to the static TEG within the acceptable variance within a specific duration. Proposal 3: Whether the time variant TEG is necessary can be FFS. The static TEG within a specific time window can be taken as the start point |
| R4-2200668 | vivo | Proposal 1: A single timing error margin associated with one TEG is define for a UE/TRP. Proposal 2: There is no need to define multiple timing error margins for multiple TEGs, which is up to UE/TRP implementation. Proposal 3: UE/TRP configures the timing error margins itself based on its implementation. Proposal 4: The single timing error margin for one TEG could be specified in the spec. Proposal 5: The timing error can be time variant but TEG is up to UE implementation, i.e., there is no need to consider time variant of TEG from RRM requirements perspective. Proposal 6: It is up to implementation how to decide when the UE Tx TEG association is changed. No further work is needed in RAN4. |
| R4-2200757 | Qualcomm Incorporated | Proposal 1: UE/TRP configures the timing error margins itself based on its implementation. Proposal 2: TEGs with configurable timing error margins, subject to UE capability, should be supported. Proposal 3: A single timing error margin is associated with each TEG, and its value is configured by the UE/TRP. Proposal 4: RAN4 should finalize margins for RSTD and UE Rx-Tx measurement accuracy in Rel-16 before deciding on timing error margins for TEGs. Proposal 5: The temporal validity or applicability of a UE Rx TEG is limited to measurements contained within the measurement report in which the Rx TEG information is provided, unless the UE can provide explicit indication of the Rx TEG validity. Proposal 6: UE Rx TEG ID #n may be configured with a different timing error margin in each measurement report. Proposal 7: Proposals 5 and 6 also apply to UE RxTx TEGs. Proposal 8: For UL+DL positioning, the temporal validity or applicability of a UE Tx TEG is limited to measurements contained within the measurement report in which the Tx TEG is information is provided, unless the UE can provide explicit indication of the Tx TEG validity. Proposal 9: For UL+DL positioning, UE Tx TEG ID #n may be configured with a different timing error margin in each measurement report. Proposal 10: For UL TDOA positioning, the temporal validity or applicability of a UE Tx TEG is limited to SRS resource instances transmitted during one reporting period. Proposal 11: For UL TDOA positioning, UE Tx TEG ID #n may be configured with a different timing error margin in each reporting period. Proposal 12: Association of transmissions/measurements to TEGs is optional. If a measurement or transmitted signal is not associated with a TEG, then no further assumption about relative timing between said measurement/signal and other measurement/signals can be made beyond what is already implied by measurement accuracy requirements in Rel-16. Observation 1: When the reference TOA and the target TOA in the RSTD measurement belong to the same Rx TEG, an improved RSTD absolute measurement accuracy may be implied, depending on the error margin of the Rx TEG. Observation 2: When the reference TOA and the target TOAs of two or more RSTD measurements belong to the same pair of Rx TEGs, an improved RSTD relative measurement accuracy may be implied, depending on the error margins of the Rx TEGs. Proposal 13: RAN4 shall discuss during the performance phase whether to introduce enhanced absolute measurement accuracy requirements for RSTD measurements for which the target and reference TRPs are associated with the same RxTEG. Proposal 14: Wait for further progress in RAN4 before deciding whether new tests will be introduced. |
| R4-2201163 | OPPO | Proposal 1: The number of timing error margins N is determined by UE/TRP itself, FFS the value of N. Observation 1: Considering hierarchical TEG reporting framework to further divide TEG sub-group with a small error margin. Proposal 2: UE/TRP configures the timing error margins itself based on its implementation. Proposal 3: Not define time variant TEG and TEG is up to UE implementation. Proposal 4: The core requirements are not impacted by TEG framework and can also postpone this issue until the timing error margins are agreed. |
| R4-2201668 | Ericsson | - Define two margin values for UE Rx TEG for different time scopes: Value 1: X, valid for all measurements in the same measurement report. Value 2: Y (< X), valid for measurements associated with same time stamp. The value of X and Y are FFS and may be dependent on PRS BW and FR. - Define number of timing error margins associated with UE Tx TEG. - Timing error margins fixed in the spec. - Temporal Rx TEG validity is up to implementation assuming that Rx TEG association is based on a per measurement report basis. - Temporal validity of Tx TEG expires when UE UL transmission timing changes due to network configured timing advance. - Temporal validity of UE RxTx TEG can be associated to the change in antenna panel used for Rx – Tx timing measurement. - Introduce measurement accuracy requirements for RSTD measurements for which UE Rx TEG association is same for both target and reference TRPs. - Introduce measurement accuracy requirements for difference between two RSTDs that have the same UE Rx TEG association to the respective target TRP. - Introduce measurement accuracy requirements for timing difference between SRS transmissions performed with same or different UE Tx TEGs. - Allow TEG association in measurement report to be optional. - Tx TEG association is considered changed when UE uplink transmission timing changes due to the network-configured timing advance command. |
| R4-2201398 | ZTE Corporation | - Proposal 1: Support multiple timing error margins per UE/TRP. - Proposal 2: NW configures fixed timing error margins to UE/TRP, or, the timing error margins can be defined in the spec. - Proposal 3: The timing error can be time variant but TEG is up to UE implementation, i.e., there is no need to consider time variant of TEG. - Proposal 4: How to report transmissions/measurements which cannot be associated with any TEG shall be discussed in RAN2. |
| R4-2201636 | Huawei, HiSilicon | Proposal 1: Define a single margin value (X) for the UE Rx TEG for the time scope of “same time stamp”, i.e. LMF assumes timing error difference between two measurements is smaller than X if they are associated with same TEG ID and same time stamp. Proposal 2: The TEG margin value is fixed in the spec. Proposal 3: RAN4 not to consider the time variation of TEG provided that time scope of TEG is defined as ‘same time stamp’. Proposal 4a: RAN4 concludes no impacts on core requirements from the TEG framework. Proposal 4b: RAN4 to discuss the following in the Performance part - the margin value for the UE Rx TEG, and - whether and how to define accuracy requirements for the TEG framework. Proposal 5: RAN4 not to further discuss whether and how to report transmissions/measurements which cannot be associated with any TEG. Proposal 6: Tx TEG association is considered to be changed if at least two SRS resources that used to belong to a same TEG no longer belong to a same TEG. Proposal 7: When Tx TEG association change occurs depends on UE implementation. |
| R4-2201982 | Nokia, Nokia Shanghai Bell | - This feature targets to mitigate residual timing error after calibrations including baseband timing compensation. The residual timing offset includes residual timing error in baseband and RF impairment error. The TEG feature, specified by RAN1, attempts to mitigate this residual timing error after calibrations. Following proposals are made: - Tx / Rx TE grouping is based on N groups with different maximum single-sided timing error for each TEG after calibration T1, T2, …, TN , where Ti = ki * Tc . Both, the number N of supported groups for Tx TEG and for Rx TEG, and the TE range Ti=±ki*Tc for each group are implementation properties of UE/TRP and hence are informed to LMF as positioning assistance information for the PRS/SRS measurement, which can be done together with the report of the UE capability support for multiple UE Tx TEGs. The TEG association information for a timing measurement (i.e. RSTD, UE Rx-Tx time difference, gNB Rx-Tx time difference, UL-RTOA) will include Rx TEG information and corresponding Tx TEG information added by the transmitter and - will be forwarded to LMF along the measurement (i.e. Rx TEG information for all considered measurement types and Tx TEG information for UE Rx-Tx time difference) or - will be requested by gNB with a configured periodicity or triggered by UE and forwarded via gNB to LMF (i.e. Tx TEG information for RTOA) or - will be reported to LMF in implementation-specific manner (i.e. Tx TEG information for RSTD and gNB Rx-Tx time difference). For defining the decision criteria, when UE shall inform gNB and LMF on the change of Tx TEG association information for UL-TDOA, RAN4 will have further investigation once the TEG framework is stable, i.e. how Tx TEG and Rx TEG association information is defined. RAN4 should inform RAN1 on this proceeding. Multiple timing error margins per UE/TRP are defined based on the maximum single-sided timing error achieved after calibration. UE/TRP configures the timing error margins itself based on its implementation. There is no need to consider time variant TEGs for UE and gNB. They can be considered to stay valid for a longer period of time and hence to have semi-static character. The impact of Rx TEGs on measurement requirements and accuracy requirements is FFS until the timing error grouping method and timing error margins are agreed. RAN4 should agree that proper functional tests are needed to test the Tx and Rx TEG association information reporting for UE and gNB. To support the case transmission/measurements cannot be associated with any TEG, allow TEG association in measurement report to be optional. |

Open issues summary

Sub-topic 1-1 Timing error margins associated with TEGs

Issue 1-1-1 The values of timing error margins associated with Rx TEGs for UE/TRP?

Proposals

- Option 1a: (CATT, ZTE)

  - Define multiple margin values (M1, M2, …) associated with TEGs per UE/TRP.

  - The association between TEG ID and margin value is decided and reported by UE/TRP itself. The margins for different TEGs can be same or different.

- Option 1b: (Nokia)

  - Multiple timing error margins per UE/TRP are defined based on the maximum single-sided timing error achieved after calibration.

- Option 1c: (OPPO)

  - The number of timing error margins N is determined by UE/TRP itself, FFS the value of N.

    - Considering hierarchical TEG reporting framework to further divide TEG sub-group with a small error margin.

- Option 2a: (Intel)

  - RAN4 shall define a static timing error margin (e.g. ≤2 fixed margin) associated with all TEGs per UE/TRP.

- Option 2b: (Ericsson)

  - Define two margin values for the UE Rx TEG for different time scopes:

    - Value 1: X, valid for all measurements in the same measurement report

    - Value 2: Y (< X), valid for measurements associated with same time stamp

    - The value of X and Y may be dependent on PRS BW and FR.

- Option 2c: (Huawei)

  - Define a single margin value (X) for the UE Rx TEG for the time scope of “same time stamp”, i.e. LMF assumes timing error difference between two measurements is smaller than X if they are associated with same TEG ID and same time stamp.

- Option 3a: (vivo)

  - A single timing error margin associated with one TEG is defined for a UE/TRP.

  - There is no need to define multiple timing error margins for multiple TEGs, which is up to UE/TRP implementation.

- Option 3b: (Qualcomm)

  - A single timing error margin is associated with each TEG, and its value is configured by the UE/TRP.

  - RAN4 should finalize margins for RSTD and UE Rx-Tx measurement accuracy in Rel-16 before deciding on timing error margins for TEGs.

- Recommended WF

  - Need more discussion

| Issue 1-1-1 The values of timing error margins associated with Rx TEGs for UE/TRP? |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 2c or 2b. In our understanding, TEG framework is introduced to achieve high accuracy positioning, so what is most relevant for the TEG framework is the margin for the same time stamp. Therefore we proposed option 2c. Of course, if companies think the margin for different time stamps is also important, we are also fine with option 2b. However, we do not see clear need for reporting TEGs with multiple margin values (different TEGs with different margin values) for the same time scope and how it would help LMF in positioning fix. It could happen that UE just report TEG with a large allowed margin value, which makes the TEG framework not useful. |
| Ericsson | We support option 2b. Since a measurement report may contain measurements from multiple time stamps, it is important that the timing error margins for Rx TEG can be defined for both time scopes. Within same time stamp margin can be tighter than error between measurements from multiple time stamps considered for the same measurement report by the UE. |
| Intel | We can support 2a,2c,2b. For Option 3b, we don’t know why the margin can be configurable. It shall be related to UE implementation itself. |
| CATT | Support option 1a. To simplify the discussion, we would like to suggest discussing this issue without considering time variation i.e. this issue is for the measurement report with the same time stamp. The case of different time stamps will be considered in issue 1-2-1. With this assumption, in our understanding, there are three aspects to be decided for this issue. The first one is how many margins to be defined for TEG association, the second one is whether the margins for multiple TEGs are the same or not and the third one is whether the margin is defined as a constant or a configurable value by UE/TRP. Without considering time variation, we think a single margin is associated with each TEG, but RAN4 can define several candidate values to adapt to different capability of UE. For the multiple TEGs of one UE/TRP, our proposal is to define different margins for different TEGs, but we can compromise to define the same margin for all the TEGs per UE/TRP. This margin is based on the timing error difference between different RF chains which depends on the UE implementation. So we think RAN4 can define several feasible constants for this margin and UE can report the used value to LMF. |
| OPPO | Option 1a and 1c. In our understanding, the residual timing error is highly dependent on the UE/TRP implementation and PRS/SRS configurations. The number of margins supported by UE/TRP should be determined by UE/TRP itself. As for how to help LMF in positioning fix, the hierarchical TEG framework can be considered. For example, TEG group #1 (level-1) is associated with the larger margin, which may correspond to the same RF chain. The measurement results in TEG group #1 can be further divided into two TEG sub-groups #1-1 (level 2) associated with the smaller margin, which may correspond to the same PRS bandwidth, or over-sampling rate, or time stamp. LMF could choose to combine measurement results within one TEG sub-group for smaller residual error, or to combine measurement results within one TEG group (cross sub-group) so that more results could be used. |
| Nokia | We support 1a, 1b, 1c, 3b. We highlight key points : Multiple timing error margins per UE/TRP are assumed, and a single timing error margin is associated with each TEG. The number of timing error margins N is determined by UE/TRP itself within the maximum number of timing error margins. RAN4 needs to determine the maximum number in the spec. FFS on Option-2b and 2c that propose reporting signalling structure on error margin associated with TEG and time stamp. |
| Qualcomm | We support option 3b. One clarification is that when we say that the value of the timing error margin is configured by the UE/TRP, it means that in the measurement report the UE would indicate the timing error value for each TEG. The motivation for supporting configurable timing error margins is that it gives flexibility to the UE/TRP to actually make use of the feature. If only a single value is supported then the benefit obtained from TEGs may be reduced. If the single value is too small the UE may not be able to associate any measurements with the TEG. E.g. different UEs may use RF components with different tolerances that may impact timing accuracy. Also, in a single measurement report a UE may report measurements for different PFLs and the UE may be capable of achieving different timing error margins for each layer. Option 1a seems similar to option 3b. Option 2a would be too restrictive, in our view. Regarding option 2b/2c, we don’t see the need to restrict the applicability of the TEG to measurements with the same timestamp. Per RAN4 agreement the association of measurements to TEGs is left up to UE/TRP implementation. The UE/TRP can choose to only associate measurements with the same time stamp with one TEG but it should not be mandated. |
| vivo | We support option 3a. In principle, we think one timing error margin would be enough for one TEG. From requirements perspective, a fixed margin can be specified. However, we are open to discuss whether further refinement is necessary. |

Issue 1-1-2 The values of timing error margins associated with UE Tx TEGs

Proposals

- Option 1: (Ericsson)

  - Define number of timing error margins associated with UE Tx TEG

- Recommended WF

  - Need more discussion

| Issue 1-1-2 The values of timing error margins associated with UE Tx TEGs |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We need more time to check. It relates to the time scope of Tx TEG as discussed in sub-topic 1-5. |
| Ericsson | We support option 1. TEG margins must also be defined for other TEG types. |
| Intel | Can support Option1. But we are also open for the further discussion. |
| CATT | Can support option 1. We agree that the margins for Tx TEG should also be defined and the values defined for Rx TEG can be reused i.e. the conclusions in issue 1-1-1 can be also applied for Tx TEG and maybe RxTx TEG. |
| OPPO | We are fine with option 1 and prefer to use the same principle for Rx TEG in issue 1-1-1. |
| Nokia | Does it mean the maximum number of timing error margins associated with UE Tx TEG? In our understanding, the number of TE associated with UE TX TEG is also up to UE report like issue 1-1-1 Option-1c : the number of timing error margins N is determined by UE/TRP itself. |
| Qualcomm | We understand that the question here is whether RAN4 should define a set of timing error margin values that may be used with Tx TEGs. If our understanding is correct, then yes; we woud support the proposal. |
| vivo | Does the proposal mean multiple timing error margins can be associated with a single UE Tx TEG? In principle, we think conclusion for Rx TEG could be reused for Tx TEG. |

Issue 1-1-3 Approaches for LMF acquiring the timing error margins associated with TEGs of UE/TRP?

Proposals

- Option 1: (CATT, ZTE)

  - NW configures requested margins to UE/TRP based on demand.

- Option 2: (CATT, vivo, Qualcomm, OPPO, Nokia)

  - UE/TRP report used margins to NW based on implementation.

- Option 2a: (Qualcomm)

  - TEGs with configurable timing error margins, subject to UE capability, should be supported.

- Option 3: (CATT, vivo, Ericsson, ZTE, Huawei)

  - The margins are fixed in the specification.

- Option 3a: (Intel)

  - RAN4 can define the maximum possible timing error margins in TS38.133. Whether UE needs to report them to NW is up to RAN1/2.

- Recommended WF

  - RAN4 define the possible timing error margins in TS 38.133.

  - FFS whether NW can configure requested margins to UE/TRP based on demand.

  - FFS whether UE/TRP need to report used margins to NW based on implementation.

| Issue 1-1-3 Approaches for LMF acquiring the timing error margins associated with TEGs of UE/TRP? |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We are fine with the recommended WF except the following update to the first bullet. The reason is that based on Issue 1-1-1, it may be possible that a single TEG margin is defined in the spec. RAN4 define the possible timing error margin(s) in TS 38.133. For the second FFS bullet, it makes some sense but it is not addressing the most typical use case of TEG (high accuracy) so we suggest to consider it as an enhancement for future releases if seen as needed. For the third bullet, our concern is that UE may just report TEG with a large allowed margin value, which makes the TEG framework not useful. |
| Ericsson | We support option 3 for the following reasons: a) there does not exist any signalling that UE can use to report timing error margins associated to its TEG to the network, b) timing error margins shall be derived to achieve target accuracy and shall be defined in the spec. The margin values can be discussed during the performance part of the WI. From recommended WF list: we support RAN4 define the possible timing error margins in TS 38.133. No further study required. |
| Intel | The recommend WF are fine for us except the following updates: “RAN4 define the possible maximum timing error margins in TS 38.133” |
| CATT | Support the recommended WF. Firstly, we think the possible margins should be defined in RAN4 and whether to define multiple values is discussed in issue 1-1-1. And then for the FFS part, we think if only one margin is defined for all the TEGs, then no NW configuration and UE report is needed. But if several candidate values are defined, then NW can configure the requested margin based on the positioning demand and UE can report the used margin based on implementation which is the same as the determination of k in report mapping. |
| OPPO | We agree to define the possible timing error margins in RAN4 spec. If multiple timing error margins are defined, then UE/TRP need to report the used margin. Therefore, we can also support the 3rd bullet. For the 2nd bullet, we are not clear about the relation between margin requested by NW and margin supported by UE capability. |
| Nokia | If timing error margin associated with TEGs is changed, the device (UE or TRP) reports used new margin(s) to NW. We assume it is a kind of on-demand report from the device. |
| Qualcomm | We agree with the following: RAN4 define the possible timing error margins in TS 38.133 The UE/TRP would inform the LMF of the timing error margin for each Rx/RxTx TEG TEG in the measurement report the UE would indicate the timing error value for each TEG. For Tx TEGs, timing error margin would be included either in the measurement report (for RTT) or in a separate report configured by the LMF for UL-only positioning. |
| vivo | RAN4 define the possible timing error margin in TS 381.33 is fine for us. Our view is that we don’t need to specify multiple timing error margins for one TEG or multiple TEGs. In addition, we don’t see any reason that UE/TRP reports the used margins to NW. How could NW use such information? |

Issue 1-1-4 Whether to send LS to RAN1 about margin value?

Proposals

- Option 1: (Huawei)

  - Yes

- Recommended WF

  - Need more discussion

| Issue 1-1-4 Whether to send LS to RAN1 about margin value? |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We support to send LS to RAN1 and also RAN2, and the LS should not be only about the value of TEG margin but also the time scope. Different WGs should have common understanding on how TEG association info is supposed to be used by LMF. |
| Ericsson | Option 1. We are open sending out LS to RAN1. Better to also capture option 2b in Issue 1-1-1 in the LS. |
| Intel | No sure is necessary to send LS to RAN1 on this issue only(margin) since it is related to UE requirements themselves. |
| CATT | Based on issue 1-1-1, the margin value need to be informed to RAN1/2 after it is decided. |
| OPPO | Agree to send LS to RAN1 if we can reach consensus in this meeting. |
| Nokia | We can come back in 2nd round after seeing 1-1-1 conclusion. |
| Qualcomm | We agree that a LS should be sent to RAN1 and RAN2 once RAN4 reaches an agreement. In our view, one important point to be included would be that timing error margins are configurable and that the timing error margin for each TEG ID would be included in the measurement report or a separate report for UL-only positioning. |
| vivo | It depends on the conclusions we are to make. If there is impact to RAN1/2, then an LS is needed. |

Sub-topic 1-2 Time variant of the TEG

Issue 1-2-1 Whether to define time-variant (semi-static or dynamic) TEGs?

Proposals

- Option 1a: (CATT, vivo, OPPO, ZTE)

  - The timing error can be time variant but TEG is up to UE implementation, i.e., there is no need to consider time variant of TEG from RRM requirements perspective.

- Option 1b: (Huawei)

  - RAN4 not to consider the time variation of TEG provided that time scope of TEG is defined as ‘same time stamp’

- Option 2a: (Intel)

  - The static TEG within a specific time window can be taken as the start point.

- Option 2b: (Nokia)

  - There is no need to consider time variant TEGs for UE and gNB. They can be considered to stay valid for a longer period of time and hence to have semi-static character.

- Option 3: (Ericsson)

  - Temporal Rx TEG validity is up to implementation assuming that Rx TEG association is based on a per measurement report basis.

  - Temporal validity of Tx TEG expires when UE UL transmission timing changes due to network configured timing advance.

  - Temporal validity of UE RxTx TEG can be associated to the change in antenna panel used for Rx – Tx timing measurement.

- Option 4: (Qualcomm)

  - The temporal validity or applicability of a UE Rx TEG is limited to measurements contained within the measurement report in which the Rx TEG information is provided, unless the UE can provide explicit indication of  the Rx TEG validity.

  - UE Rx TEG ID #n may be configured with a different timing error margin in each measurement report.

  - The principles for UE Rx TEG (i.e. first two bullets) also apply to UE RxTx TEGs

  - For UL+DL positioning, the temporal validity or applicability of a UE Tx TEG is limited to measurements contained within the measurement report in which the Tx TEG is information is provided, unless the UE can provide explicit indication of the Tx TEG validity.

  - For UL+DL positioning, UE Tx TEG ID #n may be configured with a different timing error margin in each measurement report.

  - For UL TDOA positioning, the temporal validity or applicability of a UE Tx TEG is limited to SRS resource instances transmitted during one reporting period.

  - For UL TDOA positioning, UE Tx TEG ID #n may be configured with a different timing error margin in each reporting period.

- Recommended WF

  - Need more discussion

| Issue 1-2-1 Whether to define time-variant (semi-static or dynamic) TEGs? |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 1b. We think in Rel-17 RAN4 should focus on the most basic time scope of TEG, i.e. same time stamp. If companies think the time scope of temporal validity of TEG should be beyond “same time stamp”, we can support the first bullet of option 3 for UE Rx TEG. For UE Tx TEG, it relates to the discussion in sub-topic 1-5, so more discussions are needed. For UE RxTx TEG, we need more time to check. |
| Ericsson | We are in favor of non-static TEG as elaborated in option 3. Since TEG is up to implementation, for Rx TEG temporal validity can be a measurement period to ensure less frequent change in TEG ID reporting and therefore reducing signalling overhead drastically. For Tx TEG validity can be derived based on TA command from the network. For every change in TA and adjustment in UL transmission timing, UE reports it Tx TEG ID. |
| Intel | Can support the Option 1b, 2a,2b because in our view, the static TEG can be taken as RAN4 requirements working assumption. |
| CATT | We can support option 1a and 1b. Firstly we think the margins should be defined with same stamp. Since the TEG ID is reported for each measurement report, we don’t think we should compare the measurements with different time stamps i.e. the reported association is only applied to the measurement report which the information is provided. If only one margin is defined, the same TEG ID represents the same value even if in different time stamp, but if multiple or configurable margins are defined, then the same TEG ID in different time stamp can represent different margins. |
| OPPO | Support option 1a. Timing variant can be addressed by associating to different TEG ID. |
| Nokia | Can support the Option 2a, 2b. FFS on time scope of TEG is defined as ‘same time stamp’ in 1b. |
| Qualcomm | Option 4 |
| vivo | Support option 1a. |

Sub-topic 1-3 RRM requirements

Issue 1-3-1 The impact of Rx TEGs on measurement requirements and accuracy requirements.

Proposals

- Option 1: (CATT)

  - RAN4 to define accuracy requirements for the TEG framework based on the different margins.

- Option 2: (Qualcomm)

  - RAN4 shall discuss during the performance phase whether to introduce enhanced absolute measurement accuracy requirements for RSTD measurements for which the target and reference TRPs are associated with the same RxTEG.

- Option 3: (OPPO, Nokia)

  - The core requirements are not impacted by TEG framework and can also postpone this issue until the timing error margins are agreed.

- Option 4: (Ericsson)

  - Introduce measurement accuracy requirements for RSTD measurements for which UE Rx TEG association is same for both target and reference TRPs.

  - Introduce measurement accuracy requirements for difference between two RSTDs that have the same UE Rx TEG association to the respective target TRP.

  - Introduce measurement accuracy requirements for timing difference between SRS transmissions performed with same or different UE Tx TEGs.

- Option 5: (Huawei)

  - RAN4 concludes no impacts on core requirements from the TEG framework.

  - RAN4 to discuss the following in the Performance part

    - the margin value for the UE Rx TEG, and

    - whether and how to define accuracy requirements for the TEG framework.

- Recommended WF

  - Need more discussion

| Issue 1-3-1 The impact of Rx TEGs on measurement requirements and accuracy requirements. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | It seems no company identified core impacts due to TEG, so we suggest to conclude no impacts on core requirements from the TEG framework. RAN4 can discuss whether and how to define accuracy requirements for the TEG framework in the Perf part. |
| Ericsson | UE Rx TEGs have an impact on RSTD measurement accuracy requirement. During an RSTD measurement, an accuracy requirement can be set for a case when the UE Rx TEG association is same for both target and reference TRPs. In a similar fashion, an accuracy requirement on difference between two RSTD measurements that have same UE Rx TEG association to the respective target TRPs but different UE Rx TEG for reference TRP can also be introduced. However, we agree that this is part of performance work item and can be further discussed during the performance part of the WI. |
| Intel | We thought the options above are quite similar. That is we need to discuss the performance requirements for TEG related issues instead of core part. |
| CATT | Option 1 and option 3. We agree that there is no impact on core requirements. Regarding the details on accuracy requirements, we can discuss in the performance part after the margins are decided. |
| OPPO | Option 1, 3 and 5. Agree to conclude that core requirements are not impacted due to TEG. How to update accuracy requirements can be further discussed in performance part. |
| Nokia | Option 3. We are not sure if RAN4 can reach to aligned accuracy performance study that tries to mitigate residual timing error. It needs high accuracy comparing to Rel-16 accuracy. |
| Qualcomm | We support option 2. It seems that there may be consensus that any new accuracy requirements can be discussed in the performance part. Regarding impact to core requirements, we think RAN4 may need to discuss potential impact from the RAN1 agreement below. The UE may need a longer measurement period. We suggest to discuss it in the next meeting. Agreement Make the following modification on the previous agreement made in RAN#106bis-e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability, and applies to all DL PRS positioning frequency layers - Note: If N is not explicitly included in the request, it is up to UE to determine the number of different UE Rx TEGs to measure the same DL PRS resource within its capability - The TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. - Note: All RSTD measurements are relative to a single reference timing - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. - M = [2, 3, 4, 6, 8] (FFS: other values) applies to all configured SRS resources for positioning - Note: If M is not explicitly included in the request, it is up to TRP to determine the number of different TRP Rx TEGs to measure the same SRS resources for positioning - FFS: details of the signalling, procedures |
| vivo | We agree that the positioning accuracy requirement due to TEG can be specified in the performance part. |
| CATT2 | To QC, we think the measurement requirements are for the each reported RSTD measurement which should not impacted by the number of reported RSTD. RAN1’s agreement is to request UE to report multiple RSTD measurement which will not impact the requirements for each RSTD measurement. |

Issue 1-3-2 Testing requirements for verifying the timing error mitigation

Proposals

- Option 1: (Qualcomm)

  - Wait for further progress in RAN4 before deciding whether new tests will be introduced.

- Option 2: (Nokia)

  - RAN4 should agree that proper functional tests are needed to test the Tx and Rx TEG association information reporting for UE and gNB.

- Recommended WF

  - Need more discussion

| Issue 1-3-2 Testing requirements for verifying the timing error mitigation |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 1. On option 2, we understand functional test is not in RAN4 scope. |
| Ericsson | We support option 1. Test cases are part of performance and therefore they should be discussed during the performance part. |
| Intel | Option 1. Can be done in the performance part. |
| CATT | Option 1. For core requirements, we think there is no impact, so there is no new tests are needed. For accuracy requirements, we should wait for the progress of accuracy requirements to decide whether to define new tests. |
| OPPO | Option 1. More progress is needed. |
| Nokia | We are fine to revisit test introduction discussion after completing the TEG framework. If there is no core requirement impact on accuracy, TEG feature support needs to be verified in a RAN4 test. |
| Qualcomm | Option 1 |
| vivo | Option 1. |

Sub-topic 1-4 Report for the measurement without TEG association

Issue 1-4-1 How to report transmissions/measurements which cannot be associated with any TEG

Proposals

- Option 1: (CATT, ZTE, Huawei)

  - Whether and how to report the measurement without TEG association should be within RAN1/2 scope.

- Option 2: (Qualcomm, Ericsson, Nokia)

  - Association of transmissions/measurements to TEGs is optional.

    - If a measurement or transmitted signal is not associated with a TEG, then no further assumption about relative timing between said measurement/signal and other measurement/signals can be made beyond what is already implied by measurement accuracy requirements in Rel-16.(Qualcomm)

- Recommended WF

  - Need more discussion

| Issue 1-4-1 How to report transmissions/measurements which cannot be associated with any TEG |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 1. |
| Ericsson | We support option 2. Based on RAN1 agreement a UE may report TEG ID only if it has more than 1 TEG. |
| CATT | Option 1. RAN4 is to define the requirements based on that the TEG association report is provided. As for whether and how to report the measurement is with RAN1/2 scope. |
| OPPO | Option 2. TEG association should be optional and accuracy requirements should fall back to Rel-16 if the TEG association is absent. |
| Nokia | Option 2. TEG and TE mitigation is optional feature improve ToA or RTT measurements. If a measurement or transmitted signal is not associated with a TEG, then LMF does not have no further assumption or behavior to improve accuracy. |
| Qualcomm | Option 2. It is clear that reporting of TEGs is optional according to RAN1. The key points in option 2 are that a) association of transmission/measurements to TEGs is optional and b) if there is no association, then no further assumptions about relative timing accuracy between measurements can be made. If RAN2 decides to introduce a particular way to signal that a measurement/transmission is not associated with any TEG that would be OK, but there must be a way for the UE to do it. |
| vivo | Option 1 |

Sub-topic 1-5 Reply LS to RAN1 on reporting of the Tx TEG association information

For RAN1 LS R4-2200054(R1-2112968):

Agreement - For UL-TDOA, supporting the following for the serving gNB to request a UE to report the Tx TEG association information between UE Tx TEG IDs and SRS resources for positioning, subject to UE capability of supporting UE Tx TEG: - Based on a configured periodicity, a UE may report the UE Tx TEG association for the SRS resources for positioning that have already been transmitted during the configured period - It is up to RAN2 to decide how to indicate the change of the Tx TEG association during the configured period (e.g., using the timestamps) - It is up to RAN4 to decide when the Tx TEG association is changed - The values of the configurable periodicities are up to RAN2 - Note: Tx TEG association information reporting by single request/response mode is assumed already supported with the previous agreement. - Send an LS to RAN2/RAN4 (cc: RAN3) - to RAN2, including the following RAN1’s agreement related to the reporting of the UE Tx TEG, for RAN2 to work on the signalling - to RAN4 for checking the agreement and work on how to decide when the Tx TEG association is changed

Issue 1-5-1 How to decide when the Tx TEG association is changed

Proposals

- Option 1a: (CATT, vivo)

  - The UE Tx TEG association between UE Tx TEG IDs and SRS resources for positioning is up to UE implementation, so it is not necessary and practical to define the condition when the TEG association is changed.

- Option 1b: (Huawei)

  - Tx TEG association is considered to be changed if at least two SRS resources that used to belong to a same TEG no longer belong to a same TEG.

  - When Tx TEG association change occurs depends on UE implementation

- Option 2: (Ericsson)

  - Tx TEG association is considered changed when UE uplink transmission timing changes due to the network-configured timing advance command.

- Option 3: (Nokia)

  - For defining the decision criteria, when UE shall inform gNB and LMF on the change of Tx TEG association information for UL-TDOA, RAN4 will have further investigation once the TEG framework is stable, i.e. how Tx TEG and Rx TEG association information is defined. RAN4 should inform RAN1 on this proceeding.

- Recommended WF

  - Need more discussion

| Issue 1-5-1 How to decide when the Tx TEG association is changed |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We understand there are two issues here: - Issue 1: the criteria of Tx TEG association change - Issue 2: the condition of Tx TEG association change For the criteria we support option 1b. It also relates to the time scope of TEG information. For the condition, we understand the selection of Tx antenna for transmitting an SRS resource instance is UE implementation, so it is not practical to define when Tx TEG association change occurs. |
| Ericsson | In our point of view option 2 is the least complex solution that can be effectively used to determine a change in the Tx TEG association. |
| CATT | Option 1a. It has been agreed in RAN4 that how to group the timing error is left to UE/TRP implementation. So there is no unified condition that can indicate the change of TEG association. And the TEG is more about the Rx/Tx RF chain and not related to the TA. So we think it is not feasible for RAN4 to define the condition when Tx TEG association is changed. Can the proponents of option 2 clarify why the TA change can represent the TEG association change? And for the criteria of TEG association change, we are also fine with the first bullet of option 1b. |
| OPPO | Prefer option 1a. |
| Nokia | When a UE detects needs to update Tx TEG association, it can be reported. We are fine with 1a, 2 and 3. |
| Qualcomm | Option 1a |
| vivo | Option 1a. |

Issue 1-5-2 How to handle the case when the Tx TEG association is changed

Proposals

- Option 1: (CATT)

  - Allow UE to report the TEG association information with a selected periodicity which is not larger than configured periodicity.

- Option 2: (CATT)

  - Allow UE to record the association information with time stamp when it is changed during the configured periodicity.

- Recommended WF

  - Need more discussion

| Issue 1-5-2 How to handle the case when the Tx TEG association is changed |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We understand this issue is in RAN1/2 scope. Based on the incoming LS, RAN4 is not asked to decide how to handle the case when the Tx TEG association is changed. |
| Ericsson | In our previous comment we presented our view on when Tx TEG association changes. Whenever Tx TEG association changes, the UE shall report (given that it supports more than 1 TEG) Tx TEG association information to the network. This ensures less frequent change in Tx TEG association and is more efficient in terms of signalling overhead. |
| CATT | Option 1 and option 2. Based on issue 1-5-1, we think it is not practical to define a unified condition to indicate the TEG association change, but this can be handled by UE implementation. For example, when the configured reporting periodicity is much larger, to indicate the frequent TEG association change within the periodicity, UE is allowed to report the association information with a smaller periodicity or record the association change with time stamp. This can be provided to RAN1/2 for information. |
| Nokia | Option-1 wording is preferred. We are not sure about the behavior recording the association information, but we assume the two options have the similar motivation. |
| Qualcomm | We understand that this issue (and whether to introduce timestamps) will be decided by RAN2 according to the LS from RAN1. |
| vivo | In our understanding, this issue is related to RAN2 based on the LS from RAN1. |

Issue 1-5-3 Whether to send reply LS to RAN1

Proposals

- Option 1: (CATT, vivo, Ericsson, Huawei)

  - Yes.

- Recommended WF

  - Need more discussion

| Issue 1-5-3 Whether to send reply LS to RAN1 |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 1 |
| Ericsson | We are open to send reply LS to RAN1. The Tx change association based on network provided TA command. |
| CATT | Option 1 if consensus is reached for issue 1-5-1 and issue 1-5-2. |
| Nokia | We are open to send LS based on Issue 1-5-1. |
| Qualcomm | Once RAN4 reaches agreements on the two issues above, a LS could be sent to RAN1 and RAN2 to inform them. |
| vivo | Yes if conclusions can be made in RAN4. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 1-1 Timing error margins associated with TEGs

|  | Status summary |
| --- | --- |
| Issue 1-1-1 | Issue 1-1-1 The values of timing error margins associated with Rx TEGs for UE/TRP? Tentative agreements: - Agreements in GTW (1.19) - A single timing error margin is associated with each Rx TEG - FFS if same or different margins are used for measurements with different time stamps - FFS: whether the timing error margin is the same or not for all Rx TEGs if UE/TRP has multiple TEGs Candidate options: Moderator: I try to provide the whole procedure of the TEG definition and report based on my understanding on the options in issue 1-1-1, issue 1-3-1 and issue 1-2-1. Please check and provide your suggested procedure if I misunderstood your position. - Option A: (CATT, OPPO, Nokia, Qualcomm) - Step #1: RAN4 define multiple candidate values {TE1, TE2, …} in the spec. - Step #2: UE/TRP has multiple Rx TEGs (TEG#1, TEG#2, …) associated with multiple values (M1, M2, …), which means the timing error difference between the measurements within the TEG#i is within the margin Mi where i=1,2,…. - Mi is selected from {TE1, TE2, …} - Mi can be same as or different from each other - Step #3: UE/TRP reports the corresponding margin together with Rx TEG ID during the measurement report. - Step #4: The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided. - Step #5: RRM accuracy requirements will be defined based on the different values {TE1, TE2, …}. - Option B: (Ericsson, Huawei, Intel) - Step #1: RAN4 define two fixed values {TE1, TE2} in the spec where TE1<TE2. - Step #2: UE/TRP has multiple Rx TEGs (TEG#1, TEG#2, …) associated with the same value TE1(TE2) which means the timing error difference between the measurements within the same RxTEG is within the margin TE1(TE2). - Step #3: UE/TRP only reports the Rx TEG ID during the measurement report and does not need to report the margin. - Step #4: TE1 is applied for the measurements with same time stamp and TE2 for the measurement with different time stamps. (i.e. the timing error difference between the measurements within the same TEG and with same time stamp is within the margin TE1, and the timing error difference between the measurements within the same TEG and with different time stamps is within the margin TE2) - Step #5: RRM accuracy requirements will be defined based on the different values {TE1, TE2}. - Option C: (Compromise from moderator) - Step #1: RAN4 define multiple candidate values {TE1, TE2, …} in the spec. - Step #2: UE/TRP has multiple Rx TEGs (TEG#1, TEG#2, …) associated with the same value M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - M is selected from {TE1, TE2, …} - Step #3: UE/TRP reports selected margin M before the measurement (e.g. after receiving the location request) and only report the Rx TEG ID during the measurement report. - Step #4: The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided. - Step #5: RRM accuracy requirements will be defined based on the different values {TE1, TE2, …}. - Step #6: FFS whether NW can configure requested margin to UE/TRP based on positioning demand. Recommendations for 2nd round: Further discuss the candidate options. |
| Issue 1-1-2 | Issue 1-1-2 The values of timing error margins associated with UE Tx TEGs Tentative agreements: - The values of timing error margins associated with UE/TRP Tx TEGs need to be defined in RAN4. - The values of timing error margins associated with UE/TRP RxTx TEGs need to be defined in RAN4. - FFS whether to reuse the values of UE/TRP Rx TEG. Candidate options: Issue 1-1-2a Whether to reuse the values of UE/TRP Rx TEG for UE/TRP Tx TEGs if applicable. - Option 1: - Yes - Option 2: - No Issue 1-1-2b Whether to reuse the values of UE/TRP Rx TEG for UE/TRP RxTx TEGs if applicable. - Option 1: - Yes - Option 2: - No Recommendations for 2nd round: Check the tentative agreements and if agreed, further discuss the candidate options. |
| Issue 1-1-3 | Issue 1-1-3 Approaches for LMF acquiring the timing error margins associated with TEGs of UE/TRP? Tentative agreements: - RAN4 define the possible timing error margin(s) in TS 38.133. - FFS whether NW can configure requested margins to UE/TRP based on demand. - FFS whether UE/TRP need to report used margins to NW based on implementation. Candidate options: None. Recommendations for 2nd round: None, the FFS part can be included in the discussion of issue 1-1-1. |
| Issue 1-1-4 | Issue 1-1-4 Whether to send LS to RAN1 about margin value? Tentative agreements: - Postpone after the issue 1-1-1 is concluded. Candidate options: None. Recommendations for 2nd round: None. |

Sub-topic 1-2 Time variant of the TEG

|  | Status summary |
| --- | --- |
| Issue 1-2-1 | Issue 1-2-1 Whether to define time-variant (semi-static or dynamic) TEGs? Tentative agreements: None. Candidate options: Moderator: I try to merge the options into the following approaches to define the time-variant TEGs which have also been included in issue 1-1-1. - Approach 1: The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided. (option A and C in issue 1-1-1) - Approach 2: Define a different margin value [TE2] for the measurement report with different time stamps (option B in issue 1-1-1) - Other approaches. Recommendations for 2nd round: The Time variant of Rx TEG is discussed in the issue 1-1-1. Further discuss the time variant for Tx TEG and RxTx TEG. |

Sub-topic 1-3 RRM requirements

|  | Status summary |
| --- | --- |
| Issue 1-3-1 | Issue 1-3-1 The impact of Rx TEGs on measurement requirements and accuracy requirements. Tentative agreements: - There is no impact on the core requirements from TEG framework. - Whether and how to define accuracy requirements for the TEG framework will be discussed in performance part. Candidate options: None. Recommendations for 2nd round: Check the tentative agreement. Companies please further check the impact on core requirements. |
| Issue 1-3-2 | Issue 1-3-2 Testing requirements for verifying the timing error mitigation Tentative agreements: - Wait for further progress and decide whether to define new tests in performance part. Candidate options: None. Recommendations for 2nd round: None. |

Sub-topic 1-4 Report for the measurement without TEG association

|  | Status summary |
| --- | --- |
| Issue 1-4-1 | Issue 1-4-1 How to report transmissions/measurements which cannot be associated with any TEG Tentative agreements: None. Candidate options: - Option 1: (CATT, ZTE, Huawei, vivo) - Whether and how to report the measurement without TEG association should be within RAN1/2 scope. - Option 2: (Qualcomm, Ericsson, Nokia, OPPO) - Association of transmissions/measurements to TEGs is optional. - If a measurement or transmitted signal is not associated with a TEG, then no further assumption about relative timing between said measurement/signal and other measurement/signals can be made beyond what is already implied by measurement accuracy requirements in Rel-16.(Qualcomm) Recommendations for 2nd round: Further discuss. |

Sub-topic 1-5 Reply LS to RAN1 on reporting of the Tx TEG association information

|  | Status summary |
| --- | --- |
| Issue 1-5-1 | Issue 1-5-1 How to decide when the Tx TEG association is changed Tentative agreements: - The UE Tx TEG association between UE Tx TEG IDs and SRS resources for positioning is up to UE implementation, so it is not necessary and practical to define the condition when the TEG association is changed. Candidate options: None. Recommendations for 2nd round: Discuss based on the draft LS. |
| Issue 1-5-2 | Issue 1-5-2 How to handle the case when the Tx TEG association is changed Tentative agreements: - The issue is within RAN2 scope. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 1-5-3 | Issue 1-5-3 Whether to send reply LS to RAN1 Tentative agreements: - Send the reply LS to RAN1/2. Candidate options: None. Recommendations for 2nd round: None. |

CRs/TPs

Discussion on 2nd round (if applicable)

Topic #2: Measurement in RRC_INACTIVE state

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200121 | CATT | Proposal 1: RAN4 to wait for further agreements in RAN1 and RAN2 (if any) regarding UE/gNB Rx-Tx time difference measurement applicability in RRC_INACTIVE state. Proposal 2: If UE Rx-Tx time difference measurement is applied in RRC_INACTIVE state, the framework of R16 UE Rx-Tx time difference measurement period can be the baseline. Proposal 3: Support of SDT is not a necessary condition for supporting positioning measurements in RRC_INACTIVE state. Proposal 4: RAN4 can independently work on positioning measurements in RRC_INACTIVE state and SDT requirements. Proposal 5: RAN4 deprioritize the discussion for PRS measurement requirements when RRC state transition occurs during the measurement period. Proposal 6: No need to restrict TPRS in the spec because of requirements. Proposal 7: Support the reduced number of samples for PRS measurement in RRC_INACTIVE state. Proposal 8: No need to define two set of PRS measurement period requirements in RRC_INACTIVE state. Proposal 9: Based on the assumption that only one PFL can be measured per DRX cycle, summation based approach can be used for multiple positioning frequency layers for the PRS measurement requirements in RRC_INACTIVE state. Proposal 10: Whether to define new UE processing capability in RRC_INACTIVE state is within RAN1 scope. Proposal 11: DRX cycle should be considered in the available PRS periodicity for the PRS measurement in RRC_INACTIVE state i,e. Tavailable_PRS is the least common multiple between TPRS and DRX cycle. Proposal 12: For the definition of TPRS and Teffect in the PRS measurement in RRC_INACTIVE state, the same approach as R16 can be used. Proposal 13: It is not necessary to inform the DRX cycle to LMF even if it is considered in the PRS measurement requirements in RRC_INACTIVE state. Proposal 14: Send LS to RAN1 to ask for the clarification whether the PRS processing window is applied for the PRS measurement in RRC_INACTIVE state. Proposal 15: Kcarrier_PRS is the combined number of NR inter-frequency carriers indicated by the serving cell and the number of NR inter-frequency carriers configured for idle mode CA measurements plus one positioning frequency layer. Proposal 16: Prioritizing measurements for long-periodicity PRS resources in RRC INACTIVE state is not necessary. Proposal 17: The side conditions in terms of PRS Ês/Iot defined in RRC_CONNECTED state can be applied for PRS measurements in RRC_INACTIVE state under the condition that the same number of samples is used. |
| R4-2200543 | Intel Corporation | Observation 1: With the SRS in RRC_INACTIVE (either pre-configured or Semi-persistent scheduled), it is feasible to measure the UE Rx-Tx time difference for positioning. Proposal 1: UE requirements for UE Rx-Tx time difference measurements in RRC_INACTIVE shall be specified. Proposal 2: RAN4 can use the framework of Rel-16 UE Rx-Tx time difference measurement period as a start point to derive the inactive state UE Rx-Tx time difference measurement period. Proposal 3: The same PRS measurement requirements shall be defined for all cases in which other RRM measurements are occurred. Proposal 4 : If there is status transition (e.g. RRC_INACTIVE RRC_CONNECT) within one successful measurement reporting period, UE measurement requirements can be based on the behaviour below. - UE restarts the PRS measurement Observation 2: If UE support the reduce number of samples in RRC_CONNECT state, the reduce sample can be applied, which is also desirable by UE because of power consumption benefits. Proposal 5: The requirements with less PRS measurement samples can be defined for NR positioning measurement in RRC_INACTIVE upon UE capability. Proposal 6: The PRS measurement period requirements under RRC_INACTIVE can be defined depending on UE processing capability but the exact formulation on this requirement (e.g. Teffect,i )can be postponed until conclusions on PRS processing capability in RRC INACTIVE state are reached in RAN1 Proposal 7: The summation-based approach for measurement requirements in case of multiple PFL in RRC_INACTIVE can be taken as the start point. |
| R4-2200636 | CMCC | Proposal 1: for positioning measurement delay requirements for inactive state, DRX cycle can be considered in measurement interval, for example, Tavailable could be the common multiple between TPRS and DRX cycle. Proposal 2: following RAN1 agreements, there is no impact on the Kcarrier for RRM measurement delay requirements with the introduction of PRS measurement in inactive state (i.e. only consider the frequency carriers indicated for RRM measurement) Proposal 3: for PRS measurement delay requirements in inactive state, replace CSSF with Kcarrier for inactive state measurement requirements, Kcarrier is the total number of configured carriers for positioning measurements, mobility measurements and CA measurements Proposal 4: it is proposed to consider latency reduction in RRC_INACTIVE state. Proposal 5: it is proposed to define two sets of PRS measurement period for inactive state, one set with 4-sample and the other set with reduced number of samples. |
| R4-2200760 | Qualcomm Incorporated | Proposal 1: UE requirements for UE Rx-Tx time difference measurements in RRC-INACTIVE state should be specified by RAN4. Proposal 2: The PRS measurement reporting delay requirements in RRC_INACTIVE should exclude the transmission time needed by SDT or the transition time to connected state to report positioning measurements. Proposal 3: Assume that the UE processes one PFL at a time in RRC_INACTVE, as in RRC_CONNECTED, and reuse the summation-based approach for the measurement period requirement with multiple PFLs in RRC_INACTIVE. Proposal 4: RAN4 should send an LS to RAN2/RAN3 notifying them that RAN4 has agreed that the PRS measurement period requirement will depend on the DRX cycle length configured for the UE and that, consequently, the response time requested by the LMF should be adjusted accordingly. RAN4 requests RAN2/RAN3 to introduce new signalling, if necessary. Proposal 5: In RRC_INACTIVE, the UE is required to measure and process PRS in one measurement occasion per DRX cycle. Observation 1: It would be beneficial for the LMF to be aware of the paging occasion (PO) parameters of the UE so that it can customize the PRS configuration for the UE to avoid collisions between PO and PRS. A preferred PRS configuration could be requested by the UE via on demand PRS. Proposal 6: Measurement requirements in RRC_INACTIVE do not apply to any PRS resources that overlap in time (including expected RSTD and uncertainty) with PO or SMTC configured for measurement by the network, or that are received within X ms (before or after) of PO or SMTC. X is FFS. Proposal 7: Depending on UE capability, the PRS measurement period length would be proportional to K=Kcarrier + 1 or K=1, where the latter would correspond to a UE that has a dedicated PRS processing engine. |
| R4-2201165 | OPPO | Proposal 1: Specify requirements for UE Rx-Tx time difference based on the framework of Rel-16 UE Rx-Tx time difference. Proposal 2: Not restrict PRS periodicity for defining requirements in RRC-INACTIVE state. Proposal 3: Not support reduced number of PRS samples in RRC-INACTIVE state. Proposal 4: Tavailable could be the least common multiple between TPRS and DRX cycle, where is TPRS calculated by the same method in Rel-16. Proposal 5: Send a LS to RAN2/3 asking for new signalling to indicate DRX cycle to LMF. Proposal 6: Replace CSSF with Kcarrier and Nlayer for RRC-INACTIVE state measurement requirements, only one positioning frequency layer is accounted into Kcarrier and Nlayer. Proposal 7: Support summation-based approach to calculate the total measurement period for multiple positioning frequency layers in RRC-INACTIVE state. Proposal 8: Not prioritize measurements for long-periodicity PRS resources in RRC-INACTIVE state. |
| R4-2201364 | vivo | Proposal 1: RRM requirements for UE Rx-Tx time difference measurements in RRC-INACTIVE state are specified. Proposal 2: RRM requirements for gNB Rx-Tx time difference measurements in RRC-INACTIVE state are specified. Proposal 3: Define baseline measurement requirements in INACTIVE state is prioritized. RAN4 could also consider to clarify UE behaviour for state transition if it is necessary. Proposal 4: No need to restrict TPRS in the spec because of requirements. Proposal 5: Requirements for reduced number of samples for RRC_INACTIVE state measurements should be defined. Proposal 6: UE RRM requirements for DL PRS-RSRP measurements and DL RSTD measurements in RRC-INACTIVE state are specified with summation-based approach for total frequency layers. Proposal 7: Requirements for INACTIVE state PRS measurements are defined under assumption that PRS measurements are performed once per DRX cycle, at least for DRX cycle no larger than 1.28s. Proposal 8: Replace CSSF with Kcarrier for inactive state measurement requirements, Kcarriers is the total number of configured carriers for mobility measurements and CA measurements plus one positioning frequency layer. |
| R4-2201400 | ZTE Corporation | Proposal 1: RAN4 shall send a LS out to let RAN2/RAN3 know that RAN4 assumes that the LMF would have knowledge of the DRX cycle, and ask for potential signalling support. Proposal 2: For reporting under RRC_INACTIVE mode, take small data transmission related requirements as baseline. |
| R4-2201640 | Huawei, HiSilicon | Proposal 1: Requirements for UE Rx-Tx measurements in Inactive state should be specified. Proposal 2: Whether a frequency layer for RRM measurement meets relaxed measurement criteria should not have any impact on PRS measurement. Proposal 3: Supporting SDT is not a condition for PRS measurement in Inactive state. Proposal 4: RAN4 to define PRS measurement when state transition occurs during the measurement period assuming UE may restart the PRS measurement. Proposal 5: No need to restrict TPRS for PRS measurement in Inactive state. Proposal 6: Introduce PRS processing window (as for MG-less PRS measurement in Connected state) for PRS measurement in Inactive state. UE is only required to measure PRS resource occasions within the window. Proposal 7: Requirements for PRS measurement in Inactive state apply when PRS resource is not overlapped with other DL signals/channels - no DL signals/channels is configured or scheduled during PPW and X symbols before and after PPW with DCI earlier than T+X symbols before the start of the PPW on any serving cell, for Capability 1A - no DL signals/channels is configured or scheduled during PPW and X symbols before and after PPW with DCI earlier than T+X symbols before the start of the PPW on serving cells in the same band as PRS, for Capability 1B - no DL signals/channels is configured or scheduled on any measured PRS symbol and X symbols before and after the PRS resource with DCI earlier than T+X symbols before the first measured symbol of the PRS resource on impacting serving cells, for Capability 2 - If PRS is in the initial BWP, X = 0; otherwise X is the number of symbols accounting for the retuning time. Proposal 8a: Define two sets of PRS measurement period for Inactive state, based on both 4-sample and reduced number of samples respectively. Proposal 8b: The requirements based on reduced number of samples apply provided that UE can support reduced number of samples for PRS measurement in Inactive state. Proposal 9: Use the summation based approach for defining measurement period for multiple PFLs. Proposal 10: RAN4 to follow RAN1 agreements regarding PRS processing capability for Inactive state. Proposal 11a: Tavailable for Inactive state measurement is defined as the LCM of Tprs, measurement window periodicity and DRX cycle. Proposal 11b: Teffect for Inactive state measurement is defined in the same way as in Rel-16 requirements. Proposal 12: Replace CSSF with Kcarrier for Inactive state measurement requirements, and Kcarrier should take one additional PFL into account. Proposal 13: Prioritizing measurements for long-periodicity PRS resources in Inactive state is not considered in Rel-17. Proposal 14: As a starting point, same side conditions is used for PRS measurement in Inactive state as in Connected state. Proposal 15: Re-use the PRS measurement reporting requirements of Connected state for Inactive state. |
| R4-2201983 | Nokia, Nokia Shanghai Bell | - To detect a potential collision of PRS reception with other DL channels, the UE determines the collision interval. The collision interval is based on RSTD assistance information (i.e. expected time of arrival of PRS for RSTD and RSTD uncertainty) and covers the time window for PRS reception and processing as well as RF tuning interruption. - For RF tuning interruption, this is based on RRT = 0.5 ms in FR1 and 0.25 ms in FR2. For calculating the collision interval this interruption occurs prior and after the PRS measurement. - In case of detected PRS collision with other DL channels, ordinary PRS measurement period requirements for RRC_INACTIVE state do not apply. - The PRS measurement period is extended to next PRS measurement, aligned to the DRX cycle, in case of detected collision to reach the same accuracy level. - All PRS measurement requirements in RRC_INACTIVE will be defined for gapless measurements. - Upon completion of measurement requirements for DL positioning methods, RAN4 investigates measurement requirements for periodic SRS and semi-persistent SRS in RRC_INACTIVE and develops corresponding measurement requirements for the latter. - RRM requirements for transmission of SRS for positioning do not apply in case of expected UL transmissions in other channels in the initial UL BWP in RRC_INACTIVE state. - RAN4 waits on RAN1 agreements on UE capabilities for RRC_INACTIVE for the definition of measurement capabilities for PRS in RRC_INACTIVE. - In case reduced number of samples is agreed for PRS measurements in RRC_CONNECTED state, it is reused for RRC_INACTIVE state. In this case two sets of PRS measurement periods are defined in RRC_INACTIVE state: one based on 4 PRS samples and one on the reduced number of PRS samples aligned to RRC_CONNECTED, subject to UE capability support. - The summation-based approach for total frequency layers is applied for RRC_INACTIVE state. - Measurement period requirements in RRC_INACTIVE depend on the DRX cycle length, at least for short DRX cycles. Hence RAN4 should liaise with RAN2/RAN3 that it is preferable that LMF has knowledge of the used DRX cycle to align PRS configuration for RRC_INACTIVE state. - Tavailable_PRS,I should take into account the DRX cycle length rather than MGRP for RRC_INACTIVE state. However, RAN4 should investigate if the LCM of the DRX cycle and the PRS periodicity is selected in all cases, or if the UE wakes up inbetween DRX cycles, in case of longer DRX cycles. - For determining the number of frequency layers to be measured in RRC_INACTIVE, take into account one additional PFL per measurement occasion. - Discuss and specify SDT-based signalling for NR positioning and respective RRM requirements within the SDT WI. |
| R4-2202017 | Ericsson | Type of positioning measurements in RRC_INACTIVE: - Observation #1: According to RAN1 agreements the following PRS measurement are applicable in RRC inactive state: RSTD, PRS-RSRP, PRS-RSRPP and UE Rx-Tx time difference. - Proposal #1: RAN4 to define PRS measurement requirements for RSTD, PRS-RSRP, PRS-RSRPP and UE Rx-Tx in RRC inactive state. PRS measurement requirements under relaxed measurement criteria: - Observation #2: Mobility related carrier configured with one or more relaxed measurement criteria, may also be configured as a positioning frequency layer (PFL) in RRC_INACTIVE state i.e. with same NR ARFCN. - Proposal #2: The UE is not be allowed to relax any PRS measurement on a PFL, which is also configured as a carrier frequency for mobility measurements and meet any relaxed measurement criterion. Applicability of PRS measurement requirements under PRS collisions: - Observation #3: In RRC_INACTIVE state, reception of DL PRS has lower priority compared to other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT). - Observation #4: It is unrealistic to quantify the extended PRS measurement period because it depends on network configuration/scheduling of DL signals/channels. - Proposal #3: It is sufficient to specify that longer PRS measurement period is expected due to the collision between DL signals/channels and PRS without any quantification of the extended PRS measurement period. PRS measurement requirements in RRC_INACTIVE: - Observation #5: Several issues related to latency reduction in RRC connected state are being discussed and analyzed. - Observation #6: PRS measurement requirements in RRC inactive state are fundamentally derived from the PRS measurement requirements in RRC connected state. - Proposal #4: Do not discuss latency reduction in RRC_INACTIVE state until all necessary agreements are made for latency reduction in RRC connected state. - Proposal #5: Existing Rel-16 side conditions in terms of PRS Ês/Iot under which the PRS measurements are applicable are reused for PRS measurements in RRC_INACTIVE state without latency reduction. - Proposal #6: The PRS measurement period scales with the total number of configured carriers (Kcarriers) for positioning measurements, mobility measurements and CA measurements. - Proposal #7: PRS measurement period in RRC_INACTIVE state is expressed as follows: TPRS_meas, i=Kcarrier*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i , Where: - Kcarrier is the total number of configured positioning frequency layers, NR inter-frequency carriers for mobility measurements, inter-RAT carriers for mobility measurements, NR inter-frequency carriers for CA measurements and inter-RAT carriers for CA measurements. - Tavailable_PRS,i= LCMTPRS,i,TDRX,i, the least common multiple between TPRS,i and TDRX, i. - Other parameters are the same as in the existing requirements for RSTD, PRS-RSRP and UE Rx-Tx time difference in Rel-16. Relation between PRS measurement and SDT: - Observation #7: UE can receive positioning assistance data from LMF using SDT procedure as well as non-SDT procedures. - Observation #8: UE can transmit positioning measurement data while in RRC inactive state only using SDT procedure. But the UE can also move to RRC connected state for transmit positioning measurement data to the LMF. - Observation #9: UE can also move to RRC connected state for transmit positioning measurement data to the LMF e.g. if no SDT resources are configured or if the UE does not support SDT. - Proposal #8: UE can perform PRS measurements and meet the PRS measurement requirements in RRC inactive state regardless of whether SDT is supported by the UE or whether SDT is configured for UE supporting SDT. PRS measurement requirements under RRC state transition: - Observation #10: UE configured with PRS measurements may change its RRC state any time. - Observation #11: RAN2 procedures allow the UE to transmit PRS measurement results to LMF for PRS measurements performed in RRC inactive state as well as in RRC connected state. - Observation #12: UE behavior related to PRS measurements under RRC state transition impacts the PRS measurement performance and requires RAN4 expertise. - Proposal #9: UE configured with and performing PRS measurement in RRC_INACTIVE state shall restart the PRS measurements after transition to RRC_CONNECTED state if the UE needs measurement gaps for the PRS measurement in the RRC_CONNECTED state. - Proposal #10: RAN4 should analyze at least the following 3 possible options for UE behavior under RRC state transition from RRC inactive state to RRC connected state: - Option 1: UE continues the PRS measurement - Option 2: UE restarts the PRS measurement - Option 3: UE stops the PRS measurement - Proposal #11: At least the option that the UE restarts the PRS measurement after the RRC state transition from RRC inactive state to RRC connected state is supported in Rel-17. PRS measurement requirements under cell change: - Observation #13: UE performing PRS measurements in RRC inactive state may perform cell reselection. - Observation #14: The number of carriers configured for measurement (Kcarruer) and DRX cycle may be different in the old serving cell before the cell reselection and in the new serving cell after the cell reselection. - Observation #15: UE performing PRS measurements in RRC inactive state may initiate cell selection to the selected PLMN according to section 4.2.2.2 in TS 38.133. - Proposal #12: The UE behaviour if the cell reselection occurs during the PRS measurement period is as follows: - The UE shall continue the RSTD measurement after the cell reselection - The UE shall continue the PRS-RSRP measurement after the cell reselection - The UE shall continue the PRS-RSRPP measurement after the cell reselection - The UE shall restart the UE Rx-Tx measurement after the cell reselection - Proposal #13: The measurement period for RSTD, PRS-RSRP and PRS-RSRPP, should be based on the longest of the Kcarriers and DRX cycles used among the old serving cell before the cell reselection and the new serving cell after the cell reselection. - Proposal #14: The UE upon initiating the cell selection for the selected PLMN, stops performing the PRS measurements and is not expected to meet the PRS measurement requirements. |
| R4-2201641 | Huawei, HiSilicon | CR on positioning measurements in RRC Inactive state |

Open issues summary

Sub-topic 2-1 Type of the PRS measurement requirements defined in RRC_INACTIVE state

Issue 2-1-1 Type of PRS measurement requirements to be defined in RRC_INACTIVE state.

Moderator: Companies are encouraged to provide views on each measurement (including UE/gNB Rx-Tx, PRS-RSRPP)

Proposals

- Option 1: (CATT)

  - RAN4 to wait for further agreements in RAN1 and RAN2 (if any) regarding UE/gNB Rx-Tx time difference measurement applicability in RRC inactive state.

- Option 2: (Intel, Qualcomm, OPPO, vivo, Huawei, Ericsson)

  - UE requirements for UE Rx-Tx time difference measurements in RRC-INACTIVE state should be specified by RAN4.

- Option 3: (vivo)

  - RRM requirements for gNB Rx-Tx time difference measurements in RRC-INACTIVE state are specified.

- Option 4: (Ericsson)

  - RAN4 to define PRS measurement requirements for PRS-RSRPP in RRC inactive state..

- Recommended WF

  - Need more discussion

| Issue 2-1-1 Type of PRS measurement requirements to be defined in RRC_INACTIVE state. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 2 and option 4. On option 1, in Rel-17 38.215 UE Rx-Tx measurement and PRS-RSRPP are already supported in RRC-INACTIVE state. On option 3, we think RRC state is from UE side, and there is no RRC state applicable for gNB, which can be also seen in 38.215. |
| Ericsson | We support Option 2 and Option 4. On Option 1: UE Rx-Tx time difference measurements is supported in RRC-INACTIVE state according to RAN1 specs. On Option 3: RRC states are related to UE not gNB. So gNB Rx-Tx time difference measurements in RRC-INACTIVE state is not relevant. |
| Intel | Option 2. For Option 4, it shall up to other issues (e.g. [222]) |
| CATT | We can compromise to option 2. But there is no requirement for gNB Rx-Tx time difference measurement is needed. Whether to define PRS-RSRPP in RRC_INACTIVE should be decided in RAN1. And for RAN4 it is too early to decide since the PRS-RSRPP measurement requirements in RRC_CONNECTED state are not defined. |
| OPPO | Prefer option 2. |
| Nokia | We support option 2 and option 4. For option 4 we propose to focus on first path PRS-RSRP. |
| Qualcomm | We support options 2 and 4. Same comment as Huawei regarding option 3. |
| vivo | Support Option 2 and Option 3 and option 4. For option 3, the intension is for gNB to measure gNB Rx-Tx time difference when SRS is transmitted in RRC inactive state by UE so that DL+UL positioning can be supported in INACTIVE state. We think there would be difference in gNB Rx-Tx time difference measurement accuracy between SRS is transmitted in CONNECTED state and INACTIVE state. The wording of Option 3 can be further polished. |

Issue 2-1-2 UE Rx-Tx measurement requirements in RRC_INACTIVE state if applicable

Proposals

- Option 1: (CATT, Intel, OPPO)

  - If UE Rx-Tx time difference measurement is applied in RRC_INACTIVE state, the framework of R16 UE Rx-Tx time difference measurement period can be the baseline.

- Recommended WF

  - Agree on option 1.

| Issue 2-1-2 UE Rx-Tx measurement requirements in RRC_INACTIVE state if applicable |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Support Recommended WF |
| Ericsson | We are fine with the WF |
| Intel | We are fine with the WF |
| CATT | Support the recommended WF. |
| OPPO | Support the recommended WF. |
| Nokia | Support the recommended WF. |
| Qualcomm | Support the recommended WF. |
| vivo | Support the recommended WF. |

Sub-topic 2-2 The PRS measurement requirements applicability in RRC_INACTIVE state

Issue 2-2-1 The PRS measurement requirements applicability in RRC_INACTIVE state regarding SDT.

Proposals

- Option 1: (CATT, Huawei, Ericsson)

  - Support of SDT is not a necessary condition for supporting positioning measurements in RRC_INACTIVE state.

  - RAN4 can independently work on positioning measurements in RRC_INACTIVE state and SDT requirements.(CATT)

- Option 2: (Nokia)

  - Discuss and specify SDT-based signalling for NR positioning and respective RRM requirements within the SDT WI.

- Recommended WF

  - Need more discussion

| Issue 2-2-1 The PRS measurement requirements applicability in RRC_INACTIVE state regarding SDT. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 1. On option 2, LPP messages for positioning measurement are no different from other data or signaling from SDT perspective, so we see no need to address them separately in SDT WI. |
| Ericsson | We support Option 1. On option 2: the SDT signaling is the same for reporting PRS measurement results or small data. No new impact on SDT due to positioning. |
| Intel | Option 1 |
| CATT | Support option 1. Option 2 is out of scope. The signaling is discussed in RAN2 and how to specify the SDT based requirements is not within this WI. |
| OPPO | Support option 1. |
| CMCC | Option 1. |
| Nokia | We can compromise to option 1. |
| Qualcomm | Option 1. |
| vivo | Option 1. |

Issue 2-2-2 The PRS measurement requirements applicability in RRC_INACTIVE state regarding state transition.

Proposals

- Option 1: (CATT, vivo)

  - RAN4 deprioritize the discussion for PRS measurement requirements when RRC state transition occurs during the measurement period.

- Option 2: (Intel, Huawei, Ericsson)

  - If there is status transition (e.g. RRC_INACTIVE RRC_CONNECT) within one successful measurement reporting period, UE measurement requirements can be based on the behaviour below.

    - UE restarts the PRS measurement

- Option 3: (Ericsson)

  - RAN4 should analyze at least the following 3 possible options for UE behavior under RRC state transition from RRC inactive state to RRC connected state:

    - Option 1: UE continues the PRS measurement

    - Option 2: UE restarts the PRS measurement

    - Option 3: UE stops the PRS measurement

- Recommended WF

  - Need more discussion

| Issue 2-2-2 The PRS measurement requirements applicability in RRC_INACTIVE state regarding state transition. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 2. We would like to ask clarification for option 1. Does it mean there would be no requirement if there is state transition during the measurement period? If this is the case, we are also fine with option 1. |
| Ericsson | We support Option 2 as this has least impact on RAN4 work. We are also fine with Option 1 (UE continues the PRS measurement) under Option 3. However this will need new requirements and given limited time may be challenging. On option 1: in reality the UE can do RRC state transition anytime and it can occur during PRS measurement period. Option 1 means that the UE will stop the PRS measurement, which is not good solution. |
| Intel | Option 2. |
| CATT | Support option 1. Suggest to focus on the UE behavior and PRS measurement requirements in RRC_INACTIVE state first. To Huawei’s question, yes if it is deprioritized, that means no requirements. |
| OPPO | Support option 1 and option 2. For option 2, no additional work for core requirement is required if UE restarts the PRS measurements. |
| Nokia | We support option 2. |
| Qualcomm | The UE should continue performing PRS measurements but the measurement period may be extended so that the UE may redo measurements that are interrupted by the state transition. In option 2, it’s not clear to us what is meant by “successful measurement reporting period.” FFS the behaviour for UE Rx-Tx when SRS configuration is affected by state transition. |
| vivo | Our preference is option 1 and RAN4 focus on completing baseline requirements. For state transition, it may also be fine to clarify UE behaviors. We don’t think specifying requirements for state transition is necessary. |

Issue 2-2-3 The PRS measurement requirements applicability in RRC_INACTIVE state regarding PRS periodicity.

Proposals

- Option 1: (CATT, OPPO, vivo, Huawei)

  - No need to restrict TPRS in the spec because of requirements.

- Recommended WF

  - Agree on option 1.

| Issue 2-2-3 The PRS measurement requirements applicability in RRC_INACTIVE state regarding PRS periodicity. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Support Recommended WF |
| Ericsson | Option 1 is fine |
| Intel | Support Recommended WF |
| CATT | Support the recommended WF. |
| OPPO | Support the recommended WF. |
| CMCC | OK with the recommended WF |
| Nokia | We support the recommended WF. |
| Qualcomm | Support the recommended WF. |
| vivo | Support the recommended WF. |

Issue 2-2-4 The PRS measurement requirements applicability in RRC_INACTIVE state regarding relaxed measurement criteria.

Proposals

- Option 1: (Intel, Huawei, Ericsson)

  - Whether a frequency layer for RRM measurement meets relaxed measurement criteria should not have any impact on PRS measurement.

- Recommended WF

  - Agree on option 1.

| Issue 2-2-4 The PRS measurement requirements applicability in RRC_INACTIVE state regarding relaxed measurement criteria. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Support Recommended WF |
| Ericsson | Fine with the WF |
| Intel | Support Recommended WF |
| CATT | Support the recommended WF. |
| OPPO | Support the recommended WF. |
| Nokia | We support the recommended WF. |
| Qualcomm | We understand that the proposal means that relaxed measurement criteria will not be applicable to positioning measurements in RRC_INACTIVE. If our understanding is correct, then we can support the recommended WF. |
| vivo | We support the recommended WF with same understanding as Qualcomm. |

Issue 2-2-5 The requirements applicability in RRC_INACTIVE state regarding collision of other functions.

Proposals

- Option 1: (Qualcomm)

  - Measurement requirements in RRC_INACTIVE do not apply to any PRS resources that overlap in time (including expected RSTD and uncertainty) with PO or SMTC configured for measurement by the network, or that are received within X ms (before or after) of PO or SMTC. X is FFS.

- Option 2: (Huawei)

  - Requirements for PRS measurement in Inactive state apply when PRS resource is not overlapped with other DL signals/channels

    - no DL signals/channels is configured or scheduled during PPW and X symbols before and after PPW with DCI earlier than T+X symbols before the start of the PPW on any serving cell, for Capability 1A

    - no DL signals/channels is configured or scheduled during PPW and X symbols before and after PPW with DCI earlier than T+X symbols before the start of the PPW on serving cells in the same band as PRS, for Capability 1B

    - no DL signals/channels is configured or scheduled on any measured PRS symbol and X symbols before and after the PRS resource with DCI earlier than T+X symbols before the first measured symbol of the PRS resource on impacting serving cells, for Capability 2

    - If PRS is in the initial BWP, X = 0; otherwise X is the number of symbols accounting for the retuning time.

- Option 3: (Nokia)

  - To detect a potential collision of PRS reception with other DL channels, the UE determines the collision interval. The collision interval is based on RSTD assistance information (i.e. expected time of arrival of PRS for RSTD and RSTD uncertainty) and covers the time window for PRS reception and processing as well as RF tuning interruption.

  - For RF tuning interruption, this is based on RRT = 0.5 ms in FR1 and 0.25 ms in FR2. For calculating the collision interval this interruption occurs prior and after the PRS measurement.

  - In case of detected PRS collision with other DL channels, ordinary PRS measurement period requirements for RRC_INACTIVE state do not apply.

  - The PRS measurement period is extended to next PRS measurement, aligned to the DRX cycle, in case of detected collision to reach the same accuracy level.

- Option 4: (Ericsson)

  - It is sufficient to specify that longer PRS measurement period is expected due to the collision between DL signals/channels and PRS without any quantification of the extended PRS measurement period.

- Recommended WF

  - Need more discussion

| Issue 2-2-5 The requirements applicability in RRC_INACTIVE state regarding collision of other functions. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | First, we understand no requirement apply when PRS is overlapping with other DL signals/channels. Option 1, 2 and 4 are aligned in this regards and we are fine with the wording in either of them. Next question is how to determine the overlapping. We support option 2 which is aligned with the determination for MG-less measurement in RRC-Connected. The values for T and X can be FFS. |
| Ericsson | We support option 1 and option 4. We do not see any need to have asymmetrical time offset X and T+X in Option 2. In our following approach can be used to progress this issue: - If there is collision/overlap between other DL signals/channels and PRS resources then the measurement period need to be extended. - Collision/overlap between other DL signals/channels and PRS resources occurs when: - Any other DL signals/channel occurs within the PRS resource or - Any other signals/channel occurs within X symbols before the PRS resource or - Any other signals/channel occurs within X symbols after the PRS resource. - X=FFS. |
| Intel | We can support Option 1 and 2 since there are quite similar. And in our view, the DL signals when UE monitored in RRC_INACTIVE shall not be overlapped by PRS. |
| CATT | Support option 4. It has been agreed that PRS has lower priority than other signals in RRC_INACTIVE state and in case of collision between PRS resources and other DL signals/channels, longer PRS measurement period is expected. No clear benefit to define the collision condition of PRS resources and even in RRC_CONNECTED state there is no such condition. |
| OPPO | Support option 1 and 4. The definition of “collision or overlap between PRS and other DL signal/channel” can be further discussed. For option 2, PRS processing window in RRC_INACTIVE mode is not agreed. |
| Nokia | We support option 3. To move forward, the new proposal by Ericsson can serve as baseline where the conditions for the overlap scenarios still need to be detailed. |
| Qualcomm | We support options 1 and 4. We believe there is substantial overlap between options 1 and 3. It may be beneficial to agree on the basic principle first. |
| vivo | Support proposal 4. When there exists the collision between DL signals/channels and PRS, longer PRS measurement period is necessary in RRC_INACTIVE state due to PRS has lower priority than other signals. We understand it may need further discussion on whether and how to define the overlapping. |

Issue 2-2-6 UE behavior for PRS measurement under cell change.

Proposals

- Option 1: (Ericsson)

  - The UE behaviour if the cell reselection occurs during the PRS measurement period is as follows:

    - The UE shall continue the RSTD measurement after the cell reselection

    - The UE shall continue the PRS-RSRP measurement after the cell reselection

    - The UE shall continue the PRS-RSRPP measurement after the cell reselection

    - The UE shall restart the UE Rx-Tx measurement after the cell reselection

- Recommended WF

  - Need more discussion

| Issue 2-2-6 UE behavior for PRS measurement under cell change. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We are fine with the first 3 bullets in option 1, but the impacts on the requirements should be discussed in Issue 2-2-7. On the 4th bullet, RAN1/2 has agreed that UE would stop SRS transmission after cell reselection, so the Rx-Tx measurement should be stopped by default, and there is no need to capture this in RAN4 spec. |
| Ericsson | We support Option 1. To HW: Agree TA becomes invalid after cell change. But UE gets new SRS configuration in the new cell. New SRS means also new TA since any UL allocation is associated with TA. It is the same as in HO, “ “The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.” So for UE Rx-Tx, the rule can be modified to as follows: - “The UE shall restart the UE Rx-Tx measurement after the cell reselection and after SRS reconfiguration on the target cell is complete.” |
| Intel | Support the first 3 bullets in option 1, |
| CATT | Basically we think the PRS measurement will not be impacted by cell reselection and UE shall continue the PRS measurement. But we suggest to decide it after the requirements are clearer. |
| Nokia | We support option 1 and the additional clarification by Ericsson. |
| Qualcomm | Similar comment as Huawei. In general, we suggest that impact of cell change to measurements, while it needs to be discussed, is not the highest priority at this point. FFS for now. |
| vivo | The UE behavior due to cell reselection can be further discussed. |

Issue 2-2-7 PRS measurement requirements applicability under cell change.

Proposals

- Option 1: (Ericsson)

  - The measurement period for RSTD, PRS-RSRP and PRS-RSRPP, should be based on the longest of the Kcarrier and DRX cycles used among the old serving cell before the cell reselection and the new serving cell after the cell reselection.

  - The UE upon initiating the cell selection for the selected PLMN, stops performing the PRS measurements and is not expected to meet the PRS measurement requirements.

- Recommended WF

  - Need more discussion

| Issue 2-2-7 PRS measurement requirements applicability under cell change. |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We do not support the first bullet and we prefer to not define exact requirements for the cell change case. We can either allow UE to re-start the PRS measurement or simply state that a longer measurement period is expected. The second bullet should be first discussed in RAN1/2 to clarify the UE behavior. |
| Ericsson | We support Option 1: - On first bullet: For RSTD, PRS-RSRP and PRS-RSRPP, if the UE continues the measurement then only DRX and the Kcarrier are changed. This means measurement period can easily be defined with DRX and Kcarriers of the two cells. No other parameter changes. - On second bullet: It is both RAN2 issue and RAN4 issue. Section 4.2.2.2, 38.133 defines UE requirements/behaviour when UE does not meet serving cell does not fulfil cell selection criteria S. So RAN4 can provide input to RAN2 the feasible UE behaviour/requirement for PRS measurement when UE does not meet cell selection criteria S. |
| Intel | Can be FFS upon the UE behavior is agreed in issue 2-2-6 |
| CATT | Same as issue 2-2-6. Suggest to decide it after the requirements are clearer. |
| Nokia | We agree with Intel, can be FFS. |
| Qualcomm | FFS. See response to issue 2-2-6. |
| vivo | FFS |

Sub-topic 2-3 SRS measurement  requirements in RRC_INACTIVE state

Issue 2-3-1 Whether to define SRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (Nokia)

  - Upon completion of measurement requirements for DL positioning methods, RAN4 investigates measurement requirements for periodic SRS and semi-persistent SRS in RRC_INACTIVE and develops corresponding measurement requirements for the latter.

  - RRM requirements for transmission of SRS for positioning do not apply in case of expected UL transmissions in other channels in the initial UL BWP in RRC_INACTIVE state.

- Recommended WF

  - Need more discussion

| Issue 2-3-1 Whether to define SRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We need some clarification on option 1. Is the first bullet about gNB measurement requirements? In our view RRC state is not applicable for gNB, so there seems no need to define new requirements for gNB. On the second bullet, we understand the priority between SRS and other UL channels should be discussed in RAN1/2. |
| Ericsson | We agree with Huawei that RRC states are for UE not for gNB. So RRC inactive has no impact on gNB measurements. 2nd bullet is related to RAN1. |
| Intel | Agree with HW’s view. |
| CATT | We think there is no gNB requirement needed in RRC_INACTIVE state. |
| OPPO | Agree with Huawei’s view that gNB measurements is not related with RRC state for UE. Besides, periodic and semi-persistent SRS may have impact on measurement period. But RAN4 did not defined core requirements for gNB measurement. The accuracy requirements and report mapping in RRC connected mode can be reused. |
| Nokia | We support option 1. First bullet is related to higher priority in the WID for specifying DL positioning support in RRC_INACTIVE state. Thus, RAN4 should use more time for progressing RRM requirements for DL methods than for UL methods. In fact, the UE is in RRC_INACTIVE, not gNB. We reused the title of the sub-topic. Regarding second bullet, this is proposed following RAN1 #107-e agreements below: Agreement - The following options are supported for SRS for positioning transmission by RRC_INACTIVE UEs: - Option 1: - Subject to UE capability (which is a prerequisite for option 2), a UE may be configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP. - Option 2: - Subject to UE capability, a UE may be configured with an SRS for Positioning where the following parameters are additionally configured for the transmission of the SRS for Positioning during the RRC_INACTIVE state: frequency location and bandwidth, SCS, CP length. - The UE shall not transmit the SRS for Positioning when it is expected to perform UL transmissions in the initial UL BWP in RRC_INACTIVE state.. |
| Qualcomm | Agree with Huawei’s comment. |
| vivo | Support 1st bullet of Option 1. There should be applicable gNB measurement requirements when UE is transmitting SRS in INACTIVE state. |

Sub-topic 2-4 Measurement period requirements for positioning measurement in RRC_INACTIVE state

Issue 2-4-1 Whether to support the reduced number of samples in RRC_INACTIVE state

Proposals

- Option 1: (CATT, CMCC, vivo, Nokia)

  - Yes.

- Option 2: (OPPO, Ericsson)

  - No.

- Option 3: (Intel, Huawei)

  - upon UE capability

- Recommended WF

  - Need more discussion

| Issue 2-4-1 Whether to support the reduced number of samples in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 3 or option 2 Reduced sample number should not be mandated for RRC-Inactive, since low latency is not the target for measurement in RRC-Inactive. |
| Ericsson | We are fine to support Option 1 but only after the work on reduced number of samples is completed for RRC connected state |
| Intel | Option 3. The less samples are more important in RRC_INACTIVE for power saving. So we strongly prefer PRS measurement with less sample can be supported in RRC_INACTIVE as RRC_CONNECT also. |
| CATT | Support option 1. If the reduced number of samples is agreed in the latency reduction of RRC_CONNECTED state, it can be reused in RRC_INACTIVE state. |
| OPPO | Prefer option 2 at this stage. And we are open to support the reduced number of samples if the related work is finished. |
| CMCC | Option 1. Considering the long DRX cycle, reduced number of samples provide benefit for the PRS measurement in inactive state. The latency reduction is under discussion, and the conclusion can be used directly for inactive PRS measurement. |
| Nokia | We support option 1. |
| Qualcomm | A reduced number of samples may be supported if a separate UE capability for RRC_INACTIVE is introduced (do not reuse the capability for low-latency positioning in RRC_CONNECTED). Otherwise, we support option 2. |
| vivo | We support option 1. Since DRX cycle in INACTIVE state is much longer than MG periodicity, reducing measurement delay is necessary. |

Issue 2-4-2 Whether to define two set of PRS measurement requirements based on number of samples in RRC_INACTIVE state

Proposals

- Option 1: (CATT)

  - No.

- Option 2: (CMCC, Huawei, Nokia)

  - Yes.

    - Define two sets of PRS measurement period for inactive state, one set with 4-sample and the other set with reduced number of samples.

- Recommended WF

  - Need more discussion

| Issue 2-4-2 Whether to define two set of PRS measurement requirements based on number of samples in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 2, but depending on Issue 2-4-1. If option 2 in Issue 2-4-1 is agreed, then we can agree to option 1 and there will be one set of requirements based on 4-sample. |
| Ericsson | Depends on issues 2-4-1. If requirements based on reduced samples are introduced then two sets of requirements will be needed. One reason is that at least some of the conditions will be different. So we support Option 2 |
| Intel | Option 2, but depending on Issue 2-4-1 |
| CATT | Can compromise to option 2. Based on issue 2-4-1 we agree that the requirements for reduced number of samples are needed. But we think for measurement period requirements, the difference is only the scaling factor, and the requirements for 4-samples and reduced samples can be defined as the same formula. |
| OPPO | Depending on Issue 2-4-1 |
| CMCC | Option 2. Same comments as for Issue 2-4-1 |
| Nokia | We support option 2. |
| Qualcomm | We can support option 2 assuming that there will be a separate UE capability for low-latency in RRC_INACTIVE. FFS if the reduced number of PRS samples would be the same in connected state and inactive state. |
| vivo | We are fine with option 1. However, it needs to introduce UE capability. |

Issue 2-4-3 Approach used for PRS measurement requirements of multiple positioning frequency layers in RRC_INACTIVE state

Proposals

- Option 1: (CATT, Qualcomm)

  - Summation-based approach is used based on the assumption that only one PFL can be measured per DRX cycle.

- Option 1a: (Intel, OPPO, vivo, Huawei, Nokia)

  - Summation-based approach is used.

- Recommended WF

  - Need more discussion

| Issue 2-4-3 Approach used for PRS measurement requirements of multiple positioning frequency layers in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 1a, and by option 1a we assume the measurement period for multiple PFLs will be the sum of measurement period of each individual PFLs, same as in Rel-16. We are not sure if option 1 means the same, so could proponents of option 1 please explain how the requirements for multiple PFLs will be defined following option 1? |
| Ericsson | Option 1a |
| Intel | Option 1a. The other conditions as Option 1 can be FFS. |
| CATT | Option 1. The two options are similar, i.e. both of them are suggesting summation-based approach. But the assumptions under which the requirements are defined should be clarified. Without MG limitation, if the PRS resources of multiple layers are not overlapped, UE can process multiple PFLs simultaneously. In this case, there is no need to use summation-based approach, instead, maximum-based approach can be used. So the summation-based approach should only be used under the assumption that UE only process one PFL in each measurement window. |
| OPPO | Support option 1a. |
| Nokia | We support option 1a. |
| Qualcomm | Option 1 |
| vivo | Option 1a. |

Issue 2-4-4 UE processing capability for PRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (CATT, Intel, Huawei)

  - Whether to define new UE processing capability in RRC_INACTIVE state is within RAN1 scope

- Recommended WF

  - Agree on option 1.

| Issue 2-4-4 UE processing capability for PRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Support Recommended WF |
| Ericsson | Support Recommended WF |
| Intel | Support Recommended WF |
| CATT | Support the recommended WF. |
| OPPO | Support the recommended WF |
| CMCC | Support the recommended WF |
| Nokia | We support the recommended WF. |
| Qualcomm | Option 1. If there are different PRS measurement requirements in connected and inactive, there should be a separate capability for inactive state. |
| vivo | Same view as Qualcomm. If different PRS measurement requirements are specified, then corresponding UE capability should be in RAN4 scope. |

Issue 2-4-5 How to consider DRX cycle for PRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (CATT, CMCC)

  - DRX cycle can be considered in the available PRS periodicity for the PRS measurement in RRC_INACTIVE state I,e. Tavailable_PRS is the least common multiple between TPRS and DRX cycle.

- Option 2: (Qualcomm, vivo)

  - The UE is required to measure and process PRS in one measurement occasion per DRX cycle.

- Option 3: (Nokia)

  - RAN4 should investigate if the LCM of the DRX cycle and the PRS periodicity is selected in all cases, or if the UE wakes up inbetween DRX cycles, in case of longer DRX cycles.

- Recommended WF

  - Need more discussion

| Issue 2-4-5 How to consider DRX cycle for PRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We can support the principle of option 1, i.e. to account for DRX cycle in Tavailable. On the exact definition of Tavailable, we suggest to further consider PPW and this depends on the outcome of Issue 2-4-11. On option 2, it could happen that PRS periodicity is larger than DRX cycle. On option 3, we think UE should not be required to wake up more than once per DRX cycle considering that main purpose of RRC-Inactive measurement is for UE power saving. |
| Ericsson | Option 1 |
| Intel | Option 1 as it can be more generic. |
| CATT | Support option 1. But we agree to clarify the UE behavior whether UE wake up in-between DRX cycles. We think it is possible up to implementation, but we are fine to define requirements based on the assumption that UE only measure when DRX is on. |
| OPPO | Option 1 |
| CMCC | Option 1. For HW’s comments on the PPW, we have different understanding. In our view, only TPRS and DRX cycle are considered for Tavailable for PRS measurement in inactive state, no need to consider PPW. According to RAN1 design, PPW is used for measurement without MG in connected. Whether PPW is also used for PRS measurement in inactive state depends on RAN1. |
| Nokia | We can compromise to option 1. We agree with CATT, the wake-up in between DRX active periods could be UE implementation specific. |
| Qualcomm | We support option 2. To Huawei’s comment: yes, PRS periodicity may be larger than DRX cycle but the UE may not be able to measure all PRS resources contained within one PRS period in one DRX cycle. It would depend on UE capability. |
| vivo | Support option 2. |

Issue 2-4-6 Tavailable_PRS,i calculation for PRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (CATT, CMCC, OPPO, Ericsson)

  - Tavailable_PRS,i could be the least common multiple between TPRS and DRX cycle.

- Option 2: (Huawei)

  - Tavailable_PRS,i for Inactive state measurement is defined as the LCM of Tprs, measurement window periodicity and DRX cycle.

- Recommended WF

  - Need more discussion

| Issue 2-4-6 Tavailable_PRS,i calculation for PRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | The difference between the two options is whether to account for PPW-RP. As PPW is discussed in Issue 2-4-11, we suggest to come back to Issue 2-4-6 after concluding 2-4-11. |
| Ericsson | Option 1. |
| Intel | Option 1. Can be also FFS. |
| CATT | Support option 1. Regarding PRS processing window, we think it is defined in RAN1 and should be decided in RAN1 whether to introduce it to RRC_INACTIVE state. |
| OPPO | Option 1 |
| CMCC | Option 1. For option 2, as commented in Issue 2-4-5, according to RAN1 design, PPW is used for measurement without MG in connected. Whether PPW is also used for PRS measurement in inactive state depends on RAN1. |
| Nokia | We support option 1. |
| Qualcomm | More discussion needed. See response to issue 2-4-5. |
| vivo | Further discussion. |

Issue 2-4-7 TPRS,I calculation for PRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (CATT, OPPO, Ericsson)

  - The same approach as R16 can be used.

- Recommended WF

  - Agree on option 1.

| Issue 2-4-7 TPRS,i calculation for PRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Support Recommended WF |
| Ericsson | Option 1. |
| Intel | Support Recommended WF |
| CATT | Support the recommended WF. |
| OPPO | Support the recommended WF. |
| Nokia | We support the recommended WF. |
| Qualcomm | Support option 1 but the following note should be removed: Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered. |
| vivo | Option 1 is fine. |

Issue 2-4-8 Teffct,I calculation for PRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (CATT, Huawei, Ericsson)

  - The same approach as R16 can be used.

- Option 2: (Intel)

  - Postponed until conclusions on PRS processing capability in RRC INACTIVE state are reached in RAN1.

- Recommended WF

  - Need more discussion

| Issue 2-4-8 Teffct,i calculation for PRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 1. We understand {N,T} capability is also applicable for PRS measurement in RRC-Inactive although the reported values can be different from RRC-Connected, so there is no impact to RAN4 requirements. |
| Ericsson | Option 1. |
| Intel | Option 2. As our understanding, the processing capability can be difference with that in RRC_CONNECTED. We can check RAN1’s inputs further. |
| CATT | Support option 1. Agree with Huawei’s view. Even there are new values are introduced, the principle of Teffct,i definition is the same. |
| OPPO | Option 1, we are also fine with option 2. |
| Nokia | We support option 1. |
| Qualcomm | Option 2. The Rel-16 approach can be the baseline but we should wait for RAN1 to finalize the new UE capability. |
| vivo | Option 1. |

Issue 2-4-9 How to define Kcarrier for PRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (vivo, Huawei, Nokia, CATT)

  - Replace CSSF with Kcarrier for inactive state PRS measurement requirements, Kcarriers is the total number of configured carriers for mobility measurements and CA measurements plus one positioning frequency layer.

- Option 2: (CMCC, Ericsson)

  - For PRS measurement delay requirements in inactive state, replace CSSF with Kcarrier for inactive state measurement requirements, Kcarrier is the total number of configured carriers for positioning measurements, mobility measurements and CA measurements

- Option 2a: (CMCC)

  - There is no impact on the Kcarrier for RRM measurement delay requirements with the introduction of PRS measurement in inactive state (i.e. only consider the frequency carriers indicated for RRM measurement).

- Option 3: (Qualcomm)

  - Depending on UE capability, the PRS measurement period length would be proportional to K=Kcarrier  + 1 or  K=1, where the latter would correspond to a UE that has a dedicated PRS processing engine.

- Option 4: (OPPO)

  - Replace CSSF with Kcarrier and Nlayer for RRC-INACTIVE state measurement requirements, only one positioning frequency layer is accounted into Kcarrier and Nlayer.

- Recommended WF

  - Need more discussion

| Issue 2-4-9 How to define Kcarrier for PRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We can support option 4 which is more comprehensive than option 1. On option 2, it counts total number of PFLs which is unnecessary if sum approach is used for multi-PFL requirements. On option 2a, we understand same Kcarrier would apply for PRS and RRM measurement as they are sharing the measurement resource. On option 3, we are open to consider the capability, but one question for clarification: if we take K=1 for PRS, is UE assumed to perform parallel measurement on PRS and SSB layers when PRS resource is overlapped with SSB? |
| Ericsson | We support Option 2 but Option 1 is also fine if sum approach is used for multiple PFLs (issue 2-4-3). In Option 4: Nlayer is not clear. Which carriers are included? |
| Intel | Option 1 and 2. |
| CATT | Support option 1. But we agree to clarify the processing engine of PRS measurement, and we think if PRS measurement is performed with the dedicated engine i.e. K=1, the measurement period need also to be extended when the PRS resources is overlapped with other signals. |
| OPPO | Option 4. The method in option 4 is the same as that in option 1. Option 4 additionally considers Nlayer. For cell re-selection in RRC_inactive mode, If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then only inter-frequency layers of higher priority should be measured and the requirements defined in clause 4.2.2.7 is shown below. In this scenario, we think PRS layer should be counted into Nlayer rather Kcarrier. - 4.2.2.7 General requirements The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds, where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information. For a UE configured with early measurement reporting, while T331 is running, Nlayers is the combined total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information and carriers configured for idle mode CA measurements. |
| CMCC | Support option 1, option 2, and option 2a. For PRS measurement delay requirements, both option 1 and option 2 are ok for us. Option 2a is for RRM measurement delay requirements, we just want to clarify that RRM measurement is not impacted by the PRS measurement, which is align with RAN1 agreement that the reception of other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT) is prioritized if collided with PRS resources in RRC_INACTIVE state. |
| Nokia | We support option 1. Option 3 with requirements for K=1 can be FFS. |
| Qualcomm | We support option 3. Just to clarify, in the proposal it is assumed that the definition of Kcarrier is not changed to include one additional layer for positioning. We’re also OK if other companies prefer to update the definition of Kcarrier (updated Kcarrier = current Kcarrier + 1). We will consider the modification proposed in option 4. To Huawei’s question: even with K=1 the UE would not be required to process PRS and SSB in parallel if they collide. |
| vivo | If summation-based approach is agreeable, the Kcarrier should be option 1. |

Issue 2-4-10 Measurement requirements for long periodicity PRS in RRC_INACTIVE state

Proposals

- Option 1: (CATT, OPPO, Huawei)

  - Prioritizing measurements for long-periodicity PRS resources in RRC INACTIVE state is not necessary.

- Recommended WF

  - Agree on option 1.

| Issue 2-4-10 Measurement requirements for long periodicity PRS in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Support Recommended WF |
| Ericsson | OK with WF |
| Intel | Support Recommended WF |
| CATT | Support the recommended WF. |
| OPPO | Support the recommended WF. |
| CMCC | OK with the recommended WF |
| Nokia | We support the recommended WF. |
| Qualcomm | Support the recommended WF. |
| vivo | Support the recommended WF. |

Issue 2-4-11 Clarification on the applicability of PRS processing window in RRC_INACTIVE state

Proposals

- Option 1: (CATT)

  - Send LS to RAN1 to ask for the clarification whether the PRS processing window is applied for the PRS measurement in RRC_INACTIVE state

- Option 2: (Huawei)

  - Introduce PRS processing window (as for MG-less PRS measurement in Connected state) for PRS measurement in Inactive state. UE is only required to measure PRS resource occasions within the window.

- Recommended WF

  - Need more discussion

| Issue 2-4-11 Clarification on the applicability of PRS processing window in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Option 2. We are fine to send the LS, but we understand RAN4 could make decision on the need of PPW for RRC-Inactive measurement and inform RAN1/2. Of course, if RAN1/2 have any concern, this can be revisited in RAN4. Technically, UE cannot measure PRS resources distributed anywhere in the time domain, and a localized distribution is preferable from both UE and NW perspective. This is same as PRS measurement in RRC-Connected. |
| Ericsson | Option 1 is fine |
| Intel | Option 1 |
| CATT | Option 1. We think use the location of PRS processing window and the location of PRS resources can have the same effect to make UE to find PRS resources and perform measurement. We can say in the LS that RAN4 see the benefit on introducing PRS processing window, but whether to introduce it is up to RAN1 decision. |
| OPPO | Option 1. RAN1 conclusion is needed. |
| CMCC | OK with option 1. According to RAN1 design, PPW is used for measurement without MG in connected. Whether PPW is also used for PRS measurement in inactive state depends on RAN1. Better to send LS to RAN1. |
| Nokia | We support option 1. |
| Qualcomm | Option 1. Our understanding is that PPW is not directly applicable to inactive state. We support sending LS to RAN1 if clarification is needed. |
| vivo | Option 1. |

Issue 2-4-12 Whether to send LS to RAN2/3 about DRX cycle using in RRC_INACTIVE state

Proposals

- Option 1: (Qualcomm, OPPO, ZTE, Nokia)

  - RAN4 should send an LS to RAN2/RAN3 notifying them that RAN4 has agreed that the PRS measurement period requirement will depend on the DRX cycle length configured for the UE and that, consequently, the response time requested by the LMF should be adjusted accordingly. RAN4 requests RAN2/RAN3 to introduce new signalling, if necessary.

- Option 2: (CATT)

  - It is not necessary to inform the DRX cycle to LMF even if it is considered in the PRS measurement requirements in RRC_INACTIVE state.

- Recommended WF

  - Need more discussion

| Issue 2-4-12 Whether to send LS to RAN2/3 about DRX cycle using in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We can support option 2. In our understanding, LMF is aware that PRS measurement may be performed when UE is in RRC-Inactive, and in this case, LMF should be prepared for the potential longer measurement delay. Therefore, it is more like a NW implementation issue. |
| Ericsson | RAN4 can inform RAN2/RAN3 and possibly SA2 that PRS measurement period is function of DRX cycle. In RRC inactive state the UE can also be configured with UE specific DRX cycle which involves SA2. |
| Intel | Option 2. In our understanding, this is NW implementation and optimization. |
| CATT | Support option 2. Agree with Huawei and Intel it is more related to NW implementation. |
| OPPO | Support option 1. |
| Nokia | We support option 1. |
| Qualcomm | We support option 1. Could the proponents of option 2 explain how the feature would work without LMF knowledge of UE DRX configuration? |
| vivo | Further discussion is needed on necessity of the LS. |

Sub-topic 2-5 Performance requirements for positioning measurement in RRC_INACTIVE state

Issue 2-5-1 Side condition for PRS measurement requirements in RRC_INACTIVE state

Proposals

- Option 1: (CATT, Huawei, Ericsson)

  - The side conditions in terms of PRS Ês/Iot defined in RRC_CONNECTED state can be applied for PRS measurements in RRC_INACTIVE state under the condition that the same number of samples is used.

- Recommended WF

  - Agree on option 1.

| Issue 2-5-1 Side condition for PRS measurement requirements in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | Support Recommended WF |
| Ericsson | Support Recommended WF |
| Intel | Can be FFS especially the condition for the less samples are not clear. |
| CATT | Support the recommend WF. For the reduced number of samples, the side condition can also be reused if it is decided, |
| OPPO | Support Recommended WF |
| Nokia | We support the recommended WF. |
| Qualcomm | Support the recommended WF. |
| vivo | Support the recommended WF. |

Sub-topic 2-6 Measurement reporting requirements for positioning measurement in RRC_INACTIVE state

Issue 2-6-1 Measurement reporting requirements for positioning measurement in RRC_INACTIVE state

Proposals

- Option 1: (ZTE)

  - For reporting under RRC_INACTIVE mode, take small data transmission related requirements as baseline

- Option 2: (Qualcomm)

  - The PRS measurement reporting delay requirements in RRC_INACTIVE should exclude the transmission time needed by SDT or the transition time to connected state to report positioning measurements.

- Option 2: (Huawei)

  - Re-use the PRS measurement reporting requirements of Connected state for Inactive state.

- Recommended WF

  - Need more discussion

| Issue 2-6-1 Measurement reporting requirements for positioning measurement in RRC_INACTIVE state |  |
| --- | --- |
| Company | Comments |
| XXX |  |
| Huawei | We can support option 2 from QC. |
| Ericsson | Need more discussion. There are different options to report the PRS measurement results (e.g. via SDT (different SDT methods), by going into connected state) which need further analysis. |
| Intel | Can be FFS.And in our view, this issue is also upon many issues (e.g. 2-2-1, 2-4-1,2-4-6,….) |
| CATT | Can support option 2. |
| Nokia | We share Intel’s view. |
| Qualcomm | Option 2 |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201641 (Huawei) (Structure of positioning measurements in RRC Inactive state) | Huawei: this CR can be postponed, and we can work based on the draft CR split discussed in email 222. |
|  | Company B |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 2-1 Type of the PRS measurement requirements defined in RRC_INACTIVE state

|  | Status summary |
| --- | --- |
| Issue 2-1-1 | Issue 2-1-1 Type of PRS measurement requirements to be defined in RRC_INACTIVE state. Moderator: In moderator’s understanding, the measurement requirements in this part means the core requirements not accuracy requirements. Tentative agreements: - RAN4 to define PRS measurement requirements for UE Rx-Tx time difference measurement in RRC inactive state. - RAN4 to define PRS measurement requirements for PRS-RSRPP measurement in RRC inactive state. - RAN4 not to define PRS measurement requirements for gNB Rx-Tx time difference measurement in RRC inactive state. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-1-2 | Issue 2-1-2 UE Rx-Tx measurement requirements in RRC_INACTIVE state if applicable Tentative agreements: - If UE Rx-Tx time difference measurement is applied in RRC_INACTIVE state, the framework of R16 UE Rx-Tx time difference measurement period can be the baseline. Candidate options: None. Recommendations for 2nd round: None. |

Sub-topic 2-2 The PRS measurement requirements applicability in RRC_INACTIVE state

|  | Status summary |
| --- | --- |
| Issue 2-2-1 | Issue 2-2-1 The PRS measurement requirements applicability in RRC_INACTIVE state regarding SDT. Tentative agreements: - Support of SDT is not a necessary condition for supporting positioning measurements in RRC_INACTIVE state. - RAN4 can independently work on positioning measurements in RRC_INACTIVE state and SDT requirements. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-2-2 | Issue 2-2-2 The PRS measurement requirements applicability in RRC_INACTIVE state regarding state transition. Tentative agreements: None. Candidate options: - Option 1: (CATT, vivo, OPPO) - RAN4 deprioritize the discussion for PRS measurement requirements when RRC state transition occurs during the measurement period and focus on the baseline requirements in RRC_INACTIVE state. - Option 2: (Intel, Huawei, Ericsson, OPPO, Nokia) - If there is status transition (e.g. RRC_INACTIVE RRC_CONNECT) during the measurement period, UE measurement requirements can be based on the behaviour below. - UE restarts the PRS measurement - Option 3: (Qualcomm) - If there is status transition (e.g. RRC_INACTIVE RRC_CONNECT) during the measurement period, - UE continue the PRS measurement - The measurement period requirements will be extended - FFS the behaviour for UE Rx-Tx when SRS configuration is affected by state transition. Recommendations for 2nd round: Further discuss. |
| Issue 2-2-3 | Issue 2-2-3 The PRS measurement requirements applicability in RRC_INACTIVE state regarding PRS periodicity. Tentative agreements: - No need to restrict TPRS in the spec because of requirements. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-2-4 | Issue 2-2-4 The PRS measurement requirements applicability in RRC_INACTIVE state regarding relaxed measurement criteria. Tentative agreements: - Whether a frequency layer for RRM measurement meets relaxed measurement criteria should not have any impact on PRS measurement. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-2-5 | Issue 2-2-5 The requirements applicability in RRC_INACTIVE state regarding collision of other functions. Tentative agreements: - Longer PRS measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_INACTIVE state. - Collision/overlap between other DL signals/channels and PRS resources in RRC_INACTIVE state occurs when: - Any other DL signals/channel occurs within the PRS resource or - Any other signals/channel occurs within X symbols before the PRS resource or - Any other signals/channel occurs within X symbols after the PRS resource. - X=FFS. Candidate options: None. Recommendations for 2nd round: Further discuss the value of X. |
| Issue 2-2-6 | Issue 2-2-6 UE behavior for PRS measurement under cell change. Tentative agreements: - The UE behaviour if the cell reselection occurs during the PRS measurement period is as follows: - The UE shall continue the RSTD measurement after the cell reselection - The UE shall continue the PRS-RSRP measurement after the cell reselection - The UE shall continue the PRS-RSRPP measurement after the cell reselection Candidate options: FFS the UE behaviour if the cell reselection occurs during the UE Rx-Tx measurement period: - Option 1: (Ericsson) - The UE shall restart the UE Rx-Tx measurement after the cell reselection - Option 2: (Huawei) - The UE shall stop the UE Rx-Tx measurement after the cell reselection Recommendations for 2nd round: Further discuss the UE behavior for UE Rx-Tx measurement under cell change. |
| Issue 2-2-7 | Issue 2-2-7 PRS measurement requirements applicability under cell change. Tentative agreements: None. Candidate options: Proposals - Option 1: (Ericsson) - The measurement period for RSTD, PRS-RSRP and PRS-RSRPP, should be based on the longest of the Kcarrier and DRX cycles used among the old serving cell before the cell reselection and the new serving cell after the cell reselection. - The UE upon initiating the cell selection for the selected PLMN, stops performing the PRS measurements and is not expected to meet the PRS measurement requirements. - Option 2: (Huawei) - Do not define exact requirements for the cell change case. - The UE behaviour upon initiating the cell selection for the selected PLMN is within RAN1/2 scope. Recommendations for 2nd round: Further discuss. |

Sub-topic 2-3 SRS measurement requirements in RRC_INACTIVE state

|  | Status summary |
| --- | --- |
| Issue 2-3-1 | Issue 2-3-1 Whether to define SRS measurement requirements in RRC_INACTIVE state Tentative agreements: None. Candidate options: - Option 1: (Nokia) - Upon completion of measurement requirements for DL positioning methods, RAN4 investigates measurement requirements for periodic SRS and semi-persistent SRS in RRC_INACTIVE and develops corresponding measurement requirements for the latter. - RRM requirements for transmission of SRS for positioning do not apply in case of expected UL transmissions in other channels in the initial UL BWP in RRC_INACTIVE state. - Option 2: - No gNB measurement requirements are defined in RRC_INACTIVE state. Recommendations for 2nd round: Further discuss. |

Sub-topic 2-4 Measurement period requirements for positioning measurement in RRC_INACTIVE state

|  | Status summary |
| --- | --- |
| Issue 2-4-1 | Issue 2-4-1 Whether to support the reduced number of samples in RRC_INACTIVE state Tentative agreements: None. Candidate options: - Option 1: (CATT, CMCC, vivo, Nokia, Ericsson) - Yes. - Option 2: (OPPO, Huawei, Qualcomm) - No. - Option 3: (Intel, Huawei, Qualcomm) - upon UE capability Recommendations for 2nd round: Further discuss. |
| Issue 2-4-2 | Issue 2-4-2 Whether to define two set of PRS measurement requirements based on number of samples in RRC_INACTIVE state Tentative agreements: - If reduced number of PRS samples is applicable in RRC_INACTIVE state, define two sets of PRS measurement period for inactive state, one set with 4-sample and the other set with reduced number of samples. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-4-3 | Issue 2-4-3 Approach used for PRS measurement requirements of multiple positioning frequency layers in RRC_INACTIVE state Tentative agreements: - Summation-based approach is used. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-4-4 | Issue 2-4-4 UE processing capability for PRS measurement requirements in RRC_INACTIVE state Tentative agreements: - Whether to define new UE processing capability in RRC_INACTIVE state is within RAN1 scope Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-4-5 | Issue 2-4-5 How to consider DRX cycle for PRS measurement requirements in RRC_INACTIVE state Tentative agreements: - DRX cycle can be considered in the available PRS periodicity for the PRS measurement in RRC_INACTIVE state i,e. Tavailable_PRS_i. - The detailed Tavailable_PRS_i can be discussed in issue 2-4-6. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-4-6 | Issue 2-4-6 Tavailable_PRS,i calculation for PRS measurement requirements in RRC_INACTIVE state Tentative agreements: None. Candidate options: - Option 1: (CATT, CMCC, OPPO, Ericsson) - Tavailable_PRS,i could be the least common multiple between TPRS and DRX cycle. - Option 2: (Huawei) - Tavailable_PRS,i for Inactive state measurement is defined as the LCM of Tprs, measurement window periodicity and DRX cycle. Recommendations for 2nd round: Postpone to next meeting since it is related to 2-4-11 and needs RAN1 reply. |
| Issue 2-4-7 | Issue 2-4-7 TPRS,I calculation for PRS measurement requirements in RRC_INACTIVE state Tentative agreements: - The same approach as R16 can be used. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-4-8 | Issue 2-4-8 Teffct,I calculation for PRS measurement requirements in RRC_INACTIVE state Tentative agreements: None. Candidate options: - Option 1: (CATT, Huawei, Ericsson, OPPO, Nokia, vivo) - The same approach as R16 can be used. - Option 2: (Intel, OPPO, Qualcomm) - Postponed until conclusions on PRS processing capability in RRC INACTIVE state are reached in RAN1. Recommendations for 2nd round: Further discuss. |
| Issue 2-4-9 | Issue 2-4-9 How to define Kcarrier for PRS measurement requirements in RRC_INACTIVE state Tentative agreements: None. Candidate options: - Option 1: (vivo, Nokia, CATT, Ericsson, Intel) - Replace CSSF with Kcarrier for inactive state PRS measurement requirements, Kcarriers is the total number of configured carriers for mobility measurements and CA measurements plus one positioning frequency layer. - Option 1a: (CMCC) - There is no impact on the Kcarrier for RRM measurement delay requirements with the introduction of PRS measurement in inactive state (i.e. only consider the frequency carriers indicated for RRM measurement). - Option 3: (Qualcomm) - Depending on UE capability, the PRS measurement period length would be proportional to K=Kcarrier + 1 or K=1, where the latter would correspond to a UE that has a dedicated PRS processing engine. - Option 4: (OPPO, Huawei) - Replace CSSF with Kcarrier and Nlayer for RRC-INACTIVE state measurement requirements, only one positioning frequency layer is accounted into Kcarrier and Nlayer. Recommendations for 2nd round: Further discuss. |
| Issue 2-4-10 | Issue 2-4-10 Measurement requirements for long periodicity PRS in RRC_INACTIVE state Tentative agreements: - Prioritizing measurements for long-periodicity PRS resources in RRC INACTIVE state is not necessary. Candidate options: None. Recommendations for 2nd round: None. |
| Issue 2-4-11 | Issue 2-4-11 Clarification on the applicability of PRS processing window in RRC_INACTIVE state Tentative agreements: - Send LS to RAN1 to ask for the clarification whether the PRS processing window is applied for the PRS measurement in RRC_INACTIVE state Candidate options: None. Recommendations for 2nd round: Discus the draft LS. |
| Issue 2-4-12 | Issue 2-4-12 Whether to send LS to RAN2/3 about DRX cycle using in RRC_INACTIVE state Tentative agreements: None. Candidate options: - Option 1: (Qualcomm, OPPO, ZTE, Nokia, Ericsson) - RAN4 should send an LS to RAN2/RAN3 notifying them that RAN4 has agreed that the PRS measurement period requirement will depend on the DRX cycle length configured for the UE and that, consequently, the response time requested by the LMF should be adjusted accordingly. RAN4 requests RAN2/RAN3 to introduce new signalling, if necessary. - Option 2: (CATT, Huawei, Intel) - It is not necessary to inform the DRX cycle to LMF even if it is considered in the PRS measurement requirements in RRC_INACTIVE state. Recommendations for 2nd round: Discus the draft LS. |

Sub-topic 2-5 Performance requirements for positioning measurement in RRC_INACTIVE state

|  | Status summary |
| --- | --- |
| Issue 2-5-1 | Issue 2-5-1 Side condition for PRS measurement requirements in RRC_INACTIVE state Tentative agreements: - The side conditions in terms of PRS Ês/Iot defined in RRC_CONNECTED state can be applied for PRS measurements in RRC_INACTIVE state under the condition that the same number of samples is used. Candidate options: None. Recommendations for 2nd round: None. |

Sub-topic 2-6 Measurement reporting requirements for positioning measurement in RRC_INACTIVE state

|  | Status summary |
| --- | --- |
| Issue 2-6-1 | Issue 2-6-1 Measurement reporting requirements for positioning measurement in RRC_INACTIVE state Tentative agreements: None. Candidate options: - Option 1: (Qualcomm, Huawei, CATT) - The PRS measurement reporting delay requirements in RRC_INACTIVE should exclude the transmission time needed by SDT or the transition time to connected state to report positioning measurements. - Option 2: (Ericsson, Intel, Nokia) - FFS Recommendations for 2nd round: Further discuss. |

CRs/TPs

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on NR Positioning Enhancements (Part 2) | CATT | WF to capture all the agreements and open issues. |
| Reply LS on reporting of the Tx TEG association information | Huawei | To: RAN1, RAN2; CC: RAN3 |
| LS on DRX cycle used in PRS measurement in RRC_INACTIVE state | Qualcomm | To: RAN2, RAN3; CC: SA2 |
| LS on the applicability of PRS processing window in RRC_INACTIVE state | CATT | To: RAN1 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2201641 | CR on positioning measurements in RRC Inactive state | Huawei | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| Ericsson | Muhammad Kazmi | Muhammad.kazmi@ericsson.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Nokia | Daejung Yoon | Daejung.yoon@nokia-bell-labs.com |
| Nokia | Juergen Hofmann | Juergen.hofmann@nokia.com |
| Qualcomm | Carlos Cabrera-Mercader | ccmercad@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)