3GPP TSG-RAN WG4 Meeting # 101-bis-e 											         R4-2202577

Electronic Meeting, Jan. 17-25, 2022

Agenda item:			6.24

Source:	Moderator (OPPO)

Title:	Email discussion summary for [101-bis-e][226] NR_SL_relay

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

In this email discussion, we focus on the remaining issues of RRM impact for Rel17 NR SL relay.

- Measurement requirements for SL Relay (re)selection

- Interruption requirements

- Measurement accuracy requirements

- Others

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discussion on the listed issues, and provide the comments on draft CRs.

- 2nd round: Conclude the issues identified in the 1st round.  Revise and endorse draft CRs

Topic #1: General requirements for NR_SL_relay_RRM

Main technical topic overview. The structure can be done based on sub-agenda basis.

[Moderator] The issues in Topic#1 focus on RRM requirements for NR_SL_relay

- Relay (re)Selection Procedure

- Measurement requirements

- Interruptions

- RRC Connection Re-establishment requirement

- Measurement accuracy requirement

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200324 | Qualcomm, Inc. | Proposal 1: For discover period definition, the resource reservation period in mode 2 refers to the dedicated resource pool period for discover signal or the resource reservation period defined in 38.213 clause 8.3.1 in shared resource pool for discover signal. Proposal 2: Reuse LTE ProSe relay measurement requirement. Proposal 3: DRx has no impact on RAN4 discovery signal measurement requirement with the assumption that the discovery signal period is the multiples of the common or pre-configured DRx cycle lengths to avoid transmission of discovery signal during DRx off time. Proposal 4: R16 V2X requirement specifies the SL communication configuration interruption on Uu for general SL communication, and it is applicable to SL relay discovery and communication configurations. Proposal 5: Do not to specify RRC connection re-establishment delay requirement. Proposal 6: Do not introduce SD-RSRP relative measurement accuracy requirement. |
| R4-2200330 | Qualcomm, Inc. | CR: SL discovery signal intra-frequency measurement accuracy requirements |
| R4-2201154 | OPPO | CR to 38133 on measurement requirements for Selection/reselection of relay UE |
| R4-2201155 | OPPO | Observation 1: The period of discovery can follow the corresponding pool’s resource reservation period in case of mode 2. Observation 2: The assumption of R16 NR V2X can be reused for SL relay. Observation 3: Different RRC configuration messages need to be considered for relay discovery and sidelink direct discovery (e.g., SL-ConfigDedicatedNR), which can be further decided by RAN2 Proposal 1: Reuse R16 V2X interruption requirements on WAN due to V2X sidelink communication configuration. Proposal 2: RAN4 not to consider interruptions during discovery (Based on R16 V2X sidelink assumption). |
| R4-2201619 | Huawei, Hisilicon | Proposal 1: For NR sidelink relay in R17, the interruption requirements at NR sidelink discovery configuration can be defined as follows: ### 12.y Interruptions at NR sidelink discovery configuration A UE capable of NR sidelink discovery may indicate its interest (initiation or termination) in NR sidelink discovery to the connected gNodeB using IE [TBD] in TS38.331[2]. The UE is allowed an interruption of up to the duration shown in Table 12.y-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the NR sidelink discovery configuration message [TBD] in TS 38.331[2] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell. Table 12.y-1: Interruption length at NR sidelink discovery configuration NR Slot length (ms); Interruption length (number of slots). 0; 1; 2. 1; 0.5; 3. 2; 0.25; 5. 3; 0.125; 9.. Proposal 2: For NR sidelink relay in R17, the measurement requirements for selection/reselection of NR SL relay UE can be defined as follows: ### 12.z Selection/reselection of sidelink relay UE This subclause contains the requirements related to selection and reselection of sidelink relay UE if a frequency dedicated to sidelink operation is used for sidelink communication via a ProSe relay UE. For a remote UE configured by upper layer for relay operation, when the RSRP measurement of the serving cell (RRC_IDLE) or the PCell (RRC_CONNECTED) is below [TBD], the remote UE shall search for candidate relay UEs for selection and/or reselection every discovery period. If the remote UE has a selected sidelink relay UE, then the remote UE shall measure the [RSRP] of the selected relay once in every four discovery periods and evaluate if it meets the relay selection criterion as defined in TS 38.331[2]. The remote UE shall measure [RSRP] of the candidate relay UEs every Tmeasure, SL_Relay_Intra for intra-frequency relay UEs that are detected and measured according to the measurement rules. For an intra-frequency relay UEs that are detected, but that has not been selected or reselected to, the remote UE shall be capable of evaluating that the intra-frequency relay UE has met selection or reselection criterion defined in TS 38.331[2] within Tevaluate, SL_Relay_Intra as specified in table 7.16.5-1. Table 12.z-1: Tmeasure, SL_Relay_Intra and Tevaluate, SL_Relay_intra Discovery Period [s]; Tmeasure, SL_Relay_Intra [s] (number of discovery periods); Tevaluate, SL_Relay_intra [s] (number of discovery periods). 0.04≤Discovery period≤10.24; Note 1 (4); Note 1 (16). NOTE 1: Time depends upon the discovery period. In resource allocation mode 2, the discovery period is defined as the resource reservation period. In resource allocation mode 2, the discovery period is defined as SPS transmission periodicity.. The minimum requirements are required to meet when the selected and candidate relay UEs are transmitting relay discovery message every discovery period.. |
| R4-2201620 | Huawei, Hisilicon | DraftCR on interruption requirements for NR SL relay |
| R4-2201872 | Ericsson | - Proposal #1: If Rel-17 SL relay operating scenario is similar to Rel-17 SL enhancement WI, interruptions due to SL shall be avoided on WAN during: - reception of paging, - reception of system information, - while onDurationTimer is running. Proposal 2: If Rel-17 SL relay operating scenario is similar to Rel-17 SL enhancement WI, the interruptions on WAN allowed due to SL include maximum allowed interruption rate, interruption length and location (if possibl |

Open issues summary

Sub-topic 1-1: Measurement requirements for Relay (re)selection

Issue 1-1: Clarification on Discovery period

Background: In RAN4#101e meeting, RAN4 made the following agreement for discovery period: - Use the period of the discover signal as the discovery period - Resource reservation period (mode 2) or SPS transmission periodicity (mode 1) can be used

- Proposals:

  - Option 1(QC): For discover period definition, the resource reservation period in mode 2 refers to the dedicated resource pool period for discover signal or the resource reservation period defined in 38.213 clause 8.3.1 in shared resource pool for discover signal.

  - Option 2(OPPO): The period of discovery can follow the corresponding pool’s resource reservation period in case of mode 2.

- Recommended WF

  - Moderator: Option 1 and 2 are not contradictory. For mode 2, resource reservation period is configured per resource pool. Relay discovery can be configured in shared resource pool if only shared resource pool is configured. Otherwise, it should be configured in dedicated resource pool.

  - The clarification can be captured in WF that in mode 2 discovery period can refer to the resource reservation period of shared resource pool if only shared resource pool is configured, or the resource reservation period of dedicated resource pool.

| Company | Comments |
| --- | --- |
| QC | Agree with moderator’s observation, and we can further discuss the wording and explanation in CR discussion. |
| Huawei |  |
| OPPO | Agree with the recommended WF. The clarification can be captured in WF. |

Issue 1-2: Measurement requirements for Relay (re)selection

Background: In RAN4#101e meeting, RAN4 made the following agreement for Measurement requirements: - Reuse measurement/evaluation period requirement from LTE ProSe.

- Clarification question: whether to consider (re)selection of intra-frequency and/or inter-frequency relay UEs?

  - Option 1: ONLY Intra-frequency (R4-2201619)

- the requirements related to selection and reselection of sidelink relay UE if a frequency dedicated to sidelink operation is used for sidelink communication via a ProSe relay UE.

  - Option 2: Both intra-frequency and inter-frequency (R4-2201154)

- Recommended WF

  - The measurement period for intra-frequency relay UE is defined as 4 discovery periods and the evaluation period for intra-frequency relay UE is defined as 16 discovery periods (i.e. 4 measurement periods).

Table 12.z-1: Tmeasure, SL_Relay_Intra and Tevaluate, SL_Relay_intra

| Discovery Period [s] | Tmeasure, SL_Relay_Intra [s] (number of discovery periods) | Tevaluate, SL_Relay_intra [s] (number of discovery periods) |
| --- | --- | --- |
| 0.04≤Discovery period≤10.24 | Note 1 (4) | Note 1 (16) |
| NOTE 1: Time depends upon the discovery period. In resource allocation mode 2, the discovery period is defined as the resource reservation period. In resource allocation mode 2, the discovery period is defined as SPS transmission periodicity. |  |  |

  - FFS: inter-frequency relay UE to be involved

  - Further discussion on the details can focus on the revision of draft CR (R4-2201154)

| Company | Comments |
| --- | --- |
| QC | There is no “intra/inter” frequency measurement for *remote* UE since there is no “serving carrier” concept on side link, and measurement on relay UE by remote UE is on SL only. The intra/inter frequency measurement is for *relay* UE selection of *serving cells*. We think the issue description “(re)selection of intra-frequency and/or inter-frequency relay UEs” should be revised to “Serving cell (re)selection by intra-frequency and/or inter-frequency measurement for relay UEs”. In our opinion, this is not a relevant topic for SL relay, therefore, we support the first bullet in recommended WF and there is no need to discuss inter-frequency measurement. Note that if discover signal are not all on the same frequency as SSB, but they all in the same resource pool. Therefore, “intra/inter frequency” doesn’t apply to discover signals. |
| Huawei | We support option 1. “intra-frequency” means that the candidate SL relay UE to be identified/measured and the selected SL relay UE are on the same SL carrier. |
| OPPO | The clarification on “intra-frequency” by Huawei is fine. We also support the first bullet in recommended WF, and can compromise not to consider inter-frequency relay in this release. - The measurement period for intra-frequency relay UE is defined as 4 discovery periods and the evaluation period for intra-frequency relay UE is defined as 16 discovery periods (i.e. 4 measurement periods). Not consider inter-frequency relay in this release |

Issue 1-3: DRX impact on Measurement requirements

Background: In RAN4#101e meeting, RAN4 made the following agreement: - Not to consider SL DRX for SL relay in R17.

- Proposals:

  - Option 1(QC): DRX has no impact on RAN4 discovery signal measurement requirement with the assumption that the discovery signal period is the multiples of the common or pre-configured DRX cycle lengths to avoid transmission of discovery signal during DRX off time.

- Recommended WF

  - Option 1 can be used as baseline for discussion.

| Company | Comments |
| --- | --- |
| QC | We raise this issue due to the LS from SA2. But since our conclusion is that DRx has no impact on RAN4 discovery signal measurement requirement, no additional agreement beyond what we currently have is fine for us as long as RAN4 scope is consistent with other WGs. |
| OPPO | Agree with QC that no additional impact of DRX on RAN4 discovery signal measurement requirement is considered. |

Sub-topic 1-2: Interruption requirements

Background: In RAN4#101e meeting, RAN4 made the following agreement for interruption requirements: - RAN4 agree to define interruption requirements at discovery configuration (for relay discovery and sidelink direct discovery). - Interruption requirements for NR sidelink discovery should be defined based on R16 V2X sidelink assumption. - FFS how to specify the requirements in next meeting

Issue 1-4-1: How to define the interruption requirements at discovery configuration based on R16 V2X sidelink assumption?

- Proposals:

  - Option 1(QC, HW): Define interruption on the PCell/serving cell during the RRC reconfiguration procedure

- R16 V2X requirement specifies the SL communication configuration interruption on Uu for general SL communication, and it is applicable to SL relay discovery and communication configurations.

- Recommended WF

  - Option 1 is recommended.

  - Further discussion on the details of interruption requirements can focus on draft CR (based on R4-2201620)

### 12.y Interruptions at NR sidelink discovery configuration A UE capable of NR sidelink discovery may indicate its interest (initiation or termination) in NR sidelink discovery to the connected gNodeB using IE [TBD] in TS38.331[2]. The UE is allowed an interruption of up to the duration shown in Table 12.y-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the NR sidelink discovery configuration message [TBD] in TS 38.331[2] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell. Table 12.y-1: Interruption length at NR sidelink discovery configuration NR Slot length (ms); Interruption length (number of slots). 0; 1; 2. 1; 0.5; 3. 2; 0.25; 5. 3; 0.125; 9.

| Company | Comments |
| --- | --- |
| QC | Support the recommended WF. |
| Huawei | We would like to collect the comments in 1st round. Since the signalling design for NR SL discovery has not been finished in RAN2, we leave the IE name as TBD. It can be updated once RAN2 has the conclusion on it. |
| Ericsson | In LTE ProSe interruptions on WAN are avoided during following occasions: - reception of paging - reception of system information - while onDurationTimer is running - In addition, for NR our view is that it is further important to avoid interruptions during following: - while RLF timer is running or when UE is performing CBD |
| OPPO | Support the recommended WF. Do not see the necessity of consider the case that RLF timer is running or when UE is performing CBD. Anyway, we are open to this. Whether the scenario provided by Ericsson is valid can be further discussed in CR revision. |

Issue 1-4-2: Whether to further consider the impact from R17_SL_enhancement on interruption requirements of SL relay?

  - Option 1(Ericsson): If Rel-17 SL relay operating scenario is similar to Rel-17 SL enhancement WI,

    - the interruptions on WAN allowed due to SL include maximum allowed interruption rate, interruption length and location (if possible).

    - interruptions due to SL shall be avoided on WAN during:

      - reception of paging,

      - reception of system information,

      - while onDurationTimer is running.

| Company | Comments |
| --- | --- |
| QC | This should be discussed in SL enhancement WI. |
| Huawei | Same view as QC. It shall be discussed in another R17 WI. |
| Ericsson | Since operating scenario for SL relay is similar to SL enhancement one in REl-17, we think it is relevant. RAN4 should reuse whatever agreement is reached in Rel-17 SL enhancement WI. |
| OPPO | Similar to other feature like DRX, we do not consider the cross-WID impact. Agree with QC and Huawei that this issue should be discussed just in SL enhancement WI. |

Sub-topic 1-3: RRC connection reestablishment

Issue 1-5: Requirements for RRC connection reestablishment

- Proposals:

  - Option 1(QC): Do not to specify RRC connection re-establishment delay requirement.

    - Specifying RRC connection re-establishment requirement itself can’t benefit network operation and has no improvement on system performance.

- Recommended WF

  - Option 1 is recommended.

| Company | Comments |
| --- | --- |
| QC | Agree with recommended WF. |
| Huawei | We agree with the recommended WF. |
| OPPO | Agree with recommended WF. |

Sub-topic 1-4: Measurement accuracy requirements

Issue 1-6: Requirements for Measurement accuracy requirements

Background: In RAN4#101e meeting, RAN4 made the following agreement for interruption requirements: - Define unified requirements for both SD-RSRP and SL-RSRP measurement in relay discovery and (re)selection - PSCCH-DMRS and/or PSSCH-DMRS can be used for the unified RSRP measurements. - Reuse L1 SL-RSRP measurement accuracy requirement and side condition for both SL-RSRP in indirect to direct switch, and SD-RSRP direct to indirect switch.

- Proposals:

  - Option 1(QC): Do not introduce SD-RSRP relative measurement accuracy requirement.

- Recommended WF

  - Option 1 is recommended.

  - Further discussion on accuracy requirements can focus on draft CR (based on R4-2200330)

Companies views’ collection for 1st round

Open issues

Comments are suggested to be provided directly in clause 1.2.

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201154 (measurement requirements) | QC: since R17 considers only switching between indirect and direct paths, not indirect to indirect paths, is it still accurate to specify measurement requirement as “selection/reselection of relay UE”? Reselect implies the indirect to indirect path switch. Should we consider measurement requirement as selection of relay UE only? |
|  | Huawei: we suggest to put “SD-RSRP/SL-RSRP” into [] and only intra-frequency case is considered. |
|  | OPPO: OK with QC’s observation. However, we thought the case of path switch from indirect to direct path can be also called as ‘reselection’. Selection/reselection of relay UE seems not a big issue. The suggestion from Huawei is fine and we can revise the draft CR accordingly. |
| R4-2201620 (interruption requirements) | QC: There could be multiple serving cell’s’. Since it is interruption, we suggest to include it in 12.7. |
|  | Huawei: To QC, only one serving cell is assumed for R16 V2X, which has been captured in section 12.1. This clause contains the requirements for the UE capable of V2X sidelink communication when the UE is out of coverage on the carrier used for V2X sidelink operation, as defined in TS 38.304 [1]. The requirements apply when the UE is: - in any cell selection state, or, - configured for V2X SL operation on a V2X carrier which is dedicated to only V2X SL operation and configured with only a PCell on WAN carrier. R16 V2X assumption is reused for R17 SL relay. So, only one serving cell is also assumed for R17 SL relay UE. |
|  | Ericsson: The CR is pending the open issues being discussed in issue 1-4-1 and 1-4-2. It should be updated accordingly. |
|  | OPPO: The CR is generally fine. Whether or how to capture other cases depends on the further discussion/agreement. |
| R4-2200330 (accuracy requirements) | Huawei: if only dedicate carrier for SL operation is assumed, then the requirements shall not be applied for sidelink on the serving frequency. And we suggest to put “SD-RSRP/SL-RSRP” into []. |
|  | QC: we assume Huawei’s comment is removing this: - in state RRC_IDLE or RRC_CONNECTED if the frequency used for sidelink is the serving frequency, or Including this can cover the R17 intra-band co-existence scenarios, but if companies’ prefer to follow R16 SL scope, we can remove this sentence. And we can add the square brackets. |
|  | OPPO: The clarification from QC looks good to us. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | #### Issue 1-1: Clarification on Discovery period Tentative agreements: - In mode 2 resource reservation period is configured per resource pool. Discovery period can refer to the resource reservation period of shared resource pool if only shared resource pool is configured, or the resource reservation period of dedicated resource pool. Recommendations for 2nd round: No more discussion. #### Issue 1-2: Measurement requirements for Relay (re)selection Tentative agreements: - The measurement period for intra-frequency relay UE is defined as 4 discovery periods and the evaluation period for intra-frequency relay UE is defined as 16 discovery periods (i.e. 4 measurement periods). - For intra-frequency, candidate SL relay UE to be identified/measured and the selected SL relay UE are on the same SL carrier. - Do not consider inter-frequency relay in this release Recommendations for 2nd round: - Confirm the tentative agreements. - Further discussion on the details of measurement requirements can focus on draft CR (based on R4-2201154). Issue 1-3: DRX impact on Measurement requirements Tentative agreements: - DRX has no impact on RAN4 discovery signal measurement requirement with the assumption that the discovery signal period is the multiples of the common or pre-configured DRX cycle lengths to avoid transmission of discovery signal during DRX off time. Recommendations for 2nd round: No more discussion. |
| Sub-topic #1-2 | #### Issue 1-4-1: How to define the interruption requirements at discovery configuration based on R16 V2X sidelink assumption? Tentative agreements: N/A Candidate options: - Option 1: interruption on the PCell/serving cell during the RRC reconfiguration procedure - Option 2: interruption while RLF timer is running or when UE is performing CBD Recommendations for 2nd round: - Continue discussion. The CR revision can be based on draft CR (based on R4-2201620) #### Issue 1-4-2: Whether to further consider the impact from R17_SL_enhancement on interruption requirements of SL relay? Tentative agreements: N/A Candidate options: - Option 1(Ericsson): If Rel-17 SL relay operating scenario is similar to Rel-17 SL enhancement WI, - the interruptions on WAN allowed due to SL include maximum allowed interruption rate, interruption length and location (if possible). - interruptions due to SL shall be avoided on WAN during: - reception of paging, - reception of system information, - while onDurationTimer is running - Option 2: NO, discuss the R17 impact just in R17 SL enhancement WID Recommendations for 2nd round: Continue discussion. If new issues identified, a new CR on interruption can be allocated. |
| Sub-topic #1-3 | #### Issue 1-5: Requirements for RRC connection reestablishment Tentative agreements: - Do not to specify RRC connection re-establishment delay requirement. Recommendations for 2nd round: No more discussion. |
| Sub-topic #1-4 | #### Issue 1-6: Requirements for Measurement accuracy requirements Tentative agreements: N/A Recommendations for 2nd round: - Further discussion on accuracy requirements can focus on draft CR (based on R4-2200330) |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201154 (measurement requirements) | to be revised |
| R4-2201620 (interruption requirements) | to be revised |
| R4-2200330 (accuracy requirements) | to be revised |

Discussion on 2nd round (if applicable)

  - Confirm the tentative agreements from 1st round summary, and continue discussion on open issues.

| Sub-topic #1-2 | #### Issue 1-4-1: How to define the interruption requirements at discovery configuration based on R16 V2X sidelink assumption? Tentative agreements: N/A Candidate options: - Option 1: interruption on the PCell/serving cell during the RRC reconfiguration procedure - Option 2: interruption while RLF timer is running or when UE is performing CBD Recommendations for 2nd round: - Continue discussion. The CR revision can be based on draft CR (based on R4-2201620) #### Issue 1-4-2: Whether to further consider the impact from R17_SL_enhancement on interruption requirements of SL relay? Tentative agreements: N/A Candidate options: - Option 1(Ericsson): If Rel-17 SL relay operating scenario is similar to Rel-17 SL enhancement WI, - the interruptions on WAN allowed due to SL include maximum allowed interruption rate, interruption length and location (if possible). - interruptions due to SL shall be avoided on WAN during: - reception of paging, - reception of system information, - while onDurationTimer is running - Option 2: NO, discuss the R17 impact just in R17 SL enhancement WID Recommendations for 2nd round: Continue discussion. If new issues identified, a new CR on interruption can be allocated. |
| --- | --- |

  - Revision of draft CR:

- Further discussion on the details of measurement requirements based on draft CR (based on R4-2201154).

- Further discussion on the details of interruption requirements based on draft CR (based on R4-2201620)

- Further discussion on accuracy requirements based on draft CR (based on R4-2200330)

Issue 1-4-1: How to define the interruption requirements at discovery configuration based on R16 V2X sidelink assumption?

| Company | Comments |
| --- | --- |

Issue 1-4-2: Whether to further consider the impact from R17_SL_enhancement on interruption requirements of SL relay?

| Company | Comments |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR SL relay RRM | OPPO | To capture the agreements of 1st and 2nd round |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2201154 | CR to 38133 on measurement requirements for Selection/reselection of relay UE | OPPO | To be revised |  |
| R4-2201620 | DraftCR on interruption requirements for NR SL relay | Huawei | To be revised |  |
| R4-2200330 | CR: SL discovery signal intra-frequency measurement accuracy requirements | Qualcomm | To be revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)