3GPP TSG-RAN WG4 Meeting #101-Bis-e  						        				R4-2202597

Electronic Meeting, Jan. 17-25, 2022

Agenda item:			6.9.4

Source:	Samsung

Title:	WF on uplink timing for FR2 HST

Document for:	Approval

# 1 Background

The WF is focused on the topic of Rel-17 NR HST FR2 enhancements RRM core requirements (AI 6.9.4.4) in RAN4 #101bis, with the email thread identifier “[101-bis-e][205] NR_HST_FR2_RRM_2”. In previous RAN4 meeting, WF on one shot large uplink timing adjustment has been approved in WF R4-2120416 as below:

For one shot large uplink timing adjustment - It is up to network configuration to enable one shot large uplink timing adjustment mechanism - RAN4 will further study if additional flag, e.g., unidirectional flag on top of general FR2 HST scenario flag is needed to enable one shot large uplink timing adjustment - RAN4 will further study the network configuration means to disable one shot large uplink timing adjustment. - If one shot large uplink timing adjustment is disabled, existing uplink timing adjustment, i.e., RA based mechanism, and related existing RAN4 requirements will be applied when needed - Introduce a mechanism for one shot large uplink timing adjustment for FR2 HST scenarios with UE allowed to adjust uplink timing beyond Tq - FFS for conditions and additional network assistance for UE to apply one shot large uplink timing adjustment. - The following options can be considered for triggering condition and network assistance - Option 1: No condition except DL timing difference: - UE will apply one shot large timing adjustment if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - Option 2: TCI switching without network assistance: - UE will apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - Option 3: TCI switching with network assistance of indication of inter-RRH and UE large DL timing change detection - UE will apply one shot large timing adjustment on TCI switching between RRH occasion if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH. - Option 4: TCI switching with network assistance of indication of inter-RRH but without UE large DL timing change detection - UE will apply one shot large timing adjustment on TCI switching between RRH occasions - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH. - Performance degradation and impact to signalling design shall be discussed for above procedures - RAN4 will further discuss the accuracy performance and testing issues based on conclusion of above procedures

# 2 Way Forward

### 2.1 Network signalling to enable/disable one shot large UL timing adjustment

- Agreement achieved in GTW (Jan. 21):

- Agreement - Dedicated new RRC based network signalling flag will be specified to enable/disable one shot large UL timing adjustment - Such above RRC based network signalling is not limited to a particular FR2 HST deployment and/or scenarios, i.e., bi-directional scenario or uni-directional scenario

### 2.2 The value of timing difference threshold

- Way Forward:

  - With network signaling to enable one shot large timing adjustment [and without network assistant signaling to inform UE on cross RRH TCI state switch], UE shall apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than a timing difference threshold.

    - Option 1: 9*64*Tc = CP/2

    - Option 2: Tq = 4.5*64*TC = CP/4

    - Other options are not precluded

### 2.3 Random Access (RA)-based mechanism

- Way Forward:

  - No specification change shall be introduced for the current RA procedure due to Rel-17 FR2 HST.

  - Whether to bring the text proposal to TR in future RAN4 meetings is up to proponent.

### 2.4 Scheduling Restriction

- Way Forward:

  - Scheduling restriction related to large propagation delay difference caused by inter-RRH beam switching in FR2 HST:

    - FFS the necessity of UL scheduling restriction (i.e., the UE is not expected to transmit PUCCH/PUSCH/SRS) after cross-RRH TCI state switch until the first TRS is received after the TCI state switch.

    - RAN4 introduce scheduling restriction for the symbol before and after reference symbols used for L1-RSRP measurement.

      - Such scheduling restriction shall be specified in clauses of L1 measurement (i.e., L1-SINR and L1-RSRP)

# References

[1] R4-2202556, “Email discussion summary for [101-bis-e][205] NR_HST_FR2_RRM_2”, Samsung