3GPP TSG-RAN WG4 Meeting #101-bis-e                	 R4-2202603

Electronic Meeting, 17 – 25 January, 2022

Agenda Item:	6.11.2.2

Source:	MediaTek Inc.

Title:	WF on R17 NR MG enhancements – General issues and multiple concurrent MGs

Document for:	Discussion

# Introduction

This is the WF to capture all agreements and open issues in [101-bis-e][209] NR_MG_enh_1 email discussion for RAN4#101-bis-e meeting.

# Applicability and configurations

#### Issue 2-1-1: Whether concurrent gaps are allowed in the case when only E-UTRAN measurement objectives are configured

- Open issue

  - Option 1: Yes

  - Option 1a: Yes, provided that UE supports LTE measurement with concurrent MGs, which is up to UE capability

  - Option 1b: Yes, under the condition that only one per-UE MG is configured for UE

  - Option 2: No

#### Issue 2-1-2: Additional limitation when UE is configured with both E-UTRA and NR Mos

- Open issue

  - FFS: When UE is configured with both E-UTRA and NR MOs, UE can be configured with concurrent MGs, but all E-UTRA Mos are expected to be associated with one single MG

#### Issue 2-1-3: Supporting concurrent gap in MR-DC scenario

- Agreement

  - RAN4 to ask RAN2 to decide whether concurrent MGs is supported in MR-DC scenario

# UE capability related issues

#### Issue 2-2-1: Whether to allow simultaneous configuration of per-UE gap and per-FR gap to FR gap capable UEs

- Agreement

  - UE can be configured with per-UE gap and per-FR gap simultaneously when

    - 1) UE is capable of per-FR gap and concurrent gaps, and

    - 2) per-UE gap is associated with PRS measurements

      - Note: Additional use cases incl. Rel-17 MUSIM and Rel-17 NR NTN Wis are not precluded to be included in future releases.

#### Issue 2-2-2: Max number of concurrent gap across all FRs for per-FR gap capable Ues (without considering other Wis)

- Agreement

  - The maximum number of concurrent gaps across all FRs for per-FR gap capable Ues is

    - 3 for SA case

    - FFS for MR-DC case if it is supported

#### Issue 2-2-3: UE feature list

- Agreement

  - Feature group: Concurrent measurement gaps

  - Components:

    - Support of more than 1 per-UE measurement gap configurations

    - Support of more than 1 per-FR gap measurement gap configurations in an FR, or simultaneous 1 per-UE measurement gap plus 1 per-FR measurement gap configurations in an FR, or more than 1 per-UE measurement gap configurations for UE capable of Rel-15 per-FR gap (independentGapConfig)

    - Note: The above 2 bullets are not 2 separate indications but a single indication with different interpretations, depending on the support of independentGapConfig.

  - Consequence if the feature is not supported by the UE: UE cannot be configured with concurrent gaps

  - Note: This is the baseline capability is to indicate UE support multiple concurrent gaps.

  - Mandatory/Optional: Optional with capability signalling

#### Issue 2-2-4: UE indication of supported gap combination index

- Agreement

  - Do not introduce UE capability indication of supported gap combination index 0 to 5.

| Index | # of simultaneous MG |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 | Per-FR2 | Per-UE |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3 | 1 | 0 | 1 |
| 4 | 0 | 1 | 1 |
| 5 | 1 | 1 | 1 |

# Overlapping

#### Issue 2-3-1: X value in proximity condition for overlapping in FR1.

- Agreement

  - Consider as least X=4 in proximity condition for overlapping in FR1

    - FFS to introduce X=0 as an optional UE capability

#### Issue 2-3-2: X value in proximity condition for overlapping in FR2

- Open issue

  - FFS to consider as least X=4 in proximity condition for overlapping in FR2

    - FFS to introduce X=0 as an optional UE capability

#### Issue 2-3-3: UE behavior during colliding gap occasion

- Open issue

  - Option 1: Priority rule

    - UE will only do the measurement w.r.t. the gap with higher priority on all colliding occasions

    - The priority can be configurable or fixed

  - Option 5: Compromised proposal from moderator

    - Introduce gap sharing rule.

      - Request RAN2 to reserve some RRC signaling for different sharing factors.

        - The signalling design may consider the possibility of resuming data scheduling on dropped gaps

      - Rel-17 requirements will only consider sharing ratios 0% and 100%.

      - The requirements for other sharing factors are FFS in later releases.

- Agreement: CRs can be drafted based on Option 1 with the editor’s note: “The detail UE behavior can be revised based on the later RAN4 agreement on UE behavior during colliding gap occasion.”

#### Issue 2-3-4: Whether to resume data scheduling on the dropped gap occasions

- Agreement

  - Data scheduling is resumed on the dropped gap occasions

#### Issue 2-3-5: Whether to introduce a UE capability to indicate whether UE supports only 0% and 100% gap sharing ratios or UE supports arbitrary configured sharing ratios. (If Option 5 in Issue 2-3-3 is agreed)

- Postpone to next meeting

#### Issue 2-3-6: Detail gap sharing ratios (If Option 5 in Issue 2-3-5 is agreed)

- Postpone to next meeting

#### Issue 2-3-7: Whether to introduce FO, FPO, PFO, PPO scenarios.

- Open issue

  - Option 1: Introduce all scenarios

  - Option 2: Only introduce PFO, PPO scenarios

  - Option 3: Only introduce FO, FPO scenarios

#### Issue 2-3-8: Whether to introduce UE capability for different overlapping scenarios (FO, FPO, PFO, PPO).

- Agreement

  - Do not introduce UE capability for different overlapping scenarios (FO, FPO, PFO, PPO)

# Overhead

#### Issue 2-4-1: Whether to define the overhead cap

- Open issue

  - Option 1: Yes

  - Option 2: No

  - Option 3: Up to UE capability

#### Issue 2-4-2: Definition of overhead cap (if agreed in Issue 2-4-1)

- Open issue

  - Option 1: The max overhead that UE can support in Rel-15/16

  - Option 2: Consider overhead cap with m=1NMGLmMGRPmMGLrMGRPr  when configuring multiple MG patterns.

    - m=1NMGLmMGRPmMGLrMGRPr≤1+ threshold(K)

      - N : number of multiple MG patterns

      - MGLr : MGL of referenced MG

      - MGRPr : MGRP of referenced MG

      - K is FFS

  - Option 3: When concurrent MGs are configured, the MGRP for each MG cannot be smaller than 40ms

# Measurement requirements

#### Issue 2-5-1: [Outside gap] CSSF

- Agreement

  - The definitions for the applicable measurement types specified in Section 9.1.5.1 for CSSF outside gap can be re-used as a starting point with the modification to consider more than 1 measurement gaps

#### Issue 2-5-2: [Outside gap] Kp

- Agreement

  - The following is taken a starting point to proceed and is subject to further checking in the next meeting.

  - The Kp value for a SSB frequency layer to be measured outside gap is defined as Kp = Ntotal / Navailable

  - For a window W of duration max(TSMTC,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the SSB frequency layer, and starting at the beginning of any SMTC occasion:

    - Ntotal is the total number of SMTC occasions within the window, ignoring any overlap with MG occasions within the window, and

    - Navailable is the number of SMTC occasions that are not overlapped with any MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule.

    - FFS: extension to CSI-RS based L3 measurements

  - Kp = 1 when Navailable = 0.

    - In this case, the SMTC occasions are fully overlapped by MGs and the measurement should be conducted within gap.

#### Issue 2-5-3: [Within gap] CSSF

- Agreement

  - The CSSF is calculated separately for each gap pattern. [provided that the association between measurement objects and gap pattern is configured by network.]

    - [Only the measurement objects associated to the same measurement gap pattern are counted when deriving CSSFwithin_gap,i for a target measurement object with index i.]

    - FFS: how the dropped gap occasions will not be used in deriving CSSFwithin_gap,i

  - RAN4 can revisit this agreement when the association implemented by RAN2 is clear.

#### Issue 2-5-4: [Within gap] MGRP

- Agreement

  - In the delay requirements of measurements within gap, indicate which MGRP to be selected between 2 configured measurement gaps

    - RAN4 can revisit this agreement when the association implemented by RAN2 is clear.

#### Issue 2-5-5: [Within gap] Kp

- Agreement

  - The following is taken a starting point to proceed and is subject to further checking in the next meeting.

  - The Kp value for a SSB frequency layer to be measured within gap is defined as Kp = Ntotal / Navailable

  - For a window W of duration max(TSMTC,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the SSB frequency layer, and starting at the beginning of any associated gap occasions covering the SMTC occasion:

    - Ntotal is the total number of associated gap occasions covering SMTC occasions within the window, ignoring any overlap with other MG occasions within the window, and

    - Navailable is the number of associated gap occasions covering SMTC occasions that are not overlapped with any other MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule.

      - Requirements do not apply if Navailable =0

    - FFS: extension to CSI-RS based L3 measurements

# Impact to other L1 measurements

#### Issue 2-6-1: P factor of L1 measurement

- Agreement

  - The following is taken a starting point to proceed and is subject to further checking in the next meeting.

  - The P value for a L1 resource to be measured is defined as

    - Ntotal / Navailable in FR1

    - Psharing * Ntotal / Noutside_MG in FR2 with Navailable = 0

    - Ntotal / Navailable in FR2 with Navailable > 0

  - For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as serving cell, and starting at the beginning of any gap occasions covering the L1 resource occasion:

    - Ntotal is the total number of L1 resource occasions within the window, ignoring any overlap with MG occasions or SMTC occasions within the window, and

    - Noutside_MG is the number of L1 resource occasions that are not overlapped with any MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule.

    - Navailable is the number of L1 resource occasions that are not overlapped with any MG occasion nor any SMTC occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule.

    - TL1 is periodicity of the target L1 RS.

# Others

#### Issue 2-7-1: Whether to specify transient UE behavior when concurrent MGs are re-configured

- Agreement

  - Do not specify transient UE behavior when concurrent MGs are re-configured

#### Issue 2-7-2: CR handling in RAN4#101bis-e meeting

- Agreement

  - All endorsed CRs are just a start point to move forward and are subject to further checking and revising in the next meeting.

# Reference

[1] R4-2104096, “WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98e, Feb’21

[2] R4-2105856, “WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98bis-e, Apr’21

[3] R4-2108346, “WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#99e, May’21

[4] R4-2115342, “WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#100e, Aug’21

[5] R4-2120414, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc., RAN4#101e, Nov’21