3GPP TSG-RAN WG4 Meeting #101-bis-e 					R4-2202638

Electronic Meeting, 17th – 25th Jan., 2022

Title:	WF on GNSS-related and timing requirements for NR NTN

Source:	Xiaomi

Agenda Item:	6.13.5.2 & 6.15.5.4

Document for:	Approval

- Introduction

According to the email discussion summary in [1], this document is to capture the WF on GNSS-related requirements and timing requirements for NR NTN.

- WF on GNSS related requirements

- Issue 1-1: GNSS accuracy assumption for RRM requirements

  - Agreement:

    - No further discussion on the common GNSS accuracy assumption for RRM requirements in Rel-17 NTN WI.

    - The specific GNSS accuracy assumption can be discussed if there is the specific RRM requirements requires GNSS accuracy assumption

- Issue 1-2: RRM impact due to UE Internal Coexistence between GNSS receiver and NR UL transmitter

  - Agreement:

    - RAN4 postpone the discussion on RRM impact due to UE Internal Coexistence between GNSS receiver and NR UL transmitter until RF session has the conclusion.

- WF on timing requirements

3.1 	WF on UE specific TA estimation

- Issue 2-1-1: Whether to define the update periodicity for UE specific TA estimation?

  - Agreement:

    - RAN4 not to define the update periodicity for UE specific TA estimation.

- Issue 2-1-2: Whether to define UE behaviour related to updating rate for UE specific TA estimation?

  - Agreement:

    - The UE behavior related to updating rate for UE specific TA estimation can be discussed under double correction issue (issue 2-3-1).

- Issue 2-1-3: UE behaviour on UE specific TA updating before applying TA adjustment

  - Agreement:

    - The UE behaviour on UE specific TA updating before applying TA adjustment can be discussed under double correction issue (issue 2-3-1).

3.2 	WF on UE initial transmit timing error requirements

- Issue 2-2-1: Requirement of initial transmit timing error (Te_NTN).

  - Agreement:

    - Do not revisit the agreements on Te_NTN agreed in RAN4#101e meeting.

    - RAN4 shall confirm the values of Te_NTN in brackets in RAN4#102e meeting.

- Issue 2-2-2: The reference timing for UE transmit timing.

  - Agreement:

    - The reference timing for the UE transmit timing shall be the downlink timing of the reference cell minus (NTA+NTA,UE-specific+NTA,common+NTA,offset) ×Tc.

    - FFS the additional clarification on NTA,UE-specific and NTA,common.

    - FFS the clarification on the reference propagator model UE used at UE side.

    - FFS the clarification on the downlink timing of the reference cell.

3.3 	WF on Double correction issue related to combination of open and closed loop TA control

- Issue 2-3-1: Double correction issue related to combination of open and closed loop TA control.

  - Option 1:

    - Double correction issue can be addressed by defining NTN UE initial timing accuracy requirement for all UL transmissions.

  - Option 2:

    - Double correction issue can be addressed under the framework of gradual timing adjustment accuracy requirement.

3.4		WF on gradual timing adjustment requirements

- Issue 2-4-1: The principles for defining gradual timing adjustment requirement

  - Option1:

    - Whether and how to relax the gradual timing adjustment requirement accordingly to accommodate the timing change/drift, i.e. updating Tq, Tp, and/or the rate

      - FFS on the propagation delay drift for service link and feeder link.

    - NTN UE is required to adjust its UL timing towards updated UE specific TA and DL timing gradually, according to minimum and maximum aggregate adjustment rate requirements

  - Option 2:

    - Not define gradual timing adjustment requirement for NTN UE;

    - Replace gradual timing adjustment requirement with NTN UE initial timing accuracy requirement, i.e. NTN UE initial timing accuracy requirement applies to all UL transmissions.

- Issue 2-4-2: UE behaviour for gradual timing adjustment for NTN UE.

  - Option 1:

    - UE performs timing adjustment for downlink reception timing drifting and UE specific TA change separately.

  - Option 2:

    - UE performs timing adjustment with combining downlink reception timing drifting and UE specific TA change as one adjustment.

  - Option 3:

    - For GEO, UE performs timing adjustment with combining downlink reception timing drifting and UE specific TA change as one adjustment.

    - For LEO, UE performs timing adjustment with combining downlink reception timing drifting, common TA change and UE specific TA change as one adjustment.

- Issue 2-4-3: Whether define different gradual timing adjustment requirements for different NTN topologies e.g. GEO, MEO, LEO

  - Agreement

    - RAN4 defines the same gradual timing adjustment requirements for different NTN topologies, if defined.

- Issue 2-4-4: Whether the maximum delay variation for the round trip delay should be considered in the gradual timing adjustment requirement in NTN?

  - Option 1:

    - Yes

  - Option 2:

    - No

- Issue 2-4-5: Whether the feeder link time drift should be considered in the gradual timing adjustment requirement in NTN?

  - Option 1:

    - Yes

  - Option 2:

    - No

3.5		WF on TA adjustment accuracy requirements

- Issue 2-5-1: Whether the UE position and satellite position estimation error should be accounted for TA adjustment accuracy requirement.

  - Agreement:

    - UE position and satellite position estimation error shall not be accounted for TA adjustment accuracy requirement.

- Issue 2-5-2: TA adjustment accuracy requirement in RRC_CONNECTED mode

  - Agreement:

    - The legacy NR TA adjustment accuracy requirements defined in TS 38.133 can be reused for NTN case.

    - FFS on the additional conditions for NTN TA adjustment accuracy requirement.

    - FFS the margin to accommodate UE autonomous open loop TA pre-compensation.

- Reference

[1]	R4-2202564,	Email discussion summary for [101-bis-e][213] NR_NTN_solutions_RRM_2