3GPP TSG-RAN WG4 Meeting # 101-bis-e  										             R4-2202640

Electronic Meeting, January 17-25, 2022

Agenda item:			6.14

Source:	Moderator (MediaTek inc.)

Title:	WF on RLM/BFD relaxation for UE Power Saving enhancements

Document for:	Information

- Introduction

This document is to capture the all agreements in RAN4#101-bis-e in email thread #214 on RLM/BFD relaxation for UE Power Saving enhancements.

No CR/TPs are treated during this meeting

Agreements from the GTW

WF

- Issue 1-1-1: Feature for RLM/BFD relaxation in Rel-17 feature table

- RAN4 to introduce a UE capability to indicate the support of RLM/BFD relaxation in general in Rel-17 feature table.

- Issue 1-2-1&Issue 1-2-2: Enable the RLM/BFD relaxation feature

- RLM/BFD relaxation is enabled by explicit signaling.

  - FFS the applicability conditions that UE is allowed to apply relaxed requirement.

- Issue 2-1: L3 CSI-RS to be used for Low mobility criteria

  - Option 1: L3 CSI-RS can be used for low mobility criteria evaluation for Ues supports CSI-RS based L3 measurements as well.

  - Option 2: The existing agreement to use SSB based L3-RSRP measurement of the serving cell to evaluate the low mobility criterion is sufficient.

- Issue 2-2: Accuracy and delay requirements for low mobility criteria

- The RRM measurements used for low mobility evaluation shall fulfil the accuracy requirements defined in TS 38.133 section 10.

Issue 2-2-1: the specific SSB to be measured for the per-UE low mobility criterion evaluation.

FFS

- Option 1a: The intra-frequency L3 RSRP measurement of serving cell based on SSB is derived as the intra-frequency SS-RSRP measured over single SSB, and shall fulfil the measurement requirement in TS 38.133 section 9.2.5.2 and the performance requirement in section 10.1. (Nokia)

- Option 1b: Network needs to configure the specific SSB to be measured for the per-UE low mobility criterion evaluation.

- Issue 2-4: Low mobility criteria configuration type

- Low mobility criterion is configured on per-UE basis.

- Issue 3-1: Good serving cell quality criteria for RLM/BFD

- Agreement

  - The good serving cell quality criteria for RLM/BFD is based on an offset X dB and Qx, while Qx is derived from PDCCH transmission parameters.

    - Qx = Qin for RLM

    - Qx = [Qin] for BFD

      - Note: definition of Qin for BFD needs to be clarified

    - The offset X can be configured from a set of 4 values

      - Exact values are FFS

    - One pre-defined value is used for evaluation if the offset is not configured

      - Pre-defined value X = [0] dB

    - Signalling details are up to RAN2

- Issue 3-4: Cell quality criteria configuration type

- Option 1: per-serving cell basis

- Option 2: per-UE basis

- Option 3: per-CG basis

- Issue 5-1: Lower bound of relaxed evaluation period

- The lower bound of relaxed evaluation period is NOT relaxed by K, if K > 2 is applying.

- The lower bound of relaxed evaluation period is relaxed by K, if K <=2 and K>1 is applying.

- Issue 5-2: relaxation factors

- Confirm K0, FR2, SSB = 1 for 80 ms < MAX(TDRX, TSSB) ≤ 160 ms.

Issue 5-2-2: The relaxation factor for FR1:

- Option 1:

  - K1, FR1 =4 for 40 ms < MAX(TDRX, TRS) ≤ 80 ms

  - K2, FR1 =4 for MAX(TDRX, TRS) ≤ 40 ms

- Option 2

  - For FR1 RLM: (consider only DRx <= 80ms)

    - K = 2 when DRx > 40ms *or* T310 <= 640ms;

    - K = 4 when DRx <= 40ms *and * T310>640ms,

  - For FR1 BFD: K = 2

Issue 5-2-3: The relaxation factor for FR2 SSB

  - Option 1: K1, FR2, SSB= 2 for MAX(TDRX, TSSB) ≤ 80 ms

  - Option 2:

    - K=1.5 for 60 ms ≤ MAX(TDRX, TSSB) ≤ 80 ms.

    - K=2 for MAX(TDRX, TSSB) ≤ 60 ms

- Issue 6-1-1: Relaxation criteria for multiple RLM-RS/BFD-RS

- Option 1:

  - For entering condition: the radio link quality of at least one RS resource is better than the entering threshold.

  - For exit condition: the radio link quality for all the RS resources is worse than the exiting threshold.

- Option 2

  - The UE is allowed to operate RLM/BFD in relaxed mode for a certain cell (SpCell or SCell) when the radio link quality is better than the threshold (Qout + X1) for all RLM-RS resource.

  - The UE shall exit the relaxed mode when the radio link quality is worse than the threshold (Qout + X2) for any the RLM-RS resources.

  - The values of X1, X2 can be same as those discussed for good serving cell quality.

- Issue 6-3: Specification section for relaxation criteria

FFS Capture the relaxation criteria in the separate sub-section, while RAN4 refers the definition of relaxation criteria to RAN1 or RAN2 specification