3GPP TSG- RAN WG4 Meeting #101-bis-e	R4-2202648

# Electronic Meeting, January 17-25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-1-7 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Support for relaxed BFD requirements for UEs under power saving mode is introduced in release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 8.5.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= Start of Change ===========================

#### 8.5.3.X	Minimum requirement for CSI-RS based relaxed beam failure detection

This clause contains the minimum requirements for CSI-RS based relaxed beam failure detection.

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS_Relax ms period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS_Relax ms period.

The value of TEvaluate_BFD_CSI-RS_Relax is defined in Table 8.5.3.X-1 for FR1.

The value of TEvaluate_BFD_CSI-RS_Relax is defined in Table 8.5.3.X-2 for FR2 with N=1.

The values of P, MBFD and PBFD is defined in clause 8.5.3.2.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 8.5.3.X-1: Evaluation period TEvaluate_BFD_CSI-RS_Relax for FR1

| Configuration | TEvaluate_BFD_CSI-RS Relax (ms) |  |
| --- | --- | --- |
| Max(TDRX, TCSI-RS) ≤ 40 ms | Max([50], Ceil(K1 × 1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| 40 ms < Max(TDRX, TCSI-RS) ≤ 80ms | Max([50], Ceil(K2 × 1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |  |
| 80 ms < Max(TDRX, TCSI-RS) ≤ 160 ms | Max([50], Ceil(1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| Note 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K1 = TBD, K2= TBD is the relaxation factor applicable for UE fulfilling the BFD relaxation criteria in [TBD] |  |

Table 8.5.3.X-2: Evaluation period TEvaluate_BFD_CSI-RS_Relax for FR2

| Configuration | TEvaluate_BFD_CSI-RS Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80ms | Max(100, Ceil(K2 × 1.5 × MBFD P N PBFD) Max(TDRX, TCSI-RS)) |
| 80 ms < Max(TDRX, TCSI-RS) ≤ 160 ms | Max([100], Ceil(1.5 × MBFD P N PBFD) Max(TDRX, TCSI-RS)) |
| Note 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K2 = 2 is the relaxation factor applicable for UE fulfilling the BFD relaxation criteria in [TBD] |  |

========================= End of Change ===========================