3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2202650

E-meeting, 17 – 25 January, 2022

Title:	WF on NR SL enhancements RRM requirements

Source:	LG Electronics

Agenda item:	6.15.5

Document for:	Approval

WF – 1.1 NTA_offset & NTA,SL when NR Cell is configured as synchronization reference source

Agreement

- In R17, the requirements on NTA,SL and NTA-offset for NR cell as synchronization reference are updated as follows

  - The sidelink transmissions takes place (NTA,SL+NTA-offset)×TC  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where NTA,SL=0. If uplink transmission and sidelink transmission are in the same band, NTA-offset is defined in Table 7.1.2-2, otherwise NTA-offset is defined as 0

WF – 1.2 Scheduling availability when switching TDM based intra-band con-current SL operation

Agreement

- Consider only RF switching time for scheduling availability

  - When switch from uplink transmision to NR V2X sidelink transmision occurs in sidelink slot ‘n’,

    - UE is not expected to transmit or receive on NR V2X sidelink on the sidelink slot ‘n’.

  - When switch from NR V2X sidelink transmision to uplink transmision occurs in sidelink slot ‘n-1’,

    - UE is not expected to transmit or receive on NR V2X sidelink on the sidelink slot ‘n-1’.

  - When switch from NR V2X sidelink transmision to uplink transmision occurs in Uu slot ‘n’,

    - UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n’.

  - When switch from uplink transmision to NR V2X sidelink transmision occurs in Uu slot ‘n-1’,

    - UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n-1’.

- FFS: whether the requirement is different when NTA is smaller than the switching time

WF – 1.2.1 Timeline applicability when switching TDM based intra-band con-current SL operation

Agreement

- FFS Tx preparation time by

  - SL to Uu Tx switch: K2 >= 2 slots for the first UL grant when switching from SL to Uu

  - Uu to SL Tx switch: N2 >= one slot + PSSCH preparation time for the first SL grant when switching from Uu to SL

WF – 1.3 Interruption on SL due to Uu BWP switch for FDM based intra-band con-current SL operation

Agreement

- Reuse the interruption length in Table 8.2.1.2.7-1(=Table8.2.2.2.5-1) as interruption on SL due to Uu BWP switch, excluding =3.

- Interruption length X

|  | NR Slot | Interruption length X (slots) |
| --- | --- | --- |
|  | length (ms) |  |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |

- Only allow interruption on SL within TBWPswitchDelay for DCI-based or timer-based BWP switch, or within TRRCprocessingDelay + TBWPswitchDelayRRC for RRC-based BWP switch

WF – 2.1 RRM requirements at transition when SL DRX cycle periodicity changes in the configured multiple SL-DRX cycles

Agreement

- When the SL-DRX changes, the UE is not required to meet corresponding requirements within this DRX cycle

WF – 2.2.2 SyncRef UE detection time (Tdetect,SyncRef UE_V2X) for Asynchronized SLSS measurment & search

Agreement

- 8s

WF – 2.2.3 UE Rx(Data) drop rate requirements for Asynchronized SLSS measurment & search

- Option 1: maximum drop rate(X) during Tdetect,SyncRef UE_V2X

  - Option 1a : X = 0.3%(Rel-16) (CATT, vivo, Xiaomi)

  - Option 1b : X = 0.3%(Rel-16) with additional clarification that “The dropping rate is calculated ignoring SL-DRX configuration” (Oppo)

  - Option 1c : X = max{2 slots, 0.3% of its V2X data reception} (LGE)

- Option 2 : maximum aggregated drop window(Y) during Tdetect,SyncRef UE_V2X (Oppo)

  - Y = 480ms

- Moderator’s suggestion based on 2nd round

    - Up to 24 slots(or 5%) of V2X data reception during maximum aggregated drop window of 480ms is allowed to be dropped for PSBCH monitoring

Agreement

- Further discuss in next meeting

WF – 2.2.4 Conditional SyncRef UE detection requirements for Asynchronized SLSS measurment & search

- Option 1 : Define conditional SyncRef UE detection requirements (QC, LGE, vivo)

  - Option 1a: (Qualcomm)

    - UE can skip asynchronized SyncRef UE search to save power when the following conditions are all satisfied over an evaluation period:

      - SLSS RSRP is larger than a threshold

      - SLSS RSRP variation is lower than a threshold. The SLSS RSRP variation is the average value of (instantaneous RSRP - current filtered RSRP)^2 during the evaluation period

      - Data connection is maintained with the current SyncRef UE source

    - The evaluation period is the same as SLSS Tx initiation/cease evaluation period when SLSS is the synchronization source

  - Option 1b: (LGE)

    - UE can skip asynchronized SyncRef UE search to save power when the following conditions are all satisfied over an evaluation period, Tevaluate,SLSS:

      - All SLSS RSRPs are larger than a threshold, syncTxThreshOoC.

      - Data connection is maintained with the current SyncRef UE source

  - Option 1c : FFS (vivo)

- Option 2 : Not define conditional SyncRef UE detection requirements for asynchronized SLSS measurement & search in R17 (Huawei, Xiaomi)

Agreement

- Further discuss in next meeting

WF – 2.2.5 UE Tx(Data & SLSS) drop rate requirements for Asynchronized SLSS measurment & search

Agreement

- Specify based on the previous agreements

  - Allow Tx dropping at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X async search

- If needed, revisit in next meeting

WF – 2.3.1 Whether to define interruption to WAN due to SL-DRX

Agreement (GTW)

- Define interruption requirements on NR transmission if configured due to NR SL transitions between active and non-active in SL DRX when NR SL is in SL-DRX but NR is in non-DRX

  - EN-DC can be used as baseline

  - FFS whether interruptions are applicable for the following WAN conditions and impact on SL transitions between active and non-active SL DRX if interruptions are not applicable:

    - reception of paging

    - reception of system information

    - while onDurationTimer is running

    - while RLF timer is running

    - while UE is performing CBD

- FFS on interruptions for the case when NR is in DRX and SL is in SL-DRX

WF – 2.3.1.1 Interruption length on WAN due to SL-DRX when interruption is allowed

Agreement

- Reuse table 8.2.1.2.1-1 for sync & async case in TS 38.133

WF – 2.3.1.2 Allowed probability of missed Ack/Nack on WAN due to SL-DRX when interruption is allowed

Agreement

- Allow up to 1 % probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640ms, and up to 0.625 % probability of missed ACK/NACK when the configured configured SL-DRX cycle]is 640ms or longer.

- when multiple SL-DRX cycles are configured,

  - a shortest SL-DRX cycle is applied

WF – 2.3.4 Interruption to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration

Agreement

- Define interruption

- Further discuss how to specify it in next meeting

WF – 2.3.5 Sensing requirement during SL-DRX

Agreement

- For active time, define requirements

  - ‘When partial sensing mechanism is enabled for the resource pool that UE is monitoring and selecting resource from, the UE shall be capable of performing the L1 SL-RSRP measurements on the sensing periods specified in TS38.214[26]. When SL-DRX is enabled, the UE shall be capable of performing the L1 SL-RSRP measurements and select resource during SL-DRX active time as specified in TS38.214[26].’

WF – 2.4.1 Interruption to SL due to Uu DRX

Agreement

- ACK/NACK miss probability requirements are only applied when HARQ process on SL is supported

WF – 3.1.1 L1-RSRP measurement for partial sensing

Agreement

- Existing L1 SL-RSRP measurement accuracy requirements can be applied

WF – 3.1.2 L1-RSRP measurement for inter-UE coordination

Agreement

- Existing L1 SL-RSRP measurement accuracy requirements can be applied