3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202654

# Electronic Meeting, 17th Jan. – 25th Jan. 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Selection/Reselction of V2X Synchronization Reference Source for sidelink enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-1-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Requirements for Selection/Reselction of V2X Synchronization Reference Source for sidelink enhancement due to SL-DRX operation should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added requirements for Selection/Reselction of V2X Synchronization Reference Source due to SL-DRX operation for sidelink enhancement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for Selection/Reselction of V2X Synchronization Reference Source for SL-DRX operation for sidelink enhancement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

## 12.4	Selection / Reselection of V2X Synchronization Reference Source

The requirements defined in this clause do not apply to the UEs that do not support transmission and reception of SLSS.

A SyncRef UE is considered to be detectable when

-	PSBCH-RSRP related side conditions given in Clause 10 are fulfilled for a corresponding Band,

-	V2X SCH_RP and SCH Ês/Iot according to Annex B for a corresponding Band are fulfilled.

When GNSS synchronization reference source is configured as the highest priority and

-	UE is synchronized to GNSS directly,

-	UE shall not drop any V2X SLSS and data transmission for the purpose of selection/reselection to the SyncRef UE.

-	UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly,

-	UE shall not drop any V2X data transmission for the purpose of selection/reselection to the SyncRef UE. The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2]. Tdetect,SyncRef UE_V2X is defined as 1.6 seconds at SCH Es/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 30% of its SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	in other case

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2]. Tdetect,SyncRef UE_V2X is defined as 8 seconds at SCH Es/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 6 % of its V2X data and SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE when no SL-DRX is used.

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2]. Tdetect,SyncRef UE_V2X is defined as 8 seconds at SCH Es/Iot ≥ 0 dB, provided that the UE is allowed to drop its V2X data and SLSS transmissions at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE when SL-DRX is used.

-	UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE when no SL-DRX is used.

-	Editor’s note: FFS when SL-DRX is used.

When serving cell/PCell synchronization reference source is configured as the highest priority,

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2].  Tdetect,SyncRef UE_V2X is defined as 8  seconds at SCH Es/Iot ≥ 0 dB, provided that the V2X UE is allowed to drop a maximum of 6 % of its V2X data and SLSS transmissions for the purpose of selection / reselection to the SyncRef UE when no SL-DRX is used.

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2].  Tdetect,SyncRef UE_V2X is defined as 8  seconds at SCH Es/Iot ≥ 0 dB, provided that the V2X UE is allowed to drop its V2X data and SLSS transmissions at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE when SL-DRX is used.

-	UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE when no SL-DRX is used.

-	Editor’s note: FFS when SL-DRX is used.

UE shall be capable of performing PSBCH-RSRP measurements for 3 identified intra-frequency SyncRef UE with the measurement period of  Tmeasure,PSBCH-RSRP in Table 12.4-1. It is assumed that the SyncRef UE do not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

Table 12.4-1: PSBCH-RSRP measurement period for intra-frequency SyncRef UE

| SL-DRX cycleNote 1 [ms] | Tmeasure,PSBCH-RSRP [ms] |
| --- | --- |
| No SL-DRX | 320 |
| SL-DRX cycle≤160ms | max (320, 2 x SL-DRX cycle) |
| SL-DRX cycle>160ms | 2 x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured, the SL-DRX cycle is the shortest one. |  |

When UE is synchronized to GNSS directly, before selection / reselection of the new synchronization reference source UE shall evaluate the GNSS synchronization source reliability for at least 20 seconds before changing the synchronization reference from GNSS to another synchronization reference source. UE shall be always synchronized to GNSS directly during the evaluation of GNSS synchronization source reliability.

< End of change #1 >