3GPP TSG-RAN WG4 Meeting #101-bis-e		R4-2202659

E-meeting, January 17-25, 2022

Agenda item:	6.16.7

Source:	Intel Corporation

Title:	WF on NR extension to 71 GHz RRM requirements (Part 2)

Document for:	Approval

# Introduction

In this contribution we capture the agreements achieved during the email discussion on the topic Rel-17 NR ext. to 71GHz RRM core requirements (AI 6.16.7) in RAN4#101-bis-e under email thread [101-e][217] NR_ext_to_71GHz_RRM_2. This email thread covers following sub-agenda items:

- AI 6.16.7.3	Interruption requirements

- AI 6.16.7.4	Active BWP switching delay requirements

- AI 6.16.7.5	Measurement gap interruption requirements

- AI 6.16.7.6	LBT impacts on RRM requirements

The details on the discussion can be found at the [217] thread discussion summary [1].

# Way forward

## Interruption requirements

- For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps the interruption requirements to be defined as in table below

Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

- For interruption requirements at transitions between active and non-active during DRX update Table 8.2.4.2.6-1 as below:

Table 8.2.4.2.6-1: Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |
| 5 | 0.03125 | 17 |  |
| 6 | 0.015625 | 33 |  |

- Following interruption requirements for NR-CA agreed in RAN4#100e apply to NR-DC case:

  - Interruptions at SCell addition/release (intra-band CA),

  - Interruptions at Scell activation/deactivation (intra-band CA)

  - Interruptions during measurements on deactivated SCC

  - Interruptions due to Active BWP switching

  - Interruptions at NR SRS carrier based switching

## Active BWP switching delay requirements

- BWP Switching on Multiple CCs shall follow chapter 8.6.2A, chapter 8.6.2B and Chapter 8.6.3A in TS38.133.

- Cross-carrier active BWP switching:

  - “DL slot n” in Section 8.6.2 of TS38.133 is the slot where UE receives BWP switching request

  - TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change

  - In general, the number of slots for BWP switching delay extension due to MRTD is given by MRTDTslot. For the scenarios currently considered for FR2-2 one slot of the smaller SCS among all the involved numerologies is enough to cover MRTD

  - Considering margin for cross-carrier processing, value Y in cross-carrier active BWP switching delay is define as 1 slot of the smaller SCS for FR1+FR2-2 and 1 slot of 120kHz for FR2-2+FR2-2

  - For active BWP switching delay requirements when one of the two BWPs in a BWP switching is a dormant BWP, value X is equal to 1 slot of the smaller SCS for FR1+FR2-2 and 1 slot of 120kHz for FR2-2+FR2-2

## Measurement gap interruption requirements

- For high SCSs one more slot of measurement interruptions is needed for NR-CA, synchronous NR-DC configuration and MCG in asynchronous NR-DC.

Measurement gap interruption requirements for FR2-2 shall be shown as following tables:

Table 1: Total number of interrupted slots on all serving cells during MGL for single carrier, intra-band NR CA and on all serving cells in MCG for inter-band NR-CA/DC with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 20 | 10 | 6 | 4 | 3 | 21Note3 | 11Note3 | 7Note3 | 5Note3 | 4Note3 |
| 30 | 40 | 20 | 12 | 8 | 6 | 40 | 20 | 12 | 8 | 6 |
| 60 | 80 | 40 | 24 | 16 | 12 | 80 | 40 | 24 | 16 | 12 |
| 120 | 160 | 80 | 48 | 32 | 24 | 160 | 80 | 48 | 32 | 24 |
| 480 | 640 | 320 | 192 | 128 | 96 | 640 | 320 | 192 | 128 | 96 |
| 960 | 1280 | 640 | 384 | 256 | 192 | 1280 | 640 | 384 | 256 | 192 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz, 480kHz and 960kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation. |  |  |  |  |  |  |  |  |  |  |

Table 2: Total number of interrupted slots on all serving cells in SCG for inter-band NR-CA and synchronous NR-DC with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 20 | 10 | 6 | 4 | 3 | 21 | 11 | 7 | 5 | 4 |
| 30 | 40 | 20 | 12 | 8 | 6 | 40 | 20 | 12 | 8 | 6 |
| 60 | 80 | 40 | 24 | 16 | 12 | 80 | 40 | 24 | 16 | 12 |
| 120 | 160 | 80 | 48 | 32 | 24 | 160 | 80 | 48 | 32 | 24 |
| 480 | 641 | 321 | 193 | 129 | 97 | 641 | 321 | 193 | 129 | 97 |
| 960 | 1281 | 641 | 385 | 257 | 193 | 1281 | 641 | 385 | 257 | 193 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz, 480kHz and 960kHz is only applicable to the case with per-UE measurement gap. |  |  |  |  |  |  |  |  |  |  |

Table 3: Total number of interrupted slots on all serving cells in SCG for asynchronous NR-DC with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 21 | 11 | 7 | 5 | 4 | 21 | 11 | 7 | 5 | 4 |
| 30 | 41 | 21 | 13 | 9 | 7 | 41 | 21 | 13 | 9 | 7 |
| 60 | 81 | 41 | 25 | 17 | 13 | 81 | 41 | 25 | 17 | 13 |
| 120 | 161 | 81 | 49 | 33 | 25 | 161 | 81 | 49 | 33 | 25 |
| 480 | 641 | 321 | 193 | 129 | 97 | 641 | 321 | 193 | 129 | 97 |
| 960 | 1281 | 641 | 385 | 257 | 193 | 1281 | 641 | 385 | 257 | 193 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz, 480kHz and 960kHz is only applicable to the case with per-UE measurement gap. |  |  |  |  |  |  |  |  |  |  |

Table 4: Total number of interrupted slots on FR2-2 serving cells during MGL for single carrier, intra-band NR CA and inter-band NR-CA/DC with per-UE measurement gap or per-FR measurement gap for FR2-2

| NR | Total number of interrupted slots on FR2 serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.25ms is applied |  |  |  |  |
| (kHz) | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms |
| 60 | 80 | 40 | 22 | 14 | 6 | 80 | 40 | 22 | 14 | 6 |
| 120 | 160 | 80 | 44 | 28 | 12 | 160 | 80 | 44 | 28 | 12 |
| 480 | 640 | 320 | 176 | 112 | 48 | 640 | 320 | 176 | 112 | 48 |
| 960 | 1280 | 640 | 352 | 224 | 96 | 1280 | 640 | 352 | 224 | 96 |
| NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell. |  |  |  |  |  |  |  |  |  |  |

- Synchronous MG interruption for present SCS doesn’t need to be updated.

## LBT impacts on RRM requirements

- RAN4 will define FR2-2 RRM requirements with CCA in Rel-17

- RAN4 will include the RRM requirements with LBT in FR2-2 in the clauses created for NR-U

- Adopt the relaxation methods used in NR-U (core requirements are extended to compensate the missed samples) as a baseline

- RAN4 will reflect the LBT failures by extending the RRM requirements by N*SMTC/SSB occasions when there is at least one SMTC/SSB occasion not available at UE within N*SMTC/SSB occasions, where N is:

  - Option 1: N is equal to the RX beams sweeping scaling factor

  - Option 2: N is defined on case-by-case basis

  - Other options are not precluded

- Maintain the working assumption on the number of SSB successive candidate positions of the same SSB index that UE is required to monitor for RRM requirements with CCA

- FFS whether to apply scaling to the maximum number of allowed measurement occasions to be missed due to LBT failure

- DraftCR Work split

  - DraftCR for FR2-2 LBT support in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state mobility requirements -     Intel

  - DraftCR for FR2-2 LBT support in Radio Link Monitoring and Link recovery procedures -     Huawei

  - DraftCR for FR2-2 LBT support in SCell Activation and Deactivation Delay requirements and Active TCI state switching delay requirements -     MTK

  - DraftCR for FR2-2 LBT support in Measurement procedure -     Nokia

  - DraftCR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change -     vivo

- Following RRM requirements to be defined for LBT support in FR2-2.

| RRM requirements to be defined for LBT support in FR2-2 |  |
| --- | --- |
| Frequency bands grouping | 3.5.3 NR operating bands in FR2 Note: Need to wait for RF agreements related to REFSENS and UE power class |
| Cell reselection | 4.2A.2.2 Measurement of serving cell |
|  | 4.2A.2.3 Intra-frequency |
|  | 4.2A.2.4 Inter-frequency |
| Handover | 6.1B Handover to target cell using CCA |
| RRC Connection Mobility Control | 6.2.1A RRC re-establishment with CCA |
|  | 6.2.3.2.3 RRC release with redirection to NR carrier subject to CCA |
| Signalling characteristics | 8.3A SCell activation and deactivation delay |
|  | 8.9 PSCell addition and release delay |
|  | 8.10A Active TCI state switching delay |
|  | 8.11.PSCell change |
|  | 8.11B Conditional PSCell change |
|  | 8.1A Radio link monitoring |
|  | 8.5A Link recovery procedures |
| Measurement requirements | 9.2A Intra-frequency |
|  | 9.3A Inter-frequency |
|  | 9.5A L1-RSRP measurements for reporting |

Other impacts identified at a later stage are not precluded

- Focus on 38.133 spec update first rather than on impact of FR2-2 LBT on the TS 36.133 specification.

FFS whether the agreements for NR unlicensed operation in FR2-2 can also apply for TS 36.133

# References

- R4-2202734, Email discussion summary for [101-bis-e][217] NR_ext_to_71GHz_RRM_2, Intel, RAN4 #101-bis-e, January 17-25, 2022