3GPP TSG-RAN WG4 Meeting # 101 bis-e 										             R4-2202673

Electronic Meeting, January 17-25, 2022

Agenda item:			6.20.3.4

Source:	CMCC

Title:	WF on use of NCD-SSB or CSI-RS for RedCap UE

Document for:	Approval

- Introduction

This way forward contains agreements of  NCD-SSB or CSI-RS for RedCap UE in RAN4#101-bis-e in email thread [221] NR_redcap_RRM_2 including:

- Further considerations on reply RAN1 LS R1-2110600 for NCD-SSB

- Reply to RAN1 LS R1-2112802 on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE

Further considerations on reply RAN1 LS R1-2110600 for NCD-SSB

Issue 3-1-1:  NCD-SSB is ‘QCL’-ed with CD-SSB

Agreement: RAN4 to assume NCD-SSB is ‘QCL’-ed with CD-SSB when this NCD-SSB and CD-SSB shares the same SSB index

Issue 3-1-2: RRM measurement on serving cell mode

Agreement: RRM measurement on serving cell can be based on CD-SSB or NCD-SSB.

Issue 3-1-3:  RRM measurement on neighbour cell in idle/inactive mode

Option 1: Idle/inactive mobility is only based on measurements on the CD-SSB (RAN agreement)

Option 2: Idle/inactive mobility is feasible based on measurements on both CD-SSB and NCD-SSB.

Issue 3-1-4:  RRM measurement on  neighbour cell in connected mode

- Option 1: In connected mode, RRM measurement on neighbour cell is supposed to be performed on CD-SSB.

- Option 2: In connected mode, RRM measurement on neighbour cell can be based on CD-SSB or NCD-SSB.

Issue 3-1-5:  If RRM measurement on neighbour cell can be based on NCD-SSB, what assisted information network can provide to UE

This issue depends on the outcome of issue 3-1-4.

FFS: The neighbor cell’s NCD-SSB information (frequency/SCS) shall be provided to UE if UE is configured to perform cell identification/measurement on neighbor cell’s NCD-SSB, i.e., UE is not required to read neighbor cell SIB to figure out the neighbor cell’s NCD-SSB by itself.

Issue 3-1-6:  The reference SSB to UE to perform intra-frequency measurement

- Option 1: Network indicates the reference SSB to UE to perform intra-frequency measurements, such as NCD-SSB or CD-SSB.

- Option 2: The reference SSB of serving cell for UE to perform intra-frequency measurements should be CD-SSB.

- Option 3: The reference SSB of serving cell for UE to perform intra-frequency measurements could be CD-SSB or NCD-SSB. No network indication is needed.

Issue 3-1-7:  If RRM measurement on neighbour cell can be based on NCD-SSB, intra-frequency and inter-frequency measurement definition for NCD-SSB

- Option 1: For RedCap UE supporting NCD-SSB measurement, a measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of reference SSB of serving cell (depending on issue 3-1-6) and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same.

- Option 2: For RedCap UE supporting NCD-SSB measurement, a measurement is defined as a SSB based intra-frequency measurement provided

- the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same.

- the subcarrier spacing of the two SSBs are also the same.

- Option 3: RedCap UE’s intra-frequency measurement definition based on the following scenarios:

- 	Case A: Serving cell active BWP with CD-SSB

- 	Case B: Serving cell active BWP with NCD-SSB

- 	Case C: Serving cell active BWP without SSB

Issue 3-1-8:  Serving cell measurement when both CD-SSB and NCD-SSB measurements are configured to the UE

- Option 1: If both CD-SSB and NCD-SSB measurement are configured to RedCap UE but both of them need MG, UE could choose to perform CD-SSB measurement only.

- Option 2: If both CD-SSB and NCD-SSB measurement are configured to RedCap UE, UE should perform the measurements for both SSB based on NW’s configuration.

- Option 3: If both CD-SSB and NCD-SSB measurement are configured to RedCap UE, UE should perform serving cell measurements based on SSB with active BWP.

Issue 3-1-9:  If RRM measurement on neighbour cell can be based on NCD-SSB, neighbour cell measurement when both CD-SSB and NCD-SSB measurements are configured to the UE

This issue depends on the outcome of issue 3-1-4.

FFS: When NW configures MOs with both CD-SSB and NCD-SSB measurements, UE should follow NW’s configuration to perform measurements.

Issue 3-1-6:  NCD-SSB index

- Option 1: ssb-PositionsInBurst for NCD-SSB could be a subset or equal-set of ssb-PositionsInBurst for CD-SSB from the same serving cell.

- Option 2: No change wrt to legacy NR, i.e. up to NW implementation

Issue 3-1-10:  NCD-SSB: frequency locations

- Option 1: NCD-SSB should be configured off sync raster.

- Option 2: No change wrt to legacy NR, i.e. up to NW implementation

Issue 3-1-11:  NCD-SSB: time domain locations

- Option 1: NCD-SSBs and CD-SSB from the same serving cell shall be transmitted in different half-frames when NCD-SSB and CD-SSB are contained in the same SFN.

- Option 2: No change wrt to legacy NR, i.e. up to NW implementation

Issue 3-1-10:  NCD-SSB Tx power

Agreement: No further discussion. Follow the last meeting agreement in R4-2120327

  - TX power of NCD-SSB can be same or different from those of CD-SSB

    - If TX power is different, then UE needs to be informed on the power difference between NCD-SSB and CD-SSB

    - It is RAN4 understanding that if power boosting is used for CD-SSB then it may not be always possible to use the same TX power for NCD-SSB.

Issue 3-1-11:  NCD-SSB periodicity

- Option 1: The UE may retune to receive CD-SSB when the periodicity of NCD-SSB is larger than CD-SSB periodicity.

- Option 2: More clarification on the scenario and FFS

- Option 3: up to UE implementation

Issue 3-1-12:  On potential new procedure

  - Option 1:

    - RedCap UE needs to report the RRM measurement together with the type of RS, either NCD-SSB or CD-SSB once NW doesn’t clearly indicate the ARFCN of the measurements’. For example, NW indicates ‘ServingCellConfigCommon-> downlinkConfigCommon-> FrequencyInfoDL-> absoluteFrequencySSB’ only without ‘NCD-SSB information’, but UE performs the measurements for NCD-SSB.

  - Option 2: No on new procedure for NCD-SSB transmission status acquisition

  - Option 3: up to RAN1/2

Agreement: No discussion in RAN4. Up to RAN1/2

Issue 3-1-13:  On requirements when NCD-SSB is configured

  - Option 1: RAN4 to introduce the new measurement delay requirements for RedCap UE (Ericsson)

    - Cell detection+ CD-SSB measurement reporting + NCD-SSB status detection

    - Cell detection + NCD-SSB measurement reporting

    - NCD-SSB measurement for a known cell

  - Option 2: no need

  - Option 3: FFS

Issue 3-1-14:  NCD-SSB impact on Non-RedCap UE

  - Option 1: In Rel-17, Non-RedCap UE may support the new capability of NCD-SSB transmission status detection.

  - Option 2: Changes/impact on non-RedCap UE are not expected

  - Option 3: up to RAN1/2 decision

Reply to RAN1 LS R1-2112802 on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE

Issue 3-2-1:  NCD-SSB Idle state

Agreement: Capture the following agreements in reply LS

For working assumption “If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective”, from RAN4’s view it is feasible.

Issue 3-2-2:  CSI-RS based method

Agreement: Capture the following agreements in reply LS

For working assumption: “Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities.” From RAN4’s view it is feasible.

For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0):

- A RedCap UE that supports FG 6-1a but NOT support CSI-RS based L3 measurement operates in the BWP

  - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

  - the UE can support SSB based L3 measurement, but cannot support CSI-RS based L3 measurement

- A RedCap UE that supports FG 6-1a and CSI-RS based L3 measurement operates in the BWP

  - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

  - the UE can support both SSB based L3 measurement and CSI-RS based L3 measurement with associated SSB.

- For serving cell timing related requirements, RAN4 agreed to not define requirements based on CSI-RS in Rel-17.