3GPP TSG-RAN WG4 Meeting #101 bis-e	R4-2202675

Electronic Meeting, 17 Jan. – 25 Jan., 2022

Title:	LS on RRM relaxation for Redcap

Response to:

Release:	Release 17

Work Item:	NR_redcap-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

        - Name:       Xusheng Wei

- E-mail Address: xusheng.wei@vivo.com

Attachments: -

1. Overall Description:

- For Rel-17 Redcap WI, RAN4 has discussed on how to specify Redcap RRM relaxation requirements and reached the following conclusions:

- When only Rel-17 relaxation criteria are configured, for the following scenarios, Rel-17 RRM relaxation requirements for Redcap UE in idle and inactive mode will be defined as below:

| 1 | Rel-17 stationary criterion alone is configured and satisfied | YES |
| --- | --- | --- |
| 2 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured and satisfied | YES |
| 3 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured however only Rel-17 stationary criterion is satisfied | YES (same RRM relaxation requirements as scenario 1) |
| 4 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured however only Rel-17 not at cell edge criterion is satisfied | No relaxation |
| 5 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured however none of them is satisfied | No relaxation |
| 6 | Rel-17 stationary criterion alone is configured but not satisfied | No relaxation |

- When both Rel-16 and Rel-17 relaxation criteria are configured, RAN4 agrees that the following cases will be considered in idle and inactive mode:

| 7 | Rel-16 low mobility | Rel-17 stationary | Allowed |
| --- | --- | --- | --- |
| 8 | Rel-16 not-at-cell-edge | Rel-17 stationary | NO |
| 9 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary | TBD |
| 10 | Rel-16 low-mobility | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 11 | Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | TBD |
| 12 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | TBD |

When Rel-17 stationarity is configured and fulfilled, RAN4 concludes that a scaling factor is used for the relaxation requirements.

Regarding higher priority inter-frequency measurement relaxation when only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ or both Rel-17 criteria are satisfied, RAN4 kindly ask RAN2’s guidance on what is RAN2’s methodology designed for this scenario, if there is any.

RAN4 respectfully asks RAN2 to take the above information into account for their future work and provide feedback on RAN4’s question.

- 2	Actions

To RAN WG2

ACTION: RAN4 respectfully asks RAN2 to take the above information into account for their future work and provide feedback on RAN4’s question.

- 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 Meeting #102-e		  	    21 Feb.  – 03 March, 2022, Electronic Meeting

TSG-RAN4 Meeting #103-e		  	    16 May.  – 27 May, 2022, Electronic Meeting