3GPP TSG-RAN WG4 Meeting # 101-bis-e 												R4-2202701

Electronic Meeting, 17th – 25th January 2022

Agenda item:			6.23

Source:	Nokia, Nokia Shanghai Bell, …

Title:	WF on NR_IIOT_URLLC_enh_RRM

Document for:	Approval

# Background

During RAN4#101-bis

agenda item in the RAN4#101-bis e-meeting:

6.23	Enhanced IIoT and URLLC support	[NR_IIOT_URLLC_enh]

* Incoming LS from RAN1: R1-2112834 LS on propagation delay compensation

6.23.1	General	[NR_IIOT_URLLC_enh-Core]

6.23.2	RRM core requirements	[NR_IIOT_URLLC_enh-Core]

6.23.2.1	Propagation delay compensation enhancements	[NR_IIOT_URLLC_enh-Core]

6.23.2.2	Reference point for Te requirements	[NR_IIOT_URLLC_enh-Core]

6.23.2.3	Others	[NR_IIOT_URLLC_enh-Core]

Email discussion: [101-bis-e][239] NR_IIOT_URLLC_enh_RRM

R4-2202576	Email discussion summary: [101-bis-e][225] NR_IIOT_URLLC_enh

Type: other		For: Information

Source: Moderator (Nokia)

Abstract:

Discussion:

Decision:		Return to.

Earlier agreed documents

RAN4#101:

- R4-2120335, WF on NR_IIOT_URLLC_enh_RRM

RAN4#100bis:

- R4-2115371, WF on NR_IIOT_URLLC_enh_RRM

RAN4#99:

- R4-2108368, WF on RRM for NR IIoT and URLLC,

# WF on topic#1 Propagation delay compensation enhancements

12 sub-topics were discussed during the meeting.

## Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking (Sub-topic 1-1)

Tentative Agreement proposed after 1st round:

Agree that RAN4 will not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking.

To be confirmed in 2nd round discussion.

Conclusion of 2nd Round:

Agreement:

Agree that RAN4 will not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking

## Reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC (Sub-topic 1-2)

No consensus on the proposal (option 1) based on the feedback in 1st round. Instead, moderator suggest that RAN4 will focus the discussion on agreeing on assumptions and conditions to be used for simulations.

## Reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC (Sub-topic 1-3)

Agreement:

Agree that RAN4 can reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC

Discussion is closed

## Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP (Sub-topic 1-4)

Tentative Agreement proposed after 1st round:

To progress the work RAN4 assume that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

This can be revisited in next meeting if companies find that such assumption is not feasible.

To be confirmed in 2nd round discussion

Conclusion of 2nd Round:

Agreement:

o	RAN4 assume that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

o	This can be revisited in next meeting if companies find that such assumption is not feasible.

## MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP (Sub-topic 1-5)

Tentative Agreement proposed after 1st round:

To progress the work RAN4 assume that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

This can be revisited in next meeting if companies find that such assumption is not feasible.

To be confirmed in 2nd round discussion

Conclusion of 2nd Round:

Agreement:

o	RAN4 assume that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

o	This can be revisited in next meeting if companies find that such assumption is not feasible.

## Measurement samples (Sub-topic 1-6)

No agreement among companies on the proposal in option 1.

However, there is support for option 2 and no company was against the principle proposed in option 2.

2nd Round discussion needed.

Conclusion of 2nd Round:

WF for next meeting:

- Rel-16 or Rel-17 PRS?

- Number of samples assumed (1 or 4 samples)?

## Side conditions (Sub-topic 1-7)

Side conditions - UE

Tentative Agreement proposed after 1st round:

RAN4 agree to reuse the side conditions defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method 1 with the condition that AWGN channel is assumed and RAN4 will agree on a few candidate Es/Iot values for the simulation assumptions including at least Es/Iot = -3 dB

To be confirmed in 2nd round discussion

Conclusion of 2nd Round:

Tentative Agreement proposed after 2nd round:

RAN4 agree to reuse the side conditions defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method 1 with the condition that at least AWGN channel is assumed, and at least Es/Iot = -3 dB is assumed.

RAN4 will further discuss other channels and candidate Es/Iot values for the simulation assumptions.

During GTW Monday January 24th following was agreed:

- Agreements

  - Rel-17 PDC RTT-based method using PRS as the DL reference signal

    - Reuse the side conditions defined in TS 38.133-10.1.25.2

      - AWGN channel is assumed

      - Es/Iot = -3 dB is assumed

      - Further discuss if other channels and candidate Es/Iot values shall be considered.

    - [4] measurement samples are used for requirements definition

Side conditions – gNB

Tentative Agreement proposed after 1st round:

Reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method with the condition of Es/Iot of 3dB

To be confirmed in 2nd round discussion

Conclusion of 2nd Round:

Agreement:

RAN4 agree to reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method with the condition of Es/Iot of 3dB

## Simulation assumptions (Sub-topic 1-8)

No consensus among companies during the discussion. Options are not considered mutually exclusive. More discussion is needed and companies are encouraged to propose detailed simulation assumptions the the scenarios they see necessary.

More discussion is needed in 2nd round.

Conclusion of 2nd Round:

Tentative agreement in this meeting.

Based on the discussion in 1st and 2nd round moderator propose following:

- RTT-based PDC when using PRS:

  - RAN4 assume AWGN channel, Es/Iot = -3 dB and 4 samples.

  - No simulations are needed for RTT-based PDC when using PRS.

  - RAN4 will reuse the requirements in TS 38.133-10.1.25.2 for Rel-17 RTT-based PDC when using PRS (also reflected in Issue 1-19 tentative agreement)

- PDC RTT based on TRS:

  - Moderator suggests using the updated Nokia table as base for further discussion related to PDC RTT based on TRS:

Simulation assumption for PDC RTT method with UE Rx-Tx time difference based on TRS are as defined in tables 1 and 2 (Moderator modification: [TDL-A (30 ns delay spread, 5Hz)]):

Table 1 General parameters

| Parameter | Value |
| --- | --- |
| Cell layout | 1 serving cell |
| Duplex modes | FDD and TDD |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) |
| Data and CCH load in non-TRS symbols | - 50% utilization in time - 100% RE utilization |
| Cyclic prefix | Normal |
| DRX | OFF |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD |
| Propagation conditions [TS 38.101-4] | AWGN, [TDL-A (30 ns delay spread, 5Hz)] |
| Es/Iot [dB] | -3 |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |
| UE measurement bandwidth | Full carrier bandwidth |

| Company | Comments |
| --- | --- |
| Question 2 | Are the general parameters agreeable (leaving the TDL-A channel in [])? Option 1: yes Option 2: no (if no – please indicate which changes are needed) |
| Nokia | Option 1 The general parameters are basically from the PRS simulation assumptions and we think it is fine to reuse them for TRS simulation. Since whether including fading channel for PRS is under further discussion, then we think the fading channel can also be included in the TRS simulation assumption. |
| Qualcomm | Option 1. We should also agree on the performance metric. We believe it should be the same one used for UE Rx-Tx time difference. - 90%-ile of the TUE-RX errors for serving cell In the above, - TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge). The ideal TOA should be randomized in the simulations. |
| Huawei | Option 1 |
| Ericsson | Option 1: yes. Moreover we agree that metric has to be added. The 90%-ile as proposed by Qualcomm above, is fine. Existing positioning requirements UE/gNB-Rx-Tx time difference are defined for all SCS FR1 and FR2. We think that this should be reflected in simulation assumptions, (It seems SCS=15 and SCS = 30 kHz get some special attention, but not the other SCS). We should have simulation assumptions for all SCS in FR1 and FR2. |
| MediaTek | Option 1. Also, we agree with Qualcomm’s comment regarding the performance metric. |

|  | Summary of extended discussion |
| --- | --- |
| Question 2 | Option 1: Nokia, QC, HW, Ericsson, MTK Option 2: Comments: 1) We should also agree on the performance metric 2) simulation assumptions should also be defined for FR2 Proposal (from R4-2009127): At least the following performance characteristics are to be provided for TUE-RX: - 90%-ile of the TUE-RX errors for each cell and, - TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge). Agreement: Simulation assumptions: - Use the general parameters in the table above (leaving the TDL-A channel in []) - [FFS: Add simulation assumptions for FR2 (120KHz SCS) aligned with R4-2009127] Performance characteristics are to be provided for TUE-RX: - 90%-ile of the TUE-RX errors for each cell and, - TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge). |

Table 2 CSI-RS for tracking transmission configuration parameters

| Parameter | Value |
| --- | --- |
| Number of transmit TRS antennas | 1 |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |
| Number with CSI-RS Resource sets with trs-Info | 1 |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(5MHz), 48 (20MHz),132 (50MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) 30 kHz: 128(5MHz), 64 (20MHz), 32 (50MHz) |
| TRS periodicity | 40ms |
| TRS resources number | 2, 4 |
| TRS symbol location | {4,8} |
| TRS frequency density | 3 |
| TRS samples | 1, 4 |
| Evaluation period | 40 ms, 200 ms |

| Company | Comments |
| --- | --- |
| Question 3 | Is TRS periodicity of 40ms agreeable? Option 1: yes Option 2: no (if no – please indicate which value) |
| Nokia | Option 1. It is aligned with the PRS simulation assumption. |
| Qualcomm | Option 1 |
| Huawei | Option 1 |
| MediaTek | Option 1. |

| Company | Comments |
| --- | --- |
| Question 4 | Are the following TRS parameters agreeable? - TRS resource number: 2, 4 - TRS symbol location: {4, 8} - TRS frequency density: 3 Option 1: yes Option 2: no (if no – please indicate which values) |
| Nokia | Option 1 |
| Qualcomm | For TRS resource number, we understand that 2 corresponds to two resources in one slot, and 4 corresponds to 2 resources in each of two slots. We would prioritize 4 resources so that there are 12 REs per RB per TRS period, which would be the same as one PRS comb per PRS period. The other two parameters are OK. |
| Huawei | Option 1. No strong view if we should prioritize 4 resources. @QC, just for clarification, with 4 resources, the number of REs in two slots and one PRB should be 16 instead of 12. Or do we misunderstand something? @Moderator, there is a typo in the BW row: 30 kHz: 24(10MHz), 48 (20MHz),132 (50MHz) |
| MediaTek | Are the following TRS parameters agreeable? - TRS resource number: 2, 4 - [MediaTek]: To our understating, - the TRS signal in FR1 and FR2 includes 4 CSI-RS across two consecutive slots, so the total OFDM symbols are 4*3 = 12 symbols. - The TRS signal in FR2 includes 2 CSI-RS in a slot. - Hence, we should either clarify that or we can agree on number 4 because it is the generic term in here as the following: - TRS resource number across two consecutive slots: 4 - TRS symbol location: {4, 8} - TRS frequency density: 3 - [MediaTek]: Fine with the rest. Regarding the signal BW, it is not clear why we have modified the TRS signal BW compared to PRS, where PRS signal BW from the LLS simulation assumptions: - 15 kHz: 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: 48 (20MHz),132 (50MHz), 272 (100MHz) While for the TRS, we have included 24PRBs? |
| Nokia | We agree with Huawei that there is a typo in the BW. So it should be TRS transmission bandwidth (in PRBs) - full carrier BW; - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(10MHz), 48 (20MHz),132 (50MHz). Sample rate (Tc); - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz). @MediaTek, the current BW for PRS measurement accuracy in TS 38.133-10.1.25.2 are different from the original PRS simulation proposal, and it has been adjusted to - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(10MHz), 48 (20MHz),132 (50MHz) So in order to align with the current PRS requirement, we think the BW for TRS should be the same as TS 38.133-10.1.25.2. |
| MediaTek2 (copied by moderator) | Thank you, Nokia for the clarification. Based on our understanding, the BW of the TRS signal is either 52 PRBs or follows the size of BWP, as in clause 5.1.6.1.1 [TS 38.133]. Hence, we have the following suggestion is to adjust the BW of TRS as follows: - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(10MHz), 48 52 (20MHz),132 (50MHz) |

| Company | Comments |
| --- | --- |
| Question 5 | Shall 1 sample simulations also be considered? Option 1: yes Option 2: no |
| Nokia | Option 1. Since it is for TRS and we think 1 sample can be included in the simulation to check the performance. |
| Qualcomm | We don’t have a very strong view here. We think it makes sense to prioritize 4 samples since we are planning to leverage accuracy requirements for PRS with 4 samples. |
| Huawei | Option 1. It is reasonable to align the assumption for PRS, and we understand this is only for simulation. |
| vivo | Option 1. Since new simulations are needed, it is more reasonable TRS is based 1 sample as it is typical use case. |
| MediaTek | Similar view to Qualcomm. No strong view but given short time until next meeting and to align with PRS LLS simulation we suggest to keep 4 samples only, yet if no strong view to eliminate 1 sample. |

| Company | Comments |
| --- | --- |
| Question 6 | Are the proposed evaluation periods agreeable (40ms is for 1 sample; 200ms if for 4 samples)? Option 1: yes Option 2: no (if no – please indicate which values) |
| Nokia | Option 1 |
| Qualcomm | Could it be clarified how this is expected to impact the simulations? Is it just number of samples? If yes, then option 1 is fine. |
| Huawei | Option 1. |
| Ericsson | Existing positioning requirements UE/gNB-Rx-Tx time difference are defined for all SCS FR1 and FR2. We think that this should be reflected in simulation assumptions, (It seems SCS=15 and SCS = 30 kHz get some special attention, but not the other SCS). We should have simulation assumptions for all SCS in FR1 and FR2. |
| MediaTek | We can agree on Option 1. |

|  | Summary of extended discussion |
| --- | --- |
| Question 3 | Option 1: Nokia, QC, HW, MTK Option 2: Agreement: TRS periodicity of 40ms is agreeable |
| Question 4 | Option 1: Nokia, QC, HW, MTK Option 2: Assuming: For TRS resource number, 2 corresponds to two resources in one slot, and 4 corresponds to 2 resources in each of two slots. Open: prioritize 4 resources? TRS transmission BW corrected as pointed out by Huawei Comment from MTK: - the TRS signal in FR1 and FR2 includes 4 CSI-RS across two consecutive slots, so the total OFDM symbols are 4*3 = 12 symbols. - The TRS signal in FR2 includes 2 CSI-RS in a slot. - Hence, we should either clarify that or we can agree on number 4 because it is the generic term in here as the following: - TRS resource number across two consecutive slots: 4 Based on input from MTK 268 and 272 signal BW were included. Based on comment from MTK: BW of TRS for 30KHz is corrected from 48 to 52. Agreement: Following TRS parameters are agreeable: - TRS resource number: 2, 4 - TRS symbol location: {4, 8} - TRS frequency density: 3 For TRS resource number, 2 corresponds to two resources in one slot, and 4 corresponds to 2 resources in each of two slots. At least simulation results using 4 resources should be provided. Companies are encouraged to explain the used TRS resource assumptions. |
| Question 5 | Option 1: Nokia, HW, vivo, (QC, MTK – no strong view) Option 2: Note: - QC and HW sees it reasonable to prioritize 4 samples - vivo see it reasonable to prioritize 1 sample Agreement: 1 sample is included |
| Question 6 | Option 1: Nokia, QC, HW, MTK Option 2: Comment: clarified how this is expected to impact the simulations Moderator: this would relate to measurement period and measurement delay requirement. Agreement: Evaluation periods of 40ms for 1 sample and 200ms for 4 samples are agreeable |

WF:

- For CSI-RS: clarify the assumptions related to measurement BW at FR1 15 and 30 kHz SCS? e

- How to handle FR2 and 60, 120 KHz SCS?

## Measurement accuracy requirements (Sub-topic 1-9)

Measurement accuracy requirements – gNB

No consensus

More discussion is needed in 2nd round.

Conclusion of 2nd Round:

Measurement accuracy requirements – gNB

Tentative agreement.

For PDC RTT gNB Rx-Tx time difference measurement accuracy requirements re-use existing gNB Rx-Tx requirements for 3dB side condition and SCS 15/30kHz (Rel-17 38.133, 13.2.2).

WF:

FFS: 60KHz and FR2.

Measurement accuracy requirements - UE

No consensus

More discussion is needed in 2nd round.

Conclusion of 2nd Round:

No agreement in this meeting. However, moderator would propose following based on the company views expressed during 1st and 2nd round:

PRS:

- Initially, RAN4 assume 4 samples, -3dB, AWGN channel and SCS 15/30 KHz.

- For PRS, reuse PDC RTT accuracy from R16 defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method.

FFS: Rel-17 PRS, 1 or 4 samples more discussion and simulations (whether R17 PRS is optional/mandatory is separate discussion).

TRS:

- Align the simulation assumptions for TRS based PDC RTT with the simulation assumption for PRS based PDC RTT.

- Define the TRS based PDC RTT UE measurement accuracy requirements based on simulations using aligned simulation assumption with PRS based PDC method.

- TRS measurement accuracy will be based on simulation results

FFS: 1 or 4 samples.

WF for next meeting:

PRS:

- Shall RAN4 define requirements based on Rel-16 or Rel-17, or for Rel-16 and Rel-17 PRS?

- Number of samples assumed (1 or 4 samples)?

TRS:

- 1 or 4 samples

Companies are encouraged to perform first simulation results for the next RAN4 meeting (RAN4#102)

## Measurement report value range (Sub-topic 1-10)

UE Rx-Tx measurement report value range

Tentative Agreement proposed after 1st round:

RAN4 agree to use the existing reporting range for UE Rx-Tx is re-used for PDC

To be confirmed in 2nd round discussion

Conclusion of 2nd Round:

Agreement:

RAN4 agree to use the existing reporting range for UE Rx-Tx is re-used for PDC.

gNB Rx-Tx measurement report value range

Tentative Agreement proposed after 1st round:

RAN4 agree to use the existing reporting range for UE Rx-Tx is re-used for PDC

To be confirmed in 2nd round discussion

Conclusion of 2nd Round:

Agreement:

RAN4 agree to use the existing reporting range for gNB Rx-Tx is re-used for PDC

## Link simulations (Sub-topic 1-11)

(conditioned 1-1-1)

Agreement:

Agreement: RAN4 need link simulation study for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS

Discussion is closed

## Test case (Sub-topic 1-12)

No consensus in this meeting

The Issue can be discussed further in next meeting.

# WF on topic#2 Reference point for Te requirements

1 Sub-topic was discussed during the meeting

## TP for downlink timing definition  (Sub-topic 2-1)

No consensus

More discussion is needed in 2nd round.

Conclusion of 2nd Round:

No agreement.

After discussion:

- Option 1: [Apple, Nokia, vivo]

- Option 2: [Ericsson, Huawei, QC, Intel]

- Option 3: moderator compromise proposal

RAN4 to continue discussion using the following 3 TP options:

Option 1: [Apple, Nokia, vivo]

The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna

Option 2: [Ericsson, Huawei, QC]

The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna

Option 3:

The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna

WF:

Next meeting is last meeting to reach agreement. All 3 options are open for discussion.

During GTW Monday January 24th following was listed:

Topic #2: Reference point for Te requirements

Proposals

- Option 1: [Apple, Nokia, vivo, OPPO]

  - The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna

- Option 2: [Ericsson, Huawei, QC, CMCC]

  - The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna

- Option 3:

  - The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna

  - No impact on RAN5 conformance test cases is expected

Session chair: make decision in RAN4 #101bis-e

| Company | Comments extended discussion |
| --- | --- |
| Question 7 | Is TP option 3 agreeable? Option 1: yes Option 2: no (if no – please propose updates) |
| Huawei | We appreciate the efforts from the moderator to offer compromise way out, but unfortunately we still have some concern on TP3. In our view, the reference point for defining the requirements should be “objective”, and in this contexts, the true arrival time of the first path should be used. And it should not be something decided by the UE itself. In this sense, TP3 is similar to TP1, i.e. it is still the UE who determines the first path. This is correct for UE behaviour but not for requirement definition. We still prefer TP2 based on above reason. |
| vivo | If the wording ‘first path’ is really so needed, we would like to propose following TP: - The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame detected by the UE is received from the reference cell at the UE antenna - No impact on RAN5 conformance test cases is expected |

|  | Summary of extended discussion |
| --- | --- |
| Question 7 | No agreement. |

# Simulation Assumptions

- 3.1	Simulation assumptions

Table 1 General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 1 serving cell |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in non-TRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD | - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, [TDL-A (30 ns delay spread, 5Hz)] |  |
| Es/Iot [dB] | -3 |  |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |

Table 2 CSI-RS for tracking transmission configuration parameters

| Parameter | Value |  |
| --- | --- | --- |
| Number of transmit TRS antennas | 1 |  |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |  |
| Number with CSI-RS Resource sets with trs-Info | 1 |  |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: 24(10MHz), 52 (20MHz),132 (50MHz), 272 (100MHz) | - 120 kHz: 32(50MHz), 64 (100MHz),128 (200MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz) | FFS |
| TRS periodicity | 40ms |  |
| TRS resources number | 2, 4 |  |
| TRS symbol location | {4,8} |  |
| TRS frequency density | 3 |  |
| TRS samples | 1, 4 |  |
| Evaluation period | 40 ms, 200 ms |  |

At least the following performance characteristics are to be provided for TUE-RX:

- 90%-ile of the TUE-RX errors for each cell

and,

- TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).