3GPP TSG-RAN WG4 Meeting #101-bis-e                	 R4-2202706

Electronic Meeting, 17 – 25 January, 2022

Agenda Item:	6.24

Source:	OPPO

Title:	WF on NR SL relay RRM

Document for:	Approval

- Introduction

This is the WF to capture all agreements and open issues in [101-bis-e][226] NR_SL_relay email discussion for RAN4#101-bis-e meeting.

- Sub-topic 1-1: Measurement requirements for Relay (re)selection

Issue 1-1: Clarification on Discovery period

Tentative agreements:

  - In mode 2 resource reservation period is configured per resource pool. Discovery period can refer to the resource reservation period of shared resource pool if only shared resource pool is configured, or the resource reservation period of dedicated resource pool.

Issue 1-2: Measurement requirements for Relay (re)selection

C

  - The measurement period for intra-frequency relay UE is defined as 4 discovery periods and the evaluation period for intra-frequency relay UE is defined as 16 discovery periods (i.e. 4 measurement periods).

  - For intra-frequency, candidate SL relay UE to be identified/measured and the selected SL relay UE are on the same SL carrier.

  - Do not consider inter-frequency relay in this release

Issue 1-3: DRX impact on Measurement requirements

Tentative agreements:

  - DRX has no impact on RAN4 discovery signal measurement requirement with the assumption that the discovery signal period is the multiples of the common or pre-configured DRX cycle lengths to avoid transmission of discovery signal during DRX off time.

- Sub-topic 1-2: Interruption requirements

Issue 1-4-1: How to define the interruption requirements at discovery configuration based on R16 V2X sidelink assumption?

Tentative agreements:

  - Define interruption on the PCell/serving cell during the RRC reconfiguration procedure

Issue 1-4-2: Whether to further consider the additional conditions on interruption requirements of SL relay?

Candidate options:

  - Option 1: The interruption requirements in this clause shall not apply if at least one of the following conditions is met:

- T310 timer is running for RLF on PCell,

- Performing candidate beam detection on PCell/serving cell as specified in section 8.5.5 and 8.5.6,

- While receiving paging and

- While receiving system information.

  - Option 2: NO, discuss the R17 impact just in R17 SL enhancement WID

- Sub-topic 1-3: RRC connection reestablishment

Issue 1-5: Requirements for RRC connection reestablishment

Tentative agreements:

  - Do not to specify RRC connection re-establishment delay requirement.

- Reference

[1] R4-2202577, Email discussion summary for [101-bis-e][226] NR_SL_relay, Moderator (OPPO), RAN4#101e

[2] R4-2120337, WF on R17 NR SL Relay RRM, OPPO, RAN4#101-bis-e

[3] R4-2115373, WF on R17 NR SL Relay RRM, OPPO, RAN4#100e