3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202715

Online Meeting, Jan 17 – 25, 2022

Agenda item:	8.9.4, 8.9.5

Source: 	Huawei, HiSilicon

Title: 		WF on RRM requirements for Rel-17 NB-IoT and LTE-MTC

Document for:	Approval

# Introduction

This document contains the agreements and discussion in RAN4#101-bis-e in email thread [101bis-e][229] NB_IOTenh4_LTE_eMTC6_RRM.

# Topic#1 Neighbour cell measurements in connected mode before RLF or NB-IoT

## Sub-topic 1-1 intra-frequency measurement requirement

### Issue 1-1-1: Intra-frequency requirement when DRX is not configure

Agreement:

Tdetect_intra = 1400 ms

Tmeasure_intra = 800 ms for NRS-based measurement

Tmeasure_intra = 1600 ms for NSSS-based measurement

### Issue 1-1-2: Intra-frequency requirement when DRX is configured

Agreement:

Tdetect_intra = 6 DRX cycles

Tmeasure_intra = 5 DRX cycle

## Sub-topic 1-2 inter-frequency measurement requirement

### Issue 1-2-1: General – How to formulate inter-frequency measurement

Agreement:

- Define inter-frequency measurement requirements based on number of available samples in available measurement occasions.

- Number of samples are determined based on agreed time in LS reply and sampling rate.

- UE can process multiple samples within in one measurement occasion at a time

- FFS：Add addition assumption on minimum length of measurement occasions to avoid high overhead which will be discussed in issue 1-2-3

### Issue 1-2-2: General – How to formulate inter-frequency detection

Agreement:

- Define inter-frequency detection requirements based on number of available samples in available measurement occasions.

- Number of samples are determined based on agreed time in LS reply and NPSS/NSSS interval.

- UE can process multiple samples within in one measurement occasion at a time

- FFS: Add addition assumption on minimum length of measurement occasions to avoid high overhead which will be discussed in issue 1-2-3

### Issue 1-2-3: Condition on inter-frequency measurement

Sub-1: Interruption due to data traffic

FFS:

Option 1: The measurement period requirements for inter-frequency cell detection and measurements in connected mode shall apply assuming no UL/DL scheduling for the UE during the measurement period.

Sub-2: Minimum length per occasion

FFS:

Option 1

Minimum length of occasion for measurements:  50 ms

Minimum length of occasion for cell detection:  200 ms

Option 2

Minimum length of occasion for cell detection is 21 ms.

Minimum length of occasion for NRS based measurement is 11 ms, and 21ms for NSSS based measurement.

Sub-3: Assumed sampling rate for measurement

Agreement:

- Assumed sampling rate for measurement is 20 ms for NRS and 40 ms for NSSS

Sub-4: Whether to have minimum spacing between occasions.

Agreement:

- No need to require a minimum spacing between measurement occasions

### Issue 1-2-4: Detailed requirements for inter-frequency measurement

FFS:

Option 1:

Without DRX and assuming no UL/DL scheduling, the available time for measurements during one NPDCCH monitoring period would be Tavail=TNPDCCH-LNPDCCH-Tproc-2·TRT, where

TNPDCCH is the NPDCCH monitoring period length,

LNPDCCH is the duration of a NPDCCH candidate with Rmax repetitions,

Tproc = 4 ms is the NPDCCH processing time,

TRT = 1 ms is the receiver retuning time.

Choose the duplexing mode (FDD/TDD) and carrier type (anchor, non-anchor).

For TDD, choose the UL/DL configuration.

Calculate TNPDCCH and LNPDCCH for all applicable values of (G, Rmax ).

Calculate Tavail=TNPDCCH-LNPDCCH-Tproc-2·TRT.

Given T(meas, basic) = 800 ms for NRS-based measurements in normal coverage, calculate the required number

of measurement occasions: Nocc=⌈T(meas, basic)/T(meas,occ) ⌉ .

Calculate the number of available measurement occasions per NPDDCH period:

Navail= ⌈ (Tavail-T(meas,occ))/(T(meas,occ)+Tg ) ⌉+1

Set D=Nocc-⌈Nocc/Navail ⌉·Navail.

For D>0, the measurement period is given by

Tmeas=⌈Nocc/Navail ⌉·TNPDCCH+D·(Tocc+Tg )- Tg+LNPDCCH+Tproc+2·TRT

For D=0, the measurement period is given by
    Tdetect=(⌈Nocc/Navail ⌉-1)·TNPDCCH+Navail·(Tocc+Tg )-Tg+LNPDCCH+Tproc+2·TRT

Option 3:

Requirements for inter-frequency measurement on a carrier different from serving carrier requirements is defined as:

Tmeasure_inter = ∑(i=1)N Min(5000,Ta,i )ms,

where Ta,i is the interval between available measurement samples, where Ta,i ≥ 20 ms for NRS and Ta,i ≥ 40

ms for NSSS. N = 60 for NRS-based measurement and 40 for NSSS based measurement.

UE will restart the measurement when the interval between two samples are larger than 5000 ms, and the delay requirements are extended accordingly.

FFS: Whether and how to define the limitation on total measurement time per inter-frequency carrier

### Issue 1-2-5: Detailed requirements for inter-frequency detection:

FFS:

Option 1:

Without DRX and assuming no UL/DL scheduling, the available time for measurements during one NPDCCH monitoring period would be Tavail=TNPDCCH-LNPDCCH-Tproc-2·TRT, where

TNPDCCH is the NPDCCH monitoring period length,

LNPDCCH is the duration of a NPDCCH candidate with Rmax repetitions,

Tproc = 4 ms is the NPDCCH processing time,

TRT = 1 ms is the receiver retuning time.

Choose the duplexing mode (FDD/TDD) and carrier type (anchor, non-anchor).

For TDD, choose the UL/DL configuration.

Calculate TNPDCCH and LNPDCCH for all applicable values of (G, Rmax ).

Calculate Tavail=TNPDCCH-LNPDCCH-Tproc-2·TRT.

Given T(detect, basic) = 1400 ms, calculate the required number of measurement occasions: Nocc=⌈T(detect, basic)/T(detect,occ) ⌉ .

Calculate the number of available measurement occasions per NPDDCH period:

Navail= ⌈ (Tavail-T(detect,occ))/(T(detect,occ)+Tg ) ⌉+1

Set D=Nocc-⌈Nocc/Navail ⌉·Navail.

For D>0, the detection period is given by

Tdetect=⌈Nocc/Navail ⌉·TNPDCCH+D·(Tocc+Tg )- Tg+LNPDCCH+Tproc+2·TRT

For D=0, the detection period is given by
    Tdetect=(⌈Nocc/Navail ⌉-1)·TNPDCCH+Navail·(Tocc+Tg )-Tg+LNPDCCH+Tproc+2·TRT

Option 4:

Requirements for inter-frequency detection on a carrier different from serving carrier requirements is defined as:

Tdetectinter = ∑(i=1)N Min(5000,Ta,i )ms,

where Ta,i is the interval between available measurement samples, Ta,i ≥ 40 ms and N = 70

UE will restart the measurement when the interval between two samples are larger than 5000 ms, and the delay requirements are extended accordingly.

FFS: Whether and how to define the limitation on total detection time per inter-frequency carrier

## Sub-topic 1-3 Multi carrier operation

### Issue 1-3-1: Requirements when UE is required to monitor multiple carriers

Agreement:

- Requirements for cell detection and measurement when UE is required to monitor multiple carriers are defined as:

  - Tdetect = Tdetect _intra+Nfreq* Tdetect_inter

  - Tmeasure = Tmeasure _intra+Nfreq* Tmeasure_inter

- Where Nfreq is number of inter-frequency layers to be measured according to the measurement capability.

### Issue 1-3-2: Carriers for neighbour cell measurement

FFS:

Option 1:

The UE continues measuring on at least one neighbour cell detected in RRC_IDLE/INACTIVE modes at least once every 5 seconds after transition to RRC_CONNECTED state provided that it belongs to the serving carrier frequency and meets the side condition (Es/Iot >= -6 dB).

## Sub-topic 1-4 Scenarios

### Issue 1-4-1: Whether to deprioritize defining requirements when serving cell in enhanced coverage.

Agreement:

- Define CONNECTED mode neighbour cell measurement requirements when the serving cell is in enhanced coverage

## Sub-topic 1-5 Additional triggering conditions

### Issue 1-5-1: Whether to have additional triggering conditions

FFS:

Option 1: In addition to the already agreed triggering conditions, the UE shall initiate the neighbour cell measurements if K number of out-of-sync indications are detected in the cell.

## Sub-topic 1-6 Indicate when UE needs to start inter-f measurements

### Issue 1-6-1: Whether to indicate when UE needs to start inter-f measurements

Agreements:

It is within in RAN2 scope and no need to further discuss in RAN4

# Topic#2 channel quality reporting table for 16-QAM in TS 36.133

### Issue 2-1-1: Capture channel quality reporting table for 16-QAM in TS 36.133

Agreement:

- Capture the mapping table for channel quality reporting of 16-QAM in TS 36.133, and the corresponding performance requirements will be discussed in Demod session.

### Issue 2-1-2: Capture core requirements of channel quality reporting for 16-QAM in TS 36.133

Agreement:

- The core requirements related to channel quality reporting for 16-QAM (e.g. measurement resource and/or period) will be discussed in Demod session, and the corresponding requirements can be captured in TS 36.133

# Draft CR Work split

### Work split

| Requirements | Section | Spec | Company |
| --- | --- | --- | --- |
| Connected mode intra-frequency neighbour cell measurement before RLF | 8.14.5 (new) 8.14.5.1 (new) | TS 36.133 | Huawei |
| Connected mode inter-frequency neighbour cell measurement before RLF | 8.14.5 (new) 8.14.5.2 (new) | TS 36.133 | Ericsson |
| Channel quality report |  | TS 36.133 | Ericsson |

# 3. References

[1] R4-2202746	Email discussion summary for [101bis-e][229] NB_IOTenh4_LTE_eMTC6_RRM.