3GPP TSG-RAN WG4 Meeting # 101-bis-e 												R4-2202733

Electronic Meeting, 17 – 25 January 2022

Agenda item:			6.16.7, 6.16.7.1, 6.16.7.2

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [101-bis-e][216] NR_ext_to_71GHz_RRM_1

Document for:	Information

# Introduction

In RAN #89e meeting a new WI on Extending current NR operation to 71GHz was approved. During the RAN4 #99 the initial scope of RRM work for NR_ext_to_71GHz WI was defined and captured in the way forward R4-2108354. Further discussion was split into two email threads. The discussion at RAN4 #100-e and RAN4 #101-e in the first email thread can be retraced through R4-2115405 with corresponding WF R4-2115351 and at R4-2120370 with corresponding WF R4-2120316. For the second email thread the discussion can be retraced through R4-2115406 with corresponding WF R4-2115352 for RAN4 #100-e and through R4-2120371 with corresponding WF R4-2120317 for RAN4 #101-e.

Current email discussion document focuses on the general requirements, timing requirements, scheduling restrictions and measurement procedures based on the documents submitted under AIs - 6.16.7, 6.16.7.1, 6.16.7.2

Draft CRs are accepted in this meeting and shall be treated in 2nd round

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: The following list of open issues was identified, based on the contributions, for the 1st round

  - General

    - Deployment scenarios

    - RX beam sweeping scaling factor

    - Terminology updates

    - CR split

  - Scheduling restrictions

    - Beam switching aspects

    - Synchronization aspects

  - Timing requirements

    - UE transmit timing error

    - Gradual timing adjustment

    - MRTD

    - MTTD

  - Measurement procedures

    - Channel model

    - Cell detection

    - PBCH detection for SSB index identification

    - SSB measurements

    - Intra-frequency measurements

  - RRM UE feature list

- 2nd round: The following list of open issues was identified, based on the discussion from the 1st round

  - General

    - Draft CR: Terminology updates

  - Scheduling restrictions

    - Synchronization aspects

    - Scheduling restrictions

    - Draft CR: Scheduling restrictions

  - Timing requirements

    - UE transmit timing error

    - Gradual timing adjustment

    - MRTD

    - MTTD

    - Draft CRs

  - Measurement procedures

    - Draft CRs

  - RRM UE feature list

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200125 | CATT | Proposal 1: For some relevant requirements not defined in FR2-2, the existing term (FR2) needs to be adjusted. - it is suggested to clearly point out that it is only applied to FR2-1 to avoid unnecessary confusion. |
| R4-2200654 | Vivo | Proposal 1: Rx beam scaling factor for FR2-2 need to continue waiting for the RF outcome. Proposal 10: Add the applicability of requirements of FR2 in 38.133 for corresponding requirements which are not defined for FR2-2. |
| R4-2200914 | Nokia | Proposal 1: RAN4 to clarify if EN-DC is within the scope of the requirements defined in Rel-17. |
| R4-2201194 | Huawei, HiSilicon | Observation 1: The RX beam sweeping factor should be further discussed with more RF conclusion on antenna size assumption and power class. |
| R4-2200562 | LGE | Proposal 5: RAN4 needs to revisit the Rx beam scaling factor for RRM measurements in IDLE / INACTIVE / CONNECTED mode after finalizing the antenna assumption for FR2-2. |
| R4-2200655 | Vivo | Draft CR to 38.133 Introducing applicability of requirements for FR2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Deployment scenarios

Sub-topic description: Discussion on various deployment scenarios applicable to the WI and priorities, if any.

Agreement from RAN4#101e:

RRM requirements for Ext_to_71GHz should be defined for the scenarios below in Rel-17.

- FR2-2 single carrier and CA in SA

- FR1+FR2-2 CA (FR1 is PCell)

- FR1+FR2-2 DC (FR1 is PCell)

- Other scenarios are precluded in Rel-17.

Open issues and candidate options before e-meeting:

Issue 1-1-1: Deployment scenarios

- Proposal 1 (Nokia): RAN4 to clarify if EN-DC is within the scope of the requirements defined in Rel-17.

  - Clarify whether FR1 refers only to NR or also to LTE.

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Intel | Our understanding of previous agreement is that FR1 refers only to NR, so no RRM requirements for EN-DC/NE-DC with FR2-2 will be defined in Rel-17 |
| MTK | Same understanding as Intel’s |
| LGE | same view with Intel. Further check RF status of band combination if needed. |
| Qualcomm | Agree with Intel’s views |
| Ericsson | FR1 refers to only NR not LTE. EN-DC is NOT within Rel-17 scope: The WID (RP-213540), clearly defines FR1 as NR FR1 bands (n79, n41 and n77): - For the case of FR2-2 DC or CA with an anchor in FR1 the following three example band combinations shall be considered: - n79 + Nx - n77 + Nx - n41 + Nx - where Nx is the 57-71 GHz band for unlicensed operation and the [66-71] GHz for licensed operation. |
| vivo | Same view with Intel. |
| Apple | We also share the view that FR1 refers to NR only. |

### Sub-topic 1-2: Rx beam sweeping scaling factor

Sub-topic description: Discussion on Rx beam sweeping scaling factor.

Open issues and candidate options before e-meeting:

Issue 1-2-1: Rx beam sweeping scaling factor

- Proposal 1 (Vivo, Huawei, LGE): Rx beam scaling factor for FR2-2 need to continue waiting for more RF conclusion on antenna size assumption and power class.

- Recommended WF

  - Agree on the proposal.

| Company | Comments |
| --- | --- |
| Intel | Ok with Proposal 1 |
| Nokia | Agree with Proposal 1 |
| LGE | Support the proposal 1. |
| Qualcomm | Agree with proposal 1 |
| Ericsson | The WF is fine.. |
| vivo | Support Proposal 1. The antenna size assumption for FR2-2 in RF group is still under discussion. |
| Apple | The WF is OK. |
| CATT | Agree with proposal 1. |

### Sub-topic 1-3: Terminology update

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-3-1: Terminology update for FR2-2

- Proposal 1 (CATT, Vivo): For some relevant requirements not defined in FR2-2, the existing term (FR2) needs to be adjusted.

  - It is suggested to clearly point out that it is only applied to FR2-1 to avoid unnecessary confusion.

- Recommended WF

  - Consider agreeing to the proposal.

| Company | Comments |
| --- | --- |
| Intel | Support Proposal 1 |
| Nokia | We are fine with proposal 1 |
| Qualcomm | Agree with the proposal |
| Ericsson | The WF is fine. |
| vivo | Support Proposal 1. When the FR2 in 38.133 is referred, both FR2-1 and FR2-2 should be considered. However, for some requirements (e.g., inter-band CA within FR2-2) which is not defined in FR2-2, the applicability of requirements need to be clarified. |
| Apple | Agree with the WF. |
| CATT | Support proposal 1. |

### Sub-topic 1-4: CR split

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-4-1: Current CR split

- Proposal 1 (Moderator): The following list of Draft CRs to be prepared by RAN4 were identified during RAN4#101e:

  - Draft CR for timing requirements for FR2-2 – UE transmit timing and Timing Advance – Nokia

  - Draft CR for timing requirements for FR2-2 – MRTD, MTTD - Qualcomm

  - Draft CR for interruption requirements for FR2-2 - vivo

  - Draft CR for active BWP switching delay requirements for FR2-2 - Huawei

  - Draft CR for general measurement requirements for FR2-2 - Ericsson

  - Additional Draft CR(s) shall be considered based on the outcome of open issues

  - Scheduling restriction – Mediatek

- Recommended WF

  - Companies are invited to comment if additional Draft CRs need to be considered.

    - Spec impact due to operation in unlicensed operation for the above topics can be handled in the same Draft CR.

    - Operation in unlicensed band affecting any other requirements will need additional Draft CR.

| Company | Comments |
| --- | --- |
| Intel | Additional work split for Draft CRs related to LBT operation is discussed in Thread 217. |
| Nokia | We are fine with the moderator’s proposal In our view, the additional Draft CRs required due to introducing RRM requirements with LBT in FR2-2, should be discussed in thread 217. |
| Ericsson | No further comments at this stage. |
| vivo | Draft CR for inter/intra frequency measurement related requirements may be needed depending on further discussion. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 Deployment scenarios | Issue 1-1-1: Deployment scenarios Companies’ views: Most companies think that previous agreement from RAN4#101e refers only to NR, so no RRM requirements for EN-DC/NE-DC with FR2-2 will be defined in Rel-17 Tentative agreements: Following the agreement from RAN4#101e, no RRM requirements for EN-DC/NE-DC with FR2-2 will be defined in Rel-17 Candidate options: Recommendations for 2nd round: No further discussion |
| Sub-topic #1-2 Rx beam sweeping scaling factor | Issue 1-2-1: Rx beam sweeping scaling factor Companies’ views: Most companies think that we need to continue waiting for RF conclusions on antenna size assumption and power class before deciding the Rx beam sweeping scaling factor Tentative agreements: Rx beam scaling factor for FR2-2 need to continue waiting for more RF conclusion on antenna size assumption and power class. Candidate options: Recommendations for 2nd round: No further discussion |
| Sub-topic #1-3 Terminology update | Issue 1-3-1: Terminology update for FR2-2 Companies’ views: Most companies agree with the proposal Tentative agreements: For some relevant requirements not defined in FR2-2, the existing term (FR2) needs to be adjusted. - It is suggested to clearly point out that it is only applied to FR2-1 to avoid unnecessary confusion Candidate options: Recommendations for 2nd round: No further discussion |
| Sub-topic #1-4 CR split | Issue 1-4-1: Current CR split Companies’ views: Most companies agree with the proposal Tentative agreements: The following list of Draft CRs to be prepared by RAN4 were identified during RAN4#101e: - Draft CR for timing requirements for FR2-2 – UE transmit timing and Timing Advance – Nokia - Draft CR for timing requirements for FR2-2 – MRTD, MTTD - Qualcomm - Draft CR for interruption requirements for FR2-2 - vivo - Draft CR for active BWP switching delay requirements for FR2-2 - Huawei - Draft CR for general measurement requirements for FR2-2 - Ericsson - Scheduling restriction – Mediatek - Additional Draft CR(s) shall be considered based on the outcome of open issues Note: Additional Draft CRs required due to introducing RRM requirements with LBT in FR2-2, are being discussed in thread 217 Candidate options: Recommendations for 2nd round: No further discussion |

## Discussion on 2nd round

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200655 Draft CR to 38.133 Introducing applicability of requirements for FR2 | Nokia: We suggest replacing the following sentence to make it a bit clearer: - Except for the following cases, the requirements for FR2 are applicable for both FR2-1 and FR2-2. by - Except for the following cases, or when explicitly mentioned, the requirements for FR2 are applicable to both FR2-1 and FR2-2. |
|  | vivo: A typing issue need to be revised. In this contribution R4-2200655, ‘Intra-band CA’ should be replaced with ‘Inter-band CA’. We are fine with Nokia’s comment. |
|  | Apple: Thanks for the CR. We suggest the following wording changes: Unless stated otherwise, the requirements for FR2 are applicable to both FR2-1 and FR2-2, except for the following cases: |

# Topic #2: Scheduling restrictions

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200125 | CATT | Proposal 2: We don’t think we could modify UE assumptions on the maximum timing difference between the reference cell and target cell from 2 SSB symbols to a larger value. Proposal 3: It is suggested to add additional judgment conditions for deriveSSB-IndexFromCell, and whether it is feasible is related to RAN2. - The network could judge deriveSSB-IndexFromCell is set to enabled or not enabled according to the relationship between the error and the length of two SSB symbols. Proposal 4: - Assumptions on deriveSSB-IndexFromCell - Do not change cell phase synchronization accuracy. - For 120kHz SCS - deriveSSB-IndexFromCell is always enabled - For 480kHz and 960kHz SCS - deriveSSB-IndexFromCell may be either enabled or not enabled and up to network configuration - When the network error is less than 3us and greater than 2 SSB symbols - deriveSSB-IndexFromCell is set to enabled - Introduce [2-3] symbols scheduling restriction before and/or after SSB transmission. - When the network error is less than 2 SSB symbols - deriveSSB-IndexFromCell is set to not enabled - keep [1] symbol scheduling restriction before and / or after SSB transmission. |
| R4-2200654 | Vivo | Observation 1: It is possible to distinguish the SSB index if the timing difference between reference cell and target cell can be guaranteed less than 3 symbols. Observation 2: When the coverage is 1km for FR2-2, the timing difference between cells for FR2-2 can be guaranteed within 6.33us. Proposal 2: Scheduling restrictions need to continue waiting for the conclusion about beam switching time from RF session. Proposal 3: DeriveSSB-IndexFromCell should be always enabled for 480kHz of FR2-2. And the deriveSSB-IndexFromCell tolerance requirement should be revised to 3 SSB symbols. Proposal 4: DeriveSSB-IndexFromCell may be either enabled or not enabled which is up to network configuration for 960kHz of FR2-2. And the deriveSSB-IndexFromCell tolerance requirement should be revised to 3 SSB symbols. Proposal 5: If deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified. |
| R4-2200888 | Ericsson | Observation 1: 2 SSB symbols in ’deriveSSB-IndexFromCell’ cannot be kept in terms of the RAN4 considerations on ‘the receiving timing difference between serving cell and neighbour cell does not exceed half of the minimum time difference between the first symbols of two SSBs in order to derive the correct index’. Observation 2: ‘deriveSSB-IndexFromCell’ can allow timing error far more than time length of 2 SSB symbols in terms of the RAN1 considerations on ‘’the UE should establish the frame boundary, and if required, the SFN, without having to decode the BCH payload.’. Observation 3: In 38.133, the statement ‘ the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled)’ implies enabled deriveSSB-IndexFromCell equal to synchronization. If synchronization requirement is met and UE can identify frame boundary without decoding the BCH payload, deriveSSB-IndexFromCell shall be enable. Observation 4: scheduling restriction shall be updated also upon boundary alignment tolerance change in ‘deriveSSB-IndexFromCell’. Observation 5: Scheduling restriction with 1 data symbol for 480KHz and 960KHz SCS is not enough. Proposal 1: 2 symbols for boundary alignment tolerance in ‘deriveSSB-IndexFromCell’ is kept for SCS up to 240KHz. Proposal 2: Because ‘deriveSSB-IndexFromCell is enabled’ is equal to ‘the neighbour cell is synchronous with the serving cell’ from practice perspective, we suggest that: - boundary alignment tolerance in ‘deriveSSB-IndexFromCell’ is relaxed to [4.7] μs or corresponding number of SSB/data symbols for 480KHz and 960KHz, which indeed meets synchronization requirement: the neighbour cell is synchronous with the serving cell allocated in 38.133. Proposal 3: If RAN1 can get agreement on introducing UE capability signaling on UE beam switch time, RRM shall investigate how to introduce the differentiation in scheduling restriction requirements. Proposal 4: Introduce scheduling restrictions shall apply for one symbol before and one symbol after resources (SSB, CSI-RS etc.) used for L1 measurements, if UE’s beam switching time(together with other impairments) influence adjacent symbols. Proposal 5: Assuming that cell phase synchronization accuracy is 3μs, scheduling restriction on L3 measurements needs at least: 2 SSB symbols before and after SSB symbols for 480KHz SCS; 3 SSB symbols before and after SSB symbols for 960KHz SCS Proposal 6: Psharing factor shall be 3 in any case upon 480KHz and 960KHz SCS. Proposal 7: Introduce one symbol scheduling restriction before and after SMTC for beam switch upon 480KHz and 960KHz SCS. |
| R4-2200914 | Nokia | Proposal 2: RAN4 to consider deriveSSB-IndexFromCell is always enabled for 480 and 960 kHz. Proposal 3: RAN4 to revisit deriveSSB-IndexFromCell assumption on maximum allowed frame boundary to be 3 symbols for 960 kHz SCS and 2 symbols for 480 kHz SCS. |
| R4-2200932 | Mediatek | Proposal 1: Relax deriveSSB-IndexFromCell tolerance to be 3 SSB symbols for SSB SCS of 960 kHz. No need to relax for SCS of 480 kHz. Proposal 2: If deriveSSB-IndexFromCell is enabled, introduce K symbols scheduling restriction before and after SSB transmission, where K = 2 for data symbols of 480kHz SCS and K=3 for data symbols of 960kHz SCS. Proposal 3: RAN4 to discuss whether the deriveSSB_IndexFromCell is always enabled or not in unlicensed band in FR2-2. |
| R4-2201194 | Huawei, HiSilicon | Observation 1: If the frame boundary alignment is within a tolerance not worse 3.5 SSB symbols, then UE is able to determine the SSB index without PBCH decoding. Observation 2: Legacy scheduling restriction of 1 data symbol before and after SSB to be measure is not sufficient for operation in FR2-2 with 480 and 960 KHz SCS with cell phase synchronization accuracy unchanged. Observation 3: Based on the consideration of synchronization assumption, it is reasonable to introduce 2 symbols and 3symbols scheduling restrictions before and after SSB to be measured. Observation 4: One more data may be needed for scheduling restriction for beam switching. Proposal 1: For L3 measurement, introduce scheduling restrictions of 3/4 data symbols before each consecutive SSB symbols to be measured and 3/4 data symbols after each consecutive SSB symbols to be measured for 480/960 KHz. Proposal 2: It is suggested to update the assumption of deriveSSB-IndexFromCell for operation in FR2-2 as frame boundary alignment across cells on the same frequency carrier is within a tolerance not worse than min(3.5 SSB symbols, 2 PDSCH symbol) for 480 KHz and min(3.5 SSB symbols, 3 PDSCH symbol) for 960 KHz. And deriveSSB-IndexFromCell can be assumed always enabled in FR2-2. Proposal 3: For L1 measurement, introduce scheduling restrictions of 1 data symbols before each RS to be measured and 1 data symbols after each RS to be measured for 480/960 KHz. |
| R4-2201787 | Intel | Observation 5: For 480kHz and 960kHz SCS the tolerance for frame boundary alignment should be no worse than 3 symbols. Proposal 8: For 480kHz and 960kHz SCS the deriveSSB-IndexFromCell is not always enabled. When deriveSSB-IndexFromCell is enabled frame boundary alignment tolerance is no worse than 3 symbols Proposal 9: For 480kHz and 960kHz SCSs network should schedule sufficient number of guard symbols in its UL-DL TDD configuration to avoid overlapping of UL/DL symbols of reference cell with DL/UL symbols of target cell. The number of guard symbols should depend on the operating SCS and deployment parameters. Proposal 10: For new detectable FR2-2 intra-frequency cell identification the SSB time index detection should be performed. |
| R4-2200562 | LGE | Proposal 1: deriveSSB-IndexFromCell should be up to network configuration for high SCS in FR2-2. Proposal 2: The offset of frame boundary alignment across cells should be less than min(2 SSB symbols, 1 PDSCH symbol) if deriveSSB-IndexFromCell by the network is enabled for high SCS. Observation 1: Due to the shorter symbol length and propagation delay difference between serving and neighbor cell, the SSB symbols could be partially located outside SMTC window duration for high SCS. Proposal 3: Introduce new SMTC window duration (e.g., 1.5ms) for high SCS to maintain all SSB symbols within SMTC window duration. Proposal 4: For high SCS, at least 3 data symbols before and/or after each consecutive SSB symbols should be considered. |
| R4-2200933 | Mediatek | Draft CR to 38.133 Introduction of scheduling restriction for FR2-2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: Beam switching aspects

Sub-topic description: Tentative agreement from RF based on majority view: [59 ns] for gNB beam switching time and [200] ns for UE beam switching time for all SCS

Open issues and candidate options before e-meeting:

Issue 2-1-1: Impact of beam switching time on scheduling restrictions

- Proposal 1 (Vivo): Scheduling restrictions need to continue waiting for the conclusion about beam switching time from RF session.

- Proposal 2 (Moderator): RAN4 to define scheduling restrictions due to UE beam switching based on the tentative agreement from RF - [200] ns for UE beam switching time for all SCS.

  - Note that this is based on the contributions from multiple companies seeking scheduling restriction of 1 OFDM symbol due to UE beam switching time.

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support proposal 2. |
| Intel | Generally ok with proposal 2. Two notes: - UE capability signalling from Issue 2-1-2 need to be considered - Autonomous UE beam switching need to be considered. |
| MTK | Support proposal 2. |
| Nokia | We prefer Proposal 1. |
| LGE | Support proposal 1. RAN4 needs to wait the final decision of RF room, and the impact of beam switching time on scheduling restriction should be considered together with synchronization aspects in sub-topic 2-2 |
| Ericsson | Proposal 2 We suggest continuing scheduling restrictions based on [200] ns. And it is worth noting that the final decision of beam switching time may contain two options with respect to different capability. |
| vivo | Prefer proposal 1. We notice that 200ns is only tentative agreement from RF. If other companies all tend to Proposal 2, we are ok to Proposal 2, e.g., 200ns for UE beam switching time for all SCS. |
| Apple | Either proposal is fine. |
| CATT | Agree with Proposal 1 to wait for the conclusion about beam switching time from RF session. |

Issue 2-1-2: UE capability signalling on beam-switching time

- Proposal 1 (Ericsson): If RAN1 can get agreement on introducing UE capability signalling on UE beam switch time, RRM shall investigate how to introduce the differentiation in scheduling restriction requirements.

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Intel | Support Proposal 1. |
| LGE | Fine with the proposal 1, but we need to wait the final decision of RF room. |
| Ericsson | The proposal can be valid in case of UE capability to support more than one UE beam switching time. |
| Qualcomm | Revisit if RAN1 reaches an agreement |

### Sub-topic 2-2: Synchronization aspects: deriveSSB-IndexFromCell

Sub-topic description: Agreement from RAN4#101e:

- Assumptions on deriveSSB-IndexFromCell

  - Do not change cell phase synchronization accuracy

  - For 120kHz SCS

    - deriveSSB-IndexFromCell is always enabled

  - For 480kHz and 960kHz SCS

    - Option 1: deriveSSB-IndexFromCell is always enabled

      - Option 1A: revisit UE assumptions on the maximum timing difference between the reference cell and target cell from 2 SSB symbols to a larger value

      - Option 1B: introduce [1-3] symbols scheduling restriction before and/or after SSB transmission

    - Option 2: deriveSSB-IndexFromCell may be either enabled or not enabled and up to network configuration

Open issues and candidate options before e-meeting:

Issue 2-2-1: Assumption on deriveSSB-IndexFromCell

- Proposal 1: deriveSSB-IndexFromCell is always enabled for the following in FR2-2:

  - Option 1a (Vivo, Nokia,, Ericsson): 480 kHz SCS

  - Option 1b (Nokia, Ericsson):960 kHz SCS

- Proposal 2: deriveSSB-IndexFromCell may be either enabled or not enabled and up to network configuration for the following in FR2-2:

  - Option 2a (CATT, Intel, LGE): 480 kHz SCS

  - Option 2b (CATT, Vivo, Intel, LGE): 960 kHz SCS

  - Option 2c (CATT): It is suggested to add additional judgment conditions for deriveSSB-IndexFromCell, and whether it is feasible is related to RAN2

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support proposal 2 which is different from our original proposal. |
| Intel | Support Proposal 2. For Proposal 1, at least for 960kHz, even with relaxed tolerance deriveSSB-IndexFromCell can be satisfied only for deployment with cell radius of 50m as it is shown in our paper. We prefer not to put such tough limitations on the deployment |
| MTK | Support Option 2b. For 480 kHz, it depends on how much the propagation delay difference is supported in FR2-2. If the delay difference is less than 1km, then it is likely always enabled. |
| Nokia | We are fine with Option 2b enabling and not enabling deriveSSB-IndexFromCell for 960 kHz |
| LGE | We support proposal 2 for both 480 and 960kHz. |
| Qualcomm | We think it could always be enabled with 480kHz SCS and to handle larger propagation delay differences, the number of SSB symbols could be increased to 3. The case of 960kHz SCS could also be further discussed depending on the deployment conditions. Support Option 1a and FFS on Option 1b |
| Ericsson | From historical standpoint, deriveSSB-IndexFromCell was introduced to determine if extra decoding PBCH isn’t needed or not. In previous RAN4 discussion, the criteria is that timing offset between cells shall be less than the half of timing difference between consecutive SSBs. Before that, in RAN1 discussions, e.g. R1-1811099, even with more than 3.5 symbols timing offset, RAN1 still can determine frame boundary and etc. without decoding PBCH. If RAN4 criteria must be kept, we can support Option 2b and Option1a. |
| vivo | Based on the SSB pattern from FR1, it is possible to distinguish the SSB index if the timing difference between reference cell and target cell can be guaranteed less than 3 symbols. In addition, the timing difference between cells not only depends on cell phase synchronization accuracy, but also is related to the propagation delay difference between reference cell and target cell. As mentioned in our contribution, when the coverage is 1km for FR2-2, the timing difference for cells can be guaranteed within 6.33us which is less than 3 symbols of 480kHz. Therefore, the deriveSSB-IndexFromCell is always enabled. However, for 960kHz, the deriveSSB-IndexFromCell which is enabled or not enabled is up to network configuration. If the network can guarantee that the timing difference is less than 3 symbols of 960kHz, the deriveSSB-IndexFromCell is enabled. |
| Qualcomm | Based on the below GTW agreement: - deriveSSB-IndexFromCell configuration - For 480 kHz SSB SCS - deriveSSB-IndexFromCell is always enabled by the network - For 960 kHz SSB SCS - FFS: deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled) - The agreement applies at least for the case of same SCS in the serving and neighbor cell. FFS whether and how to handle the cases with mixed SCS in the serving and neighbor cells - FFS whether to relax tolerance for UE assumptions on frame boundary alignment We can compromise to leave enabling or disabling of deriveSSB-IndexFromCell up to network configuration under the condition that we also specify maximum frame boundary alignment tolerance when it is disabled. We think we can specify the tolerance as the number of SSB symbols (with 960kHz SCS) that correspond to the agreed number of SSB symbols for 480kHz SCS in Issue 2-2-2. For example, if we agree to specify 2/3 SSB symbols to enable deriveSSB-IndexFromCell for 480kHz SCS, we should also specify 4/6 SSB symbol frame boundary tolerance for 960kHz SCS when deriveSSB-IndexFromCell is disabled. Also, based on the current spec (as highlighted below), we think same SCS on the serving cell and the neighbor cell may be assumed. So no need to consider mixed numerology. When deriveSSB-IndexFromCell is enabled, the UE assumes frame boundary alignment (including half frame, subframe and slot boundary alignment) across cells on the same frequency carrier is within a tolerance not worse than min(2 SSB symbols, 1 PDSCH symbol) and the SFNs of all cells on the same frequency carrier are the same. Proposal 3 (Qualcomm): Based on the agreement from GTW, we propose - deriveSSB-IndexFromCell configuration - For 480 kHz SSB SCS - deriveSSB-IndexFromCell is always enabled by the network - For 960 kHz SSB SCS - deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled) - NOTE: RAN4 to also specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled |
| LGE | For mixed SCS, we checked the deriveSSB-IndexFromCell is for same frequency carrier and intra-frequency measurement is for same SSB SCS. So we are fine to remove the FFS for mixed SCS case in GTW agreements. |
| Apple | We support Option 1a. Option 2b can be further discussed. |
| CATT | We agree with proposal 3. |

Issue 2-2-2: Frame boundary alignment when deriveSSB-IndexFromCell is enabled

- Proposal 1 (Ericsson, Nokia): Keep the existing requirements, i.e., 2 SSB symbols, for SCS up to 240kHz

- Proposal 2: For 480 kHz SCS

  - Option 2a (Vivo, Intel): Revise to 3 symbols

  - Option 2b (Huawei): Revise to 3.5 symbols

  - Option 2c (CATT, Nokia, Mediatek, LGE): Keep the existing requirements, i.e., 2 symbols

- Proposal 3: For 960 kHz SCS

  - Option 3a (Vivo, Nokia, Mediatek, Intel): Revise to 3 symbols

  - Option 3b (Huawei): Revise to 3.5 symbols

  - Option 3c (CATT, LGE): Keep the existing requirements, i.e., 2 symbols

- Proposal 4 (Ericsson):  Revise to [4.7] μs or corresponding SSB/data symbols for 480KHz and 960KHz

- Proposal 5 (Huawei):  Also revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | For proposal 2 and 3: We are fine with 3 symbols or 3.5 symbols Also support proposal 5, one PDSCH symbol needs to be revised. It is related to the scheduling restriction requirements on how many symbols before and after SSB-to-measure that UE is required to monitor. |
| Intel | For Proposal 1: Support. For Proposal 2 and 3: Support 3 symbols and ok with 3.5 symbols For Proposal 5: Prefer not to have differentiation between SSB and PDSCH in requirements. Otherwise, different combinations of SSB/PDSCH SCSs need to be considered |
| MTK | Option 2b & Option 3a |
| Nokia | Support Proposal 1. Support Proposal 2 and 3 with 3 or 3.5 symbols |
| LGE | Support option 2c and 3c. If deriveSSB-IndexFromCell is up to network configuration, existing tolerance for both 480 and 960kHz should be kept when deriveSSB-IndexFromCell is enable |
| Qualcomm | Support proposal 1 Support Proposal 2 and 3 and are fine with 3 or 3.5 symbols. |
| MTK2 | Fixed the Typo Support Proposal 1. Option 2c & Option 3a |
| Ericsson | Support proposal 1. If RAN4 criteria must be kept, we’re ok with 3 or 3.5 symbols for 480Khz and 960Khz. |
| vivo | Support Proposal 1. Support Proposal 2 and 3 with 3 symbols. Also fine with 3.5 symbols. |
| Qualcomm | Support proposal 1 Support Proposal 2 and 3 and are fine with 3 or 3.5 symbols for the case when deriveSSB-IndexFromCell is enabled. Proposal 6 (Qualcomm): For 960 kHz SCS, when deriveSSB-IndexFromCell is disabled Specify the number of SSB symbols (with 960kHz SCS) corresponding to (in absolute time) agreed value for 480 kHz SCS in proposal 2. |
| Apple | Proposal 1 is OK. For proposal 2 and proposal 3, option 2a/b and 3a/b are OK. |
| CATT | Support Proposal 1. Support proposal 2b and 3b which is different from our original proposal. From the perspective of the definition of deriveSSB-IndexFromCell, as long as the receiving timing difference between serving cell and neighbour cell does not exceed half of the minimum time difference between the first symbols of two SSBs, that is, 3.5 symbols, UE could derive the SSB index of the adjacent cell from the timing of the serving cell. |

Issue 2-2-3: Intra-frequency measurement if deriveSSB-IndexFromCell is not enabled

- Proposal 1 (Vivo, Intel): When deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support proposal 1 if deriveSSB-IndexFromCell can be enabled or not enabled as discussed in issue 2-2-1. |
| Intel | Support Proposal 1. Currently there is no requirement for SSB index detection for FR2, since deriveSSB-IndexFromCell is always enabled for FR2. |
| MTK | Proposal 1. |
| Nokia | Support Proposal 1 if deriveSSB-IndexFromCell is not enabled in issue 2-2-1. |
| LGE | Support the proposal 1. |
| Qualcomm | Support proposal 1 but it depends on the outcome of Issue 2-2-1 |
| Ericsson | Ok with Proposal 1. |
| vivo | Support Proposal 1. In the existing requirements, the deriveSSB-IndexFromCell for FR2 is always enabled. However, for FR2-2, the deriveSSB-IndexFromCell is not always enabled. Therefore, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified |
| Apple | Agree with Proposal 1. |
| CATT | Support proposal 1 if deriveSSB-IndexFromCell is not enabled. |

Issue 2-2-4: deriveSSB-IndexFromCell in unlicensed band

- Proposal 1 (Mediatek): Discuss whether the deriveSSB_IndexFromCell is always enabled or not in unlicensed band in FR2-2.

  - Note: It’s not always enabled in FR1 NR-U.

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | More discussion is needed. Suggest to first conclude on licensed band. |
| Intel | Prefer to focus on licensed band first |
| MTK | Discussion is needed since It’s not always enabled in FR1 NR-U. |
| Nokia | Fine with discussing it. If could be kept as FFS for this meeting. |
| Qualcomm | Need more discussion. Fine with FFS for this meeting |
| Ericsson | FFS |
| vivo | Fine with FFS. |
| Apple | Fine with FFS, but wonder if there is any reason that it should not be enabled for unlicensed band. |
| CATT | Agree with FFS. |

Issue 2-2-5: Guard symbols in UL-DL TDD configuration

- Proposal 1 (Intel): For 480kHz and 960kHz SCSs network should schedule sufficient number of guard symbols in its UL-DL TDD configuration to avoid overlapping of UL/DL symbols of reference cell with DL/UL symbols of target cell. The number of guard symbols should depend on the operating SCS and deployment parameters.

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We are wondering what is special for FR2-2 about handling the cross link interference if it is the intention of proposal 1. It is up to NW configuration considering the scenarios and SCS as mentioned in proposal 1. |
| Intel | Our proposal is mostly informative – just to mention that more guard symbols are needed for high SCSs. We don’t see any related spec impact. |
| Nokia | Do we need an agreement if there is no spec impact? |
| Ericsson | We suppose it isn’t a specific issue on FR2-2 and no need to update spec. |
| vivo | We understand this may depend on the NW configuration. |
| Apple | We have the same question: do we need such an agreement? |

Issue 2-2-6: SMTC for 480/960 kHz SCS

- Proposal 1 (LGE): Introduce new SMTC window duration (e.g., 1.5ms) for high SCS to maintain all SSB symbols within SMTC window duration.

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We prefer not to have new SMTC configuration. We believe it is similar with MG discussion where no new gap pattern is introduced. |
| MTK | prefer not to have new SMTC configuration. |
| Nokia | Not clear the final goal of the proposal. We prefer not to have new SMTC for the time being. |
| LGE | No new MG pattern is introduced in the last meeting. However, our proposal is to try to reduce data tx/rx loss due to long SMTC (MG) for high SCS. Considering cell phase synchronization accuracy when deriveSSB-IndexFromCell is not enable, SSB might be partially located outside 1ms SMTC window. If longer SMTC window is configured to cover all SSBs within SMTC window, MG length should be also longer. It would be inefficient. So we prefer to introduce new SMTC window for 480 and 960kHz, i.e., 1.5ms SMTC window. |
| MTK2 | Agree with Proposal 1 in general. Besides, it also needs to consider MRTD between CCS, i.e. the gap should be keep for MRTD + DL/UL switch. |
| Ericsson | We suppose the issue can be mitigated by proper SSBs allocation in present SMTC. |
| vivo | We understand this is kind of optimization. It should be up to RAN1 to decide. |
| Apple | Further discussion is preferred. |

### Sub-topic 2-3: Scheduling restrictions

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-3-1: Scheduling restrictions during L1 measurement

- Proposal 1 (Ericsson, Huawei): Introduce scheduling restrictions for one symbol before and one symbol after the measurement resources (SSB, CSI-RS etc.)

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support proposal 1 which is for beam switching. |
| Qualcomm | We think an additional symbol will be needed to account for the beam switching time as it exceeds the CP duration. Support proposal 1. |
| Ericsson | Because of longer beam switching time than CP length, the switching needs to reserve a symbol. |
| vivo | If the UE beam switching time for all SCS is 200ns in Issue 2-1-1, Proposal 1 is OK. |
| Apple | Proposal 1 is OK. |
| CATT | We support proposal 1 based on the impact of the beam switching. |

Issue 2-3-2: Scheduling restrictions during L3 measurement

- Proposal 1: Introduce K symbols scheduling restriction before and after SSB transmission (due to synchronization error) –

  - For 480 kHz SCS,

    - K=2 (CATT, Ericsson, Mediatek, Huawei)

    - K=3 (LGE)

  - For 960 kHz SCS,

    - K=3 (CATT, Ericsson, Mediatek, Huawei, LGE)

- Proposal 1a (Ericsson, Huawei): In addition to K, introduce scheduling restrictions for additional one symbol before and one symbol after the SSB transmission (due to beam switching delay)

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support K=2 for 480 and K=3 for proposal 1 and also support proposal 1a. It means totally 3/4 symbols for scheduling restriction. |
| MTK | Support K=2 for 480 and K=3 for proposal 1. |
| LGE | We support 3 symbols scheduling restriction for both 480 and 960kHz. |
| Qualcomm | Support K=2 for 480 and K=3 for proposal 1 and also support proposal 1a. A total of 3 symbols are needed for 480kHz and 4 symbols for 960kHz. |
| Ericsson | K=2 and K=3 for 480Khz and 960Khz SCSs. And, because of longer beam switching time than CP length, the switching needs to reserve a symbol. |
| vivo | For Proposal 1, we understand the synchronization error not only depends on cell phase synchronization accuracy, but also is related to the propagation delay difference between reference cell and target cell which depends on the conclusion of Issue 3-3-2. Therefor we suggest pending this issue until there is agreement for Issue 3-3-2. Proposal 1a is fine. |
| Qualcomm | We agree with vivo, additional symbols may be needed depending on the propagation delay difference, TA adjustment accuracy etc. Need further study depending on Issue 3-3-2. |
| Apple | Support K=2 for 480kHz and K=3 for 960kHz SCS for proposal 1. Also proposal 1a is OK. |
| CATT | We prefer proposal 1 and proposal 1a, and we also agree with vivo to wait for the conclusions on Issue 3-3-2. |

Issue 2-3-3: ‘Psharing factor’ in beam management

- Proposal 1 (Ericsson): Psharing factor in beam management, e.g., L1-RSRP, BFD and RLM shall be 3 in any case for 480KHz and 960KHz SCS

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We would like to further discuss if the symbols for L1 measurement including additional symbols for beam sweeping are not overlapped with SSB-to-measure and symbols for L3 scheduling restriction, is it still necessary to define P as 3. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1: Beam switching aspects | Issue 2-1-1: Impact of beam switching time on scheduling restrictions Companies’ views: Many companies prefer to wait for agreements on beam switching time from RF Tentative agreements: Scheduling restrictions due to beam switching time need to continue waiting for the conclusion about beam switching time from RF session Candidate options: Recommendations for 2nd round: Continue discussion if agreements on beam switching time is reached in the RF session during the first round. |
|  | Issue 2-1-2: UE capability signalling on beam-switching time Companies’ views: Some companies prefer to wait for RAN1 and/or RF agreements on UE capability signaling for beam switching Tentative agreements: Revisit how to introduce the differentiation in scheduling restriction requirements, after RAN1 agreement on UE capability signaling on beam-switching time. Candidate options: Recommendations for 2nd round: No discussion needed |
| Sub-topic 2-2: Synchronization aspects: deriveSSB-IndexFromCell | Issue 2-2-1: Assumption on deriveSSB-IndexFromCell GTW agreements: - deriveSSB-IndexFromCell configuration - For 480 kHz SSB SCS - deriveSSB-IndexFromCell is always enabled by the network - For 960 kHz SSB SCS - FFS: deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled) - The agreement applies at least for the case of same SCS in the serving and neighbor cell. FFS whether and how to handle the cases with mixed SCS in the serving and neighbor cells - FFS whether to relax tolerance for UE assumptions on frame boundary alignment Companies’ views: After the GTW agreement, some companies noted that the case of mixed SCS in the serving and neighbor cells is not applicable for deriveSSB-IndexFromCell. One company also proposed to specify the frame boundary alignment tolerance even when deriveSSB-IndexFromCell is disabled for 960kHz Candidate options: Option 3 (Qualcomm): - deriveSSB-IndexFromCell configuration - For 480 kHz SSB SCS - deriveSSB-IndexFromCell is always enabled by the network - For 960 kHz SSB SCS - deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled) - NOTE: RAN4 to also specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled - FFS whether to relax tolerance for UE assumptions on frame boundary alignment Recommendations for 2nd round: Continue discussion in the second round and see if Option 3 can be agreed. Frame boundary alignment tolerance is discussed under Issue 2-2-2 |
|  | Issue 2-2-2: Frame boundary alignment when deriveSSB-IndexFromCell is enabled Companies’ views: Most companies agree to keep the existing requirements, i.e., 2 SSB symbols, for SCS up to 240kHz. Companies that support relaxing the tolerance for 480kHz SCS are fine with 3 or 3.5 SSB symbols, some companies also prefer to keep the existing requirements of 2 symbols for 480kHz SCS. Tentative agreements: Keep the existing requirements, i.e., 2 SSB symbols, for SCS up to 240kHz FFS: For 480kHz SCS: - Option 1a: 3 SSB symbols - Option 1b: 2 SSB symbols FFS: For 960kHz SCS: when deriveSSB-IndexFromCell is enabled - Option 2a: 3 SSB symbols - Option 2b: 2 SSB symbols FFS: For 960kHz SCS: when deriveSSB-IndexFromCell is disabled - Option 3a: 6 SSB symbols - Option 3b: 4 SSB symbols FFS: Revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS Candidate options: Proposal 2: Frame boundary alignment: For 480kHz SCS: - Option 1a: 3 SSB symbols - Option 1b: 2 SSB symbols For 960kHz SCS: when deriveSSB-IndexFromCell is enabled - Option 2a: 3 SSB symbols - Option 2b: 2 SSB symbols For 960kHz SCS: when deriveSSB-IndexFromCell is disabled - Option 3a: 6 SSB symbols - Option 3b: 4 SSB symbols Proposal 3: Revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS Recommendations for 2nd round: Continue discussion in the second round |
|  | Issue 2-2-3: Intra-frequency measurement if deriveSSB-IndexFromCell is not enabled Companies’ views: Most companies agree that SSB index detection requirements for intra-frequency measurements need to be defined for 960kHz spacing in FR2-2, if deriveSSB-IndexFromCell is not enabled by the network Tentative agreements: Depending on Issue 2-2-1, if deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified for 960kHz SCS Recommendations for 2nd round: No further discussion needed. |
|  | Issue 2-2-4: deriveSSB-IndexFromCell in unlicensed band Companies’ views: Most companies prefer to keep this issue FFS in this meeting Tentative agreements: FFS whether the deriveSSB_IndexFromCell is always enabled or not in unlicensed band in FR2-2. - Note: It’s not always enabled in FR1 NR-U. Recommendations for 2nd round: No further discussion needed |
|  | Issue 2-2-5: Guard symbols in UL-DL TDD configuration Companies’ views: Most companies believe this configuration is up-to the network and no agreement specific to FR2-2 is needed on this. Tentative agreements: No agreement needed Recommendations for 2nd round: Issue closed; no further discussion needed |
|  | Issue 2-2-6: SMTC for 480/960 kHz SCS Companies’ views: Most companies believe no new SMTC window duration is needed. Tentative agreements: Candidate options: - Proposal 1 (LGE): Introduce new SMTC window duration (e.g., 1.5ms) for high SCS to maintain all SSB symbols within SMTC window duration. - Proposal 2 (Moderator): No new SMTC window duration is needed for high SCS. Recommendations for 2nd round: Continue discussion in the second round, and see if proposal 2 can be agreed. |
| Sub-topic 2-3: Scheduling restrictions | Issue 2-3-1: Scheduling restrictions during L1 measurement Companies’ views: Most companies believe that scheduling restriction for one symbol before and after the measurement resources is needed for FR2-2 during L1 measurements Tentative agreements: Introduce scheduling restrictions for one symbol before and one symbol after the measurement resources (SSB, CSI-RS etc.) during L1 measurements for 480/960kHz SCS Candidate options: Recommendations for 2nd round: No further discussion |
|  | Issue 2-3-2: Scheduling restrictions during L3 measurement Companies’ views: While most companies agree that 2/3 symbol scheduling restrictions are needed for 480/960kHz SCS, some companies note that additional symbols may be needed to account for beam switching time, propagation delay difference, TA adjustment etc. Tentative agreements: - Introduce a total of K (due to synchronization error) + L (due to beam switching time) + M (due to propagation delay difference, TA adjustment etc) symbols scheduling restriction before and after SSB transmission – - For 480 kHz SCS, - K=2 - For 960 kHz SCS, - K=3 - L = 1 for 480/960kHz SCS (based on Issue 2-3-1) - FFS: M for 480/960kHz SCS - Consider propagation delay difference of 3.33us considering FR2-2 coverage of 1km Candidate options: - FFS: M for 480/960kHz SCS Recommendations for 2nd round: Continue discussion in the second round and finalize M |
|  | Issue 2-3-3: ‘Psharing factor’ in beam management Companies’ views: Not enough comments Tentative agreements: Candidate options: - Proposal 1 (Ericsson): Psharing factor in beam management, e.g., L1-RSRP, BFD and RLM shall be 3 in any case for 480KHz and 960KHz SCS Recommendations for 2nd round: Continue discussion in the second round |

## Discussion on 2nd round

### Sub-topic 2-2: Synchronization aspects: deriveSSB-IndexFromCell

Sub-topic description: GTW Agreement from the first round:

- Assumptions on deriveSSB-IndexFromCell

  - For 480 kHz SSB SCS

    - deriveSSB-IndexFromCell is always enabled by the network

  - For 960 kHz SSB SCS

    - FFS: deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled)

  - The agreement applies at least for the case of same SCS in the serving and neighbor cell. FFS whether and how to handle the cases with mixed SCS in the serving and neighbor cells

  - FFS whether to relax tolerance for UE assumptions on frame boundary alignment

Open issues and candidate options before e-meeting:

Issue 2-2-1: Assumption on deriveSSB-IndexFromCell

- Recommended WF : Consider agreeing to the following:

  - deriveSSB-IndexFromCell configuration

    - For 960 kHz SSB SCS

      - deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled)

      - NOTE: RAN4 to also specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled

| Company | Comments |
| --- | --- |
| vivo | Agree with the Recommended WF. |
| Nokia | We don’t understand why to specify the frame boundary alignment when deriveSSB-IndexFromCell is disabled. |
| Ericsson | Agree with Recommended WF. Same question as Nokia. How to use it? |
| Huawei | Agree with the first bullet. For the second bullet, share the same view as Nokia and Ericsson, what is the impact of RRM requirements or PBCH reading requirements. |
| Qualcomm | We think deriveSSB-IndexFromCell can always be enabled for 960kHz SSB SCS as well. According to TS 38.213 (see below), a UE can determine the LSBs of the SSB index using PBCH DMRS. This means that the UE doesn’t need to decode PBCH even if the frame boundary alignment is quite large. For FR2-2, we are talking about a frame boundary misalignment of 3us (no need to consider propagation delay difference as intra-frequency cells are colocated). So, we think deriveSSB-IndexFromCell can be always enabled for 960kHz SCS as well and the tolerance could be relaxed to 3 symbols. - Proposal 1 (Qualcomm): deriveSSB-IndexFromCell can be always enabled for 960kHz SCS From TS 38.213 (Clause 4.1): The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to Lmax-1, where Lmax is determined according to SS/PBCH block patterns for Cases A through E. Lmax is a maximum number of SS/PBCH block indexes in a cell, and the maximum number of transmitted SS/PBCH blocks within a half frame is Lmax. - For operation without shared spectrum channel access, Lmax=Lmax - For operation with shared spectrum channel access, Lmax=8 for Lmax=10 and 15 kHz SCS of SS/PBCH blocks and for Lmax=20 and 30 kHz SCS of SS/PBCH blocks For Lmax=4, a UE determines the 2 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211]. For Lmax>4, a UE determines the 3 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211] |
| LGE | We are fine with the recommended WF. But we are not sure whether the NOTE is needed |
| Apple | The WF seems ok, except the note which needs further clarification. Also, our understanding is deriveSSB-IndexFromCell is for intra-freq. measurement, but it doesn’t mean neighbor cells are co-located. |
| CATT | Agree with the Recommended WF. However, we don't understand why the frame boundary alignment tolerance should be specified even if deriveSSB-IndexFromCell is disabled? For what Qualcomm mentioned, we checked the contents of deriveSSB-IndexFromCell in 38.133(Clause 7.7.1), which is reproduced below: When deriveSSB-IndexFromCell is enabled, the UE assumes frame boundary alignment (including half frame, subframe and slot boundary alignment) across cells on the same frequency carrier is within a tolerance not worse than min(2 SSB symbols, 1 PDSCH symbol) and the SFNs of all cells on the same frequency carrier are the same. According to the current spec, for intra CA, only co-located deployment is applied. According to our understanding, on the same frequency carrier is not equal to intra CA and does not mean neighbor cells are co-located. Therefore, the propagation delay cannot be ignored directly |
| Intel | Agree with recommended WF Regarding the Note, our understanding is that if deriveSSB-IndexFromCell is enabled then synchronization is within tolerance, if deriveSSB-IndexFromCell is disabled then synchronization is worse than tolerance. However, even for deriveSSB-IndexFromCell is disabled we might need some limitation on how bad synchronization is, since we are operating in TDD. Regarding the comment from QC: We agree that such approach can resolve SSB index ambiguity between 8 SSB indexes by PBCH DM-RS detection. To our understanding when deriveSSB-IndexFromCell is enabled PBCH DM-RS detection is not mandatory and such procedure is upto UE implementation. So, we prefer not to allow always enabled deriveSSB-IndexFromCell relying on that procedure. Based on implementation, it still can be used with deriveSSB-IndexFromCell disabled, but some tolerance for synchronization in that case is still required, since the procedure can resolve upto 8 SSB indexes. In that sense we are ok to define tolerance for deriveSSB-IndexFromCell is disabled. Propagation delay difference can not be ignored, since neighbor cells a not necessarily co-located |
| Qualcomm | Considering Intel’s comment, we are fine with leaving deriveSSB-IndexFromCell to be enabled or not up-to the network. And we understand that propagation delay cannot be ignored but we still think that defining the tolerance when deriveSSB-IndexFromCell is disabled is useful as it enables the UE to detect the SSB time index without decoding PBCH and using PBCH-DMRS only. We would like the group to consider this. Recommended WF: - deriveSSB-IndexFromCell configuration - For 960 kHz SSB SCS - deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled) - FFS - NOTE: RAN4 to also specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled |

Issue 2-2-2: Frame boundary alignment when deriveSSB-IndexFromCell is enabled

- Proposal 1: Frame boundary alignment:

  - For 480kHz SCS:

  - Option 1a: 3 SSB symbols

  - Option 1b: 2 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled

  - Option 2a: 3 SSB symbols

  - Option 2b: 2 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is disabled

  - Option 3a: 6 SSB symbols

  - Option 3b: 4 SSB symbols

- Proposal 2: Revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS

- Recommended WF: Consider agreeing to the following

- Frame boundary alignment:

  - For 480kHz SCS:

  - 2 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled

  - 3 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is disabled

  - 4 SSB symbols

- Revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS

| Company | Comments |
| --- | --- |
| vivo | Support Option 1a and Option 2a. For 480kHz and 960kHz, when derive SSB-IndexFromCell is enabled, we prefer that the frame boundary alignment is 3 SSB symbols based on the SSB pattern from RAN1. Support Option 3a. For 960kHz, when derive SSB-IndexFromCell is not enabled, we prefer that the frame boundary alignment is 6 SSB symbols considering the coverage is 1km and cell phase synchronization accuracy is 3us. |
| Nokia | We support Option 1b – 2 symbols for 480 kHz Option 2a - 3 symbols for 960 kHz with deriveSSB-IndexFromCell enabled Proposal 2 – 2 PDSCH symbols for 480 kHz and 3 PDSCH symbols for 960 kHz SCS. As for deriveSSB-IndexFromCell disabled, we have the same comments as in the last issue. We understand we are discussing the UE assumption on frame boundary alignment so that I can derive the index from the neighbors when deriveSSB-IndexFromCell is enabled. We need a bit more explanation why why having requirements on the UE assumption for derives-IndecFromCell when it is not enabled. Wouldn’t the cell phase synchronization accuracy be enough? |
| Ericsson | For 480KHz SCS, we are ok with 2 or 3 SSB symbols. For 960kHz SCS, support 3 SSB symbols. |
| Huawei | For 480 KHz SCS, we prefer 3 SSB symbols For 960 KHz SCS, we prefer 3 SSB symbols. Revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS It should be noted that these are also for deriveSSB-IndexFromCell. For the case when deriveSSB-IndexFromCell is disable, we share the same views as Nokia. It needs further justification. |
| Qualcomm | As commented on Issue 2-2-1, we think deriveSSB-IndexFromCell can always be enabled for 960kHz SCS as well. And since deriveSSB-IndexFromCell is used only for intra-frequency cells, which are assumed to be co-located, the propagation delay difference is zero. So only cell-phase synchronization error need to be considered. - Frame boundary alignment: - For 480kHz SCS: - 2 SSB symbols - For 960kHz SCS - 3 SSB symbols |
| LGE | Fine with the recommended WF. But it doesn’t need when dreiveSSB-IndexFromCell is disable |
| Apple | We have similar question as Nokia and Huawei about the case when deriveSSB-IndexFromCell is disable. |
| CATT | - Frame boundary alignment: - For 480kHz SCS: - 3 SSB symbols - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled - 3 SSB symbols For 480kHz SCS and 960kHz SCS when deriveSSB-IndexFromCell is enabled, we support option 1a and option 1b respectively. Considering the frame boundary alignment may include cell phase synchronization accuracy and the propagation delay, 2 SSB symbols may not enough. Taking into account the SSB pattern in RAN1, the maximum can not exceed 3.5 symbols. For 960kHz SCS when deriveSSB-IndexFromCell is disabled, it suggest to not specify or add UE frame boundary assumption. |
| Intel | We don’t see the reason, why we should differentiate SSB and PDSCH symbols. Our preference for tolerance: 480kHz: 3 symbols. Smaller tolerance will significantly limit coverage (to less than 200m cell radius) 960kHz: deriveSSB-IndexFromCell is enabled – 3 symbols deriveSSB-IndexFromCell is disabled – 6 symbols |
| Qualcomm | We understand that propagation delay needs to be considered, with that we agree with Intel’s proposal: 480kHz: 3 symbols. 960kHz: deriveSSB-IndexFromCell is enabled – 3 symbols deriveSSB-IndexFromCell is disabled – 6 symbols |

Issue 2-2-6: SMTC for 480/960 kHz SCS

- Proposal 1 (LGE): Introduce new SMTC window duration (e.g., 1.5ms) for high SCS to maintain all SSB symbols within SMTC window duration

- Recommended WF: Consider agreeing to the following:

  - No new SMTC window duration is needed for high SCS.

| Company | Comments |
| --- | --- |
| Nokia | Agree with WF |
| Ericsson | Agree with Recommended WF |
| Huawei | Agree with WF |
| Qualcomm | Agree with the recommended WF |
| LGE | Considering rx timing difference and to reduce data tx/rx loss due to long SMTC (MG) for high SCS, we prefer to consider new SMTC window duration as 1.5ms only for 480/960kHz SCS. But, we are fine with recommended WF if all companies have the same views not to introduce new SMTC window. |
| Intel | Agree with Recommended WF |

### Sub-topic 2-3: Scheduling restrictions

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-3-2: Scheduling restrictions during L3 measurement

- Proposal 1 (Moderator): Introduce a total of K (due to synchronization error) + L (due to beam switching time) + M (due to propagation delay difference, TA adjustment etc) symbols scheduling restriction before and after SSB transmission –

  - For 480 kHz SCS,

    - K=2

  - For 960 kHz SCS,

    - K=3

  - L = 1 for 480/960kHz SCS (based on Issue 2-3-1)

  - FFS: M for 480/960kHz SCS

    - Consider propagation delay difference of 3.33us considering FR2-2 coverage of 1km

- Recommended WF

  - Consider agreeing to proposal 1 and discuss values for M

| Company | Comments |
| --- | --- |
| vivo | Support the total scheduling restriction symbols should be 7 symbols and 4 symbols for 960kHz and 480kHz respectively. We understand that both synchronization error and propagation delay difference can be considered together. The total time is 6.33us include the synchronization of 3us and the delay difference of 3.33us. According to the slot length of 1.125us and 2.25us for 960kHz and 480kHz respectively, the total scheduling restriction symbols should be 7 (6+1) symbols and 4 (3+1) symbols for 960kHz and 480kHz respectively considering 1 symbol due to beam switching time. |
| Ericsson | We understand the rational of the proposal. The gap between consecutive SSB indexes is 3 symbols. Counting K+L+M, the scheduling restriction is greater than 4-5 symbols for 480KHz and 7-8 symbols for 960KHz. But, the question is how to cope with present SSB pattern. Firstly, L (due to beam switching time) doesn’t happen between SSBs indexes, One SMTC has same beam. Secondly, there are only 3 symbols between consecutive SSB indexes. Anyhow, all PDCCH/PDCCH symbols will not be used. |
| Huawei | Support WF. Whether M is needed needs further discussion. For the beam switching time, we think it is still needed. For instance, if not SSB candidate positions are transmitted, then it is still possible that for data scheduling.Then the scheduling restriction for beam switching time is needed. We think the 3 symbols between consecutive ssb indexes is not a problem. Because in legacy requirements, SSB index can be transmitted adjacently when we still have scheduling restriction on one symbol before and after SSB. |
| Qualcomm | We don’t think we need to consider propagation delay difference for scheduling restrictions in FR2-2. Scheduling restrictions are defined for intra-frequency measurements and inter-frequency measurement without gaps. For FR2-2 we only have intra-frequency measurement, where neighbor cells are assumed to be co-located. So no propagation delay difference. Support the values of K and L. We can specify M =1 to account for any TA adjustment accuracy, TA resolution error etc. @Ericsson, as per our understanding, the beam switching time is to switch the beam from SSB to DL/UL data, and because UE is switching it’s beam it cannot transmit or receive data during that time. And although there are only three symbols between consecutive SSB indexes, the UE may not be configured to measure all SSB indexes. In worst case, when the UE is configured to measure all the SSB indexes, effective scheduling restriction would be X symbols before and after the SSB burst. But this may not always be true and only few SSB indices may be configured for measurement. |
| LGE | We think the scheduling restriction is when deriveSSB-IndexFromCell is enable. For clarification, do we need M value? In our understanding, the tolerance of dervieSSB-IndexFromCell is already considered for sync error and propagation delay. |
| Apple | We agree L is still needed to accommodate UE beam switching. We also think propagation delay difference is needed for inter-freq. measurement, where collocation may not be the case. |
| CATT | Agree with vivo to consider the total scheduling restriction symbols, support 4 symbols for 480kHz and 7symbols for 960kHz. For M for 480 / 960khz SCS, as described in Issue 2-2-1, we support considering propagation delay. |
| Qualcomm | We agree that propagation delay cannot be disregarded and needs to be considered. We agree with vivo’s comment to support 4 symbols for 480kHz and 7symbols for 960kHz as total scheduling restriction time. |

Issue 2-3-3: ‘Psharing factor’ in beam management

- Proposal 1 (Ericsson): Psharing factor in beam management, e.g., L1-RSRP, BFD and RLM shall be 3 in any case for 480KHz and 960KHz SCS

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Huawei | We suggest to wait for the conclusion of scheduling restriction symbols. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200933 Draft CR to 38.133 Introduction of scheduling restriction for FR2-2 | Nokia: We are in general ok with the text, but the specific values for K should wait for further agreements. The list explaining where the K values are explained is using the wrong style. It is using "Normal" when it should be using "B1" according to 3GPP drafting rules. |
|  | vivo：In general, we agree with the content. The specific values of K need to wait for the outcome of Issue 2-3-2. |
|  | CATT: Agree to wait for the outcome of Issue 2-3-2 to determine the specific values of K. |

# Topic #3: Timing requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200126 | CATT | Proposal 1: It is reasonable to choose Te that always meets the condition TCP – TCH 2 (Te + TAC,Q /2 ) > 0. Proposal 2: For 480 kHz SCS and 960 kHz SCS, TCH reduction should be considered. Otherwise, we need to be able to accept the loss of transmission performance. - One possible method is to limit the application scenarios, such as indoor, tens of meters of coverage environment or in a good propagation. Observations: For other options with SSB SCS > UL SCS, the Te requirement is easier to meet than SSB SCS = UL SCS. Proposal 3: For SSB and UL SCS combinations, - It is reasonable to support other options with SSB SCS > UL SCS. - It may also be possible to define requirements for the combination of (120, 480) and (480, 960), depending on the channel delay spread and whether the reserved margin time is sufficient for the UE implementation. Proposal 4: The percentage of UL CP length Te can occupancy should be < 30%. Proposal 5: It may be necessary to consider the new MRTD requirements for FR2-2, and when defining the MRTD requirements in FR2-2, it is based on the following principles: - For asynchronous cases: MRTD = 0.5 slot - For synchronous cases: MRTD = TAE + propagation delay difference Proposal 6: For FR2-2 and FR2-2, there is no propagation delay difference. Proposal 7: For FR1 and FR2-2, the deployment assumptions between FR1 BS and FR2-2 BS will have an impact on MRTD. - If there are no specific limitations or large changes compared with legacy deployments, the deployment delay difference may not change. Proposal 8: The MRTD requirements for FR1 and FR2-2 inter-band CA depend on the conclusions of TAE and the propagation delay difference requirements. Proposal 9: MRTD for FR1 and FR2-2 NR DC - For inter-band synchronous cases: Wait for conclusions on TAE and the propagation delay difference requirements. - For inter-band asynchronous cases: Reuse the existing MRTD requirements for FR1 and FR2-1, that is, half the slot length with respect to the larger SCS of the MCG and SGC cells. |
| R4-2200283 | Apple | Observation 1: Considering the narrow beam in FR2-2, the actual delay spread is smaller than the ones used in the channel model. Proposal 1: When considering the upper limit of Te, 50% of UL CP length is used. Proposal 2: Requirements for the following cases can be deferred: - 120, 960 - 120, 480 - 480, 960 - Other options with SSB SCS > UL SCS Proposal 3: RAN4 considers the testing and core requirement together to speed up the progress of the core requirement discussion. |
| R4-2200661 | Vivo | Proposal 1: The maximum channel delay spread shall be 20ns for both 480kHz and 960kHz when defining the Te requirements. Proposal 2: RAN4 shall not define the Te requirements for the combination of (120kHz SSB SCS, 960kHz UL SCS) and (480kHz SSB SCS, 960kHz UL SCS). Proposal 3: For the Te requirements, update Table 7.1.2-1 for FR2-2 as below: Table 7.1.2-1: Te Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te. 1; 15; 15; 12*64*Tc. 30; 10*64*Tc. 60; 10*64*Tc. 30; 15; 8*64*Tc. 30; 8*64*Tc. 60; 7*64*Tc. 2-1; 120; 60; 3.5*64*Tc. 120; 3.5*64*Tc. 240; 60; 3*64*Tc. 120; 3*64*Tc. 2-2; 120; 120; 3.5*64*Tc. 480; 1.7*64*Tc. 480; 120; 3.5*64*Tc. 480; 1.7*64*Tc. 960; 120; 3.5*64*Tc. 480; 1.7*64*Tc. 960; 0.7*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. Proposal 4: RAN4 shall evaluate how much DL timing estimation error can be improved when using TRS. Proposal 5: For UL SCS of 480/960 kHz, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last 40 ms. Proposal 6: The propagation delay difference of FR2-2 should be 3.33us considering the coverage of 1km. Proposal 7: The MRTD requirement for intra-band non-contiguous NR CA need to continue waiting for the TAE conclusion from RF group. Proposal 8: The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2. Proposal 9: The MRTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MRTD requirements for inter-band NR carrier aggregation, update Table 7.6.4-2 for FR2-2 as below: Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8 note1. Between FR1 and FR2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA.. Proposal 10: The existing MRTD requirements for inter-band synchronous DC for FR1 and FR2-1 can be reused for inter-band synchronous DC for FR1 and FR2-2. Proposal 11: The MRTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MRTD requirements for inter-band synchronous NR DC, update Table 7.6.6-1 for FR2-2 as below: Table 7.6.6-1: Maximum receive timing difference requirement for inter-band synchronous NR DC Frequency Range; Maximum receive timing difference (µs). Cell in MCG; Cell in SCG. FR1; FR1; 33. FR2-1; FR2-1; 8. FR1; FR2; 33. Proposal 12: The MRTD requirements for inter-band asynchronous NR DC shall the half of slot length. Proposal 13: For the MRTD requirements for inter-band asynchronous NR DC, update Table 7.6.6-2 for FR2-2 as below: Table 7.6.6-2 Maximum receive timing difference requirement for inter-band asynchronous NR DC Max {Sub-carrier spacing in PCell (kHz), Sub-carrier spacing in PSCell (kHz)}; Maximum receive timing difference (µs). 15; 500. 30; 250. 60; 125. 120; 62.5. 480; 15.625. 960; 7.8125. Proposal 14: The existing MTTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2. Proposal 15: The MTTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MTTD requirements for inter-band CA, update Table 7.5.4-1 for FR2-2 as below: Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of TAGs; Maximum uplink transmission timing difference (µs). FR1; 34.6. FR2-1; 8.5 Note1. Between FR1 and FR2; 26.1. Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA.. Proposal 16: The existing MTTD requirements for inter-band synchronous NR DC for FR1 and FR2-1 can be reused for inter-band synchronous NR DC for FR1 and FR2-2. Proposal 17: The MTTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MTTD requirements for inter-band synchronous NR DC, update Table 7.5.6-1 for FR2-2 as below: Table 7.5.6-1: Maximum uplink transmission timing difference requirement for inter-band synchronous NR DC Frequency Range; Maximum uplink transmission timing difference (µs). Cell in MCG; Cell in SCG. FR1; FR1; 34.6. FR2-1; FR2-1; 8.5. FR1; FR2; 34.1. Proposal 18: The MTTD requirements for inter-band asynchronous NR DC shall the half of slot length. Proposal 19: For the MTTD requirements for inter-band asynchronous NR DC, update Table 7.5.6-2 for FR2-2 as below: Table 7.5.6-2 Maximum uplink transmission timing difference requirement for inter-band asynchronous NR DC Max {Sub-carrier spacing in PCell (kHz), Sub-carrier spacing in PSCell (kHz)}; Maximum uplink transmission timing difference (µs). 15; 500. 30; 250. 60; 125. 120; 62.5. 480; 15.625. 960; 7.8125. Observation 1：In [38.808], the channel model of FR2-2 from RAN1 is AWGN and TDL-A (5/10/20ns delay spread). |
| R4-2200915 | Nokia | Observation 1: Study phase results in 38.808 show analysis that suggest delay spreads of more than 30 ns for indoor scenarios above 52.6 GHz. Proposal 1: When defining the margin for the Te calculation, consider a maximum channel delay spread of TCH = 40 ns for 480 kHz SCS, and TCH = 30 ns for 960 kHz SCS. Observation 2: From UE perspective, the boundaries for Te can be determined considering the DL timing detection accuracy eRS and the expected clock drift eDRIFT. Proposal 2: Choose Te such that the condition eRS+eDRIFT < Te < (TCP-TCH-TAC,Q)/2 holds, where eRS is the DL timing estimation accuracy, eDRIFT is the clock drift for the RS periodicity, TCP is the CP length, TCH is the channel delay spread, TAC,Q is the timing advance command step. Observation 3: A small X implies in small period between SSBs large overhead and inefficient network resource usage and should be avoided. Proposal 3: Adopt SSB availability of X=80 ms. Observation 4: A practical eMBB scenario might be using a large SCS for enabling a wider BWP in DL, while using a narrower BWP with small SCS in the UL. Observation 5: There is no technical reason to prevent scenarios with SSB SCS > UL SCS since it can only be easier from the UE implementation perspective due to the improved DL timing estimation accuracy provided larger SSB SCS. Proposal 4: RAN4 to define Te requirements for all the cases with SSB SCS > UL SCS, i.e.: -480 kHz SCS for SSB and 120 kHz UL SCS -960 kHz SCS for SSB and 120 kHz UL SCS -960 kHz SCS for SSB and 480 kHz UL SCS Observation 6: UE capabilities discussion is still ongoing on RAN1, and it is not prevented to have a UE that supports a different set of SCS in DL and UL. That means that it is not prevented that an UE could support 960 kHz SCS in UL and not support it on DL. Observation 7: Initial access SSB configurations do not include 960 kHz SCS. Therefore for resource optimization, some network deployments might want to use 960 kHz Proposal 5: RAN4 to define Te requirements for all the cases with SSB SCS < UL SCS, i.e.: -120 kHz SCS for SSB and 480 kHz UL SCS -120 kHz SCS for SSB and 960 kHz UL SCS -480 kHz SCS for SSB and 960 kHz UL SCS Observation 8: For the combination of 960 kHz SCS in UL and 120 kHz SCS in DL, the usage of TRS enables enough UE margin for the definition of Te. Observation 9: TRS is mandatory UE feature and Ues expect it to be configured in RRC connected mode. Observation 10: Since TRS is only configured n RRC_connected, Te values derived from TRS are not applicable to PRACH in initial access, and SSB DL timing detection accuracy have to be considered in that situation. Proposal 6: The Te requirements for the SCS combination of 960 kHz in UL with 120 kHz DL can be relaxed for PRACH transmission in initial access in relation to the lower bound TCP - TCH - 2 ( Te + TAC,Q /2 ) > 0. Proposal 7: Adopt the following Te requirements for the new SCSs used in FR2-2: SCS of SSB/TRS signals (kHz); SCS of uplink signals (kHz); Te. 120; 480; 0.9*64*Tc. 960 (Note1); 0.5*64*Tc. 960 (Note 2); 0.8*64*Tc. 480; 120; 3.1*64*Tc. 480; 0.5*64*Tc. 960; 0.4*64*Tc. 960; 120; 3*64*Tc. 480; 0.45*64*Tc. 960; 0.35*64*Tc. Note 1: Considering TRS as reference signal for DL timing estimation Note 2: For PRACH only, relaxed requirements considering DL timing estimation based on 120 kHz SCS SSB.. Observation 11: Scenario with FR2-2 inter-band CA and NR-DC is not agreed to be defined in RRM before there are further agreements from RF. Proposal 8: No need to discuss propagation delay for MRTD while FR2-2 only inter-band CA and NR-DC is not included in the RRM deployment scenarios. Observation 12: The MRTD requirements for asynchronous cases are defined for the closest slot boundary without taking into account the slot index. Proposal 9: For Inter-band synchronous NR-DC between FR1 and FR2-2 reuse the existing MRTD requirements for FR1 and FR2-1. Proposal 10: For Inter-band asynchronous NR-DC between FR1 and FR2-2 reuse the existing MRTD requirements for FR1 and FR2-1, i.e. MRTD is half of the slot duration. |
| R4-2200934 | Mediatek | Observation 1: For the higher SCS, the timing error is dominated by the RF mismatch and margin. Observation 2: The timing error budget for RF mismatch and margin will be reduced to ~2.1 Ts and ~1 Ts for UL SCS of 480k Hz and 960 kHz, respectively Proposal 1: Introduce a new UE capability for supporting UL timing accuracy. Observation 3: Network could schedule one UE at one UL slot to avoid UL interference between UEs when UE cannot support accurate UL timing. Proposal 2: UE shall not transmit UL transmissions on the 1st UL symbol and the last UL symbol of scheduled UL slots, if the UE cannot meet the new Te requirement for SCS of 480 kHz / 960 kHz. Proposal 3: For intra-band non-contiguous CA, 1 slot margin is reserved for misalignment. For FR1 and FR2-2 CA, K slots (s) are reserved, where K is FFS depending on the MRTD value. Proposal 4: If the scheduling cell and scheduled cell are with different SCS, keep the current assumption which says “TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change. |
| R4-2201195 | Huawei | Observation 1: Define Te requirements to guarantee the performance for typical scenarios. Proposal 1: Define requirements for the case when SSB SCS > UL SCS in addition to the agreed cases, and discuss whether to have requirements for (120, 480) and (480, 960). Observation 2: The timing estimation accuracy based on TRS depends on bandwidth of TRS. Observation 3: The timing error of UE UL transmission is unpredictable if only defining requirements for short SSB periodicity Proposal 3: RAN4 to define Te requirements for typical SSB periodicity of 20 ms and a separate relaxed Te requirements for longer SSB periodicity. Proposal 4: Define Te requirements for following two cases: SSB is available in the last 20 ms for SSB SCS ≥ UL SCS. The percentage of UL CP that Te can occupy is 30%. SSB is available in the last 80 ms for SSB SCS ≥ UL SCS. The percentage of UL CP that Te can occupy is 40%. Table I. Te when SSB is available in last 20ms SSB SCS (kHz); UL SCS (kHz); Te; Te/CP. 480; 120; 1.6*64*Tc; 8.8%. 480; 1*64*Tc; 22.3%. 960; 120; 1.4*64*Tc; 7.7%. 480; 0.8*64*Tc; 17.8%. 960; 0.65*64*Tc; 29.0%. Table II. Te when SSB is available in last 80ms SSB SCS (kHz); UL SCS (kHz); Te; Te/CP. 480; 120; 1.9*64*Tc; 10.5%. 480; 1.2*64*Tc; 26.7%. 960; 120; 1.6*64*Tc; 8.8%. 480; 1*64*Tc; 22.3%. 960; 0.85*64*Tc; 37.9%. Observation 4: TAE requirements for FR2-2 is under discussion in RF session. Proposal 5: Take following propagation delay difference assumptions for considerations: - Same propagation delay difference as FR2-1 - Shorter propagation delay differences, e.g. 1000 m Proposal 6: For asynchronization case, define MRTD/MTTD as half slot for corresponding SCS. |
| R4-2201407 | ZTE | - Proposal 1: Keep the Te within the same percentage of the CP length as existing SCS (< 30%). - Proposal 2: Te can occupy 30% of UL CP length if UE implementations are proven to be impossible to keep current Te requirements. - Proposal 3: Define at least two sets of Te requirements based on SSB periodicity for UL SCS of 480/960 kHz. At least two Te can be defined based on the assumption that if an SSB is available in the last 40 ms and 160 ms. |
| R4-2201583 | Ericsson | Observation 1: The combination of SSB SCS = 120 kHz and UL SCS = 960 kHz is with a proposed Te = 0.63*64*Tc get very small margin. Proposal 1: Te Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te; UL CP (%). 2-2; 120; 480; 1.3*64*Tc; 28%. 120; 960; 0.63*64*Tc; 28%. 480; 120; 0.25*64*Tc; 1.3%. 480; 480; 0.25*64*Tc; 5%. 480; 960; 0.25*64*Tc; 11%. 960; 120; 0.17*64*Tc; 1%. 960; 480; 0.17*64*Tc; 4%. 960; 960; 0.17*64*Tc; 7%. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. Proposal 2: MRTD = 0.26 µs for non-contiguous intra band CA in FR2-2. Proposal 3: MRTD = 8 µs for FR2-2 CA inter band. Proposal 4: MRTD = 25 µs for inter band carrier aggregation between FR1 and FR2-2. Proposal 5: MRTD = 33 µs for inter-band asynchronous EN-DC, for SCG SCS = 480 kHz and SCG SCS = 960 kHz. Proposal 6: MRTD = 33 µs for inter-band synchronous EN-DC, for SCG SCS = 480 kHz and SCG SCS = 960 kHz. |
| R4-2201787 | Intel | Observation 1: - RMS delay spread of 40ns and larger should not be considered as typical for 960kHz operation. - For timing offset equal to 50% of CP length PUSCH BLER reaches 1% in very high SNR region for both 480kHz (with DS=40ns) and 960kHz (with DS=20ns) operation - For timing offset equal to 30%of CP length and lower no major demodulation issues observed Proposal 1: RAN4 to define the Te requirements as 1.34*64*Tc and 0.67*64*Tc for UL SCSs 480kHz and 960kHz respectively Observation 2: The Tq requirements do not put any limitation on the minimum step size or rate, but it is not reasonable to have the requirements on step size which is larger than the target tolerance. Thus, existing values of Tq can not be reused for 480kHz and 960kHz SCSs. Proposal 2: RAN4 to define Tq and Tp requirements for 480kHz and 960kHz considering that it is preferred to have them smaller than Te value. Observation 3: RF margin value considered for LTE, FR1 and FR2 is too large comparing to the proposed Te value. Proposal 3: RAN4 to define Tq requirements for 480kHz and 960kHz equal to (0.6875*64*Tc + RF_margin). RAN4 to determine RF_margin value, which could be reasonable for high frequencies of FR2-2. Observation 4: For high SCSs we see two options on how to account RTD Option 1: RTD reflects the whole time difference between carriers caused by TAE and propagation delay difference. RTD is considered between the boundaries of the slots which were sent simultaneously. Option 2: RTD reflects only slot boundary misalignment without considering the slot index. RTD is considered between the closest slot boundaries (Rel-15/Rel-16 definition) Proposal 5: In case of NR carrier aggregation RAN4 to consider the receive timing difference between carriers as the timing difference between the slot boundaries of the slots which were sent simultaneously Proposal 6: In case of NR DC RAN4 to consider the receive timing difference between carriers as the timing difference between the closest slot boundaries. Proposal 7: In case of NR DC with high SCS the UE is always in asynchronous conditions with MRTD shown in Table 2.3-1 Table 2.3-1 MRTD for NR DC Max {Sub-carrier spacing in PCell (kHz), Sub-carrier spacing in PSCell (kHz)}; Maximum receive timing difference (µs). 480; 16.625. 960; 7.8125. |
| R4-2202033 | Qualcomm | Observation 1: RAN4 specifies the UE transmit timing based on the SSB bandwidth. Observation 2: An additional margin in the timing error is specified to account for UE artifacts related to DL to UL switching. Observation 3: The upper limit on the timing error is half CP length on the uplink transmission. Observation 4: Reduction in CP length of the UL transmission for higher SCS of 480/960kHz leaves the UE with very little timing estimation error margin which does not scales down linearly with wider DL SSB bandwidth. Observation 5: Even with Te of 0.5*CP, some scenarios (e.g., SSB SCS 120, UL SCS 960) may not meet the UL timing accuracy requirements given an extremely small error margin. Observation 6: Current RAN4 requirements on availability of an SSB in the last 160ms leads to a timing drift of 16ns (with 0.1ppm frequency error) which contributes to the UL timing error margin. Proposal 1: For UL SCS of 480/960 kHz, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last 20ms. Observation 7: RAN1 assumed the following mandatory delay spread for link level simulations [TS 38.808]: TDL model as defined in of TR38.901 Clause 7.7.2: - TDL-A (5ns, 10ns, 20ns DS) CDL model as defined in of TR38.901 Clause 7.7.1: - CDL-B (20ns, 50ns DS) - CDL-D (20ns, 30ns DS) with K-factor = 10 dB Proposal 2: To evaluate RAN4 RRM requirements for FR2-2, assume the following delay spread values: - 20ms for 480kHz SCS - 10ms for 960kHz SCS Observation 8: Based on the condition TCP - TCH - 2 ( Te + TAC,Q /2 ) > 0, and assuming DS of 20ms and 10ms for 480kHz and 960kHz SCS respectively, we get a Te/TCP ratio of 37.7% Observation 9: For a Te/TCP ratio of 37.7%, the UE can meet the UL accuracy requirements for most cases if an SSB is available in the last 20ms. Proposal 3: Based on the discussed condition TCP - TCH - 2 ( Te + TAC,Q /2 ) > 0, and a delay spread of 20ms and 10ms for 480kHz and 960kHz SCS respectively, propose to use a Te/TCP ratio of 37.7% in determining UL timing accuracy requirements. FFS : 120kHz SSB SCS and 960kHz UL SCS. |
| R4-2200563 | LGE | Proposal 1: Use the following principle for MTTD/MRTD requirements in FR2-2, and overall MRTD/MTTD requirements could be defined after finalizing the TAE value discussion. - For MRTD - In synchronous case: TAE + propagation delay difference - In asynchronous case: half slot - For MTTD - MRTD + UL transmit error Proposal 2: No further discussion on propagation delay for FR2-2 MTTD/MRTD requirements is needed. Proposal 3: The shorter MRTD between FR1 and FR2-2 inter-band CA should be further discussed based on real network deployment and UE baseband processing. Proposal 4: Define 33us MRTD for 480 and 960kHz SCS of asynchronous inter-band NR-DC |
| R4-2200916 | Nokia | Draft CR adding timing requirements for FR2-2 |
| R4-2202048 | Qualcomm | Draft CR for timing requirements for FR2-2 – MRTD, MTTD |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: UE transmit timing error

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1-1: Basic principles for defining Te

- Proposal 1 (Nokia, CATT): Choose Te such that the condition Te < (TCP-TCH-TAC,Q)/2  holds.

- Proposal 2 (Nokia): RAN4 to choose Te such that the condition eRS+eDRIFT < Te < (TCP-TCH-TAC,Q)/2  holds.

  - where eRS is the DL timing estimation accuracy

  - eDRIFT is the clock drift for the RS periodicity

  - TCP is the CP length

  - TCH is the channel delay spread

  - TAC,Q is the timing advance command step

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We suggest to discuss the Te value and applicable scenarios directly. As analyzed in our paper, it is hard to come up with a general principle, which may easily lead to a Te value which exceeds UE’s capability. |
| Nokia | We agree with proposals 1 and 2. We think it is good to have general guidelines for evaluating feasible Te values. |
| Ericsson | Proposal 1 for Te is ok. Proposal 2 for e_rs + e_drift is ok. |
| vivo | If agreements can be reached for Issue 3-1-4, then we don’t need to discuss the principle. |
| Apple | We want to emphasize that UE implementation challenges need to be properly considered. |
| CATT | We agree with Nokia, and think it is better to have general guidelines for evaluating feasible Te values for analysis. Considering the UE capability, we can focus more attention on the application scenario and SCS combination. |

Issue 3-1-2: Channel delay spread for defining Te

- Proposal 1 (CATT, Apple): For 480 kHz SCS and 960 kHz SCS, TCH reduction should be considered

- Proposal 2 (Vivo): The maximum channel delay spread shall be 20ns for both 480kHz and 960kHz when defining the Te requirements

  - RAN1 considered AWGN and TDL-A (5/10/20ns delay spread)

- Proposal 3 (Nokia): When defining the margin for the Te calculation, consider a maximum channel delay spread of TCH = 40 ns for 480 kHz SCS, and TCH = 30 ns for 960 kHz SCS.

- Proposal 4 (Intel): RMS delay spread of 40ns and larger should not be considered as typical for 960kHz operation. Consider 40ns for 480KHz and 20ns for 960kHz.

- Proposal 5 (Qualcomm): To evaluate RAN4 RRM requirements for FR2-2, assume a delay spread of 20ms for 480kHz SCS and 10ms for 960kHz SCS based on RAN1 TDL-A (5/10/20ns delay spread) model

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Slight prefer proposal 2. |
| Intel | Prefer Proposal 4. Ok with Proposal 2 |
| Nokia | We agree with proposal 3. This is in line with the results from 38.808 clause 6.3 on typical scenarios for operation above 52.6 GHz. In the study phase at least one company identified DS of 45 ns, and other companies identified 30 ns or less for 80 to 90% of the UEs. Contributions on that topic include R1-2007654, R1-2007982, R1-2008615. We can compromise with Proposal 4. |
| Qualcomm | We support proposal 5. RAN1 considered delay spreads of up-to 20ns for TDL-A channels during study phase. Not to mention the timing related challenges were not considered during the study phase. Meeting timing requirements would be very challenging considering longer delay spreads and hence we support Proposal 5. We are okay to compromise to proposal 2. |
| Ericsson | We prefer 40 ns also for 960 kHz. |
| vivo | Support Proposal 2. The channel model from RAN1 can be considered as baseline. Proposal 1 and proposal 5 is also fine. |
| Apple | As observed in our paper, due to narrow beams in FR2-2, the actual delay spread is smaller than the ones used in the channel model. Proposal 5, 20ns for 480kHz SCS and 10ns for 960kHz SCS, is a good starting point. |
| CATT | Slight prefer proposal 4. |

Issue 3-1-3: SSB periodicity

- Proposal 1:  For UL SCS of 480/960 kHz, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last X ms.

  - Option 1 (Qualcomm, Apple, Huawei): X=20ms

  - Option 2 (Vivo, ZTE): X=40ms

  - Option 3 (Nokia): X=80ms

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We suggest to consider two set of requirements based on different SSB periodicity. At least one for 20 ms SSB to cover the most typical scenario. |
| Intel | Support Option 2. We can also agree on Option 1, though it provides significant overhead. |
| Nokia | We prefer Option 3. From the margin calculations we presented in our paper, we identified that it is possible to define Te values with 80 ms interval. Smaller X would imply in very heavy use of resources for reference signals; therefore we would like to avoid it to be mandated to be too small. |
| Qualcomm | We support option 1, i.e. X=20ms but can compromise to X=40ms for 480kHz SCS. For 960kHz SCS, X=20ms is required to consider defining Te requirements. |
| Ericsson | We prefer option 3 based on that we prefer to stay as close to existing 160 ms period as possible to avoid draining UE power. It is true hat one pays a price for additional click drift due to this. |
| vivo | Prefer Option 2 from the perspective of both UL timing requirements and power saving. Considering the frequency error is 0.1PPM, there only exists a 2ns difference between 20ms of SSB periodicity and 40ms of SSB periodicity. |
| Apple | Support option 1. |
| CATT | We prefer option 3 and believe that the existing 160ms should be as close as possible to avoid draining UE power. We can also agree on option 2. |

Issue 3-1-4: Percentage of UL CP length Te can occupy for UL SCS of 480/960 kHz

- Option 1 (Apple): 50%

- Option 2 (CATT, ZTE, Intel): 30%

- Option 3 (Huawei): For SSB SCS ≥ UL SCS, and SSB periodicity: 80ms – 40%, 20ms – 30%,

- Option 4 (Ericsson): 28%

- Option 5 (Qualcomm): For SSB periodicity of 20ms – 38%.

  - FFS: 120kHz SSB SCS and 960kHz UL SCS

- Recommended WF

  - Discuss the options

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support option 3. |
| Intel | Support Option 2. Our position is based on the link-level simulation results for PUSCH demodulation. |
| Nokia | Our preference is to keep to discuss Te for the individual SSB and UL SCS combination. As we discuswed before, we also brought LL results that show degradation of UL demodulation performance if Te occupies a large fraction of the CP length, so we would like to keep these values close to existing Te values in the rage of 20.30% of the CP. |
| Qualcomm | Support option 5, UE cannot meet the Te requirements below this number for SSB periodicity of 20ns and delay spread of 10/20ns. Not to mention that the case of 120,960 is challenging to meet even with 40% CP occupancy. |
| Ericsson | We prefer option 4 (28%) or option 2 (30%), based on highest value used in legacy system. |
| vivo | Proposal 5 is fine with us without restriction of SSB periodicity to 20ms. |
| Qualcomm | Based on GTW agreement: - UE transmit timing error requirements for UL SCS of 480/960 kHz are defined under the following assumptions - SCS: - SSB SCS ≥ UL SCS - FFS if other SCS combinations shall be considered - At least one SSB is available at the UE during the last: 20ms, 40ms, 80ms - Note: If multiple set of requirements are defined, then the requirements will be defined for at most for 2 periodicities - Max delay spread: - [30ns] for 480kHz - [20ns] for 960kHz - FFS if a single set or multiple sets of requirements need to be defined - FFS how to design test case for UE transmit timing error requirements Considering the agreed delay spread, we can compromise to the following Te occupancy SSB SCS; UL SCS; Te/CP ratio. T_SSB = 20ms; T_SSB = 40ms; T_SSB = 80ms. 480; 480; 0.25; 0.3; 0.35. 960; 480; 0.25; 0.3; 0.35. 960; 960; 0.3; 0.35; 0.4. 480; 960; 0.35; 0.4; 0.45. 120; 480; 0.28; 0.3; 0.35. As shown above, we think we can specify a single set of requirements with SSB periodicity of 40ms and adopt the above mentioned Te/CP occupancy. Note that the case of 120,960 is still challenging and can be kept FFS for now |
| Apple | Support option 1. We will also consider further proposals based on today’s GTW discussion. |
| Nokia | Considering the outcome of the discussions on yesterday’s GTW, we believe that it is beneficial to send an LS to RAN1. Since RAN4 has identified that some scenarios are challenging, we believe that this should be communicated and considered in RAN1 feature list. In our view, the main points to be communicated to RAN1 would include: - The list of challenging SSB SCS / UL SCS combinations - Need for considering the SSB SCS in the feature list for UEs that are capable of transmitting UL in 480 kHz and/ or 960 kHz SCS. Can the moderator allocate an LS so that we can continue the discussion on the second round? Nokia can volunteer to draft it. |
| CATT | We prefer option 3 and believe that the existing 160ms should be as close as possible to avoid draining UE power. We can also agree on option 2. |

Issue 3-1-5: Two set of requirements

- Proposal 1: Define two set of UL timing accuracy requirements:

  - Option 1a (Huawei): One for SSB periodicity of 20ms, other for 80ms

  - Option 1b (ZTE): One for SSB periodicity of 40ms, other for 160ms

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support option 1a. At least 20ms SSB periodicity should be covered. |
| Intel | Defining two sets of requirements will provide better flexibility for network. We are ok with Proposal 1. The exact values for SSB periodicities depend on the outcome of Issue 3-1-3 |
| Nokia | We think Te should be accurate enough for the worst case of SSB periodicity, and one set should be enough for the RRM requirements. |
| Qualcomm | We are open to explore this route. One set of requirements for SSB periodicity of 20ms is definitely needed. |
| Ericsson | At least 160 ms SSB periodicity should be covered. If we define other set we prefer to use 80 ms SSB periodicity. |
| vivo | We prefer to define one set of UL timing accuracy requirement based on SSB periodicity of [40ms]. |
| Qualcomm | Based on our analysis in Issue 3-1-4, we agree with vivo and prefer to define one set of requirements based on SSB periodicity of 40ms (FFS case of 120,960) |
| Apple | We are willing to further discuss proposal 1. |

Issue 3-1-6: SSB and UL SCS combinations

- Proposal 1:  Define requirements for the following (SSB SCS, UL SCS) cases:

  - 120, 960

    - Yes – Nokia

    - No – Apple, Vivo

  - 120, 480

    - Yes – Vivo, Nokia

    - No – Apple

  - 480, 960

    - Yes – Nokia

    - No – Apple, Vivo

  - FFS for these cases – CATT, Huawei, Qualcomm

    - Depending on SSB periodicity, channel delay spread and application scenario

- Proposal 2:  Define requirements for other options with SSB SCS > UL SCS:

  - Yes – CATT, Nokia, Huawei

  - No – Apple

- Proposal 3 (Mediatek):  Introduce a new UE capability for supporting UL timing accuracy.

  - Proposal 3a (Mediatek):  UE shall not transmit UL transmissions on the 1st UL symbol and the last UL symbol of scheduled UL slots, if the UE cannot meet the new Te requirement for SCS of 480 kHz / 960 kHz.

- Recommended WF

  - Discuss the options

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support proposal 2 – Yes. For (120,480) (120,960) (480,960), justifications are need. We suggest to preclude (120,960). |
| Intel | We support precluding (120,960). For other combinations we don’t see justification to preclude. |
| MTK | FFS (960, 120), (120,960) not clear the use case. Support Proposal 3 to indicate whether UE is able to meet the accurate timing requirement. Up to NW implementation, NW could further mitigate the UL interference. |
| Nokia | Proposal 1: We agree with 120+480, 120+960, 480+960. We believe that the SSB SCS < UL SCS combinations should not be precluded from the RRM requirements. UE capabilities discussion is still ongoing on RAN1, and it is not prevented to have a UE that supports a different set of SCS in DL and UL. That means that it is not prevented that an UE could support 960 kHz SCS in UL and not support it on DL. Initial access SSB configurations do not include 960 kHz SCS. Therefore for resource optimization, some network deployments might want to use 120 kHz or 480 kHz only for the SSBs. Proposal 2: we agree with SSB SCS > UL SCS We don’t see reason to prevent these options. There is no technical reason to prevent these scenarios, since it is easier from the UE implementation to meet the Te requirements if the SSB SCS is larger than the UL SCS. Additionally, a practical eMBB scenario could be using a larger SCS for a wider SCS in DL while using a smaller SCS in UL. Proposal 3 – we do not agree with this proposal. In our view, if such capability is introduced there is a large complexity level introduced to the network, and the overall system performance would be highly affected by the UEs that do not meet the Te requirements. The UEs not supporting Te accuracy would probably experience much worst UL thoughtput, since only very low MCSs could be used for them. Additionally, it would be very challenging to schedule different UEs in the same slot. |
| Qualcomm | For Proposal 1, whether these combinations could be supported or not depends on the agreements on delay spread and SSB periodicities under Issue 3-1-2 and 3-1-3. It’s almost impossible to support some of these combinations if a shorter delay spread (~10/20ns) and shorter SSB periodicity (20/40ns) is not agreed. Suggest to keep FFS and come back after reaching some agreements on Issue 3-1-2 and 3-1-3. For proposal 2, we don’t see the use case but are okay in general. Suggest to keep FFS for now and focus on more important combinations. |
| Ericsson | (120,960) can be downprioritized. Besides this, the other (DL:SSB,UL_SCS) combinations can enter the standard. |
| vivo | We prefer to define the following (SSB SCS, UL SCS) cases: (120, 120) (120, 480) (480, 120) (480, 480) (960, 120) (960, 480) (960, 960) |
| Qualcomm | We agree with Ericsson and vivo and can define requirements for all cases, under the assumption of 40ms SSB periodicity, except (120,960) which can be FFS for now |
| Apple | The main reason to limit the combinations is 1) how difficult it is for the UE to meet the requirements 2) how likely they will be deployed. More inputs are welcome. |
| CATT | We also agree with Ericsson and vivo. For proposal 2: We support to define requirements for other options with SSB SCS > UL SCS. For proposal 1: For the SSB and UL SCS combinations of (120, 480) and (480, 960), depending on SSB periodicity, channel delay spread and application scenario. For the SSB and UL SCS combinations of (120, 960), we recommend not to define Te requirement for it, as UE is difficult to implement. |

Issue 3-1-7: Using TRS for RRC connected mode UL timing

- Proposal 1 (Vivo):  Using TRS for improving DL timing estimation error needs further evaluation

- Proposal 2 (Huawei): Rely only on SSBs

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support proposal 2. But we are open to discuss on TRS based requirements. The accuracy of TRS based timing estimation also depends on BW of TRS and periodicity. It cannot assumed that TRS can also way provided better performance. |
| Intel | We are wondering whether the intention of Proposal 1 is to define new requirements for Te in RRC connected mode in addition to UE initial transmission timing error |
| Nokia | We agree with proposal 1. We have also proposed on our paper that can be good for SSB/UL SCS combinations that are more challenging. In that case initial access may have to be evaluated separately as in Issues 3-1-10 |
| Qualcomm | We are open to discuss TRS for improving DL estimation, but it needs further evaluation. |
| Ericsson | Proposal 2. |
| vivo | We would like to see justification of using TRS for improving Te requirements. |
| Apple | We are open to further discussions to understand all the details, such as the benefits and potential impact on UE. |
| CATT | Support to proposal 1, and it needs further evaluation. |

Issue 3-1-8: Initial transmit timing accuracy test

- Proposal 1 (Apple):  RAN4 considers the testing and core requirement together to speed up the progress of the core requirement discussion.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Intel | Prefer to focus on core requirements in accordance with Workplan |
| Apple | As discussed in our paper, we believe it is beneficial to discuss both the core requirements and tests at the same time to progress the work. |
| Ericsson | Agree with Intel. Prefer to focus on core requirements in accordance with Workplan. |

Issue 3-1-9: Te requirements

- Recommended WF

  - There are proposals from multiple companies on exact values for the Te requirements based on various assumptions such as CP occupancy, channel delay spread, SSB periodicity etc. It’s recommended to agree on these parameters first and the exact values can be re-visited in second round, if there are enough agreements

| Company | Comments |
| --- | --- |
| Intel | Agree with Recommended WF |
| Ericsson | The WF is ok. |

Issue 3-1-10: Te requirements during initial access

- Proposal 1 (Nokia): The Te requirements for the SCS combination of 960 kHz in UL with 120 kHz DL can be relaxed for PRACH transmission in initial access in relation to the lower bound TCP - TCH - 2 ( Te + TAC,Q /2 ) > 0.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Nokia | We agree with Proposal 1. We brough this proposal as a relaxation for the combination of SSB with 120 kHz SCS and UL signals with 960 kHz. This is to be looked in combination with the TRS proposal to be used in connected mode. Since TRS is not configured in initial access the PRACh requirements could be relaxed or this particular case. |
| Qualcomm | Since this largely depends on whether TRS are used or not, we can come back to this after some progress on Issue 3-1-7 |
| Ericsson | In issue 3-1-6 we propose to down prioritize this combination. |
| Apple | We also agree to not consider this case for now. |

### Sub-topic 3-2: Gradual timing adjustment

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-2-1: Gradual timing adjustment

- Proposal 1 (Intel): RAN4 to define Tq and Tp requirements for 480kHz and 960kHz considering that it is preferred to have them smaller than Te value.

  - Define Tq requirements for 480kHz and 960kHz equal to (0.6875*64*Tc + RF_margin).

  - Determine RF_margin value, which could be reasonable for high frequencies of FR2-2.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We suggest to further discuss it with conclusion of Te requirements. |
| Intel | With that proposal we wanted to highlight that gradual timing adjustment may need to be defined based on the new values of Te |
| Nokia | We are in general fine with Proposal 1. To be clarified is the range of RF margin, and technical reasons for RF margin. |
| Qualcomm | Need further discussion. |
| Ericsson | We agree with Huawei, to further discuss it with conclusion of Te requirements. |
| vivo | FFS |
| Apple | We prefer further discussion. |

### Sub-topic 3-3: MRTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-3-1: Basic principles

- Proposal 1 (CATT, Huawei, LGE): Define MRTD requirements in FR2-2 based on the following rules in:

  - For Async cases: MRTD = 0.5 slot

  - For sync cases: MRTD = TAE + propagation delay difference

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support proposal 1. |
| Intel | Based on current spec, MRTD by definition can not be larger than 0.5 slot. Even if we have TAE + propagation delay difference larger than slot length, MRTD will still be 0.5 slot. Option 2 in the Figure below is our understanding of current spec We propose to update the definition for RTD for CA to Option 1 from the Figure. |
| MTK | To clarify if “Async cases” is for DC? If yes, we can support Proposal 1. Agree with sync case in general. Support Option 1 definition in Intel’s figure. |
| Nokia | Fine with Proposal 1 async cases. For sync cases, propagation delay is only considered for the inter-frequency cases, but FR2-2/FR2-2 was not included in the deployment scenarios. |
| LGE | Support the proposal 1 |
| Ericsson | For async cases: we prefer to cap reduction to 33 µs. MRTD = 0.5 slot if 0.5 slot > 33 µs. MRTD = 33 µs if 0.5 slot ≤ 33 µs For cases: MRTD = TAE + propagation delay difference Proposal 1 is ok. The asynchronous MRTD = 0.5*slot follow from MRTD definition for that case, as pointed out by Intel. |
| vivo | Proposal 1 is OK. |
| Apple | Proposal 1 is OK |
| CATT | Support proposal 1 |

Issue 3-3-2: Propagation delay difference

- Proposal 1 (CATT, Nokia, Vivo, LGE): For FR2-2 and FR2-2, there is no propagation delay difference

  - Proposal 1a (Vivo): For deriveSSB-IndexFromCell, the propagation delay difference of FR2-2 should be 3.33us considering the coverage of 1km

- Proposal 3 (Huawei, CATT): For FR2-1 and FR2-2, take following propagation delay difference assumptions for considerations:

  - Same propagation delay difference as FR2-1

  - Shorter propagation delay differences, e.g. 1000 m

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support proposal 3. We believe there are some difference between proposal 1 and proposal 1a. Proposal 1 seems for FR2-2 intra-band CA MRTD requirements. Proposal 1a is about timing difference between serving cell and neighbor cells. |
| Intel | Support Proposal 1. For Proposal 1a, deriveSSB-IndexFromCell is the timing difference between different cells on the same frequency. It is not related to CA/DC MRTD For Proposal 3: FR2-1 + FR2-2 is out of Rel-17 scope. |
| MTK | Support Proposal 1. Support Proposal 3 is out of scope. |
| Nokia | Support proposal 1 As pointed out by Intel, proposal 1a is not related to MRTD and proposal 3 is out of scope |
| LGE | Support proposal 1. For proposal 3, does this mean FR2-1+FR2-2 ? |
| Qualcomm | Support proposal 1. The scenario in Proposal 3 is out of scope. |
| Ericsson | Same propagation delay difference as FR2-1 |
| vivo | We support Proposal 1 and Proposal 1a. Since there is no inter-band CA FR2-2 scenario in Rel-17, it may make no sense to discuss the propagation delay for the requirement of MRTD. However, as mentioned in Issue 2-2-1, the question related to deriveSSB-IndexFromCell need to consider the propagation delay difference. If the coverage of 1km is considered for FR2-2, the deriveSSB-IndexFromCell is always be enabled for 480kHz because the timing difference between cells can be guaranteed within 3 SSB symbols. |
| Apple | This may depend on the RF session on UE TX power. |
| CATT | Support proposal 1 |

Issue 3-3-3: MRTD for intra-band non-contiguous CA

- Proposal 1 (Vivo, Huawei, Nokia, LGE): The MRTD requirement for intra-band non-contiguous NR CA need to continue waiting for the TAE conclusion from RF group.

- Proposal 2 (Ericsson): MRTD = 0.26 µs for non-contiguous intra band CA in FR2-2

  - Assume TAE = 260 ns for non-contiguous intra band CA

- Proposal 3 (Intel): In case of NR carrier aggregation RAN4 to consider the receive timing difference between carriers as the timing difference between the slot boundaries of the slots which were sent simultaneously

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support proposal 1. For proposal 3, it is not very clear about how to define “sent simultaneously”. |
| Intel | Support Proposal 1. Proposal 3 is more related to the cases where TAE + propagation delay difference is longer than 0.5 slot |
| MTK | Support proposal 1. |
| Nokia | We agree with proposal 1, since RF is still discussing TAE. As for Proposal 3, we think the definition of MRTD doesnt need to be changed from 38.133, or: “A UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG and the closest slot timing boundary of a cell belonging to the SCG to be aggregated for NR DC operation. A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers to be aggregated in NR carrier aggregation.” |
| LGE | Support proposal 1. |
| Qualcomm | Support proposal 1 |
| Ericsson | Proposal 2. |
| vivo | Support Proposal 1. |
| Apple | Proposal 1 is OK |
| CATT | Support proposal 1 |

Issue 3-3-4: MRTD for FR1 and FR2-2 inter-band CA

- Proposal 1a (CATT): The MRTD requirements for FR1 and FR2-2 inter-band CA depend on the conclusions of TAE and the propagation delay difference requirements.

- Proposal 1b (Vivo, Ericsson): The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs

- Proposal 1c (LGE): Consider shorter MRTD for inter band carrier aggregation between FR1 and FR2-2 based on real network deployment and UE baseband processing.

- Proposal 2 (Intel): In case of NR carrier aggregation RAN4 to consider the receive timing difference between carriers as the timing difference between the slot boundaries of the slots which were sent simultaneously

- Proposal 3 (Vivo): The MRTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MRTD requirements for inter-band NR carrier aggregation, update Table 7.6.4-2 for FR2-2 as below:

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 note1 |
| Between FR1 and FR2 | 25 |
| Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA. |  |

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support option 1a and 3 for the clarification of FR2 inter-band CA |
| Intel | For Proposal 1b: Based on RTD definition in current spec, MRTD can not be 25us for 480kHz and 960kHz SCSs. The maximum RTD can be 0.5slot. That’s why Proposal 2 is needed to update definition of RTD. Support Proposal 1b + Proposal 2 Support Proposal 3 |
| MTK | Support Proposal 1a and Proposal 3 |
| Nokia | We agree with Proposal 1b. We are fine with Proposal 3 |
| LGE | We are fine with proposal 1a and proposal 3. For proposal 1c, in our understanding, 25us MRTD for FR1 and FR2 inter-band CA has been defined due to shorter symbol length, so UE baseband processing according to shorter symbol length for high SCS needs to be also considered for MRTD of FR1+FR2-2 inter-band CA together with network deployment. |
| Qualcomm | Support proposal 1a and 1c. Also fine with proposal 3. |
| Ericsson | Proposal 1b. Clarification according to proposal 3 is ok. |
| Vivo | Support proposal 1b and proposal 3. |
| Apple | Proposal 1c and proposal 3 are OK. |
| CATT | Support Proposal 1a and Proposal 3. |

Issue 3-3-5: MRTD for FR1 and FR2-2 NR DC - Synchronous

- Proposal 1 (CATT): Wait for conclusions on TAE and the propagation delay difference requirements.

- Proposal 2 (Vivo, Nokia, Ericsson): Reuse requirements of FR1 and FR2-1 synchronous NR-DC.

- Proposal 3 (Intel): In case of NR DC with high SCS the UE is always in asynchronous conditions

- Proposal 4 (Vivo): The MRTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MRTD requirements for inter-band synchronous NR DC, update Table 7.6.6-1 for FR2-2 as below:

Table 7.6.6-1: Maximum receive timing difference requirement for inter-band synchronous NR DC

| Frequency Range |  | Maximum receive timing difference (µs) |
| --- | --- | --- |
| Cell in MCG | Cell in SCG |  |
| FR1 | FR1 | 33 |
| FR2-1 | FR2-1 | 8 |
| FR1 | FR2 | 33 |

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Support option1 and 4 for the clarification of FR2 inter-band CA |
| Intel | Support Proposal 3. For DC case we don’t need RTD definition update, which we proposed in previous issues, since we don’t have cross carrier operation in DC and we don’t need slot index alignment. Considering that propagation delay difference between FR1 and FR2-2 is larger than 0.5 slot, MRTD should be equal to 0.5 slot by current definition Support Proposal 4 |
| MTK | Support Proposal 1 and Proposal 4 |
| Nokia | We agree with Proposal 2 and 4. |
| LGE | Support proposal 1 and 4. |
| Qualcomm | Support proposal 1 and fine with 4. |
| Ericsson | Proposal 2. |
| vivo | Support proposal 2 and proposal 4. |
| Apple | Proposal 1 and 4 are OK. |
| CATT | Support proposal 1 and proposal 4. |

Issue 3-3-6: MRTD for FR1 and FR2-2 NR DC - Asynchronous

- Proposal 1 (CATT, Vivo, Nokia, Intel): Reuse the existing MRTD requirements for FR1 and FR2-1, that is, half the slot length with respect to the larger SCS of the MCG and SGC cells.

| Max {Sub-carrier spacing in PCell (kHz), Sub-carrier spacing in PSCell (kHz)} | Maximum receive timing difference (µs) |
| --- | --- |
| 480 | 15.625 |
| 960 | 7.8125 |

- Proposal 2 (LGE, Ericsson): Define 33us MRTD for 480 and 960kHz SCS of asynchronous inter-band NR-DC

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 1 provided that it is the timing difference between closest slot boundaries. |
| Intel | Support Proposal 1 |
| MTK | Fine with proposal 1 |
| Nokia | Support Proposal 1 |
| LGE | We are open to half slot MRTD for this case. But the half slot MRTD for 480 and 960kHz in async case is shorter than MRTD in sync case. So, we slightly prefer 33us MRTD in async case. |
| Qualcomm | Support proposal 1 |
| Ericsson | Proposal 2. We are fine with proposal 1. The asynchronous MRTD = 0.5*slot follow from MRTD definition for that case, as pointed out by Intel in Issue 3-3-1: Basic principles. |
| vivo | Support Proposal 1. |
| Apple | Proposal 1 is OK. |
| CATT | Support proposal 1 |

### Sub-topic 3-4: MTTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-4-1: Basic principles

- Proposal 1 (LGE): Define MTTD requirements in FR2-2 based on the following rule:

  - MTTD = MRTD + UL transmit error

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Intel | If the MRTD definition will be updated according to our proposal in Issue 3-3-1, MTTD definition may also need updates. |
| MTK | Support Proposal 1 with the following clarification: The UL transmit error" should also cover 1) TA resolution error + 2) TA adjustment accuracy + 3) Timing Error Limit as R15 principle. |
| Nokia | We think Proposal 1 doesn’t contain the whole picture for the definition of MTTD. In Rel 15 we defined it considering TA resolution error, TA accuracy, and Te as discussed in the papers R4-1810927, R4-1810924. |
| LGE | Agree with MTK’s and Nokia’s further clarification for UL transmit error. |
| Ericsson | Proposal 1 is fine. |
| Apple | Agree with MTK and Nokia. |

Issue 3-4-2: MTTD for FR1 and FR2-2 inter-band CA

- Proposal 1 (Vivo): The existing MTTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2.

- Proposal 2 (Vivo): The MTTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MTTD requirements for inter-band NR carrier aggregation, update Table 7.5.4-1 for FR2-2 as below:

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2 | 26.1 |
| Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA. |  |

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 2 for the clarification of FR2 inter-band CA |
| Intel | Support Proposal 2 |
| MTK | Proposal 1 would depend on other issues, e.g. Te and Issue 3-4-1. Fine with proposal 2 |
| Nokia | We agree with proposal 2. Proposal 1 depends on the outcome of Issue 3-4-1. |
| LGE | Same view with MTK. MTTD would depend on Te. fine with proposal 2. |
| Qualcomm | FFS on proposal 1 and fine with proposal 2 |
| Ericsson | Ericsson: We are fine with proposal 1 and proposal 2. |
| vivo | Support Proposal 1. Te of 120kHz must be more than Te of 960kHz for FR2-2 and the worst case is 120kHz for both FR2-1 and FR2-2. Therefore, the existing MTTD requirements for FR1+FR2-1 CA can be reused for FR1+FR2-2 CA. Support Proposal 2. |
| Apple | Agree with proposal 2. |

Issue 3-4-3: MTTD for FR1 and FR2-2 NR DC - Synchronous

- Proposal 1 (Vivo): The existing MTTD requirements for inter-band synchronous NR DC for FR1 and FR2-1 can be reused for inter-band synchronous NR DC for FR1 and FR2-2.

- Proposal 2 (Vivo): The MTTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MTTD requirements for inter-band synchronous NR DC, update Table 7.5.6-1 for FR2-2 as below:

Table 7.5.6-1: Maximum uplink transmission timing difference requirement for inter-band synchronous NR DC

| Frequency Range |  | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| Cell in MCG | Cell in SCG |  |
| FR1 | FR1 | 34.6 |
| FR2-1 | FR2-1 | 8.5 |
| FR1 | FR2 | 34.1 |

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 2 for the clarification of FR2 + FR2 NRDC |
| Intel | Ok with Proposal 2. |
| MTK | Same comment as Issue 3-4-2. |
| Nokia | We agree with proposal 2. Proposal 1 depends on the outcome of Issue 3-4-1. |
| LGE | Need to wait Te conclusion. Fine with proposal 2. |
| Qualcomm | FFS on proposal 1 and fine with proposal 2 |
| Ericsson | Ericsson: We are fine with proposal 1 and proposal 2. |
| vivo | Support Proposal 1. Te of 120kHz must be more than Te of 960kHz for FR2-2 and the worst case is 120kHz for both FR2-1 and FR2-2. Therefore, the existing MTTD requirements for FR1+FR2-1 DC can be reused for FR1+FR2-2 DC. Support Proposal 2. |
| Apple | Agree with proposal 2 |
| CATT | Support Proposal 2. |

Issue 3-4-4: MTTD for FR1 and FR2-2 NR DC – Asynchronous

- Proposal 1 (Vivo): The MTTD requirements for inter-band asynchronous NR DC shall the half of slot length. For the MTTD requirements for inter-band asynchronous NR DC, update Table 7.5.6-2 for FR2-2 as below:

Table 7.5.6-2 Maximum uplink transmission timing difference requirement for inter-band asynchronous NR DC

| Max {Sub-carrier spacing in Pcell (kHz), Sub-carrier spacing in PSCell (kHz)} | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| 15 | 500 |
| 30 | 250 |
| 60 | 125 |
| 120 | 62.5 |
| 480 | 15.625 |
| 960 | 7.8125 |

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 1. |
| Intel | Ok with Proposal 1 |
| MTK | Fine with proposal 1. |
| Nokia | We are fine with Proposal 1. |
| Qualcomm | Fine with the proposal |
| Ericsson | For async cases: we prefer to cap reduction to 33 µs. MRTD = 0.5 slot if 0.5 slot > 33 µs. MRTD = 33 µs if 0.5 slot ≤ 33 µs For cases: MRTD = TAE + propagation delay difference |
| vivo | Support Proposal 1. |
| Apple | Agree with proposal 1. |
| CATT | Ok with Proposal 1 |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 3-1: UE transmit timing error | Issue 3-1-1: Basic principles for defining Te Companies’ views: Some companies think it’s good to have these guidelines, while most think it’s not needed and we can directly discuss the Te values. Tentative agreements: Candidate options: Recommendations for 2nd round: While these guidelines help initiate the discussion, there is no need to further discuss this as the group prefers to discuss the requirements directly. No further discussion needed in second round. |
|  | Issue 3-1-2: Channel delay spread for defining Te GTW agreements: - Max delay spread: - [30ns] for 480kHz - [20ns] for 960kHz Candidate options: Recommendations for 2nd round: Following GTW agreement, no further discussion is needed. |
|  | Issue 3-1-3: SSB periodicity GTW agreements: - At least one SSB is available at the UE during the last: 20ms, 40ms, 80ms - Note: If multiple set of requirements are defined, then the requirements will be defined for at most for 2 periodicities Candidate options: Recommendations for 2nd round: Following GTW agreement, the group to analyze which SSB periodicity may be used to define the Te requirements under Issue 3-1-4. Revisit to finalize final value after discussion on Issue 3-1-4. No further discussion needed under this issue. |
|  | Issue 3-1-4: Percentage of UL CP length Te can occupy for UL SCS of 480/960 kHz GTW agreements: - UE transmit timing error requirements for UL SCS of 480/960 kHz are defined under the following assumptions - SCS: - SSB SCS ≥ UL SCS - FFS if other SCS combinations shall be considered - At least one SSB is available at the UE during the last: 20ms, 40ms, 80ms - Note: If multiple set of requirements are defined, then the requirements will be defined for at most for 2 periodicities - Max delay spread: - [30ns] for 480kHz - [20ns] for 960kHz - FFS if a single set or multiple sets of requirements need to be defined - FFS how to design test case for UE transmit timing error requirements Candidate options: Recommendations for 2nd round: based on the GTW agreement, continue discussion in the second round. Further discuss whether there’s a need to send an LS to RAN1 on UE UL timing error issues |
|  | Issue 3-1-5: Two set of requirements GTW agreements: - FFS if a single set or multiple sets of requirements need to be defined Candidate options: Recommendations for 2nd round: Based on the GTW agreement, continue discussion in the second round. |
|  | Issue 3-1-6: SSB and UL SCS combinations GTW agreements: - SSB SCS ≥ UL SCS - FFS if other SCS combinations shall be considered Candidate options: Recommendations for 2nd round: Based on the GTW agreement, continue discussion in the second round |
|  | Issue 3-1-7: Using TRS for RRC connected mode UL timing Companies’ views: No consensus, needs further discussion Tentative agreements: Candidate options: - Proposal 1 (Vivo): Using TRS for improving DL timing estimation error needs further evaluation - Proposal 2 (Huawei): Rely only on SSBs Recommendations for 2nd round: Continue discussion in the second round |
|  | Issue 3-1-8: Initial transmit timing accuracy test GTW agreements: - FFS how to design test case for UE transmit timing error requirements Candidate options: Recommendations for 2nd round: Based on the GTW agreement, continue discussion in the second round |
|  | Issue 3-1-9: Te requirements Recommendations for 2nd round: Prioritize Issue 3-1-4. No further discussion needed in the second round |
|  | Issue 3-1-10: Te requirements during initial access Recommendations for 2nd round: Discuss Issue 3-1-7 first. No further discussion needed in the second round |
| Sub-topic 3-2: Gradual timing adjustment | Issue 3-2-1: Gradual timing adjustment Companies’ views: No consensus, needs further discussion Tentative agreements: Candidate options: - RAN4 to define Tq and Tp requirements for 480kHz and 960kHz considering that it is preferred to have them smaller than Te value. - Define Tq requirements for 480kHz and 960kHz equal to (0.6875*64*Tc + RF_margin). - Determine RF_margin value, which could be reasonable for high frequencies of FR2-2. Recommendations for 2nd round: Continue discussion in the second round |
| Sub-topic 3-3: MRTD | Issue 3-3-1: Basic principles Companies’ views: Most companies agree to the proposal. Tentative agreements: - Define MRTD requirements in FR2-2 based on the following rules in: - For Async cases: MRTD = 0.5 slot - For sync cases: MRTD = TAE + propagation delay difference Candidate options: Recommendations for 2nd round: No further discussion in the second round |
|  | Issue 3-3-2: Propagation delay difference Companies’ views: Most companies agree to the proposal 1 Tentative agreements: - For FR2-2 and FR2-2, there is no propagation delay difference Candidate options: Recommendations for 2nd round: Propagation delay difference of 3.33us considering FR2-2 coverage of 1km for deriveSSB-IndexFromCell shall be treated under Issue 2-2-1. No further discussion on this issue in the second round |
|  | Issue 3-3-3: MRTD for intra-band non-contiguous CA Companies’ views: Most companies prefer to wait for agreements on TAE requirements Tentative agreements: The MRTD requirement for intra-band non-contiguous NR CA need to continue waiting for the TAE conclusion from RF group. Candidate options: Recommendations for 2nd round: No further discussion on this issue in the second round |
|  | Issue 3-3-4: MRTD for FR1 and FR2-2 inter-band CA Companies’ views: No agreements on the MRTD requirements, terminology clarification is okay Tentative agreements: - The MRTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MRTD requirements for inter-band NR carrier aggregation, update Table 7.6.4-2 for FR2-2 as below: Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8 note1. Between FR1 and FR2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA.. Candidate options: - Option 1a (CATT): The MRTD requirements for FR1 and FR2-2 inter-band CA depend on the conclusions of TAE and the propagation delay difference requirements. - Option 1b (Vivo, Ericsson): The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs - Option 1c (LGE): Consider shorter MRTD for inter band carrier aggregation between FR1 and FR2-2 based on real network deployment and UE baseband processing. Recommendations for 2nd round: Need further discussion. Consider agreeing to Option 1a. |
|  | Issue 3-3-5: MRTD for FR1 and FR2-2 NR DC - Synchronous Companies’ views: No agreements on the MRTD requirements, terminology clarification is okay Tentative agreements: The MRTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MRTD requirements for inter-band synchronous NR DC, update Table 7.6.6-1 for FR2-2 as below: Table 7.6.6-1: Maximum receive timing difference requirement for inter-band synchronous NR DC Frequency Range; Maximum receive timing difference (µs). Cell in MCG; Cell in SCG. FR1; FR1; 33. FR2-1; FR2-1; 8. FR1; FR2; 33. Candidate options: - Proposal 1 (CATT): Wait for conclusions on TAE and the propagation delay difference requirements. - Proposal 2 (Vivo, Nokia, Ericsson): Reuse requirements of FR1 and FR2-1 synchronous NR-DC. - Proposal 3 (Intel): In case of NR DC with high SCS the UE is always in asynchronous conditions Recommendations for 2nd round: Need further discussion. Consider agreeing to Proposal 1 |
|  | Issue 3-3-6: MRTD for FR1 and FR2-2 NR DC - Asynchronous Companies’ views: most companies agree to the proposal Tentative agreements: - Reuse the existing MRTD requirements for FR1 and FR2-1, that is, half the slot length with respect to the larger SCS of the MCG and SGC cells. Max {Sub-carrier spacing in PCell (kHz), Sub-carrier spacing in PSCell (kHz)}; Maximum receive timing difference (µs). 480; 15.625. 960; 7.8125. Candidate options: Recommendations for 2nd round: Need further discussion |
| Sub-topic 3-4: MTTD | Issue 3-4-1: Basic principles Companies’ views: Most companies believe that the proposal doesn’t show the whole picture and the UL transmit error" should also cover 1) TA resolution error + 2) TA adjustment accuracy + 3) Timing Error Limit as R15 principle Tentative agreements: Candidate options: - Proposal 1a (moderator): Define MTTD requirements in FR2-2 based on the following rule: - MTTD = MRTD + TA resolution error + TA adjustment accuracy + UL transmit error Recommendations for 2nd round: Need further discussion |
|  | Issue 3-4-2: MTTD for FR1 and FR2-2 inter-band CA Companies’ views: No agreements on the MRTD requirements, terminology clarification is okay Tentative agreements: - The MTTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MTTD requirements for inter-band NR carrier aggregation, update Table 7.5.4-1 for FR2-2 as below: Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of TAGs; Maximum uplink transmission timing difference (µs). FR1; 34.6. FR2-1; 8.5 Note1. Between FR1 and FR2; 26.1. Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA.. Candidate options: Recommendations for 2nd round: No further discussion needed as the exact requirement depends on other issues. |
|  | Issue 3-4-3: MTTD for FR1 and FR2-2 NR DC - Synchronous Companies’ views: No agreements on the MRTD requirements, terminology clarification is okay Tentative agreements: - The MTTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MTTD requirements for inter-band synchronous NR DC, update Table 7.5.6-1 for FR2-2 as below: Table 7.5.6-1: Maximum uplink transmission timing difference requirement for inter-band synchronous NR DC Frequency Range; Maximum uplink transmission timing difference (µs). Cell in MCG; Cell in SCG. FR1; FR1; 34.6. FR2-1; FR2-1; 8.5. FR1; FR2; 34.1. Candidate options: Recommendations for 2nd round: No further discussion needed as the exact requirement depends on other issues. |
|  | Issue 3-4-4: MTTD for FR1 and FR2-2 NR DC - Asynchronous Companies’ views: Most companies support the proposal Tentative agreements: - The MTTD requirements for inter-band asynchronous NR DC shall the half of slot length. For the MTTD requirements for inter-band asynchronous NR DC, update Table 7.5.6-2 for FR2-2 as below: Table 7.5.6-2 Maximum uplink transmission timing difference requirement for inter-band asynchronous NR DC Max {Sub-carrier spacing in Pcell (kHz), Sub-carrier spacing in PSCell (kHz)}; Maximum uplink transmission timing difference (µs). 15; 500. 30; 250. 60; 125. 120; 62.5. 480; 15.625. 960; 7.8125. Candidate options: Recommendations for 2nd round: No further discussion needed |

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 3-1: UE transmit timing error

Issue 3-1-4: Percentage of UL CP length Te can occupy for UL SCS of 480/960 kHz

GTW agreements:

  - UE transmit timing error requirements for UL SCS of 480/960 kHz are defined under the following assumptions

    - SCS:

      - SSB SCS ≥ UL SCS

      - FFS if other SCS combinations shall be considered

    - At least one SSB is available at the UE during the last: 20ms, 40ms, 80ms

      - Note: If multiple set of requirements are defined, then the requirements will be defined for at most for 2 periodicities

    - Max delay spread:

      - [30ns] for 480kHz

      - [20ns] for 960kHz

  - FFS if a single set or multiple sets of requirements need to be defined

  - FFS how to design test case for UE transmit timing error requirements

- Recommended WF

  - Discuss Te values for different SSB periodicities and agreed delay spread when SSB SCS ≥ UL SCS

  - Further discuss the other SCS combinations

  - Discuss if there’s a need to send an LS to RAN1 regarding the following

    - The list of challenging SSB SCS / UL SCS combinations

    - Need for considering the SSB SCS in the feature list for UEs that can transmit UL in 480 kHz and/ or 960 kHz SCS.

| Company | Comments |
| --- | --- |
| vivo | For SCS combinations, we agree with not defining the requirements for (120,960) and (480,960). |
| Nokia | We are ok with listing (120,960) as challenging combination. We would prefer to keep (480,960) if possible, but would like to hear more from the companies. Additionally, we think it might be beneficial in terms of efficient SSB resource utilization if SSBs with 480 kHz can also be used for UEs transmiting with 960 kHz in UL. |
| Ericsson | We can down prioritize (120,960). We prefer to keep (480,960). |
| Huawei | We agree not define requirement for (120, 960). For (120,480) (480,960), we are fine to check whether there is chance that companies can reach an acceptable value. Regarding the necessity of sending the LS. We have following questions. We what we are discussing is from requirements perspective. If RAN4 reach an agreement that the requirements of certain combination can not be met, what the expected action from RAN1? Do some enhancement for the case (e.g. denser RS design) or just inform RAN1 that RAN4 will not define requirements for this case? |
| Qualcomm | Based on GTW agreement: - UE transmit timing error requirements for UL SCS of 480/960 kHz are defined under the following assumptions - SCS: - SSB SCS ≥ UL SCS - FFS if other SCS combinations shall be considered - At least one SSB is available at the UE during the last: 20ms, 40ms, 80ms - Note: If multiple set of requirements are defined, then the requirements will be defined for at most for 2 periodicities - Max delay spread: - [30ns] for 480kHz - [20ns] for 960kHz - FFS if a single set or multiple sets of requirements need to be defined - FFS how to design test case for UE transmit timing error requirements Considering the agreed delay spread, we can compromise to the following Te occupancy SSB SCS; UL SCS; Te/CP ratio. T_SSB = 20ms; T_SSB = 40ms; T_SSB = 80ms. 480; 480; 0.25; 0.3; 0.35. 960; 480; 0.25; 0.3; 0.35. 960; 960; 0.3; 0.35; 0.4. 480; 960; 0.35; 0.4; 0.45. 120; 480; 0.28; 0.3; 0.35. As shown above, we think we can specify a single set of requirements with SSB periodicity of 40ms and adopt the above mentioned Te/CP occupancy. Note that the case of 120,960 is still challenging and can be deprioritized or keep FFS for now For LS, we prefer waiting to reach agreements on the supported SCS combinations before sending the LS to RAN1 |
| Nokia | About the LS discussion. Our intention is with eh LS is that, if we are going to down select certain combinations of SCS, RAN1 should take this into consideration when defining UE capabilities. What we would like to avoid, is that there are UEs that can only support the combinations for which we do not have requirements. For example, if a UE only supports SSBs with 120 kHz SCS, and it does support 960 kHz SCS in UL, this UE will not have a Te requirement to follow. In our perspective, this SCS combination can only be down selected by RAN4 if this situation is not possible from UE capability. @Huawei: The expected action from RAN1 is not additional signaling, but only that the support of 960 kHz SCS in UL has a dependency on the SSB SCS as well. As a conclusion, if we are going to down select combinations on RAN4, this should reflect UE capabilities in RAN1, otherwise we could have UEs in the field without requirements. About the values proposed by Qualcomm, we try to bring another proposal on values in the next hours, but we need time for preparing the calculations. |
| Apple | We still have the view that no requirements are needed for (120,960) and (480,960). For the LS, we think it is a good idea to inform RAN1 of RAN4 decisions. We appreciate the proposed requirements from Qualcomm and other companies and would like to have more time to evaluate them. We want to come back to this issue at the next meeting. |
| CATT | For SCS combinations, we agree with not defining the requirements for (120,960). For (480,960), we suggest to consider whether the UE could implement. For sending the LS, we agree with Huawei's concerns, of course, after RAN4 reaches agreements. |
| Intel | We would like to identify the span of proposed values for Te requirements: based on the companies’ proposals the span is 20%CP < Te < 40%CP. We propose to further limit discussion to the values withing that range As for combinations downselection, we prefer to deprioritize combination (120,960) and keep other combinations. For the LS, we are ok to inform RAN1 of RAN4 decisions |

Issue 3-1-5: Two set of requirements

GTW agreements:

  - FFS if a single set or multiple sets of requirements need to be defined

- Recommended WF

  - Discuss whether a single or multiple set of requirements are needed considering Issue 3-1-4

| Company | Comments |
| --- | --- |
| vivo | We prefer to define a single set of requirements based on the SSB periodicity of [40ms]. |
| Nokia | We think at least requirements with 80 ms should be defined. We see little benefit of having multiple sets of requirements but are willing to compromise for progress. |
| Ericsson | We agree with Nokia that at least requirements with 80 ms should be defined. |
| Huawei | If the one set of requirements can be acceptable to all, then obviously one set of requirement are more sample. But actually, we see it is hard to companies to converge on one value. Then we prefer to have two set of requirements. 20/40 and 80. |
| Qualcomm | As per our analysis in Issue 3-1-4, we prefer a single set of requirements based on SSB periodicity of 40ms |
| Apple | We can keep this open. |

Issue 3-1-7: Using TRS for RRC connected mode UL timing

- Proposal 1 (Vivo):  Using TRS for improving DL timing estimation error needs further evaluation

- Proposal 2 (Huawei): Rely only on SSBs

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| vivo | Proposal 2 is also fine to us. |
| Nokia | Proposal 1. We think TRS use can be kept as FFS. From our analysis the real need for TRS use would be for the combination of 120 kHz SSB and 960 kHz SCS in UL. But I think we should not discard using TRS before we advance more on the actual Te values. We understand that proposal 1 is not stating that TRS is necessarily used, it is only leaving the option open in case it is needed. |
| Ericsson | Proposal 2. |
| Huawei | We are fine to keep TRS open for discussion. But we would like to hear views from companies whether to have conditions on TRS BW and periodicity. |
| Qualcomm | We can keep this FFS for now |
| Apple | We can keep this open. |
| CATT | Keep it FFS. |
| Intel | Same comment as for the first round: whether the intention of Proposal 1 is to define new requirements for Te in RRC connected mode in addition to UE initial transmission timing error requirement. Our understanding is that UE initial transmission timing can rely only on SSB. |

Issue 3-1-8: Initial transmit timing accuracy test

- Proposal 1 (Apple):  RAN4 considers the testing and core requirement together to speed up the progress of the core requirement discussion.

- Proposal 2 (new): RAN4 to agree that Te requirements will be considered as having statistical nature for the RRM performance requirements.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Nokia | We understand Apple's concern, and we would like to compromise here, but the text of the proposal might be too general for the moment. We understand that it might be hard for some companies to agree on Te values if the performance requirements are not of statistical nature. Therefore, we propose the following, which we hope will alleviate the final requirements for some companies: Proposal 2: RAN4 to agree that Te requirements will be considered as having statistical nature for the RRM performance requirements. |
| Huawei | For proposal 2. We would like to know whether it is only for the Te requirements testing for this WI or also intend to revisit legacy Te requirements testing? If company has strong views on this, we would like to discuss this in next meeting as we need to check whether current Te test cases are defined without statistical nature on purpose and whether there is RAN5 impact. |
| Nokia | @Huawei: Thanks for que question, and it is good you point it out. I agree that we should not define requirements that would have impact on legacy features. - Proposal 2a (new): RAN4 to agree that Te requirements for FR2-2 will be considered as having statistical nature for the RRM performance requirements. |
| Apple | We consider the change for Te requirement test case for FR2-2 only, and keep the existing test cases for FR1 and FR2-1 intact. We appreciate Nokia’s proposal 2 and make the following modification for clarity: Proposal 2: RAN4 to agree that test cases for Te requirements for FR2-2 will be designed as having statistical nature, i.e., something like “The rate of UE meeting the Te requirement observed during repeated tests shall be at least 90%.” And the detailed wording can be finalized during the RRM perf. Part. |

### Sub-topic 3-2: Gradual timing adjustment

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-2-1: Gradual timing adjustment

- Proposal 1 (Intel): RAN4 to define Tq and Tp requirements for 480kHz and 960kHz considering that it is preferred to have them smaller than Te value.

  - Define Tq requirements for 480kHz and 960kHz equal to (0.6875*64*Tc + RF_margin).

  - Determine RF_margin value, which could be reasonable for high frequencies of FR2-2.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| vivo | Prefer to discuss the issue with outcome of Te requirements. |
| Nokia | We are fine with the proposal. |
| Ericsson | Prefer to discuss the issue with outcome of Te requirements. |
| Huawei | Prefer to discuss the issue with outcome of Te requirements. |
| Qualcomm | FFS for now |
| Apple | We prefer to discuss this at next meeting. |
| CATT | Agree to give priority to discuss Te requirements. |
| Intel | Ok to postpone the discussion until agreement for Te |

### Sub-topic 3-3: MRTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-3-4: MRTD for FR1 and FR2-2 inter-band CA

- Proposal 1a (CATT): The MRTD requirements for FR1 and FR2-2 inter-band CA depend on the conclusions of TAE and the propagation delay difference requirements.

- Proposal 1b (Vivo, Ericsson): The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs

- Proposal 1c (LGE): Consider shorter MRTD for inter band carrier aggregation between FR1 and FR2-2 based on real network deployment and UE baseband processing.

- Proposal 2 (Intel): In case of NR carrier aggregation RAN4 to consider the receive timing difference between carriers as the timing difference between the slot boundaries of the slots which were sent simultaneously

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| vivo | Support Proposal 1b. |
| Nokia | We prefer Proposal 1b. |
| Ericsson | Proposal 1b. |
| Huawei | Fine with 1a or 1b |
| Qualcomm | We support proposal 1c, we are fine with 1a as well |
| LGE | Fine with 1a or 1c. |
| Apple | Option 1a or 1c is fine. |
| CATT | Ok with Proposal 1a and 1c. |
| Intel | Ok with proposal 1b. However, we would like to highlight the MRTD definition issue again. With current definition from TS38.133 MRTD is related to timing difference between the closest slot timing boundaries. Thus, MRTD can not be larger than 0.5 slot We would like to add the following in the WF: FFS whether and how to change the MRTD definition, so that it could be larger than 0.5 slot. |

Issue 3-3-5: MRTD for FR1 and FR2-2 NR DC - Synchronous

- Proposal 1 (CATT): Wait for conclusions on TAE and the propagation delay difference requirements.

- Proposal 2 (Vivo, Nokia, Ericsson): Reuse requirements of FR1 and FR2-1 synchronous NR-DC.

- Proposal 3 (Intel): In case of NR DC with high SCS the UE is always in asynchronous conditions

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| vivo | Support Proposal 2. |
| Nokia | Support Proposal 2. |
| Ericsson | Proposal 2. |
| Huawei | Fine with proposal 1 or 2 |
| Qualcomm | Support proposal 1 |
| LGE | Prefer proposal 1 |
| Apple | Support proposal 1. |
| CATT | Prefer Proposal 1 |
| Intel | We don’t see the need in having “synchronous” NR-DC which is misaligned by more than 1 slot. What “synchronous” means in that case? |

### Sub-topic 3-4: MTTD

Issue 3-4-1: Basic principles

- Proposal 1a (moderator): Define MTTD requirements in FR2-2 based on the following rule:

  - MTTD = MRTD + TA resolution error + TA adjustment accuracy + UL transmit error

- Proposal 2 (new): Define MTTD requirements in FR2-2 based on the following rule:

  - MTTD = MRTD + 2*(TA resolution error + TA adjustment accuracy + UL transmit error)

- Recommended WF

  - Consider agreeing to proposal 1a

| Company | Comments |
| --- | --- |
| vivo | Agree with Proposal 1a. |
| Nokia | We think the proposal is missing a 2X factor. So we propose Option 2. If we follow the Rel 15 discussions, we would have - MTTD = MRTD + 2*(TA resolution error + TA adjustment accuracy + UL transmit error) |
| Ericsson | Proposal 1a. |
| Huawei | Agree with Nokia. But we think it should be (TA resolution error + TA adjustment accuracy + UL transmit error) in CC1 + (TA resolution error + TA adjustment accuracy + UL transmit error) in CC2 instead of simply *2. |
| Qualcomm | Proposal 1a, but are open to consider the factor of 2. |
| LGE | Fine with Nokia’s proposal. In our understanding, 2*(TA resolution error + TA adjustment accuracy + UL transmit error) was considered as worst case uplink error between two carriers. |
| Apple | Proposal 2 is better, can consider Huawei’s refinement. |
| CATT | Agree with Nokia’s concern and we think Huawei's analysis is more accurate. |
| Intel | Ok with Huawei’s revision of Proposal 2. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200916 Draft CR adding timing requirements for FR2-2 | Huawei: Thanks for the CR. For transmit timing part, we think it is not stable yet not only the exact value. (e.g. set of requirements, whether depend on TRS and SCS combinations). |
|  | Qualcomm: ‘120 kHz of or smaller’, ‘480 kHz of or larger’ We suggest removing TRS related text since there hasn’t been much discussion on that yet and is quite open. Also remove note2 and note3 from the table |
|  | Apple: Thanks for the CR. We are not ready to agree to the values in Table 7.3.2.2-1: UE Timing Advance adjustment accuracy. At the last meeting, we proposed different values in R4-2117418. |
| R4-2202048 Draft CR for timing requirements for FR2-2 – MRTD, MTTD | Nokia: Thanks for the CR. On clause 7.7.1 We believe that this clause should not add UE frame boundary assumption when the deriveSSB-IndexFromCell is disabled. |
|  | Huawei: Same view as Nokia |
|  | CATT: Agree with Nokia, we think when deriveSSB-IndexFromCell is not enabled, it should not add UE frame boundary assumption. |

# Topic #4: Measurement procedures

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200654 | Vivo | Proposal 6: The channel model from RAN1 should be used for FR2-2. Proposal 7a: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2 if using the channel model from RAN1. Proposal 7b: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 is extended by 3 samples for FR2-2 if using the channel model of FR2-1. Proposal 8a: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1. Proposal 8b: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 3 samples compared with FR2-1 if using the channel model of FR2-1. Proposal 9: SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency inter-frequency measurements no matter if the channel model is from RAN1 or FR2-1. |
| R4-2200888 | Ericsson | Proposal 8: Considering the channel delay spread in FR2-2 is expected to be smaller than that for FR2_1, AWGN/TDL-A (5/10/20ns delay spread) maybe enough to be reference channel model to derive SSB identification delay. |
| R4-2201194 | Huawei, HiSilicon | Proposal 4: Requirements for cell detection and SSB measurement for FR2-1 can apply to FR2-2 considering channel model from RAN1. |
| R4-2201787 | Intel | Proposal 11: RAN4 to define requirements on Time period for time index detection TSSB_time_index_intra (and TSSB_time_index_intra_CCA) for FR2-2. Proposal 12: RAN4 to define requirements on Time period for time index detection as it is shown in Tables 2.4-2 and 2.4-3 Table 2.4-2. Time period for time index detection for intra-frequency measurements without measurement gaps TSSB_time_index_intra. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter. Table 2.4-3. Time period for time index detection for intra-frequency measurements with measurement gaps TSSB_time_index_intra. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter. |
| R4-2200889 | Ericsson | Draft CR to 38.133 On general measurement requirement for extending NR operation to 71GHz |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1: Channel model for measurement procedures

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 4-1-1: Channel model

- Proposal 1 (Vivo, Huawei, Ericsson): The channel model from RAN1 [38.808] should be used for FR2-2.

  - AWGN and TDL-A (5/10/20ns delay spread)

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Huawei | Support proposal 1. |
| Nokia | Fine with proposal 1 |
| Huawei | Support proposal 1. |
| vivo | Support Proposal 1. |
| Qualcomm | Fine with proposal 1 |

### Sub-topic 4-2: Cell detection

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 4-2-1: Cell detection

- Proposal 1a (Vivo, Huawei): Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2, considering channel model from RAN1 (Issue 4-1-1)

- Proposal 1b (Vivo): Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 is extended by 3 samples for FR2-2, considering channel model of FR2-1

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Support proposal 1a. |
| Nokia | Support Proposal 1a |
| Ericsson | Proposal 1a is enough. |
| vivo | Support Proposal 1a. |

### Sub-topic 4-3: PBCH detection for SSB index acquisition

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 4-3-1: PBCH detection for SSB index acquisition

- Proposal 1a (Vivo): SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1.

- Proposal 1b (Vivo): SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 3 samples compared with FR2-1 if using the channel model of FR2-1.

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Nokia | Prefer to keep as FFS. |
| vivo | Support Proposal 1a. Based on our simulation, if using the channel model from RAN1, a sample need to be extended for FR2-2 compared with the existing 5 samples of FR2-1 (without taking Rx beam sweeping factor into account). |

### Sub-topic 4-4: SSB measurements

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 4-4-1: SSB measurements

- Proposal 1 (Vivo, Huawei): SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency and inter-frequency measurements.

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Support proposal 1. |
| Nokia | We are fine with Proposal 1. |
| Ericsson | Support proposal 1. |
| vivo | Support Proposal 1. |

### Sub-topic 4-5: Intra-frequency measurements

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 4-5-1: Intra-frequency cell identification

- Proposal 1 (Intel): RAN4 to define requirements on Time period for time index detection TSSB_time_index_intra (and TSSB_time_index_intra_CCA) for FR2-2 as below:

Table 2.4-2. Time period for time index detection for intra-frequency measurements without measurement gaps

|  | TSSB_time_index_intra |
| --- | --- |
| No DRX | Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter |

Table 2.4-3. Time period for time index detection for intra-frequency measurements with measurement gaps

|  | TSSB_time_index_intra |
| --- | --- |
| No DRX | Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter |

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Huawei | Support proposal 1 if it is agreed that deriveSSB-IndexFromCell is not always enabled. |
| MTK | Support proposal 1. |
| Nokia | Prefer to keep the formulas as FFS. |
| Ericsson | Revisit the issue after agreement on Issue 2-2-1 and relevant issues. |
| vivo | Proposal 1 is OK. And the value of MSSB_index_intra can wait for the outcome of Issue 1-2-1 and Issue 4-3-1. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 4-1: Channel model for measurement procedures | Issue 4-1-1: Channel model Companies’ views: Most companies agree with the proposal Tentative agreements: - The channel model from RAN1 [38.808] should be used for FR2-2. - AWGN and TDL-A (5/10/20ns delay spread) Candidate options: Recommendations for 2nd round: No further discussion needed. |
| Sub-topic 4-1: Cell detection | Issue 4-2-1: Cell detection Companies’ views: Most companies agree with the proposal 1 Tentative agreements: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2, considering the agreed channel model (from RAN1) Candidate options: Recommendations for 2nd round: No further discussion needed. |
| Sub-topic 4-3: PBCH detection for SSB index acquisition | Issue 4-3-1: PBCH detection for SSB index acquisition Companies’ views: Not enough comments. One company prefer FFS Tentative agreements: FFS: PBCH detection for SSB index acquisition for 480/960 kHz SCS Candidate options: Recommendations for 2nd round: No further discussion needed. |
| Sub-topic 4-4: SSB measurements | Issue 4-4-1: SSB measurements Companies’ views: Most companies agree with the proposal 1 Tentative agreements: SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency and inter-frequency measurements. Candidate options: Recommendations for 2nd round: No further discussion needed. |
| Sub-topic 4-5: Intra-frequency measurements | Issue 4-5-1: Intra-frequency measurements Companies’ views: Most companies are fine with the proposal but prefer to wait for conclusion in other issues Tentative agreements: FFS: RAN4 to define requirements on Time period for time index detection TSSB_time_index_intra (and TSSB_time_index_intra_CCA) for FR2-2 as below: Table 2.4-2. Time period for time index detection for intra-frequency measurements without measurement gaps TSSB_time_index_intra. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter. Table 2.4-3. Time period for time index detection for intra-frequency measurements with measurement gaps TSSB_time_index_intra. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter. Candidate options: Recommendations for 2nd round: No further discussion needed. |

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200889 Draft CR to 38.133 On general measurement requirement for extending NR operation to 71GHz | Nokia: The values in Table 9.1.2-4b are not in line with the agreements from RAN4#100. There is also discussion on Topic #3 of thread 217. |
|  | Qualcomm: This CR is treated under thread 217 |

# Topic #5: RRM UE feature list

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200286 | Apple | Issue 5-1-1 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 5-1: RRM UE feature list

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 5-1-1: RRM UE feature list

- Proposal 1 (Apple): The following features are being proposed

  - X1 - UE support of max. CBW for supported SCS

    - Capability of supported max. CBW

      - 400MHz for 120kHz SCS

      - 1600MHz for 480kHz SCS

      - 2000MHz for 960kHz SCS

    - Note: this capability may need to be split into three capabilities, i.e. one for each supported SCS

  - X4 - UL support of 64QAM in the UL

    - Capability of support of 64QAM in the UL

  - X5 - Active BWP switching delay

    - Reuse R15 capability bwp-SwitchingDelay

    - There is another capability bwp-SwitchingMultiCCs-r16 in R16 which is likely to be reused as well, pending further discussion

  - X6 - Initial transmit timing error

    - Depend on the outcome of ongoing discussion. There is a possibility of having UE capability on Te and the supported SSB and UL SCS combinations

- Recommended WF

  - Discuss the feature list.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | As commented by companies, we X1 and X4 will be discussed in other thread. For X5, we agreed to reuse existing requirements for BWP switching related requirements. For X6. It depends on discussion about Te requirements. |
| Nokia | X1 and X4 are also being discussed in the RF session. Better to leave the discussion there. X5 – We are fine reusing R15 and R16 capability if needed X6 – we don’t agree with that capability. From the network perspective having UEs that do not support Te requirements would imply in great degradation of performance. |
| Qualcomm | Agree with others that X1 and X4 are being discussed in the RF thread. Do we need X5 as we agreed to use the existing requirements No conclusion on capability signaling for Te yet. So FFS on X6 |
| Ericsson | We agree with Huawei that issues X1 and X4 should not be discussed in RRM. For X5 we also agree to reuse the existing BWP switching related requirements, For X6, Initial transmit timing error, cannot be a feature because requirements cannot be feature. This was concluded in R16 in RAN4 and in RAN. The UE capable of certain SCS should meet Te requirements for that SCS. |
| vivo | We understand X1 and X4 is the scope of RF. For X5, it can be observed that no companies have doubt for capability bwp-SwitchingMultiCCs-r16 from the thread of 217. Therefore, we suggest that capability bwp-SwitchingMultiCCs-r16 can be reused in FR2-2 in R17. As for X6, we suggest pending the further discussion on Issue 3-1-6. |
| Apple | We are OK to further discuss X6 |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 5-1: RRM UE feature list | Issue 5-1-1: RRM UE feature list Companies’ views: X1 and X4 are being treated in the main session. For X5, existing requirements are being used, so most companies don’t see the need to discuss it. X6 depends on the discussion on Issue 3-1-6. Tentative agreements: Candidate options: - X6 - Initial transmit timing error Recommendations for 2nd round: Further discuss X6, after agreements on Issue 3-1-6 |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 5-1: RRM UE feature list

Issue 5-1-1: RRM UE feature list

- Proposal 1 (Apple): The following features are being proposed

  - X6 - Initial transmit timing error

    - Depend on the outcome of ongoing discussion. There is a possibility of having UE capability on Te and the supported SSB and UL SCS combinations

- Recommended WF

  - Further discuss X6, after agreements on Issue 3-1-4

| Company | Comments |
| --- | --- |
| Nokia | Agree with recommended WF. |
| Ericsson | For X6, Initial transmit timing error, cannot be a feature because requirements cannot be feature. This was concluded in R16 in RAN4 and in RAN. The UE capable of certain SCS should meet Te requirements for that SCS. |
| Huawei | It is bit confused to have Te requirements capability. What is the expected UE behavior when UE support the band and SCS but can not support this capability. Is UE allowed to transmit? So we prefer not to have this capability. |
| Apple | We agree with the WF. |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR extension to 71 GHz RRM requirements (Part 1) | Qualcomm | WF is supposed to capture the agreements and open issues |
| LS on UE transmit timings | Nokia | To: RAN1; Cc: RAN2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200655 | Draft CR to 38.133 Introducing applicability of requirements for FR2 | Vivo | Come back to | To be treated in 2nd round |
| R4-2200889 | Draft CR to 38.133 General measurement requirement for extending NR operation to 71GHz | Ericsson | Not Pursued | To be treated in thread 217 |
| R4-2200916 | Draft CR to 38.133 Timing requirements for FR2-2 | Nokia | Come back to | To be treated in 2nd round |
| R4-2200933 | Draft CR to 38.133 Introduction of scheduling restriction for FR2-2 | Mediatek | Come back to | To be treated in 2nd round |
| R4-2202048 | Draft CR to 38.133 Draft CR for timing requirements for FR2-2 – MRTD, MTTD | Qualcomm | Come back to | To be treated in 2nd round |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2202657 | WF on NR extension to 71 GHz RRM requirements (Part 1) | Qualcomm | Agreeable | WF captures the agreements and open issues |
| R4-2202755 | Draft CR to 38.133 Introducing applicability of requirements for FR2 | Vivo | Agreeable |  |
| R4-2202756 | Draft CR to 38.133 Timing requirements for FR2-2 | Nokia | Agreeable |  |
| R4-2202757 | Draft CR to 38.133 Introduction of scheduling restriction for FR2-2 | Mediatek | Agreeable |  |
| R4-2202048 | Draft CR to 38.133 Draft CR for timing requirements for FR2-2 – MRTD, MTTD | Qualcomm | Agreeable |  |
| R4-2202754 | Draft Big CR: RRM requirements for Rel-17 NR extension to 71GHz | Qualcomm, Intel |  | For email approval |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| Intel | Ilya Bolotin | ilya.bolotin@intel.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| Apple | Steven Chen | steven.x.chen@apple.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)