3GPP TSG-RAN WG4 Meeting # 101-bis-e 												R4-2202736

Electronic Meeting, Jan. 17th – 25th, 2022

Agenda item:			6.19.3

Source:	Moderator (Samsung)

Title:	Email discussion summary for [101-bis-e][219] NR_feMIMO_RRM

Document for:	Information

Introduction

This e-mail discussion summary captured the discussions for Rel-17 FeMIMO RRM in RAN4 #101-bis-e meeting.

In RAN4 101-e meeting, two WFs were approved.

- WF on FeMIMO RRM impact for unified TCI was approved in R4-2120320; and

- WF on FeMIMO RRM requirements for inter-cell beam management was approved in R4-2120321.

In addition, two coming LS from RAN1 would be discussed.

In this e-mail discussion, the following topics are arranged based on agenda items.

- Topic #1: Unified TCI (6.19.3.1)

- Topic #2: Inter-cell beam management (6.19.3.2)

- Topic #3: Other RRM requirements (6.19.3.3)

Based on the e-mail discussions, WF (s) is expected to collect the meeting agreements for future discussions and CRs.

Topic #1: Unified TCI

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4- 2200182 | MediaTek Inc | Proposal 1: To define the PL-RS switching delay requirement only when PL-RS and associated RS with spatial relation are the same RS. Observation 1: In R17 unified TCI state framework, for the unknown PL-RS, no switching delay requirement will be defined. Proposal 2: No requirement when the PL-RS or associated RS with spatial relation is unknown. Observation 2: For DCI based TCI state switch, the target PL-RS is maintained. For MAC CE TCI state based, the target PL-RS may be maintained or non-maintained. Proposal 3: For DCI based switch, DCI based UL TCI state switching requirements can be re-used for PL-RS switching delay, i.e., Y symbol. Proposal 4: For MAC CE based switch, the PL-RS switching delay requirement should consider the maintained and non-maintained cases. Proposal 5: For MAC-CE based UL TCI switch in separate UL/DL mode for serving cell, the delay requirement for known case is THARQ + 3Nslotsubframe,µ + NM* ( Tfirst_target-PL-RS+ 4*Ttarget_PL-RS + 2ms)/NR slot length, where NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise. Proposal 6: For the MAC-CE/DCI based TCI-pair indication in separate UL/DL TCI states switch, the UL/DL state switch delay requirements can reuse the UL/DL delay requirement defined in separate TCI mode, respectively. Proposal 7: For MAC-CE based joint UL/DL TCI states switch, to define a total switching delay requirement for known case and delay the requirement is THARQ+3Nslotsubframe,μ+max(NM×Tfirst_target_PL-RS+4×TTarget_PL-RS+2ms/NR slot length,TOk×(Tfirst-SSB+TSSB-proc)/NR slot length). Proposal 8: For DCI based switch, the new TCI state should be applied starting from the first slot that is at least Y symbols after the last symbol of the PUCCH containing ACK corresponding to DCI indication. Proposal 9: For common TCI switching delay for CA case, reuse the delay requirement as the TCI state switching for single CC, with the clarification that the first slot to apply the new TCI state is determined on the CC with the smallest SCS among the CCs which applying the beam indication. Proposal 10: For known non-serving cell, re-use serving cell's R17 MAC-CE/DCI based unified TCI switching delay requirements. |
| R4-2200277 | Apple | Known Condition Proposal #1: Re-use the existing known conditions for associated DL-RS for joint and separate DL/UL TCI for MAC CE and DCI based switch. Proposal #2: Define switching delay for UL TCI state associated with DL-RS based on command decoding time and whether the target TCI state is - Known or unknown - Associated PL-RS is maintained MAC CE based switch Proposal #3: Define MAC -CE based switching delay for UL TCI state associated with DL-RS as: THARQ + 3ms + NM*(Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) for known TCI THARQ + 3ms + TL1-RSRP + (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) for unknown TCI Observation #1: For joint TCI state switch referring to DL and UL switching delay to define joint TCI switch delay would count common processing delays in DL and UL twice, making the delay much longer than necessary. Proposal #4: Define a total switching delay for joint TCI state switching requirements. Proposal #5: Define joint TCI state switching delay from the slot switching command is received until UE can receive DL channel or transmit UL channel with target TCI state, whichever is later. Proposal #6: Define MAC CE based joint TCI state switch for known TCI state as: THARQ + 3ms + max{TOk*(Tfirst-SSB + TSSB-proc) , NM*( Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms)} Proposal #7: Define MAC-CE based switching delay requirements for unknown joint TCI state as: THARQ + 3ms + TL1-RSRP +max{TOuk*(Tfirst-SSB+ TSSB-proc) , (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms)} Proposal #8: Define DCI based TCI state switching requirements for joint and separate DL and UL TCI. DCI based switch Proposal #9: Define requirements for DCI based joint or separate DL/UL TCI state switching for known target TCI, TCI in active TCI list and maintained PL-RS. Proposal #10: Define DCI based joint or separate DL/UL TCI state switching delay as TACK +Y for known TCI state. TCI state switch associated with different PCI Observation #2: The TCI state switch associated with different PCI would be either separate DL or joint TCI. Proposal #11: Define TCI state switching requirements associated with different PCI for separate DL and joint TCI only. Proposal #12: Components of delay for TCI state switching associated with different PCI are (1) MAC-CE decoding or DCI processing time, (2) Time acquisition delay if target TCI was not in active TCI list monitored by UE, (3) Time for RX beam acquisition if target TCI state is unknown, (5) Time for Active BWP switch. Proposal #13: Discuss and define interruption requirements for TCI state switch associated with different PCI. |
| R4-2200593 | ZTE Corporation | Proposal 1: The targets of Option 1 and Option 2 are same, considering Option 1 is aligned with RAN1’s conclusion and in line with RAN4’s definition habit, we prefer Option 1. Proposal 2: For joint TCI switch, for the case of different SCS between PDSCH/PDCCH and PUSCH/PUCCH/SRS, it should be determined which SCS is used to count Y. Proposal 3: For separate TCI switch with a pair of DL TCI and UL TCI, whether the case of only one of the two target TCI states is known allowed, which should be determined. Proposal 4: Considering for applying the unified TCI state in intra-band CA case, still reuse the existing known condition is enough, not need to update the known condition. Proposal 5: For simplicity, using Option 2 for both joint mode and separate mode can be tradeoff. Proposal 6: For separate mode with DL and UL TCI state switch delay, we also prefer to using Option 2. Proposal 7: Considering the PL-RS switch is not as frequently as UL TCI state switch, and actually UE can always maintain target PL-RS, we prefer Option 2 for the case of separate UL TCI state switch. Proposal 8: Based on the assumption that UE can always maintain the target PL-RS for the case of UL TCI state switch triggering PL-RS switch, we support re-using the UL TCI state switching requirements as PL-RS switching requirements. |
| R4-2200602 | vivo | Observation 1 The switching of TCIstate/SpatialRelation/PL-RS in R15/R16 does not necessarily cause interruption. Proposal 1 The requirements for UL TCI switching delay and requirements for PL-RS update delay are specified as two separate delay. For PL-RS activation delay, only specify requirements when PL-RSs are known. Proposal 2 RAN4 works on the PL-RS update delay requirements only for the case where PL-RS is identical to the spatial relation RS in the UL TCI, unless RAN1 reaches further conclusions on other cases. Proposal 3 If a MAC CE activates only one separate UL TCI, R16 MAC CE based spatial relation info update requirements can be reused for R17 MAC-CE based UL TCI state switch, for both known case and unknown case, and R16 MAC CE based PL-RS requirements can be reused for R17 PL-RS update delay caused by UL TCI switch. Observation 2 In R17 separate TCI, for MAC-CE based TCI state list updates, one MAC CE can update a list with mixed UL TCIs and DL TCIs. Proposal 4 RAN4 specify MAC CE based TCI state list update requirements comprising UL TCI list update delay, DL TCI list update delay, and the potential maintained PL-RS list update delay. Proposal 5 If the TCI state list update include both DL TCIs and UL TCIs, the endpoint of the TCI state switching delay is the later one between DL TCI switching delay and UL TCI switching delay. Proposal 6 If there is at least one unknown DL or UL TCI in the TCI list being activated, the requirement for TCI state list update delay follow the respective unknown case, i.e. extra delay for the respective L1-RSRP measurement is considered. Proposal 7 If a MAC CE activates only one joint TCI, the delay for TCI state switch and the delay for associated PL-RS update are separately specified, and the TCI switch delay comprises the overall delay for UE to apply the sync assumption, Rx spatial filter and Tx Spatial filter. Proposal 8 For MAC CE based joint TCI state list update, only specify requirements for the case when all TCI states are known. Proposal 9 For DCI-based joint TCI and separate TCI state switching in both single carrier case and CA case, the switching delay follows RAN1 agreements on beam application time. Proposal 10 Specify requirements for common TCI state switching delay in CA scenario, i.e. the switching delay between the TCI states whose QCL-D or UL TX filter is determined by a source RS in one of the CCs, while QCL-A or QCL-C is still determined by the RS in each CC. Proposal 11 Specify per-CC known status for the common TCI state. Proposal 12-1 For MAC-CE based common TCI state or common TCI state list update delay, if all TCI-CC pairs are known, UE follows the requirements of common TCI switch delay for known case, which is identical to single CC MAC CE based TCI state or TCI state list update delay for the known case. Proposal 12-2 For MAC-CE based common TCI state or TCI state list update delay, if any TCI-CC pair is unknown, and the total number of unknown TCI-CC pairs is within UE capability ‘beamManagementSSB-CSI-RS’, UE follows the requirements of common TCI switch delay for unknown case, which is identical to single CC MAC CE based TCI state or TCI state list update delay for the unknown case. Proposal 12-3 For MAC-CE based common TCI state or TCI state list update delay, if the total number of unknown TCI-CC pairs exceeds the UE capability ‘beamManagementSSB-CSI-RS’, no RRM requirements are defined. Proposal 13 RAN4 starts discussion on the RRM requirements for the addition of the ‘cell with different PCI’, while legacy requirements for MAC-CE based SCell activation can be used as baseline. Proposal 14 For MAC-CE based TCI state activation, if the TCI state being activated belongs to a cell with different PCI, UE need to check whether the ‘cell with different PCI’ is known before checking whether the TCI state is known. Proposal 15 For the known condition for the ‘cell with different PCI’, the known condition for deactivated SCell can be reused, except that the measurement period for ‘cell with different PCI’ should be the SMTC period for the intra-frequency L3 measurements. Observation 3 The L3 measurement periodicity considered for the activation requirements in ‘cell with different PCI’ can be much shorter than that for SCell activation Observation 4 How does network know whether UE has successfully switched the TCI to a ‘cell with different PCI’ not is still unclear based RAN1/RAN2 conclusions. Observation 5 Interruption is considered in intra-frequency DAPS HO due to the baseband and RF adjustments for the activation of another cell. Proposal 16 For MAC-CE based TCI state activation, if the TCI state being activated belongs to a cell with different PCI, and no TCI in the active TCI list is associated to this PCI, the TCI state switching delay need to comprise the time for activation of ‘cell with different PCI’, in which whether an interruption window is required needs further discussion in RAN4. Proposal 17 RRM Requirements for inter-cell BM under CA scenario are deprioritized in R17, and can be delayed to R18 if no enough time. |
| R4-2200642 | CMCC | Proposal 1: for MAC-CE based UL TCI switching in separate TCI mode for serving cell, for the case that a PL-RS is included in UL TCI state or joint TCI state, the existing requirements can be reused, which means the existing MAC-CE based pathloss reference signal switch delay requirements can be reused, but the terminology of UL spatial relation need to be updated by UL TCI state. Proposal 2: for MAC-CE based UL TCI switching in separate TCI mode for serving cell, for the case that a PL-RS is associated with UL TCI state or joint TCI state, UL TCI switching delay requirements and pathloss reference signal switching delay requirements can be specified separately, and the existing requirements can be reused. Proposal 3: For MAC-CE based DL/UL TCI switching delay in Joint TCI mode for serving cell, it is proposed to define the DL switching delay requirements and UL switching delay requirements separately. Proposal 4: For DCI based TCI state switching (joint or UL TCI), the delay requirements can be defined based on RAN1 agreements of Y symbols. Observation 1: according to existing spec, when the UL spatial relation info switch for PUCCH changes both the associated DL RS and pucch-PathlossReferenceRS with the same MAC-CE activation, and if both the DL RS and pucch-PathlossReferenceRS are known, the UE shall be able to transmit PUCCH with the target UL spatial relation after the delay specified for known pathloss reference signal switch. Observation 2: for the unknown case, either the associated DL RS for uplink spatial relation switching or pucch-PathlossReferenceRS are unknown, according to existing spec, a longer switching delay is allowed, which means there are no detailed requirements. Observation 3: for MAC-CE based UL TCI switching in separate TCI mode for serving cell, the case that a PL-RS is included in UL TCI state or joint TCI state is similar like the scenario specified in existing spec that both the associated DL RS for UL spatial relation info and pucch-PathlossReferenceRS are configured with the same MAC-CE activation. |
| R4-2200649 | Nokia, Nokia Shanghai Bell | PL-RS switching delay requirement Proposal 1 : Regarding how to capture the support of the PL-RS switching to non-serving cell in the spec : - Add non-serving cell support in TS.38133 chapter 8.14 PL-RS activation requirement. The requirements in this clause apply for pathloss reference signal activated or updated on serving cell or [a cell configured with a different PCI] in MR-DC or standalone NR in clause 7.1.1 in TS 38.213. - The requirement in ‘8.14.3 MAC-CE based pathloss reference signal switch delay’ can be reused. Proposal 2 : Review known conditions for supporting switching to non-serving cell in “8.14.2 Known conditions for pathloss reference signal.” (ex : add a condition of when a cell with a different PCI is configured with association of SSB indices and PCI indices.) Active TCI switching delay to serving cell Proposal 3 : MAC-CE based TCI switching delay does not have difference for a serving cell and non-serving cell. Proposal 4 : DCI based TCI switching delay does not have difference for serving cell and non-serving cell. Switching delay for separate TCI/ Joint TCI Observation 1 : Unified TCI feature is the TCI switching framework in Rel-17. In physical layer operation, actual TCI switching is executed as DL TCI switching or UL TCI switching under Rel-17 unified TCI framework. Proposal 5 : For joint TCI, DL and UL requirements can be applicable respectively. ( also for a pair of DL and UL separate TCI ) - For DL scheduling, the UE shall be able to receive PDCCH/PDSCH with the new target DL TCI state in the joint TCI at the first slot that is after a required DL switching delay in 8.10 - For UL scheduling, the UE shall be able to transmit PUCCH/PUSCH/SRS with the new target UL TCI state in the joint TCI at the first slot that is after a required UL switching delay in 8.12 ( Reuse the current DL and UL TCI switching requirements ) Proposal 6 : For a pair of DL and UL separate TCI switching requirements, the same requirement statements in Proposal 4 can be applicable. Beam application time (BAT) for ‘indicated TCI’ Observation 2 : The first slot to apply the ‘indicated TCI’ is at least Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. The legacy DCI-based TCI switching delay requirement cannot be reused for the indicated TCI switching. Proposal 7 : Capture the statement below as a requirement for the indicated TCI switching delay to consider ‘Y symbol’ (= BeamAppTime_r17 symbols) - A UE shall be able to receive PDSCH with target TCI state of the serving cell [or non-serving cell] on which TCI state switch occurs at the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH with HARQ-ACK information. Wording of “non-serving cell” Proposal 8 : Wording of “non-serving cell” needs to be aligned with RAN1/2 spec. TS38.214 uses wording as “SS/PBCH block having a PCI different from the PCI of the serving cell” rather than directly mention ‘a cell’. |
| R4-2200787 | Intel Corporation | Observation 1: If PL-RS is included in UL TCI state, PL-RS and associated RS in UL TCI state are identical. They will be known or unknown at the same time. Proposal 1: If PL-RS is included in UL TCI state and the TCI state is known, MAC-CE based uplink spatial relation switching delay requirements can be re-used for UL TCI switching delay. Proposal 2: If PL-RS is included in the UL TCI state and the TCI state is unknown, the delay requirement for UL TCI switching delay is: n+ THARQ + 3Nslotsubframe,µ+ TL1-RSRP+ 5*Ttarget_PL-RS + 2msNR slot length Observation 2: If PL-RS is associated with UL TCI state, PL-RS and associated RS in UL TCI are QCL-typeD. They will be known or unknown at the same time. Proposal 3: When PL-RS is associated with UL TCI sate and PL-RS is activated in the same MAC-CE with UL TCI state switching, if TCI state is known, legacy MAC-CE based uplink spatial relation switching delay requirements can be re-used. Proposal 4: If associated DL RS and Pathloss Reference RS are unknown, the delay requirement is the same as the case when PL-RS is included in UL TCI state and the TCI state is unknown. Proposal 5: If PL-RS is associated with UL TCI state and PL-RS is not activated in the same MAC-CE with UL TCI state switching, the legacy MAC-CE based uplink spatial relation switching delay requirement for known case and unknown case can be re-used for UL TCI state switching. Proposal 6: MAC-CE based UL TCI switching delay will apply for PUCCH, aperiodic SRS, semi-persistent SRS and periodic SRS. Proposal 7: For MAC-CE based TCI state-pair indication, the TCI state switching delay requirement can be defined for UL TCI and DL TCI switching respectively. Proposal 8: UL TCI state switching requirements can be re-used for PL-RS switching delay if UL TCI state switching can trigger PL-RS switch. Proposal 9: Re-use the DL/UL TCI state switching delay respectively for joint TCI case. Proposal 10: For CA, DCI-based TCI switching delay is determined based on the carrier with the smallest SCS among the carrier(s). Proposal 11: For inter-cell TCI state switching, only define requirement for known TCI state case. Proposal 12: For inter-cell TCI state switching, define requirement when the BW of inter-cell is within the active BWP of the serving cell and the SCS are the same. Proposal 13: System information of inter-cell will be assumed to be known when defining delay requirement for TCI state switching for inter-cell. Observation 3: For uplink spatial info switch, no DL timing tracking is need since DL timing is unchanged. Observation 4: For UL TCI switching for inter-cell, DL timing of inter-cell may change and UE may need to perform DL timing tracking for Uplink if DL timing of inter-cell is not maintained. Observation 5: For UL TCI switching for inter-cell, UE may need to track DL timing for PL-RS if DL timing of inter-cell is not maintained. Proposal 14: For inter-cell UL TCI state switching, further discuss whether DL timing tracking for inter-cell is needed. Proposal 15: For inter-cell UL TCI state switching, further discuss whether there is any timing offset assumption. Proposal 16: Modify “non-serving cell” to “TRP with different PCI”. |
| R4-2201363 | Samsung | Proposal 1: RAN4 agreed the specification structure for switching delay requirements for unified TCI in RAN4 101bis meeting taking the above suggestions into account Proposal 2: Specify the PL-RS switching delay for known and maintained condition by specifying requirements as same delay requirements as UL TCI switching delay requirements, e.g., slot n+ THARQ + 3Nslotsubframe,µ for MACE-CE based PL-RS switching delay Proposal 3: For joint TCI with both uplink TCI and downlink TCI update case, separated delay requirements for downlink and uplink is applied. Proposal 5: It is suggested to apply known condition in current specification for both serving cell and non-serving cell TCI switch delay requirements. Proposal 6: Apply the existing TCI switch delay and spatial relation switch delay for intra-frequency non-serving cell downlink TCI and uplink TCI switch delay |
| R4-2201384 | Ericsson | Proposal 1: RAN4 to agree that when a DCI based TCI state switch command is received at UE at slot n, and sends ACK at slot n+TACK, it should be able to receive on the new beam at n+TACK+ TBAT. Where TBAT is signalled by gNB based on the UE capability. Proposal 2: For CA cross-carrier scheduling, RAN4 to agree that, when a DCI based TCI state switch command is received at slot n, and sends ACK at slot n+TACK, it should be able to receive on the new beam at n+TACK+ TBAT. Where TBAT is signalled by the gNB based on the UE capability and the slot and beam application time are based on the carrier with smallest SCS. Proposal 3: Rel-16 MAC-CE based TCI state switching requirements to be reused for unified joint TCI state switching based on MAC-CE. Proposal 4: RAN4 to agree that when a DCI based TCI state switch command is received at UE at slot n, and sends ACK at slot n+TACK, it should be able to receive on the new beam at n+TACK+ TBAT. Where TBAT is signalled by gNB based on the UE capability. Proposal 5: For CA cross-carrier scheduling, RAN4 to agree that, when a DCI based TCI state switch command is received at slot n, and sends ACK at slot n+TACK, it should be able to receive on the new beam at n+TACK+ TBAT. Where TBAT is signalled by the gNB based on the UE capability and the slot and beam application time are based on the carrier with smallest SCS. Proposal 6: Rel-16 MAC-CE based TCI state switching requirements to be reused for unified separate TCI state switching based on MAC-CE. Proposal 7: Requirements for PL-RS update under TCI framework shall use the separate UL TCI state switch requirements/joint TCI state switch requirements. Proposal 8: RAN4 to agree that TCI switch delay requirements for target TCI is associated with non-serving cell should be same as target TCI is associated with serving cell. |
| R4-2201616 | Huawei, HiSilicon | Proposal 1: The following TCI state switching delay requirements needs to be defined for R17 unified TCI framework. - MAC-CE based DL TCI state switching delay - MAC-CE based UL TCI state switching delay - DCI based DL TCI state switching delay - DCI based UL TCI state switching delay Proposal 2: The same DL TCI state switching delay requirements are applied when the target DL TCI state is indicated by a DL TCI state, a joint TCI state or a TCI pair. Proposal 3: The same UL TCI state switching delay requirements are applied when the target UL TCI state is indicated by a UL TCI state, a joint TCI state or a TCI pair. Proposal 4: The TCI state switching delay requirements for R17 unified TCI can be defined as follows: Requirement Type; Known/Unknown; Switching Delay; Applicable cases. MAC-CE based DL TCI state switching; known; THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc); DL TCI, joint TCI or TCI pair. Unknown; THARQ +3Nslotsubframe,µ + TL1-RSRP + TOuk*(Tfirst-SSB+ TSSB-proc). MAC-CE based DL TCI state switching; known; THARQ + 3Nslotsubframe,µ+ 1; UL TCI, joint TCI or TCI pair. Unknown; THARQ +3Nslotsubframe,µ + TL1-RSRP +1. DCI based DL TCI state switching; Known only; THARQ + Y symbol; DL TCI, joint TCI or TCI pair. DCI based UL TCI state switching; Known only; THARQ + Y symbol; UL TCI, joint TCI or TCI pair. Proposal 5: For R17 unified TCI, the same DL/UL TCI state switching delay requirements are applied when the associated SSB for target TCI is with the serving cell PCI or with PCI different from the serving cell. Proposal 6: When the UE needs to update a PL-RS which is included in or associated with the UL/joint TCI state by DCI, the DCI based UL state switching delay can be reused as PL-RS switching delay. |

Open issues summary

Please note that some proposals (issues) might be omitted by purpose as they are deprioritized in the 1st round discussion or out of the scope of [219].

Sub-topic 1-1: Specification Structures of Unified TCI

Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay

- Proposals

  - Proposal 1: RAN4 agreed the specification structure for switching delay requirements for unified TCI in RAN4 101bis meeting, e.g. (Samsung, Intel)

8.10.15 Active downlink TCI state switching delay for unified TCI 8.10.15.1 Introduction 8.10.15.2 Know condition for downlink TCI state 8.10.15.3 MAC-CE based downlink TCI state switch delay 8.10.15.4 DCI based downlink TCI state switch delay 8.10.15.5 Active downlink TCI state list update delay 8.10.16 Active uplink TCI state switching delay for unified TCI 8.10.16.1 Introduction 8.10.16.2 Know condition for uplink TCI state 8.10.16.3 MAC-CE based uplink TCI state switch delay 8.10.16.4 DCI based uplink TCI state switch delay 8.10.16.5 Active uplink TCI state list update delay

  - Proposal 2: Separate section for the requirement of joint UL and DL TCI case

  - Proposal 3: Separate section for the requirement of PL-RS switching delay (Intel)

  - Proposal 4: Separate section for the requirement of separate TCI list update delay (vivo)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Sub-topic 1-2 Switching delay requirements for unified TCI associated with SC

Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as

- Proposals

  - Option 1: the delay requirements can be defined based on RAN1 agreements (CMCC, Samsung, Intel)

  - Option 2: TACK +Y ( Huawei, Apple)

    - Option 2a: Specify which SCS is used to count Y if that of DL and UL are different (ZTE)

    - Option 2b: Define switching delay requirement of DCI based unified TCI state switching for cases that joint and separate TCI, known TCI, TCI I in active TCI list and maintained PL-RS. (Apple)

  - Option 3: Y symbols (MTK)

  - Option 3a: ‘Y symbols’ delay is about beam application time (BAT) that cannot reuse legacy DCI-based TCI state switch delay requirement. Define a switching requirement as RAN1 agreement. (Nokia)

    - A UE shall be able to receive PDSCH with target TCI state of the serving cell [or non-serving cell] on which TCI state switch occurs at the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH with HARQ-ACK information.

  - Option 4: UE sends ACK at slot n+TACK, receive on the new beam at n+TACK+ TBAT. Where TBAT is signalled by the gNB based on the UE capability. (Ericsson)

  - Option 5: Define DCI based DL and UL TCI state switching delay also for CA case (vivo)

    - Option 5a: For CA, DCI-based TCI switching delay is determined based on the carrier with the smallest SCS among the carrier(s). (Intel, Ericsson)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as

- Proposals

  - Option 1:  For serving cell, for known case, i.e., PL-RS and associated RS with spatial relation are known (MTK)

    - THARQ + 3Nslotsubframe,µ + NM* ( Tfirst_target-PL-RS+ 4*Ttarget_PL-RS + 2ms)/NR slot length, where NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

    - For unknown  case, i.e., PL-RS or associated RS with spatial relation is unknown , no requirement is applied.

  - Option 2: for UL TCI state associated with DL-RS as following for both known and unknow TCI state and associated PL-RS is maintained (Apple, Intel):

    - THARQ + 3ms + NM*(Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) for known TCI

    - THARQ + 3ms + TL1-RSRP + (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) for unknown TCI

  - Option 3: Reuse the existing MAC-CE based uplink spatial relation switching delay requirements  for known and unknown case (ZTE, vivo, Samsung, Huawei, Nokia)

  - Option 4: (Intel)

    - If PL-RS is included in UL TCI state, reuse uplink spatial relation switching delay requirements for known case and n+ THARQ + 3Nslotsubframe,µ+ TL1-RSRP+  5*Ttarget_PL-RS + 2msNR slot length for unknown case.

    - When PL-RS is associated with UL TCI sate and PL-RS is activated in the same MAC-CE with UL TCI state switching, if TCI state is known, legacy MAC-CE based uplink spatial relation switching delay requirements can be re-used.

    - If associated DL RS and Pathloss Reference RS are unknown, the delay requirement is the same as the case when PL-RS is included in UL TCI state and the TCI state is unknown.

    - If PL-RS is associated with UL TCI state and PL-RS is not activated in the same MAC-CE with UL TCI state switching, the legacy MAC-CE based uplink spatial relation switching delay requirement for known case and unknown case can be re-used for UL TCI state switching.

  - Option 5: for known case, if PL-RS is included in UL TCI state, reuse the existing MAC-CE based pathloss reference signal switch delay requirements; If PL-RS is associated with UL or joint TCI, UL TCI switching delay requirements and pathloss reference signal switching delay requirements are specified separately, and existing requirements on UL spatial relation switching and existing requirements on PL-RS can be reused respectively. (CMCC)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as

- Proposals

  - Option 1: No extra requirement needed for Joint TCI mode, DL and UL requirements can be applicable respectively. (Samsung, CMCC, Ericsson, Intel, Huawei, Nokia)

  - Option 2: Define a total switching delay for joint TCI state switching requirements and define the requirement from the slot switching command is received until UE can receive DL channel or transmit UL channel with target TCI state, whichever is later. (Apple, vivo, MTK)

    - Option 2a: (Apple)

      - THARQ + 3ms + max{TOk*(Tfirst-SSB + TSSB-proc) , NM*( Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms)} for known joint TCI state

      - THARQ + 3ms + TL1-RSRP +max{TOuk*(Tfirst-SSB+ TSSB-proc) , (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms)} for unknown joint TCI state

    - Option 2b:  (vivo)

      - Define a total switching delay for joint TCI state switching requirements as existing DL requirement

    - Option 2c: (MTK)

      - For known case, i.e., PL-RS and associated RS with spatial relation are known: the delay requirement is THARQ+3Nslotsubframe,μ+max(NM×Tfirst_target_PL-RS+4×TTarget_PL-RS+2ms/NR slot length,TOk×(Tfirst-SSB+TSSB-proc)/NR slot length).

      - For unknown case,  i.e., PL-RS or associated RS with spatial relation is unknown, no reuqirement is applied.

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-2-4 Define MAC-CE based TCI state-pair indication requirement

- Proposals

  - Option 1:

    - the TCI state switching delay requirement can be defined for UL TCI and DL TCI switching respectively. (Intel, MTK, Huawei, Nokia)

  - Option 2:

    - RAN4 specify MAC CE based TCI state list update requirements comprising UL TCI list update delay, DL TCI list update delay, and the potential maintained PL-RS list update delay.

    - If the TCI state list update include both DL TCIs and UL TCIs, the endpoint of the TCI state switching delay is the later one between DL TCI switching delay and UL TCI switching delay. (vivo)

Sub-topic 1-3 Switching delay requirements for unified TCI associated with NSC

Issue 1-3-1 Define TCI state switching delay (associated with different PCI) as

- Proposals

  - Option 1: Components of delay are (1) MAC-CE decoding or DCI processing time, (2) Time acquisition delay if target TCI was not in active TCI list monitored by UE, (3) Time for RX beam acquisition if target TCI state is unknown, (5) Time for Active BWP switch. (Apple)

  - Option 2: discussion on the RRM requirements for the addition of the ‘cell with different PCI’, while legacy requirements for MAC-CE based SCell activation can be used as baseline. And check whether the ‘cell with different PCI’ is known by the known condition for deactivated Scell.(vivo)

  - Option 3: MAC-CE based and DCI based TCI switching delay does not have difference for a serving cell and non-serving cell. (Nokia, Ericsson)

  - Option 4: only define requirement for known TCI state case, and the BW of inter-cell is within the active BWP of the serving cell and the SCS are the same; System information of inter-cell will be assumed to be known; For inter-cell UL TCI state switching, further discuss whether DL timing tracking for inter-cell is needed and whether there is any timing offset assumption. (Intel)

  - Option 5: Apply the existing TCI switch delay and spatial relation switch delay for intra-frequency non-serving cell downlink TCI and uplink TCI switch delay. (Samsung)

  - Option 6: For R17 unified TCI, the same DL/UL TCI state switching delay requirements are applied when the associated SSB for target TCI is with the serving cell PCI or with PCI different from the serving cell. (Huawei)

  - Option 7: For known non-serving cell, re-use serving cell's R17 MAC-CE/DCI based unified TCI switching delay requirements.(MTK)

  - Option 8: apply known condition in current specification for both serving cell and non-serving cell TCI switch delay requirements. (Samsung, ZTE, Nokia)

  - Option 9: Define switching delay requirement of TCI state associated with NSC for cases that separate DL TCI and joint TCI only. (Apple)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI

- Proposals

  - Option 1: Yes (Apple)

  - Option 2: Needs further discussion in RAN4. (vivo, Intel)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-3-3 The wording of “non-serving cell”

- Proposals

  - Option 1: Wording of “non-serving cell” needs to be aligned with RAN1/2 spec. TS38.214 uses wording as “SS/PBCH block having a PCI different from the PCI of the serving cell” rather than directly mention ‘a cell’. (Nokia)

  - Option 2: Modify “non-serving cell” to “TRP with different PCI”. (Intel)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Sub-topic 1-4 Delay requirements for TCI switching in CA case

Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario

- Proposals

  - Option 1: Specify the requirement, i.e. the switching delay between the TCI states whose QCL-D or UL TX filter is determined by a source RS in one of the CCs, while QCL-A or QCL-C is still determined by the RS in each CC. (vivo)

  - Option 2: delay requirement as the TCI state switching for single CC, with the clarification that the first slot to apply the new TCI state is determined on the CC with the smallest SCS among the CCs which applying the beam indication. (MTK, Intel)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-4-2 Known condition in CA scenario

- Proposals

  - Option 1: Specify per-CC known status for the common TCI state. And if the total number of unknown TCI-CC pairs exceeds the UE capability ‘beamManagementSSB-CSI-RS’, no RRM requirements are defined. (vivo)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Sub-topic 1-5 Requirements for PL-RS switching delay indicated by unified TCI

Issue 1-5-1 Scenarios for defining  PL-RS switching requirement

- Proposals

  - Option 1: To define the PL-RS switching delay requirement only when PL-RS is identical to the spatial relation RS in the UL TCI. (MTK, vivo)

  - Option 2:  If the PL-RS or associated RS with spatial relation is unknown, no switching delay requirement will be defined. (MTK)

  - Option 3: For PL-RS activation delay, only specify requirements when PL-RSs are known. (vivo)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-5-2 Whether the PL-RS maintained in unified TCI framework

- Proposals

  - Option 1: For DCI based TCI state switch, the target PL-RS is maintained. (MTK)

  - Option 1a: For MAC CE TCI state based, the target PL-RS may be maintained or non-maintained. (MTK, Nokia, Intel)

  - Option 2: UE can always maintain the target PL-RS for the case of UL TCI state switch triggering PL-RS switch (ZTE)

  - Option 3: No maintained or non-maintained question for unified TCI framework

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements

- Proposals

  - Option 1: re-using the UL TCI state switching requirements as PL-RS switching requirements. (ZTE, Samsung, Nokia) if UL TCI state switching can trigger PL-RS switch (Intel)

  - Option 2: The requirements for UL TCI switching delay and requirements for PL-RS update delay are specified as two separate delay. (vivo, CMCC)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 1-5-4 Define DCI based PL-RS switching delay as

- Proposals

  - Option 1: For DCI based switch, DCI based UL TCI state switching requirements can be re-used for PL-RS switching delay, i.e., Y symbol. (MTK, Huawei)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Moderator | From moderator view, the following comments are made to guide the 1st round discussion regarding Topic#1, as proposals are too complicated and diverse for these issues. I think Topic#1 (Unified TCI) is the 1st priority at present. Sub topic 1-1 Specification Structures of Unified TCI Specification structure is a critical issue for future CR drafting so we’d better agree on a structure ASAP. Sub topic 1-2 Switching delay requirements for unified TCI associated with SC For DCI-based TCI state switching, we would better follow what RAN1 has agreed and defined, as R16 did. For requirements of UL TCI and joint TCI mode, we could focus on this two issues in the 1st round. For UL TCI, companies could propose define requirements on which cases and the requirement for each case. For joint TCI mode, the question is whether the requirement is independent or dependent (e.g. UL switching needs to wait for DL switching). Sub topic 1-3 Switching delay requirements for unified TCI associated with NSC For this sub-topic, a critical issue is how to define known condition of NSC (different PCI), and then the NSC requirements could refer to that of SC. Sub topic 1-4 Delay requirements for TCI switching in CA case TBA Sub topic 1-5 Requirements for PL-RS switching delay indicated by unified TCI For PL-RS, first we need to decide whether “not maintained case” still exists in unified TCI framework. After that, whether |
| Intel | Sub topic 1-1 Specification Structures of Unified TCI Fine with option 1 and option 3. Sub topic 1-2 Switching delay requirements for unified TCI associated with SC Issue 1-2-1: Support option 1. As in legacy, for DCI based switching delay, we will refer to RAN1’s specification. Issue 1-2-2: Support option 2 and 4. No matter PL-RS is included or associated with source RS in UL TCI state, requirement for unknown case can be defined. Since we consider the beam alignment case, which means that the RX beam for PL-RS and source RS in UL TCI state is the same. Pathloss can be calculated after RX beam sweeping using the same newly founded RX beam. It’s different from legacy, since in legacy we can’t guarantee that the RX beam for PL-RS and source RS in UL TCI state is the same. Issue 1-2-3: Support option 1. While for DL and UL TCI state switching, the procedure for DL TCI state switching and UL TCI state switching are independent and the ending point for DL and UL are clear. Besides, for option 1, it’s simpler and we can re-use most of the legacy requirement. Issue 1-2-4: Support option 1. Option 1 and option 2 seems to discuss different thing. For option 2, It specify TCI state list update delay. From our understanding, TCI state list update delay will equal to DL TCI state switching delay. Sub topic 1-3 Switching delay requirements for unified TCI associated with NSC Issue 1-3-1: Support option 4. If delay requirement for SC and NSC is the same, Suggest to further discuss the pre-requisite condition, e.g. active BWP, timing offset assumption. If timing offest is still defined as smaller than CP, we are fine that DL tracking may not be neceassary for UL TCI state switching. Otherwise, DL tracking may be necessary for the following purpose: - Update UL timing according to DL timing - Tracking DL timing of PL-RS for pathloss calculation For option 9, UL TCI state switching for inter-cell is valid scanario. The RAN1’s agreement is as follows: Agreement On Rel.17 beam indication enhancements for inter-cell beam management, the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) applies to: - The channels and signals as for intra-cell beam management except for non-UE dedicated channels/signals - For the aforementioned applicable channels and signals, SSB associated with a physical cell ID different from that of the serving cell is used as an indirect QCL reference for DL TCI (in case of separate DL/UL TCI) or joint TCI, or an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI). It mentions that SSB associated with a physical cell ID different from that of the serving cell can be used as an indirect/direct QCL reference for UL TCI (in case of separate DL/UL TCI). Issue 1-3-2: Fine with option 2. If T active BWP of inter-cell is within the active BW of serving cell, it seems that no extra interruption is needed. Issue 1-3-3: Fine with option 2 and open to further discuss. Sub topic 1-4 Delay requirements for TCI switching in CA case Issue 1-4-1: Fine with option 2. Issue 1-4-2: Further discussion. Since it applies for intra-band CA. we think that known status for one CC can be applied for other CC as well. It seems not necessary to specify per-CC known status. Sub topic 1-5 Requirements for PL-RS switching delay indicated by unified TCI Issue 1-5-1: Futher discussion. For option 1, we think that there maybe two cases. Case 1 is that PL-RS is included in UL TCI state where PL-RS is identical to the source RS in UL TCI. Case 2 is that PL-RS is associated with UL TCI state where PL-RS and source RS in UL TCI are QCL-typeD. How to configure PL-RS is up to RAN2. Furthermore, since the RX beam assumption is the same for PL-RS and source RS in UL TCI, if PL-RS is unknown, it’s still possible to define requirement for PL-RS switching delay with unknown condition. Therefore, we suggest to add a new option 4. Option 4: - PL-RS switching delay requirement apply when PL-RS is identical to the source RS in the UL TCI or PL-RS is associated with UL TCI state where PL-RS and source RS in UL TCI are QCL-typeD. - PL-RS switching delay requirement for both known and unknown case can be defined. Issue 1-5-2: Fine with option 1a. Issue 1-5-3: Support Option 1. Issue 1-5-4: Further discussion. depends on RAN1’s specification. |
| Apple | Sub topic 1-1 Specification Structures of Unified TCI We have not yet agreed on whether we should have joint TCI requirements or only UL and DL requirements. Also, we don’t think there is any update to DL TCI state switch, except for DCI base switch, hence prefer not to introduce a new section for DL TCI state switch. We can add a separate section in current 8.10.4 for unified TCI. This is agreeable: 8.10.X Active uplink TCI state switching delay for unified TCI We recommend adding: 8.10.Y Active joint TCI state switching delay for unified TCI There is no change to PL-RS switching delay from Rel-16. We have agreement not to introduce any new PL-RS switching delay requirements, so we don’t see why we need proposal 3. We would also like to know the motivation behind proposal 4 to introduce separate section for TCI list update delay. If the intention to add a separate sub-section under each section for joint and UL TCI switch, we are fine with that. Sub topic 1-2 Switching delay requirements for unified TCI associated with SC Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as We support the condition for DCI based switch for joint and UL TCI - Define switching delay requirement of DCI based unified TCI state switching for cases that joint and separate TCI, known TCI, TCI in active TCI list and maintained PL-RS. Most of the proposals are similar except for ones for CA case. We prefer to discuss CA case separately as all delay requirements are affected. For single carrier case we are fine with delay requirement as n+TACK+ TBAT. Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as We support option 2. We prefer to define the delay requirements for unknown case as well. Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as We support option 2a and we prefer to define the delay requirements for unknown case as well. Issue 1-2-4 Define MAC-CE based TCI state-pair indication requirement We prefer to refer to separate UL and DL switching delay for TCI pair switch since this is not the same as joint TCI state switch. Sub topic 1-3 Switching delay requirements for unified TCI associated with NSC Issue 1-3-1 Define TCI state switching delay (associated with different PCI) as In our understanding the TCI switch to non-serving cell or different PCI is only for DL TCI state and we support option 1. The TCI state switch could be for DL TCI and joint TCI but not UL TCI in our understanding. We are fine with defining conditions under which TCI state switch to different PCI are applicable. Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI We proposed option 1, but it also depends on whether we define any conditions for the switch. Without any conditions for TCI switch to different PCI, interruptions might still be needed if there is a BWP change with TCI state switch. Issue 1-3-3 The wording of “non-serving cell” We support Option 2. Sub topic 1-4 Delay requirements for TCI switching in CA case Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario For common TCI state switch in CA scenario the TCI ID is shared and switched. The TCI state ID need not be the same TCI state, so the delay would just be per CC TCI state switch. We would like to hear more on common TCI state switch applicability to CA case. Issue 1-4-2 Known condition in CA scenario Need to decide if common TCI switch will be defined first. Sub topic 1-5 Requirements for PL-RS switching delay indicated by unified TCI Issue 1-5-1 Scenarios for defining PL-RS switching requirement We don’t need to define requirements for PL-RS switching as there are no enhancements in Rel-17 for it. For PL-RS in UL TCI switch we define requirements for beam alignment case. These are inline with agreements made in last RAN4 meeting. |
| vivo | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay To proposal 1, we are fine, except that we prefer to use 8.10.X or 8.10.X.1 instead of 8.10.15.x. For Apple’s proposal, it would be fine to merge 8.10.15.3 with 8.10.3, merge 8.10.15.4 with 8.10.4. But we see difficulties in merging 8.10.15.5 with 8.10.6, since 8.10.6 is for active PDSCH TCI list, but R17 is for active DL TCI list (for both PDSCH and PDCCH). For UL TCI, we prefer to use 8.10.Y or 8.10.X.2 instead of 8.10.16.x; To proposal 2, we fully support. This can be a new section for joint UL and DL TCI switching delay. If ‘joint UL and DL TCI’ refers to not only Joint TCI but also separate TCI list update comprising both UL and DL TCI, then we are fine to merge proposal 2 and proposal 4 in one sub-section, while add some applicability in the introduction part of this. For proposal 3, same view as Apple. We also think revising 8.14 would be more simple approach for PL-RS. The proposed structure for joint UL and DL TCI would be (either 8.10.Z or 8.10.X.3) 8.10.Z Joint downlink and uplink active TCI switching for unified TCI 8.10.Z.1 Introduction 8.10.Z.2 Know condition for joint TCI 8.10.Z.3 MAC-CE based active joint TCI state switch delay 8.10.Z.4 DCI based joint TCI state switch delay 8.10.Z.5 Active joint TCI state list update delay 8.10.Z.6 Active separate TCI state list update delay comprising downlink and uplink TCIs Note: TCI for cell with different PCI is also specified in theses sections(8.10.X, 8.10.Y, 8.10.Z). Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as Same view as Apple. Do not see fundamental difference between option 1 to 4. Option 1 to 4 can be merged into 1 proposal. It is suggested to take option 4 as the baseline wording for the requirements. For CA case, we support 5 and 5a for the intra-cell TCI state switching. Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as We support option 3. As discussed in our paper, we prefer not to mix UL TCI switching and PL-RS updated in one joint delay requirements. Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as We support option 2 and 2b, and disagree with option 1. For option 1, the main issue is that the delay for DL and UL TCI state switching can be different. As discussed in our paper, the performance of either UL or DL may not be good enough if the corresponding TCI is not ready yet. Since there is dependency between UL and DL, e.g. PDCCH for uplink grant or ACK/NACK feedback for PDSCH, we do not think it is reasonable. At least from test case set-up perspective, option 1 is not preferred. Issue 1-2-4 Define MAC-CE based TCI state-pair indication requirement We support option 2 and disagree with option 1. TCI state pair is a TCI state list with one DL TCI and one UL TCI. In this case, the same issue as the joint TCI case can be inferred. Since there is dependency between UL and DL, e.g. PDCCH for uplink grant or ACK/NACK feedback for PDSCH, the switching delay for this case should consider both DL and UL are ready. If RAN4 separates UL and DL, how to test? Issue 1-3-1 Define TCI state switching delay (associated with different PCI) as We are OK to option 2 and 8. For option 1, we do not think Time for Active BWP switch is needed. The BWP is exactly the same as serving cell. For option 3, 5, 6, 7, we are OK to the proposals if the NSC is known. For option 4, ‘the BW of inter-cell is within the active BWP of the serving cell and the SCS are the same’ in our view is redundant. System information of NSC will be the same as serving cell as agreed in RAN1/2. We also see the timing issue mentioned in option 4. For option 9, we think the NSC is not precluded from UL transmission according to RAN1/2 agreements. Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI We support option 2. We are open to further discussion. Issue 1-3-3 The wording of “non-serving cell” Do not see fundamental difference between option 1 and 2. Fine with either one Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Support option 1 and 2. Fine to further discuss in future meetings Issue 1-4-2 Known condition in CA scenario Support option 1. To intel, for intra-band CA case, from MRTD perspective, there is no guarantee that the QCL-A source will be the same. Issue 1-5-1 Scenarios for defining PL-RS switching requirement We support option 1 for current stage, and open to discuss other cases based on further RAN2 agreements. For option 2 and 3, we think they are the same if option 1 is adoptable. We prefer to focus on the known case firstly. Issue 1-5-2 Whether the PL-RS maintained in unified TCI framework Fine with option 1/1a. Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements Prefer option 2. We think PL-RS may not necessarily have the same delay as the UL TCI update. This can be discussed in UL TCI requirements, i.e., 1-2-2. Issue 1-5-4 Define DCI based PL-RS switching delay as Ok to the proposal |
| Huawei | Sub topic 1-1 Specification Structures of Unified TCI For Proposal 1, generally we can agree with it. For proposal 2, a joint UL/DL TCI indication can be considered as a special case for separate UL/DL TCI indication, where the source RSs for DL TCI and UL TCI are the same. Both DL TCI switching delay and UL TCI switching delay requirements are applied. There is no need to define separate requirements for joint UL/DL TCI indication. For proposal 3, we agree to define PL-RS switching delay requirements in separate section. For proposal 4, FFS the requirements for TCI list update delay. Sub topic 1-2 Switching delay requirements for unified TCI associated with SC Issue 1-2-1: we agree with option 1 to define delay requirements based on RAN1 agreements. Issue 1-2-2: we agree with option 3. Issue 1-2-3: we agree with option 1. Issue 1-2-4: we agree with option 1. Sub topic 1-3 Switching delay requirements for unified TCI associated with NSC Issue 1-3-1: we agree with option 3, option 6, option 7 and option 8. It seems that options 3/6/7 share the same view. Issue 1-3-2: we agree with option 2. Issue 1-3-3: we agree with option 1. We suggest to further study how and where to capture the wording in RRM requirements. Sub topic 1-4 Delay requirements for TCI switching in CA case Issue 1-4-1: we agree with option 2, which is aligned with RAN1 agreements. Issue 1-4-2: further study whether the case mentioned in option 1 would occur. Sub topic 1-5 Requirements for PL-RS switching delay indicated by unified TCI Issue 1-5-1: we agree with option 3. Issue 1-5-2: we agree with option 1 and 1a. Issue 1-5-3: we agree with option 2. Issue 1-5-4: we agree with option 1. |
| CMCC | Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as Option 1. Prefer to follow the same approach for Rel-16 DCI based switching delay requirements. Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as For unknown case, we are OK to specify requirements. Option 5 is for known case, and option 5 is udated as following to reflect our consideration: Updated Option 5: for known case, if PL-RS is included in UL TCI state, reuse the existing MAC-CE based pathloss reference signal switch delay requirements; If PL-RS is associated with UL or joint TCI, UL TCI switching delay requirements and pathloss reference signal switching delay requirements are specified separately, and existing requirements on UL spatial relation switching and existing requirements on PL-RS can be reused respectively. Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as Option 1. The joint TCI mode means that a single MAC CE is used to indicate both the DL and UL TCI switch, as for the UE behaviour, in our understanding, UE will perform the DL TCI state switch and UL TCI state switch separately. From this point of view, it is proposed to define separate switching delay requirements for UL and DL. Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements To clarify our consideration. For the case that a PL-RS is associated with UL TCI state or joint TCI state, in our view, it means that UL TCI and RL-RS are configured separately, the requirements can be specified separately, For the case that a PL-RS is included in UL TCI state or joint TCI state, in our understanding, is similar like the Rel-16 case that both the associated DL RS for UL spatial relation info and pucch-PathlossReferenceRS are configured with the same MAC-CE activation, and the Rel-16 approach can be reused, which means the PL-RS switching delay requirements and UL TCI state switching requirements are the same. |
| Nokia | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay On proposal 1, we are fine on the structure. We prefer to reuse 8.10.X or 8.10.X.X instead of a new section 8.10.15. and reuse 8.12.X instead of 8.10.16. This is because we predict most of legacy requirements can be reused. As we think, new sub-sections are required for 8.10.X. DCI based downlink TCI state switch delay for unified TCI 8.12.X DCI based uplink TCI state switch delay for unified TCI 8.14.X DCI based pathloss reference signal switch delay for unified TCI However, up to the agreements from this meeting, we can consider a new whole section like the proposal-1. On proposal 2, we don’t think it is needed. On proposal 3, we support it. MAC-CE based PL-RS switching can be reused in 8.14, but DCI based PL-RS is new anyhow in a new sub-section. On proposal 4, we don’t think it is needed. Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as Basically, we support option-1. We need specific wording anyhow. Nokia proposal-3a takes the wording from RAN1 agreement. We propose to refer to CR R1-2112949. In our understanding, the delay is not such simple like just Y-symbols in other options. Nokia contribution R4-2200649 has some interpretation with Figure 1. Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as We support option-3. Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as We support Option-1. Issue 1-2-4 Define MAC-CE based TCI state-pair indication requirement We support option-1. Issue 1-3-1 Define TCI state switching delay (associated with different PCI) as We support option-3, option-5 and option-6, option-7. We modify to : Apply the existing [serivng-cell TCI] switch delay and spatial relation switch delay requirements for intra-frequency cell(s) of which SSB for target TCI is associated with the serving cell PCI or with PCI different from the serving cell. Support option-8 as well in addition with non-serving cell support. Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI Support option-2. Issue 1-3-3 The wording of “non-serving cell” Support option-1. Or consider ‘a cell with PCI different from a serving cell’. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Firstly, RAN4 makes conclusion on issue Issue 1-2-1. Then follow RAN1 agreement. Agreement in RAN1 #106bis : On Rel-17 DCI-based beam indication, regarding application time of the beam indication for CA, the first slot and the Y symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. Issue 1-4-2 Known condition in CA scenario Need more time to understand UE capability ‘beamManagementSSB-CSI-RS’ Issue 1-5-1 Scenarios for defining PL-RS switching requirement We are ok with option-2, but visit this issue after known conditions is discussed. Issue 1-5-2 Scenarios for defining PL-RS switching requirement On option-1, FFS. Support option-1a. Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements Support Option-1 re-using the UL TCI state switching requirements as PL-RS switching requirements. Issue 1-5-4 Define DCI based PL-RS switching delay as We support Option 1. |
| MediaTek | Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as Support option 1 to follow the similar structure as legacy Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as More discussion for option 2 we are ok to FR2. But, for FR1, FFS for TL1-RSRP which is not applicable to FR1 Besides, the associated PL-RS could be maintained or non-maintained Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as More discussion for option 2a. For option 2a, for unknown case, we have concerning if there is no L1-RSRP "report" which is not included in TL1-RSRP. Because network may not know the proper DL TCI state for UE so that UE may not be able to receive the DL signals after the switch delay. Besides, for FR1, FFS for TL1-RSRP which is not applicable to FR1. Option 1 is unclear to us, what is the exact requirement? We can compromise to option 1 if the requirements are as following - DL TCI state with known condition: THARQ + 3ms + TOk*(Tfirst-SSB + TSSB-proc) - DL TCI state with unknown condition: THARQ + 3ms + TL1-RSRP + TOk*(Tfirst-SSB + TSSB-proc) - UL TCI state with known condition: THARQ + 3ms + NM*( Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) - UL TCI state with unknown condition: THARQ + 3ms + TL1-RSRP + NM*( Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) Issue 1-2-4 Define MAC-CE based TCI state-pair indication requirement Support option 1. The requirement should be the same as separate UL/DL TCI state switch Issue 1-3-1 Define TCI state switching delay (associated with different PCI) as For option 1, we agree most items except for (5) Time for Active BWP switch. To our understanding, there is no dedicated BWP configuration for NSC. UE should follow the same BWP configuration as serving cell to receive the signals from NSC. The evidance is provided as follows for reference (RAN #92e, ): - Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management for intra-cell and inter-cell scenarios to support higher UE speed and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - For inter-cell beam management, a UE can transmit to or receive from only a single cell (i.e. serving cell does not change when beam selection is done). This includes L1-only measurement/reporting (i.e. no L3 impact) and beam indication associated with cell(s) with any Physical Cell ID(s). For option 2, to our understanding, it is not needed for known non-serving cell. For option 3, it seems similar to option 7. To our understanding, UE should do L1-RSRP measurement after the L3 measurement report has been transmitted for NSC. Otherwise, UE may not know the exact timing of the SSB from NSC. Thus, we suggest RAN4 to consider option 7 as baseline to define the requirement. For option 4, similar comment as option 1. There is no BWP configuration for NSC. Besides, should we consider the timing issue in TCI state switch? To our understanding, the reason we need to consider the timing issue is because UE is required to receive the signals from both SC and NSC at a same time. But, it seems no such UE behavior (receive two signals from different cell at a time) in TCI state switch requirement. For option 5, more discussion is needed. Suggest to follow the same switch delay requirement of unified TCI state of serving cell. For option 6, agreeable with clarification in option 3 and 7. For option 7, support. Please find our comment in option 3. For option 8, support. For known condition of NSC, to our understanding, the legacy can be reuse due to no essential difference. For option 9, support. Basically, we agree with this proposal but we would like to add a note that only define the requirement for unknown non-serving cell. Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI Support option 2. As our comment in Issue 1-3-1, the BWP configuration of non-serving cell will follow serving cell. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Support option 2. To our understanding, one beam application time may be applied for CCs as following RAN1 agreement. Thus, one delay requirement for CA case should be defined in TS 38.133. Agreement in RAN1 #107e Refine the following agreement as follows: Agreement On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the UE can assume that one beam application time (BAT) for a given SCS is configured for all the CCs configured with the common TCI state ID update, - Note: It was agreed that the BAT associated with the carrier(s) (hence BWP(s)/CC(s)) on which the beam indication applies is determined based on the carrier with the smallest SCS among the carrier(s) (hence BWP(s)/CC(s)) applying the beam indication - TBD (maintenance): whether a second configured BAT is also supported, e.g. for MPUE or inter-cell BM. Issue 1-4-2 Known condition in CA scenario More discussion is needed. Issue 1-5-1 Scenarios for defining PL-RS switching requirement Support option 3 For option 4 proposed by intel, we can wait for the RAN2 decision for the definition of beam alignment. Issue 1-5-2 Scenarios for defining PL-RS switching requirement Support option 1 and 1a. To our understanding, the PL-RS in TCI state selected from RRC list by MAC CE command may be non-maintained. Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements One question for clarification. What is the exact requirement for option 1? Issue 1-5-4 Define DCI based PL-RS switching delay as Support option 1. To our understanding, RAN1 does not have such discussion. Because, for DCI based switch UL transmission, the spatial relation and PL-RS should be complete at the same time. Otherwise, the UL transmission will be failed. |
| Qualcomm | Sub-topic 1-1: For Proposal 1, for x.x.x.2 it should be “known” not “know” condition Proposal 3 also makes sense Sub-topic 1-2: Issue 1-2-1 We support Proposal 1, RAN4 has to align to RAN1. WE do not think having signaling for when to switch is useful. Issue 1-2-2: Support Option 3. what is the argument to change the requirements? Issue 1-2-3: To proponents of Option 1, how to apply the requirements? sequentially? Issue 1-2-4: Option 1 Sub-topic 1-3: Issue 1-3-3: Option 1 |
| Ericsson | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay We have slightly different view. We could have one section for Joint TCI state switching. Two sections for separate TCI state switching. For example, 8.10.15 Joint TCI state switching delay for unified TCI 8.10.15.1 Introduction 8.10.15.2 Know condition for joint TCI state 8.10.15.3 MAC-CE based joint TCI state switch delay 8.10.15.4 DCI based joint TCI state switch delay 8.10.15.5 Active joint TCI state list update delay 8.10.16 Active downlink TCI state switching delay under separate TCI state switching in unified TCI 8.10.15.1 Introduction 8.10.15.2 Know condition for downlink TCI state 8.10.15.3 MAC-CE based downlink TCI state switch delay 8.10.15.4 DCI based downlink TCI state switch delay 8.10.15.5 Active downlink TCI state list update delay 8.10.17 Active uplink TCI state switching delay under separate TCI state switching in unified TCI 8.10.16.1 Introduction 8.10.16.2 Know condition for uplink TCI state 8.10.16.3 MAC-CE based uplink TCI state switch delay 8.10.16.4 DCI based uplink TCI state switch delay 8.10.16.5 Active uplink TCI state list update delay. Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as We agree with Apple and Vivo that Option 1 and 4 are same and can be merged. Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as May be first question to answer before discussing the delay requirement is component of UL TCI state switch contains. That means at the end of UL TCI state switch, shall UE be ready with only the spatial relation info for PUCCH or SRS transmission or it needs to compute the power for PUCCH transmission also. Our understanding is UE needs to obtain both. Our view is, once the principle is agreed, we can discuss the requirement in second round. Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as I think our proposal is not option 1. It is close to option 2b. UL spatial relation info is determined using beam correspondance and since it is same DL-RS used for both DL and UL TCI state, and same actviation command, we do not see why additional delay than DL TCI state switching delay is required for DL and UL TCI state swithcing using joint TCI state switch. Issue 1-2-4 Define MAC-CE based TCI state-pair indication requirement We do not understand the issue properly. Can proponents please clarify. Is the issue talking about TCI state update delay requirement? If so, issue name/title is confusing. Issue 1-3-1 Define TCI state switching delay (associated with different PCI) as We support option 3 and we are fine to look at option 2 also. Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI We are open for further discussion whether TCI state switch involves BWP switch. Issue 1-3-3 The wording of “non-serving cell” There are several terminologies used. Serving cell, additional serving cell, non-serving cell, serving cell with different PCI. It is better to come to common understanding. Our understanding is serving cell and additional serving cell/serving cell with different PCI are the TRPs to which UE is connected for data transmission. Non-serving cell are the neighbouring cells/TRPs whom UE monitors for potential TRP change. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Not sure of the scenario. Is it for TCI state switching performed in parallel on different CC? This may need further discussion on when the TCI state is received at UE. Issue 1-4-2 Known condition in CA scenario Can be FFS till the scenario is clarified. Issue 1-5-1 Scenarios for defining PL-RS switching requirement May be a clarification question to Vivo or MTK. What does identical mean in option 1? - Option 1: To define the PL-RS switching delay requirement only when PL-RS is identical to the spatial relation RS in the UL TCI. (MTK, vivo) Issue 1-5-2 Whether the PL-RS maintained in unified TCI framework May be a clarification question to MTK. Is it correct understanding that PL-RS associated with active TCI state list is always assumed to be maintained at UE? If it is not correct understanding, how does network know if the PL-RS is maintained or not so that proper method can be used for switching, that means DCI vs MAC-CE? Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements Legacy requirements can be used as baseline. Details can be FFS. Issue 1-5-4 Define DCI based PL-RS switching delay as DCI based DL switch requirements can be reused? |
| Samsung | Sub-topic 1-1 For specification structure, we think proposal 1 is workable with following clarifications to address the concerns raised as well as other proposals - For joint TCI, i.e., regardless of whether independent requirements will be defined, joint TCI requirements can be specified in proposal 1 as a special case for DL TCI and UL TCI by adding , e.g., in DL TCI switching delay section “if uplink TCI state switching is active in joint TCI, the switching delay for active downlink TCI state switching delay is xxxxxx” - Even if no further update for DL TCI switching requirements, identical requirements can be still referred (or copy & paste) in next section, The benefit of introducing new section for DL TCI switching is readability of specification assuming Rel-17 UE is supposed to work in Rel-16 network in which unified TCI is not there. In such case, existing section is applied. - PL-RS switching delay requirements (if any) can be also specified in UL TCI switching section as special case. - Following Rel-16 approach, no need to have dedicated section for TCI list update delay For whether such new section shall be added as sub-section of existing section or independent section from existing ones, no major difference is identified. Considering readability, independent section is better approach. Overall, we think no clear drawback is identified for proposal 1 which can be agreed as baseline for preparing the CRs in the next RAN4 meeting. Issue 1-2-1 We think option 3&4 is detailed approach of option 1. Given that, we think option 3a can be used as starting point. For option 5, we agreed requirements for CA shall be specified and option 5a is also agreeable. Issue 1-2-2 For option 3, existing requirements cover both known and unknown TCI case can be reused. The only case needs further discussion is for unknown case, whether the additional delay proponent for PL-RS update, i.e., NM* 5*Ttarget_PL-RS + 2msNR slot length shall be considered for overall uplink switching delay or not. In our understanding, if we can assume PL-RS in Rel-17 unified TCI framework can be always maintained, i.e., NM is always 0, existing requirements can be applied for unknown case without considering additional delay proponent for target PL-RS measurement Issue 1-2-3 It is not clear for us, whether UE has to wait the additional delay to complete DL TCI switching, i.e., UE can start receiving PDCCH in target TCI without waiting for additional delay for uplink TCI switching (TL1-RSRP ). Maybe the remaining issue is about the test case design in such case but at least from core requirements perspective, option 1 shall be agreed as baseline. We can further discuss the test case design in performance part. Issue 1-2-4 TCI state-pair in included in the TCI list. Update pair delay requirements shall be as same as TCI list update delay which is identical as DL TCI switching delay. Sub topic 1-3 delay requirements for different PCI cell For delay requirements associated with different cell ID, first of all, we agree non-serving cell is not correct. We can go with either option 2 or Nokia’s new proposal “ a cell with PCI different from a serving cell” For delay requirements, we prefer to have same requirements for both serving cell and different PCI cell but known condition for different PCI cell shall be well specified as in Intel’s comments. To remove the additional timing tracking delay for cell with different cell ID, known condition can be specified as timing alignment between serving cell and different PCI cell. Similar considering can be applied for BWP switching delay for different PCI cell, i.e., different PCI cell shall be within the active BWP of serving cell. Otherwise (timing and BWP is not aligned with serving cell), the different PCI cell is unknown, for unknown case, additional delay for timing tracking and BWP switching can be considered. For unknown case, other alternative is not to specify the requirements for such case, ,i.e., TCI switching to different PCI cell shall be only applied in the case that timing and BWP of different PCI cell are aligned with serving cell. For topic 1-4 Delay requirement for CA, we agree with vivo i.e., option 1 in both issue 1-4-1 and 1-4-2 For topic 1-5 requirements for PL-RS update, as we commented in issue 1-2-2, we think as long as PL-RS can be assumed as always maintained in unified TCI framework, no matter of including PL-RS update in uplink TCI update, uniform requirements can be applied, i.e., no additional delay requirements for PL-RS update in Rel-17 unified TCI framework |
| ZTE | Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as Support Option 4 and Option 5a. It should be noted that RAN1 has reached latest agreement in 107 meeting as follows: - Agreement in RAN1 #107 - On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the UE can assume that one beam application time (BAT) for a given SCS is configured for all the CCs configured with the common TCI state ID update, - Note: It was agreed that the BAT associated with the carrier(s) (hence BWP(s)/CC(s)) on which the beam indication applies is determined based on the carrier with the smallest SCS among the carrier(s) (hence BWP(s)/CC(s)) applying the beam indication - The detailed signalling of the BAT is up to RAN2 - FFS: For CC(s) not configured with a common TCI state ID update. So we believe Option 4 is the most direct formula aligned with the conclusion in RAN1. For the reference SCS, Option 5a align with the above RAN1 agreement. Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as Support Option 3. Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as Support Option 1. Issue 1-3-1 Define TCI state switching delay (associated with different PCI) as Support Option 3, 6, 7, 8. We believe Option 3, 6 have same view, and Option 7, 8 have same view. Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI Support Option 2. Issue 1-3-3 The wording of “non-serving cell” Between Option 1 and 2, we do not have strong view. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Support Option 2. We believe the case that the source RS of the target TCI state is transmitted in another CC is also possible in legacy Rel-16 TCI state switch. For the TCI state of QCL type-C/D, the source RS of the target TCI state can be transmitted in other serving cell. Further more, the update of TCI state for a CC group has been supported in Rel-16, so we believe there is not any new characteristic introduced by the unified TCI state structure applying for intra-band CA. Issue 1-4-2 Known condition in CA scenario Whether specify per-CC known status for the common TCI state, further study is needed. Can the proponent interview how to realize the per-CC known status for the case that: a CC/BWP field is configured for the source RS of the target TCI state, then each CC applying the target TCI state would use the beam direction of the source RS transmitted in such CC/BWP as target beam. We concern such case do not align the “per-CC known status”. Issue 1-5-1 Scenarios for defining PL-RS switching requirement Support Option 2 and Option 3. Actually we think they are same view. Issue 1-5-2 Whether the PL-RS maintained in unified TCI framework Support Option 1 and Option 3. For the sake of pathloss measurement robustness, the beam of PL-RS should be wider then other source RS, so the PL-RS switch is not as frequently as UL TCI state switch, and actually UE can always maintain target PL-RS. Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements Support Option 1. Issue 1-5-4 Define DCI based PL-RS switching delay as Support Option 1. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay - Proposals - Option 1: Separate section and take following structure as a baseline (Samsung) 8.15 Active downlink TCI state switching delay for unified TCI 8.15.1 Introduction 8.15.2 Known condition for downlink TCI state 8.15.3 MAC-CE based downlink TCI state switch delay 8.15.4 DCI based downlink TCI state switch delay 8.15.5 Active downlink TCI state list update delay 8.16 Active uplink TCI state switching delay for unified TCI 8.16.1 Introduction 8.16.2 Known condition for uplink TCI state 8.16.3 MAC-CE based uplink TCI state switch delay 8.16.4 DCI based uplink TCI state switch delay 8.16.5 Active uplink TCI state list update delay. - Option 2: No need for downlink TCI, adding joint TCI section, and reuse 8.10.X (Apple) 8.10.X Active uplink TCI state switching delay for unified TCI 8.10.Y Active joint TCI state switching delay for unified TCI. - Option 3: separate TCI state list update section (vivo) 8.10.Z.5 Active joint TCI state list update delay 8.10.Z.6 Active separate TCI state list update delay comprising downlink and uplink TCIs. - Option 4: Also consider new section for DCI-based requirement (Nokia) Recommendations for 2nd round: Based on GTW discussion. |
| Sub-topic #1-2 | Issue 1-2-1 Define DCI based DL and UL TCI state switching delay as - Proposals - Option 1: Simply refer to RAN1 spec (i.e. n+TACK+ TBAT) and further discuss on TP, e.g. - ‘Y symbols’ delay is about beam application time (BAT) that cannot reuse legacy DCI-based TCI state switch delay requirement. Define a switching requirement as RAN1 agreement. (Nokia) - A UE shall be able to receive PDSCH with target TCI state of the serving cell [or non-serving cell] on which TCI state switch occurs at the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH with HARQ-ACK information.. Recommendations for 2nd round: Companies could propose specific Text Proposal for the requirement in 2nd round. Issue 1-2-2 Define MAC-CE based UL TCI state switching delay in separate UL/DL mode as - Proposals - Option 1: for UL TCI state associated with DL-RS as following for both known and unknow TCI state and associated PL-RS is maintained (Apple, Intel): - THARQ + 3ms + NM*(Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) for known TCI - THARQ + 3ms + TL1-RSRP + (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) for unknown TCI - Option 2: Reuse the existing MAC-CE based uplink spatial relation switching delay requirements for known and unknown case (ZTE, vivo, Samsung, Huawei, Nokia, QC) - Option 3: Additional delay component for PL-RS update for known case. For unknown case, i.e., PL-RS or associated RS with spatial relation is unknown, no requirement is applied.(MTK) - Option 4: For known case, if PL-RS is included in UL TCI state, reuse the existing MAC-CE based pathloss reference signal switch delay requirements; If PL-RS is associated with UL or joint TCI, UL TCI switching delay requirements and pathloss reference signal switching delay requirements are specified separately, and existing requirements on UL spatial relation switching and existing requirements on PL-RS can be reused respectively. (CMCC) Recommendations for 2nd round: Based on GTW discussion. Issue 1-2-3 Define MAC-CE based joint UL and DL TCI state switching delay as - Proposals - Option 1: No extra requirement needed for Joint TCI mode, DL and UL requirements can be applicable independently. (Samsung, CMCC, Intel, Huawei, Nokia) - Option 2: Define a total switching delay for joint TCI state switching requirements and define the requirement from the slot switching command is received until UE can receive DL channel or transmit UL channel with target TCI state, whichever is later. (Apple, vivo, MTK) Recommendations for 2nd round: Based on GTW discussion. Issue 1-2-4 Define MAC-CE based TCI state-pair indication requirement - Proposals - Option 1:the TCI state switching delay requirement can be defined for UL TCI and DL TCI switching respectively. (Intel, MTK, Huawei, Nokia, QC) - Option 2a: Update pair delay requirements shall be as same as TCI list update delay which is identical as DL TCI switching delay. - Option 2b: If the TCI state list update include both DL TCIs and UL TCIs, the endpoint of the TCI state switching delay is the later one between DL TCI switching delay and UL TCI switching delay. Recommendations for 2nd round: Continue discussion in the 2nd round. |
| Sub-topic #1-3 | Issue 1-3-1 Define TCI state switching delay for “NSC” as - Proposals - Option 1: Components of delay are (1) MAC-CE decoding or DCI processing time, (2) Time acquisition delay if target TCI was not in active TCI list monitored by UE, (3) Time for RX beam acquisition if target TCI state is unknown, (5) Time for Active BWP switch. (Apple) - Option 2: MAC-CE based and DCI based TCI switching delay does not have difference for a serving cell and NSC (the associated SSB for target TCI is with PCI different from the serving cell). (Nokia, Ericsson, Intel, Huawei,MTK) - Define the requirement only for the NSC is known and further consider the known condition of NSC. Recommendations for 2nd round: Based on GTW discussion. Issue 1-3-2 Whether introduce the interruption requirement due to TCI state switch associated with different PCI - Proposals - Option 1: Needs further discussion in RAN4. Recommendations for 2nd round: Continue collecting views in the 2nd round. Issue 1-3-3 The wording of “non-serving cell” - Proposals - Option 1: TS38.214 uses wording as “SS/PBCH block having a PCI different from the PCI of the serving cell” rather than directly mention ‘a cell’. Or consider ‘a cell with PCI different from a serving cell’. (Nokia) - Option 2: Modify “non-serving cell” to “TRP with different PCI”. (Intel) Recommendations for 2nd round: Continue collecting views in the 2nd round. |
| Sub-topic #1-4 | Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario - Proposals - Option 1: Specify the requirement, i.e. the switching delay between the TCI states whose QCL-D or UL TX filter is determined by a source RS in one of the CCs, while QCL-A or QCL-C is still determined by the RS in each CC. (vivo,Samsung) - Option 2: delay requirement as the TCI state switching for single CC, with the clarification that the first slot to apply the new TCI state is determined on the CC with the smallest SCS among the CCs which applying the beam indication. (MTK, Intel, Huawei, ZTE) Recommendations for 2nd round: Continue discussion in the 2nd round. Issue 1-4-2 Known condition in CA scenario - Proposals - Option 1: Specify per-CC known status for the common TCI state. And if the total number of unknown TCI-CC pairs exceeds the UE capability ‘beamManagementSSB-CSI-RS’, no RRM requirements are defined. (vivo) Recommendations for 2nd round: Continue discussion in the 2nd round. |
| Sub-topic #1-5 | Issue 1-5-1 Scenarios for defining PL-RS switching requirement - Proposals - Option 1: To define the PL-RS switching delay requirement only when PL-RS is identical to the spatial relation RS in the UL TCI and when PL-RS or associated RS with spatial relation is known. (MTK, vivo) - Option 2: PL-RS switching delay requirement apply when PL-RS is identical to the source RS in the UL TCI or PL-RS is associated with UL TCI state where PL-RS and source RS in UL TCI are QCL-typeD. PL-RS switching delay requirement for both known and unknown case can be defined. (Intel) - Option 3: No need to define requirements for PL-RS switching in unified TCI. For PL-RS in UL TCI switch, define requirements for beam alignment case. (Apple) Recommendations for 2nd round: Based on GTW discussion. Issue 1-5-2 Whether the PL-RS maintained in unified TCI framework - Proposals - Option 1: For MAC CE TCI state based, the target PL-RS may be maintained or non-maintained. (MTK, Nokia, Intel) - Option 2: UE can always maintain the target PL-RS for the case of UL TCI state switch triggering PL-RS switch (ZTE, Huawei) Recommendations for 2nd round: Continue discussion in the 2nd round. Issue 1-5-3 Define MAC-CE based PL-RS switching delay requirements - Proposals - Option 1: re-using the UL TCI state switching requirements as PL-RS switching requirements. (ZTE, Samsung, Nokia) if UL TCI state switching can trigger PL-RS switch (Intel) - Option 2: The requirements for UL TCI switching delay and requirements for PL-RS update delay are specified as two separate delay. (vivo, CMCC) Recommendations for 2nd round: Continue discussion in the 2nd round. Considering Issues 1-5-1 Issue 1-5-4 Define DCI based PL-RS switching delay as - Proposals - Option 1: Follow RAN1 agreements |

Discussion on 2nd round (if applicable)

[Moderator] The open issues are list here for your reference. In case of double efforts, companies may choose to make their comments directly on the corresponding WF.

Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay

- Proposals: Take the following structure as baseline (Samsung)

8.15 Active downlink TCI state switching delay for unified TCI 8.15.1 Introduction 8.15.2 Known condition for downlink TCI state 8.15.3 MAC-CE based downlink TCI state switch delay 8.15.4 DCI based downlink TCI state switch delay 8.15.5 Active downlink TCI state list update delay 8.16 Active uplink TCI state switching delay for unified TCI 8.16.1 Introduction 8.16.2 Known condition for uplink TCI state 8.16.3 MAC-CE based uplink TCI state switch delay 8.16.4 DCI based uplink TCI state switch delay 8.16.5 Active uplink TCI state list update delay

  - Option 1: agree on the structure as baseline (Apple, Huawei, Samsung)

  - Option 2: Use 8.10.x instead of 8.15 (Nokia)

  - Option 3: Others

    - Ericsson: we prefer having one section for joint TCI state switching

Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement for known TCI case

- Proposals

  - Option 1: Yes. Known TCI may outside 4 maintained PL RS. (Apple, MTK, Intel, Ericsson, Huawei,ZTE)

  - Option 2: No.

    - Option 2a: NM = 0 if PL-RS is included in UL TCI state (Nokia)

Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4

- Proposals

  - Option 1: two RS are identical

    - Option 1a: FFS whether “two RS are QCL Type-D” can be considered as beam alignment. (Huawei)

  - Option 2: two RS are identical or QCL Type-D. (Apple, Nokia, Intel,ZTE)

  - Option 3: Others

    - MTK: depends on RAN2 (vivo)

Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement

- Proposals

  - Option 1:the TCI state switching delay requirement can be defined for UL TCI and DL TCI switching respectively. (Intel, MTK, Huawei, Nokia, QC, Apple, vivo, ZTE, Samsung)

  - Option 2a: Update pair delay requirements shall be as same as TCI list update delay which is identical as DL TCI switching delay.

  - Option 2b: If the TCI state list update include both DL TCIs and UL TCIs, the endpoint of the TCI state switching delay is the later one between DL TCI switching delay and UL TCI switching delay.

Issue 1-3-1 Define TCI state switching delay for “NSC” as

- Proposals

  - Option 1: Components of delay are (1) MAC-CE decoding or DCI processing time, (2) Time acquisition delay if target TCI was not in active TCI list monitored by UE, (3) Time for RX beam acquisition if target TCI state is unknown, (5) Time for Active BWP switch. (Apple)

    - Option 1a: No Active BWP switch (MTK)

  - Option 2: MAC-CE based and DCI based TCI switching delay does not have difference for a serving cell and NSC (the associated SSB for target TCI is with PCI different from the serving cell). (Nokia, Ericsson, Intel, Huawei,MTK,vivo, ZTE, Samsung)

    - Option 2a: Define the requirement only for the NSC is known and further consider the known condition of NSC.

    - Option 2b: the active BWP of NSC is within active BWP of serving cell and SCS is the same. (Apple)

    - Option 2c: with the condition that timing offset between SC and NSC are within CP (Intel)

Issue 1-3-2 The wording of “non-serving cell”

- Proposals

  - Option 1: TS38.214 uses wording as “SS/PBCH block having a PCI different from the PCI of the serving cell” rather than directly mention ‘a cell’. Or consider ‘a cell with PCI different from a serving cell’. (Nokia, vivo, Samsung)

  - Option 2: Modify “non-serving cell” to “TRP with different PCI”. (Intel)

Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario

- Proposals

  - Option 1: Specify the requirement, i.e. the switching delay between the TCI states whose QCL-D or UL TX filter is determined by a source RS in one of the CCs, while QCL-A or QCL-C is still determined by the RS in each CC. (vivo, Samsung, Huawei)

  - Option 2: delay requirement as the TCI state switching for single CC, with the clarification that the first slot to apply the new TCI state is determined on the CC with the smallest SCS among the CCs which applying the beam indication. (MTK, Intel, Huawei, ZTE, Nokia, ZTE)

  - Option 3: further consider requirement for CA case next meeting (Samsung)

Issue 1-4-2 Known condition in CA scenario

- Proposals

  - Option 1: Specify per-CC known status for the common TCI state. And if the total number of unknown TCI-CC pairs exceeds the UE capability ‘beamManagementSSB-CSI-RS’, no RRM requirements are defined. (vivo)

Issue 1-5-1 Scenarios for defining  PL-RS switching requirement

- Proposals

  - Option 1: To define the PL-RS switching delay requirement only when PL-RS is identical to the spatial relation RS in the UL TCI and when PL-RS or associated RS with spatial relation is known. (MTK, vivo)

  - Option 2: PL-RS switching delay requirement apply when PL-RS is identical to the source RS in the UL TCI or PL-RS is associated with UL TCI state where PL-RS and source RS in UL TCI are QCL-typeD. PL-RS switching delay requirement for both known and unknown case can be defined. (Intel, Nokia)

  - Option 3: No need to define requirements for PL-RS switching in unified TCI. For PL-RS in UL TCI switch, define requirements for beam alignment case. (Apple, vivo)

    - Option 3a: delay component of measuring PL-RS shal be considered for not maintained and unknown TCI case.

Issue 1-5-2 Define MAC-CE based PL-RS switching delay requirements

- Proposals

  - Option 1: re-using the UL TCI state switching requirements as PL-RS switching requirements. (ZTE, Samsung, Nokia, Intel, Ericsson,ZTE)

  - Option 2: The requirements for UL TCI switching delay and requirements for PL-RS update delay are specified as two separate delay. (vivo, CMCC)

| Company | Comments |
| --- | --- |
| Apple | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay Option 1. We are fine with the proposed structure as baseline. We should also have a section for joint TCI state switch in our understanding where we refer to DL and Ul sections for specific delay requirements. Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement Option 1. Yes UE is only required to maintain 4 PL-RS. Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 Beam alignment is defined if either condition are fulfilled: - PL-RS is identical to the RS in UL or joint TCI - PL-RS is QCL Type D to the RS in UL or joint TCI Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Option 1 Issue 1-3-1 Define TCI state switching delay for “NSC” as We can compromise to option 2 but with the condition that the active BWP of NSC is within active BWP of serving cell and SCS is the same. Issue 1-3-2 The wording of “non-serving cell” Either a cell with PCI different from a serving cell or “TRP with different PCI” is fine for us. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario We have a different understanding of common TCI – the TCI ID is shared but not the TCI state, so at the end the switching delay for CA case with common TCI is the switching delay on each CC. We don’t see the need to define requirements for this. Issue 1-5-1 Scenarios for defining PL-RS switching requirement Option 3. PL-RS is part of UL or joint TCI and the switching delay includes it. PL-RS cannot be switched separately. |
| MediaTek | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay No strong view Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement Support option 1. To our understanding, for MAC CE based switch, network will activate one TCI state from RRC list. In RAN1, there is no agreement that the PL-RS in RRC list should be maintained. So, it is possible that the PL-RS may be maintained or non-maintained in MAC CE based TCI state switch Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 Option 3, depending on RAN2 design Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Support option 1, this issue seems similar to joint mode TCI state switch. Issue 1-3-1 Define TCI state switching delay for “NSC” as Option 1 and 2 seems not conflict. We support option 1 (except for BWP switch) and option 2. For option 1, to our understanding, the BWP should be the same as serving cell. Because the scenario we considered is that serving cell is not changed. That means the BWP configuration would not be changed while TCI state is changed. For option 2, the TCI state switch from serving cell to unknown non-serving cell is not a typical case. Suggest to define the TCI state switch requirement only for the non-serving cell is known. The details of known condition can be FFS. Issue 1-3-2 The wording of “non-serving cell” No strong view Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Support option 2. Option 1 and option 2 seems not conflict. We need more time to check option 1. Issue 1-4-2 Known condition in CA scenario We need more time to check this issue. Issue 1-5-1 Scenarios for defining PL-RS switching requirement This issue seems has been discussed in Issue 1-2-1 and Issue 1-2-2 Issue 1-5-2 Define MAC-CE based PL-RS switching delay requirements This issue seems has been discussed in MAC CE based delay requirement. |
| Nokia | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay We prefer option-2 but fine with option-1 up to load of new updates. In addition, how to handle issue 1-5-4 Define DCI based PL-RS switching delay? It falls in 8.14? Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement At least, if PL-RS is included in UL TCI state, NM=0 (option-2). We are open to discuss NM in a case if PL-RS is associated with UL-TCI. Come back in the next meeting. Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 Option-2. (Option-2 includes option-1). Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Option-1 Issue 1-3-1 Define TCI state switching delay for “NSC” as Option-2 Issue 1-3-2 The wording of “non-serving cell” Option-1. Signals indicating a non-serving cell is not directly given to a UE. RAN1/2 specs are rather written based RS specific signals regarding non-serving cell indication. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Option-2 is RAN1 statement, and TCI state switching delay for a CC is high layer signalled. On Option-1, we are not sure how “while QCL-A or QCL-C is still determined by the RS in each CC” impacts on switching delay? Issue 1-4-2 Known condition in CA scenario FFS. We want to come back in the next meeting. Issue 1-5-1 Scenarios for defining PL-RS switching requirement Option-2, and up to Issue 1-2-1 and Issue 1-2-2. Issue 1-5-2 Define MAC-CE based PL-RS switching delay requirements Option-1 and up to Issue 1-2-1 and Issue 1-2-2. |
| Intel | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay Option 1. Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement Option 1. Yes. Since it’s up to UE capability. Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 Option 2. - If PL-RS is included in UL TCI or joint TCI, PL-RS is identical to the source RS in UL or joint TCI - If PL-RS is associated UL TCI or joint TCI, PL-RS and source RS in UL or joint TCI is QCL-Type D. Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Option 1 Issue 1-3-1 Define TCI state switching delay for “NSC” as We are fine to define the same requirement with the condition that timing offset between SC and NSC are within CP. Otherwise, DL timing tracking for UL TCI switching for NSC is still needed for the following purpose if the target TCI is not in the active TCI state list, based on our analysis. - Update NSC UL timing according to NSC DL timing - Tracking DL timing of NSC PL-RS for pathloss calculation Issue 1-3-2 The wording of “non-serving cell” Both are fine. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Option 2. Issue 1-5-1 Scenarios for defining PL-RS switching requirement Option 2. PL-RS requirement for both known and unknown can be defined for beam alignment case. Issue 1-5-2 Define MAC-CE based PL-RS switching delay requirements Option 1. |
| Ericsson | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay Option 3. For joint TCI state switching we define single requirement. Considering that we prefer having one section for joint TCI state switching. Samsung proposal is OK for us for separate TCI state switching requirements. Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement Active TCI state list size (8 I think if I am not wrong) is more than PL-RS maintained size of 4. Hence it is possible gNB may ask UE to switch to PL-RS from the outside of 4 maintained PL-RS. Hence, we think NM can be 1 in Rel-17 for known TCI state switch requirements. Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 Can companies clarify what is identical mean here? Does it mean same RS is used as QCL source? Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Is it the TCI list update delay we are discussing? Looking at option 1 and option 2, they are directed for different issues. Need clarification on the issue. Issue 1-3-1 Define TCI state switching delay for “NSC” as Option 2. We can agree to Apples clarification on the active BWP switch and SCS. Issue 1-3-2 The wording of “non-serving cell” UE can be connected to two TRP. Any clarification on wording for these TRP? UE may need to measure other TRP than connected TRP, what is the wording for these TRP? Non-serving mean the TRP other than connected TRP? Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario SO is this means parallel TCI state switching on two CC? Need clarification on the scenario. Issue 1-4-2 Known condition in CA scenario Can discuss after 1-4-1 is clarified. Issue 1-5-1 Scenarios for defining PL-RS switching requirement We agreed to define requirements for beam alignment case. Option 1 is our understanding as per the agreement. Issue 1-5-2 Define MAC-CE based PL-RS switching delay requirements Option 1. |
| Huawei | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay We support option 1. The existing TCI state switching requirements in section 8.10 are applied for R15/R16 TCI indication. We suggest to introduce new sections for unified TCI indication. Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement Support option 1. For MAC-CE based activation, the associated PL-RS for a known TCI can be not maintained for high layer RSRP measurements. Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 At least “two RS are identical” can be considered as beam alignment. FFS whether “two RS are QCL Type-D” can be considered as beam alignment. Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Support option 1. Issue 1-3-1 Define TCI state switching delay for “NSC” as Support option 2. The known conditions for target TCI needs to be updated with that the associated SSB can be with PCI different from the serving cell. Issue 1-3-2 The wording of “non-serving cell” RAN4 firstly need to study which part of RRM requirements will be impacted. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Option 1 and option 2 are not mutually exclusive. It seems that both options are aligned with RAN1’s agreements. Issue 1-4-2 Known condition in CA scenario FFS the case mentioned in option 1. |
| vivo | Issue 1-1-1 The Spec structures of Unified TCI State Switching Delay Fine with option 1. Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement Option 1. Based on GTW agreements, NM is allowed to be equal to 1 if PL-RS is not maintained. Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 Up to RAN2. However, seems RAN2 will go in the direction of option 2. Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Can compromised to option 1 based on GTW agreements. Issue 1-3-1 Define TCI state switching delay for “NSC” as Fine to option 2 based on current situation. Issue 1-3-2 The wording of “non-serving cell” Fine to option 1. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Support option 1 but OK to consider how to reduce specification work. Issue 1-4-2 Known condition in CA scenario Same comments as issue 1-4-1 Issue 1-5-1 Scenarios for defining PL-RS switching requirement Issue 1-5-2 Define MAC-CE based PL-RS switching delay requirements Can be discussed with UL TCI together. |
| Samsung | Issue 1-1-1 We also think the option 1 could be a starting point. Considering the companies’ comments in 2nd round, maybe we can consider to use option 1 as baseline but also agree additional note as below - Companies are encouraged to bring the CRs in next meeting based on baseline specification structure - For joint TCI state switching requirements, companies which prefer new dedicated section, can also bring CRs in the next meeting for further discussions Issue 1-2-1 It is recognized even PL-RS is included in TCI list, network could still active TCI with PL-RS which is not maintained by UE due to the number of PL-RS in TCI list is beyond UE capability of number of maintained Pl-RS. With such applicability clarification, NW =1 will be specified for TCI switching delay for known TCI case. Issue 1-2-2 Beam alignment is the applicability scenario for specifying the UL TCI switching delay requirements. With this, beam alignment includes both PL-RS is included in TCI list, or PL-RS is QCLed with the RS associated with uplink TCI in the list. We think as WF, RAN4 can further check in the next RAN4 meeting Issue 1-2-3 Option 1 is certainly correct and also confirmed for both separated TCI and joint TCI case. TCI state-pair indication is included in the TCI list. Requirements of updating the list shall be also applied for the state-pair update as stated in option 2a. TCI state list update requirements is as same as DL TCI switching delay as in section 8.10.6. RAN4 can further discuss whether to reuse such requirements for Rel-17 unified TCI Issue 1-3-1 In our understanding, option 2 can be interpreted as RAN4 shall strive to specify the requirements for “NSC” as the same as serving cell with specified known defined for “NSC”. For other cases, if additional delay proponent is required, requirements can be specified for case as unknown “NSC” Issue 1-3-2 Option 1 Issue 1-4-1 We are open to discuss whether to specify the requirements for CA case in RAN4 spec. Issue 1-4-2 It is related to whether RAN4 shall specify the requirements for CA case or not Issue 1-5-1, Issue 1-5-2 In general, we support no specified requirements for PL-RS switching delay in unfied TCI given PL-RS is no longer configured separately from TCI framework As we commented before, delay proponent of measuring PL-RS, i.e., (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) shall be considered for - PL-RS is not maintained for known TCI Unknown TCI case |
| ZTE | Issue 1-2-1 Whether NM is allowed to be equal to 1 in Rel-17 requirement Option 1. Based on GTW agreements, NM is allowed to be equal to 1 if PL-RS is not maintained. Issue 1-2-2 The definition of beam alignment for UL TCI requirement in RAN4 Option 2. Issue 1-2-3 Define MAC-CE based TCI state-pair indication requirement Option 1 based on GTW agreements. Issue 1-3-1 Define TCI state switching delay for “NSC” as Option 2. Issue 1-3-2 The wording of “non-serving cell” Both fine for us. Issue 1-4-1 The requirements for common TCI state switching delay in CA scenario Support option 2. Issue 1-4-2 Known condition in CA scenario More clarification is needed. Can the proponent interview how to realize the per-CC known status for the case that: a CC/BWP field is configured for the source RS of the target TCI state, then each CC applying the target TCI state would use the beam direction of the source RS transmitted in such CC/BWP as target beam. We concern such case do not align the “per-CC known status”. Issue 1-5-2 Define MAC-CE based PL-RS switching delay requirements Support option 1. |

Topic #2: Inter-cell beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200183 | MediaTek Inc | Proposal 1: UE is not required to perform the L1-RSRP measurement on the unknown non-serving cell. Proposal 2: For the measurement behaviour for non-serving cell in FR1, L1 measurements for non-serving cell can be performed within or outside SMTC. Proposal 3: For the measurement behaviour for non-serving cell in FR2, L1 measurements for non-serving cell are assumed to be performed outside SMTC. Proposal 4: For the unknown non-serving cell, no delay requirement of the L1-RSRP measurement will be defined. Proposal 5: For the known non-serving cell, the existing R15/R16 delay requirement of the L1-RSRP measurement can be reused. Proposal 6: For inter-cell beam management/multiple TRP, to define the requirement with the condition of the timing offset between serving cell and non-serving cell is within one CP. Proposal 7: For inter-cell beam management/multiple TRP, to take one FFT as assumption to define the L1-RSRP measurement requirement. Proposal 8: For inter-cell beam management/multiple TRP, UE Rx beam assumptions for L1 and L3 measurement are same as legacy, i.e., fine beam and rough beam will be applied for L1 and L3 measurement, respectively. |
| R4-2200278 | Apple Inc. | L1-RSRP Measurements for inter-cell beam management Proposal #1: The L1-RSRP measurement requirements on cell with different PCI are applicable to inter-cell beam management and inter-cell multi-TRP. Proposal #2: Define known cell condition for L1-RSRP as In FR1 the cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds. In FR2 the cell is known if, during the last 5 seconds – - the UE has sent a valid measurement report for the cell with different PCI and - One of the SSBs measured from the cell with different PCI being configured remains detectable Otherwise, the cell is unknown. Proposal #3: Define SSB based L1-RSRP measurement period for known cell with different PCI similar to existing serving cell requirements. Proposal #4: For unknown cell, the L1-RSRP measurement period is extended by the time needed for intra-frequency cell identification and measurement. Proposal #5: Define Measurement restriction on SSB based L1-RSRP measurements for cell with different PCI, if the SSB from cell with different PCI is on the same OFDM symbol as SSB or CSI-RS from serving cell for other L1 measurements. Proposal #6: Define scheduling availability for UE performing L1-RSRP measurement on cell with different PCI. Proposal #7: RAN4 defines requirements for Nmax=1 only. Timing offset and FFT assumption Observation #1: Inter-cell beam management and inter-cell multi-TRP framework are under the assumption that UE receives signals from serving and cell with different PCI with single FFT with timing offset assumption < CP. Observation #2: Inter-cell multi-TRP assumes dedicated and non-dedicated signals to be received from serving and cell with different PCI with single FFT and with timing offset < CP. Observation #3: For inter-cell beam management, if timing offset > CP is allowed additional scheduling restriction is needed for L1-RSRP measurement and dedicated channel reception from cell with different PCI. Proposal #8: L1-RSRP measurements for cell with different PCI should be restricted to single FFT operation and timing offset assumption within CP. Rx beam and Measurement behavior assumption Observation #4: UE knows the SSB time location of cells with different PCI configured for L1-RSRP measurement. Observation #5: If L1-RSRP measurement is restricted to SMTC the measurement period would be much longer. Observation #6: UE is expected to use fine beam for L1-RSRP measurement on cells with different PCI as it is for beam management and beam indication. Proposal #9: Do not restrict L1-RSRP measurement on cells with different PCI to SMTC. Proposal #10: Define requirements for L1-RSRP measurements on cell with different PCI with similar assumptions (for e.g., RX beam) as L1 measurements on serving cell. Proposal #11: Further discuss reply LS to RAN1. |
| R4-2200594 | ZTE Corporation | Proposal 1: Not re-use the existing known/unknown condition of Handover, enhance a new known/unknown condition based on L1 measurement for NSC. Proposal 2: For MAC CE based TCI state switch for NSC, re-use existing MAC CE based requirements. Proposal 3: For DCI based TCI sate switch for NSC, re-use the corresponding requirements for serving cell. Proposal 4: In order to guarantee sufficient flexibility for L1 SSB configuration of NSC as similar as serving cell, which should not be limited by the configuration of SMTC and MG. Proposal 5: Re-using the existing collision handling in legacy Rel-16 to resolve the possible collision between L1 SSB used for NSC measurement and SMTC/MG. Proposal 6: For inter-cell BM, since one FFT applied for both serving cell and NSC is preferred, so if simultaneous RX with such different QCL sources is needed, then the timing offset requirement is needed so as to guarantee the timing offset between serving cell and NSC is within CP. |
| R4-2200603 | vivo | Proposal 1 R17 RAN4 RRM requirements for inter-cell BM should be forward compatible to R18 L1/L2-centric mobility. Proposal 2 Based on recent progress, it is suggested not to restrict the assumption of timing difference between the serving cell and a cell with different PCI to be less than CP. Proposal 3 Inter-cell L1-RSRP measurements can be used by network in making decisions on whether to activate the TCI associated to the cell with different PCI, and therefore it is not necessary to restrict assumptions that UE only perform L1 measurements on the known cells. Proposal 4 For inter-cell L1-RSRP measurement requirements, including measurement restrictions and scheduling restrictions, FR1 and FR2 are discussed separately assuming different UE behaviour, similar to the case of R15/16 intra-cell L1-RSRP measurements. Proposal 5 Except cell identification and measurements performed in SMTC, single FFT assumption should be the baseline UE behaviour assumption in specifying R17 inter-cell L1-RSRP measurement requirements. Proposal 6 RAN4 only works for SSB-based inter-cell L1-RSRP measurement requirements in R17. Proposal 7 For FR1, before a TCI state associated to the cell with different PCI is activated, for single-FFT UE, UE is only required to perform inter-cell L1-RSRP measurements within SMTC. Observation 1 In R15/16, for UEs capable of tracking more than 1 TCIs, the timing difference between tracked TCIs is assumed to be less than CP. Proposal 8 After a TCI state associated to the cell with different PCI is activated, for UEs capable of tracking more than 1 TCIs, a baseline UE is only able to track TCIs with timing difference less than CP, and RAN4 can further discuss whether to specify UE capability for those who are able to track TCIs with timing difference larger than CP. Observation 2 If a UE is only able to track TCIs with timing difference less than CP, but the actual timing difference between active TCIs from different cells is larger than CP, UE performance degradation can be expected, except the L3/L1 RSRP measurement performance within SMTCs. Proposal 9 RAN4 separates the discussion on inter-cell L1-RSRP measurement requirements into 2 cases, - Case I: the L1-RSRP measurements performed before a TCI associated to the cell with different PCI is activated - Case II: the L1-RSRP measurements performed after a TCI associated to the cell with different PCI is activated Observation 3 For FR1, if SSB-based inter-cell L1 measurements are performed within SMTC, scheduling restrictions defined for L3 measurements can be re-used, and no need to define measurement restrictions for such L1 measurements. Proposal 10 For FR1, the inter-cell L1-RSRP measurement requirements assuming UE only performs L1 measurements within SMTCs are at least specified in R17 feMIMO WI, i.e. L1 measurements within SMTCs are 1st priority over other cases of UE measurement behaviour assumption. Observation 4 For FR1, if SSB-based inter-cell L1 measurements are performed outside SMTCs, more scheduling restrictions and measurement restrictions outside SMTCs can be expected, if - network cannot ensure that timing difference from different cells is less than CP, and - SSB has a different subcarrier spacing than PDSCH/PDCCH/CSI-RS, and - UE does not support ‘simultaneousRxDataSSB-DiffNumerology’. Observation 5 For FR1, if SSB-based L1 measurements are performed outside SMTCs, RAN4 may need to further specify applicability of core/performance requirements, or scheduling/measurement restrictions, if - network cannot ensure that timing difference from different cells is less than CP, and - one of the following conditions are met - Condition 1: - SSB has a different subcarrier spacing than PDSCH/PDCCH/CSI-RS, and - UE does not support ‘simultaneousRxDataSSB-DiffNumerology’. - Condition 2: - SSB has the same subcarrier spacing as PDSCH/PDCCH/CSI-RS, and - UE is only capable of Single-FFT Observation 6 According to RAN1 LS, RRM measurement requirements are not impacted by L1-RSRP measurements on RSs with a PCI different from serving cell. Proposal 12 For FR2, before a TCI state associated to the cell with different PCI is activated, UE is only required to perform inter-cell L1-RSRP measurements within SMTC assuming the same set of Rx beams for L3 measurements is re-used, and requirements are only applicable if ‘timeRestrictionForChannelMeasurement’ is not configured. Proposal 13 For FR2, after a TCI state associated to the cell with different PCI is activated, - if the SSB for the cell with different PCIs are not fully overlapped with SMTC, the SSB-based inter-cell measurement share the same requirements as the legacy L1-RSRP measurement for one serving cell. - RAN4 can further discuss whether to allow larger beam sweeping factor if serving cell SSB and the SSB for the cell with different PCIs are fully overlapped in time domain. - If larger beam sweeping factor is allowed for the case when serving cell SSB and the SSB for the cell with different PCIs are fully overlapped in time domain, then UE is required to prioritize L1-RSRP measurements to the original serving cell for the case when serving cell SSBs and the SSBs for the cell with different PCIs are partially overlapped in time domain - if the SSB for the cell with different PCIs are fully overlapped with SMTC, inter-cell L1-RSRP measurements requirements are specified assuming the same set of Rx beams for L3 measurements is re-used, and requirements are only applicable if ‘timeRestrictionForChannelMeasurement’ is not configured. Proposal 14 Clarify the understanding of beam sweep factor N from RAN4 perspective, for SSB-based L1-RSRP measurements, as following in the reply LS, ‘UE is only required to meet the L1-RSRP measurement accuracy requirements after N samples.’ |
| R4-2200643 | CMCC | Proposal 1: for the requirements on number of cells (the number of PCIs different from the serving cell for measurement/reporting), it is not preferred to only consider Nmax = 1. Proposal 2: for the definition on requirements on number of cells (the number of PCIs different from the serving cell for measurement/reporting), it is proposed to refer to the Nmax, which was agreed in RAN1. Proposal 3: for FR1 L1-RSRP measurement of non-serving cell, it is proposed to follow the principle in Rel-15/16 that the FR1 L1-RSRP measurement of non-serving cell can be performed within SMTC or outside SMTC. And L3 and L1 measurements can be performed at the same time Proposal 4: for FR2 L1-RSRP measurement of non-serving cell, it is proposed to follow the principle in Rel-15/16 that the FR2 L1-RSRP measurement of non-serving cell is performed outside SMTC. Observation 1: according to RAN1 agreements, Nmax is up to UE capability and the candidate value of Nmax is {1, 2, 3, 7}. Observation 2: for the requirements on number of cells, only consider Nmax = 1 cannot guarantee the performance of UE which is capable of Nmax > 1. Observation 3: For FR1 L1-RSRP measurement of serving cell, the location of configured SSB resource for L1-RSRP can be within SMTC or outside SMTC. |
| R4-2200650 | Nokia, Nokia Shanghai Bell | Known cell condition for L1 measurement Proposal 1 : For L1-RSRP measurement on non-serving cell, a UE has sent at least L3-RSRP report within a given time and acquired initial sync by detecting PSSS from the non-serving cell. Proposal 2 : a UE should be configured with association information of the non-serving cell PCI and SSB to support L1 RSRP measurement on non-serving cell. Proposal 3 : SSB of a non-serving cell has the same center frequency, SCS, SFN offset in the same active BWP with a serving cell in Rel-17. Also, SSB time domain position can be also known. - If SSB time domain position is not given, a UE can search the non-serving cell using the SSB association information. Proposal 4: Assuming L3-RSRP on non-serving cell has been measured and reported to a serving cell before L1-RSRP measurement trial on non-serving cell, the serving cell can evaluate if quality of signal is good enough to measure L1-RSRP based on L3-RSRP value. ( FFS on a threshold value). Timing offset assumptions between non-serving cell and serving cell Proposal 5 : If a UE is not support simultaneous RX and if the RX timing difference between non-serving cell and serving cell is assumed within CP length, there is no strong motivation to introduce the limitation of known conditions of non-serving cell. Measurement accuracy & period for serving cell L1-RSRP Proposal 6 : If a UE does not support simultaneous RX, measurement accuracy for serving cell L1-RSRP can be reused for L1-RSRP measurement for non-serving cell. Proposal 7 : If a UE does not support simultaneous RX, and if SSB from serving cell and non-serving overlap over time, the UE should be able to sequentially measure L1-RSRP on SSB from serving and from non-serving cell. - The current measurement period for L1-RSRP can be scaled by a scaling factor. - FFS : How to apply a scaling factor (including considering inside or outside of SMTC window discussion) RX scheduling restriction due to SSB overlapping Proposal 8 : If a UE does not support simultaneous RX and if SSB from a cell and other DL signal from another cell overlap in time, RAN4 studies to set RX scheduling restriction of a UE. - Overlap between L1 measurement on a cell and another cell - Overlap between L1 measurement on a cell and L3 measurement on another cell - Overlap between L1 measurement on a cell and PDCCH/PDSCH reception from another cell Proposal 9: Assuming RX scheduling restriction and measurement conditions proposed above, SMTC window is not essentially required for L1-RSRP measurement on non-serving cell. - Also, a UE is not mandated to measure L1-RSRP measurement outside of SMTC window. |
| R4-2200788 | Intel Corporation | Observation 1: There is no conclusion about timing offset assumption between serving cell and TRP with different PCI in RAN1. Observation 2: For FR2, if different RX beams are assumed for L1 and L3 measurement of neighbor cell, more accurate beam quality measurement can be achieved and shorter measurement delay is expected for inter-cell beam measurement. However, measurement delay for L1 serving cell will be impacted and extended. Observation 3: For FR2, if same RX beam is assumed for L1 and L3 measurement of neighbor cell, L1 serving cell measurement will not be delayed. However, beam quality measurement may not be accurate and measurement delay may be longer for inter-cell beam measurement. Proposal 1: Both SSBs inside and outside SMTC can be used for L1-RSRP measurement for inter-cell. RX beam sweeping factor can be further reduced. Proposal 2: If L1-RSRP measurement for inter-cell is performed inside SMTC, it will use the same RX beam of L3 measurement. If L1-RSRP measurement of neighbor cell is performed outside SMTC, different RX can be used. Observation 4: SSB location, index and PCI information will be provided by high layer. Proposal 3: For inter-cell L1-RSRP measurement, single shot is considered as the baseline for defining measurement period and don’t consider timeRestrictionForChannelMeasurement. Proposal 4: For FR2, if SSBs outside SMTC and MG are used for inter-cell L1-RSRP measurement. It will be shared with L1 measurements of serving cell. A sharing factor X is further introduced on top of P factor. Proposal 5: For FR2, when SMTC and SSB for inter-cell L1-RSRP are fully overlapped, no requirement for NSC L1-RSRP measurement is expected or clarify that performance degradation is expected. Proposal 6: If a cell is known, the total measurement delay for inter-cell beam measurement will only consider the delay for inter-cell L1-RSRP measurements. Proposal 7: If a cell is unknown, the total measurement delay will consider extra cell searching time and SSB index acquiring time. Proposal 8: If the cell is unknown, one SSB sample can be used for cell search time. Proposal 9: If the cell is unknown, further discuss extra time needed for SSB index acquiring. Proposal 10: if NW configure UE to measure L1-RSRP for multiple TRPs with different PCIs, UE will perform the measurement in sequential method. Proposal 11: The reply LS will depend on the discussion about whether L1-RSRP for inter-cell beam measurement can be performed outside SMTC or not. |
| R4-2201385 | Ericsson | Proposal 1: RAN4 to support L1-RSRP measurement on NMAX+1 TRPs. Proposal 2: When SSB from different PCI overlap, UE should be able to measure them at the same time if following conditions are met. - Received SSB are within the active BWP - Received time difference between the SSB of different PCI is within the CP Proposal 3: RAN4 to agree that the non-serving TRPs, whose timing offset w.r.t reference TRP is less than CP to be measured at the same time. If the timing offset of the non-serving TRPs w.r.t reference TRP are more than CP, they are to be measured in TDM fashion. Where reference TRP refers to serving TRP and additional serving TRP. Proposal 4: RAN4 to agree on the assumption that two independent FFT and RF chain are used for inter-cell beam management. Proposal 5: RAN4 to agree that UE measures all the non-serving cell TRPs whose timing offset is within CP from the reference cell TRP at the same time and other TRPs whose timing offset is more than CP in TDM fashion. Where reference cell TRP is serving TRP or additional serving TRP. Proposal 6: RAN4 to agree that same RX beam can be used for L1-RSRP and L3-RSRP computation for non-serving cell inter-cell beam management in Rel-17. Proposal 7: L1-RSRP measurements for inter-cell beam management to be based on SSB only for non-serving cell inter-cell beam management in Rel-17. Proposal 8: RAN4 to re-use HO and SCell known/unknown definitions for inter-cell beam management. |
| R4-2201617 | Huawei, HiSilicon | Proposal 1: The Rx beam assumption for L1-RSRP measurements based on SSB with PCI different from the serving cell can be same as that for L1-RSRP measurements based on SSB with serving cell PCI. Proposal 2: For the timing offset between serving cell TRP and the TRP with different PCI, it is suggested to reuse R16 timing offset assumption for multi-TRP transmissions, i.e. timing offset within CP length. Proposal 3: For L1-RSRP measurements based on SSB with different PCI, the existing SSB based L1-RSRP measurement requirements in R15/R16 can be reused as baseline. Proposal 4: For SSB based L1-RSRP measurements in R17, it is suggested to introduce the measurement restrictions between SSB with serving cell PCI and SSB with PCI different from the serving cell. Proposal 5: For SSB based L1-RSRP measurements in R17, it is suggested to introduce the measurement restrictions between SSB with serving cell PCI and SSB with PCI different from the serving cell. Proposal 6: It is suggested that the existing cell detectable conditions used in SMTC based intra-frequency measurements can be reused as the known cell condition for non-serving cell SSB configured for L1-RSRP measurements. |
| R4-2201960 | Samsung | Proposal 1: RAN4 agreed the specification structure for L1-RSRP measurement requirements for configured SSB with PCI different from serving cell in RAN4 101bis meeting taking the above suggestions into account Proposal 2: Existing condition for detected intra-frequency cell can be reused for known condition for L1-RSRP for SSBs with different PCI Proposal 3: For unknown SSB with different PCI, RAN4 can further discuss whether to introduce additional delay performance for L1-RSRP. Proposal 4, Existing serving cell L3 measurement behaviour in FR1 and L1 measurement requirements in FR2 can be used as baseline for L1-RSRP measurement for SSB with different PCI fully overlapped with SMTC. Proposal 5: RAN4 can further discuss the case when SSB with different PCI not fully overlapped with SMTC by taking the scheduling restriction, measurement restriction, timing difference assumption and Rx beam assumption into account. |

Open issues summary

Please note that some proposals (issues) might be omitted by purpose as they are deprioritized in the 1st round discussion or out of the scope of [219].

Sub-topic 2-1: Specification Structures for Inter-cell L1-RSRP Measurement

Sub-topic description:

Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement

- Proposals

  - Proposal 1: Spec structure as following (Samsung)

9.12 L1-RSRP measurements for configured SSB with PCI different from serving cell 9.12.1 Introduction 9.12.2 Requirement applicability 9.12.3 Measurement reporting requirements 9.12.4 L1-RSRP measurement requirements 9.12.5 Measurement restriction 9.12.6 Scheduling availability

Sub-topic 2-2: Requirements for inter-cell L1-RSRP measurement

Sub-topic description:

Issue 2-2-1 Applicability rules for inter-cell L1-RSRP measurement requirement

- Proposals

  - Proposal 1: UE is not required to perform the L1-RSRP measurement on the unknown non-serving cell and no delay requirement will be defined. (MTK)

  - Proposal 2: Requirements applicable to inter-cell beam management and inter-cell multi-TRP. (Apple)

  - Proposal 3: Do not restrict UE only perform L1 measurements on the known cells. (vivo)

  - Proposal 4: Separates the discussion for 2 cases: performed [before / after] a TCI associated to the cell with different PCI is activated (vivo)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement

- Proposals

  - Proposal 1: The timing offset between SC and NSC is within one CP and take one FFT as the assumption. (MTK, Apple, vivo, Huawei), and similar assumptions (for e.g., RX beam) as L1 measurements on serving cell. (Apple)

  - Proposal 2: Fine beam and rough beam will be applied for L1 and L3 measurement, respectively. (MTK)

  - Proposal 3: Single shot is considered as the baseline for defining measurement period and don’t consider timeRestrictionForChannelMeasurement. (Intel)

  - Proposal 4: If the timing offset of the non-serving TRPs w.r.t reference TRP is within CP, they are to be measured at the same time. If the timing offset of the non-serving TRPs w.r.t reference TRP are more than CP, they are to be measured in TDM fashion. (Ericsson)

  - Proposal 5: The same RX beam can be used for L1-RSRP and L3-RSRP. (Ericsson)

  - Proposal 6: The same Rx beam assumption for SC and NSC (Huawei)

  - Proposal 7: RAN4 to agree on the assumption that two independent FFT and RF chain are used for inter-cell beam management (Ericsson).

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 2-2-3 Known NSC condition for L1-RSRP measurement

- Proposals

  - Proposal 1: Define known cell condition for L1-RSRP as (Apple)

In FR1 the cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds. In FR2 the cell is known if, during the last 5 seconds – - the UE has sent a valid measurement report for the cell with different PCI and - One of the SSBs measured from the cell with different PCI being configured remains detectable

  - Proposal 2: (Nokia)

For L1-RSRP measurement on non-serving cell, a UE has sent at least L3-RSRP report within a given time and acquired initial sync by detecting PSSS from the non-serving cell. UE should be configured with association information of NSC PCI. SSB of a non-serving cell has the same center frequency, SCS, SFN offset in Rel-17. Also, SSB time domain position can be also known. - If SSB time domain position is not given, a UE can search the non-serving cell using the SSB association information. Assuming L3-RSRP on non-serving cell has been measured and reported to a serving cell before L1-RSRP measurement trial on non-serving cell, the serving cell can evaluate if quality of signal is good enough to measure L1-RSRP based on L3-RSRP value. ( FFS on a threshold value).

  - Proposal 3: Re-use HO and SCell known/unknown definitions(Ericsson)

  - Proposal 4: existing cell detectable conditions used in SMTC based intra-frequency measurements can be reused as the known cell condition (Huawei, Samsung)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement

- Proposals

  - Proposal 1: For the known non-serving cell, the existing R15/R16 delay requirement can be reused. (MTK) as the baseline (Huawei).

  - Proposal 1a: Existing serving cell L3 measurement behaviour in FR1 and L1 measurement requirements in FR2 can be used as baseline within SMTC. (Samsung)

  - Proposal 2: Define SSB based L1-RSRP measurement period for known cell with different PCI similar to existing serving cell requirements. For unknown cell, period is extended by the time needed for intra-frequency cell identification and measurement. RAN4 defines requirements for Nmax=1 only. (Apple)

  - Proposal 3: It is not preferred to only consider Nmax = 1; it can refer to RAN1 agreement. (CMCC)

  - Proposal 4: if SSB from serving cell and non-serving overlap over time, the current measurement period for L1-RSRP can be scaled by a scaling factor. (Nokia)

  - Proposal 5: If a cell is unknown, the total measurement delay will consider extra cell searching time and SSB index acquiring time; one SSB sample can be used for cell search time (Intel)

  - Proposal 6: RAN4 to support L1-RSRP measurement on NMAX+1 TRPs (Ericsson)

  - Proposal 7: For unknown SSB with different PCI, RAN4 can further discuss whether to introduce additional delay performance for L1-RSRP.

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Sub-topic 2-3:  L1-RSRP measurement behaviour for non-serving cell

Sub-topic description:

Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1

- Proposals

  - Proposal 1: Both within and outside SMTC (MTK, Apple, CMCC, Intel)

  - Proposal 2: Before a TCI state associated to the cell with different PCI is activated for single-FFT UE, only required to perform within SMTC. And for R17 within SMTCs are 1st priority. (vivo)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2

- Proposals

  - Proposal 1: Outside SMTC only (MTK, CMCC)

  - Proposal 2: Both within and outside SMTC (Apple) RX beam sweeping factor can be further reduced. (Intel)

  - Proposal 3: The same as FR1 vivo’s proposal assuming the same set of Rx beams for L3 measurements is re-used, and requirements are only applicable if ‘timeRestrictionForChannelMeasurement’ is not configured. (vivo)

  - Proposal 4: Assuming RX scheduling restriction and measurement conditions, SMTC window is not essentially required for L1-RSRP measurement on non-serving cell.

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Issue 2-3-3 When inter-cell L1-RSRP measurement overlapping with other UE behaviours

- Proposals

  - Proposal 1: Define Measurement restriction if the SSB from NSC on the same OFDM symbol from serving cell and define scheduling availability for UE performing L1-RSRP measurement on cell with different PCI. (Apple)

  - Proposal 2: if SSB from a cell and other DL signal from another cell overlap in time, set RX scheduling restriction of a UE. (Nokia)

  - Proposal 3: A sharing factor X is further introduced on top of P factor for FR2 introduced. (Intel)

  - Proposal 4: UE should be able to measure them at the same time if following conditions are met:1 Received SSB are within the active BWP; 2 Received time difference between the SSB of different PCI is within the CP.

  - Proposal 5: introduce the measurement restrictions between SSB with serving cell PCI and SSB with PCI different from the serving cell. (Huawei)

  - Proposal 6: Larger beam sweeping factor if SC and NSC SSBs fully overlapped in time; prioritize SC L1-RSRP measurements if SC and NSC SSBs partially overlapped in time. (vivo)

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Sub-topic 2-4:  Reply on RAN1 LS R1-2112762

Sub-topic description:

Issue 2-4-1 Reply on RAN1 LS L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap

- Proposals

  - Proposal 1: Discuss the proposed reply LS (Apple)

For FR1, when the SSB from cell with different PCI for L1-RSRP measurement is in the same OFDM symbol as SSB from serving cell for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure SSB from both serving and cell with different PCI without any restriction. For FR2, when the SSB from cell with different PCI for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS or SSB from serving cell for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. Similar measurement restriction will apply if SSBs from one than one cell with different PCI overlaps in the same symbols in FR2. For FR2 if the UE is capable of simultaneous reception with different QCL Type D, then it can measure SSB from 2 cells simultaneously if they overlap.

  - Proposal 2: Clarify the understanding of beam sweep factor N “UE is only required to meet the L1-RSRP measurement accuracy requirements after N samples”. (vivo)

  - Proposal 3: The reply LS will depend on the discussion about whether L1-RSRP for inter-cell beam measurement can be performed outside SMTC or not. (Intel)

  - Proposal 4 (Ericsson):

  - When SSB from different PCI overlap for L1-RSRP measurements, UE should be able to measure them at the same time if following conditions are met.

    - Received SSB are within the active BWP

    - Received time difference between the SSB of different PCI is within CP

  - Proposal 5 (Nokia)

- RAN4 develops UE requirements for at least Nmax = 1, where Nmax is the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting.

  - RAN4 will further study to specify the requirements for other N_max value taking RAN1 decision into account

- RAN4 is discussing to introduce separate UE requirements of two types of UE reception capabilities and requirements in Rel-17.

- A UE not supporting simultaneous reception from different QCL-type-D sources sequentially measures one SSB and another.

- A UE supporting simultaneous reception from two (or multiple) different QCL-type-D sources can simultaneously measure L1 RSRP on the overlapping SSBs.

    - Due to lack of Rel-17 TU, the Rel-17 requirement study on simultaneous reception continues in Rel-18 WI phase. RAN4 will define requirement applicability to Rel-17 from Rel-18.

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Moderator | From moderator view, the following comments are made to guide the 1st round discussion regarding Topic#2, as proposals are too complicated and diverse for these issues. Sub-topic 2-1: Specification Structures for Inter-cell L1-RSRP Measurement Specification structure is a critical issue for future CR drafting so we’d better agree on a structure ASAP. Sub-topic 2-2: Requirements for inter-cell L1-RSRP measurement The requirement depends on Sub-topic 2-3. Sub-topic 2-3: L1-RSRP measurement behavior for non-serving cell The most critical issue we must determine is the UE behaviour for inter-cell L1-RSRP measurement, i.e. measure L1-RSRP within or outside STMC. Sub-topic 2-4: Reply on RAN1 LS R1-2112762 TBA |
| Intel | Sub-topic 2-1: Specification Structures for Inter-cell L1-RSRP Measurement Issue 2-1-1: Fine with Proposal 1. Sub-topic 2-2: Requirements for inter-cell L1-RSRP measurement Issue 2-2-1: Fine with Proposal 2, 3. Issue 2-2-2: Fine with proposal 1, 2, 3. We want to clarify that the Rx beam assumption for NSC is similar with SC, fine beam will be used. while they may not be the same if SSB for SC and NSC are overlapped. Issue 2-2-3: Generally fine with Proposal 1, 3, 4. The detail wording for known condition can be further discussed. Issue 2-2-4: Support Proposal 4, 5, 7. For proposal 4, We have similar proposal in our paper. our understanding is that SSB configuration for NSC and SC L1-RSRP measurement may be the same since it’s intra-frequency scenario. Similar as intra-frequency L3 measurement, SSB configuration for serving cell and neighbour cell is the same in real deployment. Therefore, when we perform L1-RSRP measurement for NSC outside SMTC, it will fully overlap with L1-RSRP for serving cell, then scaling factor needs to be considered. For proposal 5, for unknown case, if the timing offset is large, then extra delay will be considered. If the timing offset is small than CP, there may be no extra delay. Sub-topic 2-3: L1-RSRP measurement behavior for non-serving cell Issue 2-3-1: Support Proposal 1. Issue 2-3-2: Support Proposal 2. Considering the real deployment that SSB configuration for NSC and SC may be the same for intra-frequency case and different RX beam will be performed for NSC and SC in FR2. SSB outside SMTC will be shared between NSC and SC. Then L1 measurement of SC will be delayed. Therefore, we propose a compromised solution that RX beam sweeping factor will be reduced and some L3 measurement results inside SMTC can be used. Issue 2-3-3: Support Proposal 3. For Proposal 3, we clarify that sharing factor is defined for SSB fully overlapped case, i.e. SSB configuration for NSC and SC are the same. For case that SSB configuration is partially overlapped, we are open to discuss whether sharing factor or measurement restriction is applied. Sub-topic 2-4: Reply on RAN1 LS R1-2112762 Issue 2-4-1: Support Proposal 3. It will depend on the related discussion in sub-topic 2-3. |
| Apple | From moderator view, the following comments are made to guide the 1st round discussion regarding Topic#2, as proposals are too complicated and diverse for these issues. Sub-topic 2-1: Specification Structures for Inter-cell L1-RSRP Measurement We propose the following for Section title: 9.12 Inter-cell L1-RSRP measurements Or 9.12 L1-RSRP measurements on TRP with different PCI from serving cell The sub-section titles are acceptable. Sub-topic 2-2: Requirements for inter-cell L1-RSRP measurement Issue 2-2-1 Applicability rules for inter-cell L1-RSRP measurement requirement We support proposal 2. There was agreement to define known condition in last meeting. We think the requirements for unknown case can also be defined as proposed in our paper. Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement We support proposal 1, 3 With proposal 4 if timing offset is not within a CP, even measurement in TDM fashion would need scheduling restriction. Issue 2-2-3 Known NSC condition for L1-RSRP measurement We support proposal 1. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement We support proposal #1, we also proposed this in our paper for known cell. For unknown cell additional delay can be defined. Sub-topic 2-3: L1-RSRP measurement behavior for non-serving cell Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 We support proposal 1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 We support proposal 2. Issue 2-3-3 When inter-cell L1-RSRP measurement overlapping with other UE behaviours We support proposal 1. But out proposal was to introduce a measurement restriction. Sub-topic 2-4: Reply on RAN1 LS R1-2112762 We propose to discuss this once we have agreement or conclusion on Issue 2-3-3. |
| vivo | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement We prefer to use inter-cell L1-RSRP measurements and it can be sub-section of 9.5, e.g. 9.5.X. The applicability of inter-cell measurement requirements can be captured in 9.5. Issue 2-2-1 Applicability rules for inter-cell L1-RSRP measurement requirement We support option 2, 3, 4. Option 2 is automatically supported by RAN1 agreements. For option 3 and 4, we think L1-RSRP may not necessarily performed on known cells, especially if the L3 measurement behaviour can be re-used. Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement For proposal 1, we are fine to the first sentence if the L1 measurement is performed after TCI state is activated. For proposal 1,2,6, we think the Rx Beam assumption can be different for SC and NSC. Our understanding is that if the RX beam of L3 measurements is used for NSC, i.e the rough beam, more samples are needed to address the accuracy issue. We prefer to define requirements only for the case when ‘timeRestrictionForChannelMeasurement’ is not configured. We also support proposal 5 and disagree with proposal 3. For proposal 4, same view as Apple. TDMed fashion outside SMTC is not necessary in our view, and scheduling restriction is needed. For proposal 7, we think this is related to FR2 simul QCL-D and should not be discussed here. Issue 2-2-3 Known NSC condition for L1-RSRP measurement This issue should be merged with 1-3-1. The known condition or NSC should be the same for L1-RSRP measurement and TCI state switching. Our proposal is aligned with proposal 3, which is captured in 1-3-1. But we are also OK to proposal 1 and 4 in general. The detailed wording can be discussed in the CR phase. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement We support option 1a and we do not think option 1a is sub bullet of option 1. For option 1, we think RAN4 may come back after RAN4 addresses SSB overlapping issue. It is slightly early to re-use legacy requirements. For P2, we are generally OK. We are also OK to P3 and P6. We do not have a strong view on Nmax. For P4, we are also OK, and we think the scaling factor does not have to be an integer. For P5, we need to further check. For P7, we are also OK. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 We do not think these proposals are contradictory. Proposal 1 can be for the case after a TCI state associated to the cell with different PCI is activated. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 We disagree with proposal 1. No need to restrict to the case outside SMTCs, epecially if the L3 RX beams can be re-used. For Proposal 2, RX beam factor can be further studied for the overlapping case. For Proposal 3, we think this is feasible especially if the SSBs for NSC are fully overlapped with SMTC. We suggest RAN4 to conclude this case for FR2 firstly. For the case when SSBs for NSC are not fully overlapped with SMTC, we suggest to come back in the next RAN4 meeting considering diverged views. For Proposal 4, we think SMTC window is of course not essentially required for L1-RSRP measurements. This is the common understanding. However, UE need to perform L1-RSRP measurements within SMTC if SSBs for NSC are fully overlapped with SMTC. Moreover, before the TCI state for NSC has been activated, SSB for NSC can only be measured within SMTC if large timing difference (>= CP) is possible. Issue 2-3-3 When inter-cell L1-RSRP measurement overlapping with other UE behaviours We think other here is precluding SMTC related discussion We support Proposal 6 but also OK to proposal 3,5. Moreover, we think measurements restriction would not be enough and hence our proposal is more aligned with proposal 3. For proposal 1, 2, the scheduling restriction for SC can be re-used for NSC if timing difference is less than CP. For proposal 4, we are also fine. Issue 2-4-1 Reply on RAN1 LS L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap Agree with other companies that we may come back to the LS after we finished - Issue 2-3-1/2-3-2 for the overlapping with SMTC and - Issue 2-3-3 for overlapping with other behaviour outside SMTC. |
| Huawei | Sub-topic 2-1: Specification Structures for Inter-cell L1-RSRP Measurement Issue 2-1-1: we are OK with Proposal 1. Sub-topic 2-2: Requirements for inter-cell L1-RSRP measurement Issue 2-2-1: we are OK with Proposal 1. For Proposal 2, need to further study how to impact the L1-RSRP requirements. Issue 2-2-2: we are OK with Proposal 1 and Proposal 2. Proposal 6 indicates the same view as Proposal 2. Issue 2-2-3: we are OK with Proposal 1 and Proposal 4. Based on our proposal, rough beam is assumed for L3 measurements and fine beam is assumed for L1 measurements. Only After UE has identified the cell with different PCI and obtained rough beam information, then UE can further performs L1-RSRP measurements on the SSB with different PCI for beam refinement. Issue 2-2-4: we are OK with Proposal 1. FFS the case where SSB with different PCI configured for L1-RSRP measurement is overlapped with SSB with serving PCI configured for L1 measurements. RAN4 further discussed whether to introduce sharing factor or to apply measurement restrictions. Sub-topic 2-3: L1-RSRP measurement behavior for non-serving cell Issue 2-3-1: we are fine with Proposal 1. Issue 2-3-2: generally we can agree with Proposal 1. The principles of defining scaling factor P can be followed when SSB with different PCI is overlapped with SMTC. Issue 2-3-2: we are fine with Proposal 1 and Proposal 5, to define measurement restrictions. Sub-topic 2-4: Reply on RAN1 LS R1-2112762 It relies on the discussion on sub-topic 2-3. |
| CMCC | Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement For Proposal 1, we have one question for clarification. We would like to know whether these restrictions on timing offset and FFT in P1 are only applied when measurements are performed outside SMTC? In our understanding, these restrictions on timing offset and FFT are not needed when measurements are performed within SMTC. If so, it is better to clarify that. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement For delay requirements, we are open to discussion. We would like to provide clarification for Proposal 3. Except delay requirements, according to existing spec, we also have the requirements on number of cells and number of SSB (TS 38.133, 9.2.3). We are wonder whether we need to specify the similar requirements for inter-cell L1-RSRP measurement. If companies see the necessity to have the requirements, then only consider consider Nmax = 1 is not preferred. It is suggested to refer to the Nmax, which was agreed in RAN1. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Proposal 1. According to existing requirements, for FR1, there is no limitation on the location of configured SSB resource for L1-RSRP of serving cell, which means both within SMTC and outside SMTC are OK. Similar approach can be reused for L1-RSRP measurement of non-serving cell for FR1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Proposal 1. According to existing requirements, the L1-RSRP measurement of serving cell can only be performed on the occasions which are not overlapped with SMTC and MG. The basic assumption is that UE can only measure one direction at a time. Taking this assumption into account, for FR2 L1-RSRP measurement of non-serving cell, to avoid the impact to other measurement, it is proposed to follow the principle in Rel-15/16 that the FR2 L1-RSRP measurement of non-serving cell can be performed outside SMTC. |
| Nokia | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement Proposal 1 : It is early to foresee the structure yet. RAN4 may need to make a new section as the proposal. However, it is hard to agree the title. The title ‘L1-RSRP measurements for configured SSB with PCI different from serving cell’ gives impression that the requirements are separate between serving and non-serving cell, that we don’t know yet. We propose to revisit in 2nd round. Issue 2-2-1 Applicability rules for inter-cell L1-RSRP measurement requirement Proposal 1 : We don’t know yet what is known cell and what is unknown cell yet. In principle, we are ok that UE is not required to perform the L1-RSRP measurement on the unknown non-serving cell with certain ‘conditions’. Proposal 2 : Support. Proposal 3 : We don’t know yet conditions what is known cell and what is unknown cell. Proposal 4 : It is not clear what is changed before/after a TCI associated to the cell. Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement Proposal 1: A single FFT RX assumption is critical, when a UE simultaneously receives RSs from SC and NSC. If a UE separately process them, ‘the timing offset between SC and NSC is within one CP and take one FFT’ should be considered as UE measurement conditions? Proposal 2,6 : We are ok as baseline assumption at least for a UE not supporting simultaneous RX. Proposal 3 is FFS. Proposal 4 : support. Proposal 5,7 : we keep this RX beam assumption for further study, (especially related with simultaneous RX.) Issue 2-2-3 Known NSC condition for L1-RSRP measurement We support proposal 1,2,4. Regarding proposal 2, our point is that the conditions need to state about RRC configuration of non-serving cell. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement Proposal 1 : It is up to how to measure including SMTC and how to handle SSB overlapping. Proposal 3,6 : FFS. Proposal 4 : We support it for a UE not supporting simultaneous RX. Proposal 5,7 : Non-serving cell should be detected first with known conditions first. “searching” case on unknown non-serving cell in the proposal is FFS. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Support proposal 1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 RAN4 discusses more on measurements both within and outside SMTC in Proposal 2 with proposal 4. We think Proposal 2 and proposal 4 are possible. Issue 2-3-3 When inter-cell L1-RSRP measurement overlapping with other UE behaviours We proposed both requirement studies for UE not supporting simultaneous RX and for UE supporting it. - A UE not supporting simultaneous reception from different QCL-type-D sources sequentially measures one SSB and another. - A UE supporting simultaneous reception from two (or multiple) different QCL-type-D sources can simultaneously measure L1 RSRP on the overlapping SSBs. For the UE not supporting simultaneous reception, we are ok to set UE RX restriction in proposal 1,2, 5. Details are FFS. Issue 2-4-1 Reply on RAN1 LS L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap Support proposal 5. The simultaneous RX support in Rel-17 should be addressed in the LS reply. |
| MediaTek | Issue 2-2-1 Applicability rules for inter-cell L1-RSRP measurement requirement Support proposal 1 and 2. To our understanding, the L3 measurement can help UE to get the timing information from non-serving cell. Besides, it can also help UE to filter which SSB from non-serving cell is detectable. Otherwise, it would be a huge burden if UE is required to measure all SSBs from non-serving cell because fine beam will be used for L1-RSRP measurement on non-serving cell and the beam coverage is smaller than rough beam which is used for L3 measurement. In addition, in general, L3 measurement will be used to search the neighboring cell, regularly. Thus, UE will not have extra cost to do such work. One more important thing to be emphasis is if we allowed UE to perform L1 measurement on unknown non-serving cell, the L1 measurement accuracy requirement should be defined because, in existing requirement, there is no L1 measurement accuracy requirement for unknown cell. Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement Agree with proposal 1, 2 and 6 Disagree with proposal 4,5,7 FFS: proposal 3 For proposal 1 and 7. It is the first time to discuss the requirement for non-serving cell. Thus, we should define the requirement based on minimum UE requirement, i.e., only one FFT is applied. Based on one FFT assumption, the timing assumption should be within one CP. Otherwise, the performance degradation will happen. For proposal 2, 5 and 6, to our understanding, the Rx beam assumption for serving cell should be reused for non-serving cell. For proposal 3, because this is the first time to discuss. We would like to check it and discuss it in next meeting Disagree with proposal 4, one fundamental question is how does network know the timing offset is larger than one CP or within one CP? Issue 2-2-3 Known NSC condition for L1-RSRP measurement Support proposal 2. As our comment in Issue 2-2-1. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement Agree with proposal 4 Disagree with proposal 3 and 6 FFS proposal 1, 1a, 2, 5, 7. For proposal 1 and 7, we suggest to keep if FFS until the timing offset has a conclusion. Proposal 1a, more discussion is needed Proposal 2, more discussion on whether to introduce the L1-RSRP measurement on unknown non-serving cell is needed. Proposal 3 and 6, prefer to define the requirement based on minimum requirement, i.e., consider Nmax =1 in R17. Proposal 4, in general, the scaling factor may need For proposal 5, more discussion is needed. Does that mean UE is required to perform L1 measurement on unknown non-serving cell and the extra cell searching time and SSB index acquiring time will be considered in each L1 measurement? To our understanding, for unknown non-serving cell, UE is only required to do the L3 measurement and no L1-RSRP measurement. Based on this framework, the most requirement can be reused for non-serving cell if the timing offset is within one CP. Otherwise, RAN4 needs to define the L1-RSRP measurement accuracy requirement for unknown non-serving cell. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Support proposal 1. To follow the same L1 measurement behavior as serving cell Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Support proposal 1. To follow the same L1 measurement behavior as serving cell. For non-serving cell, the beam type should rough and fine for L3 and L1 measurement, respectively. We are wondering whether the same beam can be used to measure L1 and L3 within SMTC in FR2? It is complicated to discuss the scenario while some SSBs within SMTC (sharing between L1 and L3) while some SSBs outside SMTC (sharing between serving L1 and non-serving L1). Issue 2-3-3 When inter-cell L1-RSRP measurement overlapping with other UE behaviours Agree with Proposal 1, 2 and 5. FFS: proposal 3, 4 and 6 For proposal 1, 2 and 5, to our understanding, the measurement restriction and scheduling restriction is needed. The details can be FFS. For proposal 3 and 6, more clarification for this proposal is needed. For proposal 4, not sure whether to consider the BWP configuration for non-serving cell as our comment in the issue 1-3-1 in unified TCI state. |
| Qualcomm | Sub-topic 2-2: Issue 2-2-1: support proposal 1. Issue 2-2-2: Support proposal 1. Issue 2-2-3: Support proposal 3 Issue 2-2-4: Support proposal 1. many of the proposal here are not disjoint though. Sub-topic 2-3: Issue 2-3-1: For proposal 1, is the UE aware of where reference signals are transmitted outside the SMTC for NSC? Issue 2-3-2: For proposal 1, is the UE aware of where reference signals are transmitted outside the SMTC for NSC? Issue 2-3-3: Proposal 4. We should avoid introducing any new sharing factor as this will further delay all other measurements also and could impact mobility. Issue 2-4-1: Proposal 4. |
| Ericsson | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement We are ok with proposal 1. Issue 2-2-1 Applicability rules for inter-cell L1-RSRP measurement requirement We support option 2, 3. Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement We think RAN4 shall define the requirements in forward compatible manner. e.g., to be able to enhance the introduced requirements for Rel-18 L1/L2 mobility. Considering this we think measurement behaviour shall take L3 measurement behaviour as baseline. We also think SC L1-RSRP and NSC L1-RSRP are for different purpose. NSC L1-RSRP primarily used for non-serving cell selection (similar to L3 mobility where L3-RSRP is used for target cell selection). With this we do not think same behaviour of SC L1-RSRP can be reused for NSC L1-RSRP. Our view for different proposals is provided below. We can agree to first part of proposal 1, “the timing offset between SC and NSC is within one CP and take one FFT as the assumption”, provided the following can be agreed “Reference timing for L1-RSRP measurement may be from the any one of the two SC (UE can connect to two serving cells)” and the two SC may be more than CP apart. We do not agree with proposal 2 and 6. We should consider L3 measurement behaviour as baseline. Hence L1 can also be rough beam as L3 and can use only SSB in Rel-17. With this assumption we support proposal 5. For proposal 3, accuracy requirement may need to be further studied based on the RX beam assumption. Proposal 4, I am not sure the meaning of TDM fashion we were referring is properly conveyed by us. Our assumption for the proposal is taking two reference timings, as UE can be connected to two SC (let us say the two SC are SC0, SC1. Where SC0 is the SC and SC1 is the additional SC with different PCI). If the two SC, SC0 and SC1 are within CP, UE can measure the TRPs SC0, SC1 and other TRPs who are within the CP length based on the reference time of serving cell (SC0); and If two SC (i.e., SC0 and SC1) are not within CP, then at each measurement instance, UE shall consider measuring the group of TRPs who are within the CP of reference TRP (SC0 or SC1) timing. Our reference to TDM fashion is explained below by considering two reference timings (SC0 and SC1). - At instance 1, UE measures with reference timing of SC0 (SC0 and other TRP within CP of SC0) - At instance 2 UE measures with reference timing of SC1 (SC1 and other TRP within CP of SC1). Our reference to TDM fashion is explained above. Issue 2-2-3 Known NSC condition for L1-RSRP measurement We support proposal 1 and 3. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement We support proposals 3 and 6. We do not agree with proposal 2. This is similar to L3 mobility measurements with 1 neighbour cell. Which do not work as per our understanding. We agree with first part of proposal 1a i.e., for FR1. For FR2, L3 measurement behaviour can be assumed. Proposal 4: We think as long as they are with in active BWP and within CP, they can be measured simultaneously similar to L3 measurements. Proposal 5 and 7 we need to further check. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Support proposal 1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 - We support first part of proposal 2, that is both within and outside SMTC Issue 2-4-1 Reply on RAN1 LS L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap We agree with other companies that we can comeback after other issues are finalised. |
| Samsung | Sub topic 2-1 For specification structure, we support our proposal but open to discuss the title as “inter-cell L1-RSRP measurement”. To Nokia, even with dedicated section, it does not preclude the same requirements for serving cell and non-serving cell. No big difference between new section or new sub-section. Sub topic 2-2 First of all, single FFT assumption shall be definitely maintained as we assumed for other features in Rel-17 as well as other legacy release. For timing alignment (within CP) can be treated as one of pre-condition as known non-serving cell. With these, our understanding for sub issues are Sub issue 2-2-1 If known condition is well defined including timing alignment assumption, we are ok to only specify the requirements for known non-serving cell Sub issue 2-2-2 We support proposal 1. Rx beam assumption for non-serving cell L1-RSRP shall be aligned with assumption we made for serving cell measurement of L1-RSRP for both FR1 and FR2 . To be noted, simultaneous reception of QCL type D RS is suggested to be precluded as stated in other our papers. Sub issue 2-2-3 Known condition shall include - RAN1 agreements for non-serving cell, i.e., same center frequency, SCS, SFN offset - Cell detectable condition (existing intra-frequency measurement can be reused) - Timing alignment assumption (within CP) Issue 2-2-4 In general, we support proposal 1 and 2 For Nmax, it is better to clarify whether the delay requirements shall be scaled by number of TRP measured. Based on common understanding (if any), RAN4 can further discuss whether to specify the requirements for other Nmax except 1. Sub topic 2-3 Issue 2-3-1 For FR1, following the UE measurement behaviour for L1 measurement and L3 measurement, requirements can be specified for withing SMTC and outside SMTC, i.e., we support proposal 1 Issue 2-3-2 & 2-3-3 Overall, we agree with Nokia, the discussion shall follow the decision of core requirements for UE supporting simultaneous reception in Rel-17. RAN4 requirements shall be specified at least for UE CANNOT support simultaneous receptions (UE capability can be used to differential this case). In such case, measurement restriction, scheduling restriction shall be specified. Also, sharing factor for delay requirements shall be considered between SC and NSC measurement assuming SC and NSC shall be measured in TDM manner. For UE support simultaneous reception, unless RAN4 agree to define the full set of requirements including RRM, RF and Demod, L1 measurement for NSC shall not be specified. As commented by some companies, since non-serving cell SSB fully overlapped with SMTC shall be allowed from specifications perspective, the requirements for within SMTC has to be defined and key issue is how to determine the sharing factor for SC and NSC measurement within SMTC For outside SMTC measurement, RAN4 shall also further discuss the requirements to allow the deployment flexibility of configuring NSC SSB. Sub topic 2-4 UE capability of simultaneous reception shall be considered in the reply LS as stated in proposal 5. |
| ZTE | Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement Support Proposal 1 and Proposal 4. For Proposal 4, we believe it is not conflict with Proposal 1, it just additionally specify the case when the time offset can not be guaranteed within CP. Issue 2-2-3 Known NSC condition for L1-RSRP measurement Support Proposal 1 and Proposal 4. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement Support Proposal 1, Proposal 3 and Proposal 4. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Support Proposal 1. In FR1, based on legacy Rel-16, UE can perofrm the L1-RSRP measurement for SC and RRM measurement simultaneously, so of course support Proposal 1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Support Proposal 2. In order to guarantee sufficient flexibility for L1 SSB configuration of NSC as similar as serving cell, which should not be limited by the configuration of SMTC and MG. Re-using the existing collision handling in legacy Rel-16 to resolve the possible collision between L1 SSB used for NSC measurement and SMTC/MG. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement - Proposals - Option 1: Spec structure as following (Samsung) 9.12 L1-RSRP measurements for configured SSB with PCI different from serving cell 9.12.1 Introduction 9.12.2 Requirement applicability 9.12.3 Measurement reporting requirements 9.12.4 L1-RSRP measurement requirements 9.12.5 Measurement restriction 9.12.6 Scheduling availability. Recommendations for 2nd round: Based on GTW discussion. |
| Sub-topic #2-2 | Issue 2-2-1 Applicability rules for inter-cell L1-RSRP measurement requirement - Proposals - Option 1: UE is not required to perform the L1-RSRP measurement on the unknown non-serving cell and no delay requirement will be defined. (MTK) - Option 1a: and for further consider the known condition - Option 2: Do not restrict UE only perform L1 measurements on the known cells. (vivo) Recommendations for 2nd round: Based on GTW discussion. Issue 2-2-2 Assumptions for defining inter-cell L1-RSRP measurement requirement - Proposals - Option 1: The timing offset between SC and NSC is within one CP and take one FFT as the assumption. (MTK, Apple, vivo, Huawei) - Option 2: If the timing offset of the non-serving TRPs w.r.t reference TRP are more than CP, they are to be measured in TDM fashion. (Ericsson) - Option 3: The same Rx beam assumption for SC and NSC (Huawei) - Option 4: The same RX beam can be used for L1-RSRP and L3-RSRP. (Ericsson) Recommendations for 2nd round: Continue discussion in the 2nd round. Issue 2-2-3 Known NSC condition for L1-RSRP measurement - Proposals - Option 1: Known condition shall include - RAN1 agreements for non-serving cell, i.e., same center frequency, SCS, SFN offset - Cell detectable condition (existing intra-frequency measurement can be reused) - Timing alignment assumption (within CP) - Option 2: Others Recommendations for 2nd round: Continue discussion in the 2nd round. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement - Proposals - Option 1: For the known non-serving cell, the existing R15/R16 delay requirement can be reused (MTK) as the baseline (Huawei). - Option 2: Others Recommendations for 2nd round: Views do not converge. Continue discussion in the 2nd round. Depends on the Issue 2-2-1 and 2-3-2. |
| Sub-topic #2-3 | Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 - Proposals - Option 1: Both within and outside SMTC (MTK, Apple, CMCC, Intel) - Option 2: Others Recommendations for 2nd round: Continue discussion in the 2nd round. Vivo may need further clarify their proposal. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 - Proposals - Option 1: Outside SMTC only (MTK, CMCC, Huawei) - Option 1a: and introduce scheduling availability. - Option 2: Both within and outside SMTC (Apple, Intel, Nokia, Ericsson, ZTE) - Proposal 2a: and determine the sharing factor for SC and NSC measurement within SMTC. Recommendations for 2nd round: Based on GTW discussion. Issue 2-3-3 When inter-cell L1-RSRP measurement overlapping with other UE behaviours Recommendations for 2nd round: Views do not converge. Continue discussion in the 2nd round. Depends on the Issue 2-3-2. |
| Sub-topic 2-4 | Issue 2-4-1 Reply on RAN1 LS L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap Recommendations for 2nd round: Continue discussion on the LS draft in the 2nd round |

Discussion on 2nd round (if applicable)

[Moderator] The open issues are list here for your reference. In case of double efforts, companies may choose to make their comments directly on the corresponding WF.

Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement

- Proposals: (Samsung)

9.12 L1-RSRP measurements for configured SSB with PCI different from serving cell 9.12.1 Introduction 9.12.2 Requirement applicability 9.12.3 Measurement reporting requirements 9.12.4 L1-RSRP measurement requirements 9.12.5 Measurement restriction 9.12.6 Scheduling availability

  - Option 1: agree on the proposed structure (Samsung)

    - Option1a (Apple): L1-RSRP measurements for cell with PCI different from serving cell

  - Option 2: Others

Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement

- Proposals

  - Option 1: The timing offset between SC and NSC is within one CP and take one FFT as the assumption. (MTK, Apple, vivo, Huawei, Intel, ZTE, Samsung)

    - Option 1a: Option 1 only for outside SMTC case (CMCC, vivo)

  - Option 2: If the timing offset of the non-serving TRPs w.r.t reference TRP are more than CP, they are to be measured in TDM fashion. (Ericsson, Nokia,ZTE)

  - Option 3: The same Rx beam assumption for SC and NSC (Huawei, Apple, MTK, Intel, Samsung), i.e. rough beam for L3 and fine beam for L1

    - Option3a: Option 3 + Option 2 (Nokia)

  - Option 4: The same RX beam can be used for L1-RSRP and L3-RSRP. (Ericsson)

  - Option 5: Option 3 for outside SMTC and option 4 for within SMTC

Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement

- Proposals

  - Option 1: UE is not required to perform the L1-RSRP measurement on the unknown non-serving cell and no delay requirement will be defined. (MTK, Huawei)

    - Option 1a: and for further consider the known condition

  - Option 2: Do not restrict UE only perform L1 measurements on the known cells. (vivo, Apple, Nokia, Intel, Ericsson,ZTE, Samsung)

Issue 2-2-3 Known NSC condition for L1-RSRP measurement

- Proposals

  - Option 1: Known condition shall include (Intel, Huawei, Samsung,ZTE)

    - RAN1 agreements for non-serving cell, i.e., same center frequency, SCS, SFN offset

    - Cell detectable condition (existing intra-frequency measurement can be reused)

    - Timing alignment assumption (within CP)

  - Option1a (Apple): + known cell for Handover requirements

  - Option 1b (Ericsson): Timing alignment assumption (within CP from the reference TRP)

  - Option 2: Others conditions

Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement

- Proposals

  - Option 1: For the known non-serving cell, the existing R15/R16 delay requirement can be reused (MTK) as the baseline (Huawei, Apple,ZTE, Samsung).

    - Option 1a (Apple): For unknown cell L1-RSRP measurement is extended by the time needed for intra-frequency cell identification and measurement.

  - Option 2: Others proposals

    - Nokia: update requirement up to Assumptions.

    - Intel: if SSB for SC and NSC are fully overlapped, sharing factor will be considered. It’s FFS whether RX beam sweeping number can be reduced.

Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1

- Proposals

  - Option 1: Both within and outside SMTC (MTK, Apple, CMCC, Intel, Nokia, CMCC, Ericsson, Huawei, vivo,ZTE, Samsung)

  - Option 2: Others

Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2

- Proposals

  - Option 1: Outside SMTC only (MTK, CMCC, Huawei)

    - Option 1a: and introduce scheduling availability.

  - Option 2: Both within and outside SMTC (Apple, Intel, Nokia, Ericsson, ZTE, Nokia, CMCC, vivo, ZTE)

    - Proposal 2a: and determine the sharing factor for SC and NSC measurement within SMTC. (MTK, Huawei, Samsung) sharing factor are applied for NSC SSB L1-RSRP measurements and SMTC measurements

| Company | Comments |
| --- | --- |
| Apple | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement We are fine with spec structure. We propose the following title: 9.12 L1-RSRP measurements for cell with PCI different from serving cell Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement Option 1. Since the inter-cell L1-RSRP are for both inter-cell BM and inter-cell mTRP. We also support Option 3 Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement Option 2. We can easily define L1-RSRP measurement requirements for unknown cell. Issue 2-2-3 Known NSC condition for L1-RSRP measurement We are fine with option 1 and also adding the conditions similar to known cell for Handover requirements as we proposed in our paper: In FR1 the cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds. In FR2 the cell is known if, during the last 5 seconds – - the UE has sent a valid measurement report for the cell with different PCI and - One of the SSBs measured from the cell with different PCI being configured remains detectable Otherwise, the cell is unknown. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement For known cell option 1 For unknown cell L1-RSRP measurement is extended by the time needed for intra-frequency cell identification and measurement. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Option 1. This is same as serving cell L1-RSRP requirements Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Option 2. This is same as serving cell L1-RSRP requirements |
| MediaTek | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement No strong view Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement Support option 1 and 3. For option 1, if the CP is larger than one CP, the extra UE implementation is needed. Besides, to our understanding, in R16 eMIMO and 17 feMIMO, one CP assumption has been concluded in RAN1. Although, it has a note that RAN4 may have own assumption. But, our suggestion is to follow the RAN1’s assumption to avoid ambiguous. Conclusion The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2. - Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.. Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement This issue is depending on what is the definition of known condition. We suggest to keep it FFS in this meeting. Issue 2-2-3 Known NSC condition for L1-RSRP measurement For the known and unknown condition, it would be better to align the discussion in RAN4 first. To our understanding, there are two possible flows: We would like to know which case is discussed now in RAN4? Note: To our understanding, regardless of case 1 or 2, the TPSS/SSS_sync_intra and TSSB_time_index_intra are needed. Case1: UE will use L3 measurement to measure the non-serving cell first and the target SSB will be detected to the exact SFN and frame boundary can be obtained (Even though information of SFN and frame boundary will be configured by NW, but, considering the asynchronous case, UE still needs to perform PSS/SSS detection and SSB time index procedures to find the exact timing). After the L3 measurement report, network will know which non-serving cell’s SSB is good to UE. The L1 measurement for non-serving cell having detectable SSB will be configured for UE to perform the L1 measurement. Figure 1. Illustration of the L1-RSRP measurement with L3 measurement report. Case 2: Initially, the L1 measurement configuration will be provided for UE (For this case, network may not know which SSB is good to UE or not, i.e., UE may need to measure all configured SSBs). Similar to Case 1, before UE performs L1 measurement, the PSS/SSS detetion and SSB time index procedures should be performed first for obtaining the exact timing information, e.g., SFN and SSB index. Figure 2. Illustration of the L1-RSRP measurement without L3 measurement report. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement We are ok to take existing R15/R16 delay requirement as baseline. Besides, we also think this issue is depending on the issue of timing assumption. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Support option 1. To follow the legacy requirement Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 We can compromise to option 2 to consider the case when all SSB from non-serving cell are overlapped with SMTC, i.e., the sharing factor should be introduced for non-serving cell’s L1 and L3 measurement. |
| Nokia | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement The proposal is fine. Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement We support option-2. We are ok with option-3 at least for option-2 case (measured in TDM fashion.) option-4 : included FFS if option-4 is allowed for other UE implementations. We want to hear more from UE venders if a single L3 beam can support simultaneous RX. Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement Option-2 Issue 2-2-3 Known NSC condition for L1-RSRP measurement We can agree to the first bullet as RAN1 agreed. The second and third bullets should be more specific. For L1 measurement on non-serving cell, SSB needs to be read to acquire sync, but it is different from saying L3 is measured. ( MTK discussion point is valid to be considered FFS.) Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement Option-2; the legacy requirement will be updated up to Issue 2-2-1. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Option-1 Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Option 2 |
| Intel | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement Option 1. Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement Option 1 and option 3. Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement Option 2. Issue 2-2-3 Known NSC condition for L1-RSRP measurement Fine with option 1. The question also related to timing offset assumption, if it’s agreed that timing offset is smaller than CP, it seems that at least cell search time can be skipped. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement We need to consider the case that SSB for SC and NSC are fully overlapped case, it’s likely to be applied in real deployment for intra-frequency case. then TDM manner will be performed. Sharing factor will be considered. Besides, It’s FFS whether RX beam sweeping number can be reduced to minimize the delay impact to serving cell L1 measurement. Therefore, we propose: Option 2: if SSB for SC and NSC are fully overlapped, sharing factor will be considered. It’s FFS whether RX beam sweeping number can be reduced. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Option 1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Option 2. From our understanding, we don’t need to limit UE implementation. It’s possible that measurement in SMTC can also provide some help even through it’s rough beam. |
| CMCC | Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement For option 1, we are OK with the restrictions on timing offset and FFT only for the case that measurement is performed outside SMTC. For the case that measurement is performed within SMTC, option 1 is not OK for us. It is not necessary to have these restrictions, since in existing spec, for the L3 SSB based measurement within SMTC, we do not have such restriction. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Option 1. Same as legacy L1 measurement of serving cell. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Can compromise to option 2. |
| Ericsson | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement Option 1 is OK. Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement We support option 2. Difference between option 1 and 2 is if the time difference between two reference TRP are more than CP length, FFT time correction is applied when they are measuring in TDM fashion. We are fine to discuss some scheduling restrictions to support this. UE could be configured to measure NMax number of TRP. Since the scenario is similar to mobility (where UE measures neighbour cells), we think we could reuse the same Rx beam assumption for L1 and L3 measurements. Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement Option 2. Based on L3 measurement framework, unknown cells can be measured. Issue 2-2-3 Known NSC condition for L1-RSRP measurement NSC is considered known for L1-RSRP measurement if UE can know the time and frequency within certain error. Option 1 is fine but with slight modification to third bullet. - Option 1: Known condition shall include - RAN1 agreements for non-serving cell, i.e., same center frequency, SCS, SFN offset - Cell detectable condition (existing intra-frequency measurement can be reused) - Timing alignment assumption (within CP from the reference TRP) Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement Pending on the Rx beam assumptions Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 We are fine with Option 1 (Both within and outside SMTC) Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 We support option 2. |
| Huawei | Issue 2-1-1 Specification Structures for Inter-cell L1-RSRP Measurement Support option 1. Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement Support option 1 and option 3. For option 3, both SC and NSC assume rough beam for L3 measurements and fine beam for L1 measurements. Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement Support option 1. Issue 2-2-3 Known NSC condition for L1-RSRP measurement Generally we can agree with option 1. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement Support option 1. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Support option 1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 We can agree with option 2 and share same view as MTK. When NSC SSB occasions are fully overlapped with SMTC occasions, the sharing factor are applied for NSC SSB L1-RSRP measurements and SMTC measurements. |
| vivo | Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement For option1, same view as CMCC. This should be only for the case outside SMTC. For option 2, we think if timing difference is large, TDMed measurement is not preferred since it is not efficient, especially if the # of cell to be measured is large. For option 3, we think it is for the case outside SMTC. For option 4, we think it is for the case within SMTC. For Nokia’s question, it is related Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement Prefer option 2. Issue 2-2-3 Known NSC condition for L1-RSRP measurement Can be FFS for all the 3 bullets. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement FFS. The SSB measurement occasion and SSB overlapping issue need to be addressed firstly. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Option 1 Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Option 2. For option 2a, need to address Rx beam assumption firstly. |
| Samsung | Issue 2-1-1 Option 1. We are also open to discuss the title as suggested by Apple Issue 2-2-1 L1-RSRP measurement for NSC requirements can be specified based on option 1. If timing offset is larger than CP, requirements is not applied. We also support option 3. Issue 2-2-2 Requirements can be specified for unknown cell. RAN4 shall further discuss the unknown condition Issue 2-2-3 We support option 1 Issue 2-2-4 We support option 1 Issue 2-3-1 We support option 1 Issue 2-3-2 We support option 2 and 2a |
| ZTE | Issue 2-2-1 Assumptions for defining inter-cell L1-RSRP measurement requirement Prefer Option 1 and 2. Issue 2-2-2 Applicability rules for inter-cell L1-RSRP measurement requirement Prefer option 2. Issue 2-2-3 Known NSC condition for L1-RSRP measurement Generally fine with Option 1. Issue 2-2-4 Define delay requirement for inter-cell L1-RSRP measurement Prefer option 1. Issue 2-3-1 Where to perform inter-cell L1-RSRP measurement from NSC in FR1 Prefer Option 1. Issue 2-3-2 Where to perform inter-cell L1-RSRP measurement from NSC in FR2 Prefer Option 2. |

Topic #3: Other RRM requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200184 | MediaTek Inc. | Proposal 1: For TRP specific BFR, for both FR1 and FR2, to introduce a new scaling factor PTRP to extend the evaluation period of BFD and CBD for multiple TRPs. Proposal 2: RAN4 should further check the need of the QCL definition for UL TCI state. Observation 1: In RF session, simultaneous reception channel/RS with different QCL type-D in R17 is being discussed. Proposal 3: In R17, no RRM requirements are specified for simultaneous reception channel/RS with different QCL type D. Moderator note: Simultaneous reception should be moved to AI 6.19.1 for discussion |
| R4-2200279 | Apple Inc. | QCL Definition Observation #1: Currently the definition of QCL is only applicable to PDSCH and PDCCH. Proposal #2: Update TCI chain definition to include SRS and for PUCCH/PUSCH. TRP specific BFR Observation #2: In FR2 if UE doesn’t support simultaneous reception with multi-panel and BFD-RS / CBD-RS from TRPs are not TDMed, BFD and CBD requirements need to be enhanced. Proposal #3: Introduce sharing factor for BFD and CBD evaluation period in FR2 when BFD-RS or CBD-RS are received simultaneously from 2 TRP. BFD for HST Enhancements Proposal #4: For a CORESET with two activated TCI states, two RS indexes are included in q0 (as two BFD RS resources) for implicit BFD. |
| R4-2200535 | Intel Corporation | Proposal 1: For Rel-17, TCI chain needs to be defined for DL/UL TCI state separately. Proposal 2: For DL TCI chain, SSB associated with a different PCID is added as a source RS. Proposal 3: DL TCI chain can apply for separate DL TCI state or joint TCI state switching. Proposal 4: For UL TCI chain, SSB, SRS or CSI-RS will be considered as source RS in the UL TCI chain. the number of Reference Signals in the chain is no more than 4. Proposal 5: For TRP-specific BFD, delay requirement is defined assuming that UE will perform BFD in TDM manner. Proposal 6: For TRP-specific BFD, measurement delay will be scaled by the number of BFD-RS sets. Proposal 7: For a CORESET with two activated TCI states, UE evaluates the RLM/BFD based on single hypothetical PDCCH BLER for the CORESET. |
| R4-2200604 | vivo | Observation 1 In R17, UL TCIs are only applicable to UL signals/channels, and UL RSs can not be used as source RSs of DL TCIs or joint TCIs. Proposal 1 Applicability of QCL needs not to be updated in R17 feMIMO WI. Proposal 2 RAN4 to work on RLM/BFD requirements for the case PDCCH with two TCI states in FR1 HST-SFN scenario. Observation 2 Compared to R15/16 BFD-RSs, different use case is assumed for the BFD-RSs when one CORESET is configured with two TCI states in HST-SFN scenario. Text Proposal 1: TS 38.133 Clause 8.1.1 If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', on the [RLM-RS pair], the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell. Otherwise, on each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell. Text Proposal 2: TS 38.133 Clause 8.5.1 If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', on the [BFD-RS pair] , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of assessing downlink radio link quality of the serving cell beams. Otherwise, on each RS resource configuration in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing assessing downlink radio link quality of the serving cell beams. Proposal 3 Adopt Text proposal 1 and Text proposal 2 for clarifications on BFD and RLM requirements in R17 HST-SFN scenario, and inform RAN1 about the above change in the reply LS. Proposal 4 Ask RAN1 for confirmation on whether the term ‘RLM-RS pair’ can used for the RLM case. Proposal 5 In R17 feMIMO WI, simultaneous transmission or reception based on 2 active UE panels is not considered for RRM requirements, and the enhancements of related RRM requirements can be considered in R18 FR2 related RAN4-led WI. Proposal 6 RRM requirements for TRP-specific BFR should be specified for FR1 in R17. Proposal 7 For the CC configured with TRP-specific BFR, introduce scaling factor PTRP = 2 to the following period requirements - SSB-based BFD - CSI-RS-based BFD - SSB-based CBD - CSI-RS-based CBD |
| R4-2200651 | Nokia, Nokia Shanghai Bell | Observation 1 : The FR2 UE feature with simultaneous RX can extract full benefits both single cell and mTRP deployment. RAN1 is discussing the UE feature as one of key solutions to overcome FR2 blocking issue since Rel-16. Proposal 1 : RAN4 does not conclude if no RRM requirements will be specified for simultaneous reception channel/RS with different QCL type D in Rel-17 feMIMO WI. Proposal 2 : The study on the simultaneous RX can be postponed to Rel-18 WI phase and continued under a Rel-18 WI. Requirement applicability to Rel-17 UE should be defined after introducing the requirements. BFR/CBD/BFRQ requirements Proposal 3 : Study BFR requirements for the two cases below : (i) to receive and evaluate sequentially BFD-RS from multiple TRP. (ii) to receive and evaluate only one BFD-RS, when a serving cell beam failure is detected. Proposal 4: consider optionally setting measurement priority over a BFD-RS set. This helps quick BFR for a prioritized cell (i.e. serving cell) or when BFD-RS from all TRPs fail at the same time. Proposal 5: If a UE receives sequentially BFD-RS from multiple TRPs and if the number of BFD-RS included in the two BFD-RS sets (k=0,1) is more than 2 or higher, consider to apply scaling_factor_BFD to TEvaluate_BFD_SSB evaluation period. (FFS on scaling_factor_BFD values) Proposal 6: If a UE receives sequentially BFD-RS from multiple TRPs and if the number of BFD-RS included in the two BFD-RS sets (k=0,1) is more than 2 or higher, consider applying scaling_factor_CBD to TEvaluate_CBD evaluation period. (FFS on scaling_factor_CBD values) Proposal 7 : In this case (ii) in Proposal 3 that a UE receive and evaluate only one BFD-RS with measurement priority, reuse a current requirement for a single cell (i.e. not to apply the scaling factors to the evaluation period requirements) Proposal 8 : If a FR2 UE does not support simultaneous reception, the following scheduling restriction applies due to beam detection ( in 8.5.8.3 Scheduling availability of UE performing L1-RSRP measurement on FR2) - The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, CSI-RS for tracking or CSI-RS for CQI on reference symbols to be measured for candidate beam detection. Proposal 9 : Adopt baseline from the BFRQ requirements in TS38.133 8.5.9.2, and studies further details depending on a recovery target cell and the evaluation period above. - How to treat two PUCCH-SR resource/SR configurations for T1 calculation. - How to apply a scaling factor in TEvaluate_CBD calculation for T2 calculation. UE supporting simultaneous reception from different QCL-type-D sources Proposal 10 : Introduce separate UE requirements of two types of UE reception capability in Rel-17 (i) A UE not supporting simultaneous reception from different QCL-type-D sources (ii) A UE supporting simultaneous reception from two (or multiple) different QCL-type-D sources Proposal 11 : RAN4 studies if the UE processing capabilities below can be supported for simultaneous reception in a single carrier - One RX chain for L1 measurement, another RX chain for L1 measurement - One RX chain for L1 measurement, another RX chain for L3 measurement - One RX chain for L3 (or L1) measurement, another RX chain for PDCCH monitoring/decoding - One RX chain for L3 (or L1) measurement, another RX chain for PDSCH decoding Proposal 12 : Clarify UE beamforming capability for simultaneous reception requirement discussion : - Option-1 : Simultaneous reception capability is equal to UE RX capability with multiple RX panel from two different AoAs. - Option-2 : It is RAN4 understanding that simultaneous reception includes UE beamforming capabilities using a wide beam or two narrow beams regardless of the number of RX panels. RAN1 LS R1-2112762 reply discussion Proposal 13 : RAN4 make a reply LS draft as below RAN4 is working on UE supports for Rel-17 enhancements for inter-cell beam management and inter-cell mTRP - RAN4 develops UE requirements for at least Nmax = 1, where Nmax is the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting. - RAN4 will further study to specify the requirements for other N_max value taking RAN1 decision into account - RAN4 is discussing to introduce separate UE requirements of two types of UE reception capability and requirements in Rel-17. - A UE not supporting simultaneous reception from different QCL-type-D sources >> When there is overlap for L1-RSRP measurement for SSB associated with serving cell PCI and PCIs different from the serving cell PCI, the UE sequentially measures one SSB and another. - A UE supporting simultaneous reception from two (or multiple) different QCL-type-D sources >> When there is overlap for L1-RSRP measurement for SSB associated with serving cell PCI and PCIs different from the serving cell PCI, the UE can simultaneously measure L1 RSRP. This is up to UE RX capability indication. - Due to lack of Rel-17 TU, the Rel-17 requirement study on simultaneous reception continues in Rel-18 WI phase. |
| R4-2201387 | Ericsson | Proposal 1: RAN4 to define requirements for TRP specific BFD, CBD and BFRQ Proposal 2: RAN4 to agree that BFD has to performed on 2 BFD-RS sets in m-TRP operation. Proposal 3: RAN4 to agree table 1 and table 2 as the evaluation period for SSB based BFD for each TRP in m-TRP operation. Table 1: Evaluation period of one SSB based BFD-RS set in m-TRP operation in FR1 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 ´ P) ´ TSSB). DRX cycle ≤ 320ms; Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)). DRX cycle > 320ms; Ceil(5 ´ P) ´ TDRX. Note: TSSB is the SSB periodicity of the SSB in the set . TDRX is the DRX cycle length.. Table 2: Evaluation period of one SSB based BFD-RS set in m-TRP operation in FR2 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 ´ P ´ N) ´ TSSB). DRX cycle ≤ 320ms; Max(50, Ceil(7.5 ´ P ´ N) ´ Max(TDRX,TSSB)). DRX cycle > 320ms; Ceil(5 ´ P ´ N) ´ TDRX. Note: TSSB is the SSB periodicity of the SSB in the set . TDRX is the DRX cycle length.. Proposal 4: RAN4 to agree table 3 and table 4 as the evaluation period for CSI-RS based BFD for each TRP in m-TRP operation. Table 3: Evalution period of one CSI-RS based BFD-RS set in m-TRP operation in FR1 Configuration; TEvaluate_BFD_CSI-RS (ms). no DRX; Max(50, [MBFD P PBFD] TCSI-RS). DRX cycle ≤ 320ms; Max(50, [1.5 × MBFD P PBFD] Max(TDRX, TCSI-RS)). DRX cycle > 320ms; [MBFD P PBFD] TDRX. Note: TCSI-RS is the periodicity of the CSI-RS resource in the set . TDRX is the DRX cycle length.. Table 4: Evalution period of one CSI-RS based BFD-RS set in m-TRP operation in FR2 Configuration; TEvaluate_BFD_CSI-RS (ms). no DRX; Max(50, [MBFD P N PBFD] TCSI-RS). DRX cycle ≤ 320ms; Max(50, [1.5 × MBFD P N PBFD] Max(TDRX, TCSI-RS)). DRX cycle > 320ms; [MBFD P N PBFD] TDRX. Note: TCSI-RS is the periodicity of the CSI-RS resource in the set . TDRX is the DRX cycle length.. Proposal 5: RAN4 to agree that CBD has to performed on 2 CBD-RS sets in m-TRP operation. Proposal 6: RAN4 to agree table 5 and table 6 as the evaluation period for SSB based CBD for each TRP in m-TRP operation. Table 5: Evaluation period of one SSB based CBD-RS set in m-TRP operation of FR1 Configuration; TEvaluate_CBD_SSB (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 ´ P ´ PCBD) ´ TSSB). DRX cycle > 320ms; Ceil(3 ´ P ´ PCBD) ´ TDRX. Note: TSSB is the SSB periodicity of the SSB in the set . TDRX is the DRX cycle length.. Table 6: Evaluation period of one SSB based CBD-RS set in m-TRP operation of FR2 Configuration; TEvaluate_CBD_SSB (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 ´ P ´ N ´ PCBD) ´ TSSB). DRX cycle > 320ms; Ceil(3 ´ P ´ N ´ PCBD) ´ TDRX. Note: TSSB is the SSB periodicity of the SSB in the set . TDRX is the DRX cycle length.. Proposal 7: RAN4 to agree table 7 and table 8 as the evaluation period for CSI-RS based CBD for each TRP in m-TRP operation. Table 7: Evaluation period of one CSI-RS based CBD-RS set in m-TRP operation of FR1 Configuration; TEvaluateC_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD P PCBD) TCSI-RS). DRX cycle > 320ms; Ceil(MCBD P PCBD) TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length.. Table 8: Evaluation period of one CSI-RS based CBD-RS set in m-TRP operation of FR2 Configuration; TEvaluate_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD P N PCBD) TCSI-RS). DRX cycle > 320ms; Ceil(MCBD P N PCBD) TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length.. Proposal 8: RAN4 to agree that delay required from BFD on TRP to SR transmission on TRP for BFR procedure is given by T = T1 x Ceil((T2+D) /T1); Where: - T1 is equal to the periodicity of PUCCH configured with schedulingRequestIDForBFR. - T2 = TEvaluate_CBD is the evaluation period. D is the UE Processing time and value of D is [2ms]. |
| R4-2201618 | Huawei, HiSilicon | Proposal 1: In Rel-17, the requirements on applicability of QCL need to be updated for applicable to both DL and UL channel. Proposal 2: In Rel-17, it is suggested to define DL TCI chain and UL TCI chain respectively for unified TCI framework. Proposal 3: The existing definition of TCI chain in R15/R16 can be reused for DL TCL chain in R17. Proposal 4: In R17, a UL TCI chain - consists of an SSB, and one or more CSI-RS resources, and the TCI state of each reference signal includes another reference signal in the same TCI chain. Or - consists of an SSB, and one or more SRS resources, and the TCI state of each reference signal includes another reference signal in the same TCI chain. Or - consists of an SSB, and one or more CSI-RS resources, and one or more SRS resources, and the TCI state of each reference signal includes another reference signal in the same TCI chain. Proposal 5: The existing BFD and CBD measurement requirements in R16 can be applied to the serving cell configured with TRP-specific BFR in R17. |

Open issues summary

Sub-topic 3-1: TRP specific BFR

Please note that some proposals (issues) might be omitted by purpose as they are deprioritized in the 1st round discussion or out of the scope of [219].

Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery

- Proposals

  - Proposal 1: for both FR1 and FR2, to introduce a new scaling factor PTRP to extend the evaluation period of BFD and CBD for multiple TRPs. (MTK)

  - Proposal 2: Introduce sharing factor for BFD and CBD evaluation period in FR2 when BFD-RS or CBD-RS are received simultaneously from 2 TRP. (Apple, Nokia)

  - Proposal 3: For TRP-specific BFD, delay requirement is defined assuming that UE will perform BFD in TDM manner; measurement delay will be scaled by the number of BFD-RS sets. (Intel)

  - Proposal 4: RRM requirements for TRP-specific BFR should be specified for FR1 in R17. For the CC configured with TRP-specific BFR, introduce scaling factor PTRP = 2 to requirements SSB-based and CSI-RS based BFR. (vivo)

  - Proposal 5: Study BFR requirements for the two cases: (i) to receive and evaluate sequentially BFD-RS from multiple TRP and (ii) to receive and evaluate only one BFD-RS when a serving cell beam failure is detected or when beam failure of all cells are detected at the same time (Nokia)

  - Proposal 6: RAN4 to define requirements for TRP specific BFD, CBD and BFRQ and agree that BFD has to performed on 2 BFD-RS sets in m-TRP operation. Agree that CBD has to performed on 2 CBD-RS sets in m-TRP operation. Agree on detailed requirement Table in R4-2201387. (Erricsson)

  - Proposal 7: RAN4 to agree that delay required from BFD on TRP to SR transmission on TRP for BFR procedure is given by T = T1 x Ceil((T2+D) /T1); Where:

    - T1 is equal to the periodicity of PUCCH configured with schedulingRequestIDForBFR.

    - T2 = TEvaluate_CBD is the evaluation period.

    - D is the UE Processing time and value of D is [2ms].

  - Proposal 8: The existing BFD and CBD measurement requirements in R16 can be applied to the serving cell configured with TRP-specific BFR in R17.

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Sub-topic 3-2: QCL definition

Issue 3-2-1 QCL definition for UL TCI state

- Proposals

  - Proposal 1: RAN4 should further check the need of the QCL definition for UL TCI state. (MTK)

  - Proposal 2: Update TCI chain definition to include SRS and for PUCCH/PUSCH. (Apple)

  - Proposal 3: For DL TCI chain, SSB associated with a different PCID is added as a source RS. DL TCI chain can apply for separate DL TCI state or joint TCI state switching. (Intel)

  - Proposal 4: For UL TCI chain, SSB, SRS or CSI-RS will be considered as source RS in the UL TCI chain. the number of Reference Signals in the chain is no more than 4. (Intel)

  - Proposal 5: Applicability of QCL needs not to be updated in R17 feMIMO WI.

  - Proposal 6: the requirements on applicability of QCL need to be updated for applicable to both DL and UL channel. The existing definition of TCI chain in R15/R16 can be reused for DL TCL chain in R17. A UL TCI chain (Huawei)

    - consists of an SSB, and one or more CSI-RS resources, and the TCI state of each reference signal includes another reference signal in the same TCI chain. Or

    - consists of an SSB, and one or more SRS resources, and the TCI state of each reference signal includes another reference signal in the same TCI chain. Or

    - consists of an SSB, and one or more CSI-RS resources, and one or more SRS resources, and the TCI state of each reference signal includes another reference signal in the same TCI chain.

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Sub-topic 3-3: Reply on RAN1 LSs

Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829

- Proposals

  - Proposal 1: For a CORESET with two activated TCI states, UE evaluates the RLM/BFD based on single hypothetical PDCCH BLER for the CORESET. (Intel, vivo)

  - Proposal 2: For a CORESET with two activated TCI states, two RS indexes are included in q0 (as two BFD RS resources) for implicit BFD. (Apple)

  - Proposal 3: Adopt Text proposal 1 and Text proposal 2 for clarifications on BFD and RLM requirements in R17 HST-SFN scenario, and inform RAN1 about the above change in the reply LS. And ask RAN1 for confirmation on whether the term ‘RLM-RS pair’ can used for the RLM case. (vivo)

- Text Proposal 1: TS 38.133 Clause 8.1.1 - If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', on the [RLM-RS pair], the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell. Otherwise, on each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell. - Text Proposal 2: TS 38.133 Clause 8.5.1 - If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', on the [BFD-RS pair] , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of assessing downlink radio link quality of the serving cell beams. Otherwise, on each RS resource configuration in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing assessing downlink radio link quality of the serving cell beams.

- Recommended WF

  - Collect companies’ view for these proposals in 1st round

Companies views’ collection for 1st round

| Company | Comments |
| --- | --- |
| Moderator | Sub-topic 2-1: Specification Structures for Inter-cell L1-RSRP Measurement TBA Sub-topic 2-2: Requirements for inter-cell L1-RSRP measurement TBA Sub-topic 2-3: L1-RSRP measurement behavior for non-serving cell TBA |
| Intel | Sub-topic 3-1: TRP specific BFR Issue 3-1-1: Fine with proposal 1,2,3,4 We think proposal 1,2,3,4 are similar. Issue 3-2-1: Support proposal 3 and 4. Issue 3-3-1: Support proposal 1. From our understanding, since it’s SFN transmission, proposal 1 seems to be more reasonable. The two RS will be combined first and then generate a single BLER. How to combine the Rs is up to UE’s implementation. |
| Apple | Sub-topic 3-1: TRP specific BFR Proposal 1,2,3,4 are the same and we support them. Issue 3-2-1 QCL definition for UL TCI state We support Proposals 2,3,4 Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 We are not sure if RAN1 is expecting reply for this LS. But we need to address this in RAN4 requirements. For proposal 1 we would like more clarification on how single hypothetical PDCCH BLER is evaluated – how are the SINRs of the 2 RSs combined for single SINR? In our understanding q0 could be with the 2 RS in the 2 activated TCI states. |
| vivo | Sub-topic 3-1: TRP specific BFR Proposal 1,2,3,4 are the same and we support them. Issue 3-2-1 QCL definition for UL TCI state Open to discuss. P2 is too general. P3: SSB with different PCI needs to be considered for both DL TCI and UL TCI. In our view we prefer no spec impact. P4: In principle we are OK P6: In principle we are OK. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 We support P1. Same view as intel. If UE calculate BLER based on the respective RSs, the BF may be mistakenly triggered. We also support P3. Regarding reply LS, we are also if RAN4 just address the issue in RAN4 and does not reply RAN1, since there is no action in RAN1 LS asking for reply. However, in case there is any unclear question from RAN4 perspective, we may ask RAN1 back. |
| Huawei | Sub-topic 3-1: TRP specific BFR Issue 3-1-1: we are fine with proposal 2 and proposal 8. In FR2, sharing factor can be introduced when BFD-RSs/CBD-RSs from 2 TRP are overlapped, otherwise the existing BFD and CBD measurement requirements in R16 can be applied. Sub-topic 3-2: QCL definition Issue 3-2-1: we are fine with proposal 2, proposal 3, proposal 4 and proposal 6. Sub-topic 3-3: Reply on RAN1 LS BFR for CORESET with two activated TCI states Issue 3-3-1: we are OK with proposal 1. For proposal 2, two RSs implicitly indicated by two activated TCI states are considered as two BFD-RS resource sets. |
| Nokia | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery Proposal 1 : We are not sure why introducing a new scaling factor for FR1 UE. Proposal 2,3,4, 5 : For FR2 UE not supporting simultaneous RX, the scaling factor should be applied with conditions (i.e. measurement priority to cells). PTRP value is FFS. Proposal 5 : support. Proposal 6 : We propose to consider requirements separately for ‘FR2 UE not supporting simultaneous RX’, for ‘FR2 UE supporting simultaneous RX’ and ‘FR1 UE’. Proposal 7 : It is up to how to determine T1 and T2. The approach itself can be reused. Issue 3-2-1 QCL definition for UL TCI state we are fine with proposal 2, proposal 3, proposal 4. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 Support proposal 1. Further text is FFS. |
| Ericsson | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery We share same view as Huawei. Existing BFD and CBD requirements can be reused for each RS set/TRP. Our proposal is same as proposal 8. When they have to be received on same OFDM symbol (without different QCL type D), sharing factor can be defined. Support proposal 2 and 8. For BFR procedure, we support proposal 7 as discussed in our paper. Issue 3-2-1 QCL definition for UL TCI state We are not defining UL TCI state switching requirements based on SRS in Rel-17. Due to this proposal 2 may not be discussed. Proposal 3 looks fine. Proposal 4: SRS can be removed as we are not defining requirements based on SRS in Rel-17. Proposal 6: same comment as proposal 4. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 We agree with proposal 1. Proposal 3 also looks similar to proposal 1. We are fine to discuss proposal 3 further. |
| Samsung | Issue 3-1-1, We support to introduce sharing factor for TRP specific BFR Issue 3-3-1 We support proposal 1 |
| MediaTek | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery Agree with proposal 1,2,3 and 4. They are similar proposal which introduce a sharing factor. For proposal 5, unclear to us why we need to have two different procedures here? What is the exact scenario for case (ii)? For proposal6, is there any difference between proposal 1,2,3,4? For proposal 7, agree. The existing requirement can be reused. For proposal 8, disagree. The sharing factor should be used to extend the evaluation period if two RS are received simultaneously. Issue 3-2-1 QCL definition for UL TCI state Support proposal 3 to update the DL TCI chain for non-serving cell. Not sure should we update the UL QCL if there is no use case in spec, e.g., spatial relation requirement in TS 38.133 is based on DL-RS. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 Support proposal 2. We do see the issue here. Two BLER will be calculated for each RS and the worse one will be used to compared with the BFD threshold |

Summary for 1st round

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery - Proposals - Option 1: Introduce sharing factor for BFD and CBD evaluation period in FR2 when BFD-RS or CBD-RS are received simultaneously from 2 TRP. FFS for FR1. - Option 2: The existing BFD and CBD measurement requirements in R16 can be applied to the serving cell configured with TRP-specific BFR in R17. - Option 3: Others Recommendations for 2nd round: Continue discussion in the 2nd round. |
| Sub-topic #3-2 | Issue 3-2-1 QCL definition for UL TCI state - Proposals - Option 1: Update TCI chain definition to include SRS and for PUCCH/PUSCH. (Apple) - Option 2: For DL TCI chain, SSB associated with a different PCID is added as a source RS. DL TCI chain can apply for separate DL TCI state or joint TCI state switching. (Intel) - Option 3: For UL TCI chain, SSB, SRS or CSI-RS will be considered as source RS in the UL TCI chain. the number of Reference Signals in the chain is no more than 4. (Intel) - Option 4: Others Recommendations for 2nd round: Continue discussion in the 2nd round. Also discuss on Intel’s dCR R4-2200536 in the 2nd round. |
| Sub-topic #3-3 | Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 - Proposals - Option 1: For a CORESET with two activated TCI states, two RS indexes are included in q0 (as two BFD RS resources) for implicit BFD. (Apple) - Option 2: For a CORESET with two activated TCI states, UE evaluates the RLM/BFD based on single hypothetical PDCCH BLER for the CORESET. (Intel, vivo) Recommendations for 2nd round: Continue discussion in the 2nd round. Most company agree on Option 2, so we could agree on Option 2. No need to replay the LS unless if there is further question to RAN1. |

Discussion on 2nd round (if applicable)

[Moderator] The open issues are list here for your reference. In case of double efforts, companies may choose to make their comments directly on the corresponding WF.

Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery

- Proposals

  - Option 1: Introduce sharing factor for BFD and CBD evaluation period in FR2 when BFD-RS or CBD-RS are received simultaneously from 2 TRP (Apple, Huawei, MTK, Samsung). FFS for FR1. (Intel)

  - Option 2: The existing BFD and CBD measurement requirements in R16 can be applied to the serving cell configured with TRP-specific BFR in R17. (Ericsson)

  - Option 3: Others

Issue 3-2-1 QCL definition for UL TCI state (Also discuss on Intel’s dCR R4-2200536 Here)

- Proposals

  - Option 1: Update TCI chain definition to include SRS and for PUCCH/PUSCH. (Apple)

  - Option 2: For DL TCI chain, SSB associated with a different PCID is added as a source RS. DL TCI chain can apply for separate DL TCI state or joint TCI state switching. (Intel, Erricson,MTK)

  - Option 3: For UL TCI chain, SSB, SRS or CSI-RS will be considered as source RS in the UL TCI chain. the number of Reference Signals in the chain is no more than 4. (Intel)

  - Option 4: Others

Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829

- Proposals

  - Option 1: For a CORESET with two activated TCI states, two RS indexes are included in q0 (as two BFD RS resources) for implicit BFD. (Apple,MTK)

  - Option 2: For a CORESET with two activated TCI states, UE evaluates the RLM/BFD based on single hypothetical PDCCH BLER for the CORESET. (Intel, vivo, Ericsson, Nokia, Samsung)

  - Option 3: No need to replay the LS unless if there is further question to RAN1.

| Company | Comments |
| --- | --- |
| Apple | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery Option 1 for FR2. For FR1 existing requirements are applicable Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 Option 1. If qo set has the 2 RSs that are in the active TCI state of PDCCH, UE would evaluate for RLM and BFD based on both and declare BFD / RLM only if both are below the threshold. This seems more straightforward than having to combine the RSs for single BLER. We don’t think we need to reply to RAN1 but need to capture requirements in RAN4 spec accordingly. |
| Intel | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery Option 1. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 Option 2. Same comments in the first round. |
| Ericsson | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery We support option 2. When they are not overlapped on the same symbol, sharing factor is not needed. Issue 3-2-1 QCL definition for UL TCI state (Also discuss on Intel’s dCR R4-2200536 Here) We are not defining TCI state switching requirements with SRS as source RS for QCL. Hence, we think update of TCI chain definition to include SRS and for PUCCH/PUSCH is not needed Agree with option 2 that for DL TCI chain, SSB associated with a different PCID is added as a source RS. DL TCI chain can apply for separate DL TCI state or joint TCI state switching. We don’t think we need to update UL TCI chain, to consider SRS as source RS in the UL TCI chain. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 We agree with option 2, that for a CORESET with two activated TCI states, UE evaluates the RLM/BFD based on single hypothetical PDCCH BLER for the CORESET. |
| Nokia | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery The proposal-1 are captured at high level, can we add more specific discussions in the next meeting below? - FFS : whether to introduce sharing factor for FR1 (>> as done already) - FFS : value and conditions to apply the sharing factor Regarding Option-2, we also keep option-2 in the scope of simultaneous RX. We notice that RAN4 already enters the requirement discussion that a UE measures sequentially. We want to put FFS on UE RX capability and requirement of sequential reception or simultaneous reception on BFD-RS as the L1 discussion. Issue 3-2-1 QCL definition for UL TCI state (Also discuss on Intel’s dCR R4-2200536 Here) We are fine with the proposals. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 Option-2. The hypothesis of a single hypothetical PDCCH BLER in qo set means that at least one of the two RSs is evaluated as valid beam correspondence. Firstly, RAN4 agrees to Option-2. And then we can study further the method how to derive the single hypothetical PDCCH BLER. |
| Huawei | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery In FR1, the existing BFD and CBD measurement requirements in R16 can be applied to the serving cell configured with TRP-specific BFR in R17. In FR2, the existing BFD and CBD measurement requirements in R16 can be reused as baseline and additional sharing factor (PTRP) needs to be introduced for BFD and CBD evaluation period when BFD-RS or CBD-RS are received simultaneously from 2 TRP. Issue 3-2-1 QCL definition for UL TCI state (Also discuss on Intel’s dCR R4-2200536 Here) We can agree with option 1, option 2 and option 3. For the draftCR, we suggest to update QCL definition in the existing section. DL TCI chain and UL TCI chain are separately defined. It can be clarified that the SSB in a DL/UL TCI chain can be associated with a PCI different from the serving cell. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 We agree with option 1and option 2. We also agree with option 3. The LS is just for information and there is no question raised by RAN1 for answer. |
| vivo | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery For FR2, M-TRP transmission requires UE supporting simul QCL-D, which is not within scope of R17. For FR1, we think RAN4 should prioritize it firstly, since there is no simul QCL-D issue. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 Option 2. Same comments in the first round. |
| MediaTek | Issue 3-1-1 Requirement for TRP specific Beam Failure Recovery Share the same view as Huawei. Issue 3-2-1 QCL definition for UL TCI state (Also discuss on Intel’s dCR R4-2200536 Here) Support option 2. Issue 3-3-1 Reply on RAN1 LS BFR for CORESET with two activated TCI states R1-2112829 Prefer option 1. |
| Samsung | Issue 3-1-1 We support option 1 Issue 3-3-1 We support option 2 |

| CR/TP number | Comments Collection |
| --- | --- |
| R4-2200536 Intel | Apple: We don’t need to define Dl TCI separately. The paragraphs are just repetition of the existing spec. The existing definition can be extended for DL and joint in the first section. Also need to add: DMRS of PUCCH or PUSCH is QCLed with the reference signal in its active TCI state and any other reference signal that is QCLed, based on above criteria, with the reference signal in the active TCI state. We are fine with Applicability of Ul spatial relation section, but we need to add the following: DMRS of PUCCH or PUSCH is QCLed with the reference signal in its active TCI state and any other reference signal that is QCLed, based on above criteria, with the reference signal in the active TCI state. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on RRM impact on unified TCI in FeMIMO | Samsung | Capture agreements and WF for Topic#1 |
| WF on FeMIMO RRM requirements for inter-cell beam management | Samsung | Capture agreements and WF for Topic#2 |
| WF on other RRM requirements for FeMIMO | Huawei | Capture agreements and WF for Topic#3 |
| Reply LS on L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap | vivo | To: RAN1; CC: RAN2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200536 | DraftCR on TCI chain update for Rel-17 FeMIMO | Intel | Return to | Continue discussing the CR in the 2nd round |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2202666 | WF on RRM impact on unified TCI in FeMIMO | Samsung | Return to | Currently the final version has been uploaded based on majority view. Continue email discussion before Tue. GTW if any other concerns. |
| R4-2202667 | WF on FeMIMO RRM requirements for inter-cell beam management | Samsung | Return to | Currently the final version has been uploaded based on majority view. Continue email discussion before Tue. GTW if any other concerns. |
| R4-2202668 | WF on other RRM requirements for FeMIMO | Huawei | Agreeable | The latest version could be approved based on companies’ comments. |
| R4-2202669 | Reply LS on L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap | vivo | Return to | Currently still some tech issues are under discussion. Continue email discussion until Tue. GTW to decide if it could be approved. |
| R4-2202750 | DraftCR on TCI chain update for Rel-17 FeMIMO | Intel | To be endorsed | The latest version could be endorsed based on companies’ comments. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Samsung) | Yiyan Zhang | Yiyan.zhang@samsung.com |
| Samsung | Xutao Zhou | xutao.zhou@samsung.com |
| MediaTek | ChihKai Yang | ck.yang@mediatek.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)