3GPP TSG-RAN WG4 Meeting # 101-bis-e 													R4-2202966

Electronic Meeting, Jan 17 - 25, 2022

Agenda item:			6.12.1, 6.12.2.3

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [101-bis-e][316]NR_perf_enh2_Demod_Part1

Document for:	Information

Introduction

This email thread discusses the general part and CRS interference handling in Rel-17 further demodulation performance enhancement WI in agenda 6.12.1 and 6.12.2.3.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to provide comments in section 1.3, 2.3, 3.3, 4.3 and 5.3.

  - Comments for Topic #3 (Signalling aspects for CRS-IM) and Topic #5 (CRS-IM for 30 kHz SCS scenario, which impacts the UE capability and NWA discussion) before 23:59 UTC Tuesday are appreciated.

  - All the comments before the official round 1 commenting deadline (08:00 UTC Wednesday) will be considered in the round 1 summary.

- 2nd round: TBA

Topic #1: General

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200017 | China Telecom | Draft TR 38.833 v1.1.0: Further enhancement on NR demodulation performance (For email approval after the meeting, i.e., to implement the TPs approved during the meeting.) |
| R4-2200018 | China Telecom | TP to TR 38.833: Symbols and abbreviations |
| R4-2200905 | China Telecom | Updated work plan for Further enhancement on NR demodulation performance WI |

Open issues summary

N.A.

Comments collection for the TP and the updated work plan

| WP/TP number | Comment collection |
| --- | --- |
| R4-2200018, CTC. TP to TR 38.833: Symbols and abbreviations |  |
| R4-2200905, CTC, Updated work plan |  |

Summary for 1st round

| WP/TP number | WP/TP Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
|  | No comments received on the following tdocs: - R4-2200018 China Telecom TP to TR 38.833: Symbols and abbreviations - R4-2200905 China Telecom Updated work plan for Further enhancement on NR demodulation performance WI |

Discussion on 2nd round

Topic #2: Receiver assumptions

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200502 | Nokia, Nokia Shanghai Bell | Reference receiver for CRS-IM It was agreed in RAN-94e to not include CRS-IC in Rel.17. UE processing time impact of CRS-IC As per RAN decision, UE processing time impact of CRS-IC should not be included in Rel-17. Implementation details for LLR weighting - If an actual implementation of LLR weighting does not require additional NWA parameters compared to implementations of comparable efficiency and performance, the implementation using the minimum complexity level of NWA should be selected. As long as there is no requirement for additional NWA parameters, RAN4 should leave the LLR-weighting implementation details to the UE, if simulation results delivered in this meeting show no misalignment (Option 3 from WF). Handling of colliding of the 2nd DMRS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting - The impact of CRS to the 2nd DMRS in symbol#11 is already handled in DSS (RM) where the 2nd DMRS can be shifted to symbol#12. - It is not possible to use the shift of the 2nd DMRS symbol when Rate Matching is not enabled. The handling should be left to the UE if simulations do not show any misalignments (Option 3 from WF). |
| R4-2200517 | Intel Corporation | Observations #1: Different methods of LLR weighting processing provide rather close performance. Proposal 1: Leave up to UE implementation the details of LLR weighting implementation. Observations #2: There is no big performance difference for scenarios with different interference-plus-noise covariance matrix estimation methods. Proposal 2: Leave up to UE implementation the details of interference-plus-noise covariance matrix estimation for CRS-IM processing. |
| R4-2200801 | CMCC | Proposal 1: Using CRS power to directly scale the LLRs without equalization processing involved. Proposal 2: Support using all DMRS for Ruu estimation provided that no obvious simulation result mis-alignment is observed. |
| R4-2200898 | China Telecom | Observation 1: Option 2A is the implementation of LLR weighting we used in our simulation, which achieves 1.2 ~ 2.0dB performance gain over the reference gain in scenario 1. Proposal 1: Leave the details of LLR weighting to UE implementation if no simulation result mis-alignment issue happens. Proposal 2: For whether to use all DMRSs for Ruu estimation, leave this issue to UE implementation, if no simulation result mis-alignment is find. |
| R4-2201414 | ZTE Corporation | Proposal 1: Whether or not to turn on CRS-IM should be left up to UE implementation. Observation 1: No significant performance difference is observed between using all DMRS and using DMRS not overlapping with CRS REs. Proposal 2: Handling of colliding DMRS symbols is up to UE implementation. |
| R4-2201415 | ZTE Corporation | Simulation results on CRS-IM receiver |
| R4-2201418 | Ericsson | Observation 1: It is approved in RAN#94-e meeting to further discuss CRS-IC receiver in Rel-18 if needed. Proposal 2: Not to continue the discussion on UE processing time impact of CRS-IC in Rel-17 Proposal 3: Leave it to UE implementation if no simulation result mis-alignment due to this issue |

Open issues summary

Issue 2-1: Implementation details for LLR weighting

- Status in RAN#101-e in the WF R4-2120705:

  - Option 1: Adopt CRS power into MMSE-IRC equalization processing

    - Calculate the CRS power per receiving antenna and the power vector is ICRS

    - Update the LLR of CRS REs by adding the diag (ICRS) to interference plus noise covariance in MMSE-IRC processing.

  - Option 2: Direct scaling of LLR without equalization processing involved

    - Option 2A:

      - For each v-shift, calculate the average CRS power of all Rx antennas per PRB.

      - Use the above CRS power to scale the LLRs on the interfered REs within this PRB, rather than using it in the MMSE-IRC equalization.

    - Option 2B: (LLR weighting processing flow in section 2.1 of R4-2118004)

  - Option 3: Leave to UE implementation if no simulation result mis-alignment due to this issue

- Proposals

  - Option 1: Adopt CRS power into MMSE-IRC equalization processing

  - Option 2: Direct scaling of LLR without equalization processing involved (CMCC)

  - Option 3: Leave to UE implementation if no simulation result mis-alignment due to this issue (Nokia, Intel, [CMCC], China Telecom, Ericsson)

    - Intel: Different methods of LLR weighting processing provide rather close performance.

- Recommended WF

  - Agree option 3

Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting

- Status in RAN#101-e in the WF R4-2120705:

  - Option 1: Not use DMRS REs for Ruu estimation which are overlapping with CRS REs

  - Option 2: Use all DMRSs for Ruu estimation

  - Option 3: Leave to UE implementation if no simulation result mis-alignment due to this issue

- Proposals

  - Option 1: Not use DMRS REs for Ruu estimation which are overlapping with CRS REs

  - Option 2: Use all DMRSs for Ruu estimation (CMCC)

    - CMCC: provided that no obvious simulation result mis-alignment is observed

  - Option 3: Leave to UE implementation if no simulation result mis-alignment due to this issue (Nokia, Intel, China Telecom, ZTE)

    - Intel: There is no big performance difference for scenarios with different interference-plus-noise covariance matrix estimation methods.

    - China Telecom: we did not observe clear performance difference between option 1 and option 2.

    - ZTE: No significant performance difference is observed between using all DMRS and using DMRS not overlapping with CRS REs.

- Recommended WF

  - Agree option 3?

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting |
| China Telecom | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting Support option 3 at the current stage. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting Support option 3 at the current stage. |
| Intel | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting We support recommended WF. Based on our analysis, there is no big performance difference for different LLR weighting implementations. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting We support recommended WF. Based on our analysis, using of different methods for interference-plus-noise covariance matrix estimation leads to rather same performance. |
| CMCC | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting Option 2 is our first choice, which has less processing complexity. We think Option 2 can be the baseline. While, Option 3 is also ok for us, since most of simulation results shows no mis-alignment at the current stage. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting Ok with the recommended WF. |
| Ericsson | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting Support the recommended WF. We see deviation on implementation of LLR weighting among companies. But there is no quite large performance gap observed in phase I. Hence, our preference stands still on leaving it to UE implementation and we don’t make any assumption. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting Same as issue 2-1. |
| Apple | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting We support the recommended WF. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting We support the recommended WF. |
| Huawei | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting We support the recommended WF. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting We support the recommended WF. |
| Nokia, Nokia Shanghai Bell | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting Agree to option 3, as it seems to be consensus. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting No significant performance differences are seen in provided simulations. We agree to option 3. |
| MediaTek | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting Support the recommend WF. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting Support the recommend WF. |
| Qualcomm | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting Ok with recommended WF. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting Ok with recommended WF. |
| ZTE | Topic #2: Receiver assumptions Issue 2-1: Implementation details for LLR weighting We support the recommended WF. Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting We support the recommended WF. Based on our analysis, there is no significant performance difference is observed between using all DMRS and using DMRS not overlapping with CRS REs. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Topic #2: Receiver assumptions | Issue 2-1: Implementation details for LLR weighting Summary of round 1 discussion: All companies are ok with the following option 3: - Option 1: Adopt CRS power into MMSE-IRC equalization processing - Option 2: Direct scaling of LLR without equalization processing involved - Option 3: Leave to UE implementation if no simulation result mis-alignment due to this issue Tentative agreements: - Leave to UE implementation if no simulation result mis-alignment due to this issue Issue 2-2: Handling of colliding of the 2nd DM-RS symbol #11 in serving cell with CRS symbol #11 in neighbouring cell for scenario 2 with LLR weighting All companies are ok with the following option 3: - Option 1: Not use DMRS REs for Ruu estimation which are overlapping with CRS REs - Option 2: Use all DMRSs for Ruu estimation - Option 3: Leave to UE implementation if no simulation result mis-alignment due to this issue Tentative agreements: - Leave to UE implementation if no simulation result mis-alignment due to this issue |

Discussion on 2nd round

Topic #3: Signalling aspects for CRS-IM

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200274 | Apple Inc. | UE Capability Signaling Proposal #1: Introduce granularity of per CC, per band, per band combination for UE capability for CRS-IM. Proposal #2: UE capability for CRS-IM is only applicable for 15KHz SCS until feasibility to support this confirmed for 30KHz SCS. Network Assistance Signaling Observation #1: For INR and loading values used in Phase 1 evaluation the degrade without CRS sequence information is around 0.5 dB. With lower INR and LTE cell loading we could expect higher degradation. Proposal #3: Include LTE CRS sequence information in parameters needed for LLR weighting. Observation #2: UE cannot assume same number of ports between serving and neighbor LTE cell in scenario 1. Observation #3: Information on number of CRS ports if not indicated by network assistance information would be challenging for UE to obtain. Observation #4: If interference LTE CRS port information is not provided, the UE would assume single port (port 0) for CRS-IM. Observation #5: With single CRS port assumption the performance degrade is > 1dB with 2RX and 0.6 dB for 4RX compared to 4 CRS ports for CRS-IM. Proposal #4: RAN4 proceeds with assumption that number of CRS ports are needed for CRS-IM. Observation #6: In RAN4#101-e there was agreement to include NWA if MBSFN or CRS muting configuration is different from RAN4 assumption. Observation #7: UE would be aware of presence of LTE cell by either network assistance information or from LTE MO. Proposal #5: UE is informed via network assistance of presence of LTE neighbor cell to perform CRS-IM in both scenario 1 and scenario 2. Proposal #6: If network assistance is not configured for UE to inform of presence of LTE neighbor cell, then UE is required to perform CRS-IM only if LTE MO is configured. Observation #8: If LTE MO is configured UE would be able to obtain LTE carrier frequency and vshift information, it cannot obtain LTE bandwidth and CRS port number information. Observation #9: UE needs network assistance to obtain CRS location information – LTE carrier frequency, bandwidth, vshift and CRS port number. Observation #10: If UE is not configured the required CRS location information via network assistance, it is additional complexity for UE to obtain this information on its own. Proposal #7: UE is informed of all LTE CRS location information via network assistance. Proposal #8: If UE is not configured NWA and is expected to rely on LTE-MO, it assumed CRS port 0 and any channel BW of CRS-IM. Proposal #9: UE is informed of LTE sequence information via network assistance, otherwise performance of CRS-IM cannot be guaranteed in real deployment. |
| R4-2200381 | MediaTek Inc. | Observation 1: The impact to UE detection for the necessary information for LLR weighting, UE processing complexity and power consumption cannot be easily quantified. Observation 2: It cannot be guaranteed that PBCH is transmitted in the measurement gap. Proposal 1: RAN4 introduces network signaling for CRS-IM receiver. - For scenario 1: physical Cell ID, number of CRS antenna ports. - For scenario 2: physical Cell ID, number of CRS antenna ports, bandwidth of the LTE carrier, center of the LTE carrier. |
| R4-2200503 | Nokia, Nokia Shanghai Bell | Parameters needed for LLR weighting CRS Sequence Observation 1: It is not clear, if using the CRS sequence will improve LLR Weighting and if so, with how much. Proposal 1: Do not include the CRS sequence as parameter for LLR Weighting and in case UE requires the CRS sequence, the UE must determine it on its own. CRS Ports Proposal 2: Use 1 or 2 CRS ports as default unless simulations provided in this meeting show otherwise. How UE can obtain above parameters which can be discussed in case by case manner also depending on deployment scenarios with below candidate alternatives Observation 2: A simple way to inform the UE about the network configuration in case network does not conform to the agreed default assumptions, can fulfil the agreements from last meeting. Proposal 3: Unless informed by the network, the UE should follow assumed condition/scenarios as default. Assumptions on the network configuration: Part I (excluding CRS port number) Observation 3: UE vendors have argued that UE doing full blind detection can be challenging for the UE implementation with relation to time and power consumption. Observation 4: In case Network does not conform to the default configuration assumptions, the network can inform the UE. If not informed, a UE might enable CRS-IM based on the default configuration assumptions. UE performing CRS-IM using invalid parameters would see performance degrades, which might not be acceptable. Proposal 4: RAN4 to propose means for the network to ensure UE is aware if the network does not conform to the default configuration assumptions. This could be by RAN4 proposing RAN2 to define a new message with this information. The content level of the message is still to be decided. Assumptions on the network configuration: Part II (CRS port number) Observation 5: To simplify the level of NWA, the network configuration should target the same number of CRS ports are used across the network. Proposal 5: Assume the number of CRS ports used in serving and neighbouring cells are the same (option 1). How could UE obtain the identified parameters if not signalled by the network Observation 6: Option1 and option 2 seems both to be possible but incomplete ways for the UE to obtain the identified parameters. For example, further down we will argue, that option 1 represent a strong constraint for the NW scheduling. However, neither option 1 nor option 2 can stand alone in all deployment and scheduling conditions. Proposal 6: Both option 1 and option 2 can form a baseline for detecting the “unidentified” parameters for most deployments. Further discussion on the related technical aspects Whether inter-RAT MO can be always configured Observation 7: The network is already aware when InterRAT MO is required based on UE signalling. This would also apply in case of LTE interference. Whether InterRAT MO is enabled shall be decided by the BS implementation (Option 1). Whether inter-RAT LTE measurement is performed right after receiving the inter-RAT MO Observation 8: It can be assumed, that a well configured network will only configure the InterRAT MO when needed. Enabling of InterRAT MO are not related to a specific threshold in the UE. The InterRAT measurements shall be done by the UE when enabled by InterRAT MO without any threshold. Whether PBCH decoding is always possible in inter-RAT measurement Observation 9: It is a strong constraint on the NW scheduling to always guarantee PBCH can be decoded during InterRAT measurement gaps. The network is not required to ensure that PBCH is placed in the InterRAT measurement gap. On center frequency and bandwidth of LTE carrier for scenario 2 Observation 10: Default assumed network configuration would be expected to be the same center frequency and bandwidth for all LTE cells. If this is not the case InterRAT MO can be used to determine the center frequency for the interference cells and the bandwidth can be known by either PBCH demodulation or, if not possible, by power measurements. Whether to introduce network assistance signalling Observation 11: The CRS-RM pattern IE contains most of the information potentially required to assist in CRS-IM LLR weighing reception. Observation 12: RRC configuration of RateMatchPatternLTE-CRS IE enables CRS-RM and, thus, prevents using CRS-IM approaches to convey the information. Observation 13: The existing of RateMatchPatternLTE-CRS IE is missing some of the parameters requested by companies as required for LLR Weighting. Observation 14: Information provided in any level of NWA is non-dynamic information taken from the network configuration. The information provided is not expected to change during RRC_connected states. Proposal 7: RAN4 to ask RAN2 define new RRC signaling for CRS-IM IE, which contains the NW available information from the IE RateMatchPatternLTE-CRS and adds optional information for CRS-muting, (Max) Number of CRS ports, and v-shifts. The information provided in this new IE should be provided in a non-dynamic manner. Configuration of this message does not trigger CRS-RM functionality. Introduced parameters description Proposal 8: Introduce an optional information parameter “CRS-Muting-r17” to indicate if CRS muting is used in the network. “CRS-Muting-r17” should default to the agreed default assumed setting. Proposal 9: Introduce an optional information parameter “v-ShiftList-r17” to indicate which v-shift is used in the LTE network. “v-ShiftList-r17” should default to the agreed default assumed setting. Proposal 10: Introduce an optional information parameter “maxNrofCRS-Ports-r17” to indicate the maximum number of CRS ports used by the network. RAN4 to discuss, if a default assumption for this parameter can be chosen. Potential impact by misdetection of network parameters without network signalling Observation 15: When an interfering cell is strong enough to warrant the use of CRS-IM, the UE will also be able to correctly detect the needed parameters for LLR Weighting. Proposal 11: Do not consider any misdetection (option 1). Whether to introduce CRS-IM UE capability signalling Observation 16: If UE has informed the serving cell that it supports CRS-IM, the serving cell can assume the UE will enable CRS-IM, hence the serving cell can then disable RM for the interference cells. Proposal 12: RAN4 to further discuss the content of the UE capability signalling after the level of NWA has been agreed. As minimum, the UE capability signalling for CRS-IM shall indicate if the UE supports CRS-IM. Proposal 13: If UE indicates CRS-IM capability, the network will assume CRS-IM to be used. However, this assumption can be overruled by prior and future agreements on default deployment assumptions and CRS-IM activation assumptions thereof. |
| R4-2200518 | Intel Corporation | Proposal 1: Define per cell NWA signalling to inform UE about the configuration of the following NW parameters: CRS muting (for Scenario 1 and 2), MBSFN configuration (for Scenario 1 and 2) and LTE channel bandwidth/center frequency (for Scenario 1), if they are not aligned with default assumptions used for CRS-IM requirements definition. Proposal 2: Do not consider UE blind detection of CRS muting, MBSFN configuration and LTE channel bandwidth/center frequency for Scenario 1. Proposal 3: Do not consider UE blind detection of CRS muting and MBSFN configuration for Scenario 2. Proposal 4: Define per cell NWA signalling to inform UE about bandwidth and center of the carrier of all neighboring LTE cells for Scenario 2. Proposal 5: Define per cell NWA signalling to inform UE about physical cell IDs of all neighboring cells. Proposal 6: Assume blind detection of number of CRS ports. Proposal 7: Assume blind detection of LTE slot number within the frame. |
| R4-2201603 | Intel Corporation | Proposal 1: Define mandatory without capability signalling feature to indicate that UE supports MMSE-IRC requirements for scenarios with inter-cell interference and slot-based transmission in case requirements will be defined in a non-release independent manner. Proposal 2: Define mandatory without capability signalling feature to indicate that UE supports MMSE-IRC requirements for scenarios with intra-cell inter-user interference in case requirements will be defined in a non-release independent manner. (Note: Proposal 1 and 2 to be discussed in thread [101-bis-e][317] NR_perf_enh2_Demod_Part2) Proposal 3: Define optional with capability signalling feature to indicate support of CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR and further discuss whether to introduce single or separate features for Scenario 1 (DSS) and Scenario 2 (NR + LTE). Proposal 4: Use per-UE granularity for CRS-IM capability signalling. |
| R4-2200802 | CMCC | Proposal 1: In order to enable CRS-IM receiver (LLR weighting), only the information of the presence of CRS and CRS location is needed. Proposal 2: UE get the LLR weighting required parameters through the following method: • the presence of LTE cell  For scenario 1: by serving cell CRS-RM configuration, 7.5kHz shift configuration  For scenario 2: by inter-RAT MO configuration • CRS muting information  Follow the assumption of no CRS muting • MBSFN configuration information  For scenario 1, follow the assumption of MBSFN configuration same as serving cell  For scenario 2, follow the assumption of no MBSFN configuration For CRS location information, UE can obtain the information as follows: • v-shift and CRS port number  For Scenario 1 and Scenario 2 with NR 15kHz SCS, UE can obtain the power difference, v-shift and CRS ports information through power difference detection.  For Scenario 2 with NR 30kHz SCS, UE can obtain the power difference and the CRS ports information through power difference detection. No need to further identify the v-shift information. • LTE carrier frequency and bandwidth  For scenario 1, follow the assumption of Channel bandwidth and centre frequency aligned for the serving and neighboring cells  For scenario 2, obtain the LTE neighbor cell carrier frequency through inter-RAT MO configuration, obtain the bandwidth through inter-RAT MO and PBCH decoding. Proposal 3: For Scenario 1 and Scenario 2, only introduce the network assistance signalling to inform UE when default assumption is not aligned with NW configuration Proposal 4: The number of CRS ports in the serving and neighbouring cells can be different. Proposal 5: Introduce CRS-IM UE capability signaling as follows: |
| R4-2200899 | China Telecom | Observation 1: Either with or without CRS sequence information, LLR weighting can always achieve reasonable performance gain (1.6 ~ 2.2 dB) over the reference scheme. Observation 2: LLR weighting with CRS sequence information can achieve 0.3~0.6 dB performance gain over LLR weighting without CRS sequence information, only when there is 1 LTE interference cell per v-shift. Moreover, the gain is not significant considering the additional UE complexity to perform channel estimation of neighbour LTE cell. Observation 3: When performing LMMSE channel estimation to the neighbour cell, LLR weighting with CRS sequence information will need similar UE complexity as CRS-IC. We have concern on whether UE will really implement the LLR weighting with CRS sequence information, since there have been companies raised the UE processing time issue due to the higher complexity of CRS-IC. Observation 4: When there is combined CRS signal from 2 LTE neighbour cells on each CRS RE: 3) There is no performance difference between LLR weighting with and without sequence, when UE perform channel estimation for LTE cell 1 with INR = 10.45dB for LLR weighting with sequence. 4) When the UE perform channel estimation for LTE cell 2 with INR = 4.6dB for LLR weighting with sequence, the performance of LLR weighting with sequence is even worse than that of LLR weighting without sequence. Proposal 1: Do not consider the need of CRS sequence information for LLR weighting processing. Observation 5: With 4 port CRS transmission, S8/Save is larger than 1 dB for 96% of the cases when SNR = 11.2dB, and 99% of the cases when SNR = -0.3dB, where S8 is the power of symbol #8 containing CRS port #2 and #3, and Save is the average power of symbol#3/5/6/9/10. Proposal 2: It is feasible for UE to blind detect the presence of CRS on symbol#8 by comparison of power on different symbols. Observation 6: For the case 1 where UE always assume the wrong CRS port number, there will be 0.4dB performance degradation compare with the case where UE always know the correct CRS port number. Observation 7: With UE performing detection for the CRS port number, LLR weighting can achieve the same performance compare with the case where UE always know the correct CRS port number. Proposal 3: There is no need for UE to be informed the CRS port number to perform LLR weighting. Observation 8: For scenario 1, it is typical to assume the same CRS port number for the serving and the neighbour cell, and meanwhile, it is not necessary to add the restriction on the CRS port number based on proposal 3. Proposal 4: The number of CRS ports in the serving and neighbouring cells can be different. Observation 9: UE only needs to be indicated the presence of the neighbour LTE cell, and then UE can detect the CRS location related information. Proposal 5: The presence of the neighbour LTE can be indicated by the configuration of serving cell CRS-RM for scenario 1, and by the configuration of inter-RAT MO for scenario 2. Proposal 6: Not to introduce any new network assistance signalling, to assist the quicker use of Rel-17 CRS-IM in commercial networks. |
| R4-2200984 | Huawei, HiSilicon | Observation 1: LLR weighting with sequence always has better performance than that without sequence bur there is still large performance gain over baseline receiver for LLR weighting without sequence Proposal 1: It is not necessary for UE to acquire the CRS sequence information. Observation 2: The performance are same for UE has knowledge of CRS ports, always assume 4 ports and CRS port number blind detection. Proposal 2: Consider UE always perform LLR weighting with the assumptions of 4 CRS ports if CRS-IM is turned on without any information of CRS presence information. i.e. CRS muting configuration, MBSFN configuration, number of CRS ports. Proposal 3: Configure Inter-RAT measurement for both scenario 1 and scenario 2 Proposal 4: Not Consider NWA for Rel-17 CRS-IM receiver |
| R4-2201416 | Ericsson | Observation 1: UE only needs to know the location of CRS symbols for doing LLR weighting and don’t need any sequence information Observation 2: Since there is no performance difference between using CRS sequence or not in the LLR weighting, CRS sequence is not needed Observation 3: CRS sequence is not necessary for doing LLR weighting Proposal 1: Not to introduce any type of network assistance signaling for Rel-17 CRS-IM Proposal 2: Not to consider any misdetection in defining demodulation requirement |
| R4-2201417 | Ericsson | Simulation results for UE PDSCH throughput when doing the demodulation with and without CRS sequence. |
| R4-2201964 | Qualcomm Incorporated | Network Assistance Proposal 1: Do not consider blind detection as candidate receiver. Observation 1: We observe 4-5 dB loss with detection algorithms v/s using network assistance under scenario 1 with link adaptation enabled and SIR = -6dB. Observation 2: Detection algorithms may not perform well when interference is weaker than serving cell PDSCH. Observation 3: It was discussed and agreed to provide such network assistance in LTE for CRS interference cancellation. Observation 4: It is additional UE complexity to detect LTE center freq and BW, which may decrease overall throughput either due to misdetection or because of frequent inter-RAT measurement objects. Proposal 2: Define network assistance for CRS interference mitigation. Proposal 3: We can consider below two options for network assistance:  Option 1: Introduce network assistance same as RateMatchPatternLTE-CRS.  Option 2: Do not consider scenario 2 for RAN4 requirements and introduce network assistance for number of CRS ports and vshift. |

Open issues summary

Sub-topic 3-1: Network Assistance Signalling Aspects

Issue 3-1-1: Need of CRS sequence information for LLR weighting

- Agreements in RAN#101-e in the WF R4-2120705:

  - Enable CRS-IM receiver (LLR weighting), below parameters/information needed:

    - The presence of CRS information including: the presence of LTE cell, MBSFN configuration, [CRS muting information] if configured

    - CRS location information including: LTE carrier frequency, bandwidth, v-shift, CRS port number

    - FFS CRS sequence information needed or not which including: Cell ID, [slot number within radio frame information]

  - Companies are encouraged to bring simulation results for LLR weighting with and without CRS sequence. Based on performance comparison, RAN4 plan to draw conclusion whether CRS sequence information needed or not in Jan 2022 RAN4 meeting.

  - The complexity and power consumption impact also need to be considered when RAN4 make decision.

- Companies’ simulation results on LLR weighting with/without CRS sequence info:

  - Apple: For INR and loading values used in Phase 1 evaluation the degrade without CRS sequence information is around 0.5 dB.

  - Intel, E///: LLR weighting with and without CRS sequence have rather same performance

  - China Telecom:

    - LLR weighting with CRS sequence information can achieve 0.3~0.6 dB performance gain over LLR weighting without CRS sequence information, only when there is 1 LTE interference cell per v-shift.

    - When there is CRS signal from 2 LTE neighbour cells on each CRS RE:

      - There is no performance difference between LLR weighting with and without sequence, when UE perform channel estimation for LTE cell 1 with INR = 10.45dB for LLR weighting with sequence.

      - When the UE perform channel estimation for LTE cell 2 with INR = 4.6dB for LLR weighting with sequence, the performance of LLR weighting with sequence is even worse than that of LLR weighting without sequence.

  - HW: LLR weighting with sequence always has better performance than that without sequence but there is still large performance gain over baseline receiver for LLR weighting without sequence

- Companies’ observations on the complexity of LLR weighting with CRS sequence:

  - China Telecom: When performing LMMSE channel estimation to the neighbour cell, LLR weighting with CRS sequence information will need similar UE complexity as CRS-IC.

    - It is questionable whether UE will really implement the LLR weighting with CRS sequence information, since there have been companies raised the UE processing time issue due to the higher complexity of CRS-IC.

- Proposals:

  - Option 1: CRS sequence information is needed for LLR weighting (Apple)

  - Option 2: CRS sequence information is not needed for LLR weighting (Nokia, CMCC, CTC, HW, E///, QC)

- Recommended WF

  - Agree option 2?

Issue 3-1-2: Whether UE needs to be indicated the CRS port number

- Status in RAN#101-e in the WF R4-2120705:

  - Interested companies can bring simulation result based on the assumption without and with knowledge of number of CRS ports

    - 2Tx ports configured

    - Simulated cases:

      - Case 1: UE assuming 4Tx ports

      - Case 2: UE aware of 2Tx ports

      - Case 3: UE blind detection of number of Tx ports

  - For power difference: Interested companies can bring simulation output for power difference between symbols with and without CRS REs) for the detection of number of CRS ports.

- Companies’ simulation results on LLR weighting with/without knowledge of CRS port number:

  - China Telecom: With UE performing detection for the CRS port number, LLR weighting can achieve the same performance compare with the case where UE always know the correct CRS port number.

  - HW: The performance are same for UE has knowledge of CRS ports, always assume 4 ports and CRS port number blind detection.

  - Intel: The number of CRS ports can be rather reliable detected without significant impact on performance.

  - Apple: In the worst case the UE would assume single CRS port, in that case, we observe that with 2 RX the performance degrade is > 1dB and with 4 RX degradation is 0.6 dB.

  - QC: We observe 4-5 dB loss with detection algorithms v/s using network assistance under scenario 1 with link adaptation enabled and SIR = -6dB. Detection algorithms may not perform well when interference is weaker than serving cell PDSCH.

- Companies’ observations on how could UE obtain CRS port number info without NWA:

  - Option 1: By PBCH decoding during measurement gap (Apple)

  - Option 2: By UE detection (Intel, CMCC, CTC, E///)

  - Option 3: UE do not need to know the CRS port number (HW)

- Proposals:

  - Option 1: UE needs to be indicated the CRS port number (Apple, QC)

  - Option 2: Not to consider indicating the CRS port number (Intel, CMCC, CTC, HW, E///, [Nokia])

    - Option 2A: UE can always perform LLR weighting with the assumptions of 4 CRS ports (HW)

    - Option 2B: CRS port number can be assumed blind detected (Intel, CMCC, CTC, E///)

    - Option 2C: Use 1 or 2 CRS ports as default (Nokia)

- Recommended WF

  - TBA

Issue 3-1-3: Assumptions on the CRS port number configuration

- Status in RAN#101-e in the WF R4-2120705:

  - On CRS port number in scenario 1:

    - Option 1: The same number of CRS ports in the serving and neighbouring cells

    - Option 2: The number of CRS ports in the serving and neighbouring cells can be different

    - To be decided in the next meeting

- Proposals:

  - Option 1: Assume the same number of CRS ports in the serving and neighbouring cells (Nokia)

  - Option 2: The number of CRS ports in the serving and neighbouring cells can be different (CMCC, CTC)

    - CMCC, CTC: No need to add the restriction on the CRS port number, since it can be detected by power comparison

- Recommended WF

  - TBA

Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network

- Status in RAN#101-e in the WF R4-2120705:

  - Option 1: By inter-RAT measurement, PBCH decoding

  - Option 2:

    - For scenario 1, by the configuration of serving cell CRS-RM

    - For scenario 2, by the configuration of 7.5KHz shift and inter-RAT MO

- Companies’ observations on related technical aspects:

  - Whether inter-RAT MO can be always configured

    - Option 1: The inter-RAT MO can be always configured if neighbour EUTRA cell exists (CMCC, HW, E///, Intel)

    - Option 2: Up to BS implementation (Nokia, Intel)

  - Whether inter-RAT LTE measurement is performed right after receiving the inter-RAT MO

    - For Inter-RAT MO without any threshold

      - Option 1: The InterRAT measurements shall be done by the UE when enabled by InterRAT MO without any threshold. (Nokia, Intel, E///)

    - For Threshold triggered inter-RAT measurement

      - Option 1: Threshold triggered inter-RAT measurement is used for IDLE mode cell re-selection. (Intel)

  - Whether PBCH decoding is always possible in inter-RAT measurement

    - Option 1: The network is not required to ensure that PBCH is placed in the InterRAT measurement gap. (Nokia, MTK)

    - Option 2: Inter-RAT MG should cover LTE subframe#0. This can be done by BS setting proper gap-offset and MGL. (CMCC, HW, E///, Intel)

- Proposals:

  - For the neighbor cell LTE presence information

    - For scenario 1

      - Option 1: by serving cell CRS-RM configuration or 7.5kHz shift configuration (CMCC, CTC, [HW], E///)

    - For scenario 2

      - Option 1: by inter-RAT MO configuration (CMCC, CTC, [HW], E///, Apple)

  - For the CRS frequency location information (LTE carrier frequency, bandwidth, v-shift)

    - For vshift information for scenario 1 and scenario 2

      - Option 1: Estimate and apply the interference power per vshift, as discussed in Issue 2-1 on LLR weighting implementation (China Telecom, HW, CMCC)

        - CMCC: UE can obtain the power difference, v-shift through power difference detection.

    - For LTE carrier frequency for scenario 2

      - Option 1: by inter-RAT MO configuration (Apple, CMCC, CTC, HW, E///, [Nokia])

    - For LTE bandwidth information for scenario 2

      - Option 1: by power difference detection (CMCC, CTC, HW, E///, [Nokia])

        - QC: It is additional UE complexity to detect LTE center freq and BW, which may decrease overall throughput either due to misdetection or because of frequent inter-RAT measurement objects.

      - Option 2: by LTE PBCH decoding

  - For the CRS sequence information ([Slot number within radio frame], Cell ID) (Still FFS on whether CRS sequence information is needed)

    - Option 1: Cannot be obtained (Apple)

    - Option 2: Slot number within radio frame information can be blind detected, Cell ID cannot be obtained (Intel)

- Recommended WF

  - For the neighbor cell LTE presence information

    - For scenario 1, agree on Option 1 (by serving cell CRS-RM configuration or 7.5kHz shift configuration)

    - For scenario 2, agree on Option 1 (by inter-RAT MO configuration) if feasible?

  - For the CRS frequency location information (LTE carrier frequency, bandwidth, v-shift)

    - For vshift information for scenario 1 and scenario 2: Further discuss

    - For LTE carrier frequency for scenario 2, agree on Option 1 (by inter-RAT MO configuration) if feasible?

    - For LTE bandwidth information for scenario 2: Further discuss

  - For the CRS sequence information: On hold, pending on the conclusion of Issue 3-1-1

Based on the above recommended WF, further discuss:

  - Feasibility of utilizing inter-RAT MO configuration information, including:

    - Whether inter-RAT MO can be always configured

    - Whether inter-RAT LTE measurement is performed right after receiving the inter-RAT MO

  - Whether the option 1 for obtaining vshift information (Estimate and apply the interference power per vshift) is agreeable?

  - How to obtain LTE channel bandwidth information for scenario 2

    - Option 1: by power difference detection

    - Option 2: by LTE PBCH decoding. FFS whether PBCH decoding is always possible in inter-RAT measurement

Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption

- Status in RAN#101-e in the WF R4-2120705:

  - Option 1: Introduce network assistance on neighbour cell LTE configuration

  - Option 2: Do not consider network assistant information

- Proposals:

  - Option 1: Introduce network assistance on neighbour cell LTE configuration (Apple, MTK, Intel, QC, Nokia)

    - Option 1A: NW signaling include LTE presence, location and sequence (Apple)

    - Option 1B (MTK)

      - For scenario 1: physical Cell ID, number of CRS antenna ports.

      - For scenario 2: physical Cell ID, number of CRS antenna ports, bandwidth of the LTE carrier, center of the LTE carrier

    - Option 1C (Intel)

      - For scenario 1: physical Cell ID

      - For scenario 2: physical Cell ID, bandwidth of the LTE carrier, center of the LTE carrier

    - Option 1D (Nokia)

      - Define new RRC signaling for CRS-IM IE, which contains the NW available information from the IE RateMatchPatternLTE-CRS and adds optional information for CRS-muting, (Max) Number of CRS ports, and v-shifts.

      - Configuration of this message does not trigger CRS-RM functionality.

    - Option 1E (QC)

      - Introduce network assistance same as RateMatchPatternLTE-CRS.

    - Option 1F (QC)

      - Introduce network assistance for number of CRS ports and vshift for scenario 1

      - Do not consider scenario 2 for RAN4 requirements

  - Option 2: Do not consider network assistant signalling (CMCC, CTC, HW, E///)

- Recommended WF

  - TBA

Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid

- Status in RAN#101-e in the WF R4-2120705:

  - For scenario 1 and 2, by default, UE follow below assumption of Network configuration for CRS-IM receiver

    - no CRS muting,

    - MBSFN configuration same as serving cell for scenario 1; NO MBSFN configuration for scenario 2

    - Channel bandwidth and centre frequency aligned for the serving and neighbouring cells for scenario 1

  - If above assumption not aligned with NW configuration:

    - Network can inform to UE by NWA signalling. FFS for the details of NWA signalling

      - It’s Network decision whether need to be informed to UE even the network configuration not aligned with default assumption. From network perspective, if such information conveys to UE, network expect UE should not follow the default assumption.

    - FFS whether UE blind detection can be considered as candidate UE receiver. If such UE capability introduced, separate UE capability signalling need to be introduced for UE receiver without blind detection

- Proposals on the detailed signalling design:

  - Option 1 (Intel): NW provides the common information for all UEs within the cell, which includes: CRS muting (for Scenario 1 and 2), MBSFN configuration (for Scenario 1 and 2) and LTE channel bandwidth/center frequency (for Scenario 1). FFS whether to assume the same or different configurations for all neighbouring cells.

  - Option 2 (Nokia): Add information on MBSFN configuration, channel bandwidth and centre frequency, and optional information for CRS-muting, and all interference cells have the same parameters.

- Recommended WF

  - Further discuss whether the signalling is per interference cell or all interference cells have the same parameters, and whether some of the parameters are optional to be signaled.

Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid

- Proposals on blind detection and potential impact of mis-detection:

  - Option 1: Do not consider UE blind detection for the following NW configurations (Intel, QC)

    - Scenario 1: CRS muting, MBSFN configuration and LTE channel bandwidth/center frequency

    - Scenario 2: CRS muting and MBSFN configuration

  - Option 2: Not to consider any misdetection in defining demodulation requirement (E///, Nokia)

- Recommended WF

  - TBA

Sub-topic 3-2: CRS-IM Capability Signalling Aspects

- Agreements in RAN#101-e in the WF R4-2120705:

  - Whether to introduce CRS-IM UE capability signalling

    - Agree to introduce UE capability signaling for CRS-IM.

    - FFS on the details, including the granularity of the capability and other aspects if necessary.

Issue 3-2-1: Granularity of UE CRS-IM Capability

- Proposals

  - Option 1: Introduce granularity of per CC, per band, per band combination (Apple)

    - Apple: UE would need to support this feature only in CCs and bands that are overlapping with LTE spectrum. Proposed UE feature list in R4-2200286.

  - Option 2: Introduce granularity of per UE (CMCC, Intel)

- Recommended WF

  - Need feedback from more companies

Issue 3-2-2: Applicability of UE CRS-IM Capability

- Proposals

  - For 15kHz SCS:

    - Option 1: FR1 only, no FDD/TDD difference (Apple, CMCC)

  - For 30kHz SCS:

    - Option 1: FFS until feasibility to support CRS-IM for 30kHz SCS is confirmed (Apple)

    - Option 2: Define separate UE capability for 30kHz SCS CRS-IM, FR1 only, no FDD/TDD difference (CMCC)

  - For scenario 1 and 2:

    - Option 1: Further discuss whether to introduce single or separate features for Scenario 1 (DSS) and Scenario 2 (Intel)

- Recommended WF

  - Need feedback from more companies

Issue 3-2-3: Other Aspects on UE CRS-IM Capability

- Proposals

  - Nokia:

    - RAN4 to further discuss the content of the UE capability signalling after the level of NWA has been agreed. As minimum, the UE capability signalling for CRS-IM shall indicate if the UE supports CRS-IM

    - If UE indicates CRS-IM capability, the network will assume CRS-IM to be used.

- Recommended WF

  - Need feedback from more companies

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Issue 3-1-2: Whether UE needs to be indicated the CRS port number Issue 3-1-3: Assumptions on the CRS port number configuration Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information - How to obtain LTE channel bandwidth information for scenario 2 Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: - For 30kHz SCS: - For scenario 1 and 2: Issue 3-2-3: Other Aspects on UE CRS-IM Capability |
| China Telecom | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Option 2, according to our simulations and analysis. Issue 3-1-2: Whether UE needs to be indicated the CRS port number Option 2. For the simulation from Apple: - 1-port LTE deployment is not typical. - Detection of CRS ports by simple power comparison is not applied. - Is it assumed that LLR weighting is only applied on the REs for port 0? For the simulation results of 4-5 dB loss from QC, we wonder what are the simulation setup and assumptions. Issue 3-1-3: Assumptions on the CRS port number configuration Option 2. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information, including: - Whether inter-RAT MO can be always configured The option 1 and option 2 from companies seem not conflict with each other. We can assume option 1 for the requirement definition. - Whether inter-RAT LTE measurement is performed right after receiving the inter-RAT MO We wonder why Threshold triggered inter-RAT measurement is only used for IDLE mode. - Whether the option 1 for obtaining vshift information (Estimate and apply the interference power per vshift) is agreeable? Estimate and apply the interference power per vshift is what we did in the simulation. In practical systems, usually the interference exists on different vshift. We don’t need to judge whether the interference exists for one vshift, but just estimate the interference power per vshift. Moreover, considering that the same NWA is provided for UEs in the same cell, we don’t believe accurate NWA information on vshift can be provided for UEs in different locations. - How to obtain LTE channel bandwidth information for scenario 2 Both Option 1 (by power difference detection) and Option 2 (by LTE PBCH decoding) are possible. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Option 2. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid No strong view on the detailed signalling design. Probably it is better to assume all interference cells have the same parameters. Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid Ok with option 1. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Option 2. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: ok with option 1. - For 30kHz SCS: ok with option 2. In addition, 30kHz SCS CRS-IM capability may apply to TDD only. - For scenario 1 and 2: can discuss later |
| Intel | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Based on our understanding, we should not restrict the UE implementation and assume that only LLR weighting without CRS sequence knowledge is considered. From performance point of view, both implementations are rather close. Same time, from complexity point of view, we think that LLR weighting with CRS sequence is lower in comparison to LLR weighting without CRS sequence. Based on our analysis, LMMSE channel estimation of neighbouring cell signal is not required and simple LS channel estimation can be used. Therefore, for LLR weighting without CRS sequence, we just need to apply LS channel estimation and then interference power estimation. Same time, for LLR weighting without CRS sequence, we need to estimate receive signal power, serving signal power and then subtract serving signal power and power of residual interference-plus-noise from receive signal power to estimate the interference power. Issue 3-1-2: Whether UE needs to be indicated the CRS port number Options 1 and 2B are fine for us. Based on our analysis, using of CRS port blind detection does not lead to significant performance degradation. Same time, we understand that blind detection leads to increasing of UE complexity. From our side, we think that this is the only parameter detection of which more or less feasible and can be excluded from NWA signalling part. However, we are also fine to include indication of CRS port number in the NWA signalling. Issue 3-1-3: Assumptions on the CRS port number configuration Support Option 2. It is better not to restrict network structure. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information From UE point of view, it is feasible to use inter-RAT MO to get information about LTE centre carrier frequency. Same time, from Network point of view, we will have certain restriction that for activation of CRS-IM at the UE side, inter-RAT MO should be configured, Therefore, we don’t see big difference between NWA signalling and inter-RAT MO configuration. - How to obtain LTE channel bandwidth information for scenario 2 Based on our understanding, from UE complexity point of view, it is better to consider PBCH decoding. However, from network point of view, it is required to ensure correct measurement gap configuration, which may restrict the scheduling. Based on all above observations, we think that it is better to consider NWA signallling of LTE centre carrier and channel bandwidth instead of relying on Inter-RAT MO and correct measurement gap configurations. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Based on discussion above, we think that introduction of network assistance signalling is required to reduce the complexity of UE processing and the complexity of NW scheduling. We support Option 1A, 1B and 1C. As for 1D, we would like to check whether it is possible to include the list of physical cell IDs instead of v-shifts. In case we assume that NW can configure Inter-RAT MO, which contains the list of physical cell IDs, we think that NW has this information and can provide it. In case we can change v-shifts by physical cell IDs in Option 1D, we are fine to consider it after clarification on Issue 3-1-6. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid We would like to check several questions: Q1: Whether it is reasonable to assume and whether it is guaranteed that all neighbouring cells will have the same configuration of certain parameters (for example, MBSFN configuration)? Q2: In case answer on Q1 is NO and at least one cell has different configuration, what will be signalled to UE in case common IE is used? Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid Support Option 1. Blind detection of these parameters is very complicated. In case blind detection of these will be considered, we need to define the dedicated requirements to verify it. Same time, it is not the main purpose of CRS-IM requirements. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Support Option 2. We think that per UE should be sufficient and will reduce the complexity of capability signalling. In case Option 1 is considered, it means that UE needs to identify the NR CCs which can potentially overlap with LTE CCs and then provide the signalling. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: Option 1. - For 30kHz SCS: Option 1. - For scenario 1 and 2: The outcome of this discussion depends on decision on network assistance signaling. Issue 3-2-3: Other Aspects on UE CRS-IM Capability We are fine with proposal from Nokia. From content point of view, we assume that UE will just inform about support of CRS-IM Rx processing (i.e. similar to all other features related to enhanced processing). |
| CMCC | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting We support Option 2. Based on the discussion of Issue 2-1, both LLR weighting implementation methods don’t need the CRS sequence information. Issue 3-1-2: Whether UE needs to be indicated the CRS port number Option 2B. From our simulation, it shows that UE can acquire the CRS port information through power estimation. Issue 3-1-3: Assumptions on the CRS port number configuration Option 2. There is no need to configure such CRS port number assumption, since UE can acquire the CRS port information through power estimation. Besides, in real network, the CRS ports in the serving and neighbouring cells can be different. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information From our view, inter-RAT MO can be always configured, and it is based on BS implementation. - Whether the option 1 for obtaining vshift information (Estimate and apply the interference power per vshift) is agreeable? For scenario 1 and Scenario 2 with 15kHz SCS, we think it is feasible for UE do the power estimation, acquire the CRS location information ( including v-shift information and CRS ports information) and power difference. - How to obtain LTE channel bandwidth information for scenario 2 From our view, both Option 1 and Option 2(with proper setting of gapset) can be used to obtain the BW information. It can be decided by UE implementation. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption No need to introduce network assistance signalling, unless the practical configuration is mis-aligned with default assumption as last meeting’s agreements. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid We support the proposal from Intel, that NW provides the common information for all UEs within the cell, which includes: CRS muting (for Scenario 1 and 2), MBSFN configuration (for Scenario 1 and 2) and LTE channel bandwidth/center frequency (for Scenario 1). The parameters should be signalled when its configuration is not aligned with default assumption. Therefore, they are optional to be signalled. As for the granularity of signalling, we prefer to assume the different configurations for all neighbouring cells. Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid We think Option 1 and Option 2 are not conflict options. We support both options. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability We support Option 2. First, the CRS-IM algorithm is same for all operating bands. Besides, for Option 1, since all FDD bands support DSS, and TDD bands such as n40, n41, n34, n39, n38 support DSS, it is rather complex for UE to signal the capability. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: - We support Option 1, UE implementation and default assumption is same under FDD and TDD, therefore, no FDD/TDD difference is needed. - For 30kHz SCS: - We support Option 2 and open to discuss. For 30kHz SCS, the UE LLR processing may need larger memory, so, separate UE capability for 30kHz SCS is probably needed. - For scenario 1 and 2: - No need to differentiate Scenario 1 and Scenario 2 with 15kHz. Issue 3-2-3: Other Aspects on UE CRS-IM Capability We are fine with the propoal from Nokia, we can have more discussion when NWA related issues has been agreed. |
| Ericsson | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Support the recommended WF. The process of LLR weighting has been wild discussed during the previous meetings. And apparently, there are different implementations that can not be unified easily. From the analysis, we see that the implementation without CRS sequence works (actually it is our implementation as well). From the simulation results, we see that the performance of the implementation without CRS sequence can achieve good gain which is no worse than having CRS sequence involved. In this case, we can conclude that the CRS sequence is not necessary for doing LLR weighting. Issue 3-1-2: Whether UE needs to be indicated the CRS port number Support 2A and 2B. By observing from the simulation results submitted by different companies, we found that there is no performance difference between knowing CRS port number in advance and obtaining the CRS port number information by detection. Even the detection is wrong (assuming x ports and transmitting y ports in real, x>y), there is no performance degradation either. Hence, we think it is not necessary to inform UE the CRS port number. Issue 3-1-3: Assumptions on the CRS port number configuration We prefer to not set restriction on that. But if we need an assumption for defining requirement, then we support option 1. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information - Whether inter-RAT MO can be always configured Option 1 - Whether inter-RAT LTE measurement is performed right after receiving the inter-RAT MO Option 1 - Whether the option 1 for obtaining vshift information (Estimate and apply the interference power per vshift) is agreeable? Agreeable to us. - How to obtain LTE channel bandwidth information for scenario 2 For scenario 1, it is the same as serving cell. For scenario 2, it can be obtained by power detection. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Option 2. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid Option 2. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Need further check. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: Option1 - For 30kHz SCS: Option 1 - For scenario 1 and 2: FFS Issue 3-2-3: Other Aspects on UE CRS-IM Capability Discuss after the issue of NWA is settled. |
| Apple | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Based on our analysis we observe performance delta with no CRS sequence assumption in INR conditions used for evaluation and requirements. Since the conditions in the field will vary, we cannot determine how much the degrade will be with using without CRS sequence assumption at different INR. We cannot understand how LLR weighting with sequence would be worse than LLR weighting without CRS sequence. Also, the complexity of LLR weighting with CRS sequence is not the same as that of CRS-IC since full LMMSE channel estimation is not required for it. We cannot agree with option 2. The UE processing without CRS sequence is not trivial either since there is some processing and complexity associated with determining the interference power based on power based estimation. CRS sequence-based methods might be less complex and easier to implement in our understanding. Issue 3-1-2: Whether UE needs to be indicated the CRS port number We support option 1. Power based estimation methods only work in certain INR /SIR conditions. They cannot always be guaranteed to work in real deployment. If UE is not indicated the number of CRS ports, it could assume single CRS port in our understanding which is the same as assumption for inter-RAT measurement. UE can blindly detect if possible, but the baseline assumption should be that either the information is provided to the UE by NWA signalling or UE assumes single port transmission. Also, UE may also be able to decode PBCH to get the number of CRS ports, but it cannot be guaranteed since MG might not have LTE PBCH and also its additional complexity for the UE. Issue 3-1-3: Assumptions on the CRS port number configuration We don’t agree with either option. The baseline assumption should be that UE assumes 1 CRS port if no NWA signalling is provided. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information If the inter-RAT MO configuration is used for CRS presence and for carrier freq it might be feasible to use this for UE to perform CRS-IM, but could it be guaranteed that inter-RAT MO is always configured? So, would UE not be required to perform CRS-IM if LTE-MO is no configured? Also, based on only LTE-MO config there would be some further restrictions on what UE assumes for other CRS parameters. - Whether the option 1 for obtaining vshift information (Estimate and apply the interference power per vshift) is agreeable? Power based estimation methods would bring additional complexity to the UE and cannot be guaranteed to provide good performance in all conditions. Hence, we cannot agree with option 1. - How to obtain LTE channel bandwidth information for scenario 2 Again, UE could blindly detect a lot of the LTE parameters but taking practical implementation into account it’s no possible to expect UE to blindly detect everything. UE is either indicated this by NWA otherwise we can consider to de-prioritize scenario 2. PBCH decoding cannot always be guaranteed as mentioned earlier in other issues, and is also additional complexity for UE to decode LTE PBCH. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Yes, network assistance signalling should be introduced. We support option 1A, 1B For option 1D is this applicable to both scenario 1 and scenario 2? If all the info from RateMatchPatternLTE-CRS and Cell _ID are configured, we also support this option. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid Is it guaranteed that all inference cells have the same configuration? If not, then parameters should be per interference cell. We don’t think any of them can be optional to be signalled. Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid We support option 1, not to consider UE blind detection when default NW config assumptions are not valid. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Option 1. Since UE doesn’t need to support this on all bands and CCs it should be able to indicate this per CC, per band, per band combination rather than as a per UE feature. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: Support option 1 - For 30kHz SCS: This depends on feasibility discussion - For scenario 1 and 2: This depends on NWA discussion Issue 3-2-3: Other Aspects on UE CRS-IM Capability We would like to understand what additional UE capability signalling is expected based on the NWA. |
|  | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Option 2 Based on our simulation results, the performance gain for LLR weighting without CRS sequence is enough. @ Intel: We don’t think LLR weighting with CRS sequence is lower in comparison to LLR weighting without CRS sequence. Based on our understanding, there are two possible ways to perform LLR weighting without CRS sequence: - Estimate the power difference for RE with CRS and RE without CRS - Estimate the power difference for total power of CRS RE, serving cell and noise. Obviously, first way is the simplest implementation and has lowest complexity. For second way, power of serving cell and noise can be directly derived from channel estimation and noise estimation which is always performed in demodulation processing, so, no extra complexity will be introduced. Based on our simulation results, two ways have same performance in two scenarios. However, for LLR weighting with sequence, we can’t exclude the LMMSE channel estimation since it is up to UE implementation. In our simulation results in our Tdoc, LMMSE channel estimation is used and it shows significant performance gain. Issue 3-1-2: Whether UE needs to be indicated the CRS port number Option 2. Based on the simulation results we can observe that CRS port information has no impact on performance and it is the safest way that UE always assume 4 CRS ports. Issue 3-1-3: Assumptions on the CRS port number configuration As discussed in Issue 3-1-3, always assuming 4 CRS ports is the most feasible way to reduce the power consumption and guarantee the performance. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Whether inter-RAT MO can be always configured Option 1. Based on our understanding, inter-RAT MO should be configured if there is neighbouring EUTRA cell deployed and serious interference from LTE. Now the test of CRS-IM is trying to mitigate the interference from LTE by setting proper INR1=10.45dB, INR2=4.45dB value, this test setup is aligned with the condition that inter-RAT MO should be configured. - Whether inter-RAT LTE measurement is performed right after receiving the inter-RAT MO Based on our understanding on the core specification as extracted below, if UE is configured with following parameters, UE will perform inter-RAT measurement: - Inter-RAT MO, - Reporting configurations - Measurement identities - Quantity configurations - Measurement gaps. The details are captured in clause 5.5.3 from 38.331. As for the concern that UE may not perform inter-RAT measurement when serving cell’s RSRP is lower than certain threshold, it can be avoided by not configuring IE s-MeasureConfig. - Whether PBCH decoding is always possible in inter-RAT measurement This can be guaranteed by setting MGL to 6ms and gap offset to 0 so that PBCH can always be detected within the measurement gap. - How to obtain LTE channel bandwidth information for scenario 2 UE can estimate CRS power per serval RBs Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Based on our discussions above, we support option 2 Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid We propose to leave it up to UE blind detection Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid Option 2 Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Option 2, option 2 is the same way as LTE and is the simplest way for defining capability signalling. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: Option 1 For 30kHz SCS: Option 2 - For scenario 1 and 2: Option 1 |
| Nokia, Nokia Shanghai Bell | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Based on companies’ simulation results and observations, using CRS Sequence for LLR Weighting does not provide enough improvements compared to complexity introduced to mandate using the CRS Sequence. Support option 2. Issue 3-1-2: Whether UE needs to be indicated the CRS port number Even if performance is degraded when selecting the wrong number of CRS ports, there should still be an overall improvement compared to CRS-RM. Based on this, the default setting for the number of CRS ports, should be defined based on the most used configuration. We support any of option 2. If agreed that the difference in performance when using 4 ports as default does not have significant impact, we can also support using 4 ports instead of 1 or 2 CRS ports as default. Issue 3-1-3: Assumptions on the CRS port number configuration If issue 3-1-2 is decided, then no agreement is required for issue 3-1-3 anymore. Based on companies observations in Issue 3-1-2 where using wrong CRS-port setting for LLR Weighting does not significantly impact the performance, however we still maintain that option 1 is practically more relevant. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information We still feel Option 2 is correct, however no agreement is required to be reached, as this is only for addition observations on technical aspects. - How to obtain LTE channel bandwidth information for scenario 2 Both option 1 and 2 are fine. PBCH decoding will not always be possible in inter-RAT measurements, however no agreement is required to be reached, as this is only for addition observations on technical aspects. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Our understanding of the agreement in the WF “- Network can inform to UE by NWA signalling. FFS for the details of NWA signalling” dictates that a message shall be defined for this purpose. Our compromise proposal is to provide as much support to the UE as possible, assuming a network wide static configuration. With our proposal, it will be possible for the network to provide some degree of information about the difference in network configuration compared to the agreed default settings. We have later found, that there might be differences in the understanding of the agreement in the WF, meaning that it is not yet agreed if such a IE should be defined or not. We would like feedback/comments from companies to confirm the understanding of the agreement. We would still prefer a solution without NWA, however if this is not possible to agree on this, we would appreciate feedback from companies regarding our compromise proposal. We have chosen this compromise proposal as the information is already available in most basestation implementations and RRC based NWA avoids c-plane complications. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid Adding information per cell will go into the direction of fully dynamic NWA, hence any level of NWA should be based on network wide configuration (i.e., all interferes have the same parameters). Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid According to prior agreement if the default assumptions are not valid and BS send the not-valid flag, then the BS assumes the UE is not using CRS-IM. If UE still enables CRS-IM then we assume it can blind detect all required parameters as it has judged the interference to be strong enough to warrant this step. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability It is not required to have declaration per band combination, as the declaration should be valid for all overlapping spectrum/bands. As such we can agree to a granularity of “per UE”, but with the additional constraint of only “supporting CRS-IM on CCs and band that are overlapping with LTE spectrum”. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: - With our proposal for the granularity the applicability question does not arise. UE capabilities are appliable whenever they are signalled. - For 30kHz SCS: - Option 1. - For scenario 1 and 2: In case UEs might support only one scenario for CRS-IM, it would make sense to indicate separate support. We would like comments from UE vendors on this proposal. - This depends on NWA discussion. Issue 3-2-3: Other Aspects on UE CRS-IM Capability We appreciate the feedback from companies regarding our proposed IE. In our current understanding, there should only be a single bit to indicate support of CRS-IM, however the current discussion on NWA content may lead to more. For example, support of blind detection for certain parameters. |
| MediaTek | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-2: Whether UE needs to be indicated the CRS port number We support Option 1. Issue 3-1-3: Assumptions on the CRS port number configuration We support Option 2 that no restriction for CRS ports for the neighboring cells. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information We share the same view with Apple. It might be possible to use inter-RAT Mo to indicate the presence of LTE and carrier frequency. However, could it be guaranteed that inter-RAT MO is always configured? Can we agree that UE will not need to perform CRS-IM if LTE MO is not configured? - Whether the option 1 for obtaining vshift information (Estimate and apply the interference power per vshift) is agreeable? We understand that operators, infra-vendors and chip venders have their own preference. In RAN4, we do lots of simulation to define requirements. However, for almost all cases, we are focusing on some specific receiver algorithm. For example, for intra-cell MMSE-IRC receiver, we will assume UE knows all configurations, such as number of layers, CDM group, DMRS configuration, MCS, etc. Indeed, UE does know above information exactly through explicitly singalling, such as RRC and/or PDCCH. We are only focusing on DMRS processing. However, for CRS-IM, except noise power estimation for LLR weighting, we are now asking UE to detect lots of static network information, such as v-shift and LTE channel bandwidth. Even if UE can detect these information by itself, it will increase the complexity and power consumption of UE. We agree that we can also have lots of simulations to verify the performance of UE detection capability for v-shift or LTE channel bandwidth. However, can it be guaranteed that our simulation assumptions cover some corner cases? Why do we just inform the UE with NW setting explicitly? Why do we take the risk that UE has wrong detection for the NW setting? - How to obtain LTE channel bandwidth information for scenario 2 There is a concern that PBCH cannot be guaranteed transmitted in inter-RAT measurement. Hence, we propose to have network signalling for LTE bandwidth in scenario 2. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption We support to introduce network assistance signalling. We also support Option 1D. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid If it cannot be guaranteed that all cells are in the same configuration. NW should provide signalling for each interference cell. Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid Support Option 1. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Prefer Option 1. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: Support Option 1. - For 30kHz SCS: Support Option 1. Depends on discussion for feasibility. - For scenario 1 and 2: Support Option 1. |
| Qualcomm | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Support Option 2. Issue 3-1-2: Whether UE needs to be indicated the CRS port number Support Option 1. As mentioned in our paper, blind detection does not work well when interference is lower than serving cell. Most of the companies’ simulation results only consider the scenario being considered in RAN4 where interference is stronger than the serving cell. But this is not the only scenario that will happen in the field. Based on operators’ measurements submitted in the plenary in the beginning of this WI, they observed the performance loss due to CRS interference in all regimes, not just cell-edge. Issue 3-1-3: Assumptions on the CRS port number configuration We need clarification for this issue. Is this for actual network deployment or just for RAN4 simulations? If it is for NW deployment, it seems both Option 1 and Option 2 are possible and that is why number of CRS ports need to be indicated to the UE via NWA. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - Feasibility of utilizing inter-RAT MO configuration information We have below concerns for utilizing IRAT MO Configuration: - NW is not required to always configure IRAT MO. If NW doesn’t configure it, what is the expected UE behavior? Does UE disable CRS-IM? - Even if NW configures IRAT MO, it is not obligated to guarantee PBCH within Measurement period. - Even if NW configures IRAT MO and guarantees PBCH within measurement period, UE is not obligated to decode PBCH because RRM requirements for IRAT measurements do not include PBCH decoding. So, if UE decides to decode PBCH on its own, it may have to violate RRM requirements depending on UE implementation. Does UE get a waiver on passing those requirements in that case? - Configuring frequent IRAT MOs for all UEs will reduce overall system throughput. - Decoding PBCH at every IRAT MO will increase UE complexity and power consumption. - How to obtain LTE channel bandwidth information for scenario 2 Power difference detection has below issues: - It is not reliable when interference is lower than signal. - UE will have to compare the powers between two symbols for each PRB, which is more complex and most likely, UEs will opt out of doing this and assume something which is not always true. For LTE PBCH decoding, we have similar concerns as we mentioned for IRAT MO above. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Based on the concerns provide for previous issues, we prefer to introduce NWA. As a compromise, we already provided Option 1F where required NWA is minimal for scenario 1. Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid We prefer it to be per interfering cell because different cells could potentially have different config. Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid From UE implementation perspective, blind detection will be very expensive because there are so many hypotheses to run. So, we prefer no blind detection. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability Prefer Option 1 because UE doesn’t need to support it for all BCs. Also, as we increase number of CCs, UE may not be able to support CRS-IM for all CCs. Issue 3-2-2: Applicability of UE CRS-IM Capability - For 15kHz SCS: Support Option 1. - For 30kHz SCS: If it is confirmed to be feasible, we prefer this to be a separate capability. - For scenario 1 and 2: This is dependent on how much NWA is agreed. If UE needs to do additional processing for scenario 2 after NWA agreement, it is preferable to consider it as separate feature. Issue 3-2-3: Other Aspects on UE CRS-IM Capability It depends on NWA outcome. |
| ZTE | Sub-topic 3-1: Network Assistance Signalling Aspects Issue 3-1-1: Need of CRS sequence information for LLR weighting Option 2 is ok. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network How to obtain LTE channel bandwidth information for scenario 2 Option 2. For scenario 2, power difference detection is enough. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption We support option 2. Sub-topic 3-2: CRS-IM Capability Signalling Aspects Issue 3-2-1: Granularity of UE CRS-IM Capability We support option 2.We think that the capability should for per UE. Based on our outstanding, different UE have different capabilities. It depend on UE. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 3-1 | Issue 3-1-1: Need of CRS sequence information for LLR weighting Summary of technical observations: - Companies’ observations on the performance gain - Apple: For INR and loading values used in Phase 1 evaluation the degrade without CRS sequence information is around 0.5 dB. - Intel, E///: LLR weighting with and without CRS sequence have rather same performance - China Telecom: - When there is 1 LTE interference cell per v-shift, LLR weighting with CRS sequence information can achieve 0.3 ~ 0.6 dB performance gain compared to without CRS sequence. - When there is CRS signal from 2 LTE neighbour cells on each CRS RE, LLR weighting with sequence has the same or worse performance compared to without sequence, when UE perform channel estimation for INR = 10.45dB and INR = 4.6dB respectively. - HW: LLR weighting with sequence always has better performance than that without sequence but there is still large performance gain over baseline receiver for LLR weighting without sequence - Companies’ observations on the complexity: - CTC, HW, Nokia: LLR weighting with CRS sequence is more complex than without sequence - China Telecom: When performing LMMSE channel estimation to the neighbour cell, LLR weighting with CRS sequence information will need similar UE complexity as CRS-IC. - HW: We can’t exclude the LMMSE channel estimation since it is up to UE implementation. LMMSE channel estimation shows significant performance gain. - Apple, Intel: The complexity of LLR weighting with CRS sequence can be same or lower than without sequence - Intel: LMMSE channel estimation of neighbouring cell signal is not required and simple LS channel estimation can be used. Summary of round 1 discussion: - Option 1: CRS sequence information is needed for LLR weighting (Apple) - Option 2: CRS sequence information is not needed for LLR weighting (Nokia, CMCC, CTC, HW, E///, QC, Nokia, ZTE) - Option 3: Do not restrict the UE implementation on whether CRS sequence is needed (Intel) GTW Agreement: General agreement for NWA signalling of CRS-IM receiver - RRC based NWA signalling shall be introduced. The details up to RAN2 with necessary input from RAN4. Not restrict UE implementation, CRS sequence not needed from baseline receiver assumption for defining RAN4 minimum performance requirements By default, Cell ID information is not needed from RAN4 performance requirements aspect; Cell ID can be included into NWA signalling as optional. - FFS the maximum number of cell ID information Issue 3-1-2: Whether UE needs to be indicated the CRS port number Summary of observations on how could UE obtain CRS port number info without NWA: - Companies’ observations based on simulation results: - Option 1: By PBCH decoding during measurement gap (Apple) - Option 2: By UE detection (Intel, CMCC, CTC, E///) - Option 3: UE do not need to know the CRS port number, and do LLR weighting assuming 4 ports (HW) Summary of round 1 discussion: - Option 1: UE needs to be indicated the CRS port number (Apple, QC, Intel, MTK) - Option 2: Not to indicate the CRS port number (Intel, CMCC, CTC, HW, E///, Nokia) - Option 2A: UE can always perform LLR weighting with the assumptions of 4 CRS ports (HW, E///, [Nokia]) - Option 2B: UE always assume 1 or 2 CRS ports (Nokia) - Option 2C: CRS port number can be assumed blind detected (Intel, CMCC, CTC, E///, Nokia, ZTE) GTW Agreement: From RAN4 minimum performance requirements aspect, UE follow below default assumption without blind detection as baseline assumption - 4 CRS ports for scenario 2 - Aligned with serving cell for scenario 1 By default, number of CRS ports no need to be informed via signalling with following default assumption from RAN4 performance requirements aspect Number of CRS ports information can be included into NWA signalling (optional) Issue 3-1-3: Assumptions on the CRS port number configuration Summary of round 1 discussion: - Option 1: Assume the same number of CRS ports in the serving and neighbouring cells in scenario 1 (Nokia, E///) - Option 2: The number of CRS ports in the serving and neighbouring cells can be different, i.e., no restriction to the CRS port number between serving and interference cells (CMCC, CTC, Intel, MTK, QC, MTK, E///) - Option 3: UE assume a certain number of CRS ports if no NWA signalling is provided (Apple, HW) - Option 3a: Assumes 1 port (Apple) - Option 3b: Assume 4 ports (HW) - Moderator’s Recommendation - No more discussion based on the agreements in Issue 3-1-2. Issue 3-1-4: How could UE obtain the identified parameters if not signalled by the network - For scenario 1 and 2, whether the option 1 for obtaining vshift information (Estimate and apply the interference power per vshift) is agreeable? Summary of round 1 discussion: - Option 1: Estimate and apply the interference power per vshift, as discussed in Issue 2-1 on LLR weighting implementation (China Telecom, HW, CMCC, E///, ZTE, Intel) - Option 2: Power based estimation methods would bring additional complexity to the UE and cannot be guaranteed to provide good performance in all conditions (Apple, MTK, QC) GTW Agreement: The baseline assumption: No need to introduce NWA signaling for v-shift information - Feasibility of utilizing inter-RAT MO configuration information for scenario 2 Summary of round 1 discussion: - Whether inter-RAT MO can be always configured - Option 1: The inter-RAT MO can be always configured if neighbour EUTRA cell exists (CMCC, HW, E///, Intel, CTC) - Option 2: Up to BS implementation (Nokia, Intel) - Option 3: UE will not be required to perform CRS-IM if LTE-MO is not configured (Apple, MTK, [QC]) - Whether inter-RAT LTE measurement is performed right after receiving the inter-RAT MO - For Inter-RAT MO without any threshold - Option 1: The InterRAT measurements shall be done by the UE when enabled by InterRAT MO without any threshold. (Nokia, Intel, E///) - For Threshold triggered inter-RAT measurement - Option 1: Threshold triggered inter-RAT measurement is used for IDLE mode cell re-selection. (Intel) - Option 2: Threshold triggered inter-RAT measurement for other modes cannot be precluded (CTC) GTW Agreement: From RAN4 minimum performance requirements aspect, it’s not required to blind detect LTE carrier frequency information for CRS-IM receiver baseline assumption. The information can be awared by following possible ways: - For scenario 2, inter-RAT MO configuration information (LTE cell presence and carrier frequency) can be utilized to perform CRS-IM if configured by NW. - LTE cell carrier frequency information can be informed to UE by NWA signalling for scenario 2 (optional) If such information not conveyed to UE, UE not expected to enable CRS-IM receiver. - How to obtain LTE channel bandwidth information for scenario 2 Summary of round 1 discussion: - Option 1: by power difference detection (CMCC, CTC, HW, E///, Nokia, [ZTE]) - Option 2: by LTE PBCH decoding (Intel, CMCC, Nokia) - Option 3: Cannot be obtained without NWA (Apple, MTK, QC) - Moderator’s Recommendation for round 2 discussion - Similar with carrier frequency, companies to check whether the following can be agreeable: - For scenario 2, LTE channel bandwidth information can be awared by following possible ways: - UE can use PBCH decoding to obtain channel bandwidth information for CRS-IM if PBCH is within the configured measurement gap - LTE channel bandwidth information can be informed to UE by NWA signalling (optional) - If such information not conveyed to UE, UE is not expected to perform CRS-IM appropriately. - In the next meeting, discuss whether the test requirement for the following schemes can be the same: - Scheme#1: CRS-IM with Inter-RAT MO configured and perform PBCH decoding - Scheme#2: CRS-IM with NWA signaling - If same test requirement can be applied, further discuss the following test setup for scenario 2 in the next meeting: - Option 1: Define one set of test setup with both Inter-RAT MO and the new NMA signaling configured by the network - Option 2: Define 2 sets of test setup: 1) Only Inter-RAT MO is configured, and; 2) Only the new NWA signaling is configured. Issue 3-1-5: Whether to introduce network assistance signalling, under the default NW configuration assumption Summary of round 1 discussion: - Option 1: Introduce network assistance on neighbour cell LTE configuration (Apple, MTK, Intel, QC, Nokia) - Option 2: Do not consider network assistant signalling (CMCC, CTC, HW, E///, Nokia, ZTE) - Moderator’s Recommendation - Already covered by the previous issues, and no need to be further discussed Issue 3-1-6: Signalling design when the default NW configuration assumptions are not valid Summary of round 1 discussion: - Whether the signalling is per interference cell or all interference cells have the same parameters - Option 1: Assume all interference cells have the same parameters (CTC, Nokia) - Option 2: Per interference cell if it is not guaranteed that all inference cells have the same configuration (Apple, Intel, MTK, QC) - Whether some of the parameters are optional to be signaled: - Option 1: All parameters are optional (CMCC) - Moderator’s Recommendation - Further discuss Issue 3-1-7: Blind detection when the default NW configuration assumptions are not valid Summary of round 1 discussion: - Option 1: Do not consider UE blind detection for the following NW configurations (Intel, QC, CTC, CMCC, Apple, MTK) - Scenario 1: CRS muting, MBSFN configuration and LTE channel bandwidth/center frequency - Scenario 2: CRS muting and MBSFN configuration - Option 2: Not to consider any misdetection in defining demodulation requirement (E///, Nokia, CMCC, HW) - Nokia: By receiving the not valid flag from BS, if UE still enables CRS-IM then we assume it can blind detect all required parameters as it has judged the interference to be strong enough to warrant this step. - Tentative agreements - Not consider UE blind detection and misdetection of the following NW configurations - Scenario 1: CRS muting, MBSFN configuration and LTE channel bandwidth/center frequency - Scenario 2: CRS muting and MBSFN configuration |
|  | Issue 3-2-1: Granularity of UE CRS-IM Capability Summary of round 1 discussion: - Option 1: Introduce granularity of per CC, per band, per band combination (Apple, MTK, QC) - Option 2: Introduce granularity of per UE (CMCC, Intel, CTC, HW, ZTE) - Option 3: Introduce granularity of per UE, but only applicable for the bands that are overlapping with LTE spectrum (Nokia) - E///: Need further check - Moderator’s Recommendation - Further discuss Issue 3-2-2: Applicability of UE CRS-IM Capability Summary of round 1 discussion: - For 15kHz SCS: - Option 1: FR1 only, no FDD/TDD difference (Apple, CMCC, CTC, Intel, E///, HW, MTK, QC) - Option 2: UE capabilities are applicable whenever they are signaled (Nokia) - For 30kHz SCS: - Option 1: FFS until feasibility to support CRS-IM for 30kHz SCS is confirmed (Apple, Intel, E///, Nokia, MTK) - Option 2: Define separate UE capability for 30kHz SCS CRS-IM, FR1 only, no FDD/TDD difference (CMCC, CTC, HW, QC if 30kHz SCS confirmed feasible) - Option 3: May apply to TDD only (CTC) - For scenario 1 and 2: - Option 1: Further discuss whether to introduce single or separate features for Scenario 1 (DSS) and Scenario 2 (Intel, E///, Apple, HW, Nokia, MTK, QC) - Option 2: Not to differentiate Scenario 1 and Scenario 2 with 15kHz (CMCC) - Moderator’s Recommendation - Further discuss Issue 3-2-3: Other Aspects on UE CRS-IM Capability Summary of round 1 discussion: - Nokia, Intel, CMCC: - RAN4 to further discuss the content of the UE capability signaling after the level of NWA has been agreed. As minimum, the UE capability signaling for CRS-IM shall indicate if the UE supports CRS-IM - If UE indicates CRS-IM capability, the network will assume CRS-IM to be used. - Apple: What additional UE capability signaling is expected based on NWA? - E///, QC: Further discuss - Moderator’s Recommendation - Further discuss |

Discussion on 2nd round

Topic #4: Test setup for CRS-IM

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201419 | Ericsson | Proposal 1: Reuse the INR value from phase I and do not include other INR values in phase II Proposal 2: Not to consider other interference loading level Proposal 3: Only cover 4 LTE CRS ports for defining requirements |
| R4-2200275 | Apple Inc | Proposal #1: Introduce requirements for scenario 2 only if the necessary network assistance is introduced, otherwise do not introduce requirements scenario 2. Proposal #2: Do not introduce requirements with lower INR levels with LLR weighting. Proposal #3: Introduce requirements for CRS-IM with 4 CRS ports only |
| R4-2200519 | Intel Corporation | Proposal 1: Keep INR1 = 10.45 dB and INR2 = 4.6 dB as the only INR values for CRS-IM requirements definition in Rel-17 in case of 20% interference PDSCH loading. Proposal 2: Further discuss the following interference PDSCH loading assumptions • Option 1: Consider only 20% interference PDSCH loading • Option 2: Consider 30% interference PDSCH loading for Scenario 1 and 20% interference PDSCH loading for Scenario 2 Proposal 3: Further discuss the following settings for number of CRS ports • Option 1: Consider only 4 CRS ports • Option 2: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2. |
| R4-2200803 | CMCC | Proposal 3: For LTE CRS port, cover 2 and 4 CRS ports. |
| R4-2200900 | China Telecom | Proposal 1: Add one set of INRs and loading levels for the CRS-IM testing to ensure that UE will turn on CRS-IM in different INR conditions. Select one of the combinations that achieves about 1dB performance gain over the reference scheme: • Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB • Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB • Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB Proposal2: Cover 2 and 4 CRS ports for the neighbour cell LTE. |
| R4-2200983 | Huawei, HiSilicon | Proposal 1: Consider INR1=10.45dB and INR2=4.6dB Proposal 2: Only consider 20% loading level. Proposal 3: Only consider 4 CRS ports |
| R4-2201601 | Nokia, Nokia Shanghai Bell | General Test Setup Observation 1: It has been agreed to support a default assumption for the needed parameters with no special relation to scenario 1 or 2. Proposal 1: At this point, RAN4 should not consider defining corresponding demod requirements for scenario 2. Interference power level Observation 2: The already agreed set of INR values is sufficient for minimum performance requirement coverage as the minimum performance requirements are assured with a single profile which was already agreed. Proposal 2: Do not define any additional set of INR values. PDSCH loading level on interference cell Observation 3: Low loading levels represent a worst case for the CRS-IM feature as IM is applied to resources that do not experience interference (assuming the algorithm does not include detection of such cases). High loading levels can mask the downsides of simple CRS-IM implementations, but also highlight the performance increase potential of CRS-IM. Both scenarios should be included in testing. Proposal 3: RAN4 to also include a loading level of 80%. Common parameters for target and interfering cells Observation 4: 2 CRS ports are the most common deployment (option 3). Observation 5: Including 4 CRS ports in addition to 2 CRS ports (option 2) is not in our view required but can be an option, if requested by companies. Proposal 4: Both option 2 and option 3 are acceptable. |

Open issues summary

Sub-topic 4-1: Test setup for 15 kHz SCS scenario

Issue 4-1-1: Test scenario

- Agreements in RAN#101-e in the WF R4-2120705

  - Both scenario 1 and scenario 2 are included in the WID.

    - To perform CRS-IM, if the required parameters specific to scenario 2 cannot be obtained by UE (either by UE detection or NWA signalling), RAN4 would re-consider whether to define the corresponding demod requirements for scenario 2.

- Proposals

  - Proposal 1: Introduce requirements for scenario 2 only if the necessary network assistance is introduced, otherwise do not introduce requirements scenario 2 (Apple)

  - Proposal 2: FFS on the requirement definition for scenario 2 (Nokia)

- Recommended WF

  - First focus on the related discussion in Topic #3-1, and no comment for this issue in round 1.

Issue 4-1-2: Interference power level

- Status in RAN#101-e in the WF R4-2120705:

  - INR1 = 10.45 dB and INR2 = 4.6 dB will be used as the INR for defining requirements.

  - FFS other INR value can be included.

    - Option 1 for the other INR value: Add one set of INRs with smaller INR values where reasonable CRS-IM gain over the reference scheme can still be shown, for example, select INR values that achieve ~1dB CRS-IM gain over the reference scheme.

    - Proponents for option 1 are encouraged to provide exact numbers on the additional interference INR/loading level.

- Proposals

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (China Telecom)

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB (Ericsson, Apple, HW, Nokia)

  - Option 3: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB in case of 20% interference PDSCH loading (Intel)

- Recommended WF

  - TBA

Issue 4-1-3: PDSCH loading level

- Status in RAN#101-e in the WF R4-2120705:

  - 20% loading level with full PRB allocation will be used for defining requirements.

  - FFS other interference loading level can be included.

- Proposals

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (China Telecom)

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider 20% PDSCH loading level (Ericsson, HW, Intel)

  - Option 3: Consider 30% interference PDSCH loading for Scenario 1 and 20% interference PDSCH loading for Scenario 2 (Intel)

  - Option 4: Also include 80% loading (Nokia)

- Recommended WF

  - TBA

Issue 4-1-4: Tx antenna and LTE CRS port number

- Status in RAN#101-e in the WF R4-2120705:

  - Option 1: Only cover 4 CRS ports

  - Option 2: Cover 2 and 4 CRS ports

  - Option 3: Only cover 2 CRS ports

- Companies’ observations:

  - Nokia: 2 CRS ports are the most common deployment

  - China Telecom: 2 CRS ports is the scenario in our deployment.

  - Intel: CRS-IM performance benefits for Scenario 2 (i.e. NR + LTE) with 2 CRS ports are less than 1 dB.

- Proposals

  - Option 1: Only cover 4 CRS ports (Ericsson, Apple, HW, Intel)

  - Option 2: Cover 2 and 4 for CRS ports (China Telecom, CMCC, Nokia)

    - Option 2A: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2 (Intel)

  - Option 3: Only cover 2 CRS ports (Nokia)

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-1: Test scenario Note: No comment in round 1. Issue 4-1-2: Interference power level Issue 4-1-3: PDSCH loading level Issue 4-1-4: Tx antenna and LTE CRS port number |
| China Telecom | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-2: Interference power level Option 1. As discussed, we think it is important to verify that UE with CRS-IM can still show performance gain under smaller INR values, to ensure that CRS-IM is used in different INR conditions, and to ensure this CRS-IM feature can benefit more UEs in different locations of the cell. In addition, the other loading level and INR levels are from the LTE CRS-IM TR 36.863, which can be directly used. If companies are hesitant to consider option 1, we can first encourage simulation results in the next meeting, and then make decision. Issue 4-1-3: PDSCH loading level Option 1. Same comment as for Issue 4-1-2. Issue 4-1-4: Tx antenna and LTE CRS port number Option 2. We can use different CRS port number in the tests with different INR levels as discussed in Issue 4-1-2. |
| Intel | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-2: Interference power level For scenario 1 we are fine to consider Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB. Same time we think that it will be redundant to have two different sets of INR values and loading levels for same scenario (1 or 2). Therefore, in case we want to consider another INR value for requirements definition, we suggest to replace the baseline configuration. One of the options from our side which looks feasible from performance verification point of view is: - 30% interference PDSCH loading and INR 1 = 9.69 dB, INR 2 = 3.7 dB for Scenario 1 and 20% interference PDSCH loading and INR1 = 10.45 dB and INR2 = 4.6 dB for Scenario 2 Issue 4-1-3: PDSCH loading level Same comment as for Issue 4-1-2. Issue 4-1-4: Tx antenna and LTE CRS port number Based on our performance analysis it is better to consider Option 1 or 2A. |
| CMCC | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-1: Test scenario Note: No comment in round 1. Issue 4-1-2: Interference power level For 20% loading level, take Option 2 or Option 3. We are fine with further check CRS-IM under 30% loading level. Issue 4-1-3: PDSCH loading level Same comment as for Issue 4-1-3 Issue 4-1-4: Tx antenna and LTE CRS port number We support Option 2. It can verify whether UE has the capability to detect the CRS port number, besides, in real network, both 2 and 4 CRS ports can be used. |
| Ericsson | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-1: Test scenario Note: No comment in round 1. Issue 4-1-2: Interference power level E: We prefer to only consider the configuration of: INR1 = 10.45 dB and INR2 = 4.6 dB with 20% loading level only. Issue 4-1-3: PDSCH loading level E: Same as 4-1-2. Issue 4-1-4: Tx antenna and LTE CRS port number E: Option 1. We can observe more interference when configuring 4 ports in comparison to 2 ports. And, we think it is enough to only consider 4 CRS ports. |
| Apple | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-1: Test scenario Note: No comment in round 1. Issue 4-1-2: Interference power level For the combinations listed in option1, with loading of 30%, the INR values are not significantly smaller than what we have assumed and if higher loading is assumed, the gains from LLR weighing would be smaller. We don’t see strong motivation for additional INR levels. Proposal from Intel is a reasonable compromise, but scenario 2 is still TBD in our understanding. Issue 4-1-3: PDSCH loading level We don’t see the necessity to use higher interference loading as it would reduce the gains from LLR weighting. Issue 4-1-4: Tx antenna and LTE CRS port number Option 1. |
| Huawei | Issue 4-1-2: Interference power level - Option 2, we don’t see any necessity to introduce other INRs and loading Issue 4-1-3: PDSCH loading level - Option 2, we don’t see any necessity to introduce other INRs and loading Issue 4-1-4: Tx antenna and LTE CRS port number - Option 1, we don’t see any necessity to introduce other CRS port numbers. |
| Nokia, Nokia Shanghai Bell | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-2: Interference power level Support Option 2. Can also support Option 3 in case only 20% interference PDSCH loading is agreed. Issue 4-1-3: PDSCH loading level We understand that 80% interference level will reduce the observed gain for CRS-IM substantially, we would still request comments on the usefulness of such a additional scenario. As baseline we can support either 20% or 30%. Issue 4-1-4: Tx antenna and LTE CRS port number We support option 3 but can also accept option 2. |
| MediaTek | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-2: Interference power level Support Option 2. Issue 4-1-3: PDSCH loading level Support Option 2. Issue 4-1-4: Tx antenna and LTE CRS port number Support Option 1. |
| Qualcomm | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-2: Interference power level Prefer Option 2. No need to test several INR levels. Issue 4-1-3: PDSCH loading level Prefer Option 2. Higher loading will decrease CRS-IM gains. Issue 4-1-4: Tx antenna and LTE CRS port number Prefer Option 1. 4 CRS ports is the worst case. |
| ZTE | Sub-topic 4-1: Test setup for 15 kHz SCS scenario Issue 4-1-2: Interference power level Option 3. Our simulation was based on this interference power level. Issue 4-1-3: PDSCH loading level Same as Issue 4-1-2. Issue 4-1-4: Tx antenna and LTE CRS port number We support option 2. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 4-1: Test setup for 15 kHz SCS scenario | Issue 4-1-1: Test scenario Summary of round 1 discussion: - Following moderator’s suggestion, no comment for this issue in round 1. Issue 4-1-2: Interference power level Summary of round 1 discussion: - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (China Telecom) - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB - Option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB (Ericsson, Apple, HW, Nokia, MTK, QC) - Option 3: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB in case of 20% interference PDSCH loading (Intel, Nokia, ZTE) - Option 4: 30% PDSCH loading and INR 1 = 9.69 dB, INR 2 = 3.7 dB for Scenario 1 and 20% interference PDSCH loading and INR1 = 10.45 dB and INR2 = 4.6 dB for Scenario 2 (Intel, Apple for scenario 1) - Moderator’s Recommendation - Further discuss in the next meeting Issue 4-1-3: PDSCH loading level Summary of round 1 discussion: - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (China Telecom) - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB - Option 2: Only consider 20% PDSCH loading level (Ericsson, HW, Intel, Apple, MTK, QC, ZTE) - Option 3: Consider 30% interference PDSCH loading for Scenario 1 and 20% interference PDSCH loading for Scenario 2 (Intel) - Option 4: Also include 80% loading (Nokia) - Option 5: Further check 30% loading level (CMCC) - Option 6: Either 30% or 20% (Nokia) - Moderator’s Recommendation - Further discuss in the next meeting Issue 4-1-4: Tx antenna and LTE CRS port number Summary of round 1 discussion: - Option 1: Only cover 4 CRS ports (Ericsson, Apple, HW, Intel, MTK, QC) - Option 2: Cover 2 and 4 for CRS ports (China Telecom, CMCC, Nokia, ZTE) - Option 2A: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2 (Intel) - Option 2B: Use different CRS port number in the tests with different INR levels (China Telecom) - Option 3: Only cover 2 CRS ports (Nokia) - Moderator’s Recommendation - Further discuss in the next meeting |

Discussion on 2nd round

Topic #5: CRS-IM for 30 kHz SCS scenario

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201964 | Qualcomm Incorporated | Observation 5: There is no DSS rate matching defined in the spec for 30kHz SCS. Observation 6: UE will have different sampling rate for 30kHz SCS compared to LTE sampling rate. Observation 7: Different sampling rates will cause LTE interference to smudge all the NR REs on symbols colliding with CRS instead of just CRS REs. Observation 8: Different SCS will cause uneven interference within subcarrier, resulting into complicated or unreliable mitigation. Observation 9: UE will have to spend double the resources on measuring/detecting the interference compared to 15kHz SCS. Proposal 4: Do not consider CRS-IM for 30kHz SCS. |
| R4-2200273 | Apple | Observation #1: CRS interference mitigation for NR with 30KHz SCS would only be in non DSS deployment scenario. Observation #2: With 30KHz SCS for NR CRS-IC would lead to additional complexity and processing impact. Proposal #1: Only consider LLR weighing in Rel-17 and in future releases for CRS interference handling in 30KHz SCS. Observation #3: UE would not know the presence of LTE cell is no network assistance or LTE MO is configured to the UE. Proposal #2: UE is indicated about presence of LTE cell for CRS-IM by network assistance information. Observation #4: All CRS location information cannot be obtained by LTE inter-RAT measurement if LTE MO is configured. Proposal #3: UE is configured with network assistance information for CRS location information of interfering LTE cell. Proposal #4: It is feasible to consider CRS-IM for 30KHz with LLR weighing if the necessary LTE information is provided to UE by network assistance information. |
| R4-2200517 | Intel Corporation | Observations #3: For Scenario 2 with 30 kHz SCS - CRS-IM provides 2.8-3.0 dB performance improvement for scenario with 0% interference cell loading - CRS-IM provides 0.9-1.4 dB performance improvement for scenario with 10% interference cell loading - CRS-IM provides 0.5-1.0 dB performance improvement for scenario with 20% interference cell loading Proposal 3: Further study whether testable CRS-IM performance improvement (≥1 dB) can be achieved for practical scenarios with 30 kHz serving signal SCS. |
| R4-2200801 | CMCC | Proposal 3: UE verify the CRS port number and interfered symbols based on power comparison per symbol level. Proposal 4: There is no need to identify the CRS frequency location in Scenario 2 with NR 30kHz. Proposal 5: we propose a LLR weighting method as follows: - Acquire the CRS time domain location by power detection - Estimate the interference power distribution per RE level in the interfered symbols - Calculate the actual SINR on these interfered REs - Scaling the LLRs by post processing SINR value Observation 1: Almost all the NR REs in interfered symbols will be interfered by LTE CRS. Observation 2: If CRS ports is 4, then the front loaded DMRS will be totally interfered by CRS. |
| R4-2200803 | CMCC | Proposal 1: If the 30kHz is feasible, add TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells in test setup in Phase II. Proposal 2: Use 7DS2U with S=6D+4G+4U for the target cell with TDD 30kHz SCS. |
| R4-2200982 | Huawei, HiSilicon | In this paper, we give our discussion for CRS-IM receiver for 30 kHz SCS, the observations and proposal are: Observation 1: In the NR 30kHz+ LTE15kHz deployment, the main interference contributing to kth of NR subcarrier are 2kth, (2k-1)th and (2k+1)th LTE subcarrier. Observation 2: There are three different types interference model for NR 30 kHz. Observation 3: For 15 kHz, UE does channel estimation for 32 REs within 1ms while for 30 kHz, UE must do channel estimation for 64 REs within 0.5 ms, which bring significant complexity compared to 15kHz SCS. Observation 4: UE should do power estimation three times for three types of REs with different interference models and perform LLR weighting respectively. For LLR weighting without CRS sequence, there is no much complexity increased. Observation 5: For NR 30kHz+LTE 15kHz, there is 1.67 dB performance gain for 4T2R and 1.41dB performance gain for 4T4R for LLR weighting over baseline receiver. |

Open issues summary

Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario

Issue 5-1-1: Application scenario for 30 kHz SCS

- Proposals

  - Option 1: Scenario 2 only (QC, Apple)

- Recommended WF

  - Agree option 1

Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario

- Proposals on feasibility of LLR weighting with CRS sequence or CRS-IC

  - Option 1: Feasible

  - Option 2: Not feasible ([Huawei] , [Apple])

    - HW: For 15 kHz, UE does channel estimation for 32 REs within 1ms while for 30 kHz, UE must do channel estimation for 64 REs within 0.5 ms, which bring significant complexity compared to 15kHz SCS.

    - Apple: With 30KHz SCS for NR CRS-IC would lead to additional complexity and processing impact.

- Proposals on feasibility of LLR weighting without CRS sequence

  - Option 1: Feasible (Intel, HW, CMCC)

    - Intel: Further study whether testable CRS-IM performance improvement (≥1 dB) can be achieved for practical scenarios with 30 kHz serving signal SCS.

    - HW: For LLR weighting without CRS sequence, there is no much complexity increased.

  - Option 2: Feasible if the necessary LTE information is provided to UE by network assistance information (Apple)

  - Option 3: Not feasible  (QC)

    - QC:

      - UE will have different sampling rate for 30kHz SCS compared to LTE sampling rate.

      - Different sampling rates will cause LTE interference to smudge all the NR REs on symbols colliding with CRS instead of just CRS REs.

      - Different SCS will cause uneven interference within subcarrier, resulting into complicated or unreliable mitigation.

      - UE will have to spend double the resources on measuring/detecting the interference compared to 15kHz SCS.

- Recommended WF

  - TBA

Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario

- Companies’ simulation results

  - Intel: For Scenario 2 with 30 kHz SCS

    - CRS-IM provides 2.8-3.0 dB performance improvement for scenario with 0% interference cell loading

    - CRS-IM provides 0.9-1.4 dB performance improvement for scenario with 10% interference cell loading

    - CRS-IM provides 0.5-1.0 dB performance improvement for scenario with 20% interference cell loading

  - HW: For NR 30kHz+LTE 15kHz, there is 1.67 dB performance gain for 4T2R and 1.41dB performance gain for 4T4R for LLR weighting over baseline receiver [for 20% interference loading].

- Recommended WF

  - Encourage comments on the results

Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario

- Proposals

  - Option 1: Propose a LLR weighting method as follows (CMCC)

    - Acquire the CRS time domain location by power detection

    - Estimate the interference power distribution per RE level in the interfered symbols

    - Calculate the actual SINR on these interfered REs

    - Scaling the LLRs by post processing SINR value

  - Option 2: UE should do power estimation three times for three types of REs with different interference models and perform LLR weighting respectively. (HW)

    - In the NR 30kHz+ LTE15kHz deployment, the main interference contributing to kth of NR subcarrier are 2kth, (2k-1)th and  (2k+1)th LTE subcarrier.

Figure: Interference pattern of one RB for NR with 30 kHz SCS (HW, R4-2200982)

- Recommended WF

  - Encourage comments

Issue 5-1-5: CRS port number information

- Proposals

  - Option 1: UE verify the CRS port number and interfered symbols based on power comparison per symbol level. (CMCC)

- Recommended WF

  - TBA

Issue 5-1-6: CRS frequency location information

- Proposals

  - Option 1: There is no need to identify the CRS frequency location in Scenario 2 with NR 30 kHz.  (CMCC)

    - CMCC: Almost all the NR REs in interfered symbols will be interfered by LTE CRS.

  - Option 2: CRS location information should be configured with network assistance information. (Apple)

    - Apple: All CRS location information cannot be obtained by LTE inter-RAT measurement if LTE MO is configured.

- Recommended WF

  - TBA

Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell

- Observation

  - Option 1: If CRS ports is 4, then the front loaded DMRS will be totally interfered by CRS.  (CMCC)

- Recommended WF

  - Encourage comments

Sub-topic 5-2: Test setup for 30 kHz SCS scenario

Issue 5-2-1: CBW and SCS for target and interference cells for 30 kHz SCS

- Agreements for 15kHz SCS in RAN#101-e in the WF R4-2120705:

  - 10MHz CBW for FDD 15kHz

  - 20MHz CBW for TDD 15kHz

- Proposals

  - Option 1: If 30kHz SCS is feasible, use TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells (CMCC)

- Recommended WF

  - First focus on Sub-topic #5-1, and no comment for this issue in round 1.

Issue 5-2-2: TDD configuration for target cell with 30kHz SCS

- Agreements for 15kHz SCS in RAN#101-e in the WF R4-2120705:

  - Use 3D1S1U with S = 10D+2G+2U for the target NR cell with TDD 15kHz SCS

  - Use DSUDDDSUDD with S = 10D+2G+2U for the interference LTE with TDD 15kHz SCS.

  - Note: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame, which is aligned with the TDD-TDD EN-DC configuration in demod test.

- Proposals

  - Option 1: If 30kHz SCS is feasible, Use 7DS2U with S=6D+4G+4U for the target cell with TDD 30kHz SCS (CMCC)

- Recommended WF

  - First focus on Sub-topic #5-1, and no comment for this issue in round 1.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC - On feasibility of LLR weighting without CRS sequence Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario Issue 5-1-5: CRS port number information Issue 5-1-6: CRS frequency location information Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell Sub-topic 5-2: Test setup for 30 kHz SCS scenario Issue 5-2-1: CBW and SCS for target and interference cells for 30 kHz SCS Note: No comment in round 1. Issue 5-2-2: TDD configuration for target cell with 30kHz SCS Note: No comment in round 1. |
| China Telecom | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Option 1. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting without CRS sequence Option 1. NWA siganlling can be discussed separately. |
| Intel | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Support recommended WF Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC Support Option 2, because multiple FFT processing will be required. - On feasibility of LLR weighting without CRS sequence We support Option 1. This is the only algorithm which can be considered for CRS-IM processing in scenario with 30 kHz SCS. Same time, we agree with the following observations from QC: Different sampling rates will cause LTE interference to smudge all the NR REs on symbols colliding with CRS instead of just CRS REs. Different SCS will cause uneven interference within subcarrier, resulting into complicated or unreliable mitigation. Therefore, performance analysis of such processing is needed. Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario Can be discussed after more input from companies and alignment on RX processing. Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario For our analysis we just measure the interference power on all REs within the symbol colliding with CRS REs and make the averaging with 1 PRB granularity. We consider such method, because based on our comment for Issue 5-1-2, we will not observe that selected REs are affected by CRS interference due to loss of orthogonality of the 15 kHz SCS signal after applying of FFT processing under assumption of 30 kHz SCS. Therefore, we don’t think that Option 1 or 2 is valid processing. Issue 5-1-5: CRS port number information We suggest to focus on performance benefits in case all parameters are known to check the performance upper bound. In case testable performance will be observed, we can further analyse the blind detection on certain parameters. Issue 5-1-6: CRS frequency location information Same comment as for 5-1-6. Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell We agree with observation in Option 1. |
| CMCC | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Agree with the recommended WF. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting without CRS sequence Feasible. UE can perform LLR weighting as following steps: - Acquire the CRS time domain location by power detection - Estimate the interference power distribution per RE level in the interfered symbols - Calculate the actual SINR on these interfered REs - Scaling the LLRs by post processing SINR value We believe there is obvious complexity will be introduced if we don’t use CRS sequence and CRS-IC. Only larger memory is needed. We also observed all the NR REs on the symbols colliding with CRS will be interfered. Based on our understanding, LLR weighting will be performed on these REs. As for the uneven interference within subcarrier, we are ok with further study whether testable CRS-IM performance improvement (≥1 dB) can be achieved for practical scenarios with 30 kHz serving signal SCS. Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario Option 1. With the assumption of almost all the NR REs on the symbols colliding with CRS will be interfered. UE should first find the symbols colliding with CRS, and then estimate the interference power per RE level in the interfered symbols. Last, use the interference power to do the LLRs. Issue 5-1-5: CRS port number information We support Option1. Same with Scenario 1, UE verify the CRS port number and interfered symbols based on power comparison per symbol level. Issue 5-1-6: CRS frequency location information Option 1. Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell How to handling the DM-RS collision can be further investigated based on simulation results. Sub-topic 5-2: Test setup for 30 kHz SCS scenario Issue 5-2-1: CBW and SCS for target and interference cells for 30 kHz SCS Note: No comment in round 1. Issue 5-2-2: TDD configuration for target cell with 30kHz SCS Note: No comment in round 1. |
| Ericsson | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Ok with recommended WF. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC Option 2. Complexity and time will be increased by performing LLR weighting with CRS sequence and CRS-IC in 30kHz SCS scenario, since there should be different FFT for different SCS respectively. - On feasibility of LLR weighting without CRS sequence Option 1. It can be further evaluated to see if there is a gain (at least ≥1dB). Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario We need to further check it. Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario Share similar views with Intel. Issue 5-1-5: CRS port number information We think option 1 is reasonable Issue 5-1-6: CRS frequency location information We think option 1 is reasonable Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell We think option 1 is reasonable. |
| Apple | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS We support the recommended WF. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC Option 2 as it is additional complexity. - On feasibility of LLR weighting without CRS sequence Might be feasible if all the required LTE parameters are signalled by NWA. Cannot expect the UE to do any detection. Based on results from Intel and observations from QC, we need to further study if there are any benefits at all with CRS-IM in 30KHz. Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario Issue 5-1-5: CRS port number information We prefer to discuss LTE parameter info once feasibility is determined. Issue 5-1-6: CRS frequency location information We prefer to discuss LTE parameter info once feasibility is determined. Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell We agree with the observation and it would impact DMRS channel estimation significantly and have impact due to uneven interference on the 2 DMRS symbols. Sub-topic 5-2: Test setup for 30 kHz SCS scenario Issue 5-2-1: CBW and SCS for target and interference cells for 30 kHz SCS Note: No comment in round 1. Issue 5-2-2: TDD configuration for target cell with 30kHz SCS Note: No comment in round 1. |
| Huawei | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS We support the recommended WF. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC Option 2 - On feasibility of LLR weighting without CRS sequence Option 1 Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario According to our simulation results, there is 1.67 dB performance gain for 4T2R and 1.41dB performance gain for 4T4R for LLR weighting over baseline receiver. In the simulation, we estimate the CRS power and perform LLR weighting per interference pattern. Interference pattern 1: Subcarrier index 0,3,6,9…, Symbol index0,1,2,3,8,9 Interference pattern 2: Subcarrier index 1, 4, 7, 10,…, Symbol index0,1,2,3,8,9 Interference pattern 3: Subcarrier index 2,5,8,11…, Symbol index0,1,2,3,8,9 Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario @ Intel: We agree that the interference CRS is dispreading to all the OFDM symbols, but as discussed in our T-doc, the main interference contributing to kth of NR subcarrier is 2kth, (2k-1)th and (2k+1)th LTE subcarrier, the interference from other LTE subcarriers is too small and can be ignored. Besides, we observed that the interference power is different for different subcarrier and we should perform LLR weighting respectively. It is not reasonable to average the power of all REs within one symbol. Issue 5-1-5: CRS port number information Issue 5-1-6: CRS frequency location information Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell Agree with observation |
| Nokia, Nokia Bell Labs | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS We support option 1 Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC We agree that exploration of CRS Sequence knowledge is practically not feasible due to complexity issues and we remain without understanding that performance gain would be minimal in any case. Support Option 2. - On feasibility of LLR weighting without CRS sequence It is feasible but not desirable as Qualcomm has pointed out a single CRS RE will reduce the slot throughput greatly so the practical relevance of 30kHz/15kHz co-existence is debateable. |
| MediaTek | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Support the recommended WF. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC Support Option 2 - On feasibility of LLR weighting without CRS sequence We believe the processing of LLR weighting for 30kHz is quite different from that for 15kHz. Also, we agree the observations from QC and prefer not to consider CRS-IM for the scenario of 30kHz. |
| Qualcomm | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Ok with recommended WF. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting with CRS sequence or CRS-IC Option 2. It’s not feasible because UE will have to match the sampling rate between NR and LTE and basically have completely separate processing for LTE CRS. Cancellation is also not trivial since those CRS REs are spread over two symbols and unevenly spread within subcarrier. - On feasibility of LLR weighting without CRS sequence Option 3. Apart from the reasons mentioned in our paper as highlighted by the moderator, we think that this scenario where NR has 30kHz SCS and neighbour LTE cell has 15kHz SCS, should be fixed from NW deployment perspective rather than burdening UE to handle it. The reason is that this kind of deployment has a fundamental issue because this kind of deployment will impact all single DMRS scenarios such as special slot, PDSCH Type B and partial slot allocation and there is nothing a UE could do about it because it’s only DMRS will be impacted by LTE CRS, resulting into inaccurate Rnn estimation. Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario Discuss in the next meeting based on the outcome of current meeting. Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario It should be left up to UE implementation. Both options have their pros and cons. Option 1 is trying to estimate the interference per RE by possibly averaging out the power difference across whole symbol. This will have lower complexity and potentially less gains. Option 2 has more complexity and potentially more gains. But these options for LLR weighting will not work well for single DMRS scenario because DMRS based noise estimation will already include CRS interference. Issue 5-1-5: CRS port number information Same comment as 15kHz. Power detection is unreliable for higher SIR. This needs NWA. Issue 5-1-6: CRS frequency location information Option 2. UE will need to know the LTE BW and center frequency. In case, there is single CRS port, UE will have to know the frequency location information as well. Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell We agree with this observation. As we mentioned in our comment for Issue 5-1-2, this will cause degradation for single DMRS scenarios for which UE can’t do much about it. |
| ZTE | Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario Issue 5-1-1: Application scenario for 30 kHz SCS Support recommended WF. Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario - On feasibility of LLR weighting without CRS sequence Option 1 is feasible. Issue 5-1-5: CRS port number information We support option 1. UE can detect CRS ports by power detect. Issue 5-1-6: CRS frequency location information We support option 1. Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell We support option 1. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 5-1: Feasibility evaluation for 30 kHz SCS scenario | Issue 5-1-1: Application scenario for 30 kHz SCS Summary of round 1 discussion: All companies agree to cover scenario 2 only. Tentative agreements: - Cover scenario 2 only Issue 5-1-2: Feasibility of performing CRS-IM in 30 kHz SCS scenario Summary of round 1 discussion: - Feasibility of LLR weighting with CRS sequence or CRS-IC - Option 1: Feasible - Option 2: Not feasible (Huawei, Apple, Intel, E///, Nokia, MTK, QC) - Feasibility of LLR weighting without CRS sequence - Option 1: Feasible (Intel, HW, CMCC, CTC, E///, [Nokia], ZTE) - Note: The need of NWA siganlling is discussed separately. - CMCC, E///, Apple: Discuss the feasibility and performance gain separately. Ok with further study whether testable CRS-IM performance improvement (≥1 dB) can be achieved for practical scenarios with 30 kHz serving signal SCS. - Option 2: Feasible if the necessary LTE information is provided to UE by network assistance information (Apple) - Option 3: Not feasible (QC, MTK) - QC: - UE will have different sampling rate for 30kHz SCS compared to LTE sampling rate. - Different sampling rates will cause LTE interference to smudge all the NR REs on symbols colliding with CRS instead of just CRS REs. - Different SCS will cause uneven interference within subcarrier, resulting into complicated or unreliable mitigation. - UE will have to spend double the resources on measuring/detecting the interference compared to 15kHz SCS. - This kind of deployment has a fundamental issue because this kind of deployment will impact all single DMRS scenarios such as special slot, PDSCH Type B and partial slot allocation - Moderator’s Recommendation (to be checked in round 2) - For LLR weighting with CRS sequence or CRS-IC: not feasible - For LLR weighting without CRS sequence: - Discuss the feasibility and performance gain separately. - Discuss the need of NWA siganlling separately. - For the next meeting, companies to further evaluate whether testable CRS-IM performance improvement can be achieved for practical scenarios with 30 kHz serving signal SCS. Issue 5-1-3: Performance gain of LLR weighting in 30 kHz SCS scenario - Moderator’s Recommendation - Further discuss in the next meeting. Issue 5-1-4: Implementation of LLR weighting in 30 kHz SCS scenario Summary of round 1 discussion: - Option 1: Propose a LLR weighting method as follows (CMCC) - Acquire the CRS time domain location by power detection - Estimate the interference power distribution per RE level in the interfered symbols - Calculate the actual SINR on these interfered REs - Scaling the LLRs by post processing SINR value - Option 2: UE should do power estimation three times for three types of REs with different interference models and perform LLR weighting respectively. (HW) - In the NR 30kHz+ LTE15kHz deployment, the main interference contributing to kth of NR subcarrier are 2kth, (2k-1)th and (2k+1)th LTE subcarrier. - Interference pattern 1: Subcarrier index 0, 3, 6, 9…, - Interference pattern 2: Subcarrier index 1, 4, 7, 10,…, - Interference pattern 3: Subcarrier index 2, 5, 8, 11…, - Option 3: Measure the interference power on all REs within the symbol colliding with CRS REs and make the averaging with 1 PRB granularity. (Intel, [E///]) - Intel: we will not observe that selected REs are affected by CRS interference due to loss of orthogonality - Option 4: Up to UE implementation (QC) - QC: Discuss the single DMRS scenario separately, since the front loaded DMRS will be totally interfered by CRS in 4 CRS port scenario. - Moderator’s Recommendation - For the performance gain evaluation, further discuss is it necessary to agree on one implementation as reference receiver? - Evaluate the scenario of DMRS 1+0 and DMRS 1+1 separately. Issue 5-1-5: CRS port number information - Proposals - Option 1: UE verify the CRS port number and interfered symbols based on power comparison per symbol level. (CMCC, E///, ZTE) - Option 2: Further analyse the feasibility of blind detection if testable performance will be observed with all parameters known to UE (Intel, Apple) - Option 3: Need NWA since power detection is unreliable for higher SIR. (QC) - Moderator’s Recommendation - Further discuss Issue 5-1-6: CRS frequency location information - Proposals - Option 1: There is no need to identify the CRS frequency location in Scenario 2 with NR 30 kHz. (CMCC, E///, ZTE) - CMCC: Almost all the NR REs in interfered symbols will be interfered by LTE CRS. - Option 2: CRS location information should be configured with network assistance information. (Apple, QC) - Option 3: Further analyse the feasibility of blind detection if testable performance will be observed with all parameters known to UE (Intel, Apple) - Moderator’s Recommendation - Further discuss Issue 5-1-7: Colliding of the DM-RS in serving cell with CRS in neighbouring cell Summary of round 1 discussion: All companies agree with the following observation: - If CRS ports is 4, then the front loaded DMRS will be totally interfered by CRS. - Moderator’s Recommendation - Companies to investigate the potential performance impact by the above observation. |
| Sub-topic 5-2: Test setup for 30 kHz SCS scenario | Issue 5-2-1: CBW and SCS for target and interference cells for 30 kHz SCS Candidate option: - Option 1: If 30kHz SCS is feasible, use TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells (CMCC) - Moderator’s Recommendation - Further discuss in the next meeting. Issue 5-2-2: TDD configuration for target cell with 30kHz SCS Candidate option: - Option 1: If 30kHz SCS is feasible, Use 7DS2U with S=6D+4G+4U for the target cell with TDD 30kHz SCS (CMCC) - Moderator’s Recommendation - Further discuss in the next meeting. |

Discussion on 2nd round

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general part and 15kHz NR SCS scenario for CRS-IM receiver | China Telecom | Cover Topic #2, 3, 4 |
| WF on 30 kHz NR SCS scenario for CRS-IM receiver | CMCC | Cover Topic #5 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200017 | Draft TR 38.833 v1.1.0: Further enhancement on NR demodulation performance | China Telecom | For email approval after the meeting |  |
| R4-2200018 | TP to TR 38.833: Symbols and abbreviations | China Telecom | Agreeable |  |
| R4-2200905 | Updated work plan for Further enhancement on NR demodulation performance WI | China Telecom | Agreeable |  |
| R4-2201603 | Discussion on UE feature list and capability signalling for NR Demodulation Enhancements WI | Intel Corporation | Noted |  |
| R4-2201964 | Views on CRS Interference Mitigation in NR | Qualcomm Incorporated | Noted |  |
| R4-2200273 | Discussion on CRS-IM with 30KHz SCS | Apple | Noted |  |
| R4-2200502 | On Receiver Assumptions for CRS-IM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2200517 | General discussion on CRS-IM requirements for scenarios with overlapping spectrum for LTE and NR | Intel Corporation | Noted |  |
| R4-2200801 | Discussion on the receiver for CRS-IM | CMCC | Noted |  |
| R4-2200898 | Discussion on the receiver assumption for CRS-IM requirement definition | China Telecom | Noted |  |
| R4-2200982 | Discussion on CRS-IM receiver for 30kHz SCS | Huawei, HiSilicon | Noted |  |
| R4-2201414 | CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR | ZTE Corporation | Noted |  |
| R4-2201415 | Simulation results on CRS-IM receiver | ZTE Corporation | Noted |  |
| R4-2201418 | Discussion on remaining issues of receiver assumption for CRS-IM | Ericsson | Noted |  |
| R4-2200274 | Discussion on Network Assistance for CRS-IM in NR | Apple | Noted |  |
| R4-2200381 | Views on the network assistance signalling for CRS-IM receiver | MediaTek inc. | Noted |  |
| R4-2200503 | On Necessity of Network assistant signaling for CRS-IM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2200518 | Network assistant signaling for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR | Intel Corporation | Noted |  |
| R4-2200802 | Discussion on the network assistant signaling necessity for CRS-IM | CMCC | Noted |  |
| R4-2200899 | Discussion on the necessity of network assistance signalling for CRS-IM | China Telecom | Noted |  |
| R4-2200984 | Discussion on necessity of NWA on CRS-IM receiver for 15kHz SCS | Huawei,HiSilicon | Noted |  |
| R4-2201416 | Discussion on the network assistance signalling for CRS-IM | Ericsson | Noted |  |
| R4-2201417 | Simulation results for CRS interference handling | Ericsson | Noted |  |
| R4-2200275 | Discussion on Test setup for CRS-IM requirements | Apple | Noted |  |
| R4-2200519 | Test setup for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR | Intel Corporation | Noted |  |
| R4-2200803 | Discussion on the test setup for CRS-IM | CMCC | Noted |  |
| R4-2200900 | Discussion on the test setup for CRS-IM requirement definition | China Telecom | Noted |  |
| R4-2200983 | Discussion on test setup on CRS-IM receiver for 15kHz SCS | Huawei,HiSilicon | Noted |  |
| R4-2201419 | Discussion on the remaining issues of test setup for CRS-IM | Ericsson | Noted |  |
| R4-2201601 | On test setup for CRS-IM | Nokia, Nokia Shanghai Bell | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | Karsten Petersen | Karsten.petersen@nokia-bell-labs.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)