3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202968

# Electronic Meeting, Jan. 17th – 25th, 2022

Agenda item:			6.12.3

Source:	Moderator (Huawei, HiSilicon)

Title:			Email discussion summary for [101-bis-e][318] NR_perf_enh2_Demod_Part3

Document for:	Information

Introduction

RAN4#100-e: all open issues are finalized as per WF R4-2115748 approved in RAN4#100-e, the general simulation assumptions are captured here for information:

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz; 30kHz: 10MHz, 40MHz, 100MHz |
| SRS, PT-RS | Not configured |
| Phase noise modelling | No explicit PN modelling (Note 1) |
| Tx EVM | No explicit 3.5% Tx EVM modelling (Note 2) |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| Note 1: The phase noise impact can be included in the impairment results, but it is left up to companies Note 2: Consider 3.5% Tx EVM impact in the impaired results, but it is left up to companies. |  |

RAN4#101-e: Initial simulation alignment was performed as captured in simulation results summary R4-2119028; Further alignment rules and derivation for final performance requirements were agreed as in the approved WF R4-2120717; CR work splitting is conducted and confirmed by interested companies as captured in the summary after 2nd round discussion R4-2120739.

RAN4#101bis-e:

1st round discussion:

- Collect companies’ simulation results for further alignment

- Draft CRs review

2nd round discussion:

- Companies are encouraged to update their results if needed by Thursday after the kickoff of 2nd round discussion to reserve time for draft CRs updates to capture the derived performance requirements for both TS 38.104 and TS 38.141-1 as per the latest submitted results for this meeting.

- Endorse draft CRs after necessary revision.

Topic #1: Test parameters

Companies’ contributions summary

| T-doc number | Company | Title | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2200132 | CATT | Draft CR for TS 38.141-1 on manufacture declaration for FR1 PUSCH 256QAM performance requirements | Draft CR for manufacture declaration for TS 38.141-1 |
| R4-2200133 | CATT | Simulation results for FR1 PUSCH 256QAM performance requirement | Simulation results resubmission |
| R4-2200148 | CATT | Draft CR for TS 38.141-2, Introduction of manufacture declaration, MU and TT for FR1 PUSCH 256QAM performance requirements | Draft CR for MU and TT for TS 38.141-2 |
| R4-2200474 | Ericsson | draftCR for 38.141-1 MU and TT for PUSCH with 256QAM | Draft CR for MU and TT for TS 38.141-1 |
| R4-2200520 | Intel Corporation | Draft CR for TS 38.104: FR1 256QAM PUSCH requirements | Draft CR for PUSCH requirements for TS 38.104 |
| R4-2200752 | Samsung | draft CR on FR1 PUSCH 256QAM FRC for TS 38.104 | Withdrawn? |
| R4-2200753 | Samsung | draft CR on FR1 PUSCH 256QAM FRC for TS 38.141-1 | Draft CR for FRC for TS 38.141-1 |
| R4-2200754 | Samsung | draft CR on FR1 PUSCH 256QAM FRC for TS 38.141-2 | Draft CR for FRC for TS 38.141-2 |
| R4-2200755 | Samsung | Simulation results for FR1 PUSCH with 256QAM | Simulation results resubmission |
| R4-2200796 | CMCC | Draft CR for TS38.141-2 Requirements for PUSCH with transform precoding disabled for BS type 1-O | Draft CR for PUSCH requirements for TS 38.141-2 |
| R4-2200838 | ZTE Corporation | Simulation results for FR1 UL 256QAM demodulation requirement | - Simulation results submission |
| R4-2201018 | Huawei,HiSilicon | Updated simulation results for FR1 PUSCH 256QAM performance requirements | - Simulation results submission |
| R4-2201019 | Huawei,HiSilicon | Summary of simulation results for FR1 PUSCH 256QAM performance requirements | - Summary of simulation results |
| R4-2201020 | Huawei,HiSilicon | draftCR: Introduction of conformance testing for FR1 PUSCH 256QAM with transform precoding disabled in TS 38.141-1 | - Draft CR for PUSCH requirements for TS 38.141-1 |
| R4-2201801 | ZTE Wistron Telecom AB | Simulation results for FR1 UL 256QAM demodulation requirement | - To be submitted during the meeting? |
| R4-2201802 | ZTE Wistron Telecom AB | draftCR on FR1 PUSCH 256QAM FRC for TS 38.104 | - FRC for TS 38.104 |

Open issues summary

In this section, Simulation results alignment and draft CRs review for FR1 PUSCH 256QAM demodulation performance requirements will be discussed.

Open issues and candidate options before e-meeting:

Simulation results alignment

Issue 1: Simulation results alignment and derivation of final performance requirements

- Proposals

  - Company can update their results if needed by Thursday

  - Capture the results listed in “38.104 req” and “38.141 req” as the final performance requirements with square brackets in draft CRs to be endorsed finally.

  - Remove the square brackets during RAN4#103-e in May this year.

- Recommended WF

Companies views’ collection for 1st round

Simulation results alignment

| Company | Comments |
| --- | --- |
| Samsung | We are fine with proposal for simulation results alignment and derivation, Meanwhile, we would like to check whether the results updated is allowed for next Feb meeting? |
| NOKIA | We agree with the proposals. |
| Ericsson | We agree with proposals and open for further discussion on updating results in Feb. meeting. |
| CATT | OK with the proposal. |
| Intel | Proposal is fine for us. We think that in case no new results will be submitted in the next RAN4 meeting in February (i.e. 102), probably, square brackets can be removed in RAN4 #102 meeting. |
| ZTE | We are fine with the proposals, and our updated simulation results have been uploaded. |
| Huawei | We are fine with the proposal and open to update the results in next Feb. meeting, but prefer to remove the square brackets in May meeting at the latest time. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200132(CATT) Manufacture declaration for TS 38.141-1 | Moderator: manufacture declaration for TS 38.141-2 is missing |
|  | CATT: To moderator, the manufacture declaration is included in 38.141-1 (R4-2200148). |
| R4-2200148(CATT) MU and TT for TS 38.141-2 | Ericsson: If TE venders confirm the MU and TT value is the same as other modulation, R4-2200148 and R4-2200474 could be withdraw. Otherwise, maybe additional row is better? We ask for companies’ opinions on this. |
|  | CATT: To Ericsson, we share the similar view that R4-2200132, R4-2200148 and R4-2200474 should be withdrawn if the value remains unchanged after TE vendor confirmation. We slight prefer the same row without no redundancy. |
|  | Intel: Based on TS 38.521-4, there is no modulation order dependancy for test uncertenty for UE testing. Probablity, it is also valid for BS testing. Input from TE vendors will be helpfull. |
|  | Huawei: - No test uncertainty and TT based on modulation order is defined for LTE in TS 36.141, we don’t think that it is necessary to distinguish different modulation order. But TE vendors’ input is beneficial; - It is better to add FR1 for support of PUSCH 256QAM to distinguish PUSCH 256QAM for FR2 in the future; - If we agree to capture the minimum performance requirements for 256QAM in the existing table, the related CRs are prepared by capturing in the existing table for PUSCH during this meeting, no need to create separate row for PUSCH 256QAM TT in Table C.3.1, and just reuse the existing one for test case 8.2.1. |
| R4-2200474(Ericsson) MU and TT for TS 38.141-1 |  |
| R4-2200520(Intel) PUSCH Requirements for TS 38.104 | Intel: Draft CR will be revised to include FRC and SNR values in []. |
| R4-2201020(Huawei) PUSCH requirements for TS 38.141-1 | Huawei: To be revised to include the latest SNR values based on the latest results submitted in this meeting. |
| R4-2200796(CMCC) PUSCH requirements for TS 38.141-2 | CMCC: Draft CR will be revised to update FRC and SNR values in []. |
| R4-2200752(Samsung) FRC for TS 38.104 | Moderator: Tdoc# was reserved but not submitted, withdrawn? Samsung: Based on offline discussion, ZTE will reponse the FRC 104 CR, we will take the FRC of 141-1/2 CR to balance the workload. |
| R4-2201802(ZTE) FRC for TS 38.104 |  |
| R4-2200753(Samsung) FRC for TS 38.141-1 |  |
| R4-2200754(Samsung) FRC for TS 38.141-2 |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic# | Status summary |
| --- | --- |
| #1 Simulation results alignment | Tentative agreement: - Company can update their results if needed by Thursday for this meeting - Capture the results listed in “38.104 req” and “38.141 req” in simulation results summary of R4-2120616 as the final performance requirements with square brackets in draft CRs to be endorsed this meeting. - It is not preferred that company will update their results after Feb. meeting - The square brackets can be removed in Feb. meeting if no further results will be updated before the end of the meeting. But this can be further discussed during Feb. meeting if needed. - If no agreement to remove square brackets in Feb. meeting, RAN4 will remove the square brackets during RAN4#103-e in May meeting finally. Recommendation for 2nd round: None |
| #2 MU and TT | Tentative agreement: TE vendors need to check if the existing MU and TT defined in TS 38.141-1 and TS 38.141-2 are applicable for 256QAM or if MU and TT will be defined based on different modulation order. Recommendation for 2nd round: Based on TE vendor’s feedback to decide how to handle CRs of R4-2200148 and R4-2200474 in the 2nd round. |

CRs/TPs

As summarized in Section 3.1.

Discussion on 2nd round

Summary for 2nd round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic# | Status summary |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF for FR1 PUSCH with 256QAM performance requirements | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200132 | Draft CR for TS 38.141-1 on manufacture declaration for FR1 PUSCH 256QAM performance requirements | CATT | Agreeable |  |
| R4-2200133 | Simulation results for FR1 PUSCH 256QAM performance requirement | CATT | Noted |  |
| R4-2200148 | Draft CR for TS 38.141-2, Introduction of manufacture declaration, MU and TT for FR1 PUSCH 256QAM performance requirements | CATT | Revised | Update the manufacture declaration To confirm the MU and TT for 256QAM from TE vendors |
| R4-2200474 | draftCR for 38.141-1 MU and TT for PUSCH with 256QAM | Ericsson | Return to | To confirm the MU and TT for 256QAM from TE vendors |
| R4-2200520 | Draft CR for TS 38.104: FR1 256QAM PUSCH requirements | Intel Corporation | Revised | Include RFC and SNR values with [] as per latest results summary |
| R4-2200752 | draft CR on FR1 PUSCH 256QAM FRC for TS 38.104 | Samsung | Withdrawn |  |
| R4-2200753 | draft CR on FR1 PUSCH 256QAM FRC for TS 38.141-1 | Samsung | Agreeable |  |
| R4-2200754 | draft CR on FR1 PUSCH 256QAM FRC for TS 38.141-2 | Samsung | Agreeable |  |
| R4-2200755 | Simulation results for FR1 PUSCH with 256QAM | Samsung | Noted |  |
| R4-2200796 | Draft CR for TS38.141-2 Requirements for PUSCH with transform precoding disabled for BS type 1-O | CMCC | Revised | Include RFC and SNR values with [] as per latest results summary |
| R4-2200838 | Simulation results for FR1 UL 256QAM demodulation requirement | ZTE Corporation | Withdrawn |  |
| R4-2201018 | Updated simulation results for FR1 PUSCH 256QAM performance requirements | Huawei,HiSilicon | Noted |  |
| R4-2201019 | Summary of simulation results for FR1 PUSCH 256QAM performance requirements | Huawei,HiSilicon | Return to | To be in the 2nd round |
| R4-2201020 | draftCR: Introduction of conformance testing for FR1 PUSCH 256QAM with transform precoding disabled in TS 38.141-1 | Huawei,HiSilicon | Revised | Update SNR values as per latest results |
| R4-2201801 | Simulation results for FR1 UL 256QAM demodulation requirement | ZTE Wistron Telecom AB | Noted |  |
| R4-2201802 | draftCR on FR1 PUSCH 256QAM FRC for TS 38.104 | ZTE Wistron Telecom AB | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents