3GPP TSG-RAN WG4 Meeting #101-bis-e			  										R4-2202992 Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation assumptions for HAPS co-existence

Agenda item: 	6.13.2.2 Coexistence scenarios and Simulation assumptions [NR_NTN_solutions-Core]

Document for:	Approval

# Introduction

Simulation assumptions for HAPS co-existence study have been captured in [1][2][3] during the previous meetings.

In this document, previous simulation assumptions [3] are updated to reflect the results of on-going discussions in RAN4#101-bis.

# Discussion

## Simulation scenarios

The agreed HAPS scenarios for initial coexistence simulations (see the scenario tables in section 1) are (i) rural TN + HAPS, (ii) Urban macro TN + HAPS, and (iii) HAPS + HAPS. All these scenarios use 2 GHz frequency band and FDD duplex scheme. HAPS altitude is assumed to be 20 km. Relevant coexistence scenarios are summarized in Table 1.

For TN and HAPS coexistence, one cluster of TN cells is randomly dropped in the HAPS coverage area as shown in Figure 1. The TN cell cluster consists of 19 sites, 57 sectors with co-channel interference wrap-around as described in section 2.3.

HAPS and HAPS co-existence can be characterized by “center-to-center” inter-system distance (i.e., the distance between two coverage centers) as depicted in Figure 2. Since HAPS location is at the center of its coverage area, the inter-system distance is the separation of the two HAPS. Different inter-system distances may be simulated to evaluate the interference caused by antenna gain variation in the elevation domain.

Table 1. HAPS coexistence scenarios

| HAPS altitude | 20 Km |
| --- | --- |
| Carrier frequency | 2 GHz |
| Duplex scheme | FDD |
| Coexistence scenarios | HAPS + TN (UMa) |
|  | HAPS + TN (RMa) |
|  | HAPS + HAPS (RMa) |

Figure 1. Simulation layout between HAPS and TN

Figure 2. Coexistence scenarios of HAPS and HAPS

## HAPS assumption

A reference HAPS antenna model proposed for HIBS (HAPS as IMT base stations) study in ITU WP-5D [5] is shown in Figure 3. The antenna array is composed of seven antenna panels (six side panels and one downward facing panel). Antenna elements on each panel are co-phased to form one beam in two crossed linear polarizations to serve one cell. There are a total of seven cells in two layers, one cell in the 1st layer and six cells in the 2nd layer. Other parameters of this antenna model are listed in Table 2. The HAPS antenna model has been revised slightly to adapt to a more realistic implementation, addressing the issues raised in [6].

Figure 3. HAPS antenna array

Table 2. HAPS network parameters

| Number of cells | 7 |
| --- | --- |
| Carrier frequency | 2 GHz |
| Frequency reuse factor | 1 |
| Antenna array configuration (row x column) | 2 x 2 for 1st layer cell 4 x 2 for 2nd layer cell |
| Antenna polarization | Linear ±45° |
| Element gain1 | 7.8 dBi |
| Element HPBW horizontal/vertical | 65⁰ for both H/V |
| Element front-to-back ratio horizontal/vertical | 30 dB for both H/V |
| Element spacing horizontal/vertical | 0.7 wavelength for both H/V |
| Antenna panel tilt (from the horizon) (Note 3) | 90⁰ for 1st layer cell 23⁰for 2nd layer cell |
| EIPR/cell | 56.8 dBm (1st layer cell), 59.8 dBm (2nd layer cell) |
| EIRP spectral density/cell | 43.8 dBm/MHz (1st layer cell), 46.8 dBm/MHz (2nd layer cell) |
| Tx power per antenna panel | 43 dBm |
| Conducted power (before ohmic loss) per antenna element (dBm) | 34 dBm for 2 x 2 (x 2 polarizations for 1st layer cell) 31 dBm for 4 x 2 (x 2 polarizations for 2nd layer cell) |
| Antenna weight2 wm,n | 12 for each of 1st layer cell element 122 for each of 2nd layer cell element |
| Polarization gain for SINR | 3 dB |
| Noise figure | 5 dB |
| Indoor UE percentage | 0% |
| Coverage area (7 cells combined) | A 100 Km radius circular area centered by the serving HAPS |
| UE distribution | Uniformly distributed in the coverage area |
| Note 1: The element gain includes the Ohmic loss of 2 dB. Note 2: These antenna weights create a fixed cell beam in the antenna panel’s boresight direction. Note 3: This is mechanical tilt only. |  |

When HAPS altitude is 20 Km, the antenna gain of this model perceived on the ground is shown in Figure 4, where (a) is the gain of the 1st layer cell (i.e., the center cell) produced by the 90⁰ tilt angle panel (downward facing panel), and (b) is the gain of a 2nd layer cell (i.e., an outer cell) produced by an eastward facing antenna panel with 23⁰ tilt angle. For a 2nd layer cell, the antenna gain depends on not only the distance but also the azimuth ϕ from the boresight. With this antenna model, SINR can be calculated for a given location on the ground taking into account the propagation loss and co-channel interference. Figure 5 shows the downlink SINR as a function of distance from the coverage center for different azimuth angles (ϕ=0 is the direction of 2nd layer cell boresight, ϕ=30° is at the cell edge), assuming 2 GHz carrier frequency, 20 MHz channel bandwidth, free space path loss, 4 dB fade margin, and 7 dB UE noise figure. It can be observed that for outdoor UEs in a rural environment, where the propagation condition is close to free space path loss with shadow fading, HAPS coverage range can reach 100 km at 2 GHz frequency.

| (a) | (b) |
| --- | --- |

Figure 4. HAPS antenna gain (in dB) in a 100 Km radius area on the ground. (a) Antenna gain of the 1st layer cell. (b) Antenna gain of a 2nd layer cell.

Figure 5. HAPS single system SINR as a function of distance from coverage center assuming 2 GHz carrier frequency, free space path loss and 4 dB fade margin

The 7-cell layout of the HAPS coverage may be implemented as (a) 7 equal sized hexagons or (b) divided cells within a circular area as shown in Figure 6. UEs should be dropped in the coverage area uniformly and the serving cell is selected by the strongest RSRP or least coupling loss. (Note: Calibration data show these two implementations yield similar results.)

| (a) | (b) |
| --- | --- |

Figure 6. HAPS cell layout

## Terrestrial network assumption

The typical network layout of 19 sites, 3 sectors per site, with co-channel interference wrap-around can be adopted for the coexistence study. System parameters such as inter-site distance, BS antenna height, antenna array and array downtilt angle, indoor UE percentage, etc. should be adjusted according to the environment. Tables 3-5 summarize the assumptions for Urban macro and rural macro environments.

Table 3. Terrestrial network parameters

| Terrestrial environment | Urban macro | Rural macro |
| --- | --- | --- |
| Network layout | 19 sites (57 cells) wrap-around | 19 sites (57 cells) wrap-around |
| Inter-site distance | 750 m | 7.5 Km |
| BS antenna height | 25 m | 30 m |
| BS transmit power | 46 dBm | 46 dBm |
| Conducted power per antenna element | 25 dBm | 25 dBm |
| BS antenna array (M, N, P) | (8, 8, 2) | (8, 8, 2) |
| Polarization gain for SINR | 3 dB | 3 dB |
| BS antenna Element spacing horizontal/vertical | H 0.5λ / V 0.7λ | H 0.5λ / V 0.9λ |
| BS antenna downtilt | 10⁰ | 3⁰ |
| BS antenna element gain pattern | Tables 4, 5 | Tables 4, 5 |
| BS noise figure | 5 dB | 5 dB |
| Indoor UE percentage | 0% | 0% |
| Minimum BS-UE distance | 35 m | 35 m |

Table 4. Terrestrial BS antenna element gain pattern (TR 38.921 [11])

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

Table 5. Terrestrial BS array antenna parameters

| Parameter | Urban Macro | Rural Macro |
| --- | --- | --- |
| Element gain1 GE,max (dBi) | 6.4 | 7.1 |
| Horizontal front-to-back ratio Am (dB) | 30 | 30 |
| Vertical front-to-back ratio SLAv (dB) | 30 | 30 |
| Horizontal 3dB beamwidth φ3dB (deg.) | 90 | 90 |
| Vertical 3dB beamwidth θ3dB (deg) | 65 | 54 |
| Horizontal element spacing dh (λ) | 0.5 | 0.5 |
| Vertical element spacing dv (λ) | 0.7 | 0.9 |
| Note 1: The element gain includes the Ohmic loss of 2 dB. |  |  |

## Propagation model

Consider coexistence scenarios TN+HAPS and HAPS+HAPS. The radio link between a TN BS and an UE served by TN (TN UE) may follow either the UMa model or the RMa model in [9] depending on the terrestrial environment. The radio link between HAPS and a UE, regardless of the UE being served by TN or HAPS, follows the NTN path loss model of either “Urban” or “Rural” scenario. Penetration loss is not needed for HAPS UE (i.e., UE served by HAPS) and TN UE since all HAPS UEs and TN UEs are assumed to be outdoor. The use of channel model for different radio links in the coexistence scenarios is summarized in Table 6.

Table 6. Channel model used in HAPS coexistence study

| Scenario | Radio Link | Channel model | Reference |
| --- | --- | --- | --- |
| TN+HAPS (UMa) | TN BS to TN UE | Urban Macro | TR 38.901 [9] |
| TN+HAPS (UMa) | HAPS to TN UE | NTN Urban | TR 38.811 [8] |
| TN+HAPS (UMa) | HAPS to HAPS UE | NTN Urban | TR 38.811 [8] |
| TN+HAPS (RMa) | TN BS to TN UE | Rural Macro | TR 38.901 [9] |
| TN+HAPS (RMa) | HAPS to TN UE | NTN Rural | TR 38.811 [8] |
| TN+HAPS (RMa) | HAPS to HAPS UE | NTN Rural | TR 38.811 [8] |
| HAPS+HAPS (RMa) | HAPS to HAPS UE | NTN Rural | TR 38.811 [8] |

## UE assumption

The same UE characteristics should be used for both TN UEs and HAPS UEs. It is reasonable to assume that UE has a single omni-directional antenna element with linear cross-polarizations. The UE assumptions is summarized in Table 7.

Table 7. UE assumption

| UE antenna array (M, N, P) | (1, 1, 2) |
| --- | --- |
| UE antenna element gain | 0 dBi, omni-directional |
| UE transmit power | 23 dBm |
| UE noise figure | 9 dB |

## Channel bandwidth and scheduled bandwidth

For FDD at 2 GHz band, 20 MHz channel bandwidth and 15 KHz subcarrier spacing is assumed. For downlink simulations, UEs are scheduled in a round-robin fashion and the scheduled UE is given the full bandwidth. For uplink simulations, attention shall be paid to the power limited nature of HAPS networks, so a smaller bandwidth is allocated to a HAPS UE than a TN UE. For the terrestrial network, 3 UEs are scheduled to share the UL channel bandwidth, with each UE being allocate 35 RBs. For the HAPS network, 9 UEs are scheduled in UL with 6 RBs for each UE, and the scheduled UEs are evenly distributed in the frequency domain as shown in Figure 7. Note that the scheduled resources are aligned among different cells to model co-channel effect. Table 8 lists the parameters of DL and UL transmission bandwidth for TN and HAPS.

Since the UE uses only part of the channel bandwidth in UL, the ACI impact may differ in the frequency domain of the adjacent channel. The ACIR model in TR 36.942 [10] should be used to model the ACI from aggressor UEs in UL, where the UE’s transmission bandwidth is taken into consideration. The model is illustrated in Figure 8, where same ACIR value for a UE’s ACI is applied to a bandwidth equal to the UE’s transmission bandwidth. Using an ACIR offset XACIR as a variable in the simulations, the model determines the ACIR by the frequency offset from the edge of the aggressor’s frequency resource. Given our UL scheduled bandwidth assumption, Table 9 and Table 10 should be used respectively for the TN UE and HAPS UE as aggressor.

Table 8. Proposed DL and UL transmission bandwidth

| Parameters | Downlink | Uplink |
| --- | --- | --- |
| Subcarrier spacing (SCS) | 15 KHz | 15 KHz |
| Channel bandwidth | 20 MHz | 20 MHz |
| Scheduled bandwidth per TN UE | 20 MHz | 35 RBs |
| Number of scheduled UEs per TN cell | 1 | 3 |
| Scheduled bandwidth per HAPS UE | 20 MHz | 6 RBs |
| Number of scheduled UEs per HAPS cell | 1 | 9 |

Figure 7. UL scheduled UE bandwidth allocation

Figure 8. UL ACIR model

Table 9. ACIR value with TN UL as aggressor

| Frequency offset between aggressor and victim | ACIR value |
| --- | --- |
| 0-34 RBs | 30 + XACIR |
| 35-69 RBs | 43 + XACIR |
| >69 RBs | 43+ XACIR |

Table 10. ACIR value with HAPA UL as aggressor

| Frequency offset between aggressor and victim | ACIR value |
| --- | --- |
| 0-5 RBs | 30 + XACIR |
| 6-11 RBs | 43 + XACIR |
| >11 RBs | 43+ XACIR |

## Uplink transmission power control model

Since terrestrial NR and HAPS networks have very different coverage and topology, UL power control setting for TN and HAPS should not be the same. The same power control model may be used but the UE transmit power should depend on the allocated bandwidth. To start the initial simulations, we can use the power control model with UE transmit power Pt determined according to

where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1

UEs connected to TN and HAPS networks may have different X (transmission BW) in this model. As described in section 2.6, scheduled bandwidth for TN UE and HAPS UE are respectively 35 RBs and 6 RBs, corresponding to X=6.3 and X=1.08. Parameters used in the UL power control model are listed in Table 11 below.

Table 11. UL power control parameters

| UL power control parameter | TN | HAPS |
| --- | --- | --- |
| Pmax (dBm) | 23 | 23 |
| Rmin (dB) | -54 | -54 |
| γ | 1 | 1 |
| X, transmission bandwidth (MHz) | 6.3 | 1.08 |
| Y, BS noise figure (dB) | 5 | 5 |

# Conclusion

It is proposed to use the simulation assumptions in this paper for NTN HAPS co-existence study.

# References

- R4-2108646, Simulation assumptions for HAPS co-existence, 3GPP RAN4#99-e.

- R4-2115751, Simulation assumptions for HAPS co-existence, 3GPP RAN4#100-e.

- R4-2120672, Simulation assumptions for HAPS co-existence, 3GPP RAN4#101-e.

- R4-2110800, Qualcomm, “HAPS coexistence simulation assumptions,” 3GPP RAN4#99-e.

- SoftBank, Loon LLC, Nokia, Ericsson, “Proposed deployment and system characteristics of HIBS in the working document towards a preliminary draft new Report ITU-R M.[HIBS-CHARACTERISTICS],” ITU WP-5D contribution, Sep. 28, 2020.

- R4-2110119, Ericsson, “NTN: coexistence studies – simulation assumptions discussion,” 3GPP RAN4#98bis-e.

- 3GPP TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects.”

- 3GPP TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks.”

- 3GPP TR 38.901, “ Study on channel model for frequencies from 0.5 to 100 GHz.”

- 3GPP TR 36.942, “Radio Frequency (RF) system scenarios.”

- 3GPP TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz.”