3GPP TSG-RAN WG4 Meeting #101-bis-e 											R4-2203006

Electronic Meeting, 17th Jan 2022 - 25th Jan 2022

Agenda item:			6.9.5.3

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on BS demodulation requirement for FR2 HST

Document for:	Approval

# Background

- The following WFs were approved previously

  - R4-2106102, WF for FR2 HST Demodulation, RAN4#98-bis, Samsung

  - R4-2017828, WF on NR support for HST in FR2”, Samsung. RAN4#97-e meeting

  - R4-2103240, WF on Deployment Scenario and UE RF Requirement for FR2 HST”, Samsung. RAN4#98-e meeting

  - R4-2106100, WF on FR2 HST Deployment scenario Analysis, Samsung, RAN4#98b-e meeting

  - R4-2106101, WF on Channel Modelling for FR2 HST, Nokia, RAN4#98b-e meeting

  - R4-2106102, WF on Demodulation requirement for FR2 HST”, Samsung, RAN4#98b-e meeting

  - R4-2108860, WF on FR2 HST Deployment Scenario Analysis, Samsung, RAN4#99-e meeting

  - R4-2108637, WF for FR2 HST Demodulation, RAN4#99, Samsung

  - R4-2115726, WF on FR2 HST demodulation, RAN4#100, Samsung

  - R4-2120703, WF on BS demodulation requirement for FR2 HST, RAN4#101, Nokia, Nokia Shanghai Bell, Samsung.

- Corresponding Email summary in RAN4#101-bis-e

  - R4-2203107	Email discussion summary for [101-bis-e][315] NR_HST_FR2_Demod_Part2.

# WF on topic#1: PUSCH requirements

## Test applicability rules and manufacturer declarations

### Issue 1-1-1: Test applicability

Candidate options:

- Option 2: FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported.

- Option 3: FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. 
If more than one DMRS configuration is declared to be supported, a pass with either of the possibilities is sufficient to demonstrate compliance to the core requirement. Capture this in a note to the performance requirement.

### Issue 1-1-2: Manufacturer declaration on HST FR2 DM-RS support - PUSCH

Agreement:

Define manufacturer declaration, applicable to BS Type 2-O, for PUSCH additional DM-RS in FR2 HST scenario.
The intention is that all combinations of pos0, pos1, and pos2 should be possible to declare.
Exact wording is FFS.

## MCS selection

### Issue 1-2-1: Receiver baseline assumption for simulation

Agreement:

Assume a receiver with post FFT FOC.

### Issue 1-2-1: MCS

Candidate options:

- Option 1: Only 20.

- Option 4: Only MCS16.

- Option 5: Both MCS16 and MCS20.
With applicability rule that that the PUSCH performance requirements needs to be tested only with highest supported MCS.

- Option 6: Continue simulation results alignment with range as MCS16-MCS20.

- Option 7: Use “MCS 20 only” as baseline. Change to “MCS 16 only”, if SNR after requirement derivation is larger than 20dB.

- Option 8: Continue simulation results alignment with MCS16 and MCS20 only.

## Requirement selection

### Issue 1-3-1: Requirement selection

Candidate options:

- Option 1 (Nokia, Intel): If a large span in the PUSCH simulations results still persists, RAN4 to verify and compare the PUSCH performance in a simpler channel model: AWGN + fixed maximum Doppler offset of 19458 Hz (without bi-directional propagation channel).

- Option 2 (CATT): Further check the span based on the consensus of Issue 1-2-1.

- Option 3 (HW): Apply the method discussed in Sub-topic 4-3 to derive the performance requirements directly.

Recommended WF:

Interested companies are invited to voluntarily bring results for the propagation conditions in option 1, at least for the test case {Post-FFT/[Pre-FFT]; Type B, 10 Symbols, 120kHz/200MHz; MCS20; DM-RS 1+1}.
The results are for alignment only and not intended for deriving the requirement

# WF on topic#2: PUSCH with UL timing adjustment requirements

## Test applicability rules and manufacturer declarations

### Issue 2-1-1: Test applicability

Agreement:

Share the same applicability rule with PUSCH.

### Issue 2-1-2: Manufacturer declaration on HST FR2 DM-RS support – UL TA

Agreement:

Share the same manufacturer declaration with PUSCH.

## FRC

Agreement:

FRC for FR2 HST PUSCH UL timing adjustment with different RS configuration as following

Table 2-1 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 4480 | 18432 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 3 |
| Code block size including CRC (bits) (Note 2) | 4504 | 6176 |
| Total number of bits per slot with PT-RS (Note 3) | 6624 | 27324 |
| Total symbols per slot with PT-RS | 1656 | 6831 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table 2-2 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 2976 | 16392 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 2992 | 8232 |
| Total number of bits per slot with PT-RS (Note 3) | 5888 | 24288 |
| Total symbols per slot | 1472 | 6072 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

Table 2-3 FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos2 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-Ax-xx | G-FR2-Ax-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 |
| Allocated resource blocks | 16 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 7 | 7 |
| Modulation | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 |
| Payload size (bits) | 3496 | 14344 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | 24 |
| Number of code blocks - C | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 3512 | 7208 |
| Total number of bits per slot with PT-RS (Note 3) | 5152 | 21252 |
| Total symbols per slot | 1288 | 5313 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0, l0= 0 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |

# WF on topic#3: PRACH requirements

No agreements.

# WF on topic#4: Other

## Requirement derivation

Agreement:

Use the legacy way to derive performance requirements as baseline:

Based on all contributed results, using outlier removal with [2] dB ideal span threshold / [4] dB impairment span threshold, and averaging result [R4-1904713] [R4-19004714].
The value of ideal span can be further discussed pending on the simulation results alignment.

## CR split (informative)

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.104 |  |  |
|  | Big CR | Samsung |
| 11 | Radiated performance requirements |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.2.2 | Requirements for BS type 2-O |  |
| 11.2.2.x | Requirements for PUSCH for high speed train | Intel |
| 11.2.2.y | Requirements for UL timing adjustment | CATT |
| 11.4 | Performance requirements for PRACH |  |
| 11.4.2 | Requirements for BS type 2-O |  |
| 11.4.2.2 | PRACH detection requirements |  |
| 11.4.2.2.x | Minimum requirements for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex G.3 | High speed train condition | Nokia |
| Annex G.4 | Moving propagation conditions | CATT |
| TS 38.141-2 |  |  |
|  | Big CR | Nokia |
| 4.6 | Manufacturer's declarations | Samsung, Nokia |
| 8 | Radiated performance requirements |  |
| 8.1.2 | Applicability rule |  |
| 8.1.2.4 | Applicability of PUSCH for high speed train performance requirements | Huawei |
| 8.2 | OTA performance requirements for PUSCH |  |
| 8.2.4 | Performance requirements for PUSCH for high speed train | Ericsson, Samsung |
| 8.2.5 | Performance requirements for UL timing adjustment | CATT |
| 8.4 | OTA performance requirements for PRACH |  |
| 8.4.1 | PRACH false alarm probability and missed detection |  |
| 8.4.1.6 | Test requirement for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex E | OTA measurement system set-up | Ericsson |
| Annex J.3 | High speed train condition | Nokia |
| Annex J.4 | Moving propagation conditions | CATT |