TSG-RAN Working Group 4 (Radio) meeting #101-bis-E	R4-2203017

Electronic Meeting, 17th – 25th January 2022

Source:	Ericsson

Title:	WF on BS RF Rx requirements

Agenda item:	6.16.4.2

Document for:	Approval

# Introduction

At RAN4#101-bis-E meeting BS RF receiver requirements was discussed. In this way-forward contribution an agreements and guidance with topics for further consideration is captured.

# Agreements

Based on the discussion captured in [1] the following agreements was captured.

2.1	Reference sensitivity

For reference sensitivity specify OTA reference sensitivity level as described in Table 2-1.

Table 2-1: Reference sensitivity requirement

| Frequency Range | BS channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
|  | 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
|  | 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| FR2-2 | 100,400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
|  | 400, 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + [9] + ΔFR2_REFSENS |
|  | 400, 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + [9] + ΔFR2_REFSENS |

2.2	FRC

Postpone further FRC discussion on EIS FRCs until SU is finalized.

2.3 	ACS

For ACS define interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 32 RBs. Confirm the PRB value once SU is agreed.

2.4	In-band blocking

For in-band blocking set the interferer level 33 dB higher than the OTA REFSENS level.

2.5 	Receiver spurious emissions

For FR2-2 set fOOB to 3.5 GHz.

For FR2-2 set Fstep,x according to Table 2-2.

Table 2-2: Step frequencies for defining the radiated Rx spurious emission limits for BS type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |
| [Licensed band] | [46] | [61] | [62.5] | [74.5] | [76] | [91] |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

2.6 	Receiver intermodulation

The interferer levels for general receiver intermodulation (25 dB higher than the OTA REFSENS level) of BS type 2-O should be used for NR operation in 52.6 – 71 GHz range.

2.7 In-channel selectivity

Agreement: Reusing existing FR2-1 requirements, with scaling according to CHBWs i.e.

EISREFSENS_wanted = EISREFSENS_50M +10*log10(BWwanted/ BW50M)+( SNRwanted - SNR50M)+3

EISREFSENS_interfer = EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+ ICS-0.9

Note: ICS in above equation = 14 dBc

# References

[1]	R4-2202973, “Email discussion summary for [101bis-e][309] NR_exto71GHz_BSRF”, Nokia