3GPP TSG-RAN WG4#101bis-e	R4-2203024

Electronic meeting, 17th – 25th January 2022

Source: 	Ericsson

Title:  	Way Forward on FR1 ACRR and OOB gain

Agenda Item:	6.5.2

Document for:	Approval

# Introduction

During RAN4#101bis-e, ACRR and OOB gain has been discussed in RAN4.

Two options were discussed, which are detailed below:

- The outcome of the GTW session is captured in section 2

- After further discussion on ACRR and OOB gain, a compromise as outlined in section 3 was presented.

These options are to be further discussed and decided at RAN4#102-e.

# Option 1: GTW outcome

## OOB gain

The OOB gain is differentiated between MR/WA and LA classes.

### WA / MR repeater class

- Agreement: For below 2GHz frequencies, follow same OOB gain as E-UTRA for both DL and UL.

- For above 2GHz frequencies and passband bandwidths above 20MHz

Agreement: with the values of frequency offset breaking points with [ ] pending on further confirmation:

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| [0,2]< f_offset_CW < [4,0] MHz | 60 dB |
| [4,0]< f_offset_CW < [15,0] MHz | 45 dB |
| [15,0] MHz <f_offset_CW | 35 dB |

- For above 2GHz frequencies and passband bandwidths <=20MHz,

The baseline assumption:

Use the requirements for passband bandwidth >20MHz together with 45dBc ACRR

- Note: ACRR requirements are not applicable if single collaborating operator holds the whole band.

### LA repeater class

Agreement:

The baseline assumption: define two set of OOB gain requirements:

- The first one: in which the operator owns the whole band or collaborates with operators in the whole band and so the repeater passband covers the whole 3GPP band

    - More relax OOB gain and ACRR requirements

      - No ACRR in this case (no adjacent channel; passband covers the whole band)

      - no OOB gain requirements for first 20MHz.

- The second one: in which there are other un-coordinated operators in the band and so the repeater passband covers a portion of the 3GPP band.

    - More stringent OOB gain and ACRR

      - Same OOB gain requirement as WA in this case

## ACRR

ACRR is differentiated between UL and DL

### UL

- Agreement: For ACRR, consider the co-existence scenario with NR, LTE and UTRAN system

  - The conformance test case(s) will be decided in conformance phase.

- Agreement: For UL, assume 33dB

  - Applicable for WA and MR

  - For LA (baseline assumption):

    - 33dB not applicable if “passband is whole band” scenario, but applicable if passband is part of band.

### DL

Agreement:

For DL: [45dB] ACRR for WA/MR and A class

  - Detailed Conformance test case(s) can be decided in conformance phase including whether separate or jointly test cases for ACRR, ACLR requirements and whether test case for OOB gain requirements enough

  - For LA class, the requirement not applicable if “passband is whole band” scenario, but applicable if passband is part of band.

# Option 2: Revised ACRR and OBUE

## OOB gain

The OOB gain is differentiated between MR/WA and LA classes.

### WA / MR repeater class

- Agreement: For below 2.5GHz frequencies, follow same OOB gain as E-UTRA for both DL and UL.

- For above 2.5GHz frequencies

Agreement: with the values of frequency offset breaking points with [ ] pending on further confirmation:

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| [0,2]< f_offset_CW < [4,0] MHz | 60 dB |
| [4,0]< f_offset_CW < [15,0] MHz | 45 dB |
| [15,0] MHz <f_offset_CW | 35 dB |

Use the requirements 33dBc ACRR for both DL and UL above 2.5GHz, 33dBc for UL below 2.5GHz and 45dBc for DL below 2.5GHz

- Note: ACRR requirements are not applicable if single collaborating operator holds the whole band.

### LA repeater class

Agreement:

The baseline assumption: define two set of OOB gain requirements:

- The first one: in which the operator owns the whole band or collaborates with operators in the whole band and so the repeater passband covers the whole 3GPP band

    - More relax OOB gain and ACRR requirements

      - No ACRR in this case (no adjacent channel; passband covers the whole band)

      - no OOB gain requirements for first 20MHz.

- The second one: in which there are other un-coordinated operators in the band and so the repeater passband covers a portion of the 3GPP band.

    - More stringent OOB gain and ACRR

      - Same OOB gain requirement as WA in this case

## ACRR

ACRR is differentiated between UL and DL

### UL

- Agreement: For ACRR, consider the co-existence scenario with NR, LTE and UTRAN system

  - The conformance test case(s) will be decided in conformance phase.

- Agreement: For UL, assume 33dB

  - Applicable for WA and MR

  - For LA (baseline assumption):

    - [20]dB not applicable if “passband is whole band” scenario, but if passband is part of band.

### DL

Agreement:

For DL: [33dB] ACRR for WA/MR and [20dBc] for LA class above 2.5GHz. [45] dB ACRR applicable for MR/WA below 2.5GHz

  - Detailed Conformance test case(s) can be decided in conformance phase including whether separate or jointly test cases for ACRR, ACLR requirements and whether test case for OOB gain requirements enough

  - For LA class, the requirement not applicable if “passband is whole band” scenario, but applicable if passband is part of band.