3GPP TSG-RAN WG4 Meeting # 101-bis-e                                                  R4-2203025

# Electronic Meeting, January 17-25, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on NF equivalent requirements for FR1

Agenda Item:	6.5.2

Document for:	Approval

# Background

In this way forward it is next steps on how to define NF equivalent requirements are proposed.

# Way forward

Earlier agreement is kept and maximum one requirement of low-power EVM and within passband OBUE is defined. To proceed further which requirement to define and how it is defined, feedback to below questions is requested.

Question 1: Which requirement should be pursued?

Agreement:

Include both Low-power EVM and within passband OBUE core requirements, meanwhile no conformance testing for low-power EVM.

Question 2: In case within passband OBUE is pursued, limit should be set considering?

Candidate options:

Option 1: adjacent PRBs (ref. UE in-band emission)

Option 2: adjacent carriers (ref. absolute ACLR)

Agreement:

For UL side, option 1 adopted

For DL side, option 2 adopted

Question 3: In case low-power EVM is pursued

Candidate options:

Option 1: specify input level for which EVM needs to be met together with channel BW and modulation

Option 2: declare input level for which EVM needs to be met together with channel BW and modulation

Option 3: define core requirement for input level range for which EVM needs to be met, but test only maximum power

Agreement:

Define core requirement for input level range for which EVM needs to be met, but test only maximum power

Still open:

Further discuss candidate values for the minimum input power level for which EVM needs to be met together with channel BW and modulation.

Proposed methodologies, still needing further discussion:

- Option 1: Set budged for termal noise and other sources assuming they are non-correlated; NF could be class-dependent. Derive input power level based on the budget.

- Option 2: state that the EVM shall be met for output power levels from maximum output power down to some margin, XdB below maximum output power

Question 4: Which further details are still missing from inside passband emission limit definition ?

Candidate options:

Option 1: actual limit in dBm, both for UL and DL

Option 2: please list other missing aspects

Proposed further methodologies to set the requirement:

- Option 1: use OBUE emission level

- Option 2: use absolute ACLR

- For both options

  - Level can could be different for UL and DL

  - Level could be different based on class

Question 5: Which further details are still missing from low-power EVM definition ?

Candidate options:

Option 1: input power level, signal bandwidth, used modulation and EVM-%

Option 2: please list other missing aspects

Further open aspects:

  - NF expectation

  - Allowed EVM degradation

  - How to make core requirement applicable for all bandwidths