3GPP TSG-WG RAN4 Meeting #101-bis-e	R4-2203029

Electronic Meeting, January 17-25, 2022

WF on CRS-IM in scenarios with overlapping spectrum for LTE and NR with 30kHz SCS

CMCC

- Background

WFs approved in the previous meetings:

[1]	R4-2108662, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR, RAN4 #99e.

[2]	R4-2115740, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR, RAN4 #100e.

[3]	R4-2120705, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR, RAN4 #101e.

- WF on CRS-IM Feasibility for 30kHz SCS Scenario

Application Scenario for 30kHz SCS

- Scenario 2 only for evaluation purpose

Feasibility of performing CRS-IM in 30 kHz SCS scenario

- Feasibility of LLR weighting with CRS sequence or CRS-IC

  - Not feasible

- Feasibility of LLR weighting without CRS sequence

  - The need of NWA signalling is discussed separately, together with Scenario2 15kHz SCS

  - FFS the feasibility in next meeting

Implementation of LLR weighting in 30 kHz SCS scenario

  - Option 1: Propose a LLR weighting method as follows

  - Acquire the CRS time domain location by power detection

  - Estimate the interference power distribution per RE level in the interfered symbols

  - Calculate the actual SINR on these interfered REs

  - Scaling the LLRs by post processing SINR value

  - Option 2: UE should do power estimation three times for three types of REs with different interference models with several RBs granularity or the whole bandwidth and perform LLR weighting respectively.

  - In the NR 30kHz+ LTE15kHz deployment, the main interference contributing to kth of NR subcarrier are 2kth, (2k-1)th and (2k+1)th LTE subcarrier.

    - Interference pattern 1: Subcarrier index 0, 3, 6, 9…,

    - Interference pattern 2: Subcarrier index 1, 4, 7, 10,…,

    - Interference pattern 3: Subcarrier index 2, 5, 8, 11…,

  - Option 3: Measure the interference power on all REs within the symbol colliding with CRS REs and make the averaging with 1 PRB granularity.

  - Intel: we will not observe that selected REs are affected by CRS interference due to loss of orthogonality

  - Option 4: Up to UE implementation

  - QC: Discuss the single DMRS scenario separately, since the front loaded DMRS will be totally interfered by CRS in 4 CRS port scenario.

- FFS which LLR weighting implementation is feasible

- FFS is it necessary to agree on one implementation as reference receiver based on the performance gain evaluation

CRS port number information

  - Option 1: UE verify the CRS port number and interfered symbols based on power comparison per symbol level.

  - Option 1a: Further analyse the feasibility of blind detection if testable performance will be observed with all parameters known to UE

  - Option 2: Need NWA since power detection is unreliable for higher SIR.

  - Option 3: Reuse the GTW agreement

  - From RAN4 minimum performance requirements aspect, UE follow below default assumption without blind detection as baseline assumption

    - 4 CRS ports for scenario 2

  - By default, number of CRS ports no need to be informed via signalling with following default assumption from RAN4 performance requirements aspect

  - Number of CRS ports information can be included into NWA signalling (optional)

- Other Options are not precluded

CRS frequency location information

  - Option 1: There is no need to identify the CRS frequency location in Scenario 2 with NR 30 kHz.

  - CMCC: Almost all the NR REs in interfered symbols will be interfered by LTE CRS.

  - Option 2: CRS location information should be configured with network assistance information.

  - Option 3: Further analyse the feasibility of blind detection if testable performance will be observed with all parameters known to UE

  - Option 4: Reuse the GTW agreement

  - The baseline assumption: No need to introduce NWA signaling for v-shift information

- WF on CRS-IM Test Setup for 30kHz SCS Scenario

CBW and SCS for target and interference cells

  - Option 1: If 30kHz SCS is feasible, use TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells

  - Other options are not precluded

TDD configuration for target cell with 30kHz SCS

  - Option 1: If 30kHz SCS is feasible, Use 7DS2U with S=6D+4G+4U for the target cell with TDD 30kHz SCS

  - Other options are not precluded

- WF on CRS-IM Performance Gain for 30kHz SCS Scenario

Colliding of the DM-RS in serving cell with CRS in neighbouring cell

- If CRS ports is 4, then the front loaded DMRS will be totally interfered by CRS.

- FFS investigate the potential performance impact by the above observation.

Performance gain of LLR weighting in 30 kHz SCS scenario

- For the next meeting, encourage companies to further evaluate whether testable CRS-IM performance improvement can be achieved for practical scenarios with 30 kHz serving signal SCS.

- Evaluate the scenario of DMRS 1+0 and DMRS 1+1 separately.

Simulation assumptions

| Parameters | Value |
| --- | --- |
| CRS ports for interference cells | Prioritize 4 CRS ports and interested companies can provide results for 2 CRS ports case |
| SCS/CBW for target cell | Option 1: 30kHz/20MHz |
| SCS/CBW for interference cells | Option 1: 15kHz/20MHz |
| TDD pattern for target cell | 7DS2U with S=6D+4G+4U |
| Other parameters | Reuse the parameters from 15kHz Scenario |

- WF on CRS-IM Capability Signalling for 30kHz SCS Scenario

Applicability of UE CRS-IM Capability

  - Option 1: FFS until feasibility to support CRS-IM for 30kHz SCS is confirmed

  - Option 2: Define separate UE capability for 30kHz SCS CRS-IM, FR1 only, no FDD/TDD difference