3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2203032

Electronic Meeting, 17 January – 25 January 2022

Agenda Item:	6.20.4

Source:	Ericsson

Title:	WF on RedCap demodulation and CQI reporting requirements

Document for:	Approval

# 1	BS demodulation requirements

- Not to discuss PUSCH requirements for coverage enhancements in RedCap WI. Companies should monitor Coverage Enhancements WI in case some new tests or performance requirements impact the RedCap WI

# 2	Common for UE demodulation and CSI reporting requirements for RedCap

## 2.1	Number of Rx antennas, CBW, duplex mode, and frequency range

- 1Rx RedCap UE: RAN4 define new UE demodulation and CSI reporting requirements with the following configuration:

  - FDD SCS=15kHz in FR1

    - Option 1: CBW=10MHz (keep the same assumption as Rel-15 NR UE demodulation)

    - Option 2: CBW=20MHz (maximum defined bandwidth for RedCap UE)

  - TDD SCS=30kHz in FR1

    - CBW=20MHz

  - TDD SCS=120kHz in FR2

    - CBW=100MHz

- 2Rx RedCap UE:

  - For FDD SCS=15kHz in FR1, RAN4 selects the existing UE demodulation and CSI reporting requirements with CBW=10MHz. FFS whether RAN4 need to define the new requirements for some test cases depending on the agreed scope of requirements.

  - For TDD SCS=120kHz in FR2, RAN4 selects the existing UE demodulation and CSI reporting requirements with CBW=100MHz. No new requirements are introduced.

  - For TDD SCS=30kHz in FR1, RAN4 define new UE demodulation and CSI reporting requirements with CBW=20MHz.

## 2.2		UL/DL pattern used for FDD tests

- 1Rx RedCap UE

  - Use Full Duplex FDD as the initial simulation assumption.

  - RAN4 discuss further whether to define requirements (e.g., FRC) based on Half Duplex FDD or not. If HD-FDD is used, consider the existing TDD pattern FR1.15-1 (i.e., DDDSU)

- 2Rx RedCap UE

  - Use Full Duplex FDD

  - RAN4 reuses the existing FDD requirements

## 2.3	How to define 2Rx FR1 TDD requirements for testing of 2Rx UE RedCap UEs

- Define 2Rx FR1 TDD demodulation (e.g., PBCH, PDCCH and PDSCH) and CSI requirements (e.g., CQI, PMI and RI) with channel bandwidth equal to 20MHz.

- Keep the requirements aligned between 2Rx and 1Rx.

# 3	UE demodulation requirements

Interested companies are encouraged to provide the simulation results for the agreed test cases in RAN4#102-e.

## 3.1	PDSCH

- RAN4 define PDSCH demodulation requirements for RedCap UE.

### 3.1.1	1Rx test cases

#### 3.1.1.1	FDD 15kHz FR1

- Option 1:

  - QPSK: Type A, Table 5.2.2.1.1-3 Test 1-1, QPSK 0.30, TDLB100-400, 2Tx, Rank 1, 70% max TP

  - 16QAM: Type A, Table 5.2.2.1.1-3 Test 1-4, 16QAM 0.48, TDLC300-100, 2Tx, Rank 1, 70% max TP

  - 64QAM: Type A, Table 5.2.2.1.1-4 Test 2-1, 64QAM 0.5, TDLA30-10, 2Tx, Rank 1, 70% max TP

  - FFS for 256QAM MCS24: Type A, Table 5.2.2.1.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP

- Option 2:

  - QPSK MCS4: Type A, Table 5.2.2.1.1-3 Test 1-2?, QPSK 0.3, TDLC300-100, 2Tx, Rank 1, 70% max TP

  - 256QAM MCS24: Type A, Table 5.2.2.1.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP

#### 3.1.1.2	TDD 30kHz FR1

- Option 1:

  - QPSK: Type A, Table 5.2.2.2.1-3 Test 1-1, QPSK 0.30, TDLB100-400, 2Tx, Rank 1, 70% max TP

  - 16QAM: Type A, Table 5.2.2.2.1-3 Test 1-4, 16QAM 0.48, TLDC300-100, 2Tx, Rank 1, 70% max TP

  - 64QAM: Type A, Table 5.2.2.2.1-4 Test 2-1, 64QAM 0.50, TLDA30-1, 2Tx, Rank 1, 70% max TP

  - FFS for 256QAM MCS24: Type A, Table 5.2.2.1.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP

- Option 2

  - 64QAM MCS19: Type A, Table 5.2.2.2.1-4 Test 2-1?, 64QAM 0.50, TLDA30-10, 2Tx, Rank 1, 70% max TP

  - 256QAM MCS24: Type A, Table 5.2.2.1.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP

#### 3.1.1.3	TDD 120kHz FR2

- Option 1:

  - QPSK: Type A, Table 7.2.2.2.1-3 Test 1-1, QPSK 0.30, TDLC60-300, 2Tx, Rank 1, 70% max TP

  - 16QAM: Type A, Table 7.2.2.2.1-4 Test 2-2, 16QAM 0.48, TDLA30-300, 2Tx, Rank 1, 70% max TP

  - 64QAM: Type A, Table 7.2.2.2.1-4 Test 2-6, 64QAM 0.42, TDLA30-75, 2Tx, Rank 1, 70% max TP

  - FFS for 256QAM MCS20: Type A, Table 7.2.2.2.1-3 Test 1-4?, 256QAM 0.76, TDLD30-75, 2Tx, Rank 1, 70% max TP

- Option 2:

  - 64QAM MCS17: Type A, Table 7.2.2.2.1-4 Test 2-6?, 64QAM 0.42, TDLA30-75, 2Tx, Rank 1, 70%

### 3.1.2	2Rx test cases

#### 3.1.2.1	FDD 15kHz FR1

- Option 1

  - QPSK: Type A, Table 5.2.2.1.1-3 Test 1-1, QPSK 0.30, TDLB100-400, 2Tx, Rank 1, 70% max TP (Reuse)

  - 16QAM: Type A, Table 5.2.2.1.1-3 Test 1-4, 16QAM 0.48, TDLC300-100, 2Tx, Rank 1, 70% max TP (New with 70% of max TP)

  - 64QAM: Type A, Table 5.2.2.1.1-4 Test 2-1, 64QAM 0.5, TDLA30-10, 2Tx, Rank 2, 70% max TP (Reuse)

  - FFS for 256QAM MCS24: Type A, Table 5.2.2.1.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP

- Option 2

  - QPSK MCS4: Type A, Table 5.2.2.1.1-3 Test 1-2?, QPSK 0.30, TDLC300-100, 2Tx, Rank 1, 70% max TP (Reuse)

  - 256QAM MCS24: Type A, Table 5.2.2.1.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP (Reuse)

#### 3.1.2.2	TDD 30kHz FR1

- Option 1

  - QPSK: Type A, Table 5.2.2.2.1-3 Test 1-1, QPSK 0.30, TDLB100-400, 2Tx, Rank 1, 70% max TP (New with CBW=20MHz)

  - 16QAM: Type A, Table 5.2.2.2.1-3 Test 1-4, 16QAM 0.48, TLDC300-100, 2Tx, Rank 1, 70% max TP (New with CBW=20MHz)

  - 64QAM: Type A, Table 5.2.2.2.1-4 Test 2-1, 64QAM 0.50, TLDA30-1, 2Tx, Rank 2, 70% max TP (New with CBW=20MHz)

  - FFS for 256QAM MCS24: Type A, Table 5.2.2.2.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP (New with CBW=20MHz)

- Option 2

  - 64QAM MCS19: Type A, Table 5.2.2.2.1-4 Test 2-1?, 64QAM 0.50, TLDA30-10, 2Tx, Rank 1, 70% max TP (New with CBW=20MHz)

  - 256QAM MCS24: Type A, Table 5.2.2.2.1-3 Test 1-3?, 256QAM 0.82, TDLA30-10, 2Tx, Rank 1, 70% max TP (New with CBW=20MHz)

#### 3.1.2.3	TDD 120kHz FR2

- Option 1

  - QPSK: Type A, Table 7.2.2.2.1-3 Test 1-1, QPSK 0.30, TDLC60-300, 2Tx, Rank 1, 70% max TP (Reuse)

  - 16QAM: Type A, Table 7.2.2.2.1-4 Test 2-2, 16QAM 0.48, TDLA30-300, 2Tx, Rank 2, 70% max TP (Reuse)

  - 64QAM: Type A, Table 7.2.2.2.1-4 Test 2-6, 64QAM 0.42, TDLA30-75, 2Tx, Rank 2, 70% max TP (Reuse)

  - FFS: 256QAM MCS20: Type A, Table 7.2.2.2.1-3 Test 1-4?, 256QAM 0.76, TDLD30-75, 2Tx, Rank 1, 70% max TP (Reuse)

- Option 2

  - 64QAM MCS17: Type A, Table 7.2.2.2.1-4 Test 2-6?, 64QAM 0.42, TDLA30-75, 2Tx, Rank 1, 70% max TP (Reuse)

## 3.2	PDCCH

- RAN4 define PDCCH demodulation requirements for RedCap UE.

- Not define PDCCH demodulation requirements with Power saving scenario for RedCap UE.

### 3.2.1	1Rx test cases

#### 3.2.1.1	FDD 15kHz FR1

- Option 1: AL8 only

- Option 2: AL4 and AL8. FFS for AL16

#### 3.2.1.2	TDD 30kHz FR1

- Option 1: AL4 only

- Option 2: AL4 and AL8. FFS for AL16

#### 3.2.1.3	TDD 120kHz FR2

- Option 1: AL16 only

- Option 2: AL4 and AL8. FFS for AL16

### 3.2.2	2Rx test cases

- Apply the same configuration as 1Rx UE tests.

## 3.3	PBCH

- RAN4 define PBCH demodulation requirements for RedCap UE.

### 3.3.1	1Rx test cases

- RAN4 define PBCH demodulation requirements by reusing the existing PBCH test setup for 2Rx with SS/PBCH block index is unknown.

- For the case with SS/PBCH block index is known,

  - Option 1: RAN4 define the case with SS/PBCH block index is known for 1Rx UE

  - Option 2: RAN4 don’t define the case with SS/PBCH block index is known for 1Rx UE

#### 3.3.1.1	Agreed test cases for FDD FR1

|  | CBW / SCS | Propagation condition | Antenna configuration | SS/PBCH block index | New / Reuse |
| --- | --- | --- | --- | --- | --- |
| Table 5.4.2.1-2 Test 1 | 10 / 15 | TDLC300-100 | 1x1 | Not known | New |

#### 3.3.1.2	Agreed test cases for TDD FR1

|  | CBW / SCS | Propagation condition | Antenna configuration | SS/PBCH block index | New / Reuse |
| --- | --- | --- | --- | --- | --- |
| Table 5.4.2.2-2 Test 1 | 20 / 30 | TDLC300-100 | 1x1 Low | Not known | New |

#### 3.3.1.3	Agreed test cases for TDD FR2

|  | CBW / SCS | Propagation condition | Antenna configuration | SS/PBCH block index | New / Reuse |
| --- | --- | --- | --- | --- | --- |
| Table 7.4.2.2-2 Test 1 | 100 / 120 | TDLA30-300 | 1x1 | Not known | New |
| Table 7.4.2.2-2 Test 2 | 100 / 240 | TDLA30-75 | 1x1 | Not known | New |

### 3.3.2	2Rx test cases

All the exiting PBCH demodulation requirements with 2Rx are applicable for RedCap 2Rx UE

#### 3.3.2.1	FDD FR1

|  | CBW / SCS | Propagation condition | Antenna configuration | SS/PBCH block index | New / Reuse |
| --- | --- | --- | --- | --- | --- |
| Table 5.4.2.1-2 Test 1 | 10 / 15 | TDLC300-100 | 1x2 Low | Not known | Reuse |
| Table 5.4.2.1-3 Test 1 | 10 / 15 | TDLC300-100 | 1x2 Low | Known | Reuse |

#### 3.3.2.2	TDD FR1

|  | CBW / SCS | Propagation condition | Antenna configuration | SS/PBCH block index | New / Reuse |
| --- | --- | --- | --- | --- | --- |
| Table 5.4.2.2-2 Test 1 | 20 / 30 | TDLC300-100 | 1x2 Low | Not known | Reuse (*1) |
| Table 5.4.2.2-3 Test 1 | 20 / 30 | TDLC300-100 | 1x2 Low | Known | Reuse (*1) |
| (*1) Keep the the same SNR requirements, but may need to change the channel bandwidth configuration. |  |  |  |  |  |

#### 3.3.2.3	TDD FR2

|  | CBW / SCS | Propagation condition | Antenna configuration | SS/PBCH block index | New / Reuse |
| --- | --- | --- | --- | --- | --- |
| Table 7.4.2.2-2 Test 1 | 100 / 120 | TDLA30-300 | 1x2 Low | Not known | Reuse |
| Table 7.4.2.2-2 Test 2 | 100 / 240 | TDLA30-75 | 1x2 Low | Not known | Reuse |
| Table 7.4.2.2-3 Test 1 | 100 / 120 | TDLA30-300 | 1x2 Low | Known | Reuse |
| Table 7.4.2.2-3 Test 2 | 100 / 240 | TDLA30-75 | 1x2 Low | Known | Reuse |

## 3.4	Sustained data rate (SDR)

- Option 1: Existing SDR test methodology can be reused for RedCap 1Rx/2Rx UE.

  - Discuss whether to update MCS configuration and common test parameters for 1 Rx RedCap UE testing

- Option 2: Not introduce SDR tests for RedCap 1Rx/2Rx UE

# 4	CSI reporting requirements

## 4.1		CQI reporting requirements

- Define CQI reporting tests for both static condition and fading condition for RedCap 1Rx/2Rx UE.

  - For the fading condition define wideband CQI reporting test only.

- Use CQI table 1 for RedCap UE CQI reporting requirements.

  - For 2Rx tests for FR1 FDD,

    - Define new 2Rx tests for FR1 FDD with CQI table 1

    - FFS whether to define test with CQI table 2 for Redcap UE supporting 256 QAM. If agreed, existing test requirements can be reused

### 4.1.1	1Rx test cases

#### 4.1.1.1	FDD FR1

| Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 | New |
| Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 | New |

#### 4.1.1.2	TDD FR1

| Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 | New |
| Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 | New |

#### 4.1.1.3	TDD FR2

| Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 1 | 2 x 1 | New |
| Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 1 | New |

### 4.1.2	2Rx test cases

#### 4.1.2.1	FDD FR1

| Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 Low | New for UE not supporting 256QAM |
| Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 2 | 2 x 2 Low | New for UE not supporting 256QAM |

#### 4.1.2.2	TDD FR1

| Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 Low | New |
| Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 2 | 2 x 2 Low | New |

#### 4.1.2.3	TDD FR2

| Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 2 | 2 x 2 | Reuse |
| Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 2 | Reuse |

## 4.2	PMI reporting requirements

- RAN4 discuss further whether to define PMI reporting tests for RedCap UE.

  - Option 1: Define for both 1Rx UE and 2Rx UE

  - Option 2: Not define for both 1Rx UE and 2Rx UE

  - Option 3 Define only for 2Rx UE

- If PMI reporting test is defined, discuss further applied test cases

## 4.3	RI reporting requirements for 2Rx UE

- RAN4 discuss further whether to define RI reporting tests for RedCap 2Rx UE.

  - Option 1: Define RI tests. It requires to rerun the simulation with CQI table 1 for FR1 FDD and TDD.

  - Option 2: Not define RI tests.