3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2203033

# Electronic meeting, January 17th – 25th, 2022

Source:			CATT

Title:			WF on RF requirement for SAN type 1-H

Agenda item:		6.13.3

Document for:		Approval

RF requirements for satellite access node

General

SAN abbreviation

Introduce SAN abbreviation for Satellite Access Node

Satellite access node types

Introduce 2 SAN types

- SAN type 1-H: 	Satellite Access Node operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

- SAN type 1-O: 	Satellite Access Node operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

Operating condition

For the TS 38.108 Core requirements drafting purposes, consider both Normal as well as Extreme testing conditions.

Contiguous operation v.s. NC operation

Only contiguous spectrum operation will be considered in NTN Rel-17.

Multi-band operation

Multi-band operation is not considered in NTN Rel-17.

Modulation quality (EVM)

Include 64QAM as optional with manufacture declaration basis for SAN.

Time alignment error

The TAE requirement is not needed for satellite BS in Rel-17.

Transmitter unwanted emissions

Specify satellite access node OBUE based on TN BS OBUE/TN MSR BC1 OBUE and scaling according to ACLR

Transmitter spurious emissions (general requirement)

Use the following spurious emission requirement as starting point.

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | 43 + 10 log(P) or 60 dBc, whichever is less stringent | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. |  |  |  |

Intra system transmitter intermodulation requirements shall be defined for NTN

FFS

References

[1] R4-2200167, Remaining issue for satellite access node RF requirement - Tx part, CATT

[2] R4-2200168, Remaining issue for satellite access node RF requirement - Rx part, CATT

[3] R4-2201318, NTN - Satellite Node Access - Tx requirements, Ericsson

[4] R4-2201319, NTN - Satellite Node Access - Rx requirements, Ericsson

[5] R4-2201470, Discussion on conducted Tx requirements of satellite access node, ZTE Corporation

[6] R4-2201471, Discussion on conducted RF requirements from satellite network perspective, ZTE

[7] R4-2201818, Discussion on Satellite Access Node conducted RF requirements: Tx, Huawei

[8] R4-2201819, Discussion on 64QAM support for the Satellite Access Node operation in FR1, Huawei, HiSilicon

[9] R4-2201820, Discussion on Satellite Access Node conducted RF requirements: Rx, Huawei