3GPP TSG-RAN WG4 Meeting # 101-bis-e                                                                                   R4-2203037

Electronic Meeting, 17– 25 January, 2022

Agenda item:	8.13.4.1

Source:	Huawei

Title:	TP for 38.863 on UE transmission characteristics for satellite access

Release:	Rel-17

Document for:	Approval

1	Introduction

Based on the TR 38.863 work split, we provided a text proposal for UE transmission characteristics for satellite access in this paper.

2	Text Proposal

<< Start of text proposal  >>

7.4.2	UE Transmission characteristics for satellite access

7.4.2.1 	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0dBi is assumed. Handheld PC3 UE is assumed in Rel-17 for satellite access.

7.4.2.2	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 7.4.2.1-1: UE Power Class

| NTN satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | +2/-33 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

7.4.2.3 	MPR/AMPR

[To be updated]

7.4.2.4 	Output power dynamics

For Transmit OFF power and Power control, the framework and requirements for existing TN UE can be reused for satellite UE. The transmit OFF power is -50 dBm for 20MHz.

Since handheld UE always transmits the maximum output power for satellite access scenario based on the NR NTN calibration summary in R4-2115628, FFS whether relaxed value needed or not based on existing TN UE requirements

7.4.2.5 	Frequency error

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the Satellite Access Node.

NOTE: The requirements is applicable only when PVT ephemeris updated at least once [10] seconds.

7.4.2.6 	Transmit modulation quality

[To be updated]

7.4.2.7 	Spectrum emission mask

[To be updated]

7.4.2.8 	ACLR

[To be updated]

7.4.2.9 	Spurious emissions

The spurious emissions limits specified in TS 38.101-1 sub-clause 6.5.3 would also be applicable to NTN satellite access UEs.

7.4.2.10 	Transmit intermodulation

The transmit intermodulation limits specified in TS 38.101-1 sub-clause 6.5.4 (Table 7.4.2.10-1) would also be applicable to NTN satellite access UEs.

The requirement of transmit intermodulation is specified in Table 7.4.2.10-1.

Table 7.4.2.10-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel center | BWChannel | 2*BWChannel |
| Interference CW signal level | -40 dBc |  |
| Intermodulation product | < -29 dBc | < -35 dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCS's for the channel BW as defined in Table 6.5.2.4.1-1 from TS 38.101-1 |  |
| Measurement offset from channel center | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |

<< end of text proposal  >>