3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                       R4-2203040

Electronic Meeting, January 17th - 25th, 2022

Title: 	TP to TR 38.863 on transmitter characteristics for satellite access node

Source: 	CATT

Agenda item:	6.13.1.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on transmitter characteristic for TR 38.863 [1] based on the approved WF in [2].

- Reference

[1] TR 38.863, 0.1.0

[2] R4-2120674, WF on Tx RF requirements for satellite access node, , CATT

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

7.3	Satellite access node requirements

[Note: Titles of Section 7.3, 7.3.1.1 need to be aligned with the name to be discussed and determined in TS 38.181 and 38.08]

7.3.1	General

NTN products are assumed to use AAS BS architecture, with both 1-H and 1-O being included in Rel-17 WI. Therefore, both conducted, as well as radiated requirements are required to be considered.

7.3.1.1	Satellite access node class

[To be updated]

7.3.2	Transmission characteristics

7.3.2.1	General

<Reference point needs to be added in this section>

7.3.2.2	Conducted transmitter requirements

7.3.2.2.1	Base station output power

7.3.2.2.2	Output power dynamics

7.3.2.2.2.1	RE power dynamic range

The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power for a specified reference condition.

RE power control dynamic range requirement is define in Table 7.3.2.2.2.1-1.

Table 7.3.2.2.2.1-1: RE power control dynamic range

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the satellite access node. |  |  |

7.3.2.2.2.2	Total power dynamic range

Total power dynamic range for SAN will reuse the requirement from TN BS for the same channel bandwidth.

The downlink (DL) total power dynamic range for each carrier shall be larger than or equal to the level in table 7.3.2.2.2.2-1.

Table 7.3.2.2.2.2-1: Total power dynamic range

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.9 | 10.4 | N/A |
| 10 | 17.1 | 13.8 | 10.4 |
| 15 | 18.9 | 15.7 | 12.5 |
| 20 | 20.2 | 17 | 13.8 |

7.3.2.2.3	Transmitted signal quality

7.3.2.2.3.1	Frequency error

For SAN type 1-H, the modulated carrier frequency of each carrier configured by the satellite access node shall be accurate to within 0.05ppm observed over 1 ms. .

7.3.2.2.3.1	Modulation quality (EVM)

It is agreed to support QPSK and 16QAM for SAN. 64QAM will be optionally supported based on manufacture declaration.

For SAN type 1-H, the EVM levels of each carrier for different modulation schemes on PDSCH outlined in table 7.3.2.2.3.1-1 shall be met.

Table 7.3.2.2.3.1-1: EVM requirements for satellite access node type 1-H

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| NOTE: Support of 64QAM is based on manufacture declaration |  |

7.3.2.2.3	Unwanted emissions

7.3.2.2.3.1	ACLR

7.3.2.2.3.2	Operating band unwanted emissions

7.3.2.2.3	Transmitter spurious emission

---------------------------------------------------End of Text proposal---------------------------------------------------------