3GPP TSG-RAN WG4 Meeting #101-bis-e 					 R4-2203042

Electronic Meeting, 17th – 25th Jan., 2022

Title:	WF on general and NTN UE demodulation requirements

Source:	Qualcomm Incorporated

Agenda Item:	6.13.6.1& 6.13.6.3

Document for:	Approval

# Introduction

According to the email discussion summary in [1], this document is to capture the WF on general and NTN UE demodulation requirements.

# WF on general aspects

- Issue 1-1-1: Architecture

- Agreement:

  - The architecture defined by RAN3 as baseline. Follow the agreement to only consider transparent architecture for RAN4 demodulation part discussion.

- Issue 1-1-2: Band and frequency

- Agreement:

  - Only consider FR1 demodulation requirements in Rel-17

- Issue 1-1-3a: Frequency/time error model:

- Agreement:

  - Satellite companies are encouraged to deliver a practical frequency/timing model for service link and feeder link

  - FFS on whether to consider the floating boundary model due to the satellite motion

- Issue 1-1-3b: Power model

- Agreement:

  - Satellite companies are encouraged to deliver a practical power model, e.g., fixed gain or fixed PSD

- Issue 1-1-4: Earth fixed beam and moving beam

- Agreement:

  - No difference between earth fixed beam and moving beam from demodulation perspective.

  - RAN4 to only consider one set of cases to cover both earth fixed beam and moving beam.

- Issue 1-1-5: General assumptions for service link and feeder link

- Agreement:

  - For UL and DL, RAN4 to further discuss the Doppler shift and Delay spread after UE time and frequency compensation for service link and feeder link.

- Issue 1-1-6: UE speed on NTN demodulation

- Agreement

  - Up to 120km/h is the start point for UE speed. Further discuss whether need to define the UE speed for NTN demodulation.

# WF NTN UE demodulation requirements

## 3.1 	WF on scope of requirements

- Issue 3-1-1: Legacy TN UE Rel-15/Rel-16 demodulation requirements apply to NTN UE or not?

  - Agreement:

    - Legacy TN UE Rel-15/Rel-16 demodulation requirements shall also apply for NTN UE.

- Issue 3-1-2: Satellite-based NTN and HAPS scenarios

  - Agreement:

    - Further discuss whether RAN4 to define the sperate requirements for GEO and LEO. Lower priority for HAPS.

- Issue 3-1-3: Enhancement on time relationship

  - Agreement:

    - RAN4 to discuss how to select K_offset to ensure UL/DL timeline is met

- Issue 3-1-4: Enhancement on HARQ

  - Agreement

    - Define PDSCH performance requirements with HARQ Processes 32

      - Define applicability rule that the PDSCH performance requirements with HARQ Processes 32 shall apply for UEs which support RAN1 UE feature “26-5	Increasing the number of HARQ processes” only.

    - Define PDSCH performance requirements with disabled HARQ. The disable HARQ processes number is FFS.

      - FFS applicability rule after RAN1 make agreement on UE feature about the disabled HARQ.

    - FFS on test method for PDSCH test with disabled HARQ feedback. The candidate options are:

      - Option 1: Disabled HARQ can be tested with the number of re-Tx set to 1

      - Option 2: other option is not precluded.

## 3.2 	WF on general assumptions

- Issue 3-2-1: Channel model

  - Agreement:

    - RAN4 to select one NLOS channel model from NTN-TDL-A/B and one LOS channel model from NTN-TDL-C/D.

- Issue 3-2-2: Delay spread

  - Agreement:

    - RAN4 to further study the Delay spread after UE pre-compensation

    - RAN4 to further study time-varying timing drift model due to the satellite motion.

- Issue 3-2-3: Doppler shift

  - Agreement:

    - RAN4 to further study the Doppler shift after UE pre-compensation

    - RAN4 to further study time-varying Doppler drift model due to the satellite motion.

## 3.3 	WF on PDSCH/PDCCH/PBCH assumptions

- Issue 3-3-1: PDCSH/PDCCH/PBCH requirements

  - Agreement:

    - RAN4 to define new requirements for PDSCH

    - FFS on whether to define new requirements for PBCH and PDCCH

- Issue 3-3-2: SCS/CBW set for PDSCH requirements

  - Agreement:

    - To consider 15kHz SCS, 10MHz. FFS on 30kHz and CBW

- Issue 3-3-3: Modulation order

  - Agreement:

    - Consider QPSK and 16QAM for PDSCH. FFS on 64QAM

## 3.4		WF on CSI reporting

- Issue 3-4-1: CSI reporting requirements

  - Agreements:

    - Do not consider PMI and RI reporting requirements, FFS for CQI reporting for LEO.

# Reference

[1]	R4-2202972, Email discussion summary for [101-bis-e][322] NR_NTN_Demod