3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2203044

Electronic Meeting, 17 January – 25 January 2022

Agenda Item:	6.6.5.1

Source:	Ericsson

Title:	Way forward for NR DL1024QAM demodulation and CQI reporting requirements

Document for:	Approval

# PDSCH demodulation requirements

- Interested companies are encouraged to provide the PDSCH simulation results according to Table 1 in the next meeting to decide the SNR test points.

- RAN4 will discuss whether to add extra margin after deriving the SNR test points to achieve 70% of maximum throughput based on the impairment results.

Table 1	Simulation parameters for PDSCH demodulation for DL 1024QAM in FR1.

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLD30-5 Option 2: TDLA30-10 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

# SDR requirements

- Add the MCS indexes for SDR test with 1024QAM based on Table 2 for 2Rx.

  - Rank 1

    - Option 1: MCS25

    - Option 2: MCS23

  - Rank 2

    - Option 1: MCS23

    - Option 2: Not define SDR tests

- Add the MCS indexes for SDR test with 1024QAM based on Table 3 for 4Rx.

  - Rank 1

    - Option 1: MCS25

    - Option 2: MCS23

  - Rank 2

    - Option 1: MCS23

    - Option 2: FFS

- Interested companies are encouraged to propose MCSpracticalj to decide MCSreqj, assuming Tx EVM 2.5% where

  - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

- Simulation parameters: TS38.101-4 Table 5.5A-1

Table 2	MCS indexes for SDM test with 1024QAM for 2Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | TBD | TBD |
| 1 | 10 | 0.8 | 21 |  | TBD |
| 1 | 10 | 0.75 | 19 |  | TBD |
| 1 | 10 | 0.4 | 9 |  | TBD |
| 2 | 10 | 1 | 25 | TBD | TBD |
| 2 | 10 | 0.8 | 21 |  | TBD |
| 2 | 10 | 0.75 | 19 |  | TBD |
| 2 | 10 | 0.4 | 9 |  | TBD |

Table 3	MCS indexes for SDM test with 1024QAM for 4Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | TBD | TBD |
| 1 | 10 | 0.8 | 21 |  | TBD |
| 1 | 10 | 0.75 | 19 |  | TBD |
| 1 | 10 | 0.4 | 9 |  | TBD |
| 2 | 10 | 1 | 25 | TBD | TBD |
| 2 | 10 | 0.8 | 21 |  | TBD |
| 2 | 10 | 0.75 | 19 |  | TBD |
| 2 | 10 | 0.4 | 9 |  | TBD |

# CQI reporting tests

- Define new CQI definition requirements for CQI table with 1024QAM entries for 2Rx/4Rx UEs.

- Interested companies are encouraged to evaluate the following test setup.

  - Test parameters: Reuse the existing CQI definition tests

  - Rank 1

  - Tx EVM assumed for simulation: Aligned with PDSCH demodulation requirements.

  - Test metric: Reuse the existing CQI definition tests

  - Test points: One SNR test point corresponding to CQI index 14

    - 2Rx UE:

      - Option 1: SNR = 28/29dB

      - Option 2: SNR = 29/30dB

      - Other options not precluded

    - 4Rx UE:

      - Option 1: SNR = 25/26dB

      - Option 2: SNR = 26/27dB

      - Other options not precluded

- RAN4 will discuss whether to add extra margin on top of the SNR test point, after RAN4 derives the SNR test points for CQI tests.

# CR work split

| Clause | CR | Volunteer company |
| --- | --- | --- |
| 5.1.1.x 6.1.1.x [B.2.1.1] (*) | Applicability of demodulation requirements Applicability of CSI reporting requirements [Delay profiles for FR1] | Qualcomm |
| 5.2.2.1.x 5.2.3.1.x A.3.2.1.x | PDSCH demodulation requirements for FDD (2Rx and 4Rx) FRC for PDSCH FDD | MediaTek |
| 5.2.2.2.x 5.2.3.2.x A.3.2.2.x | PDSCH demodulation requirements for TDD (2Rx and 4Rx) FRC for PDSCH TDD | Apple |
| 6.2.2.1.x 6.2.2.2.x A.4 | CQI reporting definition under AWGN conditions for FDD (2Rx and 4Rx) CSI reference measurement channels | Huawei |
| 6.2.3.1.x 6.2.3.2.x 5.5A | CQI reporting definition under AWGN conditions for TDD (2Rx and 4Rx) Sustained downlink data rate provided by lower layers | Ericsson |

(*) If TDLD30 is agreed