3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2203062

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			WF on remaining aspects for BS RF requirements for RMR 900/1900

Agenda item:		5.4.3, 5.5.3

Document for:		Approval

# RMR 900MHz

## From RAN4#100-e

Note: The following was discussed in RAN4#100-e meeting, companies agreed on those statement but they were not captured in an appoved document.

Agreements:

- To convert the CEPT EIRP limits in conducted limits assuming a 17dBi antenna gain and capture those conducted limits in TS 38.104 together with the antenna gain assumption.

- To capture in TS 38.104 that for BS operating in n100, for uncoordinated deployment, the BS rated output power Prated,c,AC  shall not exceed: 47.5dBm/5MHz + (fDL-922.1) x 40/3 dB (considering a 17 dBi antenna gain, and with fDL being the centre frequency in MHz).

## Deployment aspects

Agreement:

Coordinated/un-coordinated deployment aspects are not to be captured in the TS.

## OBUE requirement

Agreement:

Capture additional OBUE requirement based on ECC Decision (20)02.

## Tx Spurious

Agreement:

The -66dBm/5MHz limit is confirmed as additional Tx spur limit and will be captured in TS 38.104 (assuming 17dBi antenna gain).

## General in-block requirement

Agreement:

The general in-block requirement specified in ECC Decision(20)02 will not be captured in TS 38.104

## Rx blocking

Way Forward:

According to ECC Decision(20)02, the interferer should be a 200kHz signal but FFS what exact signal it is:

- Option 1: GSM

- Option 2: 1 RB from 5MHz NR signal

- Other

# RMR 1900MHz

## From RAN4#100-e

Note: The following was discussed in RAN4#100-e meeting, companies agreed on those statement but they were not captured in an appoved document.

Agreements:

- To convert the CEPT EIRP limits in conducted limits assuming a 18dBi antenna gain and capture those conducted limits in TS 38.104 together with the antenna gain assumption.

- To capture in TS 38.104 that for BS operating in n101, the BS rated output power Prated,c,AC  shall not exceed shall not exceed 47 dBm/10MHz.

- To capture in TS 38.104 the additional spurious limit for band n101 in TS 38.104:

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | -61 dBm (NOTE) | 5 MHz |
| NOTE: considering a 18 dBi antenna gain |  |  |

## Deployment aspects

Agreement:

Coordinated/un-coordinated deployment aspects are not to be captured in the TS.

## Tx Spurious

Agreement:

-61dBm/5MHz confirmed as additional Tx spur limit and will be captured in TS 38.104 (assuming 18dBi antenna gain).

## Rx blocking

Agreement

Type of interferer: LTE (as specified in ECC Decision(20)02).

# References

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2201324 | RMR 900 MHz - BS RF | Ericsson |
| R4-2201329 | RMR 1900 MHz - BS RF | Ericsson |
| R4-2201805 | Cross-check on the implementation of ECC Decision (20)02 for RMR900 | Huawei |
| R4-2201806 | draft CR to TS 38.104: consideration of regional requirements for RMR900 | Huawei |
| R4-2201807 | Remaining aspects for BS RF requirements for RMR900 | Huawei |
| R4-2201811 | Consideration of coordinated/un-coordinated deployments for the RMR900 requirements in RAN4 specifications | Huawei |
| R4-2201812 | Cross-check on the implementation of ECC Decision (20)02 for RMR1900 | Huawei |
| R4-2201813 | Remaining aspects for BS RF requirements for RMR1900 | Huawei |
| R4-2201815 | Consideration of coordinated/un-coordinated deployments for the RMR1900 requirements in RAN4 specifications | Huawei |