3GPP TSG-RAN WG4 Meeting #101-bis-e		R4-2203070
Electronic Meeting, 17th – 25th Jan., 2022

Agenda item:	6.2.4

Source:	OPPO

Title:	WF on TRP TRS for UE with multi-antenna

Document for:	Approval

Topic 1: Test methodology for UE with multi-antenna

Sub-topic 1-1: Figure of Metric for the UE radiated performance with TAS ON

Agreement:

- RAN4 do not define new Figure of Metric for the UE transmit radiated performance with TAS ON.

Sub-topic 1-2: Priorities of the influenced factors on TAS ON

Agreement:

- Update the table of the influenced factors with priority as below.

| Factor ID | Potential Influence factors | Priority |
| --- | --- | --- |
| Factor 1 | Downlink Rx signal | High |
| Factor 2 | Near-body/object sensor | Low |
| Factor 3 | USIM card setting | Low |
| Factor 4 | Base station signalling | TBD |
| Factor 5 | Particular optimization algorithms | TBD |
| Factor 6 | Efficacy of the TAS ON methodology (i.e. ability to rank devices based on lab test to be correlated to ranking based on field performance) | NA |

- Introducing a dwell or wait time between the change in DL direction and the power measurement to ensure the switching state stable in the measurement procedure will be considered.

Sub-topic 1-3: Test methodology for UE with Tx Diversity

Agreement:

- Further discuss the test method for TxD UEs with Method A and Method B as a start point.

  - Method A: Make UE worked in the test mode which keep UE transmitting with one fixed antenna. Test TRP of one of UE antennas named as TRP1. And then keep UE transmitting with the other antenna, and test TRP of the other antenna named as TRP2. Take the sum of TRP1 and TRP2 to get the TRP for the UE with Tx Diversity.

  - Method B: Make UE worked in Tx Diversity mode. Test TRP with both of the antennas transmitting together. It is the TRP for the UE with Tx Diversity.

- Phase difference changing in TxD is FFS.

Topic 2: Test time reduction

Sub topic 2-1 Single Point Offset method to reduce TRP TRS OTA test time

WF:

- The single point offset method can be further discussed.

References

[1] R4-2202956_Summary_326_after_1st round, Moderator (OPPO), RAN4#101-bis-e, Jan., 2022

[2] R4-2203098 Email discussion summary for [101-bis-e][326] FR1_TRP_TRS_Part2, Moderator(OPPO), RAN4#101-bis-e, Jan, 2022